3GPP TSG RAN WG1 Meeting #123	R1-2508526

Dallas, US, Nov 17th – 21st, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on enhancing DL CSI acquisition

Agenda Item:	10.2.2

Document for:	Discussion and Decision

- Introduction

In RAN#109 meeting, a revised WID for NR MIMO Phase 6 was approved for Rel-20 [1]. Among items in the WID, two aspects corresponding to DL CSI acquisition enhancement were captured, i.e., early SRS/CSI/CSI-RS triggering, and CSI-RS frequency density reduction. Detailed objectives are as follows:

- On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - On phase continuity, RAN4 to study and conclude by RAN#111 whether the requirements from the legacy RAN4 spec for DMRS bundling can be retained. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

In this contribution, we elaborate our views on the above two aspects, respectively.

- Early SRS/CSI/CSI-RS triggering

In the section, we will provide our further considerations on early SRS/CSI-RS/CSI triggering under the following three scenarios based on the agreements reached in RAN1#122 bis meeting.

- Scenario-1: UE transitioning from IDLE/INACTIVE to CONNECTED mode.

- Scenario-2: UE transitioning from SCell deactivation to activation.

- Scenario-3: UE transitioning from SCell dormancy state to non-dormancy state.

  - Scenario-1: UE transitioning from IDLE/INACTIVE to CONNECTED mode

In RAN1#122bis meeting, the following procedure was agreed to achieve early SRS-AS CSI-RS/CSI triggering for scenario-1: UE transition from IDLE/INACTIVE to CONNECTED mode.

Agreement (RAN1#122bis) For early triggering of SRS-AS when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple SRS configurations based on one or multiple ‘xTyR’ UE capability assumptions. - At least one SRS configuration(s) resource sets for SRS-AS in an SRS configuration is provided for a same ‘xTyR’ - FFS: Information to be provided in SRS configuration(s) - FFS: Whether Configuration information could be SRS configuration or the CSI report configuration for PMI-free report. - FFS: Whether SRS configuration can be associated with CSI report configuration for PMI-free report. - In Step-2, UE reports through MSG3 which/whether the SRS configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the SRS configuration(s) provided in SIBx is/are supported. Agreement (RAN1#122bis) For early triggering of aperiodic CSI reporting and the associated CSI-RS for CSI when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple CSI report configurations based on one or multiple UE capability assumptions - Each CSI reporting configuration is associated with a CSI resource configuration for channel measurement. - The CSI-RS resource set(s) in the CSI resource configuration for channel measurement is provided with a same number of CSI-RS ports - Each CSI reporting configuration is associated with a CSI resource configuration for CSI-IM based interference measurement. - FFS: Support of NZP CSI-RS based interference measurement - FFS: Information to be provided in a CSI report/resource configuration - In Step-2, UE reports through MSG3 which/whether the CSI report configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the CSI report configuration(s) provided in SIBx is/are supported.

On top of the above procedure, more details need to be further clarified.

SRS-AS/CSI-RS/CSI configuration

Last RAN1 meeting, it is agreed that one or multiple SRS/CSI report configuration(s) for early SRS-AS/CSI-RS/CSI triggering are carried in SIBx. However, it is still pending on resource and/or reporting configuration-related information being included in SIBx.

To trigger CSI-RS/SRS-AS/CSI reporting by MSG4, SIBx needs to provide the necessary SRS and/or CSI reporting configurations. For SRS-AS, in order to support UEs with different xTyR capabilities, NW should offer one or more SRS configurations corresponding to each xTyR UE capability assumption. Regarding PMI-based CSI reporting, since the maximum supported number of CSI-RS ports differs across UEs, SIBx should include at least one CSI reporting configuration for each UE capability assumption(s) of supported # of CSI-RS ports. For PMI-less CSI reporting, SIBx can also provide one or more CSI reporting configurations. For each case as above mentioned, the amount of configuration(s) provided by SIBx should balance signaling overhead and configuration flexibility, further study may be needed to optimize this balance.

Regarding the above issue, we consider that SIBx should include the following information:

- For SRS-AS, one or multiple SRS configuration(s) for each of a list of xTyR AS UE capability assumption(s), where each SRS configuration includes one or multiple SRS resource set(s).

- For PMI-based CSI reporting, one or multiple CSI report configuration(s) for each of a list of UE capability assumption(s) of supported # of CSI-RS ports.

- For PMI-less CSI reporting, one or multiple CSI report configuration(s).

Note that, to reduce broadcast signaling overhead, SIBx should only provide UE-common resource/report configuration information, e.g., number of SRS/CSI-RS ports, codebook configuration, etc.

Proposal 1: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), SIBx provides at least the following information for SRS-AS/CSI-RS/CSI:

- For SRS-AS, one or multiple SRS configuration(s) for each of a list of xTyR AS UE capability assumption(s).

- For PMI-based CSI reporting, one or multiple CSI report configuration(s) for each of a list of UE capability assumption(s) of supported # of CSI-RS ports.

- For PMI-less CSI reporting, one or multiple CSI report configuration(s).

Proposal 2: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), SIBx only provides UE-common resource/report configuration information, such as, the number of SRS/CSI-RS ports, codebook configuration, etc.

UE Capability indication in MSG 3 PUSCH

Regarding capability indication, RAN1 has agreed that UE reports via MSG3 which/whether the SRS/CSI report configuration(s) provided in SIBx is/are supported. For the specific method of UE indicating supported configuration(s), our views are as follows.

The scope of early SRS/CSI-RS/CSI triggering has been specified in the WID. Specifically, for SRS, the scope is limited to SRS for antenna switching. For CSI-RS reporting, the scope is limited to aperiodic wideband CSI reporting with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via legacy UCI. Therefore, UE capability can be indicated in the following manner:

- For AS-SRS, UE indicates supported xTyR AS UE capability assumption(s).

- UE supports all SRS configurations associated with the indicated xTyR AS UE capability assumption(s).

- For PMI-based CSI reporting, UE indicates the supported maximum number of CSI-RS ports, or alternatively, a supported CSI report configuration corresponding to the supported maximum number of CSI-RS ports.

- UE supports all CSI report configurations with the number of CSI-RS ports of the CMR not exceeding the maximum supported by UE.

- For PMI-less CSI reporting, UE only reports whether this type of CSI reporting is supported or not.

Proposal 3: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), UE capability indication in MSG3 can be provided as follows:

- For AS-SRS, UE indicates supported xTyR AS UE capability assumption(s).

- UE supports all SRS configurations associated with xTyR AS UE capability assumption(s) indicated in MSG3.

- For PMI-based CSI reporting, UE indicates the supported maximum number of CSI-RS ports, or alternatively, a supported CSI report configuration corresponding to the supported maximum number of CSI-RS ports.

- UE supports all CSI report configurations with the number of CSI-RS ports of the CMR not exceeding the maximum supported by UE.

- For PMI-less CSI reporting, UE only reports whether this type of CSI reporting is supported or not.

TRS for tracking

For TRS, RAN1 has reached the following agreement to achieve early SRS-AS/CSI-RS/CSI triggering for scenario-1:  UE transition from IDLE/INACTIVE to CONNECTED mode.

Agreement (RAN1#122bis) For early TRS reception when UE transition from IDLE/INACTIVE to CONNECTED mode, down-select one from the followings in RAN1#123 meeting: - Alt-1: Support aperiodic TRS triggered by MSG4 only - Alt-2: Support periodic TRS triggered by MSG4 and aperiodic TRS triggered by MSG4 - Alt-3: Support periodic TRS triggered by MSG4 only

Regarding TRS, periodic TRS triggered by MSG4 does not demonstrate any additional benefit over aperiodic TRS triggered by MSG4. If the goal is to trigger/initiate TRS transmission via MSG4, it is straightforward that we only use aperiodic TRS for greater flexibility instead of further introducing periodic TRS in such case. That is, we do NOT support periodic TRS triggered by MSG4. As each UE may have different requirements of the number of TRS bursts for tracking, UE should indicate via MSG 3 how many TRS bursts are needed, where the corresponding candidate values may be none, one, or two.

Proposal 4: For early SRS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), support only AP TRS triggered by MSG4.

- UE can indicate via MSG3 the number of AP TRS burst(s), with candidate values of none, one or two, supported by the UE for early CSI.

Triggering mechanism for AP SRS-AS/CSI-RS/CSI

Regarding triggering mechanism, the following relevant agreement was reached in RAN1#122bis meeting:

Agreement (RAN1#122bis) For UE transition from IDLE/INACTIVE to CONNECTED mode, using MAC CE in the PDSCH for MSG4 to trigger early aperiodic SRS-AS/CSI-RS/CSI - Note: How to determine the PUSCH for aperiodic CSI reporting is separately discussed.

We believe that when MAC CE in the PDSCH carrying MSG4 is used to trigger AP SRS/CSI-RS/CSI, the PUSCH for AP CSI reporting should also be indicated in the MAC CE to avoid additional signaling overhead and latency. Therefore, the following information should be carried in the MAC CE included in the PDSCH carrying MSG4:

- Triggered TRS/SRS/CSI-RS resource/resource set or CSI report;

- UL-grant information;

- Additional UE specific resource/report configuration information, e.g., CSI-RS triggering offset, RBs for CSI-RS transmission, SRS triggering offset, RBs for SRS transmission, RI restriction, etc.

Proposal 5: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), MAC CE triggering aperiodic SRS/CSI-RS/CSI in the PDSCH carrying MSG4 should indicate the following information:

- Triggered TRS/SRS/CSI-RS resource/resource set or CSI report;

- UL-grant information;

- Additional UE specific resource/report configuration information, e.g., CSI-RS triggering offset, RBs for CSI-RS transmission, SRS triggering offset, RBs for SRS transmission, RI restriction, etc.

Timeline for AP SRS-AS/CSI-RS/CSI

For the reference time point for triggering AP SRS-AS, AP CSI-RS, and AP CSI reporting , one of the following alternatives can be considered, which is different from the legacy DCI triggered CSI/CSI-RS/SRS:

- Option-1: The slot of the PDSCH carrying MSG4.

- Option-2: The slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying MSG4 to trigger early CSI-RS/CSI/SRS.

- Option-3: 3ms after the slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying MSG4 to trigger early CSI-RS/CSI/SRS.

If UE successfully decodes the PDSCH carrying MSG4, any of the three options works. While if UE fails to decode the PDSCH carrying MSG4, the following issues may arise. The SRS/CSI-RS/CSI or PUSCH triggering/scheduling information indicated by MAC CE in retransmitted PDSCH carrying MSG4 (e.g., trigger offset, UL-grant, CSI-RS/SRS Resource/Resource set) may NOT be valid any more, and UE cannot successfully transmit/receive/report the RS/report. For this case, we believe that UE should discard the SRS/CSI report triggered by MAC CE in the retransmitted PDSCH carrying MSG4. That is, if UE detects a non-toggled ‘New Data Indicator’ field in the DCI triggering the PDSCH carrying MSG4, which means that the PDSCH is retransmitted, the SRS/CSI-RS/CSI report triggered by MAC CE in the PDSCH should be dropped.

Proposal 6: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), reference time point for triggering AP SRS/CSI-RS/CSI can be selected from on one of the following options:

- Option-1: The slot of the PDSCH carrying MSG4.

- Option-2: The slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying MSG4 to trigger CSI-RS/CSI/SRS.

- Some of candidate value(s) on triggering offset to AP CSI-RS/CSI/SRS can be less than 3ms (for early triggering), besides for being larger than or equal to 3 ms.

- Option-3: 3ms after slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying MSG4.

- FFS: timeline restrictions.

Proposal 7: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), if UE detects a non-toggled ’New Data Indicator’ field in the DCI triggering the PDSCH carrying MSG4, the SRS/CSI-RS/CSI report triggered by MAC CE in the PDSCH carrying MSG4 should be dropped.

  - Scenario-2: UE transitioning from SCell deactivation to activation

In RAN1#122bis meeting, the following relevant agreement was reached for scenario-2: UE transition from SCell deactivation to activation.

Agreement (RAN1#122bis) On triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting for a SCell, based on the legacy SCell activation activating the SCell, down-select one from the followings in RAN1#123 meeting: - Alt-1 (Implicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to RRC configuration. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to RRC configuration - Alt-2 (Explicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to an indication in SCell activation command. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command. Agreement (RAN1#122bis) For SCell transition from deactivation to activation, support the followings for aperiodic CSI reporting triggered based on legacy SCell activation command: - For PMI-based reporting, support the report quantity ‘CRI-RI-PMI-CQI’ - FFS: ‘CRI-RI-LI-PMI-CQI’ - For PMI-free reporting, support the report quantity ‘CRI-RI-CQI’

Triggering mechanism for AP SRS-AS/CSI-RS/CSI

For scheduling flexibility, we prefer Alt2: the SRS resource set(s)/CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command. We tend to reuse the enhanced SCell activation MAC CE to trigger AP SRS/CSI-RS/CSI. After checking the existing specification, we notice that only one reserved bit in the existing enhanced SCell Activation/Deactivation MAC CE as in Figure 6.1.3.55-1 and Figure 6.1.3.55-2 of clause 6.1.3.55 of TS 38.321 can be used to indicate new feature or function. Based on this, we believe the new fields need to be introduced in the Scell activation MAC CE, and the reserved bit in the existing enhanced SCell Activation/Deactivation MAC CE can be used to indicate whether additional field(s) exist for triggering SRS/CSI-RS/CSI.

#### TS 38.321 #### 6.1.3.55 Enhanced SCell Activation/Deactivation MAC CEs The Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of seven C-fields, one R-field and zero or more TRS IDj fields in ascending order based on the ScellIndex for SCells indicated by the Ci field(s) to be activated. The Enhanced SCell Activation/Deactivation MAC CE of with one octet Ci field is defined as follows (Figure 6.1.3.55-1). The Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of 31 C-fields, one R-field and zero or more TRS IDj fields in ascending order based on the ScellIndex for SCells indicated by the Ci field(s) to be activated. The Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field is defined as follows (Figure 6.1.3.55-2). - Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [5], this field indicates the activation/deactivation status of the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated and that a TRS IDj field is included for the SCell. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated and that no TRS ID field is included for this SCell; - TRS IDj: If TRS IDj is set to a non-zero value, it indicates the corresponding TRS address by scellActivationRS-Id as specified in TS 38.331 [5] is activated. If TRS IDj is set to zero, it indicates that no TRS is used for the corresponding SCell; - R: Reserved bit, set to 0. Figure 6.1.3.55-1: Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field Figure 6.1.3.55-2: Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field

Similar to the triggering mechanism in Scenario-1, the enhanced SCell Activation MAC CE should indicate the following information when used to trigger aperiodic SRS/CSI/CSI-RS:

- Triggered SRS/CSI-RS resource/resource set or CSI report;

- UL-grant information.

Proposal 8: For early SRS-AS/CSI-RS/CSI triggering in scenario-2 (UE transitioning from SCell deactivation to activation), strive to reuse the enhanced SCell activation/Deactivation MAC CE to trigger AP SRS/CSI-RS/CSI.

- New fields need to be introduced in the enhanced SCell activation/Deactivation MAC CE.

- The reserved bit in the enhanced SCell Activation/Deactivation MAC CE can be used to indicate whether additional field(s) exist for triggering SRS/CSI-RS/CSI.

Proposal 9: For early SRS-AS/CSI-RS/CSI triggering in scenario-2 (UE transitioning from SCell deactivation to activation), strive to reuse the triggering mechanism in scenario-1, the enhanced SCell Activation MAC CE that triggers aperiodic SRS/CSI-RS/CSI should indicate the following information:

- Triggered SRS/CSI-RS resource/resource set or CSI report;

- UL-grant information.

Timeline for AP SRS-AS/CSI-RS/CSI

For starting point of triggering offset to AP CSI-RS transmission, AP CSI reporting and AP SRS-AS, the following options can be considered:

- Option-1: The slot of the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

- Option-2: The slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

- The triggering offset to CSI-RS/CSI/SRS can be smaller than 3ms (for early triggering) in addition to being larger than or equal to 3ms.

- Option-3: 3ms after the slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

Option-4: A common offset relative to the slot where the enhanced SCell Activation/Deactivation MAC CE is received.

Furthermore, if the UE fails to decode the PDSCH carrying the enhanced SCell activation MAC CE, for the same reason as discussed in Scenario-1, any SRS/CSI-RS/CSI reporting triggered by that MAC CE should be dropped.

Proposal 10: For early SRS-AS/CSI-RS/CSI triggering in scenario-2 (UE transitioning from SCell deactivation to activation), the starting point of triggering offset to AP SRS transmission and AP CSI-RS measurement and AP CSI reporting can be determined based on one of the following potential options:

- Option-1: The slot of the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

- Option-2: The slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

- Some of candidate value(s) on triggering offset to CSI-RS/CSI/SRS can be smaller than 3ms (for early triggering), besides for being larger than or equal to 3ms.

- Option-3: 3 ms after the slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

- Option-4: A common offset relative to the slot where the enhanced SCell Activation/Deactivation MAC CE is received.

Proposal 11: For early SRS-AS/CSI-RS/CSI triggering in scenario-2 (UE transitioning from SCell deactivation to activation), if UE detects a non-toggled ’New Data Indicator’ field in the DCI triggering the PDSCH carrying the enhanced SCell activation MAC CE, the SRS/CSI-RS/CSI report triggered by the enhanced SCell activation MAC CE should be dropped.

  - Scenario-3: UE transitioning from SCell dormancy state to non-dormancy state

In legacy, it has been specified that UE should stop monitoring PDCCH on/for the SCell, but continues performing CSI measurement if configured according to the definition of dormant BWP in clause 3.1 of TS 38.321.

#### 3.1 Definitions Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signaling. In the dormant BWP, UE stops monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured.

Similar to scenario-1 and 2, at least AP CSI reporting, AP SRS and AP CSI-RS should be supported for scenario-3.

To minimize spec impact, legacy DCI formats should be reused as much as possible for early SRS/CSI-RS/CSI triggering:

- For AP CSI-RS/CSI triggering: Legacy DCI formats 0_1/0_3 with ‘SCell dormancy indication’ and ‘CSI request’ fields can support simultaneous switching out of dormancy and aperiodic CSI reporting for one or multiple SCells. Specifically, “CSI request” field in the DCI format 0_1/0_3 received on PCell can indicate an aperiodic trigger state, where the aperiodic trigger state is associated with multiple aperiodic CSI reporting configurations provided on PCell and each aperiodic CSI reporting configuration can be linked with a CSI-RS resource configuration on the non-dormant BWP of a SCell of these SCells, and then the aperiodic CSI report(s) including measurement results from one or more SCells is/are transmitted on the PCell. So, no enhancement is needed for triggering aperiodic CSI-RS/CSI based DCI format 0_1/0_3 containing ‘SCell dormancy indication’ and ‘CSI request’ fields.

- For AP SRS triggering: Legacy DCI formats 0_1/0_3/1_1/1_3 include both ‘SCell dormancy indication’ and ‘SRS request’ fields. However, for DCI formats 0_1/0_3/1_1/1_3 received on PCell, the ‘SRS request’ field is limited to the PCell and cannot trigger AP SRS for a SCell. Thus, to enable joint triggering of AP SRS and AP CSI-RS/CSI via the existing DCI (e.g., DCI format 0_1/0_3) received on PCell, each AP CSI-RS resource set configured on the non-dormant BWP of a SCell can be configured with an associated AP SRS resource set. For example, when an AP CSI-RS for a SCell is triggered by a DCI format 0_1/0_3 received on PCell, the SRS associated with the AP CSI-RS will be also triggered simultaneously.

Proposal 12: For early SRS-AS/CSI-RS/CSI triggering in scenario-3 (UE transitioning from SCell dormancy state to active state), strive to reuse existing DCI formats (e.g., DCI formats 0_1/0_3) to trigger early SRS/CSI-RS/CSI, where

- For AP CSI-RS/CSI triggering

- CSI-RS measurement and CSI report for out-of-dormancy BWP of SCells can be triggered by ‘CSI request’ field in DCI formats 0_1/0_3 based on legacy mechanism, thereby no enhancement is needed.

- For AP SRS triggering

- Each AP CSI-RS resource set can be configured with an associated AP SRS resource set.

- When the AP CSI-RS for an out-of-dormancy BWP of a SCell is triggered by DCI formats 0_1/0_3, the associated AP SRS for the out-of-dormancy BWP of the SCell is triggered simultaneously.

- CSI-RS frequency density reduction

  - Aggregation of CSI-RS resources with lower frequency density

As stated in the WID, to support backward compatibility, introducing lower CSI-RS frequency densities should NOT impact the legacy specification for rate matching on CSI-RS REs. More specifically, for a cell where exist both legacy and new UEs, NW may configure and transmit two sets of CSI-RS. For example, NW may transmit a first CSI-RS with 32 ports and 1/2 frequency density, and a second CSI-RS with 128 ports and 1/8 frequency density. Then the legacy UEs need to perform rate matching on the REs occupied by the second CSI-RS based on ZP CSI-RS resources. It should be noted that, the legacy UEs only support frequency densities of 3, 1, 1/2 for ZP CSI-RS resource. To avoid resource waste, the REs occupied by the second CSI-RS should match with those occupied by a ZP CSI-RS resource with frequency density 1 or 1/2 (frequency density 3 is only applicable for CSI-RS for beam management or CSI-RS for tracking).

To further elaborate the above issue, Figure 1 gives two examples where a legacy UE performs rate matching on the REs occupied a 128-port CSI-RS with frequency density 1/8. In both cases, the 128-port CSI-RS is achieved by aggregating 4 32-port NZP CSI-RS resources, in case#0 the NZP CSI-RS resources are distributed on even or odd RBs in every 8 RBs, while in case#1 the NZP CSI-RS resources are distributed on the first or second 4 RBs in every 8 RBs. Then in cases #0 and #1, a legacy UE need to perform rate matching on REs corresponding to a ZP CSI-RS with frequency density 1/2 and 1, respectively. It can be observed that, the NZP CSI-RS aggregation pattern in case #1 causes additional resource waste.

Figure 1 Example cases of rate matching for low-frequency-density CSI-RS performed by legacy UEs

Figure 2 shows possible aggregation patterns of low-frequency-density NZP CSI-RS resources to avoid similar cases as case #1. It can be observed that,

- In patterns #0 and #3, the REs occupied by the aggregated low-frequency-density NZP CSI-RS resources are same as those occupied by a ZP CSI-RS resource with frequency density of 1;

- In patterns #1, #2, and #6, the REs occupied by the aggregated low-frequency-density NZP CSI-RS resources are same as a ZP CSI-RS resource with frequency density of 1/2;

- In patterns #4 and #5, the REs occupied by NZP CSI-RS resources #0 and #2, or NZP CSI-RS resources #1 and #3, are same as those occupied by a ZP CSI-RS resource with frequency density 1/2. Note that, in pattern #4, the two NZP CSI-RS resources in one RB can be either TDMed or FDMed.

(a)                    (b)                     (c)                      (d)

(e)                            (f)                            (g)

Figure 2 Possible NZP CSI-RS aggregation patterns

From the above patterns, it can be concluded that, the number of aggregated low-frequency-density NZP CSI-RS resources N and the CSI-RS frequency density 𝜌 should satisfy the condition that Nρ = 1 or 1/2. Besides, the aggregated low-frequency-density NZP CSI-RS resources should be allocated in one or two consecutive slots as legacy.

Proposal 13: For CSI-RS overhead reduction for 48, 64, and 128 ports, the REs occupied by aggregated low-frequency-density NZP CSI-RS resources should be same as those occupied by one or more ZP CSI-RS resources with frequency density of 1 or 1/2.

- The number of aggregated low-frequency-density NZP CSI-RS resources N and the CSI-RS frequency density 𝜌 should satisfy the condition that Nρ = 1 or 1/2.

- The aggregated low-frequency-density NZP CSI-RS resources should be allocated in one or two consecutive slots.

Subsequently, Table 1 lists possible combinations of CSI-RS frequency density 𝜌, the number of aggregated NZP CSI-RS resources N, the number of ports in each NZP CSI-RS resource X, the number of total CSI-RS ports PCSI-RS, and the aggregation pattern.

Table 1 Possible combinations of CSI-RS frequency density 𝜌, the number of aggregated CSI-RS resources N, the number of ports in each aggregated CSI-RS resource X, the number of total CSI-RS ports PCSI-RS, and the aggregation pattern

|  | 𝜌 | N | X | PCSI-RS | Aggregation pattern |
| --- | --- | --- | --- | --- | --- |
| 0 | 1/3 | 3 | 16 | 48 | #0 |
| 1 | 1/6 | 3 | 16 | 48 | #1 |
| 2 | 1/4 | 2 | 24 | 48 | #2 |
| 3 | 1/4 | 2 | 32 | 64 | #2 |
| 4 | 1/4 | 4 | 16 | 64 | #3~5 |
| 5 | 1/4 | 4 | 32 | 128 | #3~5 |
| 6 | 1/8 | 4 | 16 | 64 | #6 |
| 7 | 1/8 | 4 | 32 | 128 | #6 |

  - CSI reporting based on low-frequency-density CSI-RS

Regarding CSI reporting based on low-frequency-density, at least Rel-19 Type-I and eType-II codebooks should be supported. Then, both wideband and subband CSI reporting should be supported for Rel-19 Type-I or eType-II codebooks.

For subband CSI reporting based on Rel-19 Type-I and eType-II codebooks, it is expected that there exists at least K (K ≥ 1) channel measurements in each subband/PMI subband, to guarantee valid CSI calculation. Herein, K (K ≥ 1) could be a predefined integer. The above requirements can be expressed as

- For Rel-19 Type-I codebook,

NRBSBρ≥K

- For Rel-19 eType-II codebook,

NRBSBρR≥K

where NRBSB is the number of RBs in a subband, ρ is the CSI-RS frequency density, and R is the number of PMIs in each subband for eType-II codebook.

Proposal 14: For CSI-RS overhead reduction for 48, 64, and 128 ports, at least Rel-19 Type-I and eType-II codebooks should be supported for CSI reporting based on low-frequency-density CSI-RS,

- Both wideband and subband CSI reporting should be supported for Rel-19 Type-I or eType-II codebooks,

  - For subband CSI reporting based on Rel-19 Type-I and eType-II codebooks, there should exist at least K (K ≥ 1) channel measurements in each subband/PMI subband.

Additionally, if the subband size does not match the frequency density 𝜌, the number of REs/RBs occupied different CSI-RS ports may be different within a subband. As the example given in Figure 3, when 𝜌 = 1/3 (with CSI-RS aggregation patterns #0 in figure 2) and subband size is 4 PRBs, each CSI-RS port in resource #0 occupies 2 REs in subband #0, while each CSI-RS port in resource #1/2 occupies 1 RE in subband #0. In this case, it will bring accuracy degradation of CSI calculation or unnecessary UE implementation complexity of CSI for reporting. Besides, the restriction on configuration of csi-ReportingBand in TS 38.214 should be maintained, i.e., ‘a UE is not expected to be configured with csi-ReportingBand which contains a subband where a CSI-RS resource linked to the CSI Report setting has the frequency density of each CSI-RS port per PRB in the subband less than the configured density of the CSI-RS resource’. Therefore, to address these issues, new subband size values should be introduced in addition to the legacy ones, as listed in the table 2.

Figure 3 Illustration of mismatch between frequency density and subband size

Table 2 Configurable subband sizes

| Bandwidth part (PRBs) | Subband size (PRBs) |
| --- | --- |
| 24 – 72 | 4, 6, 8 |
| 73 – 144 | 8, 12, 16 |
| 145 – 275 | 16, 24, 32 |

Proposal 15: For CSI-RS overhead reduction for 48, 64, and 128 ports, introduce new subband size values, i.e., 6/12/24 PRBs, for the frequency-domain densityρ = 1/3 or 1/6 to guarantee the restriction of configuration of csi-ReportingBand in current spec, where

- Subband size of 6 PRBs apply for bandwidth part of 24 – 72 PRBs;

- Subband size of 12 PRBs apply for bandwidth part of 73 – 144 PRBs;

- Subband size of 24 PRBs apply for bandwidth part of 145 – 275 PRBs.

Moreover, considering the buffering and calculation resources would be dramatically reduced for CSI processing when the CSI-RS frequency density is reduced, the number of occupied CPUs, the timeline requirements (Z/Z’), and the number of resources/ports counted as activated, can be reduced for a CSI report procedure.

Proposal 16: For CSI-RS overhead reduction for 48, 64, and 128 ports, further study whether/how the number of occupied CPUs, the timeline requirements (Z/Z’), and the number of resources/ports counted as activated, should be reduced for a CSI report procedure.

- Conclusion

In summary, we have the following observations and proposals on enhancing DL CSI acquisition:

Proposal 1: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), SIBx provides at least the following information for SRS-AS/CSI-RS/CSI:

- For SRS-AS, one or multiple SRS configuration(s) for each of a list of xTyR AS UE capability assumption(s).

- For PMI-based CSI reporting, one or multiple CSI report configuration(s) for each of a list of UE capability assumption(s) of supported # of CSI-RS ports.

- For PMI-less CSI reporting, one or multiple CSI report configuration(s).

Proposal 2: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), SIBx only provides UE-common resource/report configuration information, such as, the number of SRS/CSI-RS ports, codebook configuration, etc.

Proposal 3: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), UE capability indication in MSG3 can be provided as follows:

- For AS-SRS, UE indicates supported xTyR AS UE capability assumption(s).

- UE supports all SRS configurations associated with xTyR AS UE capability assumption(s) indicated in MSG3.

- For PMI-based CSI reporting, UE indicates the supported maximum number of CSI-RS ports, or alternatively, a supported CSI report configuration corresponding to the supported maximum number of CSI-RS ports.

- UE supports all CSI report configurations with the number of CSI-RS ports of the CMR not exceeding the maximum supported by UE.

- For PMI-less CSI reporting, UE only reports whether this type of CSI reporting is supported or not.

Proposal 4: For early SRS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), support only AP TRS triggered by MSG4.

- UE can indicate via MSG3 the number of AP TRS burst(s), with candidate values of none, one or two, supported by the UE for early CSI.

Proposal 5: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), MAC CE triggering aperiodic SRS/CSI-RS/CSI in the PDSCH carrying MSG4 should indicate the following information:

- Triggered TRS/SRS/CSI-RS resource/resource set or CSI report.

- UL-grant information.

- Additional UE specific resource/report configuration information, e.g., CSI-RS triggering offset, RBs for CSI-RS transmission, SRS triggering offset, RBs for SRS transmission, RI restriction, etc.

Proposal 6: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), reference time point for triggering AP SRS/CSI-RS/CSI can be selected from on one of the following options:

- Option-1: The slot of the PDSCH carrying MSG4.

- Option-2: The slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying MSG4 to trigger CSI-RS/CSI/SRS.

- Some of candidate value(s) on triggering offset to AP CSI-RS/CSI/SRS can be less than 3ms (for early triggering), besides for being larger than or equal to 3 ms.

- Option-3: 3ms after slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying MSG4.

- FFS: timeline restrictions.

Proposal 7: For early SRS-AS/CSI-RS/CSI triggering in scenario-1 (UE transitioning from IDLE/INACTIVE to CONNECTED mode), if UE detects a non-toggled ’New Data Indicator’ field in the DCI triggering the PDSCH carrying MSG4, the SRS/CSI-RS/CSI report triggered by MAC CE in the PDSCH carrying MSG4 should be dropped.

Proposal 8: For early SRS-AS/CSI-RS/CSI triggering in scenario-2 (UE transitioning from SCell deactivation to activation), strive to reuse the enhanced SCell activation/Deactivation MAC CE to trigger AP SRS/CSI-RS/CSI.

- New fields need to be introduced in the enhanced SCell activation/Deactivation MAC CE.

- The reserved bit in the enhanced SCell Activation/Deactivation MAC CE can be used to indicate whether additional field(s) exist for triggering SRS/CSI-RS/CSI.

Proposal 9: For early SRS-AS/CSI-RS/CSI triggering in scenario-2 (UE transitioning from SCell deactivation to activation), strive to reuse the triggering mechanism in scenario-1, the enhanced SCell Activation MAC CE that triggers aperiodic SRS/CSI-RS/CSI should indicate the following information:

- Triggered SRS/CSI-RS resource/resource set or CSI report.

- UL-grant information.

Proposal 10: For early SRS-AS/CSI-RS/CSI triggering in scenario-2 (UE transitioning from SCell deactivation to activation), the starting point of triggering offset to AP SRS transmission and AP CSI-RS measurement and AP CSI reporting can be determined based on one of the following potential options:

- Option-1: The slot of the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

- Option-2: The slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

- Some of candidate value(s) on triggering offset to CSI-RS/CSI/SRS can be smaller than 3ms (for early triggering), besides for being larger than or equal to 3ms.

- Option-3: 3 ms after the slot of a PUCCH or PUSCH with HARQ-ACK information corresponding to the PDSCH carrying the enhanced SCell Activation/Deactivation MAC CE.

- Option-4: A common offset relative to the slot where the enhanced SCell Activation/Deactivation MAC CE is received.

Proposal 11: For early SRS-AS/CSI-RS/CSI triggering in scenario-2 (UE transitioning from SCell deactivation to activation), if UE detects a non-toggled ’New Data Indicator’ field in the DCI triggering the PDSCH carrying the enhanced SCell activation MAC CE, the SRS/CSI-RS/CSI report triggered by the enhanced SCell activation MAC CE should be dropped.

Proposal 12: For early SRS-AS/CSI-RS/CSI triggering in scenario-3 (UE transitioning from SCell dormancy state to active state), strive to reuse existing DCI formats (e.g., DCI formats 0_1/0_3) to trigger early SRS/CSI-RS/CSI, where

- For AP CSI-RS/CSI triggering

- CSI-RS measurement and CSI report for out-of-dormancy BWP of SCells can be triggered by ‘CSI request’ field in DCI formats 0_1/0_3 based on legacy mechanism, thereby no enhancement is needed.

- For AP SRS triggering

- Each AP CSI-RS resource set can be configured with an associated AP SRS resource set.

- When the AP CSI-RS for an out-of-dormancy BWP of a SCell is triggered by DCI formats 0_1/0_3, the associated AP SRS for the out-of-dormancy BWP of the SCell is triggered simultaneously.

Proposal 13: For CSI-RS overhead reduction for 48, 64, and 128 ports, the REs occupied by aggregated low-frequency-density NZP CSI-RS resources should be same as those occupied by one or more ZP CSI-RS resources with frequency density of 1 or 1/2.

- The number of aggregated low-frequency-density NZP CSI-RS resources N and the CSI-RS frequency density 𝜌 should satisfy the condition that Nρ = 1 or 1/2.

- The aggregated low-frequency-density NZP CSI-RS resources should be allocated in one or two consecutive slots.

Proposal 14: For CSI-RS overhead reduction for 48, 64, and 128 ports, at least Rel-19 Type-I and eType-II codebooks should be supported for CSI reporting based on low-frequency-density CSI-RS,

- Both wideband and subband CSI reporting should be supported for Rel-19 Type-I or eType-II codebooks,

  - For subband CSI reporting based on Rel-19 Type-I and eType-II codebooks, there should exist at least K (K ≥ 1) channel measurements in each subband/PMI subband.

Proposal 15: For CSI-RS overhead reduction for 48, 64, and 128 ports, introduce new subband size values, i.e., 6/12/24 PRBs, for the frequency-domain densityρ = 1/3 or 1/6 to guarantee the restriction of configuration of csi-ReportingBand in current spec, where

- Subband size of 6 PRBs apply for bandwidth part of 24 – 72 PRBs;

- Subband size of 12 PRBs apply for bandwidth part of 73 – 144 PRBs;

- Subband size of 24 PRBs apply for bandwidth part of 145 – 275 PRBs.

Proposal 16: For CSI-RS overhead reduction for 48, 64, and 128 ports, further study whether/how the number of occupied CPUs, the timeline requirements (Z/Z’), and the number of resources/ports counted as activated, should be reduced for a CSI report procedure.

- References

- RP-252510, Revised WID: NR MIMO Phase 6, Moderator(Samsung)