3GPP TSG-RAN WG1 Meeting #123	R1-2508537

Dallas, USA, Nov 17th – 21st, 2025

Source:	TCL

Title:	Discussion on other aspects for CSI compression

Agenda item:	10.1.1.2

Document for:	Discussion and Decision

Introduction

The study items on Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface[1][2] explored the benefits of augmenting the air-interface with features enabling improved support of AI/ML. The project studied the 3GPP framework for AI/ML for air-interface corresponding to spatial/temporal/frequency CSI compression (hereinafter referred to as AI/ML-based CSI compression) regarding aspects such as performance, complexity, and potential specification impact.

As for the use case of AI/ML-based CSI compression, the CSI spatial/frequency compression without temporal aspects (Case 0) is recently approved as one objective in the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements in RAN#108 meeting[3]. Furthermore, the WID identifies the following specification support aspects related to AI/ML-based CSI compression:

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air, including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

In the recent RAN1#122-bis meeting, there were full discussions on data collection, monitoring, and model paring. Some conclusions were drawn as follows[9]:

Agreement: For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantization), consider the following definition for evaluation: - Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall 2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2. - Note: Companies to report whether real value and imaginary value are quantized, or amplitude value and phase value are quantized. - Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part. Uniform quantization is assumed. - Note: Companies to report whether/how the normalization is performed, and whether it is normalized within or across 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts. - Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values. - Note: It is up to company to use number of samples S>1 in their evaluations. In this case, companies need to provide information how S samples are quantized in their evaluation. Agreement: For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection. - L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits - Company to report the quantization step for reference amplitude for 6bits. - No restriction to the total number of non-zero coefficients for a specific rank. - Other combinations can be considered and reported by companies. Agreement: For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered: - Benchmark#1: FP32 (k1=k2=32 bits) - Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2) - CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer. - Tx port: 32, 64, or 128 up to companies. - Subband number: 13, or 19 up to companies. - Performance metric: - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option. - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution - Overhead per sample and per layer SGCS results are reported per layer - Note: Complexity should be analysed. - For Option 1, consider k1+k2=4, 6 ,7, 8, 16, 32 - Other assumptions follow R18 study EVM Agreement For CPU counting of UE side data collection, - OCPU=1 - FFS: The CPU occupancy duration

In this contribution, we continue to discuss the other aspects related to AI/ML-based CSI compression, focusing on data collection for training, performance monitoring, and model paring.

Discussions

  - Data collection for training

Data collection for training can be performed at both NW side and UE side.

NW side data collection

For data collection at NW side, the target CSI samples can be collected by higher layer based signaling as previously discussed. The quantization format of target CSI is also under discussion. Option 1 (scalar quantization) and Option 4 (eType II like codebook with new parameter) are supported by the majority of companies and will be further studied. For Option 1, we propose to quantize the amplitude value and phase value, which align more closely with physical meaning. The phase value is naturally distributed between -π and π, whereas the amplitude value is non-negative and often presents a large dynamic range compared to the phase value. Hence, it is unnecessary to normalize the phase value. The phase value can then be quantized to fewer bits with relatively limited information compared to the amplitude. The difference of amplitude among different dimension values may be large, where the dimensions include antenna ports, subbands and layers. The large dynamic range will cause the poor quantization accuracy. In this case, it is better to normalize the amplitude for each dimension value. For example, if the amplitude of one subband is much larger than that of another subband, the amplitude can be normalized within each subband. Considering that UE is aware of the amplitude distribution of measured precoding matrices, it can report the dimension(s) within which the amplitude is normalized along with the target CSI samples to facilitate the NW side data collection.

In addition, the two-sided model must be trained on a suitable dataset of channel state information to achieve effective CSI compression. Hence, the quality information can be provided along with target CSI samples to denote its reliability for NW side data collection and then NW can choose data for model training according to the quality information. The content of the quality information needs to be further discussed. For example, the measured CSI-SINR or CQI can serve as the quality information. The larger CSI-SINR or CQI implies the higher quality. A quality indicator ranging from 0 to 1 can also serve as the quality information and the larger quality indicator indicates the higher quality. The calculation of the quality indicator is based on the UE implementation. The quality threshold can be also configured by NW. In this way, UE can choose data for NW side training beforehand so that the overhead of both the dataset of CSI and associated quality indicators can be reduced.

Proposal 1: For scalar quantization of target CSI, it is recommended to quantize the amplitude value and phase value.

Proposal 2: The amplitude can be normalized for each dimension value and the dimension(s) within which the amplitude is normalized can be reported by UE for NW side data collection.

Proposal 3: The quality information such as CSI-SINR and CQI can be provided along with target CSI samples or the quality threshold can be configured by NW for reliability of target CSI samples.

UE side data collection

For data collection at UE side, each UE can locally collect target CSI samples over time. It can collect data in both an opportunistic and proactive manner. The opportunistic manner means the UE collects available data from ongoing channel measurement sessions including the channel measurement associated with CSI-ReportConfig used for configuring the resources for data collection purpose without CSI report, and the proactive manner means the UE requests channel measurements from NW when it needs data collection. In practice, the UE can log target CSI samples periodically or during specific channel conditions to capture a wide range of scenarios. This on-device data collection leverages the UE’s direct perception of the channel, ensuring the generalization performance of the model. The target CSI samples can be also provided by NW for initial training of the encoder. In one mechanism (Option 4-1), the NW sends a dataset to the UE containing the target CSI samples and corresponding labels, i.e., CSI feedback produced by encoder. By receiving such data, the UE can train or fine-tune its encoder model to align with the NW’s decoder expectations. This approach is flexible. The provided target CSI could originate from the NW’s own measurements, simulations, or even other UEs (e.g., crowd-sourced data), allowing cross-user or network-wide learning without the UE needing to independently experience every channel condition.

Observation 1: The UE side data collection for training can be performed in both an opportunistic and proactive manner.

  - Performance monitoring

Once the AI/ML-based CSI compression is operational, it is critical to monitor its performance and ensure the model continues to meet accuracy requirements. The performance monitoring for CSI compression includes end-to-end monitoring and root-cause analysis, enabling timely detection of performance degradation and triggering the model update.

End-to-end monitoring

The end-to-end performance monitoring was fully discussed in the SI stage and the conclusions are captured as follows[4][7].

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. Conclusion UE-side monitoring is feasible. Observation Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

The performance monitoring can be performed at NW side or UE side. For NW side performance monitoring, NW evaluates the quality of the reconstructed CSI (output of the decoder) against the label to gauge end-to-end performance. For CSI spatial/frequency compression without temporal aspects, this label could be the actual measured CSI. The label acquisition can be based on the existing CSI report framework. For example, both target CSI and CSI feedback are configured as the report quantities and the periodical report type is configured so NW can compare the reconstructed CSI with the actual measured CSI periodically reported by UE to assess overall accuracy.

For UE side performance monitoring, multiple options were considered. For example, UE side monitoring can be performed based on the output of the CSI reconstruction model at UE or via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI, which can reduce the signaling overhead. For the sake of root-cause analysis and model control, the decision-making can be performed at NW side. In this way, UE needs to report the monitoring-related information to NW for final decision-making and two kinds of report quantities can be considered. One option is that UE reports monitoring metric values such as the SGCS between target CSI and reconstructed CSI, and NW judges the model performance based on the monitoring metrics. The other option is that UE judges the model performance by itself based on the threshold configured by NW. UE only needs to report the monitoring outcome, e.g., good performance or bad performance, to NW. Using threshold-based triggering ensures that the UE reports are event-driven, i.e., only when performance falls outside desired bounds, thereby reducing unnecessary signaling. The UE’s report could be a binary flag or a small codepoint representing the performance category, further minimizing overhead.

Observation 2: The UE side performance monitoring can be performed by UE implementation.

Proposal 4: For NW side performance monitoring, the label acquisition could be based on the existing CSI report framework, e.g., both target CSI and CSI feedback are configured as the report quantities.

Proposal 5: For UE side performance monitoring, UE needs to report the monitoring metric values or monitoring outcome.

Root cause analysis

The root cause analysis was discussed in the SI stage and the conclusion is captured as follows[6].

Conclusion For NW-first training, in inter-vendor training collaboration Direction A and C, identification of root cause for performance issues can be achieved at least by the following - Data distribution mismatch that is identified by either NW side or UE side - Via target CSI and CSI feedback pair report, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part. - Note that this conclusion does not preclude other methods. - Note that triggering of root cause detection may be after detecting performance degradation

If the end-to-end performance degrades, it may infer the issue lies with distribution shift in the input data, i.e., data drift. Hence, the root cause analysis can be triggered to further identify whether the data drift affects the autoencoder or the quantization. In such a case, the NW can request additional UE-side monitoring data. For instance, the UE might be requested to send samples of target CSI, encoder output and quantizer output for analysis. The NW can firstly compare the quantizer output with the output from the NW side reference CSI generation part to identify if the decoder generalization has degraded, or rather, is out-of-date. Then the NW can compare the target CSI with the decoder output which is derived from encoder output directly to identify if the autoencoder generalization has degraded. Similarly, the NW can continue to compare the encoder output with the de-quantizer output which is derived from quantizer output to identify if the quantization generalization has degraded. The acquisition of samples of target CSI, encoder output and quantizer output can be all configured based on the existing CSI report framework. Overall, the system can identify whether a performance drop is due to the AI model effects or quantization effects, ensuring that appropriate actions can be taken with confidence.

Proposal 6: The request of target CSI, encoder output and quantizer output could be based on the existing CSI report framework for the root cause analysis.

  - Model paring

The model paring was discussed in the SI stage and in the last meeting, and the achieved agreements are captured as follows[5][7][8].

Agreement For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, - ID-X can be used for pairing the UE-part and the NW-part of a two-sided model - FFS: other information needed for pairing Agreement In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for NW-side data collection - FFS: whether ID/even same ID is needed for monitoring configuration - FFS: where the ID is assigned - Note: whether the purpose for pair will be specified will be discussed separately. Agreement In Direction C, the fully standardized reference model is associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for NW-side data collection - FFS: whether ID/even same ID is needed for monitoring configuration - FFS: where the ID is assigned or how the ID is specified - Note: whether the purpose for pair will be specified will be discussed separately. Agreement For model pairing procedure for inference, consider the applicability report procedure of Rel-19 AI/ML beam management for UE side model as a starting point. - Pairing ID(s) (i.e., ID for pairing related discussion) is indicated (e.g., in CSI-reportConfig) in Step 3. - FFS whether/how to link the Pairing ID with the inter-vendor collaboration - FFS whether/how Paring ID(s) are reported in Step 4 - FFS whether to support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only). - FFS CSI compression specific inference configuration or inference related parameters.

It is concluded that the paring ID should be introduced for pairing the UE-part and the NW-part of a two-sided model. In Options 3a-1 and 4-1, the paring ID can be exchanged along with the model parameters or dataset for model identification. Using the paring ID, NW and UE follow a clear procedure to ensure they are using a paired model. For inference, if NW decides to use a new encoder due to an update or adaptively switching for changing conditions, it can indicate the paring ID of the corresponding encoder to UE in Step 3 of the applicability report procedure as discussed in the last meeting. Additionally, if multiple encoders are applicable, e.g., one for low mobility and another for high mobility scenarios, the paring ID allows UE to distinguish them and apply the correct one as instructed by the NW. The encoder at UE and the decoder at NW thus maintains alignment.

Furthermore, the paring ID can be used in the UE-side data collection and NW-side data collection. For UE-side data collection, the paring ID can indicate that the configured channel measurement resources are to be used for the encoder identified by that ID. If the network later provides incremental training data, e.g., the data from other UEs, to facilitate the UE-side model fine-tuning, it can also tag it with the paring ID to indicate that those samples are to be used for the encoder identified by that ID. For NW-side data collection, the paring ID can be also used for NW-side model fine-tuning. For example, UE will report the logged training data associated with the paring ID which is indicated by NW or reported along with the training data.

Considering there exists many identifiers in the previous discussions about AI/ML for NR air interface, such as model ID, functionality ID, ID-X, paring ID, associated ID. We propose to further standardize a unified ID to make the specification concise. The unified ID is defined as a deterministic mapping from multiple identifiers mentioned above. UE should support recovering the identifiers from the unified ID by de-mapping. For instance, UE can recover the paring ID from unified ID so that the encoder and decoder remain paired. In this way, the UEs with the same unified ID are deemed to be configured with the same identifier set or have the similar model conditions, which unifies the ID-related configuration.

Observation 3: The paring ID can be exchanged along with the model parameters (Option 3a-1) or dataset (Option 4-1) for model identification.

Proposal 7: The paring ID can be used for model paring procedure for inference, UE-side data collection and NW-side data collection.

Proposal 8: It is recommended to introduce a unified ID to streamline the ID-related configuration.

Conclusions

In this contribution, we have the following observations and proposals:

Observation 1: The UE side data collection for training can be performed in both an opportunistic and proactive manner.

Observation 2: The UE side performance monitoring can be performed by UE implementation.

Observation 3: The paring ID can be exchanged along with the model parameters (Option 3a-1) or dataset (Option 4-1) for model identification.

Proposal 1: For scalar quantization of target CSI, it is recommended to quantize the amplitude value and phase value.

Proposal 2: The amplitude can be normalized for each dimension value and the dimension(s) within which the amplitude is normalized can be reported by UE for NW side data collection.

Proposal 3: The quality information such as CSI-SINR and CQI can be provided along with target CSI samples or the quality threshold can be configured by NW for reliability of target CSI samples.

Proposal 4: For NW side performance monitoring, the label acquisition could be based on the existing CSI report framework, e.g., both target CSI and CSI feedback are configured as the report quantities.

Proposal 5: For UE side performance monitoring, UE needs to report the monitoring metric values or monitoring outcome.

Proposal 6: The request of target CSI, encoder output and quantizer output could be based on the existing CSI report framework for the root cause analysis.

Proposal 7: The paring ID can be used for model paring procedure for inference, UE-side data collection and NW-side data collection.

Proposal 8: It is recommended to introduce a unified ID to streamline the ID-related configuration.

References

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #96, June 6 – 9, 2022.

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, RAN #107, March 12 – 14, 2025.

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, RAN #108, June 9 – 13, 2025.

- RAN1 Chair’s Notes, RAN1 #117, May 20th – 24th, 2024.

- RAN1 Chair’s Notes, RAN1 #119, Nov. 18th – 22nd, 2024.

- RAN1 Chair’s Notes, RAN1 #120, Feb. 17th – 21st, 2025.

- RAN1 Chair’s Notes, RAN1 #120-bis, April 7th – 11th, 2025.

- RAN1 Chair’s Notes, RAN1 #122, Aug. 25th – 29th, 2025.

- RAN1 Chair’s Notes, RAN1 #122-bis, Oct. 13th – 17th, 2025.