3GPP TSG RAN WG1 #123												  R1-2508553

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	10.2.1

Source:	NEC

Title:              	Discussion on improvement of SRS capacity and coverage

Document for:	Discussion and Decision

- Introduction

For RAN1#122bis meeting [1], agreements on SRS enhancements were achieved as:

Agreement

To determine the time-domain location of a cross-slot SRS resource,

- ‘l0’ is interpreted as the starting symbol in the starting slot for an SRS resource.

- ‘l'+l0’ is interpreted as the symbol index in a slot or two consecutive S and U slots.

- The restriction ‘loffset≥NsymbSRS-1’ is removed.

Agreement

- For intra-repetition hopping for SRS repetition symbols within each SRS frequency, support periodic, semi-persistent and aperiodic SRS.

- For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support all SRS usages (i.e., codebook, nonCodebook, antennaSwitching and beamManagement)

Agreement:

- Support cross-slot SRS for all SRS resource types, i.e., for periodic, semi-persistent, and aperiodic SRS.

- Support cross-slot SRS for usages of codebook, non-codebook, antenna switching, beam management

Agreement:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support Alt-1 from the agreement in RAN1#122bis as the following:

- when PF=4 and K=2, select one of the following patterns:

  - Alt 1: {0,2}

- when PF=K=4, select one of the following patterns:

  - Alt 1: {0,2,1,3}

Agreement:

- For an aperiodic cross-slot SRS resource set in scenario 1, the slot offset of the SRS resource set refers to the first of the two slots spanned by the SRS resource set.

- Note 1: Whether 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) can be supported in scenario 1 is discussed separately.

- Note 2: If the transmission of 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) is supported, for the resource(s) in the second slot, whether/how to utilize the slot offset configured for the SRS resource set is discussed separately.

- Note 3: 0 or more SRS resource with time-domain resource entirely in the first slot (i.e. S slot) can be supported in scenario 1.

Working Assumption:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support Alt-2:

- when PF=K=2, select at least one of the following basic patterns:

  - Alt 1 (consecutive mapping): {0,…,0,1,…,1}

  - Alt 2 (non-consecutive mapping): {0,1,,…,0,1}

- when PF=4 and K=2, down select from the following basic patterns:

  - Alt 1 (consecutive mapping):{0,…,0,2,…,2}

  - Alt 2 (non-consecutive mapping):{0,2,…,0,2}

- when PF=K=4, down select from the following basic patterns:

  - Alt 1 (consecutive mapping): {0,…,0,2,…,2,1,…,1,3,…,3}

  - Alt 2 (non-consecutive mapping): {0,2,1,3,…,0,2,1,3}

In this contribution, we provided our views on the SRS enhancements.

- Discussion

Multiple frequency domain starting positions

Last meeting, there was hot discussion on hopping pattern for intra-repetition hopping, i.e. consecutive or non-consecutive, and working assumption was made to support non-consecutive mapping as basic pattern. We support to confirm the working assumption, as non-consecutive mapping can have more benefit, not limited on co-scheduling for UEs with different value of R, supporting on non-integer value of NsymbSRSPF*R, and so on. So we propose:

Proposal 1: Confirm the working assumption for intra-repetition hopping for SRS, i.e. supporting Alt-2 (non-consecutive mapping).

In current spec, one SRS resource can contain NsymbSRS symbols, and in case of repetition (e.g. R<NsymbSRS and NsymbSRSR>1 is restricted as an integer), there are NsymbSRSR repetitions, each repetition with R symbols. And for partial frequency sounding, the factor PF can be configured as 2 or 4. The starting position is defined as

The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH+noffsetRPFS+noffset2FH where k0pi=nshiftNscRB+kTCpi+koffsetl'+fcohnf,ns,fμ,l'' mod KTC and kTCpi=kTC+3KTC4 mod KTCif NapSRS=8, pi∈1003, 1007, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1002, 1006, and nSRScs,max=6kTC+KTC4 mod KTCif NapSRS=8, pi∈1001, 1005, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, and nSRScs,max=12kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, nSRScs,max=8, and nSRScs≥nSRScs,max2kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, nSRScs,max∈8, 12,and nSRScs≥nSRScs,max2kTCotherwise noffsetFH=b=0BSRSmSRS,bNscRBnb noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF noffset2FH=ninithop+nSRSTxHopping mod Nhop-ninithopmSRS,0-moverlaphopNscRB

For the formula determining the frequency domain starting position, it’s preferred to maintain a unified formula. For Rel-17 partial frequency sounding, the new factor for determining starting position is noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF. For multiple starting positions, we can further introduce one factor of hop (e.g. khop') or replace the factor khop. For example, noffsetRPFS=NSCRBmSRS,BSRSkF+khop+khop'mod PFPF. And the new factor of hop khop' can be a function of l', R and PF.

Proposal 2: For multiple frequency domain starting positions, a new hop factor khop' as function of l', R and PF can be introduced on top of current formula.

Regarding the combination values of NsymbSRS, R and K, it can be further discussed whether RK can be non-integer. In our understanding, non-integer value of RK can have some benefit for network scheduling.

Proposal 3: Prefer to support configuration for intra-repetition with non-integer value of RK.

Cross slot SRS

For cross-slot SRS, it needs further discussion on whether 0 or more SRS resource entirely in second slot or in first slot can be supported. Based on the WID description, the first slot of two consecutive slots is defined as S slot, if this restriction is clearly defined in TS, at least we should support the case that 0 or more SRS resource with time domain resource entirely in the second slot. For the first slot, it’s OK not to support SRS resource entirely located in the S slot, in other words, the SRS resource cross two slots can be the first one in time domain within the two consecutive slots.

Proposal 4: For cross-slot SRS, support 0 or more SRS resource with time domain resource entirely in the second slot.

In the WID, for single SRS resource across two slots, a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots should be applied. While it’s possible the TCI state is updated within the two slots, especially in case of DCI based TCI state update. The UE behaviour can be further defined, such as UE doesn’t expect such kind of update, only follow the old TCI state (e.g. delaying the TCI state application timing), or UE drop all or part of the SRS resource in two slots.

Proposal 5: Further discuss UE behavior on a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots.

In addition, both of the multiple frequency domain starting positions and SRS across two slots are valuable features for commercial deployment. We think they should be supported simultaneously (according to UE capability), such as multiple frequency domain starting positions for one SRS resource across two slots.

Proposal 6: Support multiple frequency domain starting positions for one SRS resource across two slots.

- Conclusion

In this contribution, we provided our views on SRS enhancements, and we propose:

Proposal 1: Confirm the working assumption for intra-repetition hopping for SRS, i.e. supporting Alt-2 (non-consecutive mapping).

Proposal 2: For multiple frequency domain starting positions, a new hop factor khop' as function of l', R and PF can be introduced on top of current formula.

Proposal 3: Prefer to support configuration for intra-repetition with non-integer value of RK.

Proposal 4: For cross-slot SRS, support 0 or more SRS resource with time domain resource entirely in the second slot.

Proposal 5: Further discuss UE behavior on a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots.

Proposal 6: Support multiple frequency domain starting positions for one SRS resource across two slots.

- References

- Chair notes, 3GPP TSG RAN WG1 #122bis, Prague, Czech Republic, Oct 13th – 17th, 2025