3GPP TSG RAN WG1 Meeting RAN1#123 		R1-2508566

Dallas, USA, Nov 17th – 21st, 2025

Title:	[Draft] Reply LS on SBFD and CA

Response to:	R1-2508304 [R2-2507746]

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	CATT [RAN1]

To:	RAN2

Cc:	RAN4

Contact person:	Shupeng Li

lsp@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN1 would like to thank RAN2 for the LS in R2-2507746 on SBFD and CA. In the LS, RAN2 asked RAN1 to provide TP for supporting SBFD with CA in the stage-2 specification (TS 38.300).

RAN1 would like to respond as follows:

RAN1 agreed that SBFD operation is supported on one carrier in multi-carrier scenario in RAN1#120 meeting.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

From the RAN1 perspective, SBFD and CA should be captured in the stage-2 specification to provide a clearer description of SBFD operation. SBFD can be configured for only one carrier in a full-duplex multi-carrier scenario, as agreed in RAN1. The change to the stage-2 specification (TS 38.300) is provided below.

## 23.1 General Sub-Band Full Duplex (SBFD) operation is supported for a TDD carrier, enabling simultaneous downlink transmission and uplink reception at the gNB on their non-overlapping respective sub-bands. From UE perspective, full duplex is not supported. The configurations of cell-specific SBFD time and frequency resources are provided through SIB1 or dedicated signalling. A UE can be semi-statically configured with SBFD sub-bands in downlink symbols and flexible symbols provided by tdd-UL-DL-ConfigurationCommon and this UE is referred to as an SBFD aware UE. The maximum number of UL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is one. The maximum number of DL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is two. A UE can be configured with SBFD symbols on only one TDD carrier in full-duplex CA scenario. In an SBFD symbol, except for cross-link interference measurements, a UE transmits or receives only in RBs that are both in the active UL BWP and in the UL sub-band, or both in the active DL BWP and the DL sub-band(s), respectively. A UE can be configured to transmit or receive only in non-SBFD symbols, only in SBFD symbols, or across both SBFD symbols and non-SBFD symbols for multiple transmission or reception occasions.

2	Actions

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above TP into account for TS 38.300.

3	Dates of next TSG RAN WG1 meetings

RAN1#124	9th – 13th February 2026	 		Stor-Göteborg, Sweden

RAN1#124bis	13th – 17th April 2026	 		EU