3GPP TSG RAN WG1 #123                                                                                    R1-2508621

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	11.3.1

Source:	Lenovo

Title:	Discussion on 6GR Waveform

Document for: 	Discussion

# Introduction

In RAN#108, the study item for 6G Radio (6GR) was discussed and formally approved [1]. It was agreed that the physical layer waveform for 6GR will continue to be based on OFDM technology, encompassing both CP-OFDM and DFT-s-OFDM schemes. The study will focus on overall waveform flexibility to meet the diverse requirements anticipated in future deployments, e.g., coverage enhancement, energy efficiency improvement, and support of sensing. 6GR waveform study will include evaluating potential improvements to the current waveform structures with a detailed assessment of their suitability for future use cases and deployment scenarios.

# Discussion

During RAN1#122-bis meeting some agreements have been reached mainly related to UL waveform evaluation. During the meeting, there were multiple discussions on DL waveform, however, there were no related agreements. In RAN1#123, we expect a continuation of the discussions on DL waveform, especially whether DFT-s-OFDM for DL will be studied and evaluated.

## DFT-s-OFDM for Downlink

## 2.1.1 Motivation

DFT-s-OFDM can achieve lower PAPR/CM which improves PA efficiency without the need for aggressive signal clipping and back-off compared to CP-OFDM. Performance metrics listed below can be enhanced with the support of DFT-s-OFDM in the downlink:

- Network energy savings is important in the low and medium load situation, where BS typically reduces or mutes antennas and beams thereby requiring higher PA power and repetitions to compensate for the coverage loss, having low PAPR waveform such as DFT-s-OFDM in downlink recovers this loss in coverage and reduces number of required repetitions for cell edge UEs. In the other hand, with lower back-off at PA for a same targeted coverage, power consumption at Tx can be reduced due to enhanced PA efficiency.

- Improvement in overall coverage. There may be a potential increase to downlink coverage target by +10 dB vs. 5G NR, which is yet to be agreed in RAN#110. Such extended terrestrial network coverage helps supporting low data rate MBB traffic e.g., multimedia messaging, voice, etc. Supporting such downlink coverage extension vs. 5G NR may need additional repetitions impacting network energy consumption. Adopting DFT-s-OFDM for cell edge UEs in the extended coverage scenarios minimizes number of required downlink repetitions for the similar BLER target thereby benefitting network energy savings compared to CP-OFDM. In addition to that, avoiding the need for repetitions in DL reduces the burden on UE Rx processing and hence improves UE power savings. Furthermore, cell edge UE throughput can be improved thanks to the Tx power boosting due to higher PA efficiency with same power consumption.

We therefore propose to study the adoption of DFT s-OFDM waveform in DL for a selected target physical channel(s) to evaluate the representative gain against baseline CP-OFDM

Proposal 1: Study adopting Downlink DFT-s-OFDM for low data rate MBB in the terrestrial network as part of network energy savings and coverage.

- Target channel(s): Two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH).

- Evaluation methodology:

  - LLS to start with focusing on key metrics such as: Net Gain and NES

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @PHY channel target BLER

    - Realistic PA model for gNB

  - NES gain (%) – baseline against corresponding CP-OFDM

    - Further discuss details on how to do without SLS

## 2.1.2 Target Downlink Channels for DFT-s-OFDM

The evaluation methodology addresses the usage of DFT-s-OFDM in downlink for the target PHY channel(s) with the aim to evaluate a representative gain over CP-OFDM (baseline) for the above use cases. The evaluation can focus on two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH).

Proposal 2: Target Downlink Channels to evaluate DFT-s-OFDM can be narrowed down to two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH).

## 2.1.3 MRSS compatibility

DFT-s-OFDM maintains occupied bandwidth, power spectral density compared to CP-OFDM similar to uplink, hence there should be no observable issues in supporting with MRSS.

Observation 1: DFT-s-OFDM maintains MRSS compatibility with 5G.

## 2.1.4 Multi-user multiplexing/Scheduling flexibility

DFT-s-OFDM can maintain the same scheduling flexibility as it maps the DFT spread data to the same subcarrier grid as that of CP-OFDM, while the time domain scheduling of slots and symbols is also same as that of CP-OFDM.

Observation 2: DFT-s-OFDM maintains same scheduling flexibility compared with 5G.

Per UE DFT: DFT transform precoding can be applied to each downlink channel of a UE in a time slot where each DFT size is same as the number of occupied subcarriers of the downlink signal of a UE.  The benefit of DFT-s-OFDM decreases as the PAPR/CM gain reduces with the increase in the number of UEs multiplexed in the same time slot.

Sub-band DFT: A single DFT transform precoding can be applied to downlink channel of a group of UEs where DFT size can be larger than the number of assigned subcarriers of each UE as shown in Figure 1. A single DFT transform precoding provides significantly lower PAPR/CM due to the larger spreading gain. RAN1 can further discuss on how to group such UEs to a single transform precoding.

Figure 1: Block diagram of Downlink Transmitter showing per-UE DFT and sub-band DFT

Proposal 3: Downlink DFT transform precoding can be applied in following two ways:

- Per UE DFT: DFT transform precoding can be applied to each downlink channel of a UE in a time slot where each DFT size is same as the number of occupied subcarriers of the downlink signal of a UE

- Sub-band DFT: A single DFT transform precoding can be applied to downlink channel of a group of UEs where DFT size can be larger than the number of assigned subcarriers of each UE.

Sub-band DFT can achieve much less PAPR/CM comparing with per-UE DFT as depicted in Figure 2. Furthermore, DFT spreading of UE data over larger bandwidth leads to improvement of SNR at UE receiver due to frequency diversity gain. Figure 3 shows BLER performance comparison between CP-OFDM, per UE DFT-s-OFDM and sub-band DFT-s-OFDM. The simulation assumptions, except for delay spread, are provided in Table 1.

| Figure 2: Comparison of PAPR/CM for sub-band DFT |  |
| --- | --- |

Figure 3: Comparison of frequency diversity gain of sub-band DFT

Observation 3: Sub-band DFT transform precoding scheme gains from frequency diversity of the channel, SNR at UE receiver due to frequency spreading is enhanced due to applying single DFT transform precoding over a larger bandwidth (5MHz BW) than UE allocated frequency resources,

- 4dB SNR gain can be obtained compared to CP-OFDM and per UE DFT-s-OFDM for UEs each with 1 RB allocation

- 1.5 dB gain can be obtained for UEs each with 6RBs allocation.

The boost of Tx power and correspondingly the improvement of SNR at Rx, achieved by adopting DFT-s-OFDM in DL, can help in supporting low data rate MBB traffic e.g., multimedia messaging, voice, etc., at cell edge. The improvement of SNR at UE Rx can be seen as an extension of coverage or as an increase of DL throughput of cell edge UEs. Figure 4 shows a simulation of an MBB use case with at cell edge. In which the Tx power boost of DFT-s-OFDM due to lower back-off enables using higher MCS value at Rx compared to CP-OFDM.

| 2GHz, BW:20MHz@15kHz, 4UEs@24RBs | 7.5GHz, BW:100MHz@60kHz, 8UEs@16RBs |
| --- | --- |
| Figure 4: Throughput enhancement at cell edge |  |

The achieved gain of throughput at same coverage due to the improvement of SNR is summarized in the table below.

| Carrier frequency | Target SNR [dB] | SB DFT MCS value | OFDM MCS value | OFDM Thp [Mbps] | SB DFT Thp [Mbps] | Throughput Gain [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 2GHz | 3 | 3 | 1 | 1.047 | 1.676 | 60.2 |
|  | 6 | 5 | 3 | 1.697 | 2.425 | 42.9 |
| 7.5GHz | 1.8 | 3 | 1 | 2.84 | 4.31 | 67.5 |

Observation 4: Sub-band DFT for DL can enhance the throughput of low data rate MBB traffic at cell edge. At same coverage 40% to 60% gain in the throughput can be achieved compared to CP-OFDM

## 2.1.5 Channel multiplexing

Channel Multiplexing: Similar to multiplexing UEs, multiplexing different PHY channels in the frequency domain and different waveforms, i.e., CP-OFDM and DFT-s-OFDM with OFDMA leads to increase in PAPR/CM depending on the ratio of RBs with CP-OFDM to RBs with DFT-s-OFDM. In general, the achievable reduction in PAPR/CM varies depending on modulation schemes of the multiplexed UEs/channels, number and sizes of DFT sub-bands, number of multiplexed UEs or channels that are FDMed either based on DFT-s-OFDM or CP-OFDM. Figure 5 shows different PAPR/CM reduction gains of multiplexed UEs /channels. Depending on the requirements in terms of coverage and NES and the connected UEs at a time, the scheduler may multiplex UEs with different waveforms and/or different DFT grouping. This leads to variation in the PAPR/CM of the generated signal. TDM multiplexing of DFT-s-OFDM and CP-OFDM is also possible.

Figure 5: PAPR/CM for multiplexed UE with different waveforms

Observation 5:  PHY channels can be multiplexed in the frequency domain using the different waveform.

- PAPR/CM of the signal with multiplexed waveforms varies depending on the allocation ratio between DFT-s-OFDM-based channel and CP-OFDM-based channel and other factors such as modulation etc.,

  - For e.g., with 50:50 ratio between SB-DFT and CP-OFDM more than 1dB PAPR gain is observed

Observation 6:  PHY channels can be multiplexed in the frequency domain using the same DFT-s-OFDM waveform. PAPR/CM reduction gain depends on the number of sub-bands, sub-band sizes, other factors such as modulation etc.,

- For e.g., Multiplexing 2xSB-OFDM waveform in a slot still provides 3.5dB PAPR gain compared to CP-OFDM

## 2.1.6 Transmitter/Receiver complexity and impact to power consumption

Complexity:

At the base station transmitter, an extra DFT transform precoding step needs to be applied compared to the CP-OFDM which has moderate complexity considering that base station already equipped with IDFT transform precoding from the PUSCH UL receiver which may require updates to perform DFT operation.

At the UE receiver, in case of sub-band DFT, each UE receiver complexity depends on how the DMRS REs are multiplexed, whether in the frequency domain or in the time domain and whether channel estimation and equalization are performed in time or in frequency domain. In case of frequency domain receiver processing, channel estimation and equalization are performed on DMRS REs that cover entire sub-band with sub-band IDFT. However, such complexity can be reduced using the time domain receiver processing. In which e.g., equalization can be performed via match filtering with a length that combines only the length of UE time domain resources and the length of channel impulse response.

Observation 7:  BS transmitter needs module update to apply DFT transform precoding

Observation 8:  UE receiver complexity depends on DMRS REs multiplexing and whether frequency domain or time domain receiver processing is applied.

Network energy saving: With assuming dynamic Back-off at PA based on signal CM, applying DFT-s-OFDM for certain slots, even with multiplexing with CP-OFDM, can still lead to reasonable energy saving at base station. Figure 6 shows a simple example of Back-off at PA for different slots assuming the scheduler randomly schedules UEs with different waveforms (CP-OFDM only, per UE DFT FDMed with OFDM, SB DFT FDMed with OFDM, full SB DFT, or full per UE DFT) assuming scheduling length of 10 slots. The Back-off varies from more than 5dB down to 2.2 dB. More than 1.3 dB average gain can be obtained in the Back-off over 3000 slots compared to the baseline (CP-OFDM). Figure 7 shows the corresponding energy saving obtained from the different instants of dynamic Back-off. Assuming PA efficiency of 35%, the energy saving gain varies from 0% and can reach 30%. An average of 16% energy saving gain can be obtained over 3000 slots.

Figure 6: Illustration of dynamic Back-off assuming UE multiplexing using different waveforms

Figure 7: Illustration of energy saving gain assuming UE multiplexing using different waveforms

Observation 9: Even in worst case scenario of applying random scheduling of UEs with different waveforms, the back-off can vary from 5dB down to 2 dB. Over 3000 slots, more than 1.3 dB average gain can be obtained in the Back-off compared to CP-OFDM and the corresponding average network energy saving obtained from the different instants of dynamic Back-off over 3000 slots can reach 16%.

## 2.1.7 Impact on synchronization and initial access

The impact to the synchronization and initial access depends on whether the DFT-s-OFDM can be applied to the common channels or not. The specification impact on the redesigning synchronization signal, PBCH, CORESET#0 for DFT-s-OFDM can be high, while the specification impact is smaller when the DFT-s-OFDM is applied only on the data channel during the connected mode.

Observation 10: Specification impact of DFT-s-OFDM for the common channel/signal is higher compared to the unicast data channel (PDSCH).

## 2.1.8 Other motivation

DFT-s-OFDM IoT: For IoT use cases, DFT-s-OFDM at the base station offers benefits due to low PAPR/CM. These benefits translate into improved coverage for cell-edge UEs, fewer required repetitions (which is needed to reach deep coverage for such device types), and thus reduced energy consumption at both network and UE. Low tier IoT usually supports single-antenna and complex MIMO configurations is avoided for power saving purpose. This makes DFT-s-OFDM in DL a good candidate for such low-complexity device type.

Figure 8, and Figure 9 illustrate the BLER performance as a function of the number of repetitions for an example scenario involving IoT devices operating in the 900 MHz band with a 5 MHz system bandwidth and 15 kHz subcarrier spacing (SCS). The IoT devices are frequency-multiplexed, each allocated the same number of resource blocks (RBs): 6 RBs (Figure 8), and 12 RBs (Figure 9). The target coverage corresponds to a maximum coupling loss (MCL) of 154 dB. Additional simulation assumptions are summarized in Table 1.

Table 1: Simulation parameters corresponding to Figures 8, 9, 10

| Parameter | Value |
| --- | --- |
| Channel Model | TDL-C |
| Delay Spread | 1000 ns |
| UE speed | 0 |
| MCS | Fixed to index 0 |
| Modulation | QPSK |
| Channel Coding Scheme | 5G LDPC with 24-bit CRC |

It is observed that the sub-band DFT scheme achieves approximately a 2.75 dB cubic metric (CM) gain compared to the CP-OFDM waveform, and a 1.31–2.68 dB gain over per UE DFT-s-OFDM, depending on the RB allocation, as shown in Figure 10. These CM gains translate into reductions of more than 30% in the number of repetitions required to achieve a 10% BLER target compared to CP-OFDM and per UE DFT-s-OFDM.

| Figure 8: BLER vs. number of repetitions for CP-OFDM, per UE DFT-s-OFDM, and SB-DFT-s-OFDM for 6 RB allocations | Figure 9: BLER vs. number of repetitions for CP-OFDM, per UE DFT-s-OFDM, and SB-DFT-s-OFDM for 12 RB allocations | Figure 10: Cubic Metric for CP-OFDM, per UE DFT-S-OFDM, and SB-DFT-S-OFDM in a system with 5MHz BW for [1, 6, 12] RB allocations |
| --- | --- | --- |

Observation 11: Sub-band/group based DFT scheme achieves approximately a 2.75 dB cubic metric (CM) gain compared to the CP-OFDM waveform, and a 1.31–2.68 dB gain over per UE DFT-S-OFDM which translates into reductions of more than 30% in the number of repetitions required to achieve a 10% BLER.

## Enhancements to Existing 5G Waveforms

During the 5G waveform study, various methods and enhancements were explored for both DFT-s-OFDM in the uplink and CP-OFDM in both uplink and downlink. Several of these enhancements focused on improving energy efficiency and reducing PAPR which are especially important in uplink-dominant or low-power scenarios. Reducing the PAPR/CM of a signal allows for a lower back-off requirement in the PA, which is effectively equivalent to power boosting, providing higher transmission power without increasing hardware constraints. Some approaches for PAPR/CM reduction rely on techniques such as signal clipping, DPD, or commanding (compression at the transmitter and expansion at the receiver). Though these methods can provide significant reductions in PAPR/CM, they often introduce signal distortion, resulting in degraded SNR and compromised receiver performance. Therefore, a trade-off must be carefully considered between the amount of PAPR/CM reduction achieved and the impact on signal quality.

Besides these methods there are different techniques can be applied to DFT-s-OFDM and CP-OFDM that give moderate reduction in PAPR/CM while preserving signal quality with near distortion-less signal recovery at Rx.

    - Enhancements to CP-OFDM Waveform

One method to reduce PAPR/CM of CP-OFDM is the application of Selected Mapping (SLM) scheme. It depends on generating multiple phase sequences at Tx multiplied elementwise with frequency domain signal. The time domain signal version with lowest PAPR/CM is chosen for transmission. SLM-CP-OFDM has demonstrated potential for ~2 dB PAPR reduction, depending on the number and selection of candidate sequences. However, there are some issues with SLM-CP-OFDM. The complexity at Tx increases with increasing the number of sequences in addition to the need of storing signal versions before selecting the suitable one as well as the need to signal sequence information to Rx. Trade-off between the PAPR reduction gains and the complexity introduced at transceiver needs to be studied. Figure 11 illustrates example results of PAPR/CM of SLM-OFDM-based PDCCH and PDSCH with 4 and 8 sequences.

| Figure 11(a): SLM-OFDM for PDCCH | Figure 11(b): SLM-OFDM for PDSCH |
| --- | --- |

As shown in Figure 11, applying SLM with four phase sequences to CP-OFDM can achieve a PAPR reduction gain of approximately 1–2 dB compared to conventional CP-OFDM. However, the improvement in CM is more modest, typically less than 1 dB. The actual gain depends heavily on the type and number of sequences and the selection strategy used for choosing the optimal phase sequence, whether the criterion is specifically optimized for minimizing PAPR or for reducing CM.

Observation 12: With selected mapping scheme, the number of used sequences is important for PAPR/CM reduction, e.g., a gap of 0.5dB can be seen between 8 and 4 sequences.

Another method is Tone reservation (TR) that reduces PAPR/CM in CP-OFDM by reserving a subset of subcarriers that carry no data and adjusting them in the frequency domain to suppress time-domain peaks. While TR can achieve ~2 dB PAPR reduction after several transmitter-side iterations, it increases complexity and reduces spectral efficiency due to unused tones. Receiver-side recovery requires only knowledge of reserved tone locations for rate matching. While TR avoids signal distortion and may not need explicit signaling, it comes with increased Tx complexity. Further gains require adapting tone placement based on channel conditions, potentially introducing dynamic signaling overhead.

Proposal 4: Study and evaluate CP-OFDM waveform enhancement techniques including PAPR/CM reduction techniques such as Selected Mapping (SLM) and Tone Reservation (TR) for coverage enhancement and energy efficiency improvement, and compare to implementation-based techniques in terms of complexity, signal distortion, and spectral efficiency.

    - Enhancements to DFT-s-OFDM Waveform

In the uplink, DFT-s-OFDM remains a critical waveform due to its inherent low PAPR characteristics. However, for 6GR further enhancements may need to be considered to satisfy 6G KPIs in terms of further improving the coverage and energy efficiency. One example of enhancing DFT-s-OFDM is by applying frequency domain spectrum shaping filters (e.g., root-raised cosine, Hamming, Hanning, etc.) at DFT output to further smoothen out the generated peaks of time domain signal. By selecting a suitable shaping filter, 1 to 2 dB reduction gain can be achieved compared to legacy DFT-s-OFDM. While basic shaping methods are effective, combining Frequency Domain Spectrum Shaping (FDSS) with spectrum extension may achieve even lower PAPR/CM, but with extra complexity and SE degradation. Further optimization of filter design including methods that take advantage of signal statistics (e.g., signal variance/distribution) may yield additional gains. Below figures show the achieved gain in PAPR/CM reduction by applying FDSS compared to legacy DFT-s-OFDM and CP-OFDM.

| Figure 12(a): PAPR for FDSS-DFT-s-OFDM with different FDSS schemes | Figure 12(b): CM for FDSS-DFT-s-OFDM with different FDSS schemes |
| --- | --- |

Observation 13: FDSS using conventional filter methods (e., root-raised cosine, Hamming, Hanning, etc.) can give a good reduction in PAPR, however, the reduction of CM can be minor in some cases.

Another technique to enhance DFT-s-OFDM can be based on extending the DFT precoder. The extension on the DFT precoder is performed in time domain inputs while the frequency domain outputs are preserved using a non-square DFT matrix. The extension plays a dual role, first modifying single-carrier shaping filter associated with the DFT-S-OFDM waveform (i.e., by time-domain oversampling, or alternatively, time-domain extension) for lower PAPR, and second enhancing the degrees of freedom of the input space for creating time domain sequences that can generate low PAPR. Consequently, this results in a new transform-based single-carrier waveform preserving key underlying properties of DFT-S-OFDM yet better setup for delivering lower PAPR.

This method enables potential ~1 dB PAPR reduction. However, it comes with challenges such as self-interference, which require analysis of receiver complexity and precoder design for practical deployment.

Proposal 5: Study enhancing DFT-s-OFDM waveform by incorporating PAPR/CM reduction techniques such as FDSS, DFT precoder extension, etc.

As a summary, evaluation of different enhancement techniques for both CP-OFDM and DFT-s-OFDM is illustrated in Figure 13.

| Figure 13: PAPR/CM of waveform enhancement techniques |  |
| --- | --- |

From the figure it can be noticed that different enhancement techniques can lead to a noticeable reduction in PAPR. However, when examining the CM, which is a more accurate representation of the required power amplifier back-off, the benefits of some of these methods can be less significant. In several cases, the CM reduction achieved through some of the schemes is minor or inconsistent. This discrepancy arises because PAPR is sensitive to isolated signal peaks, while CM accounts for both the average and peak signal behavior in a way that aligns more closely with the power amplifier's non-linear characteristics. Given this, the evaluation of waveform enhancement techniques should aim to prioritize the reduction of CM over PAPR. While PAPR reduction can be an initial step, targeting CM directly ensures that the resulting waveform aligns with the practical constraints of RF front-end design.

Proposal 6: The study and evaluation of waveform enhancements should focus on CM characteristic of the waveform

# Conclusions

Proposal 1: Study adopting Downlink DFT-s-OFDM for low data rate MBB in the terrestrial network as part of network energy savings and coverage.

- Target channel(s): Two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH).

- Evaluation methodology:

  - LLS to start with focusing on key metrics such as: Net Gain and NES

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @PHY channel target BLER

    - Realistic PA model for gNB

  - NES gain (%) – baseline against corresponding CP-OFDM

    - Further discuss details on how to do without SLS

Proposal 2: Target Downlink Channels to evaluate DFT-s-OFDM can be narrowed down to two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH).

Observation 1: DFT-s-OFDM maintains MRSS compatibility with 5G.

Observation 2: DFT-s-OFDM maintains same scheduling flexibility compared with 5G.

Proposal 3: Downlink DFT transform precoding can be applied in following two ways:

- Per UE DFT: DFT transform precoding can be applied to each downlink channel of a UE in a time slot where each DFT size is same as the number of occupied subcarriers of the downlink signal of a UE

- Sub-band DFT: A single DFT transform precoding can be applied to downlink channel of a group of UEs where DFT size can be larger than the number of assigned subcarriers of each UE.

Observation 3: Sub-band DFT transform precoding scheme gains from frequency diversity of the channel, SNR at UE receiver due to frequency spreading is enhanced due to applying single DFT transform precoding over a larger bandwidth (5MHz BW) than UE allocated frequency resources,

- 4dB SNR gain can be obtained compared to CP-OFDM and per UE DFT-s-OFDM for UEs each with 1 RB allocation

1.5 dB gain can be obtained for UEs each with 6RBs allocation

Observation 4: Sub-band DFT for DL can enhance the throughput of low data rate MBB traffic at cell edge. At same coverage 40% to 60% gain in the throughput can be achieved compared to CP-OFDM

Observation 5:  PHY channels can be multiplexed in the frequency domain using the different waveform.

- PAPR/CM of the signal with multiplexed waveforms varies depending on the allocation ratio between DFT-s-OFDM-based channel and CP-OFDM-based channel and other factors such as modulation etc.,

  - For e.g., with 50:50 ratio between SB-DFT and CP-OFDM more than 1dB PAPR gain is observed

Observation 6:  PHY channels can be multiplexed in the frequency domain using the same DFT-s-OFDM waveform. PAPR/CM reduction gain depends on the number of sub-bands, sub-band sizes, other factors such as modulation etc.,

- For e.g., Multiplexing 2xSB-OFDM waveform in a slot still provides 3.5dB PAPR gain compared to CP-OFDM

Observation 7:  BS transmitter needs module update to apply DFT transform precoding

Observation 8:  UE receiver complexity depends on DMRS REs multiplexing and whether frequency domain or time domain receiver processing is applied.

Observation 9: Even in worst case scenario of applying random scheduling of UEs with different waveforms, the back-off can vary from 5dB down to 2 dB. Over 3000 slots, more than 1.3 dB average gain can be obtained in the Back-off compared to CP-OFDM and the corresponding average network energy saving obtained from the different instants of dynamic Back-off over 3000 slots can reach 16%.

Observation 10: Specification impact of DFT-s-OFDM for the common channel/signal is higher compared to the unicast data channel (PDSCH).

Observation 11: Sub-band/group based DFT scheme achieves approximately a 2.75 dB cubic metric (CM) gain compared to the CP-OFDM waveform, and a 1.31–2.68 dB gain over per UE DFT-S-OFDM which translates into reductions of more than 30% in the number of repetitions required to achieve a 10% BLER.

Observation 12: With selected mapping scheme, the number of used sequences is important for PAPR/CM reduction, e.g., a gap of 0.5dB can be seen between 8 and 4 sequences.

Proposal 4: Study and evaluate CP-OFDM waveform enhancement techniques including PAPR/CM reduction techniques such as Selected Mapping (SLM) and Tone Reservation (TR) for coverage enhancement and energy efficiency improvement, and compare to implementation-based techniques in terms of complexity, signal distortion, and spectral efficiency.

Observation 13: FDSS using conventional filter methods (e., root-raised cosine, Hamming, Hanning, etc.) can give a good reduction in PAPR, however, the reduction of CM can be minor in some cases.

Proposal 5: Study enhancing DFT-s-OFDM waveform by incorporating PAPR/CM reduction techniques such as FDSS, DFT precoder extension, etc.

Proposal 6: The study and evaluation of waveform enhancements should focus on CM characteristic of the waveform

# References

- RP-251881, New SID: Study on 6G Radio

# Appendix

## Agreements

## Agreements achieved in RAN1#122 on AI 11.3.1

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc. Note: Proponents are encouraged to provide more detailed information on their proposals for the next meeting, e.g.: Proponents to characterize the main motivation for modification/additional waveform proposals: - Targeted link direction, i.e. DL, UL or both - Targeted use case (e.g. NTN, specific frequency range, etc.), if any - Potential motivations metrics used, and quantified gains for a proposal, e.g. - Coverage - Network energy efficiency - UE energy efficiency - Spectral efficiency - High speed tolerance - Scheduling flexibility - Integration with ISAC Proponents provide information on the following aspects, if applicable - MRSS compatibility - Target channels/signals, e.g. all channels, PxSCH only, etc. - MIMO (SU and MU-MIMO) compatibility - Target modulations, and impact to other modulations, if applicable - Multi-user multiplexing/scheduling flexibility - Multiplexing/coexistence with baseline waveforms - Impact on synchronization and initial access - Expected specification impact - Transmitter/receiver complexity and impact to power consumption.

## Agreements achieved in RAN1#122 on AI 11.3.1

Agreement Draft LS R1-2508068 is endorsed with following revision: - removing “Additionally, if time permits, any feedback for CP-OFDM PAPR reduction/MPR values achievable by implementation is also appreciated.” Agreement Final LS R1-2508069 is endorsed. Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

## Coverage evaluation of DL DFT-s-OFDM

To evaluate the gain in DL coverage using DFT-s-OFDM, link level simulation was performed for some DL PDSCH messages using DFT-s-OFDM and compared to CP-OFDM as illustrated in Figure 14. Simulation parameters are summarized in table 2.

Table 2: Configurations of the Link level simulated PDSCH

| Message | TPS (bits) | MCS (CR) | #OFDM Symbols | #RBs | System BW(MHz) | Allocated BW (MHz) | LLS-Parameters |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SIB1 | 990 | 4(0.301) | 11 + 1DMRS | 12 | 100 | 4.32 | Channel model: CDL-A300 BW:100MHz Carrier frequency: 7.5GHz UE speed:3km/h SCS: 30kHz MIMO: Disabled (single layer transmission) |
| Msg2 | 11 | 0(0.117) | 2 + 1DMRS | 2 | 100 | 0.72 |  |
| Msg4 | 160 | 5(0.370) | 3 + 1DMRS | 6 | 100 | 2.16 |  |
| Unicast MBB DL (peak 47Mbps) | 1700 | 14(0.54) | 11 + 1DMRS | 6 | 100 | 2.16 |  |

| Figure 14: BLER performance for DFT-s-OFDM-based PDSCH |  |
| --- | --- |

DFT-s-OFDM shows slightly lower performance compared to CP-OFDM in the CDL-A-300 scenario at low MCS levels using QPSK modulation. However, the BLER degradation is minor, typically less than 0.3 dB. A more noticeable performance loss (approximately 1 dB) is observed for DFT-s-OFDM in MBB use cases, particularly at higher MCS levels. Simulation results indicate that when DFT-s-OFDM is used for DL with low MCS schemes and single-layer transmission, the slight BLER degradation compared to CP-OFDM is negligible. In such cases, the advantage of reduced CM becomes dominant, making DFT-s-OFDM a suitable choice.

Figure 14 shows calculation of the potential gain in DL coverage in case of DFT-s-OFDM for the above-mentioned messages using a required SNR @1%BLER from LLS, and assuming RF/deployment parameters summarized in Table 3.

| Figure 9: Range comparison between DFT-s-OFDM and CP-OFDM for some DL signals |  |
| --- | --- |

Range gain of >300m can be achieved for Macro cell, for the cases with low MCS values and single MIMO layer, while very minimum gain in range can be achieved for MMB with single layer.

Table 3: RF and deployment parameters

| Parameter | Value | Parameter | value |
| --- | --- | --- | --- |
| Carrier frequency | 7.5 GHz | Cable loss | 3 dB |
| System BW | 100 MHz | UE noise figure | 9 dB |
| BS height | 25 m | Thermal noise density | -174 dBm/Hz |
| UE height | 1.5 m | Rx interference density | -165.7 dBm/Hz |
| #Tx antenna elements | 128, 256, 512, 1024 | diversity gain | 0 dB |
| #Rx antenna elements | 1 | Rx implementation margin | 2 dB |
| UE antenna gain | 0 dBi | shadow fading margin | 4.84 dB |
| BS antenna gain | 8 dBi | penetration loss | 9 dB |
| body loss | 1 dB | Max total Tx power | 53 dBm |