3GPP TSG RAN WG1 #123		R1-2508715

Dallas, United States, Nov. 17th-21st, 2025

Source: 	OPPO

Title:	Inter-vendor training collaboration for AI/ML CSI compression

Agenda Item:	10.1.2

Document for:	Discussion and Decision

Introduction

In 3GPP RAN1#122bis, the specification support for inter-vendor training collaboration for two-sided AI/ML CSI compression has been discussed. In this contribution, some sequential critical issues are discussed.

Dataset content and format

RAN1#122

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s).

- FFS: Target CSI type and format

- FFS: CSI feedback type and format

- FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size.

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, down select one of the following options:

- Option 1: The exchanged CSI feedback is the latent message before quantization.

- Option 2: The exchanged CSI feedback is the binary sequence at the output of quantization.

- Note: Quantization codebook is exchanged in addition if not specified in the spec.

- Note: Whether or not using the Quantization codebook for model training, is up to UE implementation.

RAN1#122bis

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, for the association between target CSI and CSI feedback in the exchanged dataset.

- Support 1:M mapping between one target CSI sample/set of samples to M CSI feedback samples/sets of samples associated with different CSI payload size configurations

- FFS: Association between Target CSI and CSI feedback, including scalability related information for different number of Tx port, sub bands related information.

- FFS: Whether/how Target CSI samples/set of samples within an exchanged dataset, with different Tx port and sub-band related information are organized.

- FFS: Whether the Target CSI is per layer or per rank when Target CSI is precoder matrix.

- FFS: Whether and how to support CSI feedback structure with different payload scalability options

- FFS: Other details

Above agreements about dataset content and format, and association between target CSI and CSI feedback have been approved in RAN1#122 and RAN1#122bis, where some issues remained FFS are discussed as follows:

Target CSI type and format: In 10.1.1.1 agenda for inference stage, RAN1 has agreed that precoding matrix as the target CSI type, and FFS for full channel. Therefore, here we should follow the same principle, i.e., support precoding matrix as target CSI type for exchanged dataset(s). Similarly, for further clarification, we are support of the precoding matrix in spatial-frequency domain. The target CSI type in beam-delay domain or beam-frequency domain are not supported. Reasons are given as follows:

- Performance issue: In the Rel-18/19 study phase, precoding matrix in spatial-frequency domain is used as the input of encoder and output of decoder for evaluations by majority companies. The performance with precoding matrix in spatial-frequency domain has been proved. As comparison, there is no sufficient evaluations with precoding matrix in beam-delay or beam-frequency domain, hence the performance gain versus legacy codebook should be further approved.

- Scalability issue: the corresponding reference model structure (i.e., Transformer backbone with potential embedding and adaption layer) is suggested by RAN1. The scalability issue over different Tx ports, sub-bands and CSI feedback payloads for precoding matrix in spatial-frequency domain has been addressed. However, for precoding matrix in beam-delay or beam-frequency domain, which kind of reference model structure should be used and whether the scalability issue can be fully addressed should be further studied.

Besides above two reasons, in our understanding, the one/two-dimensional DFT pre-processing from spatial-frequency domain precoding matrix to beam-delay or beam-frequency domain can be replaced by an additional adaption layer at the beginning of the encoder. At the NW-side, the output of decoder can be still precoding matrix in spatial-frequency domain, no matter how the encoder is implemented at the UE-side. Therefore, we think that

Based on these aspects, we are only support of specifying precoding matrix in spatial-frequency domain as the basic target CSI type for NW-side data collection. It is not necessary to specify precoding matrix in different domains, whether the input of precoding matrix in spatial-frequency domain is transformed into other forms can be left to UE-side encoder implementation.

Observation 1: Performance and scalability issues are not fully addressed for target CSI type with precoding matrix in beam-delay or beam-frequency domain.

Proposal 1: Only support CSI eigenvector in spatial-frequency domain as the target CSI type in exchanged dataset(s).

- Whether the input of precoding matrix in spatial-frequency domain is transformed into other forms can be left to UE-side encoder implementation.

Regarding the format of target CSI, it would be better if the format of exchanged target CSI is aligned with the format used for NW-side data collection for model training. As we presented in 10.1.1.2 agenda, scalar quantization on the real and imaginary part of target CSI can be used as the baseline format for NW-side data collection. Therefore, for the precoding matrix in exchanged dataset(s), we also prefer using the same way as the baseline format. The determination of quantization parameters for target CSI exchange related can be further discussed.

Proposal 2: Support linear normalization and uniform scalar quantization on real and imaginary part as baseline format of target CSI in exchanged dataset(s).

- Determination of quantization related parameters can be further discussed.

Regarding the CSI feedback type and format, we prefer option 2 with the binary sequence, which is enough for UE-side model training with additional quantization codebook exchanged along with the dataset. In our view, it is not necessary to exchange latent message before quantization, since it cannot provide additional performance gain for UE-side model training.

Proposal 3: Support binary sequence after quantization as the format of CSI feedback in Option 4-1.

For the association between target CSI and CSI feedback in the exchanged dataset, the 1:M mapping is supported between one target CSI sample / set of samples to M CSI feedback samples / sets of samples associated with different CSI payload size configurations. The basic structure and association within the dataset is promoted as follows. The dataset is associated with one pairing ID and includes N target CSI sets, wherein each target CSI set has different Tx port configs and/or sub-band configs. For each target CSI set, it is associated with M CSI feedback sets, wherein each CSI feedback set has different payloads. Note that it is not necessary to distinguish different layers and/or different ranks in the target CSI set, since the layer-common and rank-common model is utilized. The target CSI with the same Tx config and sub-band config may include target CSI from different layers, then UE-side can use this target CSI and associated CSI feedback to train the layer-common and rank-common model.

Table 1 Dataset structure and association detail

| Dataset |  |
| --- | --- |
| Pairing ID |  |
| Target CSI # Tx port config 1 # sub-band config 1 | CSI feedback # payload 1 |
|  | … |
|  | CSI feedback # payload M |
| Target CSI # Tx port config 2 # sub-band config 2 | CSI feedback # payload 1 |
|  | … |
|  | CSI feedback # payload M |
| … | … |
| Target CSI # Tx port config N # sub-band config N | CSI feedback # payload 1 |
|  | … |
|  | CSI feedback # payload M |

Table 1 shows the basic structure of exchanged dataset. Other information including performance target, quantization codebook, and other necessary information can be exchanged additionally with association to target CSI and/or CSI feedback.

Proposal 4: Support the dataset structure and 1:M association between one target CSI set with specific Tx port and sub-band config with M CSI feedback sets with different payloads (as shown in Table 1)

Performance target

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, both average SGCS and average NMSE as performance target in the exchanged dataset are supported. Both average SGCS and average NMSE are provided.

- NW side can optionally send NMSE and/or SGCS values at X-percentiles.

Agreement about the performance target exchange has been achieved in last meeting, wherein SGCS and/or NMSE can be considered, wherein average SGCS and/or NMSE is supported, since it is the most straightforward metric to measure the performance. Regarding the format of average SGCS and/or NMSE, two options to indicate the performance target can be considered (take SGCS as example):

- Option A: direct SGCS performance indication, wherein one SGCS is indicated for each layer, as {SGCS1,SGCS2,⋯,SGCSV}, where SGCSv,1≤v≤V is the performance target for the v-th layer, V is the rank of associated dataset exchanged from NW-side to UE-side. By this way, the end-to-end SGCS performance of UE-side model for the v-th layer, denoted as SGCSvUE should satisfy SGCSvUE≥SGCSv

- Option B: differential SGCS performance indication, wherein one differential SGCS is indicated for each layer, as {ΔSGCS1,ΔSGCS2,⋯,ΔSGCSV}, where ΔSGCSv,1≤v≤V is the performance target gain for the v-th layer. By this way, the end-to-end SGCS performance of UE-side model for the v-th layer, denoted as SGCSvUE should satisfy SGCSvUE≥SGCSv,legacy+ΔSGCSv, wherein SGCSv,legacy is the performance of legacy codebook for the v-th layer. The codebook type and parameter combinations of legacy codebook can be additionally indicated by NW-side or up to implementation of UE.

Obviously, Option A indicates the absolute performance target of UE-side model, while Option B indicates the performance gain compared to legacy codebook. As per our understanding, the performance gain versus legacy codebook under the same dataset is more meaningful, and we prefer to use Option B as the format of performance target indication.

Proposal 5: Regarding the format of average SGCS and/or NMSE, support:

- Option A: direct value

- Option B: differential value compared to legacy codebook

CSI feedback payload determination

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, if the quantization codebook for CSI feedback is not fixed by the specification, support exchange of quantization codebook along with dataset exchange

- FFS: Quantization codebook exchange method (e.g, look up table, or how to exchange quantization codebook related information, or other methods)

- FFS: Common or different quantization codebook for different CSI payload size

- Note: leveraging the discussion in agenda item 10.1.1.1 when applied

In RAN1#122 meeting, we have agreed that if the quantization codebook is not specified, the quantization codebook should be exchanged along with dataset. Other than quantization codebook, the exchange of mapping relationship between parameter combinations (PCs) and CSI feedback payloads should be supported as well. In 10.1.1.1 agenda, RAN1 is aiming to specify a standardized PC table associated to fixed value of CSI feedback payload.

In addition to the specified quantization PCs, the non-specified exchanged parameter combination should be supported. The mapping between CSI feedback payload and its associated pair of {dl,v,Ql,v} or triplets of {dl,v,L,Ql,v} can be exchanged along with the dataset in Option 4-1. By this way, different NW-vendors may have the freedom and flexibility to implement different kinds of parameter combinations, e.g., VQ, SQ with different dl,v, Ql,v values, which is potential to achieving a higher performance suitable for local region than specified alternatives. Then UE-side can use the exchanged non-specified quantization parameter combination, instead of the specified ones for sequential model training and inference. In detail, two different ways can be considered for PC exchange.

- Explicit quantization parameters indication: for each pair of target CSI and CSI feedback dataset with one specific payload, the corresponding quantization parameters, e.g., dl,v,Ql,v, can be explicitly indicated. Consider that 1:M mapping is supported where one target CSI set can be associated with multiple CSI feedback sets with different CSI payloads, specific quantization parameters can be explicitly indicated along with each CSI feedback set.

- Implicit quantization parameters indication: the table of PCs and associated CSI feedback payloads as well as quantization parameters, e.g., dl,v,Ql,v, is firstly exchanged along with the whole dataset. The corresponding PC is exchanged along with each CSI feedback set to implicitly indicate the quantization parameters for this CSI feedback set. By this way, for different CSI feedback sets with different CSI feedback payloads, their associated PCs should be from the common mapping table.

Proposal 6: Support exchanged quantization parameter combination and indication along with dataset in Option 4-1.

Pairing ID

In RAN1#122 meeting, RAN1 has agreed to support exchange of single pairing ID along with dataset. The first bullet says one pairing ID is assigned to one dataset, which means that one dataset is only associated with one pairing ID. But on the other hand, different datasets can be assigned with the same pairing ID, which means that UE-side can use these datasets with the same pairing ID to train one unified encoder.

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single pairing ID along with dataset exchange.

- One pairing ID is assigned to one dataset.

  - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed).

- From RAN1 perspective, the uniqueness of the pairing ID needs further studied.

FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

Two remaining issues are discussed as follows:

Uniqueness of the pairing ID: in our view, associated ID is not needed in two-sided model, and the function of associated ID discussed in BM agenda can be included in pairing ID. Regarding the associated ID, RAN2 has agreed that the associated ID value is unique within a PLMN. Therefore, pairing ID should follow the same rule to make the pairing ID usable between multiple cells.

Impact on pairing ID(s) when additional data samples are added to an exchanged dataset: this issue includes two different conditions, which should be discussed separately:

- Additional CSI feedback data samples without target CSI: in this case, the updated NW-side encoder can generate updated output CSI feedback with the same target CSI. The target CSI dataset has been exchanged to the UE-side, and only the updated CSI feedback needs to be exchanged additionally. To facilitate UE to find the associated target CSI for the newly exchanged CSI feedback for UE-side model updating/fine-tuning, the same pairing ID should be assigned for the updated CSI feedback dataset. By this way, the newly exchanged CSI feedback dataset should replace the old exchanged CSI feedback, and the pairing ID should be remained unchanged.

- Additional CSI feedback data samples with target CSI: the exchanged datasets include new pairs of CSI feedback and target CSI, in this case, UE-side may have different behaviors according to the pairing ID:

Table 2 UE-side behavior and pairing ID

| Case | UE-side behavior | Same or different pairing ID for new dataset and old dataset |
| --- | --- | --- |
| Case 1 | Train/update/fine-tune the UE-side encoder with both of new dataset and old dataset | Same |
| Case 2 | Train/update/fine-tune the UE-side encoder with only new dataset based on current encoder trained on old dataset | Same |
| Case 3 | Train the UE-side encoder with only new dataset, not based on current encoder trained on old dataset | Different |

In Case 1, the new dataset and old dataset are just two parts of a whole large dataset, which possesses similar feature in spatial and frequency domain of target CSI. Therefore, the UE-side encoder in Case 1 can be trained/updated/fine-tuned to achieve a better performance on both of new dataset and old dataset with more data samples. Obviously, the pairing ID of new dataset and old dataset should be the same.

In Case 2, the new dataset may provide some new features not included in old dataset, which are more accommodated to current channel environments. In this case, UE-side can train/update/fine-tune the encoder on new dataset with current encoder trained on old dataset as a starting point. By this way, the trained encoder can be more suitable to new dataset, and can achieve a better performance on new dataset. To let UE-side select the right encoder as the starting point, the same pairing ID of new dataset and old dataset should be assigned.

We can find UE-side cannot distinguish Case 1 and Case 2 with only the same pairing ID, thus the UE-side behavior may be unclear. To address this issue, another ID, e.g., version ID, can be introduced to indicate the relationship between new dataset and old dataset. For Case 1, the new dataset and old dataset are assigned to the same pairing ID and same version ID, e.g., version ID = 0 to represent the initial version of dataset. For Case 1, the new dataset and old dataset are assigned to the same pairing ID but different version ID, e.g., version ID = 0 for old dataset but version ID = 1 for new dataset. According to both of pairing ID and version ID, UE-side can select the most suited behavior to train/update/fine-tune the UE-side encoder.

Proposal 7: Support of unique pairing ID within a PLMN.

Proposal 8: Support of the same pairing ID if additional CSI feedback data samples are added to an exchanged dataset.

Proposal 9: Support of additionally using version ID to indicate the relationship between new dataset and exchanged dataset.

Conclusion

In this contribution, we provide our views on inter-vendor training collaboration for two-sided AI/ML based CSI compression. Observations and proposals are listed as following:

Observation 1: Performance and scalability issues are not fully addressed for target CSI type with precoding matrix in beam-delay or beam-frequency domain.

Proposal 1: Only support CSI eigenvector in spatial-frequency domain as the target CSI type in exchanged dataset(s).

- Whether the input of precoding matrix in spatial-frequency domain is transformed into other forms can be left to UE-side encoder implementation.

Proposal 2: Support linear normalization and uniform scalar quantization on real and imaginary part as baseline format of target CSI in exchanged dataset(s).

- Determination of quantization related parameters can be further discussed.

Proposal 3: Support binary sequence after quantization as the format of CSI feedback in Option 4-1.

Proposal 4: Support the dataset structure and 1:M association between one target CSI set with specific Tx port and sub-band config with M CSI feedback sets with different payloads (as shown in following table)

| Dataset |  |
| --- | --- |
| Pairing ID |  |
| Target CSI # Tx port config 1 # sub-band config 1 | CSI feedback # payload 1 |
|  | … |
|  | CSI feedback # payload M |
| Target CSI # Tx port config 2 # sub-band config 2 | CSI feedback # payload 1 |
|  | … |
|  | CSI feedback # payload M |
| … | … |
| Target CSI # Tx port config N # sub-band config N | CSI feedback # payload 1 |
|  | … |
|  | CSI feedback # payload M |

Proposal 5: Regarding the format of average SGCS and/or NMSE, support:

- Option A: direct value

- Option B: differential value compared to legacy codebook

Proposal 6: Support exchanged quantization parameter combination and indication along with dataset in Option 4-1.

Proposal 7: Support of unique pairing ID within a PLMN.

Proposal 8: Support of the same pairing ID if additional CSI feedback data samples are added to an exchanged dataset.

Proposal 9: Support of additionally using version ID to indicate the relationship between new dataset and exchanged dataset.

References

- R1-2507668, FL summary # 3 for inter-vendor training collaboration, Prague, Czech Republic, Oct. 13th-17th, 2025.