3GPP TSG RAN WG1 #123                                         R1-2508773

Dallas, USA, November 17th – 21st, 2025

Agenda item:	8.2

Source: 	Samsung

Title: 	Remaining issue on NR MIMO Phase 5

Document for:	Discussion and Decision

# Introduction

The Rel-MIMO 19 WID which has been finally revised in RAN#105 [1] includes the following objectives.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

This contribution provides Samsung’s view regarding the remaining issues on NR MIMO Phase 5.

# UE-initiated (UEI) beam reporting

In this section, we focus on some remaining issues related to the UEI beam reporting including trigger event(s) – along with the measurement RS(s) configuration/determination, UL signaling content(s)/report format(s) and the UCI based UL signaling procedure(s) design, and present our views on the related subject matters, e.g., in form of text proposals (TPs) based on the endorsed specification(s).

# Event(s) and triggering condition(s)

A few remaining issues related to event evaluation(s) are discussed here.

Case (a)

Case (b)

Figure 1

In RAN1#120b, it was agreed that the UE would reset the event instance counting for all new beams when a UE receives a TCI state update. The intention of specifying such counting resetting behavior is to ensure that the reported beam is based on the latest indicated TCI state instead of a previous and invalid one. One remaining issue is to further clarify details of such update – i.e., detailed means of determining reference time for the “current” beam RS. The basic principle should be that the first PUCCH is only allowed to be transmitted if the triggering event is based on the current/latest TCI state at the transmission time of the first PUCCH. One simple case is that the measurement is performed in the time window after the TCI state update and there is no other DCI indicating the TCI state is received during the time window for event evaluation as show in Case (a) of Figure 1. Another example is given in Case (b) where the later received DCI (DCI #3) indicates a same TCI state as that indicated in an earlier received DCI (DCI #2). In RAN1#122, the following proposal regarding how to determine the reference time for the “current” beam RS was provided and discussed:

Proposal 2.2: Regarding UE-initiated/event-driven beam reporting, on both Mode-A and Mode-B, if the current beam is configured to always be reported, down-select one of the following options on the reference time for deriving the current beam RS in a report instance: - Option-1: - For a single CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state indicated in a latest PDCCH before a first PUCCH associated with the second PUSCH. - For a multi-CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state, indicated in a latest PDCCH, for the serving cell of the new beam RS, where the latest PDCCH is before a first PUCCH associated with the second PUSCH. - Option-2: The current beam RS is the same as the RS derived by the indicated TCI state in the slot of the CSI reference resource corresponding to the report instance. - Option-3: - For a single CC case and a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the latest TCI state, applied [in a latest slot] before a first PUCCH associated with the second PUSCH. - For a multi-CC case and a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the latest TCI state, applied [in a latest slot] before a first PUCCH associated with the second PUSCH, for the serving cell of the new beam RS.

We support Option-1 in the above proposal. If the TCI state indicated in a latest PDCCH/DCI before a corresponding first PUCCH is the same as the “indicated” TCI state that triggers the event hence the UEI-BR (meaning that the current beam RS that determines the event is not changed/updated prior to the corresponding first PUCCH transmission occasion), the corresponding first PUCCH can be transmitted. For instance, as shown in the Figure 1, because the latest DCI (DCI #3) received prior to the corresponding first PUCCH transmission occasion indicates a same TCI state (i.e., TCI #2) as that indicated in DCI #2 which has been used for triggering the event and the corresponding UEI-BR (i.e., Case (b)), the UE can determine that the corresponding first PUCCH can be transmitted. If there is no DCI #3, DCI #2 would become the latest DCI and the UE can also transmit the first PUCCH (i.e., Case (a)).

Furthermore, we do not think that Option-2 is needed or has any specification impact(s). We only need to clarify which indicated TCI state the UE needs to refer to for deriving current beam RS for event evaluation (not necessarily for deriving the report content(s)). In contrast to Option-1, Option-3 focuses on application of a TCI state (and when) – but it is still unclear about how to determine this indicated TCI state.

Based on the above, we would like to present the following proposal for both single-cell and multi-cell cases. It is worth noting here that for the multi-cell case, the principle is the same as the single-cell case. The only difference for the multi-cell case is that the indicated TCI state can apply to multiple serving cells, and the serving cell configured with new beam RS(s) should be one of these multiple serving cells that apply the indicated TCI state.

Proposal 1: Regarding UE-initiated/event-driven beam reporting, on both Mode-A and Mode-B, if the current beam is configured to always be reported, support the following Option-1 on the reference time for deriving the current beam RS in a report instance:

- Option-1:

  - For a single CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state indicated in a latest PDCCH before a first PUCCH associated with the second PUSCH.

  - For a multi-CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state, indicated in a latest PDCCH, for the serving cell of the new beam RS, where the latest PDCCH is before a first PUCCH associated with the second PUSCH.

Regarding counting resetting based on change or update of “current” beam RS, it should be clarified that

- when the candidate beam RS is an NZP CSI-RS, the event instance counting for the candidate beam RS is reset only when the reference signal in in the indicated TCI state is updated, and

- when the candidate beam RS is an SSB, the event instance counting for the candidate beam RS is reset only when the SSB QCLed with the reference signal in the indicated TCI state is updated.

Otherwise, the following unintended cases are supported by current descriptions in the TS 38.214: (1) the event instance counting for the candidate beam RS being an NZP CSI-RS is reset if the SSB QCLed with the reference signal in the indicated TCI state is updated, and (2) the event instance counting for the candidate beam RS being an SSB is reset if the reference signal in the indicated TCI state is updated. Hence, we propose the following TP #1 to clarify the correct UE behaviors.

TP #1:

38.214-j10

5.2.1.5.4.1	UE Initiated CSI reporting

<Unchanged parts are omitted>

The counter of the event instances for such reference signal is reset:

-	if the reference signal in the indicated TCI state is updated and newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition

-	or if the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated and newBeamResourceSet is a CSI-SSB-ResourceSet.

<Unchanged parts are omitted>

Proposal 2: Adopt TP #1 in clause 5.2.1.5.4.1 of the TS 38.214.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation: - Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality. - Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality. - Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs Note: For Event-2, we have the following definition for scheme-1 and scheme-2 - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Regarding Event-7, it is unclear whether multiple “current” beam RSs are allowed to be determined for event evaluation with respect to a same timer/time window because the Q-th best quality hence the corresponding “current” beam RS may change from one measurement occasion to another. Obviously, without restricting the number of “current” beam RS(s) that can be determined for event evaluations within a same timer/time window, the overall UEI beam reporting procedure would be inevitably prolonged and very complicated. Furthermore, to reuse the same design as Event-2 (as in the above agreement), we propose to support that for a same timer/time window, the UE would only apply one same “current” beam RS for event evaluations, where for simplicity, the fixed “current” beam RS can be that used for starting the corresponding timer/time window – or equivalently that used for determining the (first) event instance that starts the timer/time window. Hence, we propose the following.

Proposal 3: Regarding determining Event-7 instance(s) within a timer/time window, support to only use the “current” beam RS with the Q-th best quality that determines the event instance that starts the timer/time window.

# Report content(s)/format(s)

Condition(s) of applying differential L1-RSRP needs to be clarified for UEI beam reporting. That is, when nrofReportedRS-UEIBR is configured to be greater than one, the UE would report differential L1-RSRP(s) for at least one reported new beam RS. The above applies to all events. Note that clarifying the condition(s) of applying differential L1-RSRP reporting is a common practice for all other CSI/beam reporting settings than the UEI beam reporting. In particular, we provide the following TP #2:

TP #2:

38.214-j10

5.2.1.5.4.1a	UE Initiated CSI reporting for event 2

<Unchanged parts are omitted>

After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP if nrofReportedRS-UEIBR is configured to be larger than one and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report

<Unchanged parts are omitted>

5.2.1.5.4.1b	UE Initiated CSI reporting for event 1

<Unchanged parts are omitted>

After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs if nrofReportedRS-UEIBR is configured to be larger than one, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report

<Unchanged parts are omitted>

5.2.1.5.4.1c	UE Initiated CSI reporting for event 7

<Unchanged parts are omitted>

After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP if nrofReportedRS-UEIBR is configured to be larger than one and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report

<Unchanged parts are omitted>

Proposal 4: Adopt TP #2 in clause 5.2.1.5.4.1a of the TS 38.214.

For event-1, when current beam L1-RSRP is configured to be reported, two absolute L1-RSRPs are in a single report instance. It is needed to clarify that which absolute L1-RSRP is used as a reference for computing differential L1-RSRP(s). In this case, the absolute L1-RSRP corresponding to the reference signal in candidate beam set should be used which is consistent with event-2.

Proposal 5: For event-1, when current beam is configured to be reported, the absolute L1-RSRP corresponding to CRI or SSBRI is used as a reference for differential L1-RSRP in the same reporting instance.

- Adopt TP #3 in clause 5.2.1.5.4.1b of the TS 38.214.

TP #3:

38.214-j10 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 <Unchanged parts are omitted> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. When enabledCurrentBeamReport is configured, differential L1-RSRPs are with a reference to the absolute L1-RSRP corresponding to CRI or SSBRI. The UE sends the CSI report <Unchanged parts are omitted>

# Collision handling for overlapping channels

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support Option-3 of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

It has been agreed in RAN1#121 that when the 1-bit first PUCCH is collided/overlapped with a PUSCH, Option-3 is supported, i.e., piggybacking/appending the 1-bit indication into the PUSCH. The details of multiplexing UEI-RI in the PUSCH were further discussed in RAN1#122 and there are two options on the table for down-selection, as shown below.

Proposal 3.1.1B: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing first PUCCH(s) with UEIRIs collided/overlapped with a PUSCH, down select one of the following options: - Option-1: The field of UEIRI is jointly encoded with HARQ-ACK by appending the UEIRI to the HARQ-ACK information bits. - Note: UE does not expect to multiplex UEIBR in a CG PUSCH with CG-UCI/UTO-UCI, - Note: From spec perspective, multiplexing UEIRI in PUSCH reuses the same mechanism of CG-UCI. - Option-2: The field of UEIRI is jointly encoded with CSI Part-1 by adding the UEIRI before CSI part-1 information bits. - Note: Multiplexing between first PUCCH and PUSCH is realized by assuming UEIRI as one type of CSI part-1.

Option-1 has minimum standard impacts by reusing the legacy UCI multiplexing design for CG-UCI/UTO-UCI, i.e., jointly encoded with HARQ-ACK by appending the UEIRI to HARQ-ACK information bits. In addition, the design principle of Option-1 is consistent with, if not exactly the same as, the UL power prioritization rules agreed in the last meeting RAN1#122 (the corresponding agreement is copied below), where the priority of UEI-RI is the same as HARQ-ACK and is higher than CSI based on the highlighted texts.

Agreement: Adopt the following changes in TS38.213 Section 7.5: - Reason for change: When the transmission powers of multiple uplink transmissions exceed the UE’s maximum transmit power, TS 38.213 specifies a prioritization mechanism to determine which transmissions should be reduced or dropped. This prioritization covers all conventional UCI types, such as SR, CSI reports, and HARQ-ACK, ensuring consistent behavior across implementations when power reduction is needed. However, the UE-initiated CSI reporting indicator (UEIRI), introduced as a new UCI type to support UE-initiated CSI reporting, is not currently included in the priority order defined in TS 38.213. - Summary of change: Capture the priority rule for UEIRI, i.e., the same priority as LRR/SR. - Consequences if not approved: Without explicit prioritization, UE behavior may vary between implementations, potentially causing inconsistent reporting of UE-initiated CSI under power-limited conditions. This could result in the unintended suppression of UEIRI or, conversely, excessive reduction of other UCI types, impacting system performance and interoperability ## 7.5 Prioritizations for transmission power reductions For single cell operation with two uplink carriers or for operation with carrier aggregation or for operation with a candidate cell configured by LTM-Config or for operation with a single cell configured with sTx-2Panel, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cell(s) or on a candidate cell, if any, in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cell(s) or on a candidate cell, if any, in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i. If the UE transmits SRS on multiple SRS resources according to Clause 6.2.1.4 of [6, TS 38.214], the UE allocates power so that all REs of the SRS transmission have same power. For the purpose of power allocation in this clause, if a UE is provided uci-MuxWithDiffPrio and the UE multiplexes HARQ-ACK information in a PUSCH, a priority index of the PUSCH is the larger of (a) the priority index of the PUSCH according to clause 9 and (b) the larger priority index of the HARQ-ACK information. When determining a total transmit power for serving cells or a candidate cell, if any, as described in Clause 21 in a frequency range in a symbol of transmission occasion i, the UE does not include power for transmissions starting after the symbol of transmission occasion i. The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot. - PRACH transmission on a candidate cell, if any, as described in Clause 21 - PRACH transmission on the PCell - PUCCH or PUSCH transmissions with larger priority index - For PUCCH or PUSCH transmissions with same priority index - PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, and/or UEIRI, or PUSCH transmission with HARQ-ACK information of the priority index - PUCCH transmission with CSI or PUSCH transmission with CSI - PUSCH transmission without HARQ-ACK information of the priority index or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell - If the UE is configured with prioSCellPRACH-OverSP-PeriodicSRS-r17 - Aperiodic SRS transmission or PRACH transmission on a serving cell other than the PCell - Semi-persistent and/or periodic SRS transmission - otherwise, - SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell =========unchanged omitted===========.

Based on the above, we provide the following proposal:

Proposal 6: On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case where the first PUCCH is collided/overlapped with a PUSCH, i.e., the 1-bit indication of first PUCCH is multiplexed into the PUSCH,

- Support joint coding with HARQ-ACK by appending the UEIRI to HARQ-ACK information bits.

Another remaining issue is the multiplexing of multiple UEI-RIs in a PUSCH, and two options were provided and discussed in RAN1#122 as shown below.

Proposal 3.1.1A (updated after offline): On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH(s) with UEIRIs collided/overlapped with a PUSCH, down select one of the following options: - Option-1: A field of bit sequence with a length of L bit is piggyback into the PUSCH. - Each of bits in the bit sequence corresponds to respective first PUCCH(s) by an ascending order of the values of PUCCH resource ID associated with the first PUCCHs. - Option-2: A field of log2L+1 bits of representing at most one of positive UEIRI - The codepoints other than all-zero value in the field are ordered based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs to represent a positive UEIBR. - An all-zero value for the log2L+1 bits represents a negative UEIRI value across all L UEIRIs.

In Rel-15, similar issue was discussed for overlapping multiple SRs and another PUCCH, and the current design is to report up to one positive SR to save the UCI payload considering that the probability of positive SR is not that high. For multiple UEI-RIs, the situation is quite similar such that more than one positive UEI-RI is not very likely; hence, in terms of overhead reduction, we believe that Option 2 is more feasible and aligned with the WID objectives of reducing overhead.

More importantly, Option 1 would result in additional implementation complexity for both UE and network. This is because the UE would perform intra-UE multiplexing according to the following four steps as specified in TS 38.213 clause 9. The design principle is that the outcome of the previous step is the input of the next step and there is no go-back operation.

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 including repetitions if any, - first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6, if any - second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 - third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 - fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause.

In the example depicted in Figure 3, two PUCCHs with positive UEI-RIs can overlap with a PUCCH format 2/3/4 and a PUSCH. In step 2, the UE would first multiplex UEI-RIs in the PUCCH format 2/3/4 with only one positive UEI-RI. In step 4, the UE would multiplex HARQ-ACK and UEI-RIs in the PUSCH. Option 1 requests the UE to keep all the original information of UEI-RI after performing step 2, which means that it would require a go-back operation which is not friendly for UE’s implementation. The same implementation issue also exists for the network. On the contrary, Option 2 only requires the result or outcome of step 2 by multiplexing the same UCI carried in the PUCCH in the PUSCH.

Figure 3

Based on the above, we provide the following proposal:

Proposal 7: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH(s) with UEIRIs collided/overlapped with a PUSCH, support Option-2:

- Option-1: A field of bit sequence with a length of L bit is piggyback into the PUSCH.

  - Each of bits in the bit sequence corresponds to respective first PUCCH(s) by an ascending order of the values of PUCCH resource ID associated with the first PUCCHs.

- Option-2: A field of log2L+1 bits of representing at most one of positive UEIRI

  - The codepoints other than all-zero value in the field are ordered based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs to represent a positive UEIBR.

  - An all-zero value for the log2L+1 bits represents a negative UEIRI value across all L UEIRIs.

  - Note: Option-2 reuses the same rule as multiplexing multiple UEI-RIs in PUCCH format 2/3/4.

# Priority rules for UEI-BR for multiple CSI configurations

In case of UEI-BR for multiple CSI configurations, the value of s needs to be revisited. In such case, NW does not know the associated UEI-BR is initiated by which CSI report configuration until the content of the report is decoded. However, for the handling of overlapping resources for CSI reports according to priority values, s should be known by NW before the report is decoded. Hence, the s should not be the value of CSI report configuration ID for the UEI-BR. Instead, s should be a fixed value which is known by NW regardless of which CSI report configuration is initiated. For example, s can be the minimum value of the configuration IDs of the multiple CSI report configurations.

Observation 1: In case of UEI-BR associated with multiple CSI report configurations, NW and UE does not have common understanding on the priority value of the corresponding UEI-BR since it is up to UE to select one of the multiple CSI report configurations.

Proposal 8: For the determination of a priority value PriiCSIy,k,c,s of a UEI-BR associated with multiple CSI report configurations and the same first PUCCH channel, s is the minimum value of the configuration IDs of the multiple CSI report configurations.

- Adopt TP #4 in clause 5.2.5 of the TS 38.214.

TP #4:

38.214-j10 5.2.5 Priority rules for CSI reports <Unchanged parts are omitted> CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where - for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeA’, y =1 for aperiodic CSI reports to be carried on PUSCH, y = 2 for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeB’,y = 3 for semi-persistent CSI reports to be carried on PUSCH, y = 4 for semi-persistent CSI reports to be carried on PUCCH and y = 5 for periodic CSI reports to be carried on PUCCH; - for CSI reports carrying L1-RSRP, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI or L1-SINR and for CSI reports not carrying L1-RSRP, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI or L1-SINR; - c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells; - for a CSI report configured with ltm-CSI-ReportConfig, c is the serving cell index value where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. - for a CSI report configured with ltm-CSI-ReportConfig, s is the ltm-CSI-ReportConfigId and Ms is the value of the higher layer parameter maxNrofLTM-CSI-ReportConfigurations - for a CSI report associated with multiple CSI-ReportConfig with same higher layer parameters eventType and PUCCHResource, s is the minimum value of the reportConfigIDs of the multiple CSI-ReportConfig <Unchanged parts are omitted>

# Remaining issue(s) related to Mode B

Agreement (RAN1#118) On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. Agreement (RAN1#121) On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the value of X symbols for determining available transmission occasion of the second UL channel on Mode-B - Support Option-1 of the following as RRC candidate values for X symbols - Option-1: 0, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512 - Note: X is based on the SCS of the first PUCCH. - Minimum value of X is subject to UE capability - Regarding ‘available’ transmission occasion of the second UL channel, the transmission restriction for of the second UL channel reuses the legacy rule of PUSCH with SP-CSI.

The above agreement made in RAN1#121 is for clarifying the ‘available’ transmission occasion of the second UL channel which is not clearly captured in the specifications. In legacy, PUSCH with SP-CSI on a serving cell would not be transmitted if the PUSCH overlaps with a set of symbols include

- another PUSCH on the same serving cell as specified in TS 38.214 clause 5.2.5 and TS 38.213 clause 9, or

- overlaps with a PUCCH with a higher priority as specified in TS 38.213 clause 9, or

- overlaps with a PUCCH with SR as specified in TS 38.213 clause 9, or

- overlaps with non-active period of cell DRX if cell DRX is activated for the serving cell

- overlaps with non-active period of DRX as specified in TS 38.321 clause 5.7

The issue was discussed in RAN1#122 due to the incoming RAN2 LS R1-2506721 copied below.

1. Overall Description: Regarding Rel-19 two-TA configuration and UE-initiated CSI reporting, RAN2 made the following agreements. RAN2 clarifies that the parameter that enables Rel-19 two-TA configuration (singleDCI-MultiTRP-2TA) is applied for both Rel-19 intra-cell 2TA without mDCI mTRP (including asymmetric TRP and symmetric TRP) and inter-cell 2TA without mDCI mTRP (i.e., ICBM). The corresponding parameter name and its field description will be updated to capture this agreement, and the agreement will be informed to RAN1 by LS. UE does not transmit PUCCH and PUSCH for mode-B if either PUCCH or PUSCH (first valid type-1 CG occasion) is outside DRX Active Time, and if the PUCCH has been transmitted, UE does not transmit the PUSCH if type-1 CG occasion is outside DRX Active time. Inform RAN1 about this conclusion. 2. Actions: To RAN WG1 ACTION: RAN2 respectfully asks RAN1 to take the above agreements into account for future work.

The highlighted yellow text is not aligned with the above RAN1 agreement. It is necessary to clearly capture the RAN1#121 agreement in RAN1 specifications to avoid potential misunderstanding.

Proposal 9: Capture the RAN1#121 agreement for the clarification of ‘available’ transmission occasion of the second UL channel in RAN1 specifications.

- Adopt TP #5 in clause 5.2.1.5.4.1a of the TS 38.214.

TP #5:

38.214-j10 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion, subjecting to limitations for UE transmission of a PUSCH with SP-CSI reports due to DRX or cell DRX operation [11, TS 38.321] or as described in clauses 9, 11.1, 11.1.1, 11.2A,15 and 17.2 including repetitions if any [6, TS 38.213] or as described in clause 5.2.5, numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

# Conclusion

In this contribution, the following observation and proposals/TPs are given:

Proposal 1: Regarding UE-initiated/event-driven beam reporting, on both Mode-A and Mode-B, if the current beam is configured to always be reported, support the following Option-1 on the reference time for deriving the current beam RS in a report instance:

- Option-1:

  - For a single CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state indicated in a latest PDCCH before a first PUCCH associated with the second PUSCH.

  - For a multi-CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state, indicated in a latest PDCCH, for the serving cell of the new beam RS, where the latest PDCCH is before a first PUCCH associated with the second PUSCH.

Proposal 2: Adopt TP #1 in clause 5.2.1.5.4.1 of the TS 38.214.

TP #1:

38.214-j10

5.2.1.5.4.1	UE Initiated CSI reporting

<Unchanged parts are omitted>

The counter of the event instances for such reference signal is reset:

-	if the reference signal in the indicated TCI state is updated and newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition

-	or if the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated and newBeamResourceSet is a CSI-SSB-ResourceSet.

<Unchanged parts are omitted>

Proposal 3: Regarding determining Event-7 instance(s) within a timer/time window, support to only use the “current” beam RS with the Q-th best quality that determines the event instance that starts the timer/time window.

Proposal 4: Adopt TP #2 in clause 5.2.1.5.4.1a of the TS 38.214.

TP #2:

38.214-j10

5.2.1.5.4.1a	UE Initiated CSI reporting for event 2

<Unchanged parts are omitted>

After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP if nrofReportedRS-UEIBR is configured to be larger than one and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report

<Unchanged parts are omitted>

5.2.1.5.4.1b	UE Initiated CSI reporting for event 1

<Unchanged parts are omitted>

After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs if nrofReportedRS-UEIBR is configured to be larger than one, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report

<Unchanged parts are omitted>

5.2.1.5.4.1c	UE Initiated CSI reporting for event 7

<Unchanged parts are omitted>

After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP if nrofReportedRS-UEIBR is configured to be larger than one and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report

<Unchanged parts are omitted>

Proposal 5: For event-1, when current beam is configured to be reported, the absolute L1-RSRP corresponding to CRI or SSBRI is used as a reference for differential L1-RSRP in the same reporting instance.

- Adopt TP #3 in clause 5.2.1.5.4.1b of the TS 38.214.

TP #3:

38.214-j10 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 <Unchanged parts are omitted> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. When enabledCurrentBeamReport is configured, differential L1-RSRPs are with a reference to the absolute L1-RSRP corresponding to CRI or SSBRI. The UE sends the CSI report <Unchanged parts are omitted>

Proposal 6: On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case where the first PUCCH is collided/overlapped with a PUSCH, i.e., the 1-bit indication of first PUCCH is multiplexed into the PUSCH,

- Support joint coding with HARQ-ACK by appending the UEIRI to HARQ-ACK information bits.

Proposal 7: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH(s) with UEIRIs collided/overlapped with a PUSCH, support Option-2:

- Option-1: A field of bit sequence with a length of L bit is piggyback into the PUSCH.

  - Each of bits in the bit sequence corresponds to respective first PUCCH(s) by an ascending order of the values of PUCCH resource ID associated with the first PUCCHs.

- Option-2: A field of log2L+1 bits of representing at most one of positive UEIRI

  - The codepoints other than all-zero value in the field are ordered based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs to represent a positive UEIBR.

  - An all-zero value for the log2L+1 bits represents a negative UEIRI value across all L UEIRIs.

  - Note: Option-2 reuses the same rule as multiplexing multiple UEI-RIs in PUCCH format 2/3/4.

Observation 1: In case of UEI-BR associated with multiple CSI report configurations, NW and UE does not have common understanding on the priority value of the corresponding UEI-BR since it is up to UE to select one of the multiple CSI report configurations.

Proposal 8: For the determination of a priority value PriiCSIy,k,c,s of a UEI-BR associated with multiple CSI report configurations and the same first PUCCH channel, s is the minimum value of the configuration IDs of the multiple CSI report configurations.

- Adopt TP #4 in clause 5.2.5 of the TS 38.214.

TP #4:

38.214-j10 5.2.5 Priority rules for CSI reports <Unchanged parts are omitted> CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where - for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeA’, y =1 for aperiodic CSI reports to be carried on PUSCH, y = 2 for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeB’,y = 3 for semi-persistent CSI reports to be carried on PUSCH, y = 4 for semi-persistent CSI reports to be carried on PUCCH and y = 5 for periodic CSI reports to be carried on PUCCH; - for CSI reports carrying L1-RSRP, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI or L1-SINR and for CSI reports not carrying L1-RSRP, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI or L1-SINR; - c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells; - for a CSI report configured with ltm-CSI-ReportConfig, c is the serving cell index value where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. - for a CSI report configured with ltm-CSI-ReportConfig, s is the ltm-CSI-ReportConfigId and Ms is the value of the higher layer parameter maxNrofLTM-CSI-ReportConfigurations - for a CSI report associated with multiple CSI-ReportConfig with same higher layer parameters eventType and PUCCHResource, s is the minimum value of the reportConfigIDs of the multiple CSI-ReportConfig <Unchanged parts are omitted>

Proposal 9: Capture the RAN1#121 agreement for the clarification of ‘available’ transmission occasion of the second UL channel in RAN1 specifications.

- Adopt TP #5 in clause 5.2.1.5.4.1a of the TS 38.214.

TP #5:

38.214-j10 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion, subjecting to limitations for UE transmission of a PUSCH with SP-CSI reports due to DRX or cell DRX operation [11, TS 38.321] or as described in clauses 9, 11.1, 11.1.1, 11.2A,15 and 17.2 including repetitions if any [6, TS 38.213] or as described in clause 5.2.5, numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

# References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105.