3GPP TSG RAN WG1 #123			                 R1- 2508902

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.3.2.3

Source: 	LG Electronics

Title: 	Other procedures for Device 2b/C

Document for:	Discussion and decision

- Introduction

In the RAN1 #122bis meeting, the following agreement were made for other procedures for Device 2b/C for Rel-20 A-IoT [1]:

Agreement For R2D, study at least the following aspects for the study of broadcast information: - Information included in the broadcast information - Whether to use PRDCH to carry the broadcast information - Transmission of broadcast information: - Option 1: periodic - Option 2: aperiodic - Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal. Agreement Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures. - FFS: other cases, except for data transmission Agreement For the study of binary sequence-based Msg1 for the study of necessity and feasibility of D2R CDM(A) for Device 2b and for Device C, study at least the following aspects: - Sequence type (e.g. Gold sequence, m-sequence) - Length of the sequence - Number of sequences Agreement For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options: - Option 1: Msg1-like content - Option 2: DO-A data payload - FFS: with or without device ID - Option 3: DO-A scheduling request Agreement For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility: - Option 1: Periodic D2R resource allocation - Option 2: Aperiodic D2R resource allocation Agreement For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility: - Option 1: necessary information provided by broadcast information - Option 2: necessary information provided by a paging-like R2D message - Option 3: necessary information derived from a synchronization signal and provided by broadcast information Agreement Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader. Agreement Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios - Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123 Continue studying feasibility of the following open loop power control methods considering device complexity: - Methods of open loop power control - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader - Required information at the device side at least includes reader Tx power, received target power at the reader, etc. - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range. - The RSRP range is based on RSRP threshold(s) configured by reader or predefined - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s) - FFS details (e.g., required measurements, required information) Continue studying the necessity and feasibility of closed loop power control for D2R.

In this contribution, we discuss the other procedures for Device 2b/C for Rel-20 A-IoT.

- Discussion

  - Initial frequency/broadcast information acquisition procedure

In RAN1 #122bis meeting, the agreement was made regarding the study of broadcast information as follows:

Agreement For R2D, study at least the following aspects for the study of broadcast information: - Information included in the broadcast information - Whether to use PRDCH to carry the broadcast information - Transmission of broadcast information: - Option 1: periodic - Option 2: aperiodic - Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

As discussed in our contribution on R2D aspects [2], a synchronous environment between the reader and the device is required to support the device’s initial access procedure and R2D/D2R transmission and reception for DO-A traffic. To this end, it is beneficial to support a periodic synchronization signal. Following the reception of the periodic synchronization signal, it is also beneficial for broadcast information to be transmitted periodically. For example, the broadcast information can be defined to be transmitted at a specific time offset after the synchronization signal. In terms of frequency resources, it is beneficial for the broadcast information to be transmitted on the same frequency resource as the periodic synchronization signal.

Observation #1: It is beneficial to transmit broadcast information periodically (with a time offset from a periodic synchronization signal), and to use the same frequency resource for both the synchronization signal and the broadcast information to enable efficient and synchronized reception at the device.

Regarding broadcast information, introducing a new R2D channel is not preferred. Instead, it is preferred to transmit broadcast information via the existing PRDCH. Specifically, when the amount of broadcast information is large (e.g., time/frequency resources for DO-A, indication of request/polling usage for DO-A operation, configuration of the periodic synchronization signal), a paging-like message transmitted over PRDCH can be considered. Conversely, when the amount of broadcast information is small (e.g., only the configuration of the periodic synchronization signal), a short PRDCH transmission similar to an access trigger message can be considered.

Observation #2: It is preferred to reuse existing PRDCH for broadcast information rather than introducing a new R2D channel dedicated for the broadcast information.

  - Enhancements on random access from procedural aspects

    - Enhanced TDMA for Msg1/Msg3

In RAN1 #122 meeting, the agreement was made regarding the enhanced TDMA for Msg1/Msg3 as follows:

Agreement At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following: - X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message) - X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

In Rel-19 A-IoT, Msg1 transmission is defined to use up to two TDM resources (i.e., X1 = 2). However, Device 2b/C may exhibit smaller residual SFO compared to Device 1, which allows for the consideration of using more than two TDM resources (e.g., X1 = 3 or 4). When more than two TDM resources are supported, further study may be required to enhance the timing offset (i.e., TR→D). Specifically, if the device selects a Msg1 TDM resource that is positioned later among the available TDM resources, a larger TR→D value may be needed to ensure reliable reception and synchronization. Furthermore, if the X1 value becomes large, it may be worth considering the transmission of additional synchronization signal(s) (e.g., CFO calibration signal(s)) at specific Msg1 occasion(s), to support accurate time and frequency synchronization at the device side.

Observation #3: Regarding the Msg1 TDM enhancement, the following observations are made:

- Device 2b/C may exhibit smaller residual SFO compared to Device 1, which can be beneficial in enabling the use of more than two TDM resources (e.g., X1 = 3 or 4).

- When more than two TDM resources are supported,

  - a larger TR→D value may be needed to ensure reliable reception and synchronization.

  - considering additional synchronization signaling may be beneficial to ensure reliable Msg1 transmission.

Additionally, for similar reasons, Msg3 transmission using TDM (i.e., X3 > 1) may be considered in Rel-20 A-IoT. If TDM is introduced for Msg3 transmission, enhancements may be needed for Msg2 PRDCH to indicate the corresponding Msg3 transmission resources. As one possible solution, Msg3 TDM/FDM resources could be indicated per echoed random ID via Msg2 PRDCH. For example, the Msg3 transmission resources may be signaled through either a dedicated signaling field or a joint signaling field. Furthermore, enhancements related to timing offset (i.e., TR→D) may also be required.

Observation #4: Regarding the Msg3 TDM enhancement, the following observations are made:

- Device 2b/C may exhibit smaller residual SFO compared to Device 1, which can be beneficial in enabling the use of multiple TDM resources (e.g., X3 > 1).

- Enhancements may be needed for Msg2 PRDCH to indicate the corresponding Msg3 transmission resources, for example by indicating Msg3 TDM/FDM resources per echoed random ID.

- When multiple TDM resources are supported, a TR→D value similar to that for Msg1 may be needed to ensure reliable operation.

    - CDMA for Msg1

In RAN1 #122bis meeting, the agreement was made regarding the CDMA for Msg1 as follows:

Agreement For the study of binary sequence-based Msg1 for the study of necessity and feasibility of D2R CDM(A) for Device 2b and for Device C, study at least the following aspects: - Sequence type (e.g. Gold sequence, m-sequence) - Length of the sequence - Number of sequences Agreement Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures. - FFS: other cases, except for data transmission

During the last meeting, some companies expressed interest in a feasibility study on Msg1 CDMA. However, considering that Msg1 TDMA/FDMA has already been defined in Rel-19 A-IoT, and that further enhancements to Msg1 TDMA are under consideration, the necessity of Msg1 CDMA remains unclear, given the potential for sufficient Msg1 resource capacity.

Observation #5: Msg1 TDMA/FDMA has already been defined in Rel-19 A-IoT, and Msg1 TDMA enhancements are under consideration; thus, the necessity of Msg1 CDMA remains unclear.

Nevertheless, as agreed in the previous meeting, further study on binary sequence-based Msg1 is to be pursued. Both Gold sequence and m-sequence can be considered as candidate options for Msg1 sequence type. Gold sequences are generated by combining two m-sequences and provide good cross-correlation properties, which help reduce interference among multiple devices. In contrast, m-sequences are generated using linear feedback shift registers and are simpler to implement. They offer good auto-correlation properties, which are useful for timing detection, but their cross-correlation performance is generally inferior to that of Gold sequences.

In Rel-19 A-IoT, Msg1 used a 16-bit random number (RN16) with an additional 6-bit CRC, resulting in a total length of 22 bits. If binary sequence-based Msg1 is considered, excluding CRC may be preferable. Therefore, the sequence length can be defined within 16 bits, similar to Rel-19 A-IoT. As both Gold and m-sequences allow sequence generation with length 15, this length can be considered appropriate.

The number of sequences is related to the multiplexing capacity to be supported. If higher capacity is required, Gold sequence may be preferable, as up to 15 distinct sequences can be defined for length 15. Conversely, if lower capacity suffices, m-sequence can be used, which provides up to 2 distinct sequences for length 15. Cyclic shifts may also be considered for both sequence types to increase the number of available sequences.

Observation #6: If Msg1 CDMA is required in Rel-20, the following observations are made:

- Regarding sequence type:

  - Gold sequence: Offers superior cross-correlation properties for interference mitigation, but is more complex to implement.

  - m-sequence: Simpler to implement with good auto-correlation for timing detection, but has relatively poor cross-correlation performance.

- Regarding sequence length, it is observed that Rel-19 A-IoT used a 16-bit random number plus 6-bit CRC, but for binary sequence-based Msg1, excluding CRC and adopting a sequence length of around 15 bits appears reasonable.

- Regarding number of sequences, it is observed that multiplexing capacity depends on the number of sequences (e.g., for length 15, Gold sequence supports up to 15 distinct sequences, while m-sequence supports up to 2 distinct sequences).

Moreover, if Msg1 CDMA is to be considered, some enhancements to the random access procedure may be required. First, the reader may provide the successfully received binary sequence (or sequence index) to the device via Msg2 PRDCH, along with the time/frequency index of Msg1 transmission. Upon receiving this information, the device can verify whether the indicated binary sequence (or sequence index) matches its own transmission and, if time/frequency index information is provided, confirm whether the indicated transmission occasion is correct. Based on this verification and the Msg3 resource information included in Msg2, the device can transmit Msg3 PDRCH.

Next, if binary sequence-based Msg1 allows devices to select different D2R modulations, the cross-correlation properties between sequences may be distorted. Therefore, binary sequence-based Msg1 should be transmitted using a predefined D2R modulation. Subsequently, based on the reader’s decision after receiving Msg1, the reader may indicate the D2R modulation to be used for Msg3 PDRCH transmission via the Msg2 PRDCH payload. If the reader does not explicitly indicate the modulation in Msg2, the device may determine the D2R modulation for Msg3 PDRCH transmission based on its own implementation.

Proposal #1: If Msg1 CDMA is required in Rel-20, the device expects the reader to include the binary sequence (or sequence index) in Msg2 PRDCH.

Proposal #2: If Msg1 CDMA is required in Rel-20, the device shall transmit Msg1 using a predefined D2R modulation.

- The D2R modulation for Msg3 PDRCH transmission can be indicated via Msg2 PRDCH.

    - Timing offsets for random access

In RAN1 #122 meeting, the agreement was made regarding the timing offset as follows:

Agreement At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g., - timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) - timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min) - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4 - timing offset between R2D and corresponding subsequent D2R provided by a reader - timing offset between D2R and corresponding subsequent R2D provided by a reader At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g., - timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

As discussed in our contribution on R2D aspects [2], if FEC is introduced for R2D, further study may be required to assess whether the TRD and/or TDR timing values need to be adjusted. Specifically, from the device perspective, decoding time is required upon PRDCH reception, which may necessitate an increase in the minimum TRD value. On the other hand, from the reader perspective, additional encoding time for PRDCH may be needed after receiving PDRCH, which may require an increase in the minimum TDR value.

Observation #7: If R2D FEC is introduced:

- from the device perspective, decoding time is required upon PRDCH reception, which may necessitate an increase in the minimum TRD value.

- from the reader perspective, additional encoding time for PRDCH may be needed after receiving PDRCH, which may require an increase in the minimum TDR value.

The method of providing the timing offset between R2D and the corresponding subsequent D2R, or between D2R and the corresponding subsequent R2D, allows the device to anticipate the timing of the next transmission or reception event based on the previous one. This enables timely CFO calibration, synchronization, and preparation for decoding. However, if the interval between the two events is long, frequency drift may occur, which can affect the accuracy of the subsequent transmission or reception.

Observation #8: Providing the timing offset between R2D and the subsequent D2R (or vice versa) enables the UE to anticipate the timing of the next communication event, facilitating timely CFO calibration and synchronization. However, if the interval is long, frequency drift may degrade the accuracy of the subsequent transmission or reception.

In Rel-19 A-IoT, no dedicated Msg2 monitoring window is defined within the contention-based access procedure. That is, after transmitting Msg1, the device is defined to continuously monitor Msg2 until Msg2 is received or the access triggering message is received. However, if this behavior is reused, it may be inefficient from an energy-saving perspective. Therefore, it may be beneficial to define a Msg2 monitoring window for Device 2b/C within the contention-based access procedure. In this case, the device remains in the ON state during the Msg2 monitoring window to perform Msg2 monitoring, and transitions to the SLEEP or OFF state after the window ends to perform energy harvesting. Here, the information for Msg2 monitoring window can be provided by the reader, for example, via a paging message.

Proposal #3: Study the Msg2 monitoring window in Rel-20 A-IoT.

In Rel-19 A-IoT, the maximum number of access occasions for Msg1 PDRCH corresponding to a paging message (or an access triggering message) and the maximum number of random numbers that a reader can provide via a single Msg2 PRDCH are defined differently. For example, from the reader’s perspective, up to 16 random numbers can be successfully received via Msg1 PDRCH corresponding to a paging message (or an access triggering message), whereas only up to 8 random numbers can be delivered via a single Msg2 PRDCH. As a result, if a device does not decode its random number in the Msg2 PRDCH, it should continuously monitor subsequent Msg2 PRDCH transmissions without knowing when the next one will be transmitted. This behavior, similar to the reasons mentioned above, is inefficient from an energy-saving perspective. To address this, transmission of multiple Msg2 PRDCHs in succession could be considered. In this context, the timing offset between two consecutive R2D transmissions (TR2D_R2D) may also need to be considered.

Proposal #4: Study the timing offset between two consecutive R2D transmissions (TR2D_R2D) for Msg2 PRDCH transmission.

  - DO-A

In RAN1 #122bis meeting, the agreement was made regarding the first D2R transmission for DO-A for Device 2b and for Device C as follows:

Agreement For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options: - Option 1: Msg1-like content - Option 2: DO-A data payload - FFS: with or without device ID - Option 3: DO-A scheduling request Agreement For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility: - Option 1: Periodic D2R resource allocation - Option 2: Aperiodic D2R resource allocation Agreement For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility: - Option 1: necessary information provided by broadcast information - Option 2: necessary information provided by a paging-like R2D message - Option 3: necessary information derived from a synchronization signal and provided by broadcast information Agreement Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader.

For the content of the first D2R transmission for DO-A, three options have been agreed upon for further study. These options may be applicable under different scenarios. One scenario considers the case where DO-A traffic is triggered before devices have completed the inventory procedure. In this scenario, the reader may allocate a common resource for the first D2R transmission for DO-A, allowing devices to transmit Msg1-like content in a contention-based manner. Specifically, multiple devices may utilize the first D2R transmission opportunity to send Msg1-like content. Among these devices, those that successfully complete the inventory procedure may be granted a D2R resource for DO-A transmission. Upon receiving the D2R grant, the corresponding devices can transmit the DO-A data payload. The inventory procedure in this scenario may follow a slotted ALOHA-based operation, similar to the Rel-19 A-IoT mechanism. While this approach introduces latency due to the inclusion of the inventory process within the DO-A procedure, it offers the advantage that devices are not required to perform inventory in advance of DO-A traffic generation.

Another scenario assumes that devices have already completed the inventory procedure and have been configured with the resource for the first D2R transmission for DO-A prior to the arrival of DO-A traffic. In this case, the reader may allocate the resource for the first D2R transmission in a device-specific manner, allowing each device to directly transmit the DO-A data payload. This corresponds to a contention-free DO-A transmission. This scenario is particularly advantageous when the DO-A data payload must be delivered with low latency or under urgent conditions. However, it requires that devices perform the inventory procedure in advance of DO-A traffic generation.

A third scenario may also be considered in which the device transmits a scheduling request after DO-A traffic has been triggered. In this scenario, the reader may allocate a common resource for the first D2R transmission for DO-A, allowing devices to transmit a DO-A scheduling request (SR) using the allocated resource. The scheduling request serves as the first D2R transmission for DO-A. Upon successful reception of the scheduling request, the reader may allocate D2R resource(s) via a D2R grant, enabling the device to transmit the DO-A data payload. This approach has the advantage of requiring only a small D2R resource for the SR, rather than allocating full D2R resources for the DO-A data payload in advance. However, if multiple devices (without prior inventory) simultaneously transmit SRs and receive D2R grants, collisions may occur due to concurrent DO-A payload transmissions. To mitigate such collisions, the SR transmission resource may be randomized across devices. As another approach, an inventory step may be inserted following the D2R grant to minimize potential transmission collisions. The scenarios described above can be illustrated as shown in Figures 1, 2, and 3.

Figure 1. The first DO-A transmission occurs before the inventory procedure is completed.

Figure 2. The first DO-A transmission occurs after the inventory procedure is completed.

Figure 3. Device transmits a SR after DO-A traffic has been arrived.

Observation #9: For the content of the first D2R transmission for DO-A, the following observations are made:

- Option 1 (i.e., Msg1-like content): Uses Msg1-like content for first D2R transmission, enabling contention-based access but increasing latency due to integrated inventory.

- Option 2 (i.e., DO-A data payload): Directly transmits DO-A data payload for low latency, but requires prior inventory and resource configuration.

- Option 3 (i.e., Scheduling Request (SR)): Starts with a small SR for resource efficiency, but risks collisions if multiple devices transmit payloads simultaneously.

For the resource allocation method for the first D2R transmission for DO-A, two options have been agreed upon for further study. One option is to periodically allocate resources for the first D2R transmission for DO-A, as shown in Figure 1, 2, and 3, which provides a straightforward and predictable mechanism for resource availability. However, if few devices utilize these resources, this approach may lead to inefficient use of D2R resources. Alternatively, the reader may transmit a polling signal/channel to check whether any devices have DO-A traffic, as shown in Figure 4. Based on the polling result, the reader can allocate resources for the first D2R transmission for DO-A. In Figure 4, it is assumed that the first D2R transmission for DO-A corresponds to a scheduling request. This approach effectively enables aperiodic resource allocation for the first D2R transmission and offers improved efficiency in D2R resource utilization.

Figure 4. The device transmits a SR after the DO-A traffic has arrived and following the reader polling.

Observation #10: For the resource allocation method for the first D2R transmission for DO-A, periodic allocation provides predictable resource availability but may waste resources when device activity is low, whereas polling-based aperiodic allocation improves efficiency by assigning resources only when DO-A traffic is detected.

For providing necessary information for the first D2R transmission for DO-A, three options have been agreed upon for further study. When allocating resources for the first D2R transmission for DO-A, it may be preferable to deliver the necessary information via broadcast information or a paging-like R2D message (Option 1 or Option 2). Conversely, Option 3, which delivers resource information through synchronization signals, is considered less desirable because it requires blind detection of synchronization signals, thereby increasing device complexity.

Observation #11: For providing necessary information for the first D2R transmission for DO-A, broadcast information or paging-like messages are preferred for delivering resource information, while synchronization signal-based delivery is less desirable due to increased device complexity.

For the period of necessary information for the first D2R transmission for DO-A, the necessary information for the first D2R transmission for DO-A may be provided periodically with each broadcast information transmission, or alternatively, the reader may transmit paging-like R2D messages aperiodically to provide resource information. Ultimately, the choice depends on how efficiently the reader aims to utilize R2D resources.

Observation #12: For the period of necessary information for the first D2R transmission for DO-A, the necessary information can be provided periodically or aperiodically, and the choice depends on how efficiently the reader aims to utilize R2D resources.

  - Power control

In RAN1 #122bis meeting, the agreement was made regarding the D2R Tx power control for Device 2b and for Device C for outdoor scenarios as follows:

Agreement Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios - Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123 Continue studying feasibility of the following open loop power control methods considering device complexity: - Methods of open loop power control - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader - Required information at the device side at least includes reader Tx power, received target power at the reader, etc. - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range. - The RSRP range is based on RSRP threshold(s) configured by reader or predefined - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s) - FFS details (e.g., required measurements, required information) Continue studying the necessity and feasibility of closed loop power control for D2R.

In outdoor scenarios, supporting Device 2b/C may require consideration of D2R Tx power control to maintain link reliability under varying channel conditions. Outdoor environments typically involve larger coverage areas and higher path loss compared to indoor deployments, which can impact communication performance. Adjusting transmit power based on link conditions can help compensate for channel variations such as shadowing and fading, while also reducing unnecessary interference. In addition, power control may contribute to improved energy efficiency for devices operating on limited power sources. Furthermore, in scenarios where multiple devices are located at varying distances from the reader, the near-far problem may arise, potentially degrading the performance of devices with weaker links. Therefore, transmit power control may be considered as a mechanism for enhancing connectivity, mitigating near-far effects, and improving overall resource efficiency in outdoor deployments.

Observation #13: D2R Tx power control may help maintain link reliability and improve resource efficiency in outdoor scenarios, particularly by mitigating the near-far problem, but its implementation adds complexity to device operation.

For the open loop power control methods, two options have been agreed upon for further study. Option 1 relies on pathloss-based power calculation using a predefined formula and configuration parameters provided by the reader. This approach enables accurate and environment-adaptive power control, which can lead to improved power efficiency and more reliable communication. Although it may introduce some implementation complexity due to the need for pathloss estimation and parameter handling, it offers better adaptability to varying radio conditions. In contrast, Option 2 uses RSRP-based power selection, where the device maps measured RSRP to a predefined transmit power level based on configured thresholds. While this method simplifies implementation and reduces computational burden, it may result in suboptimal power settings due to the limited granularity and indirect nature of RSRP as a channel quality indicator.

Observation #14: For the open loop power control methods, the following observations are made:

- Option 1: Pathloss-based power control offers higher accuracy and adaptability to varying radio conditions, making it suitable for scenarios where efficient and reliable communication is critical.

- Option 2: RSRP-based power control provides a simpler implementation with lower device complexity, but may be less effective in dynamically changing environments due to its coarser granularity.

- Conclusions

In this contribution, we discussed the other procedures for Device 2b/C for Rel-20 A-IoT, we have the following proposals and observations.

Observation #1: It is beneficial to transmit broadcast information periodically (with a time offset from a periodic synchronization signal), and to use the same frequency resource for both the synchronization signal and the broadcast information to enable efficient and synchronized reception at the device.

Observation #2: It is preferred to reuse existing PRDCH for broadcast information rather than introducing a new R2D channel dedicated for the broadcast information.

Observation #3: Regarding the Msg1 TDM enhancement, the following observations are made:

- Device 2b/C may exhibit smaller residual SFO compared to Device 1, which can be beneficial in enabling the use of more than two TDM resources (e.g., X1 = 3 or 4).

- When more than two TDM resources are supported,

  - a larger TR→D value may be needed to ensure reliable reception and synchronization.

  - considering additional synchronization signaling may be beneficial to ensure reliable Msg1 transmission.

Observation #4: Regarding the Msg3 TDM enhancement, the following observations are made:

- Device 2b/C may exhibit smaller residual SFO compared to Device 1, which can be beneficial in enabling the use of multiple TDM resources (e.g., X3 > 1).

- Enhancements may be needed for Msg2 PRDCH to indicate the corresponding Msg3 transmission resources, for example by indicating Msg3 TDM/FDM resources per echoed random ID.

- When multiple TDM resources are supported, a TR→D value similar to that for Msg1 may be needed to ensure reliable operation.

Observation #5: Msg1 TDMA/FDMA has already been defined in Rel-19 A-IoT, and Msg1 TDMA enhancements are under consideration; thus, the necessity of Msg1 CDMA remains unclear.

Observation #6: If Msg1 CDMA is required in Rel-20, the following observations are made:

- Regarding sequence type:

  - Gold sequence: Offers superior cross-correlation properties for interference mitigation, but is more complex to implement.

  - m-sequence: Simpler to implement with good auto-correlation for timing detection, but has relatively poor cross-correlation performance.

- Regarding sequence length, it is observed that Rel-19 A-IoT used a 16-bit random number plus 6-bit CRC, but for binary sequence-based Msg1, excluding CRC and adopting a sequence length of around 15 bits appears reasonable.

- Regarding number of sequences, it is observed that multiplexing capacity depends on the number of sequences (e.g., for length 15, Gold sequence supports up to 15 distinct sequences, while m-sequence supports up to 2 distinct sequences).

Proposal #1: If Msg1 CDMA is required in Rel-20, the device expects the reader to include the binary sequence (or sequence index) in Msg2 PRDCH.

Proposal #2: If Msg1 CDMA is required in Rel-20, the device shall transmit Msg1 using a predefined D2R modulation.

- The D2R modulation for Msg3 PDRCH transmission can be indicated via Msg2 PRDCH.

Observation #7: If R2D FEC is introduced:

- from the device perspective, decoding time is required upon PRDCH reception, which may necessitate an increase in the minimum TRD value.

- from the reader perspective, additional encoding time for PRDCH may be needed after receiving PDRCH, which may require an increase in the minimum TDR value.

Observation #8: Providing the timing offset between R2D and the subsequent D2R (or vice versa) enables the UE to anticipate the timing of the next communication event, facilitating timely CFO calibration and synchronization. However, if the interval is long, frequency drift may degrade the accuracy of the subsequent transmission or reception.

Proposal #3: Study the Msg2 monitoring window in Rel-20 A-IoT.

Proposal #4: Study the timing offset between two consecutive R2D transmissions (TR2D_R2D) for Msg2 PRDCH transmission.

Observation #9: For the content of the first D2R transmission for DO-A, the following observations are made:

- Option 1 (i.e., Msg1-like content): Uses Msg1-like content for first D2R transmission, enabling contention-based access but increasing latency due to integrated inventory.

- Option 2 (i.e., DO-A data payload): Directly transmits DO-A data payload for low latency, but requires prior inventory and resource configuration.

- Option 3 (i.e., Scheduling Request (SR)): Starts with a small SR for resource efficiency, but risks collisions if multiple devices transmit payloads simultaneously.

Observation #10: For the resource allocation method for the first D2R transmission for DO-A, periodic allocation provides predictable resource availability but may waste resources when device activity is low, whereas polling-based aperiodic allocation improves efficiency by assigning resources only when DO-A traffic is detected.

Observation #11: For providing necessary information for the first D2R transmission for DO-A, broadcast information or paging-like messages are preferred for delivering resource information, while synchronization signal-based delivery is less desirable due to increased device complexity.

Observation #12: For the period of necessary information for the first D2R transmission for DO-A, the necessary information can be provided periodically or aperiodically, and the choice depends on how efficiently the reader aims to utilize R2D resources.

Observation #13: D2R Tx power control may help maintain link reliability and improve resource efficiency in outdoor scenarios, particularly by mitigating the near-far problem, but its implementation adds complexity to device operation.

Observation #14: For the open loop power control methods, the following observations are made:

- Option 1: Pathloss-based power control offers higher accuracy and adaptability to varying radio conditions, making it suitable for scenarios where efficient and reliable communication is critical.

- Option 2: RSRP-based power control provides a simpler implementation with lower device complexity, but may be less effective in dynamically changing environments due to its coarser granularity.

- References

RAN1 Chair’s Notes in the 3GPP TSG RAN WG1 #122bis

R1-2508900, “R2D air interface for Device 2b/C”, LG Electronics, RAN1#123, Dallas, USA, November 2025.