3GPP TSG- RAN WG1 #123 Meeting                                                                             R1-2508917

Dallas, US, Nov 17th – 21st, 2025

Agenda item:	11.3.1

Title:	Discussions on 6G Waveforms

Source:	Lekha Wireless Solutions

Document for:	Discussion/ Decision

- Introduction

In RAN1#122 [1] meeting, the following agreements have been achieved on the suggestions for 6G waveforms.

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded.

A broad support for CP-OFDM and DFT-s-OFDM has been observed to be the baseline for 6GR waveform for uplink. Similarly, a broad support for CP-OFDM has been observed to be the baseline for 6GR waveform for downlink [2].

Also, in RAN1#122-bis meeting, the following agreements were proposed/made [3].

Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - Practical modulation and coding should be used - Evaluation metrics maybe revisited in the future. - Note: Companies to report how to calculate the Tx power gain - Other metrics, e.g. effective occupied BW, In-band emissions, etc. are to be used, if applicable Ericsson proposal for system level - For uplink low-PAPR proposals, the system level evaluations taking MPR into account are used to establish the overall benefit of schemes. - The MPR is determined by using the PA model. DL evaluation for low-PAPR Feature lead proposal: - For downlink low-PAPR proposals the primary evaluation criterion is one of (depending on the proposal) - Net Gain [dB] = Tx power gain - link loss relative to the reference @10% BLER - A realistic base station PA model should be used - Network energy saving - Other metrics, e.g. occupied BW, are to be used, if applicable

- DFT-s-OFDM in DL

In LTE and 5G NR, the downlink (base station → user) primarily uses CP-OFDM (Cyclic Prefix OFDM). That’s because base stations can handle the high PAPR (Peak-to-Average Power Ratio) of OFDM easily since they have powerful and efficient amplifiers. Downlink needs flexible allocation of subcarriers across multiple users, which OFDM handles well.

Similarly, for uplink (user → base station), DFT-s-OFDM (also called SC-FDMA in LTE) is preferred. User devices (UEs) have power-limited amplifiers. Plain OFDM would cause high PAPR, reducing power efficiency and draining battery faster. DFT spreading gives a single-carrier-like waveform with lower PAPR, more suited for handheld devices.

Normally, downlink does not use DFT-s-OFDM. But there are exceptions in 5G NR. 5G NR allows DFT-s-OFDM for downlink transmissions, but only in certain cases:

- Uplink-like scenarios in downlink testing (e.g., device-to-device communication or when evaluating PAPR efficiency).

- Optional feature in FR1 (sub-6 GHz) for PDSCH (Physical Downlink Shared Channel) to reduce PAPR in some deployments.

- In massive MIMO or high-power downlink cases, lower PAPR can reduce distortion at power amplifiers, hence some vendors may choose it.

5G NR adds the option of using DFT-s-OFDM in the downlink (something LTE did not do).

The motivations behind using DFT-s-OFDM in DL are:

1.	Lower PAPR leads to better power efficiency

    - Even though gNB amplifiers are strong, massive MIMO and small-cell transmitters may need more power-efficient operation.

    - DFT-s-OFDM reduces nonlinear distortion and improves coverage at the cell edge.

2.	Improved UE receiver performance

    - UEs may benefit from a cleaner received signal (lower distortion, lower EVM).

    - Especially useful for PDSCH (Physical Downlink Shared Channel) carrying user data.

3.	Consistency with uplink

    - Since uplink uses DFT-s-OFDM, supporting it in the downlink too can simplify some designs (e.g., for device-to-device relaying or symmetry in testing scenarios).

In 3GPP 5G NR, downlink DFT-s-OFDM is optional:

    - FR1 (≤ 6 GHz): CP-OFDM (mandatory), DFT-s-OFDM (optional).

    - FR2 (mmWave): Only CP-OFDM.

It is supported mainly for PDSCH, but not for PDCCH (control channels). Typical use cases include:

    - Coverage-limited cells (e.g., rural deployments, cell-edge users).

    - Low-frequency bands (FR1) where spectral efficiency is less critical, but robust coverage is more important.

    - Small-cell base stations where power amplifier efficiency is a concern.

Proposal 1: DFT-s-OFDM has benefits in terms of PAPR and receiver performance. Due to the multi-carrier capabilities of CP-OFDM, it is more preferred in DL. However, there are use-cases like coverage-limited cells, small-cell BSs etc., where DFT-s-OFDM can help in power efficient transmission.

2.1 Tx-Power Gain Calculation and Other Metrics

(i) Adopting a realistic PA model

Using a nonlinear PA model that reflects UE or small-cell hardware:

- Rapp model or memory polynomial model calibrated to measured PA data.

- Rapp model (soft limiter):

y(t)=x(t)(1+(∣x(t)∣/Asat)2p)1/(2p)

with p=2–3, Asatthe saturation amplitude.

(ii) Define Reference and Proposed Tx Powers

Use the same spectral efficiency (same MCS, code rate, modulation).
For each waveform:

- Determine PA back-off required to meet EVM and ACLR limits from 3GPP TS 38.104 (for gNB) or TS 38.101-1 (for UE):

  - ACLR limit (FR1): 45 dB (TS 38.104 Section 6.6.3.2).

  - EVM limits: QPSK < 17.5 %, 16QAM < 12.5 %, 64QAM < 8 %, 256QAM < 3.5 %. (TS 38.104 Section 6.5.2.2).

- Record the average transmitted power at which these limits are met (after PA and any filtering).

- Tx Power Gain [dB] = difference in average output power between the proposed and reference waveform under the same constraints:

Tx Power Gain [dB]=Ptx,prop-Ptx,ref

Interpretation: a 3 dB gain means the proposed waveform can transmit twice the average power while satisfying linearity constraints.

Secondary Evaluation Metrics for Uplink Low-PAPR Techniques

The following table summarizes the recommended secondary indicators for evaluating uplink low-PAPR waveform proposals in addition to the primary Net Gain metric. These metrics help assess linearity, spectral efficiency, and implementation impact in alignment with 3GPP RAN4 evaluation methodology.

| Metric | Purpose | Calculation / Notes |
| --- | --- | --- |
| Effective Occupied Bandwidth (EOB) | Checks spectral spreading due to nonlinearities. | Bandwidth containing 99% of total transmitted power. Evaluated using PSD integration (99% OBW). |
| ACLR (Adjacent Channel Leakage Ratio) | Verifies linearity and spectral compliance. | Measured using 3.84 MHz (FR1) or 12 MHz (FR2) channel filters; compared against mask limits. |
| EVM (Error Vector Magnitude) | Quantifies in-band distortion introduced by PAPR reduction. | EVM=Sref-Sout2Sref2×100%. Evaluated after PA or DPD if applied. |
| SNR Degradation (dB) | Measures link-quality penalty under fading conditions. | Difference in SNR required to achieve 10% BLER relative to reference waveform. |
| PA Efficiency (%) | Evaluates energy efficiency improvement of low-PAPR signal. | η=Pout,avgPDC, at given back-off. It can be used to compute Net Energy Gain. |
| Complexity / Side Information | Reports algorithmic and signaling overhead of the PAPR reduction method. | Includes FFT operations, tone reservation overhead, side-information bits per symbol. |

Proposal 2: Transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Complementary metrics—such as SNR degradation, effective occupied bandwidth, and PA efficiency can also be analysed to provide a comprehensive assessment of waveform linearity and spectral behaviour.

3.  OFDM enhancements for the downlink (DL)

In the downlink, the transmitter is usually the base station (BS), which has more processing power, less energy constraints, and a need to serve multiple UEs simultaneously with high throughput and robustness. Plain OFDM (as used in LTE downlink) works well but has some challenges. It suffers from high PAPR (Peak-to-Average Power Ratio) leading to inefficient PA usage. It has sensitivity to phase noise, Doppler, and CFO. It suffers from out-of-band emissions due to rectangular pulse shaping. It has limited flexibility for mixed numerologies. Hence, enhancements and alternative OFDM-based schemes have been explored.

Enhancements in Classical OFDM for DL

(a) Windowing & Filtering: Smooth symbol edges (e.g., raised cosine windowing) which reduces OOB leakage and helps spectral coexistence with other systems.

(b) Sub-band Filtering: Each group of subcarriers are filtered separately (f-OFDM, UF-OFDM). It improves spectral localization and supports mixed numerologies. Also, useful in 5G for coexistence of eMBB, URLLC, mMTC in DL.

(c) Cyclic Prefix Variants: Variable CP length to adapt to channel delay spread. Short CP for low-latency DL, long CP for highly dispersive channels.

(d) DFT-spread OFDM (DFT-s-OFDM): Mostly used in uplink for lower PAPR. In downlink, it can be used in special cases (e.g., enhanced coverage DL). It reduces PAPR enabling more efficient PA usage at BS and UEs.

(e) Precoding and MIMO Enhancements: Advanced precoders like ZF, MMSE etc. improve spectral efficiency. Beamforming in DL enhances robustness to Doppler and multipath.

| Waveform | Key Idea | Downlink Benefit(s) |
| --- | --- | --- |
| f-OFDM (Filtered-OFDM) | Sub-band filtering of OFDM symbols | Good for flexible DL spectrum allocation, low OOB leakage |
| UF-OFDM (Universal Filtered-OFDM) | Sub-band wise FIR filtering | Better spectral confinement, shorter filter achieving low latency |
| FBMC (Filter Bank Multicarrier) | Subcarrier filtering with offset QAM | Excellent frequency localization, but higher complexity and MIMO integration challenges |
| GFDM (Generalized FDM) | Flexible block-based multicarrier | Suited for fragmented DL spectrum, but high complexity |
| WOLA-OFDM (Windowed Overlap and Add) | Uses overlapping windowed symbols | Reduced OOB, backward compatible with OFDM |
| DFT-s-OFDM (a.k.a. SC-FDMA in LTE UL) | Spreading in frequency domain | Lower PAPR that could be used in DL for energy-limited scenarios |
| Zak-OTFS | Delay-Doppler domain modulation with less complex Zak transform | Robust in high Doppler DL (e.g., fast-moving UEs, V2X) with less complexity |

Proposal 3: Plain OFDM works well in DL but has some limitations such as high PAPR, sensitivity to phase noise and Doppler, out-of-band emissions, and limited flexibility to mixed numerologies. Hence, enhancements and alternative OFDM-based schemes need to be explored including windowing and filtering, sub-band filtering, cyclic prefix-based enhancements, precoding and MIMO enhancements, and DFT-s-OFDM.

  - Tx power-gain calculations and other metrics for DL

(i) Base Station PA Model

- Use a realistic BS-class PA model (typically Doherty or GaN-based) incorporating nonlinearity and memory effects.

- Recommended models:

  - Rapp model (p = 3–4) for soft-limiting AM-AM compression.

  - Memory polynomial model for wideband (>100 MHz) carriers.

- Include PA efficiency vs. back-off characteristics based on measured or literature data.

(ii) Definition of Tx Power Gain

- The Tx Power Gain [dB] quantifies how much more power can be transmitted before hitting EVM or ACLR constraints:

Tx Power Gain [dB]=Ptx,prop-Ptx,ref

- Both waveforms must use identical modulation and coding (same spectral efficiency).

- Ptxis the average output power at which ACLR and EVM limits in 3GPP TS 38.104 section 6.6 are just satisfied:

  - ACLR: 45 dB (FR1)

  - EVM: 64QAM ≤ 8 %, 256QAM ≤ 3.5 %

(iii) Link Loss (SNR Degradation)

- Calculate link loss as the difference in the required post-equalization SNR to achieve 10 % BLER under fading:

Link Loss [dB]=SNRprop-SNRref

- Simulate under realistic downlink channels (e.g., TDL-C, CDL-D) with imperfect channel estimation and MMSE equalization.

(d) Net Gain Calculation

Net Gain [dB]=Tx Power Gain [dB]-Link Loss [dB]

A positive Net Gain indicates that the low-PAPR waveform provides overall benefit in radiated power versus performance trade-off.

Secondary Evaluation Metrics for Downlink Low-PAPR Techniques

The following table summarizes the recommended secondary indicators for evaluating downlink low-PAPR waveform proposals in addition to the primary Net Gain metric. These metrics capture PA efficiency, energy saving potential, spectral behavior, and implementation impact under realistic base station conditions.

| Metric | Purpose | Computation / Note |
| --- | --- | --- |
| PA Efficiency (%) | Quantifies energy benefit from reduced PA back-off. | η=Pout,avgPDC. Evaluate at the same ACLR/EVM compliance point for both reference and proposed signals. |
| Network Energy Saving (%) | Estimates system-level energy gain due to higher PA efficiency and reduced power consumption. | Compute total DC power saving at cell or network level under equal throughput assumptions. |
| Occupied Bandwidth (OBW) | Checks for spectral regrowth and compliance with emission masks. | Determine the frequency span containing 99% of total transmitted power and compare it with mask limits. |
| ACLR (Adjacent Channel Leakage Ratio) | Assesses linearity and spectral containment of the downlink signal. | Measure ACLR using standardized filters (e.g., 3.84 MHz for FR1, 12 MHz for FR2). |
| EVM (%) | Evaluates in-band distortion caused by PAPR reduction and PA nonlinearities. | EVM=Sref-Sout2Sref2×100%. Evaluate per modulation format (64QAM, 256QAM). |
| Complexity / Implementation Impact | Quantifies DSP and DPD overhead that are introduced by the low-PAPR method. | Report computational load, DPD adaptation requirements, and side-information signaling overhead. |

Proposal 4: Downlink transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Evaluation is based on the Net Gain metric (Tx power gain – link loss at 10% BLER) using a realistic base station PA model, complemented by secondary metrics such as PA efficiency, ACLR, EVM, network energy savings, and occupied bandwidth.

- OFDM enhancements for the uplink (UL)

The story is different from downlink because here the transmitter is the UE, which has strict power and complexity constraints. UE transmit power is limited, hence need waveforms with low PAPR for efficient PA usage. They must be robust to mobility, CFO, Doppler since synchronization is harder from UEs. They need spectral efficiency and coexistence with other UL transmissions.

Enhancements in Classical OFDM for UL

- DFT-s-OFDM (a.k.a. SC-FDMA in LTE UL): Key UL waveform in LTE, also supported in 5G NR UL. It applies DFT precoding before OFDM modulation which converts multicarrier to single-carrier-like transmission. Advantages include lower PAPR (more efficient UE power amplifiers), maintains OFDM equalization simplicity, and supports MIMO.

- Windowing and Filtering: Windowed OFDM or WOLA-OFDM (Windowed Overlap and Add). These reduces OOB leakage which is important in UL where multiple UEs transmit simultaneously in adjacent bands.

- Flexible CP Length: Short CP for low latency UL, long CP for large delay spreads.

- SC-FDMA Variants: f-DFT-s-OFDM (Filtered DFT-s-OFDM): Sub-band filtering for better spectral shaping. ZP-SC-FDMA: Uses zero padding instead of CP, slightly improves spectral efficiency.

| Waveform | Key Idea | Uplink Benefit(s) |
| --- | --- | --- |
| f-OFDM (Filtered-OFDM)/ UF-OFDM (Universal Filtered-OFDM) | Sub-band filtering of OFDM symbols | Improved coexistence of multiple UEs with different numerologies |
| FBMC (Filter Bank Multicarrier) | Subcarrier filtering with offset QAM | Excellent spectral localization, could enable asynchronous UL users, but higher complexity |
| GFDM (Generalized FDM) | Flexible block-based multicarrier | Supports non-contiguous UL allocation, but complex |
| WOLA-OFDM (Windowed Overlap and Add) | Uses overlapping windowed symbols | Low complexity, reduced OOB which is suitable for UL coexistence |
| DFT-s-OFDM (a.k.a. SC-FDMA in LTE UL) | Spreading in frequency domain | Lower PAPR, main choice in LTE/5G NR UL |
| ZP-SC-FDMA | Zero-padding instead of CP | Reduced PAPR and improved spectral efficiency |

Proposal 5: UE transmit power is limited; hence uplink need waveforms with low PAPR for efficient PA usage, robust to mobility, CFO, and Doppler and need spectral efficiency and coexistence with other UL transmissions. Enhancements to current waveforms can be done using windowing and filtering techniques, CP length variations, SC-FDMA variants, and DFT-s-OFDM.

- DFT-s-OFDM enhancements for the uplink (UL)

User Equipment (UE) has limited power amplifier (PA) efficiency. High PAPR requires linear PA, which is expensive and power-hungry and reduces battery life. Hence, DFT-s-OFDM (a.k.a. SC-FDMA) was chosen for LTE UL instead of plain OFDM because it already offers lower PAPR. Still, additional PAPR reduction techniques are desired.

PAPR Reduction Techniques in DFT-s-OFDM

a. DFT Precoding (inherent feature): Converts multiple subcarriers into a single-carrier-like signal. Already reduces PAPR compared to OFDM.

b. Subcarrier Mapping Schemes

- Localized Mapping (LFDMA): Consecutive subcarriers mapped aiding lower implementation complexity.

- Distributed Mapping (IFDMA): Interleaved subcarriers help giving the lowest PAPR but needs strict frequency synchronization.

- DFDMA (Distributed FDMA): Hybrid of above two, gives a tradeoff between PAPR and frequency diversity.

c. Tone Reservation / Tone Injection: Reserving some subcarriers to generate a cancellation signal that reduces peaks or moving constellation points to reduce PAPR while maintaining Euclidean distance.

d. Clipping and Filtering: A simple method is to clip the peaks of the signal, then filter to remove out-of-band distortion. Advantages include easy and effective implementation. However, it can cause in-band distortion which increases BER.

e. Selective Mapping (SLM): Multiple candidate signals are generated by applying different phase sequences. And then we transmit the one with lowest PAPR. However, it requires side information signaling which increases overhead.

f. Partial Transmit Sequence (PTS): Partitioning the input symbols into sub-blocks. It optimizes phase factors of sub-blocks to minimize PAPR. It gives good performance but is computationally complex.

g. Precoding-Based Techniques: Applying precoding matrices before DFT spreading. Prominent examples include Zadoff-Chu sequences, Walsh-Hadamard transform, etc. They improve PAPR characteristics.

Proposal 6: Due to restricted power requirements in uplink, DFT-s-OFDM was chosen for LTE UL instead of plain OFDM because it already offers lower PAPR. Still, additional PAPR reduction techniques are desired. Some good PAPR reduction techniques include DFT precoding, sub-carrier mapping, tone reservation or injection, clipping and filtering, selective mapping, partial transmit sequence, and precoding-based techniques.

- UL DFT-s-OFDM with Rank > 1

DFT-s-OFDM is the baseline waveform for NR uplink because of its low Peak-to-Average Power Ratio (PAPR) and high PA efficiency. In NR Rel-15/16, DFT-s-OFDM supports Rank = 1, i.e., single-layer transmission only. The Rank > 1 extension introduces spatial multiplexing (multi-layer transmission) while preserving DFT precoding on each layer.

## Impact and Design Considerations

| Aspect | Impact / Description |
| --- | --- |
| Throughput | Enables multi-layer MIMO (Rank > 1) in UL, improving spectral efficiency in high SINR regions. |
| PAPR | Slightly increases with rank but remains significantly lower than CP-OFDM. However, the gap with CP-OFDM decreases with increase in rank. |
| PA Efficiency | Improved vs. CP-OFDM; slight reduction as rank increases due to multi-layer envelope summation. Again, the gap with CP-OFDM is reduced. |
| DMRS Design | Requires orthogonal or quasi-orthogonal DMRS per layer to ensure separation. |

  - Rank Extension Feasibility

In general, rank up to 2 is currently supported because it gives good balance between spectral efficiency (throughput gain) and signal quality / PA efficiency / receiver complexity. Rank>2 is generally not supported because envelope distortion and DMRS overhead outweigh throughput gains.

Proposal 7: UL DFT-s-OFDM with Rank > 1 offers a practical means to extend uplink throughput while preserving low PAPR and high PA efficiency. Practical deployments are recommended up to Rank = 2, balancing efficiency, complexity, and link performance relative to CP-OFDM with higher ranks.

- Conclusion

In this proposal, we discussed and provide our views on suggestions for 6G waveforms with following proposals:

Proposal 1: DFT-s-OFDM has benefits in terms of PAPR and receiver performance. Due to the multi-carrier capabilities of CP-OFDM, it is more preferred in DL. However, there are use-cases like coverage-limited cells, small-cell BSs etc., where DFT-s-OFDM can help in power efficient transmission.

Proposal 2: Transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Complementary metrics—such as SNR degradation, effective occupied bandwidth, and PA efficiency can also be analysed to provide a comprehensive assessment of waveform linearity and spectral behaviour.

Proposal 3: Plain OFDM works well in DL but has some limitations such as high PAPR, sensitivity to phase noise and Doppler, out-of-band emissions, and limited flexibility to mixed numerologies. Hence, enhancements and alternative OFDM-based schemes need to be explored including windowing and filtering, sub-band filtering, cyclic prefix-based enhancements, precoding and MIMO enhancements, and DFT-s-OFDM.

Proposal 4: Downlink transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Evaluation is based on the Net Gain metric (Tx power gain – link loss at 10% BLER) using a realistic base station PA model, complemented by secondary metrics such as PA efficiency, ACLR, EVM, network energy savings, and occupied bandwidth.

Proposal 5: UE transmit power is limited; hence uplink need waveforms with low PAPR for efficient PA usage, robust to mobility, CFO, and Doppler and need spectral efficiency and coexistence with other UL transmissions. Enhancements to current waveforms can be done using windowing and filtering techniques, CP length variations, SC-FDMA variants, and DFT-s-OFDM.

Proposal 6: Due to restricted power requirements in uplink, DFT-s-OFDM was chosen for LTE UL instead of plain OFDM because it already offers lower PAPR. Still, additional PAPR reduction techniques are desired. Some good PAPR reduction techniques include DFT precoding, sub-carrier mapping, tone reservation or injection, clipping and filtering, selective mapping, partial transmit sequence, and precoding-based techniques.

Proposal 7: UL DFT-s-OFDM with Rank > 1 offers a practical means to extend uplink throughput while preserving low PAPR and high PA efficiency. Practical deployments are recommended up to Rank = 2, balancing efficiency, complexity, and link performance relative to CP-OFDM with higher ranks.

4. References

[1] RAN1#122 Chair’s Notes, August 2025.

[2] R1-2506595 FL Summary 11.3.1 Moderator, August 2025.

[2] R1-2508043 FL Summary 11.3.1 Moderator, October 2025.