3GPP TSG RAN WG1 Meeting #123							R1-2508920

Dallas, USA, Nov 17th – 21st, 2025

Title:	Draft reply LS on SBFD and CA

Response to:	R1-2508304 (R2-2507746)

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN WG1

To:	RAN WG2

Cc:	RAN WG4

Contact person:	Qinyan JIANG

jiangqinyan@fujitsu.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN1 thanks RAN2 for the LS on SBFD and CA in R2-2507746.

RAN1 would like to provide the following TP for supporting SBFD with CA in the stage-2 specification (TS 38.300).

## 23.1 General Sub-Band Full Duplex (SBFD) operation is supported for a TDD carrier, enabling simultaneous downlink transmission and uplink reception at the gNB on their non-overlapping respective sub-bands. From UE perspective, full duplex is not supported. The configurations of cell-specific SBFD time and frequency resources are provided through SIB1 or dedicated signalling. In CA scenario, only one cell, either the Pcell or a Scell, can be configured with SBFD time and frequency resources. A UE can be semi-statically configured with SBFD sub-bands in downlink symbols and flexible symbols provided by tdd-UL-DL-ConfigurationCommon and this UE is referred to as an SBFD aware UE. The maximum number of UL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is one. The maximum number of DL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is two. In an SBFD symbol, except for cross-link interference measurements, a UE transmits or receives only in RBs that are both in the active UL BWP and in the UL sub-band, or both in the active DL BWP and the DL sub-band(s), respectively. A UE can be configured to transmit or receive only in non-SBFD symbols, only in SBFD symbols, or across both SBFD symbols and non-SBFD symbols for multiple transmission or reception occasions.

2	Actions

To RAN WG2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above TP into account.

3	Dates of next TSG WG1 meetings

TSG RAN1 Meeting #124			Feb 9th – 13th, 2026				Gothenburg, SE

TSG RAN1 Meeting #124-bis		Apr 13th – 17th, 2026				Malta, MT