3GPP TSG RAN WG1 #123	R1-2508923

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	8.3

Source: 	Fujitsu

Title:	Remaining issues on SBFD operation

Document for:		Discussion and decision

Introduction

This contribution will discuss some remaining issues regarding SBFD TX/RX/measurement procedures and SBFD random access operation.

Discussion

SBFD TX/RX/measurement procedures

SBFD TX/RX/measurement procedures

SBFD symbols are configured based on referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon assuming normal CP. As clarified in spec, referenceSubcarrierSpacing is smaller or equal to the SCS of any DL/UL BWP. For a BWP configured with normal CP, it is obvious that a symbol of the BWP would overlap with only one SBFD symbol or only one non-SBFD symbol of the referenceSubcarrierSpacing. However, for a BWP configured with extended CP, a symbol may overlap with SBFD symbols and non-SBFD symbols of referenceSubcarrierSpacing, as shown in Figure 1. Therefore, it is necessary to clarify how to determine the type of symbols at least for the BWP with extended CP.

Figure 1. Symbol types for BWP with extended CP

Proposal 1: For a BWP with extended CP, clarify how to determine the type (SBFD/non-SBFD) of symbols.

- Adopt TP#1 for TS 38.213.

Power control for PUSCH repetition type B

In the legacy, the UE determines transmission power per transmission occasion which is defined as below. For PUSCH repetition Type B, a PUSCH transmission occasion refers to a nominal repetition. That is, the UE would determine the transmission power per nominal repetition.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

In SBFD operation, if the UE is configured with separate power control parameters for SBFD symbols and non-SBFD symbols, when a nominal repetition spans both SBFD symbols and non-SBFD symbols, the applicable power control parameters become ambiguous. On the other hand, according to the agreement below, each actual repetition would be either in SBFD symbols or in non-SBFD symbols. To maintain consistent implementation of separate power control for different symbol types, the UE should determine transmission power individually for each actual repetition, instead of for each nominal repetition.

Agreement For PUSCH repetition type B, adopt Option 1. Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

Proposal 2: In SBFD operation, for PUSCH repetition Type B, the UE determines transmission power per actual repetition (instead of nominal repetition in the legacy).

- Adopt TP#2a or TP#2b for TS 38.213

Separate closed-loop power control

Agreement (RAN1#119) The maximum number of power control loops is the same as legacy (i.e. no more than 2) for SBFD operation in Rel-19. Agreement (RAN1#119) For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

According to the above agreements, separate power control loop for SBFD and non-SBFD symbols is supported.

However, in the current spec, it is not clear how to apply the TPC command in a DCI scheduling PUSCH in the case of separate power control loop for different symbol types.

In general, we think the TPC command should be applied to the loop associated with the symbol type where the UE would transmit PUSCH scheduled by the DCI. For example:

- For a DCI scheduling a single PUSCH transmission occasion, the TPC command should be applied to the loop associated with the symbol type of the PUSCH.

- For a DCI scheduling multiple PUSCH transmission occasions,

  - For Configuration 1, the TPC command should be applied to the loop associated with the valid symbol type.

  - For Configuration 2, if any PUSCH transmission occasion scheduled by the DCI is in SBFD/non-SBFD symbols, the TPC command should be applied to the loop associated with SBFD/non-SBFD symbols, respectively.

Proposal 3: Adopt TP#3 for TS 38.213 on TPC command application.

HARQ process ID

In RAN1#122bis meeting, HARQ process ID allocation for multi-PDSCH/PUSCH scheduling via a single DCI and multi-PUSCH configured grant. Companies agreed on the following principles but would like to simplify the TP.

To determine HARQ process ID for multi-PUSCH scheduled by a single DCI or multi-PUSCH configured grant,

- In Configuration 1,

  - If the valid symbol type is SBFD symbols, HARQ process ID is not allocated to a PUSCH overlapping with a non-SBFD symbol or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

  - If the valid symbol type is non-SBFD symbols, HARQ process ID is not allocated to a PUSCH overlapping with an SBFD symbol, or a non-SBFD symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

- In Configuration 2,

  - HARQ process ID is not allocated to a PUSCH overlapping with a non-SBFD symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, and is not allocated to a PUSCH across SBFD symbols and non-SBFD symbols.

To determine HARQ process ID for multi-PDSCH scheduled by a single DCI,

- In Configuration 1,

  - If the valid symbol type is SBFD symbols, HARQ process ID is not allocated to a PDSCH overlapping with a non-SBFD symbol.

  - If the valid symbol type is non-SBFD symbols, HARQ process ID is not allocated to a PDSCH overlapping with an SBFD symbol, or a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- In Configuration 2,

  - HARQ process ID is not allocated to a PDSCH overlapping with a non-SBFD symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and is not allocated to a PDSCH across SBFD symbols and non-SBFD symbols.

Proposal 4: Adopt TP#4~6 for TS 38.214 on HARQ process ID allocation.

Carrier aggregation

In RAN1#120 meeting, the following agreement was achieved for SBFD in CA.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

Regarding the FFS, in our understanding, ‘half-duplex CA case’ refers to directional collision handling between reference and other cell(s) for half-duplex operation in TDD CA. In the legacy, if the UE is configured with directional handling, it should determine a reference cell as below.

the UE determines a reference cell for a symbol as an active cell with the smallest cell index among - the configured multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - the cells of each band respectively if the UE is capable of simultaneous transmission and reception by simultaneousRxTxInterBandCA for the configured multiple serving cells, where the symbol is configured as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol.

With that understanding, if the SBFD cell is determined as the reference cell, how to determine the link direction of the symbol needs to be enhanced because of the new symbol types, i.e. SBFD symbols and non-SBFD symbols, of the reference cell. The spirit of the current rule should be followed.

Proposal 5: For half-duplex CA, if the SBFD cell is determined as the reference cell of a symbol, the symbol is configured as

- downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the symbol is not an SBFD symbol

- uplink, if the symbol is a flexible or SBFD symbol and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol

- downlink, if the symbol is a flexible or SBFD symbol and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol.

PUCCH repetition

In RAN1#122bis meeting, the following TP for PUCCH repetition was discussed.

### 9.2.6 PUCCH repetition procedure For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in clause 11.1, or flexible symbol, or an SBFD symbol as described in clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and - consecutive UL symbols, as described in clause 11.1, or flexible symbols, or consecutive SBFD symbols as described in clause 11.1, respectively, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols, and - if the UE is not provided sbfd-Config2-Transmission, and - if the first repetition is in SBFD symbols, the UE shall only consider slots containing consecutive SBFD symbols starting from the first symbol, with a length equal to or larger than the number of symbols indicated by nrofsymbols and do not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - if the first repetition is in non-SBFD symbols, the UE shall only consider slots containing consecutive non-SBFD symbols starting from the first symbol, with a length equal to or larger than the number of symbols indicated by nrofsymbols, and do not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

However, it has some editorial issues, resulting in potential misalignment with the legacy principle and the following agreement.

Agreement (RAN`#120bis) For PUCCH repetition, PUSCH repetition with available counting and TBoMS with Configuration 1, - In case the valid symbol type is SBFD symbol, a slot is determined as available if the symbols allocated for PUCCH/PUSCH in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - In case the valid symbol type is non-SBFD symbol, a slot is determined as available if the symbols allocated for PUCCH/PUSCH in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

Proposal 6: Adopt TP#7 for TS 38.213 on PUCCH repetition.

SBFD random access operation

RO type definition

In RAN1#122bis meeting, the following TP for RO type definition was discussed. The highlighted part is controversial.

# 8 Random access procedure Prior to initiation of the physical random access procedure, Layer 1 receives from higher layers a set of SS/PBCH block indexes and provides to higher layers a corresponding set of RSRP measurements. Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a Type-1 random access procedure, as described in clauses 8.1 through 8.4, or a Type-2 random access procedure as described in clauses 8.1 through 8.2A. Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions each including only symbols that are indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon and not configured by sbfd-RACHDualConfig, or - second PRACH occasions, that are - only within SBFD symbols and including at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon when the UE is provided sbfd-RACHSingleConfig, or - only within SBFD symbols or non-SBFD symbols configured by sbfd-RACHDualConfig, or - starting from an SBFD symbol and end in a non-SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes

In RAN1#118 meeting, the following agreement was reached. In our understanding, by a TP following the definition of additional-RO in this agreement, the ambiguity in whether a RO in uplink or flexible symbols and configured by sbfd-RACHDualConfig can be resolved without adding the highlighted part for the first ROs.

Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

Proposal 7: Adopt TP#8 for TS 38.213 on RO type definition.

RO validation

The TP for RO validation agreed in RAN1#122bis is intended to retain the exact same rules for the first (legacy) ROs. However, one condition was accidentally not removed.

Proposal 8: Adopt TP#9 for TS 38.213 on RO validation.

Msg3 repetition

Currently, the resource allocation in time domain for SBFD is captured in TS 38.214 as below. However, there are still some issues related to Msg3 repetition.

- Issue 1: Msg3 repetition with Configuration 1 should be requested only by PRACH transmissions with the second RO. When the first RO is used, the legacy Msg3 repetition should be applied. However, the condition of the RO type is missing.

- Issue 2: CBRA may be triggered in RRC connected mode, and Msg3 repetition with Configuration 1 may be requested by PRACH transmission with the second RO. However, with the if condition in yellow, when the UE is configured with sbfd-Config2-Transmission, the UE cannot apply Msg3 repetition with Configuration 1.

- Issue 3: PUSCH scheduled by fallbackRAR UL grant should not be involved. For 2-step RA, the first RO should be used, but it is not used to request Msg3 repetition with Configuration 1.

- Issue 4: In the red part, the case of Msg3 repetition with Configuration 1 is missing.

#### 6.1.2.1a Resource allocation in time domain for SBFD For a UE scheduled with PUSCH transmission occasions across SBFD symbols and non-SBFD symbols in different slots, - if the UE is not configured with sbfd-Config2-Transmission, - the UE transmits only the PUSCH in a valid symbol type; - For Type 1 PUSCH transmissions with a configured grant, the valid symbol type is provided by symbolType in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig. - For Type 2 PUSCH transmissions with a configured grant or PUSCH transmissions scheduled by DCI scrambled with SP-CSI-RNTI, the valid symbol type is the symbol type of the first PUSCH transmission occasion associated with activation DCI. For Type 2 PUSCH transmissions with a configured grant of PUSCH repetition type B, the valid symbol type is the symbol type of the first actual repetition associated with activation DCI. - For PUSCH transmissions scheduled by DCI format 0_1, 0_2, 0_3, 0_0 with CRC scrambled by TC-RNTI, RAR UL grant or fallbackRAR UL grant, the valid symbol type is the symbol type of the first PUSCH transmission occasion indicated by the scheduling DCI, the RAR UL grant or the fallbackRAR UL grant. For PUSCH repetition type B scheduled by DCI format 0_1 or 0_2, the valid symbol type is the symbol type of the first actual repetition occasion indicated by scheduling DCI. The UE does not expect that the first PUSCH transmission occasion indicated by scheduling DCI, the RAR UL grant or the fallbackRAR UL grant is mapped to both SBFD symbols and non-SBFD symbols, except for PUSCH repetition type B. - For PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1 or TB processing over multiple slots or PUSCH repetition type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, RAR UL grant or fallbackRAR UL grant, - a slot containing the transmission occasion that is not in the valid symbol type is not counted in the number of N∙K slots. - In case the valid symbol type is SBFD symbol, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - In case the valid symbol type is non-SBFD symbol, if the PUSCH repetition type A is scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, RAR UL grant or fallbackRAR UL grant, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon, if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. Otherwise, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - For PUSCH repetition type B, UE drops an actual repetition if the actual repetition is not in the valid symbol type. - otherwise, the UE transmits the PUSCH occasions in SBFD symbols and non-SBFD symbols after applying collision handling in clause 11.1 of [6, TS 38.213], if any. For PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1 or TB processing over multiple slots, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, or the symbols allocated for the transmission occasion in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For a UE configured with SBFD symbols and scheduled with a PUSCH transmission occasion that is mapped to SBFD symbols and non-SBFD symbols within a slot, - If the PUSCH transmission occasion is scheduled for PUSCH repetition type A with AvailableSlotCounting is enabled and K>1 or TB processing over multiple slots, the slot is not counted in the number of N∙K slots. - If the PUSCH transmission occasion is a nominal repetition for PUSCH repetition type B, the nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. If the UE is not configured with sbfd-Config2-Transmission, UE drops an actual repetition if the actual repetition is not in the valid symbol type. - Otherwise, the UE does not transmit the PUSCH transmission occasion.

Therefore, we have the following proposal to address the issues.

Proposal 9: Adopt TP#10 for TS 38.214 to correctly capture Msg3 repetition with Configuration 1.

Msg4 PUCCH

In RAN1#122bis meeting, the following text proposal was discussed in the FL summary.

### 7.2.1 UE behaviour <Unchanged parts omitted> - PO_PUCCH,b,f,c(qu) is a parameter composed of the sum of a component PO_NOMINAL,PUCCH, provided by p0-nominal or, if p0-nominal-sbfd is provided and, the PUCCH transmission is in SBFD symbols and associated with a PRACH in the second PRACH occasions, or the PUCCH transmission is in SBFD symbols in the RRC_CONNECTED state, by p0-nominal-sbfd, or PO_NOMINAL,PUCCH=0 dBm if p0-nominal is not provided, for carrier f of primary cell c and, if provided, a component PO_UE_PUCCH(qu) provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP b of carrier f of primary cell c, where 0≤qu<Qu. Qu is a size for a set of PO_UE_PUCCH values provided by maxNrofPUCCH-P0-PerSet. The set of PO_UE_PUCCH values is provided by p0-Set. If p0-Set is not provided to the UE, PO_UE_PUCCH(qu)=0, 0≤qu<Qu

The highlighted part was controversial, and we would like to share our views below.

In the CONNECTED mode, the UE can also perform CBRA. In this case, if PRACH is transmitted on the first PRACH occasions, the gNB may not know the UE capability of SBFD when scheduling Msg4 PUCCH, e.g. if the prior Msg3 does not convey C-RNTI of the UE. The issue of Msg4 PUCCH in the CONNECTED mode is similar to that in the initial access. Therefore, regardless of the RRC state, same principle for Msg4 PUCCH in the SBFD symbols should be followed, i.e., p0-nominal-sbfd is used only if the associated PRACH is transmitted in the second PRACH occasions.

For PUCCH other than Msg4 PUCCH, if it is in SBFD symbols, p0-nominal-sbfd is used.

With the above, TP#11 is suggested.

Proposal 10: Adopt TP#11 for TS 38.213 to correctly capture the case where p0-nominal-sbfd is used for PUCCH.

Conclusion

SBFD TX/RX/measurement procedures

According to the discussions in 2.1, we have the following proposals.

Proposal 1: For a BWP with extended CP, clarify how to determine the type (SBFD/non-SBFD) of symbols.

- Adopt TP#1 for TS 38.213.

Proposal 2: In SBFD operation, for PUSCH repetition Type B, the UE determines transmission power per actual repetition (instead of nominal repetition in the legacy).

- Adopt TP#2a or TP#2b for TS 38.213

Proposal 3: Adopt TP#3 for TS 38.213 on TPC command application.

Proposal 4: Adopt TP#4~6 for TS 38.214 on HARQ process ID allocation.

Proposal 5: For half-duplex CA, if the SBFD cell is determined as the reference cell of a symbol, the symbol is configured as

- downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the symbol is not an SBFD symbol

- uplink, if the symbol is a flexible or SBFD symbol and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol

- downlink, if the symbol is a flexible or SBFD symbol and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol.

Proposal 6: Adopt TP#7 for TS 38.213 on PUCCH repetition.

SBFD random access operation

According to the discussions in 2.2, we have the following proposals.

Proposal 7: Adopt TP#8 for TS 38.213 on RO type definition.

Proposal 8: Adopt TP#9 for TS 38.213 on RO validation.

Proposal 9: Adopt TP#10 for TS 38.214 to correctly capture Msg3 repetition with Configuration 1.

Proposal 10: Adopt TP#11 for TS 38.213 to correctly capture the case where p0-nominal-sbfd is used for PUCCH.

# Appendix I: Text Proposals

## TP#1 for TS 38.213

## 11.1 Slot configuration ## *** Unchanged parts are omitted *** SBFD symbols are consecutive, start from a first slot provided by SBFD-StartingSlotIndex and from a first symbol in the first slot provided by SBFD-StartingSymbolIndex, and end in a second slot provided by SBFD-EndingSlotIndex and in a second symbol in the second slot provided by SBFD-EndingSymbolIndex. SBFD symbols can be provided in any of pattern1 and, if provided, pattern2. A configuration period for SBFD symbols is P msec when only pattern1 is provided, or P+P2 when pattern2 is additionally provided. If a BWP in the serving cell is configured with μ=2, a type for a symbol with extended CP is determined from a type for a symbol of referenceSubcarrierSpacing with normal CP. A UE determines an extended CP symbol to be an SBFD symbol if the overlapping normal CP symbols are SBFD symbols. A UE determines an extended CP symbol to be a non-SBFD symbol if one of the overlapping normal CP symbols is a non-SBFD symbol. *** Unchanged parts are omitted ***

## TP#2a for TS 38.213

- 7 Uplink Power control *** Unchanged parts are omitted *** A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is an actual repetition if the UE is configured with SBFD symbols or a nominal repetition otherwise [6, TS 38.214]. *** Unchanged parts are omitted ***

## TP#2b for TS 38.213

- 7 Uplink Power control *** Unchanged parts are omitted *** A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is an actual repetition if p0AlphaSetforPUSCH-SBFD is provided or a nominal repetition otherwise [6, TS 38.214]. *** Unchanged parts are omitted ***

## TP#3 for TS 38.213

### 7.1.1 UE behaviour *** Unchanged parts are omitted *** - For the PUSCH power control adjustment state fb,f,c(i,l) for active UL BWP b of carrier f of serving cell c in PUSCH transmission occasion i - δPUSCH,b,f,c(i,l) is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in clause 11.3 - l∈0,1 if the UE is configured with twoPUSCH-PC-AdjustmentStates and l=0 if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in clause 8.3 - if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell - if the SRS resource set indicator value is 00, l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig - if the SRS resource set indicator value is 01, l is equal to the value of powerControlLoopToUse2 in ConfiguredGrantConfig - if the SRS resource set indicator value is 10 or 11, a first l and a second l respectively associated with the first and second SRS resource set are respectively equal to powerControlLoopToUse and powerControlLoopToUse2 in ConfiguredGrantConfig - else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and is provided p0-PUSCH-Alpha2, for a transmission of a configured grant Type 1 PUSCH and for active UL BWP b of carrier f of serving cell - a first l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig that is associated with the first srs-ResourceIndicator in rrc-ConfiguredUplinkGrant - a second l is equal to the value of powerControlLoopToUse2 in ConfiguredGrantConfig that is associated with the second srs-ResourceIndicator in rrc-ConfiguredUplinkGrant - else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format 0_0 and for active UL BWP b of carrier f of serving cell - l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig - else, for a PUSCH (re)transmission configured by ConfiguredGrantConfig, the value of l∈0,1 is provided to the UE by powerControlLoopToUse in ConfiguredGrantConfig. - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the first and second SRI fields and the l values provided by sri-PUSCH-ClosedLoopIndex, and determines the l values mapped to the values of the first and second SRI fields corresponding to each SRS resource set with usage set to 'codebook', respectively - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the first SRI field value and the l values provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook', and - the value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and the l value(s) provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the value corresponding to the second SRS resource set with usage set to 'nonCodebook' - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission does not include an SRI field - If the UE is provided twoPUSCH-PC-AdjustmentStates - the UE determines l=0 for the PUSCH transmission corresponding to the first SRS resource set with usage set to 'codebook' or 'nonCodebook', and l=1 for the PUSCH transmission corresponding to the second SRS resource set with usage set to 'codebook' or 'nonCodebook' - else - the UE determines l=0 for the PUSCH transmission - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided twoPUSCH-PC-AdjustmentStates - If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1 - If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value - If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value - If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUSCH transmission - If the UE is provided p0AlphaSetforPUSCH-SBFD - If the UE is not provided with sbfd-Config2-Transmission - If the DCI only schedules a single PUSCH transmission occasion in SBFD or non-SBFD symbols, or the valid symbol type for the PUSCH transmission occasions scheduled by the DCI is SBFD or non-SBFD symbols, the UE applies the TPC command value for l provided by p0AlphaSetforPUCCH-SBFD or p0AlphaSetforPUSCH, respectively. - If the UE is provided with sbfd-Config2-Transmission - If any PUSCH transmission occasion scheduled by the DCI is in SBFD or non-SBFD symbols, the UE applies the TPC command value for l provided by p0AlphaSetforPUCCH-SBFD or p0AlphaSetforPUSCH, respectively. - If the UE is provided SRI-PUSCH-PowerControl, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the l value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the l value that is mapped to the SRI field value - If the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE, l=0 - If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUSCH-RNTI, the l value is provided by the closed loop indicator field in DCI format 2_2 *** Unchanged parts are omitted ***

## TP#4 for TS 38.214

## 6.1 UE procedure for transmitting the physical uplink shared channel *** Unchanged parts are omitted *** When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided and, if the UE is configured with SBFD symbols, not indicated as an SBFD symbol by tdd-UL-DL-ConfigurationCommon, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if the symbols indicated by the indexed row of the used resource allocation table in the slot include both SBFD and non-SBFD symbols or if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided and, if the UE is configured with SBFD symbols, not indicated as an SBFD symbol by tdd-UL-DL-ConfigurationCommon, or a symbol of invalid symbol type if the UE is configured with SBFD symbols and not provided with sbfd-Config2-Transmission, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationUL-r16 or outOfOrderOperationUL-r18, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i on a scheduling cell,, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i of the scheduling cell. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1, 0_2 or 0_3 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. *** Unchanged parts are omitted ***

## TP#5 for TS 38.214

## 6.1 UE procedure for transmitting the physical uplink shared channel *** Unchanged parts are omitted *** When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource in an SRS resource set with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell. When nrofSlotsInCG-Period is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the first configured PUSCH grant and each subsequent valid configured PUSCH grant within a periodicity of the configuration is determined as in clause 5.4.1 of [10, TS 38.321], where a valid configured PUSCH grant is the one not colliding with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided and, if the UE is configured with SBFD symbols, not indicated as SBFD symbol(s) by tdd-UL-DL-ConfigurationCommon, and not colliding with a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213], and, if the UE is configured with SBFD symbols, not colliding with invalid symbol type if the UE is not provided with sbfd-Config2-Transmission or not across SBFD symbols and non-SBFD symbols if the UE is provided with sbfd-Config2-Transmission. *** Unchanged parts are omitted ***

## TP#6 for TS 38.214

## 5.1 UE procedure for receiving the physical downlink shared channel ## *** Unchanged parts are omitted *** A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1, or 4_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided, or with modulo operation of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if the symbols indicated by the indexed row of the used resource allocation table in the slot include both SBFD and non-SBFD symbols or if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of invalid symbol type if the UE is configured with SBFD symbols and not provided with sbfd-Config2-Transmission. When a UE is configured by the higher layer parameter repetitionScheme set to 'tdmSchemeA', the PDSCH includes two PDSCH transmission occasions. For each PDSCH, if either PDSCH occasion overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, the PDSCH is not received and HARQ process ID is not increment for the PDSCH. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH if the UE is not capable of receiving FDMed unicast and multicast PDSCH per slot per carrier. When HARQ feedback for the HARQ process ID is not disabled, or for the HARQ process associated with the first SPS PDSCH when HARQ-feedbackEnablingforSPSactive is provided and enabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i on a scheduling cell, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i of a scheduling cell,. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the PDCCH ending in symbol i is determined based on the PDCCH candidate that ends later in time. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration , with N=13 for =0, N=13 for =1, N=20 for =2, N=24 for =3, N=96 for m=5, and N=192 for m=6. ## *** Unchanged parts are omitted ***

## TP#7 for TS 38.213

### 9.2.6 PUCCH repetition procedure For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in clause 11.1, or flexible symbol, or an SBFD symbol as described in clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and - consecutive UL symbols, as described in clause 11.1, or flexible symbols, or consecutive SBFD symbols as described in clause 11.1, respectively, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols, and - if the UE is configured with SBFD symbols and not provided sbfd-Config2-Transmission, - if the first repetition is in SBFD symbols, the UE shall only consider slots containing consecutive SBFD symbols, that are not symbols of an SS/PBCH block with index provided by ssb-PositionsInBurst, starting from the first symbol, equal to or larger than the number of symbols indicated by nrofsymbols. - if the first repetition is in non-SBFD symbols, the UE shall only consider slots containing consecutive non-SBFD symbols, that are not DL symbols indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or symbols of an SS/PBCH block with index provided by ssb-PositionsInBurst, starting from the first symbol, equal to or larger than the number of symbols indicated by nrofsymbols,

## TP#8 for TS 38.213

# 8 Random access procedure ## *** Unchanged parts are omitted *** In the remaining of this clause, when a symbol is not stated as an SBFD symbol, the symbol is a non-SBFD symbol. Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions each including only symbols that are indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon and considered as uplink for the random access procedure, or - second PRACH occasions, that are in RBs that are both in the active UL BWP and in the UL sub-band, and associated either only - within SBFD symbols and that includinge at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon when the UE is provided either sbfd-RACHSingleConfig or - configured by sbfd-RACHDualConfig, or start from an SBFD symbol and end in a non-SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes The procedures in clause 8.1 apply separately for the first PRACH occasions and the second PRACH occasions. Only a Type-1 random access procedure is applicable for the second PRACH occasions. ## *** Unchanged parts are omitted ***

## TP#9 for TS 38.213

- 8 Random access procedure ## *** Unchanged parts are omitted *** - if a UE is provided tdd-UL-DL-ConfigurationCommon for a cell, a PRACH occasion for the cell in a PRACH slot is valid if - for a first PRACH occasion - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot, if it is only in UL symbols, and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or in SSB-MTC-AdditionalPCI corresponding to the cell, as described in clause 4.1 - for a second PRACH occasion - it is in RBs that are both in the active UL BWP and in the UL sub-band, and - it starts at least Ngap symbols after a last non-SBFD downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and does not overlap with a SS/PBCH block symbol, and - it is only within SBFD symbols, including at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon, if the UE is provided sbfd-RACHSingleConfig, or - it is only within SBFD symbols if the UE is provided sbfd-RACHDualConfig, or - it starts from an SBFD symbol and ends in a non-SBFD symbols if the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes ## *** Unchanged parts are omitted ***

## TP#10 for TS 38.214

#### 6.1.2.1a Resource allocation in time domain for SBFD For a UE scheduled with PUSCH transmission occasions across SBFD symbols and non-SBFD symbols in different slots, - if the UE is not configured with sbfd-Config2-Transmission or for PUSCH transmissions scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI or RAR UL grant and associated with a PRACH transmission in a second PRACH occasion, - the UE transmits only the PUSCH in a valid symbol type; - For Type 1 PUSCH transmissions with a configured grant, the valid symbol type is provided by symbolType in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig. - For Type 2 PUSCH transmissions with a configured grant or PUSCH transmissions scheduled by DCI scrambled with SP-CSI-RNTI, the valid symbol type is the symbol type of the first PUSCH transmission occasion associated with activation DCI. For Type 2 PUSCH transmissions with a configured grant of PUSCH repetition type B, the valid symbol type is the symbol type of the first actual repetition associated with activation DCI. - For PUSCH transmissions scheduled by DCI format 0_1, 0_2, or 0_3, or PUSCH transmissions scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, RAR UL grant and associated with a PRACH transmission in a second PRACH occasion or fallbackRAR UL grant, the valid symbol type is the symbol type of the first PUSCH transmission occasion indicated by the scheduling DCI or, the RAR UL grant or the fallbackRAR UL grant. For PUSCH repetition type B scheduled by DCI format 0_1 or 0_2, the valid symbol type is the symbol type of the first actual repetition occasion indicated by scheduling DCI. The UE does not expect that the first PUSCH transmission occasion indicated by scheduling DCI or, the RAR UL grant or the fallbackRAR UL grant is mapped to both SBFD symbols and non-SBFD symbols, except for PUSCH repetition type B. - For PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1 or TB processing over multiple slots or PUSCH repetition type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, RAR UL grant and associated with a PRACH transmission in a second PRACH occasion or fallbackRAR UL grant, - a slot containing the transmission occasion that is not in the valid symbol type is not counted in the number of N∙K slots. - In case the valid symbol type is SBFD symbol, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - In case the valid symbol type is non-SBFD symbol, if the PUSCH repetition type A is scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, or RAR UL grant and associated with a PRACH transmission in a second PRACH occasionor fallbackRAR UL grant, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon, if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. Otherwise, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - For PUSCH repetition type B, UE drops an actual repetition if the actual repetition is not in the valid symbol type. - otherwise, the UE transmits the PUSCH occasions in SBFD symbols and non-SBFD symbols after applying collision handling in clause 11.1 of [6, TS 38.213], if any. For PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1 or TB processing over multiple slots, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, or the symbols allocated for the transmission occasion in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For a UE configured with SBFD symbols and scheduled with a PUSCH transmission occasion that is mapped to SBFD symbols and non-SBFD symbols within a slot, - If the PUSCH transmission occasion is scheduled for PUSCH repetition type A with AvailableSlotCounting is enabled and K>1 or TB processing over multiple slots or the PUSCH transmission is scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI or RAR UL grant and associated with a PRACH transmission in a second PRACH occasion, the slot is not counted in the number of N∙K slots. - If the PUSCH transmission occasion is a nominal repetition for PUSCH repetition type B, the nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. If the UE is not configured with sbfd-Config2-Transmission, UE drops an actual repetition if the actual repetition is not in the valid symbol type. - Otherwise, the UE does not transmit the PUSCH transmission occasion.

## TP#11 for TS 38.213

### 7.2.1 UE behaviour <Unchanged parts omitted> - PO_PUCCH,b,f,c(qu) is a parameter composed of the sum of a component PO_NOMINAL,PUCCH, provided by p0-nominal or, if p0-nominal-sbfd is provided and, the PUCCH transmission is in SBFD symbols and associated with a PRACH in the second PRACH occasions, or the PUCCH transmission is in SBFD symbols and not associated with a PRACH, by p0-nominal-sbfd, or PO_NOMINAL,PUCCH=0 dBm if p0-nominal is not provided, for carrier f of primary cell c and, if provided, a component PO_UE_PUCCH(qu) provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP b of carrier f of primary cell c, where 0≤qu<Qu. Qu is a size for a set of PO_UE_PUCCH values provided by maxNrofPUCCH-P0-PerSet. The set of PO_UE_PUCCH values is provided by p0-Set. If p0-Set is not provided to the UE, PO_UE_PUCCH(qu)=0, 0≤qu<Qu