3GPP TSG RAN WG1 #123		R1-2508943

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.1.1.2

Source:	Google

Title:	Other aspects for AI/ML based CSI Compression

Document for:	Discussion/Decision

# Introduction

In RAN1 #122b, the following on other aspects for AI/ML based CSI compression was agreed.

Agreement: For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantization), consider the following definition for evaluation: - Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall 2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2. - Note: Companies to report whether real value and imaginary value are quantized, or amplitude value and phase value are quantized. - Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part. Uniform quantization is assumed. - Note: Companies to report whether/how the normalization is performed, and whether it is normalized within or across 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts. - Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values. - Note: It is up to company to use number of samples S>1 in their evaluations. In this case, companies need to provide information how S samples are quantized in their evaluation. Agreement: For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection. - L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits - Company to report the quantization step for reference amplitude for 6bits. - No restriction to the total number of non-zero coefficients for a specific rank. - Other combinations can be considered and reported by companies. Agreement: For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered: - Benchmark#1: FP32 (k1=k2=32 bits) - Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2) - CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer. - Tx port: 32, 64, or 128 up to companies. - Subband number: 13, or 19 up to companies. - Performance metric: - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option. - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution - Overhead per sample and per layer SGCS results are reported per layer - Note: Complexity should be analysed. - For Option 1, consider k1+k2=4, 6 ,7, 8, 16, 32 - Other assumptions follow R18 study EVM Agreement For CPU counting of UE side data collection, - OCPU=1 - FFS: The CPU occupancy duration

In this contribution, we provide some discussion on the other aspects for AI/ML based CSI compression, including the data collection for training, performance monitoring and model pairing related issues.

# Data collection

## Network side data collection

For network side data collection, the following was agreed in RAN1 #122.

Agreement: For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,: - Option 0: Reusing legacy e-Type II codebook - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI. - Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization. - Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix. - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) - Note: the quantizaton format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling. - Note: At least take precoding matrix as Target CSI type. FFS channel matrix. - FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline.

The data collection procedure should not require higher complexity in UE side. Both the computing complexity and the UE memory should be considered. The computing complexity for option 2 is much smaller than other options. The memory for option 3 could be smaller than other options. For Option 4, another aspect is to increase the value of O1 and O2, e.g., (O1, O2) = (8, 8).

Proposal 1: For network side data collection, with regard to UE’s memory constraints and computing complexity, option 2 and 3 should be prioritized.

- (O1, O2)=(8, 8) can be considered for Option 4 if supported

# Performance monitoring

The performance monitoring can be either in network side or UE side. Rel-19 supports UE-assisted performance monitoring for beam prediction and CSI prediction by reporting the RS-PAI and CSI-PAI. Different from CSI prediction, it is not easy for UE to get the outcome of the reconstructed CSIs in the network side. Therefore, if UE-assisted performance monitoring is to be supported, the precoded CSI-RS would be necessary for the UE to identify the precoded channel, and the UE can calculate the SGCS between the precoded channel from the precoded CSI-RS and the target CSI and the estimated channel for the CSI-RS for AI/ML based CSI report.

Proposal 2: If UE assisted performance monitoring is supported, support the UE to report the SGCS between the channel measured from a precoded CSI-RS and the target CSI and channel measured from a CSI-RS for AI/ML based CSI report.

- Reuse the R19 framework for the SGCS report

Another possible way is the network side monitoring. Thus, the network configures the UE to report a certain CSI which can be assumed as the target CSI. This would require higher overhead, but requires less complexity in UE side compared to the UE assisted performance monitoring. The possible options could be similar to the NW-side data collection. But for usually the performance monitoring happens more frequently than the data collection. Therefore, the overhead should be more important for the down-selection. With regard to the tradeoff between the accuracy and overhead, the Option 4 (eType2 with enhanced parameters) can be considered. Further, for NW-side performance monitoring, the reported CSI can also be used for CSI acquisition. Thus, the NW can also configure the UE to report CQI.

Proposal 3: If NW-side performance monitoring is supported, support the NW configures the UE to report the CSI based on eType2 codebook with enhanced parameters based on the eType2 CSI report framework

- Support the UE to report CQI additionally to facilitate CSI acquisition

# Model pairing

The model pairing can be based on a model ID. The network can configure a list of model IDs in a CSI report configuration, where different model IDs correspond to different payload sizes. The UE can report the CSI corresponding to a model ID that fits for the requirement of the maximum payload size. Then the CSI omission can be avoided.

Proposal 4: Support the network to configure a list of model IDs in a CSI report configuration

- Different models can be associated with different payload size

- UE reports CSI corresponding to a model ID that fits for the requirement of the maximum payload size to avoid CSI omission

# Conclusion

In this contribution, we provided discussion on the other aspects for AI/ML based CSI compression. Based on the discussion, the following proposals are provided.

Proposal 1: For network side data collection, with regard to UE’s memory constraints and computing complexity, option 2 and 3 should be prioritized.

- (O1, O2)=(8, 8) can be considered for Option 4 if supported

Proposal 2: If UE assisted performance monitoring is supported, support the UE to report the SGCS between the channel measured from a precoded CSI-RS and the target CSI and channel measured from a CSI-RS for AI/ML based CSI report.

- Reuse the R19 framework for the SGCS report

Proposal 3: If NW-side performance monitoring is supported, support the NW configures the UE to report the CSI based on eType2 codebook with enhanced parameters based on the eType2 CSI report framework

- Support the UE to report CQI additionally to facilitate CSI acquisition

Proposal 4: Support the network to configure a list of model IDs in a CSI report configuration

- Different models can be associated with different payload size

- UE reports CSI corresponding to a model ID that fits for the requirement of the maximum payload size to avoid CSI omission