3GPP TSG RAN WG1 #123	R1-2509019

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:		11.3.2

Source:				IMU

Title:					Discussion on 6GR Frame Structure

Document for:		Discussion and Decision

# Introduction

In RAN1#122-bis meeting, the following agreements were made:

Agreement For communication, 6GR considers NR frame structure used as a starting point for the study item, - Resource defined by one subcarrier and one symbol is called as resource element (RE). - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12 - Radio Frame length is 10ms - Each radio frame is split into 10 subframes, each with a duration of 1 ms - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design. - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings. Agreement 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. - FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1. - Note: ISAC is separately discussed in ISAC session. Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

The moderator suggested the following work plan for this meeting.

- Numerologies related

  - Remaining issues for SCS and CP types

  - Maximum bandwidth/FFT for sub-6GHz, around 7GHz, around 15GHz and FR2

    - Study on how to achieve 400MHz bandwidth around 7GHz

  - Note: minimum bandwidth is up to AI 11.1

  - Note: ISAC related is up to ISAC session

- Frame structure related

  - Frame format configurations/indications

  - Any further high-level consideration of slot concept, including scheduling unit related issues.

- Duplex related

  - Study of any duplex related issues, focuses on the duplex types that has been agreed to study.

- MRSS:

  - Rate matching and sharing related is to be discussed in AI11.1.

  - MRSS related to frame structure and numerology issues is discussed in AI 11.3.2

- Other aspects,

  - frequency resource adaptation is not treated unless related to frame structure, numerology, etc.

  - other related

This contribution builds on the agreements from RAN1#122bis, including the baseline 6GR frame structure and the agreement to study 30kHz and 60kHz SCS for the 7GHz band. We provide observations and proposals on the key open issues identified in the work plan for RAN1#123, with a specific focus on finalizing the 7GHz numerology and defining the configuration for advanced duplexing schemes. This contribution is organized as follows:

1	Introduction	1

2	Discussion and Proposals	2

2.1	On 7GHz Numerology: SCS, CP, and FFT Size	2

2.2	On Duplexing Configuration and Indication	3

3	Conclusion	3

# Discussion and Proposals

## On 7GHz Numerology: SCS, CP, and FFT Size

There is a fundamental trade-off in selecting the SCS for 7GHz, which the current simulation results do not fully capture. 30kHz SCS showed strong performance in the completed large delay spread (e.g., 300ns) simulations. However, these tests were limited to speeds up to 120 km/h. Further simulations are necessary to verify the expected performance degradation at higher speeds. Conversely, 60kHz SCS is inherently robust to high-speed scenarios but performed poorly with Normal CP in the 300ns large delay spread simulations. It is noted that longer CP types are already being considered for NTN and Sensing cases.

- The current simulation results are incomplete, as they confirm 30kHz SCS's strength in large delay spread but fail to test its known weakness at high speeds. The poor performance of 60kHz SCS with Normal CP may be resolved by testing with a longer CP, which is already under study for NTN/Sensing.

To find a balanced solution for 7GHz, RAN1 should study the performance of 60kHz SCS combined with a longer CP. This study is necessary to verify if this combination can provide robustness to both high-speed (Doppler) and large delay spread (ISI) scenarios. This study should also consider harmonization with longer CP structures already under discussion for NTN and Sensing. Furthermore, companies should conduct link-level simulations comparing the spectral efficiency of a single 400MHz carrier with 60kHz SCS + Longer CP against a 2x200MHz implementation with 30kHz SCS + Normal CP.

- To find a balanced solution for 7GHz, RAN1 should study the performance of 60kHz SCS combined with a longer CP. This study should include link-level simulations comparing the spectral efficiency of a single 400MHz carrier with 60kHz SCS + Longer CP against a 2x200MHz implementation with 30kHz SCS + Normal CP.

A critical and unresolved issue is the UE implementation complexity for 400MHz bandwidth. The baseline 30kHz SCS requires a 16k-point FFT to support 400MHz bandwidth. This is a four-fold increase from NR's 4k FFT, breaking the two-fold progression from LTE (2k) to NR (4k), and may impose an unacceptable implementation burden (e.g., ADC power, silicon cost, design risk) on the UE. Conversely, 60kHz SCS supports 400MHz bandwidth with an 8k-point FFT. This represents a more reasonable "doubling" of NR's FFT size and aligns with the 8k/16k FFT sizes under study.

- The baseline 30kHz SCS imposes a significant UE implementation burden by requiring a 16k-point FFT for 400MHz bandwidth. 60kHz SCS achieves the same bandwidth with a more practical 8k-point FFT, at least on the UE side.

We propose to prioritize 8k FFT as the maximum FFT size for 6GR UEs to ensure reasonable implementation complexity. Based on this, 60kHz SCS should be re-evaluated as the baseline for 7GHz, as it uniquely achieves both 400MHz CBW support and an 8k FFT. This provides a practical path for UE implementation while still meeting the 400MHz bandwidth target.

- Prioritize 8k FFT as the maximum FFT size for 6GR UEs. This practical limit implies that 60kHz SCS should be re-evaluated as the baseline for 7GHz.

## On Duplexing Configuration and Indication

There is a clear consensus from the RAN1#122bis meeting that the legacy 5G NR DCI-based slot-level SFI (Slot Format Indicator) for dynamic TDD is "deprioritized for study due to complexity". Simpler alternatives, such as a "Frame Pattern Indicator (FPI)", have been suggested as a path forward.

- The group has agreed to deprioritize 5G's complex DCI-based SFI for dynamic TDD. Simpler alternatives, like FPI, are now the focus.

There is strong consensus to study BS-side semi-static SBFD. The primary open issue is the method for configuration, with discussions diverging between Version 1 (defining "new additional symbol type(s)") and Version 2 (using "NR symbol/slot types as a starting point"). There is a noted preference from many companies for Version 2 to maintain NR alignment and reduce specification impact.

- While BS-side semi-static SBFD is agreed for study, the key open point is whether to create a new symbol type (Version 1) or reuse NR's symbol types (Version 2).

6GR should support BS-side semi-static SBFD by reusing the NR frame structure and symbol types (i.e., Version 2). A new, dedicated "SBFD symbol" is unnecessary for this use case and would add significant, avoidable complexity to specification. The SBFD resource pattern (e.g., UL/DL subband locations) should be configured via cell-specific RRC. The gNB can then utilize NR's flexible symbol concept, managing UL and DL transmissions via scheduling. This RRC + flexible symbol framework is simpler, maintains NR alignment, and is a sufficient and extensible baseline for studying future enhancements like dynamic SBFD.

- Support BS-side semi-static SBFD by reusing NR's flexible symbol concept (Version 2), with subband patterns configured by cell-specific RRC. This avoids a complex new symbol type and is a simpler, forward-compatible solution.

# Conclusion

In conclusion, the following proposals are provided for consideration in the study of the 6G Air Interface:

- CP-OFDM. To find a balanced solution for 7GHz, RAN1 should study the performance of 60kHz SCS combined with a longer CP. This study should include link-level simulations comparing the spectral efficiency of a single 400MHz carrier with 60kHz SCS + Longer CP against a 2x200MHz implementation with 30kHz SCS + Normal CP.

- Prioritize 8k FFT as the maximum FFT size for 6GR UEs. This practical limit implies that 60kHz SCS should be re-evaluated as the baseline for 7GHz.

- Support BS-side semi-static SBFD by reusing NR's flexible symbol concept (Version 2), with subband patterns configured by cell-specific RRC. This avoids a complex new symbol type and is a simpler, forward-compatible solution.