3GPP TSG RAN WG1 #123                                                   R1-2509054

Dallas, USA, Nov 17th – 21st, 2025

Source: Google

Title:	Discussion on Performance Evaluations for Rel-20 ISAC

Document for: Discussion and Decision

# Introduction

In RAN #108, the SID on Study on Integrated Sensing And Communication (ISAC) for NR was agreed [1]. The objective of the Rel-20 SI on ISAC for NR is to evaluate the performance of gNB-based mono-static sensing for the UAV use case. Several key metrics and baseline assumptions have been established. This contribution focuses on providing proposals to finalize the remaining key metrics and open issues.

# Evaluation Metrics

In RAN1#122, the following agreements have been reached. RAN1 has agreed on the sensing overhead calculation, standardizing two distinct methods. The methods differ based on whether resources utilized for both communication and sensing are included in the overhead accounting.  This assessment is important for quantifying the overhead introduced by sensing and for determining whether communication signals are efficiently reused for both sensing and communication functions.

Agreement

For the evaluation on NR ISAC,

- 3 kinds of resources are defined

- Type 1: resources that are used for sensing signal transmission

- Type 2: part of Type 1 resources that are used for communication purpose

  - Note: it is possible the resource type 2 doesn’t exist

- Type 3: resources that are not used for sensing signal transmission, and cannot be used for communication purpose due to sensing operation

- Both options should be reported for sensing resource ratio

- Option 1: (Type_1 + Type_3) resources over all radio DL and UL resources

- Option 2: (Type_1 - Type_2 + Type_3) resources over all radio DL and UL resources

- Note: if Type 2 resource doesn’t exist, two options are the same

For the evaluation on NR ISAC,

- Company should report T/F RE mapping of sensing RS, assumed TDD UL/DL configuration if applicable.

- Company should report which sensing RS resources are considered as Type 2 resource and related reason.

To determine the reusability of a sensing signal for communication, beam characteristics should be a primary consideration. Wide-beam signals are generally assumed to be reusable, as their broad coverage can simultaneously illuminate a sensing target (e.g., a UAV) and serve ground-based UEs or UEs in tall buildings.

Conversely, the reusability of narrow beams is highly dependent on the pointing direction. A beam pointed vertically at a UAV is unlikely to serve a ground-based UE or even UEs in tall buildings, whereas a beam aimed at a UAV near the horizon may have a signal footprint that can covers ground-based UEs and UEs in tall buildings, thus enabling reuse.

Furthermore, power configuration is a critical factor. The power level configured for accurate sensing of the UAV must also be evaluated to determine if it is sufficient to meet the coverage requirements for the communication link.

- Establish a formal framework for classifying a sensing signal reusability for communication

- Define a set of rules, based on beam geometry (width, direction) and power configuration, to determine whether a sensing signal can be simultaneously reused for communication.

# Measurements

In RAN1#122b, the following proposal was discussed offline for the type of measurements and the sensing metrics to be reported:

### Proposal:

- The following measurements are identified in the study of NR ISAC.

  - Level 1: raw data, which is the time-domain samples or subcarrier-domain samples of the channel response from each antenna port, or anything equivalent

  - Level 2: (partial raw data)

    - Amplitude and phase profile of delay, and/or Doppler, and/or angle

      - E.g., Amplitude and phase profile of delay-angle

  - Level 3 per path (subset of Level 2):

    - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp

      - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles

        - A path can correspond to one or multiple detected points

        - A path can correspond to one or multiple detected objects

  - Level 4 per point:

    - Option 1: Range, velocity, angle, and power per detected point for a given time stamp

      - Definition: A point is defined by one range/delay, one Doppler and one angle

    - Option 2: Position, velocity, and power per detected point for a given time stamp

      - Definition: A point is defined by one location and one velocity

  - Level 5: e.g., {position, velocity, no target ID}

    - Definition: Position and velocity for a detected object/target for a given time stamp

      - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target

      - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported

  - Level 6: e.g., {position, velocity, target ID}

    - Definition: Position and velocity for a detected object/target for a given time stamp

      - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target

      - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported

  - SINR, RSRP from perspective of sensing

  - Time stamp

- Study the following levels of measurements for potential down-selection within Rel-20 SI

  - Level 2

  - Level 3

  - Level 4

  - Level 5

  - Level 6

  - SINR, RSRP from perspective of sensing

We believe it is premature to down-select to a single measurement level during this Study Item phase. Such a decision requires a clear purpose and a comprehensive evaluation of the trade-offs for each level, which has not yet been completed. Forcing a selection now would preemptively constrain future specification work, which is contrary to the exploratory nature of a study. For the current evaluation activities, a specific reporting level is unnecessary as results can be generated via offline post-processing from a common dataset, allowing for a fair comparison without a binding decision.

If choice is going to be made, our preference is to support Level 6 ({position, velocity, target ID}) when the gNB possesses sufficient processing capacity, as this provides the most complete, actionable information.

However, we recognize that for certain use cases, such as those requiring centralized sensing fusion or advanced processing at a CN sensing function, lower levels like Level 3 (per path) or Level 4 (per point) are more appropriate.

Also, for some deployment, a gNB might lack the capability to compute final results and Level 3 may also maintain flexibility for the CN Sensing Function.

It is crucial to clarify that this selection methodology applies strictly to mono-static TRP scenarios, as we must avoid obligating the UE with heavy processing burdens in other configurations like TRP-to-UE.

- Do not down-select to a single measurement reporting level during the Study Item phase, as this is premature and would unnecessarily constrain future specification work.

- For the purpose of SI evaluations, a specific reporting level selection is not required, as results for different levels can be derived via offline post-processing from a common dataset.

- Clarify that the study on measurement levels and their selection applies to mono-static TRP scenarios to avoid implying new processing obligations on the UE in other configurations.

- If a selection of a measurement level is needed, we support Level 6.

Further clarification is required regarding the reporting granularity for the proposed measurement levels. It is currently ambiguous whether a given report (e.g., Level 4, 5, or 6) is intended to be generated per CPI or if it represents a result that has been filtered or averaged across multiple CPIs.

Moreover, for a base station controlling multiple TRPs, it must be clarified whether the sensing measurements are to be reported on a per-TRP basis or aggregated as a single report per gNB.

This ambiguity also extends to complex phenomena like micro-Doppler; for instance, it is unclear whether its detection should result in reporting multiple measurements within a single CPI or if a single, averaged measurement is expected.

- Clarify whether the defined measurement levels are reported per CPI or are filtered/averaged across multiple CPIs.

- Clarify whether a gNB with multiple TRPs reports sensing measurements on a per-TRP basis or as an aggregated report per gNB.

- Study the reporting mechanism for micro-Doppler, including whether it requires reporting multiple measurements within a single CPI or an averaged result.

# Association Method

In RAN1#122, the following agreement has been reached for the association of a detected object and the true target.

Agreement

For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions:

  - One true target is associated with at most one detected object

  - One detected object is associated with at most one true target

  - The same association applies to miss detection, false alarm probability Type 2 (if defined) and positioning/velocity accuracy

  - Companies to report the method used for association of the detected object(s) and the true target(s)

It was agreed that object association should be left to individual companies implementations, with each company reporting the specific method used for the association. The association threshold may be based on both position and velocity. Furthermore, the association algorithm can be more sophisticated, incorporating information such as micro-Doppler or target characteristics. We support the requirement that the association method used, and particularly the information utilized in the association, be reported alongside the submitted evaluation results.

- Association method used, and particularly the information utilized in the association (e.g., position, velocity,  micor-doppler, target characteritics, …) be reported alongside the submitted evaluation results.

# Conclusion

In this contribution, we have presented several proposals for the evaluation of the performance of gNB-based mono-static sensing for UAV use case:

- Establish a formal framework for classifying a sensing signal reusability for communication

- Define a set of rules, based on beam geometry (width, direction) and power configuration, to determine whether a sensing signal can be simultaneously reused for communication.

- Do not down-select to a single measurement reporting level during the Study Item phase, as this is premature and would unnecessarily constrain future specification work.

- For the purpose of SI evaluations, a specific reporting level selection is not required, as results for different levels can be derived via offline post-processing from a common dataset.

- Clarify that the study on measurement levels and their selection applies to mono-static TRP scenarios to avoid implying new processing obligations on the UE in other configurations.

- If a selection of a measurement level is needed, we support Level 6.

- Clarify whether the defined measurement levels are reported per CPI or are filtered/averaged across multiple CPIs.

- Clarify whether a gNB with multiple TRPs reports sensing measurements on a per-TRP basis or as an aggregated report per gNB.

- Study the reporting mechanism for micro-Doppler, including whether it requires reporting multiple measurements within a single CPI or an averaged result.

- Association method used, and particularly the information utilized in the association (e.g., position, velocity,  micor-doppler, target characteritics, …) be reported alongside the submitted evaluation results.

# References

- RP-242348, “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T

- R1-2505777	Work plan for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

- R1-2505778	TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

- R1-2508159 Session notes for 10.5 Study on Integrated Sensing And Communication (ISAC) for NR, Ad-Hoc Chair (Huawei), 3RAN1#122bis, Prague, Czech Republic, Oct 13th – 17th, 2025