3GPP TSG RAN WG1 #123	R1-2509119

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.3.1

Source: 	InterDigital, Inc.

Title:	Evaluations for Active Devices in Ambient IoT

Document for:	Discussion

# 1. Introduction

3GPP RAN has recently approved a new SI  to study a second phase of RAN aspects for outdoor scenarios of Ambient IoT [1]. In the previous meeting, agreements on evaluation assumptions were made [2, 3]. In addition, a link budget template was agreed for coverage analysis. The template can be found in Section 2 including the specific values used for the coverage analysis presented here.

In this contribution, R2D/D2R coverage analysis is provided. Link level simulation results can be found in [4].

# Link Budget

R2D Coverage:

Table 1 to Table 4 provide the link budget for UMa assuming penetration loss of 0 and 20 dB. We can see from the results that R2D coverage meets the 500m goal when penetration loss is not considered but coverage enhancement is needed when penetration loss is 20 dB.

Observation 1: R2D coverage is achievable at link distances of 500m without significant modification of the Rel-19 air interface if penetration loss is not considered.

Observation 2: R2D coverage enhancement is needed when penetration loss is considered.

Table 1 R2D coverage at 10-1 BLER (penetration margin 0 dB)

| Tx power | Device C | Device 2b |
| --- | --- | --- |
| 33 dBm | 1827m | 1624m |
| 43 dBm | 3294m | 2927m |

Table 2 R2D coverage at 10-1 BLER (penetration margin 20 dB)

| Tx power | Device C | Device 2b |
| --- | --- | --- |
| 33 dBm | 562m | 500m |
| 43 dBm | 1014m | 901m |

Table 3 R2D coverage at 10-2 BLER (penetration margin 0 dB)

| Tx power | Device C | Device 2b |
| --- | --- | --- |
| 33 dBm | 887m | 789m |
| 43 dBm | 1600m | 1422m |

Table 4 R2D coverage at 10-2 BLER (penetration margin 20 dB)

| Tx power | Device C | Device 2b |
| --- | --- | --- |
| 33 dBm | 273m | 243m |
| 43 dBm | 492m | 437m |

D2R coverage:

Table 5 provides the D2R coverage for Device C and Device 2b using the agreed CFO values of 10 ppm and 100 ppm, respectively. When penetration loss is 0 dB, Device C can achieve 10% BLER with MSK modulation, however for Device 2b the target BLER cannot be achieved even with 30 dB SNR due to high CFO. In this case the coverage of Device C is about 1300m while the 70m coverage for Device 2b corresponds to 30 dB SNR which cannot be achieved, so the D2R coverage for Device 2b is captured as less than 70m.

On the other hand, when penetration loss is assumed to be 20 dB, the coverage of Device C  falls to 400m and cannot meet the 500m target.

Table 5 D2R coverage at 10-1 BLER

| CFO | 5 dBm Tx power (Device C) | -10 dBm Tx power (Device 2b) |
| --- | --- | --- |
| 10 ppm | 1300m (penetration loss 0 dB) 400m (penetration loss 20 dB) | N/A |
| 100 ppm | N/A | <70m |

Observation 3: D2R coverage may be achievable by Device C at link distances of 500m with MSK modulation and 0 penetration loss. When penetration loss is 20 dB, coverage falls below 500m.

Observation 4: D2R coverage is not achievable by Device 2b with Rel-19 modulation and amble design at link distances of 500m.

Observation 5: D2R coverage is not achievable by Device C with Rel-19 modulation and amble design at link distances of 500m when penetration loss is 20 dB.

Proposal 1: R2D coverage should be enhanced for high penetration loss scenarios.

Proposal 2: D2R coverage should be enhanced for low and high penetration loss scenarios and should

should prioritize enhancements to mitigate the effect of CFO.

Link budget template:

Table 5 Link budget template

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | Deployment scenario 4 with topology 1 with no external carrier wave | Deployment scenario 4 with topology 1 with no external carrier wave |
| [0B] | Device 2b/C | Device 2b/C | Device 2b/C |
| [0C] | Center frequency (MHz) | 900MHz | 900MHz |
| [0D] | Pathloss model | UMa NLOS defined in TR38.901 | UMa NLOS defined in TR38.901 |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | 2 | 1 |
| [1E] | Total Tx Power (dBm) | For standalone: 43 dBm For in-band: 33 dBm | Device 2b: -10 dBm Device C: 5 dBm |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180 kHz | 180 kHz |
| [1G] | Tx antenna gain (dBi) | 17dBi | For A-IoT device, 0dBi |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0dB |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | 3dB | N/A |
| [1M] | EIRP (dBm) | Calculated | Calculated |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | 1 | 2 |
| [2B] | Bandwidth used for the evaluated channel (Hz) | 180 kHz | 180 kHz |
| [2C] | Receiver antenna gain (dBi) | 0 dBi | 17 dBi |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | 3 dB |
| [2D] | Receiver Noise Figure (dB) | Device 2b: 15 dB Device C: 13 dB | 5dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2E1] | Receiver interference density (dBm/Hz) | -999 dBm/Hz | -999 dBm/Hz |
| [2F] | Effective Noise Power (dBm) | Calculated | Calculated |
| [2G] | Required SNR/CNR | From LLS | From LLS |
| [2H] | Ambient IoT on-object antenna penalty | 0dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt2 | Budget-Alt2 |
| [2L] | Receiver Sensitivity (dBm) | Calculated | Calculated |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | 6 dB | 6 dB |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB | 0 dB |
| [3D] | Other gains (dB) (if any please specify) | N/A | N/A |
| [3E] | Penetration margin (dB) | 0, 20 dB | 0, 20 dB |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated | Calculated |
| [4B] | Distance (m) | Calculated | Calculated |
| （5）Other |  |  |  |
| [5A] | Other notes | N/A | N/A |

# Summary

In this contribution, a link budget analysis is provided to study the coverage for active devices in outdoor scenarios. The following are proposed and observed:

Observation 1: R2D coverage is achievable at link distances of 500m without significant modification of the Rel-19 air interface if penetration loss is not considered.

Observation 2: R2D coverage enhancement is needed when penetration loss is considered.

Observation 3: D2R coverage may be achievable by Device C at link distances of 500m with MSK modulation and 0 penetration loss. When penetration loss is 20 dB, coverage falls below 500m.

Observation 4: D2R coverage is not achievable by Device 2b with Rel-19 modulation and amble design at link distances of 500m.

Observation 5: D2R coverage is not achievable by Device C with Rel-19 modulation and amble design at link distances of 500m when penetration loss is 20 dB.

Proposal 1: R2D coverage should be enhanced for high penetration loss scenarios.

Proposal 2: D2R coverage should be enhanced for low and high penetration loss scenarios and should

should prioritize enhancements to mitigate the effect of CFO.

# References

- RP-251884, New SI: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, Huawei (moderator)

- Final Report of 3GPP TSG RAN WG1 #122

- Final Report of 3GPP TSG RAN WG1 #122bis

- R1-2507755, “Evaluations for Device 2b/C in Ambient IoT,” InterDigital, Inc.