3GPP TSG RAN WG1 #123	R1-2509145

Dallas, USA, Nov 17th-21st, 2025

Source:	MediaTek Inc.

Title: Channel coding for 6GR interface

Agenda item:		11.4.1

Document for:		Discussion and Decision

# Introduction

RAN1 122bis agreements: Working Assumption - Study 6G data channel coding for higher throughput than 5G with acceptable performance-complexity tradeoff for both NW side and UE side, - Target peak data rate is assumed to be 2 times of the target peak data rate defined in TR38.913 Note: The other target throughput is up to company to report. Note: Applicability of the potential channel code will be further discussed. Agreement - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G LDPC - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed - For 6G Polar code - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Necessity for control information beyond NR range is to be further discussed - Polar code maximum mother code length is kept as 1024. - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

In RAN1#122bis, it has been agreed that LDPC is used for 6G data channel coding and Polar code is used for 6G large payload (>11bits) control channel coding. This contribution provides an overview on the aspects to be improved and studied for 6G channel coding together with evaluation methodology.

# 6G data channel coding enhancement

## Motivation

Peak data rate is one of the KPIs to enable 6G applications, e.g., AR/XR, immersive communication. In IMT-2030 and company contributions to RAN1#122, peak data rate in the range of 30-200 Gbps are being actively discussed. In RAN PLAN #109, the minimum requirements for peak spectral efficiencies have been finalized to be 60bit/s/Hz and 30bit/s/Hz for downlink and uplink, respectively. Although there is no consensus on the maximum peak data rate yet, the new spectral efficiencies requirements are 2 times of 5G requirements, which suggests at least 2 times maximum peak data rate from 5G. With the new available spectrum, e.g., additional 400MHz in 6G, the expected maximum peak data rate is expected to be around 4 times of 5G peak data rate. In RAN1 #122bis, a design goal of doubling the 5G peak data rate has been adopted as a working assumption to align with the 6G peak spectral efficiency requirements, even though there is no consensus yet on the additional data rate enabled by the new 6G spectrum. The design goal can be revisited if further progress on the 6G peak data rate and meanwhile, companies can report the achievable data rates from their respective designs.

Given the working assumption and pending 6G peak data rate requirement, it is desirable to pursue designs capable of achieving 2-4 times peak data rate of 5G design, while minimizing the additional performance and complexity trade-offs required to move from a 2 to 4 times increase. Under such design criteria, simply scaling the existing 5G LDPC design by deploying additional parallel decoders is not ideal. While this approach may technically meet the peak data rate requirements, it is not the most efficient in terms of area utilization. Furthermore, implementing extra decoders solely to meet peak data rate requirements introduces inefficiencies, as these resources remain underutilized during lower-throughput operation.

Observation 1: Due to the 6G maximum SE decision made in RAN and newly introduced spectrum, 2x-4x peak data rate improvement from the 5G QC-LDPC design is anticipated in 6G.

Proposal: Study 6G QC-LDPC design to achieve 2x-4x peak data rate of 5G with acceptable performance-complexity tradeoff for both NW side and UE side provided that additional performance and complexity trade-offs required from 2 to 4 times peak data rate increase is minimized

To address the 6G throughput challenge more efficiently, it is useful to analyze the key factors influencing QC-LDPC decoder throughput. The data rate of QC-LDPC decoder for a given operating frequency rate f, BG associated with given code rate, and lifting size Z can be approximated as

Tput≈NI∙(Kb×Z)∙fI×C

,

where NI is the number of decoders, Kb is the number of information column in the BG, I is the number of iterations deployed at decoding algorithm, and C is the number of decoding cycle per iteration. There are several design factors of QC-LDPC which contribute to Tput, i.e., Kb, Z, I, C, and the impact from each factor is discussed in the following.

Based on the decoder data rate estimation formula, the increasement of Kb with fixed Z seems to benefit the data rate outcome if other terms are fixed. However, larger Kb generally increases C accordingly given the optimum degree distribution within the BG design. Therefore, the associated data rate improvement from increased Kb and potentially increased C should be carefully studied. Furthermore, additional implementation cost associated with larger Kb or Kb×Z (CB size) needs to be considered, which is elaborated more in section 1.3.

Another direction to improve data rate is to reduce I and/or C. The iteration I is a random variable which depends on various factors such as channel realization, code rate, CB size, etc. Due to the randomness and the dependency across several factors, contribution of I to data rate might not be straightforward to be quantified and the impact might not be predictable. To demonstrate the impact of iteration, BLER performance examples of different I for BG1 are presented in Figure 1. As shown, 0.2dB degradation is noticed to achieve same BLER from I=20 to I=10 at high code rate and 0.4dB degradation is noticed from I=20 to I=10 at R=648.51024. The performance degradation becomes more pronounced with further iteration reduction, e.g., from I=20 to I=5, where 0.6 dB loss is shown at R=9481024 and 1.2dB loss is shown at R=648.51024. These results indicate that the impact of reducing I on BLER performance varies in different scenarios and the performance degradation can be dB level for small I, especially at low code rates. Therefore, the trade-off between BLER performance and throughput gain based on reducing I should be carefully evaluated.

Observation 2: dB level BLER performance degradation is observed when reducing decoding iteration I from 20 to 5 for 4X peak data rate gain.

Figure 1 BG1 BLER performance under different layer BP iterations

Increasing Z and/or reducing C from current 5G design can be another data rate enhancement directions. Increasing Z, the lifting size, provides the feasibility of processing information exchange between more variable and check nodes simultaneously, which has direct impact on the peak data rate, i.e., double the max lifting size Zmax from 5G design can directly double the peak data rate, regardless of BG design in QC-LDPC structure. The side-effect of large Z is the implementation cost and how to achieve the trade-off between data rate enhancements and implementation cost from larger lifting size should be further studied. The factor C provides another dimension of data rate enhancement and it solely depends on BG design. Unlike the implementation cost from the data rate enhancement of large lifting size, reducing C generally provides implementation benefits given a certain type of decoder implementation. In addition to the peak Tput enhancements, reducing C also provides benefit to daily use scenario in terms of decoding latency. On the other hand, reduction on C, e.g., less edge in BG, can lead to potential BLER performance loss. The trade-off between BLER performance and throughput gain based on reducing C should be further evaluated.

Observation 3: Increasing maximum lifting size Z and/or reducing decoding cycles per decoding iteration C  from current 5G design provides potential Tput enhancements.

Observation 4: Reducing decoding cycles per decoding iteration C from current 5G design provides potential decoding latency benefit.

As for the encoding latency, 5G design already addresses this aspect by incorporating efficient structure, such as an accumulator (dual diagonal structure) in the high code part of parity check matrix and a Raptor code structure in the low code rate extension part of parity check matrix. Such encoding latency beneficial structure should be preserved in 6G design.

Observation 5: Dual diagonal structure and Raptor code structures introduced in 5G QC-LDPC code can be leveraged in 6G QC-LDPC code design to achieve encoding latency reduction.

## Evaluation methodology

### Iteration and decoding cycle per iteration evaluation

The total decoding cycle amount to decode a code block (CB) can be expressed as I×C, where I is the decoding iteration number and C is the number of decoding cycles per iteration. As discussed in section 1.1, the value of I is influenced by several factors including channel realization, code rate, CB size, etc. To demonstrate that, for fixed code rate and CB size, Figure 1 shows the required I to meet different target BLER depends on SNR. For example, to achieve BLER=0.01, it requires I=20 at around SNR=6.8 dB whereas I=10 is required at around SNR=7dB. To facilitate the analysis of I, following definition can be considered:

I: number of iterations to achieve target BLER at target SNR

The selection of target BLER and target SNR is critical and it should be carefully discussed. For target BLER, at least 10-2 should be considered for CB BLER target to ensure TB reliability. For the target SNR, acceptable performance/throughput trade-off from 5G design should be considered, e.g., 0.5dB SNR degradation compared to the reference SNR associated with 5G BG1 under layer BP with I=20 can be acceptable. Example of target SNR and reference SNR is shown in Figure 2. Furthermore, due to the early termination check embedded in each LDPC decoding iteration, the actual number of iterations executed to decoder a CB may be less than the maximum iteration. Therefore, the average of I can be considered in order to accurately reflect the required decoding iteration number.

Figure 2 Illustration of reference SNR and target SNR

As for the estimation of C, it depends on the decoding algorithm. For a typical layer decoding algorithm, C can be estimated by the number of edges in LDPC parity check matrix divided by number of edges which can be processed simultaneously. For QC-LDPC, C can be simplified to the number of edges in BG divided by number of edges available to be processed simultaneously.

Proposal: Consider I×C as the estimation of total decoding cycles per CB

- C: decoding cycle per iteration

  - C can be approximated by e/M, where e is the number of edge in a BG and M is the number of edges available to be processed simultaneously, if any

- I: [Average number or max number] of iterations to achieve target BLER at target SNR

  - Target BLER: [0.01]

  - Target SNR=Reference SNR+[<0.5]dB

  - Reference SNR: The SNR where BG1 achieves target BLER under Layer BP with 20 iterations

### Peak data rate evaluation

From the perspective of a channel coding decoder, the peak data rate can be determined by the maximum TB size of a TB which can be decoded within a slot duration T. Since TB size is a function of RE numbers within T multiplied by the highest code rate, we first evaluate the maximum RE number a decoder can process within T as follows.

To begin, consider a decoder operating at a given maximum operating frequency fmax. The maximum number of CBs of MCS=X in a TB which can be decoded within T while achieving target BLER is

NCB,MCS=X,T=fmax×TImcs=X×Cmcs=X,

where Imcs=X is the number of iterations to decode an MCS=X CB to achieve target BLER at target SNR and Cmcs=X is the number of decoding cycles per iteration to decoder an MCS=X CB. From NCB,MCS=X,T, it can be derived that the maximum number of REs, REMAX,MCS=X,T, the decoder can support such that the decoder can finish decoding of all the MCS=X CB within T and meet the target BLER is given by

REMAX,MCS=X,T=NCB,MCS=X,T×CBsizeRmcs=X×1QAMMCS=X=fmax×TImcs=X×Cmcs=X×CBsizeRmcs=X×1QAMMCS=X ,

where Rmcs=X is the code rate associated with MCS=X and QAMMCS=X is the modulation order associated with MCS=X. It’s important to point out that REMAX,MCS=X is MCS dependent, which means the maximum number of REs the decoder can support such that all the MCS=X CB within T can be decoded to achieve the target BLER vary for different MCS values. This variation comes from the different decoding cycles per iterations, code rates, modulation orders, and potential different number of iterations to achieve target BLER for different MCSs. Therefore, to ensure the decoder can finish decoding of all the CBs within T with robust performance across all possible MCS, the maximum RE number within T the decoder can support has to be the minimum REMAX,MCS=X,T across all possible MCS, which can be expressed as

REMAX,T=minMCS=XCBsizefmaxTImcs=X×Cmcs=X×1RMCS=X×1QAMMCS=X.

This ensures that the decoder’s capability is not overestimated for any MCS, thereby guaranteeing consistent performance. Consequently, the peak data rate under MCS=27 given REMAX,T is

REMAX,T×QAMMCS=27×RMCS=27=minMCS=XCBsizefmaxImcs=X,BG1×Cmcs=X,BG1×RMCS=27RMCS=X×QAMMCS=27QAMMCS=X

This formula provides a direct link between the decoder’s computational capacity and the achievable peak data rate, taking into account the constraints imposed by decoding complexity and error performance requirements under different MCS configurations.

To illustrate the significance of determining REMAX,T with the consideration of all the possible MCS=X, Figure 3 presents a simple example. In this scenario, the maximum number of CBs of MCS=27 in a TB increases by a factor of 4 when Imcs=27 decreases from 20 to 5, which implies that REMAX,MCS=27,T and the peak data rate improve by 4X assuming each MCS=27 CB requires only 5 iteration instead of 20 iterations to meet the performance requirement. Moreover, in Figure 3, we also demonstrate that if the same REMAX,MCS=27,T resources are scheduled with MCS=20, then the number of MCS=20 CB can be estimated by NCB,MCS=27×Rmcs20Rmcs27 and each of the CB will be allocated decoding iteration number of NCB,MCS=27×(Imcs=27=5)×Cmcs=27NCB,MCS=27×Rmcs20Rmcs27×Cmcs=20 to comply with the constraint of decoder’s computation capacity. For BG1, Cmcs=27 and Cmcs=20 can be estimated by the corresponding edge number in the BG, which is 76 and 150, respectively. Consequently, the number of iteration of each MCS=20 CB is  NCB,MCS=27×(Imcs=27=5)×Cmcs=27NCB,MCS=27×Rmcs20Rmcs27×Cmcs=20 ≈3.5. However, as shown in Figure 1, when the iteration number decrease to less than 5, the BLER degradation by more than 1dB compared with 20 iterations BLER performance. Such dB level degradation may not satisfy the required BLER target, which highlights the necessity of estimating peak data rate with the consideration performance of other MCS configurations.

Proposal: To ensure BLER performance is acceptable for all MCS, consider the following metric to facilitate peak data rate evaluation for a decoder operating at maximum frequency fmax

- minMCS=XCBsizefmaxImcs=X×Cmcs=X×RMCS=27RMCS=X×QAMMCS=27QAMMCS=X

  - Imcs=X is the minimum iterations to satisfy BLER performance requirement

Figure 3: Illustration of MCS=27 iteration reduction impact on peak data rate and the associated iteration reduction on MCS=20

As highlighted in the peak data rate evaluation formular, the MCS with the largest Imcs=X×Cmcs=X×RMCS=X×QAMMCS=X determines the peak data rate bottleneck. For BG1 design, the value of Cmcs=X×RMCS=X×QAMMCS=X across different MCS is provided in Figure 4 assuming the Imcs=X is the same for different MCS. As demonstrated in Figure 4, the bottleneck case to determine peak data rate of BG1 is MCS=20.

Figure 4 Values of Cmcs=X×RMCS=X×QAMMCS=X across MCS

Observation 6: Bottleneck case to determine peak data rate of BG1 is MCS=20 based on peak data rate estimation minMCS=XCBsizefmaxImcs=X×Cmcs=X×RMCS=27RMCS=X×QAMMCS=27QAMMCS=X.

## Enhancement directions

As discussed in section 1.1, 6G data channel coding study should investigate designs to achieve 2 times peak data rate improvement goal stated in the working assumption while providing flexibility to accommodate pending 6G peak data rate requirement. Therefore, it is desirable to pursue designs capable of achieving 2-4 times peak data rate of 5G design with minimized additional performance and complexity trade-offs while scaling from 2 to 4 times increased in peak data rate. In the following, approaches to achieve 4X peak data rate and the associated performance/complexity are discussed.

To achieve 4X peak data rate target, discussion in section 1.1 suggests that reducing decoding iteration I by 1/4 in BG1 may cause performance degradation, e.g., reducing I from 20 to 5 shows >0.5dB loss for BG1 at high code rates and >1dB loss at low code rates as shown in Figure 1. Additionally, as illustrated in Figure 3, reducing I from 20 to 5 at highest code rate limits the number of iterations available for decoding a low code rate CB to fewer than 5 due to a greater number of decoding cycles per iteration is required, which further aggravates the performance degradation at low code rates. With a careful design targeting fast convergence, it is possible to reduce the performance loss at high code rates to <0.5dB at 5 iteration but still challenging to have acceptable performance loss at low code rates at <=5 iterations compared with 20 iterations. Therefore, to further enhance peak data rate of 6G QC-LDPC with acceptable performance compromise, it is essential to investigate approaches to reduce decoding cycle per iteration C compared to 5G QC-LDPC design in addition to the reduction of I, especially at low code rates. Such approaches, as highlighted by the proposed metric, are crucial for achieving the desired peak data rate improvements while maintaining robust error correction performance.

Recall that the decoding cycle per LDPC decoding iteration can be estimated by C=e/M, where e is the number of edges in the associated submatrix of BG at given MCS and M is the parallelism factor (number of edges can be processed simultaneously) exiting in the BG design. Therefore, to reduce C, a new BG with peak data rate enhancement can aim to introduce fewer edges than 5G design. The feature of less edges within a BG not only provides the benefit of throughput, but the associated implementation complexity can also be reduced, which will be discussed with more details later in this section. One possible approach is to reduce edges from 5G BG1 design. However, given the optimized degree distribution in BG1, further edge reduction to support 2-4 times data rate enhancement can cause significant performance loss. Alternatively, a new compact BG with smaller size than BG1 is proposed in [8]. The challenge of introducing a new compact BG lies in finding an optimized degree distribution to achieve comparable BLER performance to BG1 at high code rates since optimized degree distribution for high code rates typically requires large row weights. Additionally, to match the same CB size with a compact BG, the lifting size must be increased which translates to increased implementation complexity. In particular, the barrel shifter used to implement circular shift function has complexity ~ZlogZ that grows nonlinearly with lifting size Z. For example, for a new compact BG with half size of BG1 information columns as Kb=11, the corresponding maximum lifting size needs to be doubled from 384 to 768 to achieve the same maximum CB size as NR BG1, which introduces >2 scaling of barrel shifter implementation complexity. Therefore, the trade-off between Tput, implementation, and performance of a compact BG should be further studied.

Another direction to improve C can be introducing parallelism structure within the BG such that the decoder can exploit the parallelism and boost the throughput. One possible parallelism structure is to establish orthogonality between pairs of rows of a BG. One example of BG with pair-wised orthogonality is illustrated in Figure 5 with shift coefficients specified, where ‘-1’ represent zero in the BG and a Z×Z all zero matrix in the parity check matrix. Similarly, nonzero elements represent ones in the BG and a Z×Z circular matrix in the parity check matrix with shift coefficient value specified by the element. The 2i-th row and 2i+1-th row in the BG example are orthogonal, i.e., no common “1”s between the two rows in any column. This orthogonality provides the feasibility of enabling processing two blocks of VNs in two blocks of CN nodes associated with the 2i-th row and 2i+1-th row of the BG concurrently. Consequently, the C is reduced by half (due to the parallelism factor M=2) comparing with legacy layer decoder which can only process CN nodes one by one sequentially. In fact, by processing a 2Z×2Z blocks in the BG example, the BG example in Figure 5 can be interpreted as a “smaller” BG with a “lifting size” of 2Z. From the barrel shifter implementation point of view, the pair-wised orthogonality in the BG example allows a simplified barrel shifter implementation of complexity 2×ZlogZ, compared with the barrel shifter associated with lifting size 2Z which has complexity ~2Zlog(2Z). On the other hand, due to the equivalence to a compact BG, the same challenge of introducing a new compact BG with an optimized degree distribution to achieve comparable BLER performance to BG1 at high code rates remains.

Figure 5 Example of pair-wise orthogonal structure inside a BG with Kb=22

From the above analysis, it is challenging to apply a fixed BG to maintain the decent BLER performance with optimized degree distribution at high code rate while also having a small number of edges at low code rate BG to enhance overall throughput across code rates. One possible solution is to consider QC-LDPC designs having adaptive BG sizes according to code rates. One example is shown in Figure 6 where the entire BG (Kb=44) is used when code rate is >5/6 and a submatrix associated with first 22 information blocks (Kb=22) is used for other low code rates. By exploring the orthogonality structure and 2Z×2Z blocks processing, the full BG (Kb=44) can be interpreted as an underlying BG with Kb=22 and lifting size=2Z, which is sufficient to achieve optimized degree distribution at high code rates. Similarly, the submatrix (Kb=22) used for low code rates can be interpreted as an underlying BG with Kb=11, which provides reduced number of edges at low code rates. Such relatively balanced number of edges associated with different code rates allows the number of available iterations for high and low code rates to be more evenly distributed such that sufficient iterations are available for decoding low code rate CBs. For example, reducing I from 20 to 5 at highest code rate translates to the number of iterations available for decoding a code rate=23 CB to be more than 5 in this example due to the reduced number of edges at low code rate, which alleviates the performance degradation concern of BG1 as illustrated in Figure 3. Consequently, such design is capable of achieving 2-4 times peak data rate of 5G design with minimized additional performance and complexity trade-offs when scaling from 2 to 4 times increase in peak data rate.

Figure 6 Example adaptive BG with Kb=44 (CR>5/6) and Kb=22 (CR<=5/6)

The BLER performance comparison of the BG example in Figure 6 and BG1 is shown in Figure 1. It can be observed that the BG example provides better BLER performance than BG1 across all iterations at code rates except a minor degradation (~0.1dB) at code rare 2/3 and 5/6 at high iterations is observed. Note that a compact BG in Figure 6 with lesser edges than BG1 are utilized for those code rates, which implies a much better throughput is achieved as the trade-off from ~0.1dB BLER degradation.

In addition to the decoding latency and Tput benefits, the encoder also can enjoy the orthogonality structure to reduce encoding latency by allowing parallel computation of parity check bits, further reducing encoding latency. The 5G BG1 design already includes the pair-wised orthogonality in the extension part of BG, i.e., the additional rows within the submatrix associated with lower code rate. However, the submatrix of BG1 associated with high code rate remain very dense and extending the pair-wised orthogonality to high code rate part in 6G QC-LDPC BG design should greatly improve the Tput and latency.

Independent to BG design, one straight forward way to achieve data rate enhancement is to increase maximum CB size limit from 5G, which is 384×22=8448 information bits in 5G BG1. Since the maximum CB size can be represented by Kb×Zmax, increasing Kb or Zmax can achieve the similar impact on improving maximum CB size. However, increasing Kb typically introduces more edges into the BG, which can negatively impact the C and anticipated Tput gain. On the other hand, enlarging Zmax improves the decoder implementation parallelism and Tput accordingly. The trade-off from larger lifting size is the increased implementation complexity, which is discussed in the following using simplified metrics.

For a commonly used block-parallel min-sum decoder for QC-LDPC, two types of implementation complexities are considered, memory area and logic area. The memory area can be roughly categorized into 4 blocks: LLR memory, R memory, Q memory, and sign memory. The LLR memory is used to store the input LLRs and the intermediate LLRs during the decoding and it can be estimated by c×(8+8)×Zmax, where c is the number of columns (VNs) in the BG and 8 bits are assumed to represent each of LLRs. R memory is used to store the minimum and second minimum of the absolute value of input LLR of a check node and the corresponding indices of minimal LLRs. The estimated R memory is r ×(8+8+ceil(log2(c))) ×Zmax, where r is the number of rows in the BG. The Q memory is used to store the extrinsic LLR from a VN to a CN and it can be estimated by w×(8)×Zmax, where w is the maximum row weight in the BG. The sign memory is used to store the sign of extrinsic LLR of each edge from a CN to a VN and it can be estimated by e×(1)×Zmax, where e is the number of edges(1s) in the BG. For logic area, the estimation is more complicated, but it can be generally assumed to be proportional to Zmax. From the above area estimation, it can be concluded that although increasing maximum lifting size Zmax directly improves the Tput, both memory area and logic area of a decoder also increase proportionally. Therefore, careful study on the trade-off between Tput and memory area cost is necessary while considering increasing maximum lifting size.

Proposal: To improve 2-4x peak data rate from 5G, study following design aspects of QC-LDPC with acceptable error correction performance and implementation complexity trade-off

- Reduced number of iterations

- Reduced BG edges

- Parallelism structure

- Maximum lifting size>384

- BG size adaptation for different code rates

- Combination of above aspects

# 6G Polar enhancement

## Motivation

While Polar codes have been adopted in 5G NR for Downlink Control Information (DCI) transmission due to their strong error correction performance at short block lengths, several limitations and issues have been identified in practical deployments, both from the base station and UE perspectives.

From the base station perspective, the 5G Polar code design for DCI exhibits significant limitations in terms of resource efficiency. A primary drawback is the restricted payload size, as the code construction supports only a narrow range of payload lengths up to 140 bits. Due to potential wilder 6G carrier bandwidth than in 5G, the number of bits required for the frequency domain resource assignment field (FDRA) in DCI is expected to be increased. Table 1 provides the estimated 6G DCI payload sizes for different bandwidths. Compared with 100 MHz bandwidth for 5G, maintaining similar or better scheduling flexibility in 6G requires an additional 17 bits for FDRA for a 200 MHz bandwidth when using RA type 0 with RBG = 16. Moreover, at RAN1 #122bis, it was agreed that the maximum channel bandwidth (CBW) at the network side is assumed to be 400 MHz around 7 GHz. Considering implementation cost and maximum attainable SNR, supporting contiguous 400 MHz bandwidth at UE can be deployed as 2 × 200 MHz. One potential solution is a multi-carrier cell that aggregates two 200 MHz carriers into a single cell to achieve better scheduling efficiency than legacy CA. When 400 MHz is considered, FDRA bit number increases further: compared with 5G, an additional 51 bits may be required for a PXSCH scheduling, which can result in a DCI payload size > 140 bits. Required bits increase further when scheduling PxSCH for multiple cells in a single DCI is supported. This 140 bits payload size constraint results in suboptimal utilization of control channel resources and limits scheduling flexibility, making it challenging to efficiently accommodate diverse DCI payload requirements in potential 6G application. Furthermore, due to the false alarm requirements, each DCI is appended with 24 bits CRC regardless of the payload size, which introduce another layer of resource inefficiency if multi-carrier scheduling is implemented through several DCIs. In addition, the current 5G Polar code implementation for DCI is confined to QPSK modulation. This limitation further hinders resource efficiency, especially in high-SINR scenarios where higher-order modulation could significantly enhance control channel spectral utilization. Such drawbacks in the 5G Polar code design are fundamentally misaligned with the anticipated demands of 6G applications, which will require greater flexibility, scalability, and efficiency to support a broader range of use cases. Addressing these issues is therefore essential for the evolution of Polar code design in 6G.

| Scenario | DCI payload size estimation for a PXSCH scheduling | Notes |
| --- | --- | --- |
| Scenario #1: Bandwidth 100 MHz | ~ 100 bits | DCI format 0_1/1_1 payload size based on Rel-15 NR, assuming a flexible configurations |
| Scenario #2: Bandwidth 200 MHz | ~ 100 bits + 17 bits | Additional 17 bits are required for FDRA when scheduling a 200 MHz BW using RA type 0 with RBG=16 |
| Scenario #3: Bandwidth 2x200 MHz | ~ 100 bits + 51 bits | Assume an MC cell that aggregates two 200 MHz carriers into a single cell and uses a single FDRA field to schedule 1100 RBs Additional 51 bits are required for FDRA when scheduling a 400 MHz BW using RA type 0 with RBG=16 |

Table 1 Estimated 6G DCI payload size for different BWs (single PXSCH scheduling is assumed)

Observation 7: Due to the limitation of maximum 140bit payload sizes and the support of only QPSK QAM, the 5G Polar code design for DCI is limited on scheduling flexibility and resource efficiency to support 6G applications.

Observation 8: 6G’s wider bandwidth substantially increases DCI FDRA bit requirements and current 140bits limit is not sufficient.

From UE perspective, the 5G Polar code for DCI is not optimized with respect to decoder complexity and early termination. The decoding process, especially for blind decoding of DCI, can be computationally intensive, leading to increased power consumption and latency at the UE. To address the blind decoding complexity issue, the 5G polar code introduces distributed CRC bits to enable early termination functionality when all the candidates in the decoding list failed to comply with the distributed CRC bits. Due to the encoding complexity concern, a single interleaver is employed to distribute CRC bits across payload regardless of the payload size. This design choice often results in an insufficient number of distributed CRC bits, which significantly diminishes the effectiveness of early termination. In particular, as shown in the interleaving table [3], only the first 3 CRC bits (the 141st, 142nd, and 143rd bits from the input of interleaver) are distributed to achieve limited amount of early termination when payload size is large. For a typical SCL Polar decoder with list size of 8, the probability that at least one of the eight candidates passes the check for these three CRC bits is relatively high. As a result, UEs frequently process the entire codeword, even when a decoding failure is inevitable, which fail the intended benefits of early termination and further impact UE decoding efficiency. Moreover, the distributed CRC interleaver is subject to maximum payload size of 140bits, which further limit the applicability in 6G application. Addressing the blind decoding effectiveness will be essential for 6G, where UEs are expected to support even more demanding use cases with tighter power and latency constraints.

Observation 9: The 5G distributed CRC design for DCI delivers limited early termination benefit at the UE, resulting in increased power consumption and latency.

Additionally, the 5G Polar code design for DCI is susceptible to RNTI (Radio Network Temporary Identifier) false alarm issues. During blind decoding, the UE must examine multiple candidate codewords, each subjected to RNTI descrambling. In the current 5G design, RNTI bits are directly scrambled with the CRC bits at the end of the codeword, which increases the likelihood of false alarms—where a UE erroneously detects a DCI intended for another UE. In a typical SCL decoder, a list of candidates is checked against CRC bits after RNTI descrambling. Under high SNR conditions, the candidate codewords in the list often differ by only a few bits, with errors typically accumulate at the end of the payload. As a result, when neighboring UEs have similar RNTIs, differing by only a few bits, there is a significant probability that a neighboring UE may incorrectly detect and act upon an unintended DCI after RNTI descrambling. Such false alarms can lead to unnecessary processing and increased power consumption at the UE, and also raise concerns regarding the reliability of control signaling. For example, neighbor UE might mistakenly decode an uplink grant intended for the target UE and cause resource collisions on the uplink, which degrade system performance and potential service disruptions. As 6G networks are expected to support even denser deployments and more stringent reliability requirements, addressing RNTI false alarm issues will be essential to ensure robust and efficient control channel operation.

Observation 10: The 5G RNTI scrambling mechanism for DCI is not optimized for RNTI-based false alarm rate among UEs with different RNTIs and endanger DCI reliability especially in denser and more demanding 6G scenarios.

In summary, while 5G Polar codes provide robust error correction for DCI, their current design presents notable challenges in terms of resource efficiency, decoder complexity, and reliability. Addressing these issues is essential for the evolution of Polar code design in 6G, where greater flexibility, efficiency, and robustness will be required to meet more demanding control channel requirements.

## Outer code design enhancements

### Scalable data integrity check design with early termination feature

In 5G polar code design for DCI, the outer code consists of 24 bits CRC encoding followed by a fixed size interleaver to realize distributed CRC feature for a maximum payload size of 140bits. Such payload size restriction from the distributed CRC design limits the flexibility of multi-carrier scheduling where potential more carriers are considered in 6G. Moreover, each DCI is attached with 24 bits CRC bits and segmentation of payloads for multi-carrier scheduling introduce additional CRC overhead. Therefore, a revisit on the DCI payload size limit should be further discussed in 6G.

To accommodate a potentially larger payload size in 6G as discussed in section 2.1, current 5G distributed CRC should be redesigned since the current design has no scalability to larger payload size. The intention of the 5G distributed CRC design is to provide the early termination feature in polar successive cancellation list decoding, where payload bits are decoded in a sequential manner. Particularly, the distributed CRC bits can be used to perform early check on the list of decoding candidates and execute the decoding termination when all the candidates fail to comply with the CRC checks. Such feature is crucial when considering the computationally intensive DCI blind detection procedure, especially if payload size is increased in 6G. However, the distributed CRC design in 5G only distributed limited CRC bits and the distributed locations are not at the beginning part of payload. Consequently, the early termination benefit is very limited especially when the list size is large. In particular, the probability that a random candidate can pass an m CRC bits check is around 2-m and the probability that at least one of L random candidates pass an m CRC bits check is around L2-m. Therefore, there is a very low possibility of early termination when considering typical polar decoder of list size 8 under current distributed CRC design where less than m=4 CRC bits are distributed in most of cases. In Figure 7, we provide the evaluation result of early termination rate for current 5G design. The early termination rate is defined as 
RET=Ekterminatek+m

where k is the number of total information bits excluding CRC bits, m is the number of total CRC bits, and kterminate the smallest number of decoded information bits (including CRC bits) when none of the candidates in the list can pass the current CRC check after decoding kterminate informations bits. As shown in Figure 7, >50% information bits are processed before decoding termination, which should be further enhanced in 6G.

Figure 7 RET  performance of 5G DCI

In addition to the inferior early termination rate, the implementation of distributed CRC check at decoder requires extra efforts in addition to legacy CRC check due to the irregular distributed CRC bit locations according to different payload size and ALs.

To address the early termination rate and scalability in 6G design, a new data integrity check mechanism which accommodates larger payload size and allows the data integrity check to be performed at earlier decoding stage to provide reasonable early termination benefit should be further studied.

Proposal: Study a new data integrity check mechanism with the aim to improve payload size scalability and early termination rate over 5G distributed CRC design

Proposal: Consider following metric and methodology to facilitate early termination rate evaluation

- Early termination rate: RET=Ekterminatek+m

  - k : payload size

  - m : number of total data integrity check bits

  - kterminate: the smallest number of decoded information bits (including data integrity check bits) when none of the candidates in the list can pass the current CRC check(s) after decoding kterminate informations bits

  - E[kterminate] is estimated under SCL with the assumption of list size=8 and pure AWGN noise is transmitted.

### RNTI-FA reduction design

In current 5G outer code encoding for DCI, 24 CRC bits are first generated followed by scrambling the last 16CRC bits with 16 RNTI bits. During typical polar list decoder, a list of candidate codewords are generated and each candidate is examined by descrambling the last 16 CRC bits using the RNTI. The candidate with the highest likelihood that passes the CRC check after RNTI descrambling is selected as the decoder output. The primary objective of RNTI scrambling is to ensure that only the intended UE, possessing the correct RNTI, can successfully detect and decode the transmitted DCI.

However, due to the nature of list decoding, both the target UE and neighboring UEs process the same or similar multiple candidates in the list, including the transmitted DCI and other incorrect DCIs. While neighboring UEs typically fail the CRC check on the candidate associated with the transmitted DCI due to incorrect RNTI descrambling, there remains a non-negligible probability that a neighbor UE may accidentally pass the CRC check for a candidate associated with incorrect DCI after applying its own RNTI, resulting in an RNTI false alarm. This issue is particularly observed during blind decoding, where the UE must evaluate multiple candidate codewords without prior knowledge whether there is an intended DCI or not. For example, consider two UEs, UE-A and UE-B, assigned with RNTIs of 0000000000000000 and 0000000000000001, respectively, which differ by only a single bit. In high SNR conditions, the candidates produced by polar list decoding may differ by only a few bits, and the similarity in RNTI patterns increases the likelihood that UE-B may incorrectly pass the CRC check for a DCI intended for UE-A, leading to an RNTI false alarm. In this example, if any of the incorrect candidates in the list and the candidate associated with the intended DCI is differed by the last bit, then UE-B can “correctly” decode the incorrect candidate after incorrect RNTI descrambling. Due to the bit difference only occurs at the CRC bit, UE-B will acquire the content of the DCI without knowing that this DCI is actually intended to UE-A. For example, if the DCI is an UL grant, then both UE-A and UE-B will proceed with UL transmission, which cause collision at the network side. Unlike other types of false alarms, the RNTI false alarm rate tends to worsen at high SNR, as the “noise” is introduced by the difference among list of candidates rather than by channel.

To demonstrate the severeness of the RNTI false alarm issue, we first evaluate the FAR from UE perspective. Figure 8 shows the RNTI FA rate of the worst RNTI patterns across several payload sizes for AL=1. The worst RNTI pattern FA is defined as follows.

PRNTIWorst=maxRNTI=0216-1P∃u∈L s.t  ϕu,RNTI passes CRC checks,

where ϕu,RNTI is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTI, L is the list of candidates from SCL decoding. It can be observed that 5G DCI design encounter >10-2 FA rate for the payload size 40-60 bits, which significantly impacts the system reliability for typical DCI configuration. This phenomenon has also been observed in practical studies, such as [5], where the RNTI false alarm rate of the current 5G RNTI scrambling design can remain significantly high, e.g., 10-1 at high SNR region.

Figure 8 Worst case RNTI FAR from 5G DCI Polar design

From a system-level perspective, it is important to assess the reliability of DCI delivery, particularly the risk that a DCI intended for a target UE may be erroneously detected by neighboring UEs. Figure 9 presents an evaluation of this scenario by assuming 11 UEs, each assigned with different RNTIs. In particular, the probability of sending a DCI to one of the UE (target UE) but unexpectedly detected by any of 10 other UEs is simulated across different payload sizes. The results indicate that the probability of sending a DCI to one of the UE (target UE) but mistakenly detected by any of 10 other UEs is between 10-3~10-4. As the number of neighboring UEs increases, this probability is expected to scale proportionally, which amplifies the risk of false alarms in dense network deployments. It is important to note that Figure 9 captures the RNTI FAR for a single transmission event. In practical scenarios, where multiple DCI transmissions occur over time, the cumulative effect of these false alarms can become significant, potentially leading to persistent resource misallocation and degraded system performance. As 6G networks are expected to support even higher densities and more stringent reliability requirements, it is crucial to explore enhanced designs to mitigate false alarm risks in future systems.

Figure 9 Probability of sending a DCI to a target UE and decoded by neighbor UEs (assuming 10 neighbor UEs with different RNTIs from target UE)

To address the reliability issue of current RNTI scrambling mechanism, one possible approach is to introduce additional RNTI based scrambling directly applied to the coded bits. This approach ensures that the intended DCI is indistinguishable from random noise after incorrect descrambling. In fact, in current spec, after the coded bits are generated from polar encoder, a subsequent scrambling associated with RNTI is performed according to [6], which is shown below. This coded bit scrambling can effectively prevent the RNTI FAR. However, the scrambling is only applied for USS. In contrast, for CSS, the scrambling is only a function of cell ID and does not incorporate the RNTI, which causes RNTI-FAR issue. However, if a similar scrambling considered in USS is applied in CSS, then the receiver requires extra descrambling followed by extra decoding, which might have decoding latency issue given the tight budge of blind decoding.

The UE shall assume the block of bits b0, …, b(Mbit-1), where Mbit is the number of bits transmitted on the physical channel, is scrambled prior to modulation, resulting in a block of scrambled bits b0, …, b(Mbit-1) according to bi=bi+c(i) mod 2 where the scrambling sequence is given by clause 5.2.1. The scrambling sequence generator shall be initialized with where - for a UE-specific search space as defined in clause 10 of [5, TS 38.213], equals the higher-layer parameter pdcch-DMRS-ScramblingID if configured; - for a PDCCH with the CRC scrambled by G-RNTI, G-CS-RNTI, MCCH-RNTI, or multicast-MCCH-RNTI in a common search space as defined in clause 10 of [5, TS 38.213], nID∈0,1,…,65535 equals the higher-layer parameter pdcch-DMRS-ScramblingID if configured in a common MBS frequency resource; - nID=NIDcell otherwise and where - is given by the C-RNTI for a PDCCH in a UE-specific search space if the higher-layer parameter pdcch-DMRS-ScramblingID is configured, and - otherwise.

In RAN1#122bis meeting, the RNTI-FAR issue was investigated in [9] and two possible solutions are proposed. The first solution relies on proper gNB implementation by avoid assigning RNTI patterns which cause severe FAR rates to a group of UEs, assuming only few RNTI patterns are concerned. However, our analysis indicates that the RNTI patterns which leads to elevated RNTI-FAR rate depends on payload size, AL, and channel conditions such that multiple RNTI patterns can cause severe FAR. For example, through simulation analysis, there are at least 14 RNTI patterns cause high RNTI-FAR>0.1 for AL=1 and K=23,46, respectively, which are listed in Table 2. The analysis implies that any two UEs with RNTI pattern difference matches to one of the patterns in Table 2 have high probability to decode each other’s DCIs. In other word, to resolve RNTI-FAR issue for a group of neighbor UEs, gNB needs to properly assign RNTIs to the group of UEs such that any two UEs within the group won’t have RNTI difference coincide with any of the problematic RNTI patterns at least in Table 2. This becomes increasingly challenging as the number of UEs grows. Furthermore, for different values of K and AL, additional RNTI patterns may need to be considered, making a gNB implementation-based solution potentially impractical.

| Payload size | RNTI patterns |
| --- | --- |
| 23 | 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 1 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 1 0 0 0 0 0 1 0 1 0 0 0 0 0 1 0 1 0 1 0 1 0 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 0 0 1 1 0 0 0 0 0 0 0 0 0 0 1 0 0 0 1 0 0 0 1 0 0 0 1 0 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 1 1 0 0 0 0 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 1 1 1 1 0 0 0 0 1 1 1 1 0 0 0 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 |
| 46 | 0 0 0 0 1 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 1 1 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 1 0 1 0 1 0 1 0 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 1 1 0 0 1 1 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 1 1 0 0 0 0 1 1 1 1 0 0 0 0 1 0 1 0 0 0 0 0 1 0 1 0 |

Table 2 RNTI pattern examples with RNTI-FAR>0.1

The second solution in [9] needs modification on Polar SCL decoder. In particular, SCL decoder stop splitting and pruning the list of candidates at the last 16 CRC bits where RNTI is scrambled with. Such decoding modification indeed can reduce the RNTI-FAR as claimed. However, the BLER might be impacted since the optimal SCL performance is not fully exploited at the last 16 CRC bits. The BLER performance associated with K=44-48 are shown in Figure 10 where nonnegligible performance degradation is observed.

Figure 10 BLER performance for SCL v.s. SCL(no path splitting at the last 16bits)

Without sacrificing the BLER performance and SCL decoder modification, another possible approach is to modify the CRC bit and RNTI scrambling location within the payload. As illustrated in Figure 11, 24 CRC bits are appended at the end of the payload and 16 RNTI bits are scrambled with the last 16 CRC bits in current 5G design. Due to the list decoding nature where candidates in the list are prone to share the same decoded bits in the beginning but diverge the end, there is a tendency that the list contains candidates associated with correct payload but scrambled with RNTIs belongs to other UEs, which causes RNTI FAR as previously discussed. By recognizing such characteristic of SCL decoding, one effective way to mitigate the RNTI FAR issue is to simply shift the CRC bits scrambled by RNTI to the beginning of the payload as indicated in Figure 11. By doing so, the list decoding process, which typically produces candidates with matching initial bits, will result in all candidates having the same correctly scrambled CRC bits at the start. When these candidates are descrambled using incorrect RNTIs, the CRC bits are likely to be incorrect, which significantly increases the probability of failing the CRC check. As a result, neighboring UEs with different RNTIs are much less likely to successfully decode the DCI, thereby greatly reducing the RNTI FAR. The effectiveness of the simple scheme is demonstrated in Table 3 where the observed RNTI FAR is significantly reduced across the evaluated payload sizes.

Figure 11 Proposed RNTI scrambling scheme

| Payload size | 5G | Proposed scheme |
| --- | --- | --- |
| 44 | 1.17×10-3 | <10-6 |
| 46 | 1.06×10-3 | <10-6 |
| 48 | 1.05×10-3 | <10-6 |

Table 3 Probability of sending a DCI to a target UE and decoded by neighbor UEs (assuming 10 neighbor UEs with different RNTIs from target UE) for 5G and proposed scheme

Proposal: Study new mechanism of RNTI scrambling to reduce RNTI-FA rate

Proposal: Consider following methodology to facilitate DCI RNTI false alarm evaluation.

- Given a target UE RNTItarget, random payload with size K, aggregation level AL, and AWGN channel

  - Alt1:Evaluate PRNTIWorst=maxRNTI ≠RNTItarget Pr∃u∈L s.t  ϕu,RNTI passes CRC checks

    - L is the list of 8 candidates from SCL decoding

    - ϕu,RNTI is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTI

    - Opration @SNR with BLER=1%

  - Alt2: Evaluate Pr∃u∈L s.t  ϕu,RNTIi passes CRC checks for i∈1,2,…,N

    - L is the list of 8 candidates from SCL decoding

    - ϕu,RNTIi is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTIi

    - {RNTIi,  for i∈1,2,…,N } are distinct and generated uniformly from all possible RNTI ≠RNTItarget for each DCI transmission

    - Opration @SNR with BLER=1%

## Polar+high order QAM for UL/DL

In the current 5G standard, bit interleaved coded modulation (BICM) scheme is adopted for polar code with high order QAM in UCI. While BICM is widely used due to its simplicity and compatibility with existing systems, it is well recognized as a suboptimal approach from capacity perspective. In contrast, multi-level coding (MLC) based coded modulation theoretically achieves channel capacity, offering superior performance over BICM under ideal conditions. However, it is challenging to realize such theoretical capacity achieving property of MLC in practice especially when short block length is considered. In particular, the optimal rate allocation across multiple levels requires careful investigation to fully leverage the performance benefits from MLC. As a result, the performance improvements over BICM has not been widely accepted/adopted for most standards.

Recently, constellation shaping begins to draw attention in 6G RAN 1 meeting. Different from legacy shaping techniques where separated blocks for distribution matcher and the channel coding are considered, it has been shown in [4] that multi-level polar code can be naturally combined with shaping. This combination allows overall scheme to enjoy both the MLC gain and shaping gain. In the following section, detailed description of such scheme and several examples are provided to demonstrate the performance advantage.

First, MLC description is as follows. Let X be the symbol alphabet, and the m=log2X be the number of bits representing each symbol. The multi-level coding schemes is composed of m layers of codes with different code rates Rl. When the decoding procedure is performed serially with the information from previous layers, the sum of capacity for each bit layer matches the capacity, i.e.,

l=1mIBl;YB1l-1=IB1,…Bm;Y=IX;Y,

where Bl is the random variable denoting the bit of the lth layer, and X and Y are the transmitted and received symbol, respectively. By allocating the code rate for each layer Rl based on IBl;YB1l-1, it can be shown that MLC is theoretical optimal while provides robust finite block length performance with careful design on rate allocation for each layer. Regarding the bit to symbol mapping, as suggested by literatures, set partitioning bit mapping is used in MLC scheme at encoder side, and the bit layer with least capacity is decoded first at decoder end.

Under the framework of MLC, a joint design of coding and modulation to realized shaping gain is provided as follows. Let n be the number of symbols. The overall procedures of the polar-coded modulation with shaping in the case of m=2 are shown in Figure 12.

Figure 12 Block diagram of joint polar coding and shaping

Polar bit indices are partitioned into the frozen set F, the information set I, and the shaping set J with size fs,l, ks,l and ps,l, respectively. Given the number of total payload size k=lks,l and the target symbol distribution PA, the size of these sets can be determined by finding a surrogate channel WY|X such that the corresponding capacity IWY|X;PA matches the information rate k/n. After serial partitioning WY|X into bit channels {WY|B1l}l=1m, the number of information and shaping bits for each layer l is given by

ks,l= I(WY|B1l;PBl|B1l-1)
ps,l = 1-H(PBl|B1l-1).

The transmitter follows the decode-then-encode procedure by first determining the value of ps,l shaping bits for each layer of polar code. The lth polar codeword cl=cl1,…,cln is then utilized to update the likelihood ratio, i.e. LLRl+1,i for all i∈1;n, required to determine the shaping bits of the next layer, where

LLRl,i=logPBl|B1,…,Bl-1(cl(i)=0|c1l-1(i))PBl|B1,…,Bl-1(cl(i)=1|c1l-1(i)).

More information how to determine the number of information bits and shaping bits for each layer and surrogate channel WY|X can be found in [10]. The symbol sequence x1,…,xn are then generated from cl,… cm with set partitioning bit mapping rule. Since the scheme is MLC, the receiver needs to decode the polar codes layer by layer with the information of previous codewords. Unlike the case of uniform QAM, the demodulator needs to consider the prior information of symbols, i.e., the MB distribution with parameter ν, when calculating the likelihood ratio of each coded bit. After finishing SC/SCL decoding, information bits are those in the indices set I.

BLER performance of MLC w/ and w/o combining shaping are evaluated across different payload sizes in Figure 13  BLER performance of Joint Polar Coding and Shaping with K=140, 16QAM, symbol length=64Figure 13, Figure 14, Figure 15, and Figure 16, where values of fs,l, ks,l and ps,l are specified under each figure, and the list size of SCL decoder with 8 is used for both transmitter and receiver. As demonstrated in the examples, 0.2-0.3 dB gain over 5G BICM is observed solely on MLC framework. By incorporating shaping bits, >0.5dB gain is observed across the payload sizes examined.

| Figure 13 BLER performance of Joint Polar Coding and Shaping with K=140, 16QAM, symbol length=64 w/ Shaping: ks,1=62, ks,2=102, ps,1=0, ps,2=23 w/o Shaping: ks,1=45, ks,2=119, ps,1=0, ps,2=0 MB parameter ν=0.13437 for shaping. | Figure 14 BLER performance of Joint Polar Coding and Shaping with K=300, 16QAM, symbol length=128 w/ Shaping: ks,1=134, ks,2=190, ps,1=0, ps,2=62 w/o Shaping: ks,1=87, ks,2=237, ps,1=0, ps,2=0 MB parameter ν=0.1592 for shaping. |
| --- | --- |
| Figure 15 BLER performance of Joint Polar Coding and Shaping with K=500, 16QAM, symbol length=256 w/ Shaping: ks,1=203, ks,2=321, ps,1=0, ps,2=175 w/o Shaping: ks,1=105, ks,2=419, ps,1=0, ps,2=0 MB parameter ν=0.19869 for shaping. | Figure 16 BLER performance of Joint Polar Coding and Shaping with K=700, 16QAM, symbol length=256 w/ Shaping: ks,1=309, ks,2=415, ps,1=0, ps,2=93 w/o Shaping: ks,1=229, ks,2=495, ps,1=0, ps,2=0 MB parameter ν=0.13437 for shaping. |

Unlike UL in 5G spec, only QPSK is considered in DCI 5G polar code design. The justification of this limitation should be reviewed in 6G especially in high-SINR scenarios where higher-order modulation could be utilized to achieve better SE.

To show the feasibility of considering DCI with high order QAM, the measure SINR from dense urban SLS simulation result is shown in Figure 17. The data indicates that a significant proportion of the measured SINR values—over 80%—exceed 16 dB. Based on CQI reporting guidelines, 16QAM is recommended when the measured SINR falls within the 7 dB to 16 dB range. Consequently, in the majority of scenarios, high order QAM schemes can be effectively deployed. This suggests that utilizing high-order QAM for DCI transmission to enhance resource efficiency is feasible and reasonable in the context of 6G system.

Figure 17 CDF of measured SINR with 2port transmission

Proposal: Study high order QAM polar code for UCI performance enhancement and DCI SE enhancement.

# Conclusion

In summary, followings are the observations and proposals.

Observation 1: Due to the 6G maximum SE decision made in RAN and newly introduced spectrum, 2x-4x peak data rate improvement from the 5G QC-LDPC design is anticipated in 6G.

Observation 2: dB level BLER performance degradation is observed when reducing decoding iteration I from 20 to 5 for 4X peak data rate gain.

Observation 3: Increasing maximum lifting size Z and/or reducing decoding cycles per decoding iteration C  from current 5G design provides potential Tput enhancements.

Observation 4: Reducing decoding cycles per decoding iteration C from current 5G design provides potential decoding latency benefit.

Observation 5: Dual diagonal structure and Raptor code structures introduced in 5G QC-LDPC code can be leveraged in 6G QC-LDPC code design to achieve encoding latency reduction.

Observation 6: Bottleneck case to determine peak data rate of BG1 is MCS=20 based on peak data rate estimation minMCS=XCBsizefmaxImcs=X×Cmcs=X×RMCS=27RMCS=X×QAMMCS=27QAMMCS=X.

Observation 7: Due to the limitation of maximum 140bit payload sizes and the support of only QPSK QAM, the 5G Polar code design for DCI is limited on scheduling flexibility and resource efficiency to support 6G applications.

Observation 8: 6G’s wider bandwidth substantially increases DCI FDRA bit requirements and current 140bits limit is not sufficient.

Observation 9: The 5G distributed CRC design for DCI delivers limited early termination benefit at the UE, resulting in increased power consumption and latency

Observation 10: The 5G RNTI scrambling mechanism for DCI is not optimized for RNTI-based false alarm rate among UEs with different RNTIs and endanger DCI reliability especially in denser and more demanding 6G scenarios.

Proposal: Study 6G QC-LDPC design to achieve 2x-4x peak data rate of 5G with acceptable performance-complexity tradeoff for both NW side and UE side provided that additional performance and complexity trade-offs required from 2 to 4 times peak data rate increase is minimized

Proposal: Consider I×C as the estimation of total decoding cycles per CB

- C: decoding cycle per iteration

  - C can be approximated by e/M, where e is the number of edge in a BG and M is the number of edges available to be processed simultaneously, if any

- I: [Average number or max number] of iterations to achieve target BLER at target SNR

  - Target BLER: [0.01]

  - Target SNR=Reference SNR+[<0.5]dB

  - Reference SNR: The SNR where BG1 achieves target BLER under Layer BP with 20 iterations

Proposal: To ensure BLER requirement is satisfied for all MCS, consider the following metric to facilitate peak data rate evaluation for a decoder operating at maximum frequency fmax

- minMCS=XCBsizefmaxImcs=X×Cmcs=X×RMCS=27RMCS=X×QAMMCS=27QAMMCS=X

  - Imcs=X is the minimum iterations to satisfy BLER performance requirement

Proposal: To improve 2-4x peak data rate from 5G, study following design aspects of QC-LDPC with acceptable error correction performance and implementation complexity trade-off

- Reduced number of iterations

- Reduced BG edges

- Parallelism structure

- Maximum lifting size>384

- BG size adaptation for different code rates

- Combination of above aspects

Proposal: Study a new data integrity check mechanism with the aim to improve payload size scalability and early termination rate over 5G distributed CRC design

Proposal: Consider following metric and methodology to facilitate early termination rate evaluation

- Early termination rate: RET=Ekterminatek+m

  - k : payload size

  - m : number of total data integrity check bits

  - kterminate: the smallest number of decoded information bits (including data intefrity check bits) when none of the candidates in the list can pass the current CRC check(s) after decoding kterminate informations bits

  - E[kterminate] is estimated under SCL with the assumption of list size=8 and pure AWGN noise is transmitted.

Proposal: Study new mechanism of RNTI scrambling to reduce RNTI-FA rate

Proposal: Consider following methodology to facilitate DCI RNTI false alarm evaluation.

- Given a target UE RNTItarget, random payload with size K, aggregation level AL, and AWGN channel

  - Alt1:Evaluate PRNTIWorst=maxRNTI ≠RNTItarget Pr∃u∈L s.t  ϕu,RNTI passes CRC checks

    - L is the list of 8 candidates from SCL decoding

    - ϕu,RNTI is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTI

    - Opration @SNR with BLER=1%

  - Alt2: Evaluate Pr∃u∈L s.t  ϕu,RNTIi passes CRC checks for i∈1,2,…,N

    - L is the list of 8 candidates from SCL decoding

    - ϕu,RNTIi is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTIi

    - {RNTIi,  for i∈1,2,…,N } are distinct and generated uniformly from all possible RNTI ≠RNTItarget for each DCI transmission

    - Opration @SNR with BLER=1%

Proposal: Study high order QAM polar code for UCI performance enhancement and DCI SE enhancement.

# Reference

- RP-251881, New SID: Study on 6G Radio

- 3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities"

- 3GPP TS 38.212: "NR; Multiplexing and channel coding"

- J. Honda and H. Yamamoto, "Polar Coding Without Alphabet Extension for Asymmetric Models," in IEEE Transactions on Information Theory, vol. 59, no. 12, pp. 7829-7838, Dec. 2013

- K. Tian, H. Wang, M. Zhang and X. Yang, "A Novel Scheme to Mitigate the RNTI-FA in Blind Detection of 5G Polar Codes," 2022 IEEE 95th Vehicular Technology Conference

- 3GPP TS 38.211: "NR; Physical channels and modulation"

- R1-2507611

- R1-2507256

- R1-2507061

- B. Wu, K. Niu, J. Dai and Y. Yuan, "Joint Design of Channel Coding and Modulation Toward 6G: Probabilistically-Shaped Polar-Coded Modulation," in IEEE Transactions on Communications, vol. 73, no. 8, pp. 5538-5553, Aug. 2025