3GPP TSG RAN WG1 #123			R1-2509255

Dallas, USA, Nov. 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on LS on SBFD and CA

Agenda Item:	5

Document for: 	Discussion and Decision

- Introduction

In this contribution, RAN2 LS on SBFD and CA and corresponding RAN1 action are discussed.

- Discussion

At the RAN2#131bis meeting, RAN2 has discussed stage-2 specification impact to support SBFD with CA according to the following agreement made at RAN1#120.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

However, RAN2 has not reached a conclusion on how to capture SBFD operation with CA in the stage-2 specification (TS 38.300), and RAN2 asks RAN1 to provide TP for supporting SBFD with CA in the stage-2 specification (TS 38.300) [1].

In addition, at the RAN1#121 meeting, the following agreement was achieved on SBFD and NR-DC.

Agreement - From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier. - Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly. ### 7.6.2 NR-DC The UE procedures described in this clause are not applicable if the UE is provided scg-State [12, TS 38.331]. If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1 for FR1 and/or by nrdc-PCmode-FR2 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, or semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or FR2 as defined in [8-3, TS 38.101-3]. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE determines a transmission power for the MCG or for the SCG as described in clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 - if the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG or SCG, the UE determines a transmission power for the MCG or for the SCG as described in clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively - if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, or is indicated as SBFD symbol to a UE by tdd-UL-DL-ConfigurationCommon, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on SCG or the MCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in clauses 7.1 through 7.5 without considering PSCG or PMCG respectively <unchanged texts omitted>. - It is up to RAN2 to decide whether or not to support simultaneous configuration of SBFD and DC. - CC: RAN3, RAN4. Reply LS on simultaneous configuration of SBFD and DC. Final LS in R1-2504858.

RAN1 also received an LS [2] from RAN2, and RAN2 informed RAN1 that SBFD and DC can be supported with limited impact such as stage-2 impact only.

RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion.

Based on the above situation, RAN1 should make a TP for TS 38.300 to capture not only the support of SBFD+CA but also the support of SBFD+NR-DC based on the RAN1 agreements.

Proposal 1: TP#1 for TS 38.300 is adopted to capture the support of SBFD+CA and SBFD+NR-DC.

- Proposed TP

TP#1

TS 38.300, v19.0.0 #### 23 SBFD ## 23.1 General Sub-Band Full Duplex (SBFD) operation is supported for a TDD carrier, enabling simultaneous downlink transmission and uplink reception at the gNB on their non-overlapping respective sub-bands. From UE perspective, full duplex is not supported. The configurations of cell-specific SBFD time and frequency resources are provided through SIB1 or dedicated signalling. A UE can be semi-statically configured with SBFD sub-bands in downlink symbols and flexible symbols provided by tdd-UL-DL-ConfigurationCommon and this UE is referred to as an SBFD aware UE. The maximum number of UL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is one. The maximum number of DL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is two. A UE can be configured with SBFD symbols on only one TDD carrier when the UE is configured with CA. A UE can be configured with SBFD symbols on only one TDD carrier when the UE is configured with inter-band NR-DC and the UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA. In an SBFD symbol, except for cross-link interference measurements, a UE transmits or receives only in RBs that are both in the active UL BWP and in the UL sub-band, or both in the active DL BWP and the DL sub-band(s), respectively. A UE can be configured to transmit or receive only in non-SBFD symbols, only in SBFD symbols, or across both SBFD symbols and non-SBFD symbols for multiple transmission or reception occasions. <*****************************************omitted******************************************>

- Conclusion

In this contribution, we discussed RAN2 LS on SBFD and CA and corresponding RAN1 action. We have the following proposals:

Proposal 1: TP#1 for TS 38.300 is adopted to capture the support of SBFD+CA and SBFD+NR-DC.

- References

- R1-2508304 (R2-2507746), LS on SBFD and CA

- R1-2506719 (R2-2506492), Reply LS on simultaneous configuration of SBFD and DC

- 3GPP TS 38.300 V19.0.0, NR; NR and NG-RAN Overall Description; Stage 2 (Release 19)

- 3GPP, RAN1#120 meeting, Chairman’s notes

- 3GPP, RAN1#121 meeting, Chairman’s notes