3GPP TSG RAN WG1 #123			R1-2509260

Dallas, US, November 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Maintenance on enhancements of network energy savings for NR

Agenda Item:	8.6

Document for: 	Discussion and Decision

- Introduction

In this tdoc, we discuss maintenance issues within the scope of A.I. 8.6 for RAN1 normative work of eNES [1].

- Discussion for OD-SSB

2.1	Interpretation for SSB types Regarding SSB-less operation

Agreements@RAN2#131bis [2]

- When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is not configured, the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation).

- When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is deactivated, RAN2 assume that the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation as legacy Rel-18 behavior).

- The field description of absoluteFrequencySSB is updated as follows:

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is ab-sent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. Thids is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

It was agreed in the RAN2#131bis about the change of the text regarding absoluteFrequencySSB in TR38.331 so that the statement took into account the UE behavior for Rel-19 UEs. Based on the agreement, the UE behavior for obtaining timing reference in SSB-less SCell operation is allowed for the following cases.

- Case a: If the SSB is not configured in SCell and OD-SSB configuration is not configured.

- Case b: If the SSB is not configured in SCell and OD-SSB is configured, but not activated.

However, in TS38.213 clause 4.1, the UE behavior for SSB-less operation might not clearly be captured for both cases. In the spec., it says that [3],

For a serving cell without transmission of SS/PBCH blocks, a UE acquires time and frequency synchronization with the serving cell based on receptions of SS/PBCH blocks on the PCell, or on the PSCell, or on an SCell if applicable as described in [10, TS 38.133], of the cell group for the serving cell.

Here the SS/PBCH blocks that is not transmitted can have two interpretations:

- Interpretation A: SS/PBCH blocks refer to as SSB whose location is defined in absoluteFrequencySSB, but not SSB whose location is defined in od-ssb-absoluteFrequency.

- Interpretation B: SS/PBCH blocks refer to as SSB whose location is defined in absoluteFrequencySSB, or SSB whose location is defined in od-ssb-absoluteFrequency, but OD-SSB is deactivated.

In our view, we should take interpretation B to align with the RAN2 agreement. However, this does not mean that UE is always required to be configured with SSB-less SCell when OD-SSB case #1 is configured. In other words, when UE is configured with both OD-SSB and SSB-less SCell, UE can use another cell for timing reference when the OD-SSB is deactivated.

Observation 1:

The interpretation of “a serving cell without transmission of SS/PBCH blocks” captured in TS38.213 clause 4.1 is unclear.

Proposal 1:

For SS/PBCH blocks captured in TS38.213 clause 4.1, we interpret SS/PBCH blocks as those whose location is determined by absoluteFrequencySSB, or by od-ssb-absoluteFrequency.

Another aspect we would like to discuss is to clarify whether OD-SSB can be configured in a SCell that can be used for UEs to acquire timing reference. In our view, at least, SCell with OD-SSB case #2 should be a reference cell since the SCell has always-on SSB. Besides, it should be another discussion point whether only AO-SSB or both AO and OD-SSB (, which is up to UE implementation) can be used as a timing reference.

Proposal 2:

Clarify that SCell with OD-SSB can be a reference cell for another SCell with SSB-less.

- Conclusions

In this contribution, we provided the following observations and proposals of on-demand SSB SCell operation for network energy saving.

Observation 1:

The interpretation of “a serving cell without transmission of SS/PBCH blocks” captured in TS38.213 clause 4.1 is unclear.

Proposal 1:

For SS/PBCH blocks captured in TS38.213 clause 4.1, we interpret SS/PBCH blocks as those whose location is determined by absoluteFreqencySSB, or by od-ssb-absoluteFrequency.

Proposal 2:

Clarify that SCell with OD-SSB can be a reference cell for another SCell with SSB-less.

References

[1] RP-250343, Revised WID on enhancements of network energy savings for NR, Mar. 2025

[2] R2-2507768, LS on OD-SSB, RAN2#131bis, Oct. 2025

[3] TS 38.213