3GPP TSG RAN WG1 #123	R1-2509331

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:		11.5

Source:	Nordic Semiconductor ASA

Title:					Energy Efficiency in 6GR

Document for:		Discussion and Decision

# Introduction

NR introduced already several features for UE power savings and network energy savings (NES). In 6GR, energy efficiency is also listed as one of the primary targets as indicated in 6GR SID [1]:

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In the previous meeting, RAN1#122bis, the following agreements were made under this agenda item:

Agreement

At least the following NR metrics,

- Network energy saving gain relative to baseline for BS

- UE energy saving gain relative to baseline for UE

- Impact to UPT (User-Perceived Throughput), if applicable,

as well as the metrics

- Impact to latency, if applicable

- Impact to QoS/delay budget satisfaction rate, if applicable

are used for 6G energy efficiency evaluation.

Agreement

Apply the following evaluation methodology framework for Quantitative analysis,

- For NW unloaded/empty load case or UE idle/inactive mode:

- For energy saving: analytical calculation

- For performance impact: analytical calculation, LLS

- For loaded cases and connected-mode UEs

- For energy saving: SLS

- For performance impact: LLS, SLS

Agreement

For evaluation purposes, expand the existing BS power model reference configuration with a set for ~7 GHz operation with the following parameters:

| Property | Configuration for Set 4 around 7 GHz |
| --- | --- |
| Duplex | TDD |
| BW | [100, 200, 400] MHz |
| SCS | [30 kHz, 60 kHz] |
| Number of TRP | 1 |
| Total number of DL TX RUs | [128, 256] |
| Total DL power level | [56] dBm |
| Total number of UL Rx RUs | [128, 256] |

Note: Bracketed values to be confirmed. Other values are not precluded.

The above configuration has no implication on supported BW, SCS for 6GR.

Agreement

Study whether/how to further update the BS model considering the following aspects, e.g.,

- Whether to downselect between Cat.1 and Cat. 2,

- Updates of parameter values (including defining a new Cat),

- Updates of power scaling, power states (including additional PSs)

- Etc.

Note: The defined BS power models does not preclude use case-specific enhancements regarding, e.g., multi-TRP, SBFD, multi-carrier etc

Agreement

Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to:

BS assumptions:

- Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g.,

  - Clustered provisioning of different cell-common signaling,

  - On-demand provisioning of different cell-common signaling,

- UE-specific signaling (for low, light, medium loads), e.g.,

  - Clustered provisioning with cell-common signaling,

  - Unclustered provisioning with cell-common signaling,

UE impact:

- Cell search complexity and latency, including frequency search latency,

- UE Power consumption,

- Sync signal detection, coverage and tracking performance,

- RRM, mobility,

- Beam management,

- Other properties are not precluded,

- Improvements to address identified impact, e.g.,

  - Additional sync signal needs,

  - Adaptation of sync signal transmission periodicity,

  - Sparser synch raster.

Agreement

Study and evaluate on-demand and/or periodic SIB-1 transmission with respect to

- NW energy savings potential and UE power consumption impact,

- SIB-1 acquisition delay,

- NW and UE complexity,

- Coverage,

- Applicable deployment scenarios, e.g.,:

  - Standalone cell/carrier,

Multiple TRPs/cells/carriers

Agreement

For 6GR energy efficiency evaluation purposes, reuse the existing UE power consumption model FR1 and FR2 reference configurations in TR 38.840 for operation up to around 7GHz and within 24.25 GHz – 52.6 GHz, respectively.

- Scaling rules can be updated, including additional rule(s) for scaling UE power consumption, and including around 7GHz specific update

  - FFS: details.

- Power value and transition time update, if necessary, including around 7GHz specific update

- No implication on supported BW, SCS, modulation and antenna setting for 6GR

- Revisit if SCS for around 7GHz is different with respect to the reference configuration

Agreement

Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects:

- Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other)

- Measurements and/or synchronization.

- System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS.

- RRC states

Other functionalities

Agreement

For evaluation purposes, study extending NR UE power consumption scaling w.r.t. at least BW and/or antenna setting, considering at least the different characteristics in RF/BB power consumption and static/dynamic power consumption.

Agreement

For evaluation purposes, study extending NR UE power consumption model for UE operation with DL WUS of OFDM-based sequence, regarding the following aspects:

- Power state(s), sleep and non-sleep, and corresponding characteristics and power value(s)

- Transition energy and time for each of sleep state(s)

- Companies to report the assumption(s) for achieving the proposed power value(s), e.g., time/frequency domain detection, noise figure assumption(s), synchronization assumption(s), BW/antenna assumption(s), etc.

Agreement

Study and evaluate on-demand sync signal(s) mechanisms for 6GR energy efficiency, considering, e.g.,:

- On-demand sync signal(s) for single cell/carrier, multi-carrier/cell, multi-TRP,

- Network-triggered and UE-triggered on-demand sync signal(s),

- Idle and/or connected modes,

- Other mechanisms/aspects/signals/channels are not precluded.

Agreement

Study and evaluate multi-carrier/cells/TRPs mechanisms for 6GR NES, considering, e.g.,:

- Sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs, including potential extensions to additional deployments and scenarios,

- RRC states,

- UE energy consumption and complexity,

- Other mechanisms/aspects/signals/channels are not precluded.

In this contribution, we will address some of the remaining energy efficiency aspects of UE and network in RRC idle mode and RRC connected mode. As a general note, energy efficiency should be considered together with other important aspects of the communication system, like user experience and UE implementation aspects.

# Remaining issues in Energy efficiency study

NR introduced most of the power saving features after the initial 5G NR specification was released, and thus those concepts were forced to handle compatibility issues and other restrictions of the initial technical choices made in the initial release. In 6GR, energy efficiency (EE) aspects should be taken into account from the very beginning and the relevant technical solutions should be included in the initial release of 6GR. Whether some of the EE features are made mandatory for UE or network can be discussed separately after features are agreed upon. From the deployment point of view, it is important that EE features are made attractive both from UE and network point of view and, if some feature is made mandatory for UE, it should be mandatory for network as well.

Proposal 1: RAN1 should strive for energy efficiency features that are attractive both for UE and network and they should be included in an initial 6GR specifications.

Proposal 2: If some of EE features are made mandatory for UE they should be mandatory for network as well.

A major energy saving feature for NW is to increase periodicity of synchronization signals and other broadcast type of common signaling. Guidelines for that study was agreed at the last meeting and one of the study items is clustering of different common signaling from NW perspective. Along the same lines, also clustering of RACH and paging operations should be studied. From the FL summary [2], we can find the FL proposal:

Study and evaluate energy efficiency of PRACH and paging resources with respect to:

- PRACH and paging resource clustering with sync signals and SIB-1

- NW and UE savings potential,

- PRACH and paging periodicity and relation with sync signal or SIB-1 periodicity,

- Impact on latency and potential mitigation techniques,

- Preamble sequence set size and paging procedures,

Etc.

In general, we are supportive of that kind of study, but the fifth sub-bullet is unnecessary at this phase of study. So, we propose the following:

Proposal 3: Study and evaluate energy efficiency of PRACH and paging resources with respect to:

- PRACH and paging resource clustering with sync signals and SIB-1

- NW and UE savings potential,

- PRACH and paging periodicity and relation with sync signal or SIB-1 periodicity,

- Impact on latency and potential mitigation techniques

Another NES feature is to adapt BS operations with respect to e.g. BW, number of Tx antennas, and transmit power when there is no load or very low load at the network. This is to facilitate low-power mode of operation for BS when traffic load in the network is low. The FL proposal for that type of study was formulated in the FL summary [2] as follows:

Study and evaluate BS aspects for NES regarding common signal transmission at least for initial access and the impact on UE performance and user experience, including, e.g.,

- Power domain, e.g., Tx power,

- Frequency domain, e.g., BW,

- Spatial domain, e.g., #antennas,

- Sleep state transitions.

We are supportive of that study as long as impact on UE performance is carefully considered, also taking into account UEs with ononly e Rx antenna.

Proposal 4: When various NES features are studied, including various adaptation methods for facilitating lower power mode for BS, impact on UE performance should be carefully considered, also taking into account UEs with only one Rx antenna.

Guidelines for the study of DL LP-WUS was already agreed at the last meeting. The only missing part is the interplay of DL LP-WUS with DRX in the RRC idle and connected mode. For example, we should study the interaction of LP-WUS and eDRX in the idle mode and, furthermore, if long eDRX could even be dropped out of 6GR toolbox. In the final feature lead summary [2], the following proposal on this topic was made:

Proposal 5.1.2.1a:

Study and evaluate UE DRX operations (iDRX, cDRX, eDRX) for 6GR EE improvement, considering at least the following aspects:

- Coordination and avoiding duplication with UE operation based on DL WUS

Mechanism for configuration update/switch

We are supportive of the study along the lines expressed in FL summary.

Proposal 5: Study UE DRX operations in RRC idle and connected modes, including interaction between LP-WUS and DRX.

As discussed at the previous meetings, the network energy may be further reduced by joint cell DTX/DRX and UE DRX, facilitating for example dynamic activation/deactivation of capacity cells in the network. If going into that direction, its implications to UE implementation and communication latency should be carefully studied.

Proposal 6: If joint cell DTX/DRX and UE DRX is pursued in 6GR, a special emphasis should be placed on UE implications such as UE complexity and communication latency.

Various adaptation methods may be employed to improve energy efficiency of UE or jointly network and UE. These may be roughly categorized into spatial domain, frequency domain, time domain, and power domain methods. In spatial domain, for example, beam management methods may be simplified so that UE power consumption is reduced. In frequency domain, UE receiver bandwidth may be adjusted to smaller or larger value depending on state where UE is operating on, similar to what is allowed in NR already in the form of BWP switching. The BWP switching in NR has, however, known weak points which should be addressed and improved towards 6G.

Proposal 7: Study adaptation methods in spatial/power, frequency, and time domains for energy efficiency enhancement in 6GR, by leveraging existing 5G adaptation solutions. at least for downlink control channel monitoring.

For cellular IoT UEs, time domain adaptation methods are maybe the most important ones, including e.g. adaptation of downlink control channel monitoring. In NR, multiple similar downlink monitoring adaptation methods were introduced, like PDCCH monitoring skipping and SSSG switching and finally connected-mode LP-WUS in Rel-19. This kind of wasteful and overlapping specification work should be absolutely avoided in 6GR, and we are favorite of harmonized design for adaptation of downlink control channel monitoring, encompassing at least C-DRX, SSSG switching, and OFDM-based WUS as a starting point.

Proposal 8: Study a harmonized adaptation for downlink control channel monitoring, by leveraging existing C-DRX, SSSG switching, and OFDM-based WUS techniques as a starting point.

Most of the UE power saving features above have focused on DL operation of UE. However, a large chunk of proposals to enhance UL transmission efficiency from UE point of view and thus to reduce UE power consumption were also seen from many companies in the last meeting. Consequently, the feature lead [2] has summarized those proposals into the following form:

Proposal 5.6.2.1a:

Study and evaluate power consumption reduction mechanism(s) related to UE's UL data transmission, taking into account the following enhancement aspects:

- Time domain aggregated transmission (UE DTX related)

- Enhanced uplink scheduling for reducing unnecessary uplink transmission, including UL skipping

- Ensuring no late changes to UL transmissions

- Autonomous PUSCH transmission mode without grant

- Releasing unused configured grant resources

We also think that improving UL transmission efficiency from UE point of view is worth studying further towards 6G and thus we are supportive of the FL proposal above.

Proposal 9: Study mechanisms to enhance UL transmission efficiency from UE point of view.

# Conclusions

In this contribution we discussed issues related to energy efficiency of UE and network in 6GR, and we have the following proposals:

Proposal 1: RAN1 should strive for energy efficiency features that are attractive both for UE and network and they should be included in an initial 6GR specifications.

Proposal 2: If some of EE features are made mandatory for UE they should be mandatory for network as well.

Proposal 3: Study and evaluate energy efficiency of PRACH and paging resources with respect to:

- PRACH and paging resource clustering with sync signals and SIB-1

- NW and UE savings potential,

- PRACH and paging periodicity and relation with sync signal or SIB-1 periodicity,

- Impact on latency and potential mitigation techniques

Proposal 4: When various NES features are studied, including various adaptation methods for facilitating lower power mode for BS, impact on UE performance should be carefully considered, also taking into account UEs with only one Rx antenna.

Proposal 5: Study UE DRX operations in RRC idle and connected modes, including interaction between LP-WUS and DRX.

Proposal 6: If joint cell DTX/DRX and UE DRX is pursued in 6GR, a special emphasis should be placed on UE implications such as UE complexity and communication latency.

Proposal 7: Study adaptation methods in spatial/power, frequency, and time domains for energy efficiency enhancement in 6GR, by leveraging existing 5G adaptation solutions. at least for downlink control channel monitoring.

Proposal 8: Study a harmonized adaptation for downlink control channel monitoring, by leveraging existing C-DRX, SSSG switching, and OFDM-based WUS techniques as a starting point.

Proposal 9: Study mechanisms to enhance UL transmission efficiency from UE point of view.

# References

RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, 3GPP TSG RAN#108, Prague, Czech Republic, June 9-13, 2025

R1-2508187, Final summary of 6GR energy efficiency study, Moderators (Ericsson, MediaTek)