3GPP TSG RAN WG1 #123		R1-2509344

Dallas, USA, Nov 17th – 21st, 2025

Source:	CEWiT

Title:	Discussion on D2R signals, channels, waveform and procedures for Ambient IoT

Agenda Item:	10.3.2.2

Document for:	Discussion and Decision

- Introduction

At RAN#109, a revised study item was approved to explore enhancements for Ambient IoT (Internet of Things) in NR outdoor environments targeting active devices [1]. The study aims to address the following main objectives:

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 … This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

In this contribution, we discuss the potential impacts on D2R signals, channels, waveform and procedures for A-IoT device 2b/C.

- Discussion

In RAN1#122-bis meeting [2], the following agreement on D2R modulation was achieved.

Agreement Regarding D2R modulation for device 2b and device C, further study the following: - Option 1: BPSK - FFS the corresponding preamble and midamble design for coherent detection at reader side - Option 2: OOK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side - Option 3: DBPSK - FFS the corresponding preamble and midamble design for non-coherent detection at reader side - Option 4: MSK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side Agreement The study assumes single-carrier waveform for D2R. FFS whether to use 2SB or 1SB for D2R

In Release-19, Device 1 relied on backscatter communication for its D2R uplink, where modulation occurred by reflecting an external carrier signal. This reflection inherently generated a double-sideband (2SB) signal, creating both upper and lower sidebands around the carrier frequency. With Release-20, the introduction of active Device 2b/C brings significant advancements. These devices are equipped with their own RF oscillators, allowing them to generate and transmit signals independently, rather than relying on a reflected carrier. This new capability makes it feasible to adopt single-sideband (1SB) modulation for D2R transmissions. By transmitting only one sideband, the device can concentrate its energy within a single, narrow frequency band, which results in enhanced spectral efficiency and reduced interference. Moreover, since all transmit power is confined to the desired sideband, filtering at the reader side becomes simpler, as there is no need to handle redundant spectral components. Therefore, adopting 1SB single-carrier modulation is the most practical and efficient choice for D2R transmission in Release-20 A-IoT systems.

Proposal 1: Support Single-side band modulation for D2R transmissions.

In Release-19, modulation schemes such as OOK, FSK, and BPSK were evaluated for D2R transmissions, and ultimately OOK and BPSK were standardized. Devices could choose either scheme for Msg1, maintaining the same modulation type for subsequent D2R transmissions. Moving to Release-20, with active Ambient IoT devices now capable of generating their own RF signals, a reassessment of these modulation techniques is essential in light of their performance, complexity, and robustness to CFO. BPSK provides strong noise resistance and higher spectral efficiency, making it well-suited for long-range outdoor coverage. However, it requires coherent detection and is sensitive to CFO, which can degrade its performance if not properly compensated. OOK, on the other hand, supports envelope detection, offering high robustness to CFO and low implementation complexity, ideal for low-power devices. Its main drawback lies in reduced spectral efficiency and potential harmonic interference, which may demand additional filtering. DBPSK improves CFO tolerance through differential detection, but its performance is generally worse than BPSK under low SNR due to noise amplification. MSK, although spectrally efficient, is hardware-intensive and highly sensitive to CFO, making it less practical for energy-constrained A-IoT devices.

Proposal 2: For D2R modulation for device 2b and device C, OOK and BPSK should be supported as a baseline.

In Release-19, the D2R transmission framework already incorporated block-level repetition to enhance reliability. For Release-20, as the deployment focus expands from controlled indoor environments to outdoor scenarios with longer coverage requirements, maintaining robust connectivity under weaker signal conditions becomes a key challenge. To ensure sufficient link margin and resilience against noise and fading over longer distances, it is essential to strengthen the transmission reliability mechanisms. One straightforward and effective enhancement is to increase the repetition factor of D2R transmissions beyond the values used in Release-19 for instance, adopting a block repetition factor greater than two. This approach directly improves decoding probability in low-SNR environments without requiring major architectural changes. Therefore, to meet the extended outdoor coverage objectives of Rel-20, adopting a higher block-level repetition factor for D2R transmissions is recommended as a practical and efficient enhancement.

Proposal 3: D2R block-level repetition with larger repetition numbers should be supported to meet the extended outdoor coverage objectives of Rel-20.

In the case of A-IoT active devices, the D2R signal is generated internally by the device’s own RF chain, without depending on any external carrier wave from the reader. Consequently, the Release-19 mechanism that determined the D2R frequency location based on the reader’s carrier reference is no longer applicable for these active devices. Therefore, the small frequency shift factor in the specification is no longer necessary for Device 2b/C. This introduces the need for a new method to define and derive the D2R transmission frequency.

Proposal 4: The small frequency shift factor is not required for Device 2b/C, as these active devices can internally generate RF signals without relying on an external carrier reference.

Release 19 supports time-domain and frequency-domain multiplexing for D2R, and TR 38.769 [3] further examines code-division multiple access (CDMA). Prior analyses suggest that CDMA-style spreading can improve resource utilization with only a modest increase in device complexity. The open questions concern its necessity and practicality given device capability limits and the cost benefit trade-off. Since Devices 2b and C are expected to be more capable than Device 1 for example, active designs can calibrate SFO/CFO more accurately. Therefore, CDMA for D2R appears viable for Device 2b/C and merits further study.

Proposal 5: Study to support CDMA for D2R transmission in devices 2b/C.

Rel-19 targets indoor backscatter Device-1, while Rel-20 adds outdoor operation with actively generated D2R for Devices 2b/C. With their higher power budgets and much tighter SFO (≈10³ ppm vs ≈10⁵ ppm for Type-1), the conservative random access (RA) of Rel-19 can be relaxed and allow more than Msg1 occasions, and permit more than 1 Msg3 occasions, these changes improve RA success probability, reduce latency, and use resources more efficiently.

Proposal 6: Study to support for increasing the number of time-domain access occasions for Msg1 for Device Types 2b/C.

Proposal 7: Study to support TDMA for Msg 3 for device 2b/C.

In RAN1#122 meeting, following agreement was made for time offsets for device 2b and device C.

Agreement At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g., - timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) - timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min) - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4 - timing offset between R2D and corresponding subsequent D2R provided by a reader - timing offset between D2R and corresponding subsequent R2D provided by a reader. At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g., - timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists.

In Rel-19, no specification was made for the timing offset between two consecutive R2D or D2R transmissions. However, in Rel-20, the need for such an offset becomes relevant, particularly for DT or DO-DTT cases where R2D control information is coded with a separate CRC from the data. In such scenarios, two PRDCHs may be transmitted consecutively, and the question arises whether a time gap between them is necessary. The necessity largely depends on whether the R2D control must be decoded prior to R2D data reception. If this is the case, a timing offset is required to account for the processing time needed to decode the control information.

In general, multiple cases exist in Rel-20 where two consecutive R2D or D2R transmissions for the same device may occur. For R2D, one example is when an R2D sync signal is followed immediately by an R2D transmission, such as R2D paging or SSB like R2D transmission. If the gap between these transmissions is too small, the device may still be processing the previous one and thus fail to decode the next. Therefore, a minimum offset between two consecutive R2D transmissions is necessary, and devices should not be expected to perform the latter reception if this condition is not met. Similarly, for D2R, consecutive transmissions may also occur when different traffic types are supported by the same device. Therefore, the study of consecutive R2D/D2R transmissions should be considered for Devices 2b and C.

Proposal 8: Support to study the timing offset between consecutive D2R transmissions.

- Conclusion

In this contribution, we discuss the potential impacts on D2R signals, channels, waveform and procedures for A-IoT device 2b/C. The proposals are summarized as follows.

Proposal 1: Support Single-side band modulation for D2R transmissions.

Proposal 2: For D2R modulation for device 2b and device C, OOK and BPSK should be supported as a baseline.

Proposal 3: D2R block-level repetition with larger repetition numbers should be supported to meet the extended outdoor coverage objectives of Rel-20.

Proposal 4: The small frequency shift factor is not required for Device 2b/C, as these active devices can internally generate RF signals without relying on an external carrier reference.

Proposal 5: Study to support CDMA for D2R transmission in devices 2b/C.

Proposal 6: Study to support for increasing the number of time-domain access occasions for Msg1 for Device Types 2b/C.

Proposal 7: Study to support TDMA for Msg 3 for device 2b/C.

Proposal 8: Support to study the timing offset between consecutive D2R transmissions.

# References

[1] RP-252964, “Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#109, September, 2025.

[2] RAN1#122-bis meeting Chairman notes.

[3] 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, V19.0.0 (2024-12)