____________________________________________________________________

3GPP TSG RAN WG1 #123			R1-2509347

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	10.6.1

Source: 	CEWiT

Title: 	Discussion on GNSS Resilient Operation for NR NTN

Document for:	Discussion and Decision

____________________________________________________________________

# Introduction

In the RAN#108 meeting, a new WI on NTN for NR Phase 4 was approved [1]. This work item introduces a set of objectives aimed at improving the robustness and reliability of NR-NTN operations, particularly in scenarios where the GNSS may be unavailable, degraded, or unreliable. The objectives of GNSS-resilient NR-NTN operations, as outlined in [1], target the development of mechanisms that allow UE and network to maintain UL synchronisation, such as TA, despite GNSS interruptions.

The detailed objectives of GNSS-resilient operation for NR-NTN include:

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

As outlined in the WID [1], this contribution focuses on the discussion of uplink synchronization mechanisms in scenarios where the GNSS becomes temporarily unavailable. The objective is to analyze potential solutions and enhancements that enable the UE to obtain UL synchronization with the network during such GNSS outages, ensuring reliable communication and minimizing service degradation. Additionally, this contribution assumes the satellite is acting as a regenerative payload.

# Previous Agreements

Work on this SI kicks started in RAN1#122 meeting and following agreements are reached in RAN1#122bis toward evaluations. Based on these agreements subsequent sections provide the further details.

Agreement: For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions: - For scenario 1 and 2 - The following time/frequency differences should be evaluated: - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area. - For UL performance evaluation 2 times differential one-way delay should be considered. - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area. - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported. Agreement: For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria: - PRACH tolerance is exceeded if - min (, Sequence duration) Differential RTT, where; - Ncp is cyclic prefix duration - max is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211 - L is the ZC sequence length - is an additional timing error. - Companies to report how Te is derived. - Differential RTT is 2 time one-way delay. - Note: the delay spread is neglected - GP can be considered and reported for the case of consecutive RO. - Or (scaling factor*(Differential Doppler) +2* fe×fc) ≥ - For restricted set A, For restricted set B, . For non-restricted set, - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively. - Othe values of can be used and reported with justification - ppm - is the uplink carrier frequency - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset. - The same scaling factor as per the previous agreement is to be reported Note: the above is applicable to single PRACH transmission. Agreement: RAN1 to assess the impacts on connected mode during NR NTN GNSS-resilient operation, including at least: - Impact on signalling overhead. Observation: For the study on GNSS resilient NR-NTN operation, the following solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty are listed based on inputs/discussions in RAN1#122bis: - Solution 1: To improve robustness: - Solutions based on modified random access procedures, using existing PRACH formats: - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats - Solution 1B: New PRACH restricted sets. - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command) - Solution 1E: Selection of a set of existing PRACH root - Solution 1F: Using TA margin and RO masking. - Other solutions are not precluded - Solution 2: Solutions based on reducing time-frequency uncertainty - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications. - Solution 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area) - Solution 2C: Solutions based on broadcasting DL timestamp(s) - Solution 2D: UE side time/frequency pre-compensation based on reference location. - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position - Solution 2F: UE side time/frequency pre-compensation based on reference signal. - Other solutions are not precluded - Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay. - Other Solutions are not precluded Agreement: In RAN1#123, based on the following alternatives representing two calculation methodologies, select a calculation methodology with potential adjustments, including correction of equations: Alt 1: For the calculation of the differential one-way delay and the differential one-way Doppler/frequency offset the following could be considered: Geometry model and equations could be considered. For the variables in the equations, refer to Figure below. For non-nadir beam For nadir beam. Note 1: UE is at same height as the reference point. Note 2: For the purpose of evaluation, fixed size of uncertainty area is used within the whole satellite coverage footprint is a baseline. Other size of uncertainty area is not precluded. FFS : Whether the minimum elevation is at farthest w.r.t satellite point of the uncertainty area or at the center of the uncertainty area FFS: How to take into account the UE altitude uncertainty. Alt 2: For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered. For the variables in the equations, refer to Figure-X. Considering satellite location at s=0,0,RE+h One-way delay between a point u and satellite: done-wayu=¦u-s¦2c One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along k=cosα,sinα,0 with speed vue: dopplerone-wayu=vcfc <u-s,x>done-wayu+vuecfc<s-u,k>done-wayu Let R be the location uncertainty area. Differential one-way delay = maxu∈R done-wayu-minu∈R done-wayu Differential one-way doppler = maxu∈R dopplerone-wayu-minu∈R dopplerone-way(u) For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ϕ from the orbital plane (x-y plane). Note: Assume satellite and UE moves parallel to the x-y plane. <⋅,⋅> is the vector inner product, ¦¦.¦​2 is the vector magnitude.

# Evaluation Methodology

During the previous meeting, two methodologies for calculating the differential one-way delay and Doppler were discussed. It was agreed that a single method should be selected for this evaluation.

We recommend Alternative 2, as it is significantly simpler and more straightforward to implement. Furthermore, Alternative 1 requires corrections to several equations. For instance, the calculations for dfar and drefnear should follow the same method, differing only in the input angle, as both are derived from the same obtuse triangle geometry shown in the figure (refer to agreement section). drefnear is a function of φ₁, which in turn depends on φ. To compute φ, the value of dfar is required. Instead, the drefnear can also be computed independently, assuming that the UE is located at a specific position and sees the satellite with a given  angle γ.

Additionally, we suggest clarifying the notation by using hnear and hfar instead of the ambiguous hue for both near and far cases, respectively. For these reasons, we propose adopting Alternative 2.

Proposal 1: The following methodology (Alt 2 from last meeting) can be used to evaluate one-way differential delay and one-way differential Doppler:

For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered. For the variables in the equations, refer to Figure-X in the agreements section.

Considering satellite location at s=0,0,RE+h

One-way delay between a point u and satellite:
done-wayu=|u-s|2c

One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along k=cosα,sinα,0 with speed vue:

dopplerone-wayu=vcfc  <u-s,x>done-wayu+vuecfc<s-u,k>done-wayu

Let R be the location uncertainty area.

Differential one-way delay = maxu∈R done-wayu-minu∈R done-wayu

Differential one-way doppler = maxu∈R dopplerone-wayu-minu∈R dopplerone-way(u)

For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ϕ from the orbital plane (x-y plane).

Note: Assume satellite and UE moves parallel to the x-y plane. <⋅,⋅> is the vector inner product, ||.|​2 is the vector magnitude.

# UL synchronisation during temporary GNSS unavailability

The purpose of TA is to ensure that uplink signals from the UE reach the gNB time-aligned, despite

the large propagation delays in NTN (LEO/MEO/GEO satellites). In absence of the GNSS based UE location to derive the UE specific TA value, following two ways can be used.

(i) Reference Point–Based TA

The gNB assumes a reference location, typically the beam or cell centre on the ground, and computes the TA based on that point. This approach is simple and incurs minimal overhead. However, UEs located near the beam edges experience additional propagation delay relative to the reference point, which leads to residual timing and frequency errors.

(ii) RAT-based Positioning (e.g., TDoA)

In this method, the TA is derived from the UE’s estimated position using RAT-dependent positioning techniques such as Time Difference of Arrival (TDoA). This allows the gNB to adjust TA for each UE with higher precision, thereby reducing timing and frequency errors across the beam. However, it requires positioning support in the network and is comparatively more complex than the reference point–based TA approach.

## 3.1 Timing Advance Determination Using Reference Point

When GNSS is unavailable,  as described in [3], additional reference point (RP2) within the beam or cell coverage area can be defined as shown in Fig. 1. TA is computed as if the UE is located at this reference point. This RP2 serves as the basis for determining a reference TA value applicable to all UEs within that beam/cell. The common TA can be computed by the network based on the known location of the RP2 and the satellite position.

Figure 1: The method to calculate the TA using the reference point in a beam/cell.

Once the reference TA is calculated, it can be signalled to the UE in SIB1 allowing the UE to apply this reference TA when transmitting its preamble. This method is simple and has less signaling overhead. But UEs at the beam edge experience extra propagation delay compared to reference point. This causes residual timing errors (difference in propagation time) and frequency errors (due to different Doppler shifts w.r.t. beam centre).

Proposal 2: The satellite should compute reference TA using its location and the geometry of the cell/beam. The reference TA is TA between the satellite and reference point on the ground (RP2).

Proposal 3: The satellite should transmit the reference TA in SIB1 for the UE’s preamble transmission.

## 3.2 Timing and Frequency Error based on Reference-Point TA Computation

The timing and frequency error is evaluated with respect to a reference point located at the centre of the beam or cell. In the absence of GNSS, the UE assumes this central point as its location. However, in reality, the UE may be positioned anywhere within the beam, which leads to deviations in both timing and frequency. These deviations, in turn, impact synchronization and can degrade the receiver’s ability to correctly decode the signals. The evaluation is carried out for three types of satellite orbits:

- LEO at 600 km altitude

- LEO at 1200 km altitude

- GSO (Geostationary Orbit) at ~35,786 km

Each of these orbital scenarios is analysed for both S-band and Ka-band and the results are shown below.

Figure 2: Timing and Frequency Error for LEO 600, S band.

Figure 3: Timing and Frequency Error for LEO 600, Ka band.

Fig. 2 and Fig. 3 illustrate the timing and frequency errors for the LEO 600 km scenario in S-band and Ka-band, respectively. The timing error is dominated by the extra propagation path when the UE is located at the beam edge compared to the reference point at the beam centre. As a result, the timing error is largest at the beam edge and gradually reduces to zero at the beam centre. In contrast, the frequency error is caused by Doppler shift, which depends on the radial velocity component of the satellite along the line of sight. Here, the frequency error is smallest at the beam edge and reaches its maximum at the beam centre where the radial velocity is the highest.

Observation 1: For Reference-Point based TA computation in LEO at 600 km with a beam radius of 25 km, the maximum timing error is 82.08 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 1.040 ppm, observed at an elevation angle of 90°.

Figure 4: Timing and Frequency Error for LEO 1200, S band.

Figure 5: Timing and Frequency Error for LEO 1200, Ka band.

Fig. 4 and Fig. 5 present the timing and frequency errors for the LEO 1200 km scenario with a beam radius of 45 km in S-band and Ka-band, respectively. Similar to the LEO 600 scenario, the timing error increases as the UE moves from the beam centre toward the edge due to the additional propagation distance compared to the reference point at the beam centre. As a result, the timing error is maximum at the beam edge and approaches zero at the beam centre. Compared to the LEO 600 km case, the timing error is larger here because of the higher satellite altitude and the wider beam radius, both of which increase the differential path length across the beam.

The frequency error, caused by Doppler shift, depends on the projection of the satellite’s velocity along the line of sight. In this case, the frequency error is smallest at the beam edge and largest at

Figure 6: Timing and Frequency Error for GSO, S band.

the beam centre. However, the overall frequency error is smaller than that of the LEO 600 km case, since the higher altitude of the satellite reduces the radial velocity component observed at the UE.

Observation 2: For Reference-Point based TA computation in LEO at 1200 km with a beam radius of 45 km, the maximum timing error is 147.75 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 0.936 ppm, observed at an elevation angle of 90°.

Figure 7: Timing and Frequency Error for GSO, Ka band.

Fig. 6 and Fig. 7 present the timing and frequency errors for the GSO scenario with a beam radius of 125 km in S-band and Ka-band, respectively. Similar to the LEO 600 and LEO 1200 scenario, the timing error is maximum at the beam edge and approaches zero at the beam centre. Compared to the LEO 600 and LEO 1200 case, the timing error is larger here because of the higher satellite altitude and the wider beam radius, both of which increase the differential path length across the beam.

The frequency error, caused by Doppler shift, depends on the projection of the satellite’s velocity along the line of sight. In this case, the frequency error is smallest at the beam edge and largest at the beam centre. However, the overall frequency error is smaller than that of the LEO 600 and LEO 1200 cases, since the higher altitude of the satellite reduces the radial velocity component observed at the UE.

Observation 3: For Reference-Point based TA computation in GSO at 35786 km with a beam radius of 125 km, the maximum timing error is 410.35 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 0.0359 ppm, observed at an elevation angle of 90°.

## UE Transmit Timing Error Limit for Satellite Access

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |

Table 1: Timing Error Limit for FR1-NTN

Table 1 presents the NTN timing error limits, as shown in TS 38.133, for the FR1 band, accounting for the propagation delays caused by the large distance between the satellite and the UE. The basic time unit is Tc =1/ (480000*40961) = 0.508 ns. Accordingly, the maximum timing error limit for uplink signals with 15 kHz SCS is 0.944 μs.

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |

Table 2: Timing Error Limit for FR2-NTN

Table 2 presents the NTN timing error limits for the FR2 band, accounting for the propagation delays caused by the large distance between the satellite and the UE. The basic time unit is Tc =1/(480000*40961) = 0.508 ns. Accordingly, the maximum timing error limit for uplink signals with 15 kHz SCS is 0.423 μs.

The maximum timing error varies across different satellite orbits: it is 82.08 μs for LEO at 600 km, 147.75 μs for LEO at 1200 km, and 410.35 μs for GSO, whereas the maximum timing error limit is 0.944 μs. These values clearly indicate that the NTN system experiences significant timing errors when TA is computed based solely on a reference point at the center of the beam or cell. This highlights the need for enhancements to compute the TA. To address this, the following section presents a method for determining TA based on TDoA technique.

Observation 4: For successful UL transmission (e.g. PUSCH) in NTN, the maximum allowable timing error is 0.944μs. This indicates that NTN scenarios cannot be accommodated by reference-location-based TA determination.

There are two types of PRACH formats: long and short PRACH formats. Among the long PRACH formats, format 1 can accommodate timing errors of up to approximately 340 μs, which is sufficient to support most NTN scenarios. In contrast, the short preamble formats in the current NR specifications cannot handle the larger propagation delays typical in NTN, and therefore PRACH detection may fail in these cases. To enable reliable access in the absence of GNSS, additional techniques are required to reduce the UE’s position errors and ensure successful PRACH detection.

Observation 5: For Reference-Point based TA Computation, detection of short PRACH formats may not be successful in NTN scenarios. Except for long PRACH format 1, other long PRACH formats (0,2, and 3) are unlikely to be detected reliably, as their timing error limits are too low to accommodate the larger delays typical of NTN.

## Timing Advance Determination Using TDoA Estimation using SSB

When GNSS is temporarily unavailable, the UE requires assistance from the satellite network to maintain UL synchronization. In such situations, UE is configured to detect multiple SSBs from multiple satellites. This approach leverages the multilateration technique (sometimes referred to as triangulation), where two or more satellites transmit SSB signals to the UE, as illustrated in Fig. 1. The UE measures the Time of Arrival (ToA) of the SSB from each satellite and then computes the Time Difference of Arrival (TDoA) between pairs of satellites. From the TDoA, the UE derives the difference in propagation delay between the two satellites. The set of all possible UE positions that produce the same delay difference forms a hyperbola (in 2D) or a hyperboloid (in 3D). When more than two satellites are involved, the number of hyperbolas increases, and their intersection point corresponds to the UE’s location. Once the UE determines its position, it can estimate the Timing Advance (TA) needed for UL transmission and apply it to the PRACH preamble to ensure proper alignment at the satellite receiver. The locations and velocities of the satellites, required for this computation, are broadcast to the UE in SIB19 through ephemeris data. While the UE’s internal clock can introduce errors in ToF-based ranging, the TDoA method inherently cancels out common clock offsets, making it more robust to such inaccuracies. All the TDoA and ToA estimation at the UE is implementation specific, and it will not use positioning framework of NR. Proposals here is to enable UE to use SSB to estimate TDoA and ToA for its positioning using SIB19 information.

Figure 2: This figure illustrates the method to calculate the TA for the UL transmission of a UE without GNSS assistance in NTN systems. This illustrates the TA method based on multilateralization where multiple NTN-satellite will participate to determine the TA.

The TDoA between two satellites is first calculated. Using measurements from multiple satellites, the UE’s location is determined, from which the estimated TA is derived.

Tarrive1-Tarrive2=d1-d2c

d1-d2=ddiff

ddiff=xue-x12+yue-y12+zue-z12-xue-x22+yue-y22+zue-z22

Proposal 4: The UE should be able to estimate the location using SSB and SIB19 when GNSS temporarily unavailable. UE should estimate TDoA using SSB from multiple satellites to estimate its location. UE specific timing advance estimated using UE location and the ephemeris information from the SIB19.   The TDoA estimation at the UE is implementionation specific, and it will not use positioning framework of NR.

These equations can be solved simultaneously to get the UE location xue,yue,zue . Here minimum four equations are necessary as there are four unknowns in the set of equations. xi,yi,zi is the location of the ith satellite. Once the estimated TA (Test) is calculated from each satellite it can be scaled by two to get the timing advance.

Proposal 5: The UE should employ multilateration technique to determine its location based on measurements from multiple satellites. The determined location is used to calculate the corresponding TA.

To avoid the over estimation, the estimated TA shall then be quantized using the predefined step size (TAstep). This TAstep shall be set to at least one of the following:

- The cyclic prefix (CP) duration of the physical random-access preamble configured for the UE

- One of the possible CP durations corresponding to the frequency band of operation

- Any other specific value explicitly signalled by the NTN-gNB to the UE.

To get the total number of steps in multiples of TAstep., the UE divides the estimated TA with the TAstepand floors it to the lower integer.   The final value of TA,  TAapplied, will be the number of steps multiplied with the step size. To reduce the over estimation error further, TA margin subtracted from the applied TA. This final value of TA is to be applied by UE for its UL transmission including PRACH transmission. This is formulated by below equations

TAapplied=⌊TAestTstep⌋*Tstep-TAmargin

TAapplied=⌊TAestTcprach⌋*Tcprach-TAmargin

Figure 3: The method to calculate the TA for the UL transmission of a UE in NTN systems in presence of overestimation error.

Proposal 6: In order to avoid TA overestimation, the UE should quantize the estimated TA using the predefined step size and subsequently subtract the TA margin from the quantized value.

In cases where TAstep is not transmitted, or the estimated TA is already sufficiently accurate, the UE may directly determine the applied TA as follows:

TAapplied=TAest-TAmargin

Proposal 7: If the step size is not signalled by the satellite, the UE should subtract the TA margin from the estimated TA.

## Closed loop TA Correction

After this initial alignment, any remaining fine TA adjustments required to achieve precise uplink synchronization can be provided to the UE through the RAR message. This approach enables robust initial access even in the absence of GNSS, while minimizing the risk of preamble misalignment due to excessive propagation delay.

Proposal 8: The satellite should provide TA adjustments required to achieve precise uplink synchronization to the UE through the RAR message.

The location estimated by the UE using TDoA may not always be accurate, which results in errors in the Timing Advance (TA). Consequently, the correction to TA can be either positive or negative, since the TA computed from TDoA may be larger or smaller than the actual value. However, the current specification only supports positive TA values and does not account for negative corrections. Therefore, the TA mechanism needs to be enhanced to support negative values as well, enabling more accurate timing alignment.

Proposal 9: Enhancement of Timing Advance should be considered to support negative corrections in the RAR message.

## Conclusion

This contribution includes the estimation of timing advance through TDoA to transmit the preamble.

Proposal 1: The following methodology (Alt 2 from last meeting) can be used to evaluate one-way differential delay and one-way differential Doppler:

For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered. For the variables in the equations, refer to Figure-X in the agreements section.

Considering satellite location at s=0,0,RE+h

One-way delay between a point u and satellite:
done-wayu=|u-s|2c

One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along k=cosα,sinα,0 with speed vue:

dopplerone-wayu=vcfc  <u-s,x>done-wayu+vuecfc<s-u,k>done-wayu

Let R be the location uncertainty area.

Differential one-way delay = maxu∈R done-wayu-minu∈R done-wayu

Differential one-way doppler = maxu∈R dopplerone-wayu-minu∈R dopplerone-way(u)

For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ϕ from the orbital plane (x-y plane).

Note: Assume satellite and UE moves parallel to the x-y plane. <⋅,⋅> is the vector inner product, ||.|​2 is the vector magnitude.

Proposal 2: The satellite should compute reference TA using its location and the geometry of the cell/beam. The reference TA is TA between the satellite and reference point on the ground (RP2).

Proposal 3: The satellite should transmit the reference TA in SIB1 for the UE’s preamble transmission.

Observation 1: For Reference-Point based TA computation in LEO at 600 km with a beam radius of 25 km, the maximum timing error is 82.08 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 1.040 ppm, observed at an elevation angle of 90°.

Observation 2: For Reference-Point based TA computation in LEO at 1200 km with a beam radius of 45 km, the maximum timing error is 147.75 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 0.936 ppm, observed at an elevation angle of 90°.

Observation 3: For Reference-Point based TA computation in GSO at 35786 km with a beam radius of 125 km, the maximum timing error is 410.35 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 0.087 ppm, observed at an elevation angle of 90°.

Observation 4: For successful UL transmission (e.g. PUSCH) in NTN, the maximum allowable timing error is 0.944μs. This indicates that NTN scenarios cannot be accommodated by reference-location-based TA determination.

Observation 5: For Reference-Point based TA Computation, detection of short PRACH formats may not be successful in NTN scenarios. Except for long PRACH format 1, other long PRACH formats (0,2, and 3) are unlikely to be detected reliably, as their timing error limits are too low to accommodate the larger delays typical of NTN.

Proposal 4: The UE should be able to estimate the location using SSB and SIB19 when GNSS temporarily unavailable. UE should estimate TDoA using SSB from multiple satellites to estimate its location. UE specific timing advance estimated using UE location and the ephemeris information from the SIB19.   The TDoA estimation at the UE is implementation specific, and it will not use positioning framework of NR.

Proposal 5: The UE should employ multilateration techniques to determine its location based on measurements from multiple satellites. The determined location is used to calculate the corresponding TA.

Proposal 6: In order to avoid TA overestimation, the UE should quantize the estimated TA using the predefined step size and subsequently subtract the TA margin from the quantized value.

Proposal 7: If the step size is not signalled by the satellite, the UE should subtract the TA margin from the estimated TA.

Proposal 8: The satellite should provide TA adjustments required to achieve precise uplink synchronization to the UE through the RAR message.

Proposal 9: Enhancement of Timing Advance should be considered to support negative corrections in the RAR message

## References

  - RP-251863, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 4,” RAN#108, June 2025.

  - 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)".

  - 3GPP TR 38.821: “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”.