3GPP TSG RAN WG1 #123	R1-2509366

Dallas, 17 – 21 November 2025

Agenda item:		11.1

Source:	Rakuten

Title:	Discussion on the Overview of 6GR Air Interface

Document for:		Discussion and Decision

# Introduction

The SID of Study on 6G Radio in RAN #108 includes the following objectives [1]:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In this report, we share our views and studies on the air interface for 6GR.

# Discussion

## Scalable Design for Diverse 6GR Device Types

RAN1 #122 meeting [2] reached the following agreements

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

which outlined the direction for a scalable design that is starting from the principle of the existence of functionalities that are commonly applicable for all 6G device types. During RAN1 #112bis meeting [3], further details on the study of functionalities have been discussed and the companies moving forward to an agreement for further detailing the functionalities commonly applicable for all 6G device types. The proposal in FL summary of remaining issues [4] is quoted as follows:

Proposal 3.1b: - For scalable 6GR design for diverse device types, RAN1 to consider - Striving for functionality designs that can be commonly applied to all 6G device types - The functionalities commonly applicable to all 6G device types include, but not limited to - Basic PHY features, such as waveform, coding, frame structure, single numerology per band - Basic idle mode procedures from RAN1 perspective - Basic initial access procedures - Basic DL/UL control, scheduling/HARQ - MRSS - [Coverage enhancements] - [Energy saving] - Note: adjustment on the design is allowed for a certain device type

We would like to give our views and comments to improve the proposal.

We agree that the principle of “striving for functional designs” should guide our actions toward the goal of being “commonly applicable across all 6G device types.” However, this point serves more as directional guidance than as a concrete, achievable target - something essential for any study. Therefore, we also aim for the study to deliver a comprehensive list of functionalities.

While we recognize that basic PHY features are fundamental to defining functionalities, we recommend avoiding specific examples, such as waveform, coding, or frame structure, as these may create the impression that the list is exhaustive or unintentionally diminish the importance of other features not mentioned.

The phrase “Basic idle mode procedure” does not fully capture the scope of idle mode functionalities. We recommend revising it to: “PHY foundation for idle mode procedures, including measurement and PHY resources for paging.” This wording provides greater clarity and emphasizes the essential components involved.

While we agree that functionalities related to initial access procedures are important, we believe the current wording lacks sufficient detail. We recommend revising it to: “PHY foundation for initial access procedures, including basic preamble structure, PHY resources for preamble, preamble power control, and RAR window.” This phrasing provides clarity and ensures all critical elements are explicitly addressed.

We recommend retaining “Basic DL/UL control” as a set of functionalities enabling the UE to receive and transmit PHY control data, without linking this aspect to scheduling or HARQ. In our view, scheduling and HARQ are not essential components of commonality and should remain separate.

MRSS functionalities are generally transparent to UEs and not universally applicable across all device types. Therefore, we recommend addressing them in a separate study.

We are not in a position to support the inclusion of coverage enhancements within the study scope. The primary objective of this study is to define common PHY functionalities applicable across all 6G device types. Coverage enhancements, while important in specific deployment scenarios, are highly implementation-dependent and vary significantly by use case, spectrum, and network architecture. Including them would dilute the focus on establishing a universally applicable baseline and introduce complexity that is outside the intended scope of commonality.

We are open to including the “energy saving” aspect and welcome discussion on refining its wording to make it clearer and more impactful.

Based on the above discussions, we propose the following:

Proposal 1: Revise Proposal 3.1b in FL summary [4] as follows:

- For scalable 6GR design for diverse device types, RAN1 to consider - Striving for functionality designs that can be commonly applied to all 6G device types - Aiming to establish a comprehensive list of functionalities commonly applicable to all 6G device types. - The functionalities commonly applicable to all 6G device types include, but not limited to - Basic PHY features - PHY foundation for idle mode procedures, including idle measurement and PHY resources for paging - PHY foundation for initial access procedures, including basic preamble structure, PHY resources for preamble, preamble power control, and RAR window - Basic DL/UL control - Energy Saving - Note: adjustment on the design is allowed for a certain device type

## Smallest Maximum Supported RF and BB UE BW without Spectrum Aggregation

RAN1 #122 meeting [2] reached the following agreement

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

which provided options and directions to study the smallest maximum supported RF and baseband UE bandwidth without spectrum aggregation. During RAN1 #112bis meeting [3], additional details regarding the aspects to be considered in the study were discussed, with some issues remaining open.. The proposal outlined in the FL summary of remaining issues [4] is quoted as follows:

Proposal 4.1b: - For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least - Overall device complexity - Note: also taking into account other complexity reduction techniques than BW reduction - Overall system performance impact - Energy efficiency for both BS and UE - Minimum spectrum allocation - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority - Note: other aspects (e.g. economies of scale) can be considered by TSG RAN when they make decision on the BW

We support the proposal in principle and emphasize that “minimum spectrum allocation” is critical when studying of the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework. Accordingly, we provide our perspective in the following proposal:

Proposal 2: Support Proposal 4.1b in FL summary [4].

## Approach for Design of the Common Signals/Channels for Initial Access as Dependency on Minimum Spectrum Allocation

RAN1 #122 meeting [2] reached the following agreement

Agreement - Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

which provided a guidance for the study of sync signal structure and associated periodicity. In general, sync signals design plays is crucial for initial access. The configuration of the common signals/channels for initial access is closely tied to the minimum spectrum allocation, another open topic for study. During RAN1 #112bis meeting [3], several approaches for designing these common signals/channels were discussed, as reflected in the proposal summarized in [4]:

Proposal 4.1b: - RAN1 to consider following to operate 6GR on the minimum spectrum allocation - Opt1: common signals/channels BW for initial access are punctured to fit into the minimum spectrum allocation, if the minimum spectrum allocation is smaller than the common signals/channels BW for initial access (if this case is supported) - Opt2: Scalable design of the common signals/channels for initial access for the minimum spectrum allocation from other spectrum allocations , if the minimum spectrum allocation is smaller than the common signals/channels BW for initial access for other spectrum allocations - Opt3: A single design of the common signals/channels for initial access which is applicable to any spectrum allocations

We support the proposal and favour Option 3, as we believe it offers the lowest uncertainty in achieving a solution with predictable performance. Accordingly, we present our position in the following proposal:

Proposal 3: Support Proposal 4.1b in FL summary [4] with a preference for selecting Opt3.

## Aspects Impacting on the NR-6GR MRSS Support

RAN1 #122 meeting [2] reached the following agreement

Agreement - Identify the high-level aspects which impact on the NR-6GR MRSS support - Including the lessons learned from LTE-NR DSS

which provided a guidance to study aspects impacting on the NR-6GR MRSS support. During RAN1 #112bis meeting [3], additional details on aspects to be considered in the study were discussed, with some issues remaining unresolved. The proposal outlined in the FL summary of remaining issues [4] is quoted as follows:

Proposal 6.2b: - High-level aspects to consider for NR-6GR MRSS include, but not limited to - UE/NW implementation complexity - Resource allocation coordination between NR-6GR - Including whether NR and 6GR TRP are always co-located or not - Radio resource utilization - Signalling overhead - Operating bands at least existing FR1 - Alignment in time/frequency resource (e.g., numerology, RB, slot, symbol, UL/DL direction in TDD operation) - Reliance on availability of specific NR functionalities Note: Focus on existing NR deployments (NW and UE)

While we agree with certain aspects of the proposal, we believe revisions are necessary. Specifically, ‘signalling overhead’ should be excluded. NR does not feature persistent signals or channels, except for SSB, which uses a relatively small portion of bandwidth. 6GR network can configure resources for other signals and channels to avoid overlapping with the resources for NR and no signalling to 6G UEs is required. MRSS can be designed to function transparently for 6GR UEs with minimal signalling overhead and, hence, preventing interference caused by NR.

The aspect "Reliance on availability of specific NR functionalities" requires reconsideration. Since we are not aware of any scenario where the absence of NR functionalities would cause MRSS to fail, we believe this aspect should be excluded from the proposal.

Based on the above discussions, we propose the following:

Proposal 4: Revise Proposal 6.2b in FL summary [4] as follows:

- High-level aspects to consider for NR-6GR MRSS include, but not limited to - UE/NW implementation complexity - Resource allocation coordination between NR-6GR - Including whether NR and 6GR TRP are always co-located or not - Radio resource utilization - Operating bands at least existing FR1 - Alignment in time/frequency resource (e.g., numerology, RB, slot, symbol, UL/DL direction in TDD operation) Note: Focus on existing NR deployments (NW and UE)

## Lessons Learned from NR BWP Framework

According to the following agreement in RAN1 #122 meetings [2],

Agreement Study and identify the lessons learned from NR BWP framework.

RAN1 has agreed to initiate a study focused on identifying the lessons learned from NR BWP framework. During RAN1 #112bis meeting [3], several key lessons were identified and captured in a proposed observation within the FL summary of remaining issues [4], quoted bellow:

Proposed observation 8.1c: - The lessons learned from NR BWP framework include, but not limited to - excessive BWP-specific BB/RF configuration parameters, which leads to UE long BWP switch latency - SCS switching under BWP framework is complicated - Some scenarios (e.g. non-overlapped BWPs) where DCI-based BWP switching can have reliability issue - lack of early RAN4 involvement, which caused sub-optimal design

While we agree with the points listed in the proposed observation, we believe the wording should be revised for clarity and accuracy. Specially, ‘SCS switching under BWP framework is complicated’ should be rephrased as ‘complication of SCS switching under BWP framework’ and ‘Some scenarios (e.g. non-overlapped BWPs) where DCI-based BWP switching can have reliability issue’ should be revised to ‘potential reliability issues associated with DCI-based BWP switching in some scenarios’.

In addition, we view that BWP switching for a UE should serve to adapt to traffic demand and the radio conditions. However, the absence of BWP-level QoS association in the NR BWP framework limits the practical use of BWP switching in implementation.

Based on the above discussions, we propose the following:

Proposal 5: Revise Observation 8.1c in FL summary [4] as follows:

- The lessons learned from NR BWP framework include, but not limited to - excessive BWP-specific BB/RF configuration parameters, which leads to UE long BWP switch latency - complication of SCS switching under BWP framework - potential reliability issues associated with DCI-based BWP switching in some scenarios - lack of early RAN4 involvement, which caused sub-optimal design - absence of BWP-level QoS association that limits the effective use of BWP switching.

## Lessons Learned from NR Spectrum Utilization and Aggregation Framework

According to the following agreement in RAN1 #122 meetings [2],

Agreement Study and identify the lessons learned from NR spectrum utilization and aggregation framework. - DC is subject to RANP decision in June 2026. - Note: MRSS aspects are separate discussion.

RAN1 has agreed to initiate a study focusing on identifying the lessons learned from NR spectrum utilization and aggregation framework. During RAN1 #112bis meeting [3] several lessons were identified and documented in a proposed observation within the FL summary of remaining issues [4], quoted bellow:

Proposed observation 9.1b: - The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to - CA has been a beneficial feature in previous generations - Not all functionalities are available from initial release - Operating scenarios of CA and DC have some overlap - Some functionalities are supported only on Pcell - Slow and complex activation of additional carrier - Inefficiency from coupling DL and UL carriers for a cell - Utilizing fragmented spectrum is not considered well - Features (such as HARQ) defined per carrier leads to sub-optimal performance - Signalling/configuration overhead and UE processing complexity of PHY channels due to per CC constraint - limited applicable scenario of SSB adaptation for Scell - Late introduction of UL TX switching leads to restricted applicability/performance

The proposed observation includes several valid points regarding lessons learned from the NR spectrum utilization and aggregation framework. However, we recommend revising the wording as follows: “Some functionalities are supported only on Pcell” should be rephrased as “Inefficiency due to unavailability of some functionalities in SCell(s)”. In addition, since the phrase “Inefficiency from coupling DL and UL carriers for a cell” is unclear, we suggest omitting it from the observation

Based on the above discussions, we propose the following:

Proposal 6: Revise Observation 9.1b in FL summary [4] as follows:

- The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to - CA has been a beneficial feature in previous generations - Not all functionalities are available from initial release - Operating scenarios of CA and DC have some overlap - Inefficiency due to unavailability of some functionalities in SCell(s) - Slow and complex activation of additional carrier - Utilizing fragmented spectrum is not considered well - Features (such as HARQ) defined per carrier leads to sub-optimal performance - Signalling/configuration overhead and UE processing complexity of PHY channels due to per CC constraint - limited applicable scenario of SSB adaptation for Scell - Late introduction of UL TX switching leads to restricted applicability/performance

## Dynamic Service Redistribution

While 5G embraces the principle of continuous improvement, i.e., continuously adding new features to enhance the technologies, 6GR must go further by enabling continuous deployment. Traditionally, networks have not waited for full installation of all nodes and transmission/reception points (TRPs) before becoming operational. Instead, they expand incrementally, either to cover broader areas or to increase density in phases. This incremental deployment model becomes even more critical in 6G.

To support this, the network architecture and air interface should be designed for a plug-and-play model, allowing new nodes or TRPs to be added seamlessly. These additions should immediately contribute to service delivery, enhancing user experience while minimizing disruption to existing infrastructure.

Additionally, network traffic fluctuates over time and across locations. This variability presents opportunities for energy savings and intelligent resource management. The air interface should facilitate dynamic reallocation of wireless services across locations, adapting efficiently to shifting demand, ideally without requiring additional hardware and while maintaining energy efficiency. Based on this discussion, we propose the following:

Proposal 7: Strive to design 6GR with high deployment flexibility from PHY perspective to meet two key objectives:

- Seamless integration of new nodes without disrupting ongoing operations.

- Dynamic, software-driven service migration across locations over time.

## Enabling Technologies for Enhanced Spectral Efficiency and Low Latency

The UE traffic is often diverse, with varying quality-of-service requirements. While not all UEs in the network demand the same performance, certain use cases, such as FWA or autonomous vehicles require extremely high data rates and/or ultra-low latency. To meet these demands, technologies that aggregate radio resources have been introduced since 4G, including carrier aggregation (CA), alongside enhancements like increased antenna elements and wider operating bandwidths. From a spatial perspective, resource aggregation can also be achieved through distributed TRPs, enabling more flexible transmission models.

Since Release 16 of NR, the concept of transmitting different layers from separate TRPs over the same frequency has been explored and standardized under the non-coherent joint transmission (NCJT) framework. However, the traditional cellular architecture, based on discrete cells, limits the optimal selection of TRP groups for serving a single UE. We propose that 6G should eliminate these architectural constraints, enabling full flexibility in NCJT. This vision will be significantly advanced by the capabilities introduced with 6GR air interface, as outlined in our forthcoming proposal.

Proposal 8: Strive to design 6GR to enable full flexibility in non-coherent joint transmission (NCJT) and non-coherent joint reception (NCJR) to enable optimized selection of serving TRPs from PHY perspective to enhance high data rate and low latency services for targeted subsets of UEs within the network.

## Enabling Technologies for Enhanced and Uniform Network Coverage

The traditional philosophy of cellular networks, comprising multiple fixed-location nodes, is to provide wireless coverage over a defined geographical area. As networks have evolved through successive generations, the focus has expanded beyond mere coverage to include cell throughput, typically measured in Mbps per square kilometre, for both downlink and uplink. However, coverage and throughput do not always align. Techniques like frequency reuse can degrade performance for cell-edge UEs. Moreover, due to geographical constraints, some UEs may be located in areas where connecting to a single node is inherently difficult, e.g. indoor UEs served by a network of outdoor TRPs. In such cases, the lack of coordination among nodes in current 5G architectures can limit opportunities for improving coverage.

To address this, coordinated techniques such as coherent joint transmission (CJT) and coherent joint reception (CJR) offer promising solutions. These methods require tight synchronization in time, frequency, and phase across multiple nodes. Therefore, we propose the following design principle for 6G:

Proposal 9: Strive to design 6GR to support high-precision synchronization between TRPs from PHY perspective to enhance signal strength in coverage-challenged areas (dead zones), thereby significantly improving overall network coverage.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:

Proposal 1: Revise Proposal 3.1b in FL summary [4] as follows:

- For scalable 6GR design for diverse device types, RAN1 to consider - Striving for functionality designs that can be commonly applied to all 6G device types - Aiming to establish a comprehensive list of functionalities commonly applicable to all 6G device types. - The functionalities commonly applicable to all 6G device types include, but not limited to - Basic PHY features - PHY foundation for idle mode procedures, including idle measurement and PHY resources for paging - PHY foundation for initial access procedures, including basic preamble structure, PHY resources for preamble, preamble power control, and RAR window - Basic DL/UL control - Energy Saving - Note: adjustment on the design is allowed for a certain device type

Proposal 2: Support Proposal 4.1b in FL summary [4].

Proposal 3: Support Proposal 4.1b in FL summary [4] with a preference for selecting Opt3.

Proposal 4: Revise Proposal 6.2b in FL summary [4] as follows:

- High-level aspects to consider for NR-6GR MRSS include, but not limited to - UE/NW implementation complexity - Resource allocation coordination between NR-6GR - Including whether NR and 6GR TRP are always co-located or not - Radio resource utilization - Operating bands at least existing FR1 - Alignment in time/frequency resource (e.g., numerology, RB, slot, symbol, UL/DL direction in TDD operation) Note: Focus on existing NR deployments (NW and UE)

Proposal 5: Revise Observation 8.1c in FL summary [4] as follows:

- The lessons learned from NR BWP framework include, but not limited to - excessive BWP-specific BB/RF configuration parameters, which leads to UE long BWP switch latency - complication of SCS switching under BWP framework - potential reliability issues associated with DCI-based BWP switching in some scenarios - lack of early RAN4 involvement, which caused sub-optimal design - absence of BWP-level QoS association that limits the effective use of BWP switching.

Proposal 6: Revise Observation 9.1b in FL summary [4] as follows:

- The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to - CA has been a beneficial feature in previous generations - Not all functionalities are available from initial release - Operating scenarios of CA and DC have some overlap - Inefficiency due to unavailability of some functionalities in SCell(s) - Slow and complex activation of additional carrier - Utilizing fragmented spectrum is not considered well - Features (such as HARQ) defined per carrier leads to sub-optimal performance - Signalling/configuration overhead and UE processing complexity of PHY channels due to per CC constraint - limited applicable scenario of SSB adaptation for Scell - Late introduction of UL TX switching leads to restricted applicability/performance.

Proposal 7: Strive to design 6GR with high deployment flexibility from PHY perspective to meet two key objectives:

- Seamless integration of new nodes without disrupting ongoing operations.

- Dynamic, software-driven service migration across locations over time.

Proposal 8: Strive to design 6GR to enable full flexibility in non-coherent joint transmission (NCJT) and non-coherent joint reception (NCJR) to enable optimized selection of serving TRPs from PHY perspective to enhance high data rate and low latency services for targeted subsets of UEs within the network.

Proposal 9: Strive to design 6GR to support high-precision synchronization between TRPs from PHY perspective to enhance signal strength in coverage-challenged areas (dead zones), thereby significantly improving overall network coverage.

# References

| [1] | RP-251881, “New SID: Study on 6G Radio,” NTT Docomo. |
| --- | --- |
| [2] | RAN1 #122, “RAN1 Chair's Notes,” August 2025. |
| [3] | RAN1 #122bis, “RAN1 Chair's Notes,” October 2025. |
| [4] | “Remaining issues on AI11.1 - Overview of 6GR Air Interface,” NTT Docomo, October 2025. |