3GPP TSG RAN WG1 #123			R1-2509446

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.6

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session Notes of AI 10.6

Document for:	Discussion, Decision

## Non-Terrestrial Networks (NTN) for NR Phase 4

Please refer to RP-251933 for detailed scope of the SI for NR-NTN Phase 4.

[123-R20-NR-NTN] Email discussion on Rel-20 NR-NTN – Mohamed (Thales)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2509446	Session Notes of AI 10.6	Ad-Hoc Chair (Ericsson)

R1-2508470	Work plan for NR NTN Phase 4	Rapporteur (Thales)

R1-2508479	TR 38.742 skeleton for study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation	Rapporteur (Thales)

### NR-NTN GNSS resilience

R1-2508333	Discussion on Rel-20 GNSS resilient NR NTN operation	FUTUREWEI

R1-2508360	PRACH with GNSS unavailable	Eutelsat Group

R1-2508385	Discussion on NR NTN GNSS resilience	Spreadtrum, UNISOC

R1-2508429	Discussion on NR-NTN GNSS resilience	vivo

R1-2508452	Discussion on NR-NTN GNSS resilience	CMCC

R1-2508465	Considerations on NR-NTN Resilience to GNSS Unavailability and Degradation	THALES

R1-2508481	Considerations for GNSS-resilient NR-NTN operation 	ST Engineering iDirect

R1-2508489	GNSS resilience for NR-NTN	Huawei, HiSilicon

R1-2508555	Discussion on NR-NTN GNSS resilience	NEC

R1-2508594	Discussion on NR-NTN GNSS resilience	CATT

R1-2509456	Discussion on NR-NTN GNSS resilience	CATT

(Revision of R1-2508594)

R1-2508626	Considerations for GNSS-resilient NR-NTN operation	ESA

R1-2508681	Discussion on NR-NTN GNSS resilience	Xiaomi

R1-2508724	Discussion on NR-NTN GNSS resilience	OPPO

R1-2508754	Discussion on NR-NTN GNSS resilience	Fraunhofer IIS, Fraunhofer HHI

R1-2508760	Discussion on GNSS Resilient NR-NTN Operation	Tejas Network Limited

R1-2508799	Discussion on NR-NTN GNSS resilience	Samsung

R1-2508843	On NR-NTN GNSS resilience	Ericsson

R1-2508854	Discussion on NR-NTN GNSS resilience	ZTE Corporation, Sanechips

R1-2508872	Discussion on NR-NTN GNSS resilience	Airbus

R1-2508875	Discussion on NR NTN GNSS Resilience	Amazon Web Services

R1-2508889	PRACH transmission and detection in NR NTN GNSS resilient operation	Sharp

R1-2508970	Discussion on NR-NTN GNSS resilient operations	ETRI

R1-2508984	Discussion on the GNSS resilient NR-NTN operation	TCL

R1-2508992	Discussion on NR-NTN GNSS resilience	HONOR

R1-2509007	Discussion on GNSS resilient operation for NR over NTN	Nokia

R1-2509016	NR-NTN GNSS resilience	InterDigital, Inc.

R1-2509017	Discussion on NR-NTN GNSS resilience	IMU

R1-2509018	Discussion on GNSS resilient NR-NTN operation	Lenovo

R1-2509071	On NR-NTN GNSS resilience approaches	Sony

R1-2509107	Discussion on NR-NTN GNSS resilience	Apple

R1-2509131	Discussion on NR-NTN GNSS resilience	Ofinno

R1-2509137	Discussion on GNSS resilient NR-NTN operation	TOYOTA Info Technology Center

R1-2509163	GNSS resilient operations in NR NTN	MediaTek Inc.

R1-2509178	Discussion on NR-NTN GNSS resilient operation using synchronization signals	ViaSat Satellite Holdings Ltd, Inmarsat

R1-2509185	Discussion on NR-NTN GNSS resilience	China Telecom

R1-2509228	NR-NTN GNSS resilience	Qualcomm Incorporated

R1-2509498	NR-NTN GNSS resilience	Qualcomm Incorporated

(Revision of R1-2509228)

R1-2509277	Discussion on NR-NTN GNSS resilience	NTT DOCOMO, INC.

R1-2509296	Discussion on GNSS resilient NR-NTN	Panasonic

R1-2509298	Discussion on NR-NTN GNSS resilience	Google Korea LLC

R1-2509301	Discussion of NR-NTN GNSS Resilience	Johns Hopkins University APL

R1-2509304	Discussion on GNSS resilient NR-NTN operation	LG Electronics

R1-2509336	Discussion on GNSS resilience for NR-NTN	CSCN

R1-2509347	Discussion on GNSS Resilient Operation for NR NTN	CEWiT

R1-2509434	Considerations on timing error budget under GNSS resilience in NR-NTN	ISSDU

R1-2509476	Compilation of company proposals on NR-NTN GNSS resilience	Moderator (THALES)

R1-2508466	FL Summary #1: Study on GNSS resilient NR-NTN operation		Moderator (THALES)

Agreement:

For the calculation of one-way differential delay and one-way differential Doppler, adopt Alt2 in RAN1#112bis agreement with the following correction on the one-way Doppler:

Note: Earth’s rotational motion is neglected for LEO.

Agreement:

The inequation for the limit on differential roundtrip delay in the RAN#122bis Agreement (copied below), is not applicable for unrestricted set.

For unrestricted set PRACH tolerance is exceeded if

min (, Sequence duration)   Differential RTT.

Agreement (RAN1#122bis):

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (, Sequence duration)   Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max  is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) ≥

    - For restricted set A,  For restricted set B, . For non-restricted set,

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Othe values of  can be used and reported with justification

    - ppm

    - is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

Agreement:

For PRACH performance evaluation,  used for the limit on differential roundtrip delay determination is to be set according to TS 38.133, Table 7.1.2-1.

Working assumption:

For UL performance evaluation, UL differential Doppler/frequency offset is calculated based on scaled DL one-way differential Doppler/frequency offset with the following scaling factor:

Scaling factor = 2* /

and   are the uplink and downlink carrier frequency respectively.

Agreement

For the study of GNSS resilient operation, for evaluation purposes, it is assumed that the UE pre-compensates RTT and Doppler on the service link for UL transmission. This may include, but not limited to:

- At least for Scenario 1, based on assistance information provided by the Network.

- For Scenario 2, based on at least the last known UE location.

R1-2508467	FL Summary #2: Study on GNSS resilient NR-NTN operation		Moderator (THALES)

Working assumption

Capture in the TR 38.742 the following tables of differential one-way delay reflecting results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1: within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.3 |  |  | 357.4 |
|  | 30 | 143.3 | 257.8 |  | 57.6 | 115.1 |  |
|  | 90 | 1.9 | 3.3 | 4.8 | 0.3 | 0.7 | 0.9 |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1451.9 |  |  | 906.6 |
|  | 30 | 255.3 | 535.3 |  | 143.1 | 257.1 |  |
|  | 90 | 6.1 | 14.9 | 15.5 | 1.9 | 3.3 | 6.0 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.2 |  |  | 357.4 |
|  | 30 | 142.9 | 257.7 |  | 57.5 | 114.9 |  |
|  | 90 | 1.9 | 3.3 |  | 0.3 | 0.7 |  |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1451.8 |  |  | 906.6 |
|  | 30 | 255.3 | 535.2 |  | 143.0 | 257.0 |  |
|  | 90 | 6.1 | 14.9 |  | 1.9 | 3.3 |  |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.515 | 32.554 | 65.106 | 162.779 |
|  | 30 | 5.8 | 28.8 | 57.6 | 143.2 | 5.8 | 28.8 | 57.646 | 143.603 |  |  |  |  |
|  | 90 | 0.0 | 0.1 | 0.3 | 1.8 | 0.0 | 0.0 | 0.1 | 0.9 | 0.0 | 0.0 | 0.0 | 0.2 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.5 | 32.5 | 65.1 | 162.8 |
|  | 30 | 5.8 | 28.8 | 57.5 | 142.9 | 5.8 | 28.8 | 57.6 | 143.3 |  |  |  |  |
|  | 90 | 0.003 | 0.1 | 0.3 | 1.9 | 0.0 | 0.0 | 0.2 | 1.0 | 0.0 | 0.0 | 0.0 | 0.2 |

Note: For the above results, UE altitude of 0 km is assumed.

Note: These values do not take into account UE altitude uncertainty.

Note: For the above results for Set1/2 parameters, it is assumed that the area served by the cell or beam (corresponding to the nadir beam size as defined in 38.821) is fixed within the satellite coverage.

Working assumption

Capture in the TR 38.742 the following tables of differential one-way Doppler reflecting results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1: within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.011 |  |  | 0.005 |
|  | 30 | 0.371 | 0.390 |  | 0.146 | 0.168 |  |
|  | 90 | 2.112 | 1.843 | 0.046 | 0.848 | 0.767 | 0.020 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.023 |  |  | 0.012 |
|  | 30 | 0.754 | 0.921 |  | 0.410 | 0.425 |  |
|  | 90 | 3.790 | 3.861 | 0.102 | 2.118 | 1.843 | 0.052 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.041 |  |  | 0.024 |
|  | 30 | 1.212 | 1.125 |  | 0.497 | 0.514 |  |
|  | 90 | 2.100 | 1.805 | 0.044 | 0.855 | 0.821 | 0.024 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.070 |  |  | 0.054 |
|  | 30 | 2.191 | 2.386 |  | 1.224 | 1.136 |  |
|  | 90 | 3.776 | 3.799 | 0.079 | 2.138 | 1.849 | 0.062 |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.000 | 0.000 | 0.001 | 0.002 |
|  | 30 | 0.015 | 0.075 | 0.150 | 0.387 | 0.009 | 0.043 | 0.086 | 0.219 |  |  |  |  |
|  | 90 | 0.085 | 0.407 | 0.861 | 2.034 | 0.041 | 0.208 | 0.415 | 1.038 | 0.000 | 0.002 | 0.004 | 0.009 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.001 | 0.002 | 0.003 | 0.007 |
|  | 30 | 0.047 | 0.235 | 0.472 | 1.189 | 0.024 | 0.121 | 0.243 | 0.610 |  |  |  |  |
|  | 90 | 0.084 | 0.421 | 0.841 | 2.102 | 0.040 | 0.202 | 0.404 | 1.010 |  |  |  |  |

Note: The above results do not take into account the Earth rotation.

Note: These values are for UE speed of 3 km/h.

Note: For the above results, UE altitude of 0 km is assumed.

Note: These values do not take into account UE altitude uncertainty.

Note: For the above results for Set1/2 parameters, it is assumed that the area served by the cell or beam (corresponding to the nadir beam size as defined in 38.821) is fixed within the satellite coverage.

Working assumption

Differential roundtrip delay limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Unrestricted | 839 | 103,13 | 419 | 0,39 | 800 | 102,34 |
| 0 | type A | 839 | 103,13 | 237 | 0,39 | 800 | 102,34 |
| 0 | type B | 839 | 103,13 | 137 | 0,39 | 800 | 102,34 |
| 1 | unrestricted | 839 | 684,38 | 419 | 0,39 | 800 | 683,59 |
| 1 | type A | 839 | 684,38 | 237 | 0,39 | 800 | 225,20 |
| 1 | type B | 839 | 684,38 | 137 | 0,39 | 800 | 129,85 |
| 2 | unrestricted | 839 | 152,60 | 419 | 0,39 | 800 | 151,82 |
| 2 | type A | 839 | 152,60 | 237 | 0,39 | 800 | 151,82 |
| 2 | type B | 839 | 152,60 | 137 | 0,39 | 800 | 129,85 |
| 3 | unrestricted | 839 | 103,13 | 419 | 0,39 | 200 | 102,34 |
| 3 | type A | 839 | 103,13 | 237 | 0,39 | 200 | 55,71 |
| 3 | type B | 839 | 103,13 | 137 | 0,39 | 200 | 31,88 |

Differential roundtrip delay limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 15 | 139 | 9.38 | 69 | 0.39 | 66.67 | 8.59 |
| A1 | 30 | 139 | 4.69 | 69 | 0.33 | 33.33 | 4.04 |
| A1 | 60 | 139 | 2.34 | 69 | 0.11 | 16.67 | 2.12 |
| A1 | 120 | 139 | 1.17 | 69 | 0.11 | 8.33 | 0.94 |
| A2 | 15 | 139 | 18.75 | 69 | 0.39 | 66.67 | 17.97 |
| A2 | 30 | 139 | 9.38 | 69 | 0.33 | 33.33 | 8.72 |
| A2 | 60 | 139 | 4.69 | 69 | 0.11 | 16.67 | 4.46 |
| A2 | 120 | 139 | 2.34 | 69 | 0.11 | 8.33 | 2.12 |
| A3 | 15 | 139 | 28.13 | 69 | 0.39 | 66.67 | 27.34 |
| A3 | 30 | 139 | 14.06 | 69 | 0.33 | 33.33 | 13.41 |
| A3 | 60 | 139 | 7.03 | 69 | 0.11 | 16.67 | 6.80 |
| A3 | 120 | 139 | 3.52 | 69 | 0.11 | 8.33 | 3.29 |
| B1 | 15 | 139 | 7.03 | 69 | 0.39 | 66.67 | 6.25 |
| B1 | 30 | 139 | 3.52 | 69 | 0.33 | 33.33 | 2.86 |
| B1 | 60 | 139 | 1.76 | 69 | 0.11 | 16.67 | 1.53 |
| B1 | 120 | 139 | 0.88 | 69 | 0.11 | 8.33 | 0.65 |
| B2 | 15 | 139 | 11.72 | 69 | 0.39 | 66.67 | 10.94 |
| B2 | 30 | 139 | 5.86 | 69 | 0.33 | 33.33 | 5.21 |
| B2 | 60 | 139 | 2.93 | 69 | 0.11 | 16.67 | 2.70 |
| B2 | 120 | 139 | 1.46 | 69 | 0.11 | 8.33 | 1.24 |
| B3 | 15 | 139 | 16.41 | 69 | 0.39 | 66.67 | 15.63 |
| B3 | 30 | 139 | 8.20 | 69 | 0.33 | 33.33 | 7.55 |
| B3 | 60 | 139 | 4.10 | 69 | 0.11 | 16.67 | 3.87 |
| B3 | 120 | 139 | 2.05 | 69 | 0.11 | 8.33 | 1.82 |
| B4 | 15 | 139 | 30.47 | 69 | 0.39 | 66.67 | 29.69 |
| B4 | 30 | 139 | 15.23 | 69 | 0.33 | 33.33 | 14.58 |
| B4 | 60 | 139 | 7.62 | 69 | 0.11 | 16.67 | 7.39 |
| B4 | 120 | 139 | 3.81 | 69 | 0.11 | 8.33 | 3.58 |
| C0 | 15 | 139 | 40.36 | 69 | 0.39 | 66.67 | 39.58 |
| C0 | 30 | 139 | 20.18 | 69 | 0.33 | 33.33 | 19.52 |
| C0 | 60 | 139 | 10.09 | 69 | 0.11 | 16.67 | 9.87 |
| C0 | 120 | 139 | 5.05 | 69 | 0.11 | 8.33 | 4.82 |
| C2 | 15 | 139 | 66.67 | 69 | 0.39 | 66.67 | 65.89 |
| C2 | 30 | 139 | 33.33 | 69 | 0.33 | 33.33 | 32.67 |
| C2 | 60 | 139 | 16.67 | 69 | 0.11 | 16.67 | 16.45 |
| C2 | 120 | 139 | 8.33 | 69 | 0.11 | 8.33 | 8.11 |

Working assumption

Theoretical differential Doppler limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0,1,2 | unrestricted | 1250 | 0 | 0.1 | 2 | 2 | 2.0 | 0.21 | 0.85 |
| 0,1,2 | type A | 1250 | 1 | 0.1 | 2 | 2 | 2.0 | 0.53 | 2.1 |
| 0,1,2 | type A | 1250 | 2 | 0.1 | 2 | 2 | 2.0 | 0.84 | 3.35 |
| 0,1,2 | type B | 1250 | 3 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 0,1,2 | type B | 1250 | 4 | 0.1 | 2 | 2 | 2.0 | 1.46 | 5.85 |
| 3 | unrestricted | 5000 | 0 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 3 | type A | 5000 | 1 | 0.1 | 2 | 2 | 2.0 | 2.40 | 9.6 |
| 3 | type A | 5000 | 2 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
| 3 | type B | 5000 | 3 | 0.1 | 2 | 2 | 2.0 | 4.90 | 19.6 |
| 3 | type B | 5000 | 4 | 0.1 | 2 | 2 | 2.0 | 6.15 | 24.6 |

Theoretical differential Doppler limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
|  | 30 | 30000 | 0 | 0.1 | 2 | 2 | 2.0 | 7.40 | 29.6 |
|  | 60 | 60000 | 0 | 0.1 | 30 | 20 | 2.5 | 1.08 | 54 |
|  | 120 | 120000 | 0 | 0.1 | 30 | 20 | 2.5 | 2.28 | 114 |

R1-2508468	FL Summary #3: Study on GNSS resilient NR-NTN operation		Moderator (THALES)

R1-2508469	FL Summary #4: Study on GNSS resilient NR-NTN operation		Moderator (THALES)

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following solutions at least for initial access procedure at least to increase PRACH tolerance and/or reducing time/frequency uncertainty:

- Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

- Solution 1B: New PRACH restricted sets.

- Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in single/multiple RAR MAC PDU scheduling multiple msg3 with different TA/FA.

- Solution 1D: Signalling enhancements for Msg2/Msg4 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command).

- Solution 1E: configuration/selection of a set of existing PRACH root for a Single PRACH transmission.

- Solution 1F: PRACH transmission using TA margin indicated by Network and RO masking.

- Solution 1G: Adaptation of PRACH configuration.

- Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

- Solution 2B: Multiple random access attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

- Solution 2C: Solutions based on broadcasting DL timestamp(s).

- Solution 2D: UE side time/frequency pre-compensation based on reference location or TA/Doppler compensation information provided by gNB.

- Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

- Solution 2F: UE side time/frequency pre-compensation based on reference signal.

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

Other Solutions are not precluded.

As part of the evaluation of the solutions above, at least the following aspects should be assessed:

- specification impact when relevant (including whether the solutions remain within the scope of the SID)

- performance,

- applicability to different scenarios,

- complexity,

- Coexistence with legacy UEs

- the signalling overhead.

Agreement:

With respect solution 2A, for the evaluation of DL-only UE-based positioning method, using DL-TDOA, consider the following evaluation assumptions and parameters:

| Parameter | Description/Value |
| --- | --- |
| Satellite Orbit | As per RAN1#122 agreement |
| Satellite parameters | As per RAN1#122 agreement |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | As per RAN1#122 agreement |
| BW | To be reported by companies. Baseline: 10 MHz for FR1 and 50 MHz for FR2 |
| Subcarrier spacing | 15 kHz for FR1-NTN, 120 kHz for FR2-NTN |
| Number of satellites in view | Case 1: 1 Satellite Case 2: At least 2 Satellites |
| UE type | As per RAN1#122 agreement |
| Positioning signals | DL-PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL: Comb-2 |
| Number of symbols used per occasion | 6 |
| Number of DL-PRS occasions | To be reported. |
| Time window for measurement collection | To be reported. |
| UE speed | As per RAN1#122 agreement |
| UE minimum elevation angle | For Case 1, as per RAN1#122 agreement. For Case 2, to be reported. |
| Ue clock drift/error | To be reported |
| Performance metrics | UE 2D as baseline. 3D can be reported by companies. Residual UL timing and frequency error after applying pre-compensation based on positioning |
| UE position estimation algorithme | To be reported. |

Agreement:

The skeleton for TR 38.742 proposed in R1-2508479 is endorsed in R1-2509611 with the following updates in red:

# 4 Background and motivation ## 4.1 Review of previous NR-NTN releases ## 4.2 Needs for GNSS resilience NR NTN operation ## 4.3 Use cases and deployment scenarios # 5 Assumptions and study parameters ## 5.1 GNSS availability scenarios ## 5.2 System assumptions ## 5.3 Evaluation scenarios and parameters # 6 Evaluation of impacts of GNSS temporary unavailability ## 6.1 Impact on synchronization in initial access ## 6.2 Impact on synchronization in connected mode # 7 Evaluation of solutions for GNSS resilient NR-NTN ## 7.1 Potential solutions for initial access ## 7.1.1 Evaluation of solutions ## 7.2 Potential solutions for RRC connected mode ## 7.2.1 Evaluation of solutions

# 8 Conclusions and recommendations ## 8.1 Summary of findings ## 8.2 Open issues and future work ## 8.3 Recommendations for normative work

R1-2509609	FL Summary #5: Study on GNSS resilient NR-NTN operation		Moderator (THALES)

Agreement:

To assess the impacts on connected mode during NR-NTN GNSS-resilient operation, evaluate the signalling overhead for a UE in RRC Connected mode under the following assumptions:

| Parameter | Description/Value |
| --- | --- |
| Satellite Orbit | As per RAN1#122 agreement |
| Satellite parameters | As per RAN1#122 agreement |
| Cell type | Quasi-Earth Fixed cell - Moving cell may be considered and reported. |
| Range of elevation angle | Baseline: From 90 degree to minimum elevation angle. Other values outside orbital plane to be reported. In this case, starting point is the highest elevation angle, and ending at the minimum elevation angle. |
| UE pre-compensation of RTT/Doppler on the service link | Case 1: At least for Scenario 1, based on assistance information provided by the Network Case 2: For Scenario 2, based on at least the last known UE location Details to be reported. Other pre-compensation methods can be reported by companies. |
| Duty cycle | Baseline: Continuous UL transmission every slot Other approaches if used, to be reported. |
| UE speed | Baseline: 3 km/h To be reported |
| Timing tolerance when connected (Note 1) | To be reported |
| Frequency tolerance when connected (Note 1) | To be reported |
| Initial timing error | Baseline: 0 To be reported |
| Initial frequency error | Baseline: 0 To be reported |
| Other | To be reported Note: Details of deriving UL timing and frequency error at gNB to be reported. |
| Performance metrics | Minimum period between two consecutive adjustment commands (in seconds) Maximum period between two consecutive adjustment commands (in seconds) Average number of adjustment commands per second Average number of times frequency error exceeds tolerance limit per second Average number of times timing error exceeds tolerance limit per second Other metrics may be considered and reported. |
| Note 1: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits and , respectively. |  |

Conclusion:

A Post-meeting Email discussion for Rel-20 NR-NTN is assigned for the following tasks:

- Collection of companies evaluation results for PRACH performance.

  - Note: The spreadsheet for PRACH performance evaluation embedded in R1-2509609 is used to collect companies’ results.

- Endorsement of a template for collection of results for Connected mode.

Duration of Email discussion: 2nd to 4th of February 2026.

Moderator: THALES (Mohamed)

R1-2509610	FL Summary #6: Study on GNSS resilient NR-NTN operation		Moderator (THALES)