3GPP TSG RAN WG1 #123			R1-2509502

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.7.1

Source:	Moderator (Thales)

Title:	FL Summary #2 - Reply LS on OCC for RACH-less HO

Document for:	Discussion, Decision

## Introduction

RAN2 evaluated the potential application of OCC for RACH-less handover and communicated with RAN1 via an LS [1] to determine whether OCC could be supported for RACH-less handover without requiring further updates to RAN1 specifications.

This Feature Lead Summary (FLS) document aims to gather and align companies’ views regarding the potential support of OCC for RACH-less handover from the RAN1 perspective. It provides a summary of related contributions under agenda item 5 at TSG-RAN WG1#123, as well as a draft LS reply based on the discussion.

# Companies’ contributions

| Company | Proposals |
| --- | --- |
| vivo | Answer: RAN1 discussed the application of OCC for RACH-less HO, and confirmed that if NW could maintain the orthogonality of OCC among multiplexed UEs, the use of OCC for NTN RACH-less handover is feasible and can be supported without any further updates to the RAN1 specifications. |
| NEC | Observation 1: In RACH-less HO, the first message in the target cell is Msg5. Observation 2: OCC can be applied to Msg5 directly follows the OCC on PUSCH rules. Proposal 1: Reply to RAN2 that there are no RAN1 specification impacts on supporting OCC length 2 or OCC length 4 for RACH-less HO. Whether or how to support OCC length 4 for RACH-less HO without potential performance loose can be left to the network implementation. |
| CATT | Observation 1: As uplink synchronization with the target cell has not yet been completed before RACH-less handover, the network is unaware of the UE's frequency offset and not able to group OCC-multiplexed users. And it will cause demodulation performance degradation and make handover failure. Observation 2: The primary purpose of initial uplink transmission in RACH-less handover is to simplify handover procedure rather than enhance capacity. Proposal 1: The use of OCC for NTN RACH-less handover is NOT feasible as uplink synchronization with the target cell has not yet been completed before RACH-less handover. The proposals to reply RAN2 is listed as following: Reply to RAN2: From RAN1’s perspective, the use of OCC for NTN RACH-less handover is NOT feasible as uplink synchronization with the target cell has not yet been completed before RACH-less handover and handover performance is not guaranteed. |
| Xiaomi | Proposal 1: Support OCC for RACH-less handover without any RAN1 specification impact. |
| OPPO | RAN1 response: At least the following RAN1 specification updates are required to support the use of OCC for NTN RACH-less handover: - the mapping relationship between SSB index and valid OCC groups for CG-PUSCH. - the rule for OCC group validation for CG-PUSCH. - the rule for OCC group dropping for CG-PUSCH. |
| Samsung | RAN1 response: RAN1 confirm that “OCC can be supported for RACH-less handover without any further update in RAN1 specifications.” |
| Ericsson | Observation 1 The configuration of OCC for CG PUSCH or DG PUSCH in the target cell after RACH-less handover is supported by the specification. Observation 2 Information regarding UE characteristics such as received UL power level in UL and carrier frequency offset can be used to find suitable candidates for pairing of UEs for OCC. This information is not available in the target gNB that prepares the RRC reconfiguration command. Observation 3 To support RACH-less handover with OCC, it may be necessary to convey additional UE information from source to target gNB before the handover, e.g., Power headroom report, UL received power level and UL carrier frequency offset measurement. Proposal 1 Reply to RAN2 with the following message: • It is RAN1's understanding that configuration of OCC for both DG and CG PUSCH in the target cell after RACH-less handover is supported by the specification • Information regarding UE characteristics that are needed to find suitable candidates for pairing of UEs for OCC is not available in the target gNB that prepares the RRC reconfiguration command. • To support RACH-less handover with OCC, it may be necessary to convey additional UE information from source to target gNB before the handover, e.g., power headroom report, UL received power level and UL carrier frequency offset measurement. |
| ZTE | Proposal 1: Reply RAN2 that OCC can be supported for NTN RACH-less handover without any further update to RAN1 specification. |
| MediaTek | RAN1 Reply: From RAN1’s perspective, introducing OCC for NR-NTN requires RAN1 specifications change. Therefore, RAN1 cannot confirm RAN2’s assumption “that OCC defined for NR NTN Rel-19 can be supported for NR NTN without any further update to RAN1 specifications |
| Qualcomm | Proposal: Endorse the following reply to RAN2 LS R1-2508308: - Support of OCC for NTN RACH-less handover is feasible and can be supported without any further update to RAN1 specifications. |
| Huawei | Proposal 1: OCC for NTN RACH-less handover is feasible and can be supported without any further update to the RAN1 specification. |

# Summary of companies’ contributions

Most companies, including vivo, NEC, Xiaomi, Samsung, ZTE, Qualcomm, and Huawei, support using OCC for RACH-less handover, stating that current RAN1 specifications are sufficient and no updates are needed, though some note that network implementation challenges such as UE pairing and performance degradation may arise. Ericsson agrees the specifications support OCC but highlights the practical need for additional UE information between gNBs to effectively multiplex users.

In contrast, CATT argue OCC is not feasible or needs further specification changes due to issues with uplink synchronization and grouping OCC-multiplexed UEs. MediaTek also argue that further specification changes would be required. OPPO also expresses concerns about limited benefits and significant specification efforts required for proper OCC support in RACH-less handover procedures.

In short:

|  | Companies | Key arguments |
| --- | --- | --- |
| Support OCC, no spec change needed | Vivo, NEC, Xiaomi, Samsung, ZTE, Qualcomm, Huawei | Feasible under current specs; some note practical or performance issues should be handled via implementation. |
| Support OCC, highlight network challenges | Ericsson | Spec supports, but pairing UEs for OCC may require additional info/signaling between nodes. |
| Oppose/require spec changes or not feasible | CATT, OPPO, MediaTek | Not feasible due to lack of sync/grouping (CATT); large spec effort required for validation/mapping (OPPO); spec change explicitly needed (MediaTek). |

# Initial proposal

## Proposal 1-1

Based on the above discussion the following initial proposal is made

Proposal 1-1-v0

Endorse the following response to RAN2 LS R1-2508308 (R2-2507909): OCC for RACH-less handover in NTN is feasible and can be supported without further updates to RAN1 specifications.

Companies are encouraged to comment on Proposal 1-1-v0

| Companies | Comments |
| --- | --- |
| Ericsson | It seems relevant to mention that while OCC for RACH-less handover is supported by the specifications, there are challenges with UE pairing. We propose to add the following information to the response: - Information regarding UE characteristics that are needed to find suitable candidates for pairing of UEs for OCC is not available in the target gNB that prepares the RRC reconfiguration command. - To support RACH-less handover with OCC, it may be necessary to convey additional UE information from source to target gNB before the handover, e.g., power headroom report, UL received power level and UL carrier frequency offset measurement. |
| Vivo1 | We support this proposal. From the perspective of RAN1 specification impact, we do not observe any impact to allow OCC for RACH-less HO. By contrast, if OCC is not supported for RACH-less HO, additional specification text, such as “UE is not expected to be configured or enabled with OCC during RACH-less HO”, would be needed in RAN1 or RAN2 spec. Regarding the feasibility, we agree with some companies that it may be challenging for the NW to determine UE pairs for the target cell. However, this is a scheduling issue and can be left to the NW’s decision. If the target cell is confident about the UE pairing, it can enable OCC in the DG/CG for RACH-less HO. If it is not, it can disable or simply not configure OCC for RACH-less HO. |
| Samsung | Support. We should focus exclusively on the RAN2 question. We don't need to address feasibility in our reply. Their question is simply about whether this is allowed by the current specification. |
| Huawei, HiSilicon | The current RAN1 spec configuration can support OCC during RACH-less handover for both the CG PUSCH and DG PUSCH. If the performance is not ensured, just not configure OCC by NW‘s implementation. Any further enhancement is not necessary. |
| DCM | Support. Any spec change is unnecessary, and if applying OCC is challenging, then just disabling OCC should be fine. No further optimization is necessary. |
| ZTE | Support. There is no restriction regarding application of OCC in RACH-less handover in RAN1 spec. |
| Qualcomm | Support. We think we should not confuse RAN2 with additional text on performance, that can be solved by proper implementation (or, if the network thinks it is not able to guarantee orthogonality, it will disable this feature). |
| OPPO | Thanks for this proposal. The point is whether or not additional RAN1 specification change for OCC for RACH-less handover in NTN is needed (similar view as Samsung). We think people may be encouraged to clarify the impact on OCC group validation/dropping rules for CG-PUSCH as we mentioned above. |
| Nokia | We find that although some companies are of the opinion that there are not fundamental blocking points from specification point, it is still important that RAN1 acknowledges some of the issues and shortcomings that will be seen if/when a RACH-less handover happens and UE continues OCC operation in the new target cell. Ericsson already pointed out a few items that my be mentioned in a LS reply to RAN2. On top of this, it would also be worth mentioning that the target gNB will not have any means to track/estimate the CFO error of the UE, which may hinder any UE pairing during the RACH-less handover. |
| CATT | We don’t support this proposal. The feasibility is questionable. For RACH less handover, the OCC pairing will have the problem, in which gNB can’t group the UE with same frequency offset. So the performance is not guaranteed. |

# Second round proposal

## Proposal 1-2

Proposal 1-2-v0

Endorse the following response to RAN2 LS R1-2508308 (R2-2507909):

OCC for RACH-less handover in NTN can be supported without further updates to RAN1 specifications. However, feasibility depends on successful UE pairing, which might be challenged by the lack of necessary UE characteristic information at the target gNB during RRC reconfiguration. Further, to support OCC for RACH-less handover, it may be necessary to exchange additional information from the source to the target gNB before handover, such as power headroom reports, UL received power levels, and UL carrier frequency offset measurements. It should also be noted that RAN1 has not performed any performance evaluation on OCC for RACH-less handover.

Companies are encouraged to comment on Proposal 1-2-v0

| Companies | Comments |
| --- | --- |
| OPPO | Thanks for this updated proposal. The situation is RAN2 is that RAN2 do not prevent the use of OCC for RACH-less. But from RAN1 perspective, I assume everybody knows that RAN1 should be more responsible w.r.t the assessment of feasibility of this technique. Apart from the UE paring issue. The performance concern should also be addressed, e.g., several companies such as Nokia, xiaomi, huawei, CATT point out that performance degradation may arise due to large frequency offset range. ---------------- I suggest to have following update in red: From RAN1 perspective, OCC for RACH-less handover in NTN can be feasible when frequency offset range is smallsupported without further updates to RAN1 specifications. However, feasibility depends on successful UE pairing, which might be challenged by the lack of necessary UE characteristic information at the target gNB during RRC reconfiguration. Further, to support OCC for RACH-less handover, it may be necessary to exchange additional information from the source to the target gNB before handover, such as power headroom reports, UL received power levels, and UL carrier frequency offset measurements. It should also be noted that RAN1 has not performed any performance evaluation on OCC for RACH-less handover. ---------------- |
| Vivo2 | First sentence: support. Regarding the OPPO’s revision, I think the first setence is to answer RAN2’s quesstion whether RACH-less HO OCC can be supported without RAN1 spec change, it is not about feasibilitly. Moderator’s proposal is more appriopriate. The second sentence : Potential information exchange between gNBs goes beyond the scope of RAN1 discussion, we prefer no to dicuss this aspect in RAN1. The feasibility of UE pairing may be implementation-dependent, and according to the online session, companies currently did not share a common view on whether such pairing can always be ensured in practical deployments. We suggest simply reflecting that there is no consensus at this stage regarding the feasibility. The thrid sentence: support. But we suggest refining the wording. Proposal 1-2-v0 Endorse the following response to RAN2 LS R1-2508308 (R2-2507909): OCC for RACH-less handover in NTN can be supported without further updates to RAN1 specifications. However, feasibility depends on successful UE pairing, which might be challenged by the lack of necessary UE characteristic information at the target gNB during RRC reconfiguration. Further, to support OCC for RACH-less handover, it may be necessary to exchange additional information from the source to the target gNB before handover, such as power headroom reports, UL received power levels, and UL carrier frequency offset measurements. It should also be noted that RAN1 has not performed any performance evaluation on OCC for RACH-less handover and cannot achieve consensus at this stage regarding the feasibility. |
| Samsung | Similar view with vivo. But, following sentence is preferred. OCC for RACH-less handover in NTN can be supported without further updates to RAN1 specifications. However, RAN1 has not performed any evaluation for the feasibility of OCC in case of RACH-less handover. |
| ZTE | Regarding the FO issue, since UE can perform accurate pre-compensation in RACH-less handover, the FO will be smaller than 0.1 ppm. And I think at least OCC2 can work with such FO without pairing from network. Moreover, the pairing is implementation related and may be possible at least for some scenarios according to online discussion, e.g., intra-gNB handover. Hence, we think it’s better to only reflect the current status in RAN1 and remove the second paragraph. |
| Nokia | In general OK with the proposed LS response. In the section where potential feasibility is discussed, we may have benefit of highlighting the e.g. UL carrier frequency offset is a component that is not currently defined as a measurement. Hence, we propose to have the following change: OCC for RACH-less handover in NTN can be supported without further updates to RAN1 specifications. However, feasibility depends on successful UE pairing, which might be challenged by the lack of necessary UE characteristic information at the target gNB during RRC reconfiguration. Further, to support OCC for RACH-less handover, it may be necessary to exchange additional information from the source to the target gNB before handover, such as power headroom reports, UL received power levels, and UL carrier frequency offset measurements, where the latter currently have no formal RAN1 definition or measurement defined. It should also be noted that RAN1 has not performed any performance evaluation on OCC for RACH-less handover. |
| Ericsson | We are fine with the proposed text and also with the proposed addition by Nokia. RAN2 asks for RAN1's opinion about the feasibility of OCC for RACH-less HO. The proposed text correctly describes the identified challenges regarding feasibility. If there is no consensus about whether these challenges can be managed or not, our view is that the proposed text is still more informative than just stating that we have no consensus without further explanation. In particular, it is useful for RAN2 to know that information is missing in the target gNB for the purpose of UE pairing, even if the decision whether to do anything about it (and if so, how to do it) is outside RAN1 scope. |

# Proposal for online session

## Proposal 1-3

Proposal 1-3-v0

Endorse the following response to RAN2 LS R1-2508308 (R2-2507909):

OCC for RACH-less handover in NTN can be supported without further updates to RAN1 specifications.

However, feasibility depends on successful UE pairing, which might be challenged by the lack of necessary UE characteristic information at the target gNB during RRC reconfiguration. Further, to support OCC for RACH-less handover, it may be necessary to exchange additional information from the source to the target gNB before handover, such as power headroom reports and UL received power levels.

It should also be noted that RAN1 has not performed any performance evaluation on OCC for RACH-less handover.

# Conclusion

Agreement:

The draft LS in R1-2509589 is endorsed with the following changes:

The final LS in endorsed in R1-2509590

RAN1 response

OCC for RACH-less handover in NTN can be supported without further updates to RAN1 specifications.

However, feasibility depends on successful UE pairing, which might be challenged by require knowledge of the lack of necessary UE characteristic information (such as UL received power levels and CFO) at the target gNB during RRC reconfiguration. Further, to support OCC for RACH-less handover, it may be necessary to exchange additional information from the source to the target gNB before handover, such as power headroom reports and UL received power levels.

It should also be noted that RAN1 has not performed any performance evaluation on OCC for RACH-less handover.

# References

- R1-2508308 	R2-2507909, LS on OCC for RACH-less HO, RAN2.

- R1-2508398	Draft reply LS on OCC for RACH-less HO	vivo

- R1-2508554	Discussion on RAN2 LS on OCC for RACH-less HO		NEC

- R1-2508565	Discussion on LS on OCC for RACH-less HO	CATT

- R1-2508656	Discussion on LS on OCC for RACH-less HO	Xiaomi

- R1-2508704	Discussion on OCC for RACH-less HO	OPPO

- R1-2508705	Draft Reply LS on OCC for RACH-less HO	OPPO

- R1-2508765	Discussion on RAN2 LS on OCC for RACH-less HO	Samsung

- R1-2508846	Discussion on RAN2 LS on OCC for RACH-less HO	Ericsson

- R1-2508847	Discussion on the LS on OCC for RACH-less HO	ZTE Corporation, Sanechips

- R1-2509158	Reply LS on OCC for RACH-less HO for NR NTN Ph3	MediaTek Inc.

- R1-2509198	Discussion on OCC for RACH-less HO	Qualcomm Incorporated

- R1-2509426	Discussion on OCC for RACH-less HO	Huawei, HiSilicon

- R1-2509427	Draft LS reply on OCC for RACH-less HO	Huawei, HiSilicon