3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	10.3

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 10.3 Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

Document for:	Discussion, Decision

## Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

Please refer to RP-252964 for detailed scope of the SI

[122bis-R20-A-IoT] Email discussion on Rel-20 A-IoT – Jay (LGE)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

[Post-122bis-R20-AIoT-evaluation] – Zhe (Huawei)

Email discussion on coverage evaluation results of link budget analysis, link-level simulation for R20 A-IoT study from November 5th until 14th

• Companies are encouraged to provide link budget analysis and link-level simulation results until November 7th

• Zhe to collect results, provide consolidated tables, proposals for observations until November 14th

The discussions are to be carried over to RAN1#123.

R1-2507357	Updated work plan for Rel-20 Ambient IoT SI	LG Electronics

### Evaluations

Including necessary evaluation assumptions of deployment scenarios for coverage and coexistence, evaluations of achievable cell edge data rate and link budget, as well as applicability and necessity of Device 2b and Device C to given scenarios.

R1-2506747	Discussion on Evaluation Methodology for R20 A-IoT	FUTUREWEI

R1-2506788	Evaluation for Rel-20 AIoT	Nokia

R1-2506808	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum, UNISOC

R1-2506827	Discussion on evaluation of active Ambient IoT device	ZTE Corporation, Sanechips

R1-2506892	Evaluation methodologies, assumptions and results for R20 AIoT	vivo

R1-2506944	Evaluation for active A-IoT device in outdoor scenario	Huawei, HiSilicon

R1-2506983	Discussion on evaluation methodology for Ambient IoT in NR outdoor for active devices	Xiaomi

R1-2507009	Discussion on evaluation assumptions	CMCC

R1-2507024	Evaluation assumptions for outdoor Ambient IoT	Tejas Network Limited

R1-2507066	Discussion on Rel-20 A-IoT evaluation assumptions and results	Ericsson

R1-2507113	Evaluation methodology for A-IoT outdoor deployment scenarios	CATT

R1-2507170	Discussion on EVM for R20 A-IoT	OPPO

R1-2507209	Evaluation results for Device 2b&C for Ambient IoT	HONOR

R1-2507247	Evaluation methodology and assumptions for Rel-20 Ambient IoT	Samsung

R1-2507299	Evaluations for Ambient IoT	NEC

R1-2507326	Discussion on evaluations for Ambient IoT	China Telecom

R1-2507358	Evaluations for Rel-20 Ambient IoT SI	LG Electronics

R1-2507591	Evaluation of Ambient IoT for outdoor active device	Sony

R1-2507619	Ambient IoT evaluations	MediaTek Inc.

R1-2507671	On Rel-20 Ambient IoT evaluations	Apple

R1-2507715	Evaluations	Qualcomm Incorporated

R1-2507755	Evaluations for Device 2b/C in Ambient IoT	InterDigital, Inc.

R1-2507807	Study on evaluations for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

R1-2507839	Evaluations for outdoor AIoT devices	Indian Institute of Tech (M)

R1-2507944	Evaluations for Rel-20 AIoT	IIT Kanpur

R1-2507958	FL summary #1 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

Agreement

For Rel-20 study, values of “W” and “h” used for RMa pathloss model are clarified as follows (as per TR38.901):

- W (avg. street width) = 20m

- h (avg. building height) = 5m

Agreement

For the bullet of “FFS: 40dB as additional assumption for Device 2b with IF-ED” under Device 2b noise figure assumption in link budget for evaluation purpose:

FFS: 40dB as additional assumption for Device 2b with IF-ED

Agreement

For Rel-20 study, the value of X dBm/Hz for “Receiver interference density (dBm/Hz)” in the link budget template is from the followings:

- For R2D, X = -169.3 dBm/Hz

- For D2R, X = -165.7 dBm/Hz

- Companies can also report other higher value(s) for R2D or -999dBm/Hz along with their assumption

Agreement

For reference data rate in LLS assumption for evaluation purpose, RAN1 select one option from the followings:

- Option 1: Remove the FFS bullet, i.e.

  - FFS whether to add additional value(s)

Agreement

For message size in LLS assumption for evaluation purpose, update the previous agreement as follows:

- FFS whether to add value of 800 bits.

Agreement

For the SFO and CFO in LLS assumption of Device C, update the previous RAN1 agreement (in RAN1#122) as follows,

Agreement

For Rel-20 study, SFO and CFO in LLS assumption for evaluation purpose is from the followings:

- Device C is smaller than Device 2b

|  | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| SFO after clock sync/calibration | 10^2 (O), 10^3 (M) ppm | [10] ppm |
| Initial CFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| CFO after clock sync/calibration | 100 (M), 200 (O), 50 (O) ppm | [10] ppm |

Agreement

For Rel-20 study, following template is used for companies to report energy harvesting/storage assumptions to obtain the reported data rates,

| Source | Energy harvesting/storage assumptions |
| --- | --- |
| Source [x] | Energy source: FL notes: Company to report energy source e.g. RF energy source and/or solar etc. Energy storage capacity: FL notes: Company to report energy storage capacity values in Joules or in farad with a voltage value. Reported data rate: FL notes: Company to report data rate for R2D and/or D2R respectively. Other information (if any): FL notes: In addition, companies can also report any related information they would like to indicate e.g. message size, reception/transmission duration, device power consumption, voltage, etc. Companies can also report how they derive the reported energy storage capacity. |

R1-2507959	FL summary #2 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

Agreement

For Rel-20 study, “Link budget template” is updated as followings,

Link budget template for Rel-20 study

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | Deployment scenario 4 with topology 1 with no external carrier wave | Deployment scenario 4 with topology 1 with no external carrier wave |
| [0B] | Device 2b/C | Device 2b/C | Device 2b/C |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Pathloss model | UMa NLOS defined in TR38.901 RMa NLOS defined in TR38.901 Note: companies can also report to use RMa LOS defined in TR38.901 UMi NLOS defined in TR38.901 | UMa NLOS defined in TR38.901 RMa NLOS defined in TR38.901 Note: companies can also report to use RMa LOS defined in TR38.901 UMi NLOS defined in TR38.901 |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | Refer to LLS table [0h1] and [0h2] | 1 |
| [1E] | Total Tx Power (dBm) | For standalone: 43 dBm For in-band: 38 or 33 dBm over R2D transmission BW Note: companies should report any PSD limitation they assume in their RAN1 evaluations | Device 2b: {-10, -20} dBm Device C: {-3, 0, 5} dBm |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1a] | Refer to LLS table [2a1] |
| [1G] | Tx antenna gain (dBi) | 17dBi | For A-IoT device, 0dBi |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0dB or 0.9dB or 4.7dB |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | 3dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a3] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | Device 2b: 15 dB FFS: 40dB as additional assumption for Device 2b with IF-ED Device C: 13 dB | For BS as reader: 5dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2E1] | Receiver interference density (dBm/Hz) | -169.3 dBm/Hz Companies can also report other higher value(s) for R2D or -999 dBm/Hz along with their assumption | -165.7 dBm/Hz Companies can also report -999 dBm/Hz along with their assumption |
| [2F] | Effective Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0dB or 0.9 dB or 4.7 dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt2 | Budget-Alt2 |
| [2L] | Receiver Sensitivity (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For UMa NLOS: 6dB For RMa NLOS: 8dB For UMi NLOS: 7.82dB | For UMa NLOS: 6dB For RMa NLOS: 8dB For UMi NLOS: 7.82dB |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB | 0 dB |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| [3E] | Penetration margin (dB) | {0, 20, 80} dB | {0, 20, 80} dB |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

Note 1: Calculated values are derived according to the following.

[1M]:

-	For R2D,

-	[1M] = [1E] + [1G] - [1N]

-	For D2R

-	[1M] = [1E] + [1G] - [1J]

[2F]:

-	[2F] = lin2dB(dB2lin([2D] + [2E]) + dB2lin([2E1])) +lin2dB([2B])

[2G]

-	For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth.

-	For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver.

-	For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth. Receiver bandwidth is the bandwidth used at the reader side to filter the D2R signals for calculating noise and interference (if any) power.

[2L]:

-	For R2D,

-	[2L] = [2G] - lin2dB([2B] / [1F]) + [2F]

Note E: The term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR.

-	For D2R,

-	[2L] = [2G] + [2F]

[4A]

-	For R2D

-	 [4A] = [1M] + [2C] - [2H] - [2L] - [3A] - [3B] + [3C] + [3D] - [3E]

-	For D2R

-	 [4A] = [1M] + [2C] - [2X] - [2L] - [3A] - [3B] + [3C] + [3D] - [3E]

[4B]

-	 [4B] is derived by assuming pathloss is [4A] and using the pathloss formula as agreed.

Agreement

For Rel-20 study, “Coverage evaluation assumptions for link-level simulation” is updated as followings,

Coverage evaluation assumptions for link-level simulation for Rel-20 study

| No. | Parameters |  | Assumptions |
| --- | --- | --- | --- |
| R2D/D2R common parameters |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template |
| [0b] | SCS |  | 15 kHz as baseline |
| [0c] | Block structure |  | Blocks as agreed in Rel-19 with update reported by companies |
| [0d] | Channel model |  | TDL-C |
| [0e] | Delay spread |  | an RMS delay spread of 300 ns |
| [0f] | Device velocity |  | 3 km/h Companies can also report 10 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 or 4 |
| [0h2] |  | Number of TXRUs | 2 or 4 |
| [0m] | Reference data rate |  | 1 kbps (M) 5 - 7 kbps (M) 48 - 60 kbps (O) 0.1 kbps for message size of 20 bits or 96 bits (O) Other data rates can be reported by companies Note 1: Companies to report the exact data rate. Note 2: The exact data rate is close to the values listed above. Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads(e.g., CRC, pre/mid/post-ambles if present). Note 4: The exact data rate may be related to coding scheme, repetition and etc. Note 5: All data rates considered are for evaluation purpose only FFS whether to add additional value(s) |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size FFS whether to add value of 800 bits. |
| [0p] | BLER target |  | 1%, 10% |
| [0q] | Sampling frequency |  | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: 50 ppm SFO after clock sync/calibration: For Device 2b: 10^3 (M) ppm, 10^2 (O) ppm For Device C: 10 ppm Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution Note: Above values are only for sampling purpose. Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. Note: SFO after clock calibration can be applied to Fe. Initial CFO: For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: 50 ppm CFO after clock sync/calibration: For Device 2b: 100 (M) ppm, 200 (O) ppm, 50 (O) ppm For Device C: 10 ppm Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only |
| [0r] | Device 2b/C |  | ZIF/IF-ED |
| R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180kHz Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report |
| [1b] | ED bandwidth |  | Up to company report with considering: R2D transmission bandwidth Device Rx LO uncertainty (e.g. CFO ppm assumption) |
| [1c] | BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at half of ED bandwidth. Companies to report X = {3, 5}. |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK-4 Companies to report M chips per OFDM symbol |
| [1f] | Line code |  | Manchester |
| [1g] | FEC |  | Companies who report evaluations using FEC should also report evaluations without FEC |
| [1h] | ADC bit width |  | 4-bit |
| [1j] | Detection/decoding method for Line code |  | Companies to report |
| [1k] | Repetition |  | Companies who report R2D evaluations using repetition should also report R2D evaluations without repetition |
| D2R specific parameters |  |  |  |
| [2a1] | Transmission bandwidth |  | 15kHz Other values are not precluded and up to company report Companies to report which of 2SB or 1SB modulation is assumed for D2R transmission bandwidth |
| [2a2] | Chip rate |  | Companies to report |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. Assume the receiver matches the transmitter's modulation, i.e., to receiver uses 1SB when transmitter uses 1SB, receiver uses 2SB when transmitter uses 2SB. Companies to report the value, and further down-selection of the values and 2SB/1SB is not precluded. |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, no line coding |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit |
| [2j] | D2R receiver |  | Companies to report, e.g., coherent receiver / non-coherent receiver |
| [2k] | Repetition |  | Companies to report based on update of Rel-19 if any |
|  | Other assumptions |  |  |
| [3a] | Other assumptions |  | To be reported by company |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |

R1-2507960	FL summary #3 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

### Study of air interface for Device 2b/C

Please refer to the first paragraph of objective 1 for the given conditions. Including study necessary and feasible changes to the Rel-19 air interface for Device 2b/C.

R1-2506748	Discussion on Air Interface for Device 2b/C	FUTUREWEI

R1-2506789	Air interface for Device 2b/C for Rel-20 AIoT	Nokia

R1-2506809	Discussion on study of air interface for Device 2b/C	Spreadtrum, UNISOC

R1-2506828	Discussion on air interface of active Ambient IoT device	ZTE Corporation, Sanechips

R1-2506893	Discussion on air interface for Device 2b/C for R20 AIoT	vivo

R1-2506945	Study of air interface for A-IoT 2b/C	Huawei, HiSilicon

R1-2506984	Discussion on the air interface for Device 2b/C	Xiaomi

R1-2507010	Discussion on air interface for Device 2b/c	CMCC

R1-2507025	Study the air interface for Device 2b and C for outdoor coverage 	Tejas Network Limited

R1-2507067	Study of air interface for Device 2b/C	Ericsson

R1-2507114	Study of A-IoT enhancement for device 2b/C	CATT

R1-2507171	Discussion on air interface enhancement for device 2b/C	OPPO

R1-2507190	Discussion on air interface for Device 2b and Device C	Transsion Holdings

R1-2507210	Views on air interface for Device 2b&C for Ambient IoT	HONOR

R1-2507248	Study on air interface design for active devices	Samsung

R1-2507300	Study of air interface for active device	NEC

R1-2507323	Discussion on air interface for Device 2b/C	Lenovo

R1-2507327	Discussion on air interface for Device 2b/C for Ambient IoT	China Telecom

R1-2507359	Air interface for Device 2b/C	LG Electronics

R1-2507382	Discussion on A-IoT Air Interface for Device 2b/C	Panasonic

R1-2507463	Views on AIoT outdoor for active devices	Ofinno

R1-2507501	Discussion on air interface for device 2b and device C	ETRI

R1-2507568	Discussion on air interface for active devices	Sharp

R1-2507570	Discussion on Air Interface Enhancements for R20 A-IoT	Fraunhofer HHI, Fraunhofer IIS

R1-2507592	Air interface for Ambient IoT device type 2b / C	Sony

R1-2507620	Study of air interface for Device 2b/C	MediaTek Inc.

R1-2507631	Study of air interface and power control for outdoor active ambient IoT devices	ROBERT BOSCH GmbH

R1-2507672	On Rel-20 Ambient IoT air interface for device 2b/C	Apple

R1-2507716	Study of air interface for Device 2b/C	Qualcomm Incorporated

R1-2507754	Air interface for Device 2b/C in Ambient IoT	InterDigital, Inc.

R1-2507787	Discussion on A-IoT active device	ASUSTeK

R1-2507808	Study on air interface for Ambient IoT device 2b/C	NTT DOCOMO, INC.

R1-2507833	Discussion on air interface for Device 2b/C	Quectel

R1-2507855	Discussion on air interface for active devices for Ambient IoT	TCL

R1-2507856	On AIoT air interface enhancement for Rel.20 Device 2b/C	Sequans Communications

R1-2507887	Study of A-IoT air interface for device 2b/C 	Wiliot Ltd.

R1-2507899	Discussion on air interface for Device 2b/C	CEWiT

R1-2507943	Discussion on air interface for active device types	IIT Kanpur

R1-2508046	FL summary #1 for 10.3.2 “Study of air interface for Device 2b/C	Moderator (LG Electronics)

Proposal 4.2a

For PRDCH, study necessity and feasibility of scrambling for Device 2b and for Device C.

Agreement

Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects

- Necessary information to be included in the L1 R2D control

- Details on how the L1 R2D control information is transmitted

R1-2508047	FL summary #2 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

Agreement

Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects:

- complexity of device to support FDM

- how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

Agreement

Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures.

- FFS: other cases, except for data transmission

R1-2508048	FL summary #3 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

Agreement

For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP:

- Option 1:  binary sequence-based SIP (e.g M-sequence or Golay sequence).

- Option 2: Reuse the Rel-19 SIP.

Proposal 4.4b

For the functionality of PRDCH chip duration determination, study the following options:

- Option 1: Use the L1 R2D control information to indicate the M value to be used for the PRDCH.

- Option 2: Enhance the Rel-19 CAP.

- Option 3: Reuse the Rel-19 CAP.

Agreement

For the functionality of indicating the end of the PRDCH, study the following option:

- Option 1: Use the L1 R2D control information to indicate the TBS.

Proposal 4.4.d

For the functionality of CFO estimation/LO calibration, study a CFO calibration signal with the following aspects:

- Whether the signal is single tone or multi-tone.

- Whether the CFO calibration signal is a standalone signal,

- The time/frequency location of the CFO calibration signal.

- Whether the CFO calibration signal is always transmitted with each R2D transmission

Proposal 5.1a

Considering 2SB is feasible for D2R, further study the necessity and feasibility of supporting 1SB for D2R.

R1-2508049	FL summary #4 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

[H] Proposal 4.2a-r1

For PRDCH, study necessity and feasibility of scrambling for Device 2b and for Device C to achieve the following:

- Interference randomization

- To reduce continuous 1s or 0s if needed

Agreement

For the functionality of chip duration determination of the data payload in the PRDCH, study the following options:

- Option 1: same as the chip duration of the corresponding L1 R2D control information

- Option 2: indicated by the corresponding L1 R2D control information

R1-2508103	FL summary #5 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

Agreement

For the functionality of chip duration determination of the L1 R2D control information, study at least the following options:

- Option 1: Fixed chip duration. No use of CAP for chip duration determination.

- Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- Option 3: Use CAP

  - Alt.3-1 Rel-19 CAP pattern

  - Alt.3-2 Enhanced CAP

    - Alt.3-2a: Rel-19 CAP pattern with repetition

    - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information

- Option 4: Use a set of different binary sequences with fixed or variable length

  - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination.

  - Alt.4-2: using binary sequences for CAP

- Option 5: Use broadcast information (if any) to indicate the chip duration

Agreement

For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects:

- Whether the signal is

  - Option a: unmodulated sinusoid single tone

  - Option b: unmodulated multiple sinusoid single tones

  - Option c: modulated multiple tones

    - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported)

- Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically)

- The time location(s) of CFO calibration signal.

  - Option 1: in relation to the L1 R2D control information

  - Option 2: in relation to the data payload of the PRDCH

  - Option 3: in relation to the PDRCH

  - Option 4: in relation to the synchronization signal (if supported)

  - Option 5: in relation to broadcast information (if supported)

  - Combination of Options is not precluded

- Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Proposal 4.4.f

For R2D signals, for the functionality of SFO calibration, study the following options:

- Option 1: Reuse existing R2D signals and/or Manchester encoding

- Option 2: Time-domain repetition of Rel-19 CAP

Proposal 4.4.g

For R2D signals, for the functionality of timing synchronization/tracking, study the following options:

- Option 1: New periodic synchronization signals

- Option 2: R2D midamble(s)

- Option 3: Manchester encoding

Agreement

Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking.

- Alt1: periodic synchronization signal

- Alt2: aperiodic synchronization signal

R1-2508104	FL summary #6 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

Agreement

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility:

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Agreement

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility:

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Agreement

Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader.

Agreement

For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options:

- Option 1: Msg1-like content

- Option 2: DO-A data payload

  - FFS: with or without device ID

- Option 3: DO-A scheduling request

Agreement

The study assumes single-carrier waveform for D2R.

- FFS whether to use 2SB or 1SB for D2R

Agreement

Regarding D2R preamble for PDRCH, study the following aspects of the D2R preamble.

- Preamble length (e.g., longer than Rel-19 D2R preamble)

- Sequence type

- Number of sequences using the same length (e.g., for reader/device differentiation)

- Considering other factors affecting preamble design (e.g., D2R modulation and detection schemes)

Agreement

Regarding D2R midamble for PDRCH, study the following aspects of the D2R midamble.

- Midamble pattern(s) including the length(s) and interval(s) within (and possibly at the end of) a PDRCH

- Sequence type, and whether the same or different sequence type is used for D2R preamble and D2R midamble

- Considering other factors affecting midamble design (e.g., D2R modulation and detection schemes)

Proposal 4.1a

For R2D, it is beneficial to add M=1 to the set of M values from Rel-19 in order to support larger coverage distances.

R1-2508129	FL summary #7 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

R1-2508130	FL summary #8 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)