3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	10.5

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 10.5 Study on Integrated Sensing And Communication (ISAC) for NR

Document for:	Discussion, Decision

## Study on Integrated Sensing And Communication (ISAC) for NR

Please refer to RP-252819 for detailed scope of the SI.

[122bis-R20-ISAC] Email discussion on Rel-20 ISAC – Yingyang (Xiaomi)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2507421	Updated work plan for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

R1-2507422	Updated TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR			Xiaomi, China Telecom

### Evaluation assumptions and performance evaluation

Including study of metrics, measurements, and relevant measurement quantization for UAV use case.

R1-2506811	Discussion on evaluation assumptions and metrics for ISAC	Spreadtrum, UNISOC

R1-2506895	Evaluation methodology and assumptions for 5G-A ISAC	vivo

R1-2506920	Evaluation assumptions and performance evaluation of ISAC for NR	InterDigital, Inc.

R1-2506946	Performance metric, methodologies, and initial evaluation results for ISAC	Huawei, HiSilicon

R1-2506986	Discussion on performance evaluation for ISAC	Xiaomi

R1-2507011	Discussion on ISAC evaluation methodology and assumptions	CMCC

R1-2507116	Discussion on evaluation assumptions and performance evaluation for R20 ISAC for NR	CATT, CICTCI

R1-2507173	Discussion of ISAC evaluation in 5GA	OPPO

R1-2507193	Discussion on 5G-A ISAC evaluation	ZTE Corporation, Sanechips

R1-2507204	Associating a detected object with its true counterpart in ISAC	Tejas Network Limited

R1-2507250	Discussion on ISAC evaluation assumptions and performance evaluation	Samsung

R1-2507332	Joint views on ISAC measurement report	China Telecom, ZTE, CAICT, CATT, Pengcheng Laboratory, Sony

(Withdrawn)

R1-2507337	Evaluation assumptions and performance evaluation for ISAC	EURECOM

R1-2507367	Discussion on ISAC Performance Evaluation for 5G-A   	NIST

R1-2507369	Views on evaluation assumptions and performance evaluation for ISAC	Sharp

R1-2507376	Discussion on evaluation assumptions and performance evaluation	Panasonic

R1-2507380	Discussion on evaluation assumptions and performance evaluation for NR ISAC	LG Electronics

R1-2507401	Views on R20 ISAC Study	SK Telecom

R1-2507428	Views on evaluation assumptions for NR ISAC	KPN N.V., TNO

R1-2507472	Evaluation assumptions and performance evaluations of ISAC for NR	Nokia, Nokia Shanghai Bell

R1-2507473	On ISAC performance evaluations and assumptions	Lenovo

R1-2507503	Discussion on evaluation assumptions and performance evaluation for NR ISAC	ETRI

R1-2507555	Joint views on ISAC measurement report	China Telecom, ZTE, CAICT, CATT, Pengcheng Laboratory

R1-2507573	Discussion on ISAC for NR	Ericsson

R1-2507593	Discussion on Evaluation of ISAC for NR	Sony

R1-2507627	Discussion on ISAC evaluation assumptions and performance evaluation	MediaTek Inc.

R1-2507634	Discussion on Performance Evaluations for Rel-20 ISAC	Google

R1-2507674	On Rel-20 Evaluation assumptions and performance evaluation for 5G-A ISAC	Apple

R1-2507718	Considerations on UAV gNB-monostatic sensing	Qualcomm Incorporated

R1-2507762	Views on Performance Metrics and Evaluation Methodology for ISAC	Tiami Networks

R1-2507810	Evaluation assumptions and performance evaluation for ISAC	NTT DOCOMO, INC.

R1-2507836	Evaluation assumptions and performance evaluation for NR ISAC	Hanbat National University

R1-2507423	Summary #1 on evaluations for NR ISAC	Moderator (Xiaomi)

Agreement

On False alarm probability Type 2 (targets dropped in simulation area), the parameter K is the number of drops with at least one detected object.

Conclusion

Sensing resolution is not considered as a performance metric for evaluation of NR ISAC.

Proposed conclusion

Sensing service latency and refreshing rate are not considered as performance metrics for evaluation of NR ISAC.

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |  |
| --- | --- | --- |
| BS antenna configuration | For 4GHz, 4.9GHz: - Option 1 as optional - Tx: (12,16, 2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) - Option 2 - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) | For 6GHz: - Option 1 as optional - Tx: (16,16, 2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) - Option 2: - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) |
|  | Mandatory: dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization |  |

- Note: Tx antenna elements and Rx antenna elements are operating simultaneously

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 |
| BS antenna mechanic tilt (downtilt angle without electrical tilt) | 90° in GCS (pointing to horizontal direction) |
| BS antenna electrical tilt (downtilt angle after mechanic and electrical tilt) | Option 1: no electrical tilt Option 2: 102° in GCS |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |

R1-2507424	Summary #2 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2507425	Summary #3 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2507426	Summary #4 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2507427	Summary #5 on evaluations for NR ISAC	Moderator (Xiaomi)