3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	8.7

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 8.7 Maintenance on Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode

Document for:	Discussion, Decision

## Maintenance on Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode

[122bis-R19-NTN] Email discussion on Rel-19 NTN enhancement – Mohamed (Thales)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### Maintenance for Rel-19 NR NTN

Note: Maximum one contribution for NR_NTN_Ph3, and potential RAN1 impact from NR_NTN_Ku_bands and NR_IoT_NTN_req_test_enh.

Note: For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- NR_NTN_Ph3

- RAN1 impact from NR_NTN_Ku_bands and NR_IoT_NTN_req_test_enh

R1-2506786	Maintenance for Rel-19 NR-NTN	Ericsson

R1-2506799	Remaining issues on NR-NTN	Spreadtrum, UNISOC

R1-2506877	Maintenance on Rel-19 NR NTN	vivo

R1-2506910	Remaining issues on Rel-19 NR NTN	ZTE Corporation, Sanechips

R1-2506936	Maintenance for Rel-19 NR NTN	Huawei, HiSilicon

R1-2506967	Maintenance for Rel-19 NR NTN	Xiaomi

R1-2507123	Maintenance for Rel-19 NR NTN	CATT

R1-2507139	Maintenance for Rel-19 NR NTN	OPPO

R1-2507233	Maintenance for Rel-19 NR NTN	Samsung

R1-2507322	Maintenance for Rel-19 NR NTN	THALES

(Withdrawn)

R1-2507533	Maintenance of uplink capacity/throughput enhancement for Rel.19 NR-NTN Ph3	Panasonic

R1-2507559	Discussion on Maintenance for Rel-19 NR NTN	Nokia

R1-2507700	Maintenance for Rel-19 NR NTN	Qualcomm Incorporated

R1-2507794	Maintenance of R19 NR-NTN	NTT DOCOMO, INC.

Maintenance for NR NTN (e)RedCap UEs

R1-2508078

Conclusion

Rel-19 NTN doesn’t support TA report enhancement for HD-FDD (e)RedCap.

Conclusion

No further discussion in Rel-19 for the case of a HD-FDD (e)RedCap collision in NTN involving more than two UL transmissions or DL receptions.

Agreement

The TP in section 5 of R1-2508078 is endorsed for the TS38.213 alignment CR.

Maintenance for NR NTN DL coverage enhancements

R1-2508065	FL Summary #1 - Maintenance on NR-NTN downlink coverage enhancements	 Moderator (Thales)

Proposed conclusion

The TP in Proposal 5-1-v1 is not pursued in Rel-19.

Conclusion

It is RAN1 understanding that Msg 3 PUSCH retransmission carry indication of support for msg4-NumberofRepetitions when initial Msg 3 PUSCH transmission carry indication of support for msg4-NumberofRepetitions.

- FFS: RAN1 specification impact

R1-2508066	FL Summary #2 - Maintenance on NR-NTN downlink coverage enhancements	Moderator (Thales)

R1-2508067	FL Summary #3 - Maintenance on NR-NTN downlink coverage enhancements	Moderator (Thales)

Maintenance for RAN4-led NR NTN WIs (NR_NTN_Ku_bands and NR_IoT_NTN_req_test_enh)

R1-2508070	FL Summary #1 - Impact from RAN4-led wor	Moderator (Thales)

R1-2508071	FL Summary #2 - Impact from RAN4-led work items	Moderator (Thales)

Agreement

The TP in section 3.1 of R1-2508071 for 38.211 is endorsed for the support of 3MHz CBW for NTN.

Agreement

The TP in section 3.2 of R1-2508071 for 38.213 is endorsed for the support of 3MHz CBW for NTN.

Agreement

The TP in section 3.3 of R1-2508071 for 38.214 is endorsed for the support of 3MHz CBW for NTN.

Proposal

The TP in section 3.4 of R1-2508071 for 38.212 is endorsed for the support of 3MHz CBW for NTN.

Conclusion

At RAN1#122bis, no RAN1 specification impact was identified to be needed for the support for Ku band for NTN in Rel-19.

R1-2508072	FL Summary #3 - Impact from RAN4-led work items	Moderator (Thales)

Maintenance for NR NTN uplink capacity/throughput enhancements and RAN1 impact of CB-Msg3-EDT

R1-2507989	Feature lead summary #1: NR-NTN uplink capacity and throughput enhancements	Moderator (MediaTek)

Proposed conclusion

There is no consensus at RAN1#122bis to specify further enhancements for phase continuity and power consistency for OCC with PUSCH in Release 19.

Proposal 2.3.1

Adopt TP_2_3_1 to TS 38.213 Clause 9

Agreement

TP_2_4_2 in section 2.4 of R1-2507989 is endorsed for TS 38.214 Clause 6.1.2.1.

R1-2507990	Feature lead summary #2: NR-NTN uplink capacity and throughput enhancements	Moderator (MediaTek)

R1-2507991	Feature lead summary #3: NR-NTN uplink capacity and throughput enhancements	Moderator (MediaTek)

LS on power ramping and RRC configuration for CB-Msg3-EDT

R1-2507987

Agreement

RAN1 response on power ramping for CB-Msg3-EDT: RAN1 will discuss and update corresponding RAN1 specifications based on RAN2 agreement to support power ramping for both NB-IoT and eMTC in CB-Msg3-EDT.

Agreement

RAN1 response for CB-Msg3-EDT configuration: RAN1 has no concern regarding the additional TBS size configured for CB-Msg3, i.e., 144 bits, for NB-IoT and eMTC.

Proposal

RAN1 response for support MPDCCH narrowband configuration: From RAN1 perspective, introducing 2 MPDCCH narrowbands requires additional RAN1 standard effort. RAN1 would like to inform RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

### Maintenance for Rel-19 IoT NTN

Note: Maximum one contribution for IoT_NTN_Ph3 and IoT_NTN_TDD

Note. For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- IoT_NTN_Ph3

- IoT_NTN_TDD

R1-2506878	Maintenance on Rel-19 IoT-NTN	vivo

R1-2506911	Remaining issues on Rel-19 IoT NTN	ZTE Corporation, Sanechips

R1-2506937	Maintenance for IoT NTN Phase 3 and TDD mode	Huawei, HiSilicon

R1-2506968	Maintenance for Rel-19  IoT NTN	Xiaomi

R1-2507124	Maintenance for Rel-19 IoT NTN	CATT

R1-2507140	Maintenance for Rel-19 IoT NTN	OPPO

R1-2507234	Maintenance for Rel-19 IoT NTN	Samsung

R1-2507264	Maintenance for Rel-19 IoT NTN	Ericsson

R1-2507294	Maintenance for Rel19 IoT-NTN	Nokia, Nokia Shanghai Bell

R1-2507312	TP for HARQ feedback resource of CB-Msg4	NEC

R1-2507701	Maintenance for Rel-19 IOT NTN	Qualcomm Incorporated

Maintenance for IoT NTN uplink capacity/throughput enhancement

R1-2508044	FL Summary #1 for Rel-19 IoT-NTN maintenance	Moderator (Sony)

Conclusion

Specification changes for handling UL gaps for synchronisation (from Rel-13) are not supported in Rel-19.

Conclusion

OCC for multi-tone transmissions of NPUSCH format 1 is not supported in Rel-19.

Agreement

Endorse TP#1_CB4 in section 5 of R1-2508044 for Clause 16.4.2 and 10.1.2.1 of TS36.213.

Agreement

Endorse TP#2_CB4 in section 5 of R1-2508044 for NB-IoT Clause 6.4.3.2 of TS36.212

Agreement

Endorse TP#3_CB4 in section 5 of R1-2508044 for eMTC Clause 5.3.3.1.12 of TS36.212

Proposal

| TP_4_1_1v1 TS36.211 clause 10.1.3.6 |  |
| --- | --- |
| Spec | TS36.211 |
| Reason for change: | When segment is configured, UE is not expected to be enabled with OCC if a segment length is not a multiple of the OCC group length, otherwise, the OCC orthogonality will be broken. The segment gap for pre-compensation configured by higher layer parameters may not be aligned with the OCC group , and thus break the OCC orthogonality. |
| Summary of change: | Clarify in the spec that, UE is expects that segment length is a multiple of the OCC group length if OCC is enabled. If segmentation gaps are configured, the segmentation gap length is equal to the OCC group length. |
| Consequences if not approved: | OCC orthogonality cannot be maintained when segmented transmission and segmented gaps are configured. |
| Clauses affected: | 10.1.3.6 |
| ============================ Unchanged Text Omitted =================================== For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1 for FDD, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ntn-SegmentedPrecompensationGaps-r17 as specified in 3GPP TS 36.331 [9] when OCC is not enabled; otherwise, a transmission gap of the minimum integer multiple of X slots not smaller than Ngapprecompensation time units is inserted, where X=4 for ∆f=3.75kHz and X=2 for ∆f=15kHz. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled. When the higher layer parameter npusch-OCC-Enabled is configured, OCC is indicated as enabled in DCI Format N0 as described in [3], and MrepNPUSCH≥2, the UE expects that the quantitiy Nsegmentprecompensation provided by higher layers is a multiple of tX slots. ============================ Unchanged Text Omitted =================================== |  |

Maintenance for IoT NTN TDD mode

R1-2507993	Feature lead summary #1 for IoT NTN TDD mode	Moderator (Qualcomm)

Agreement

The following TP is endorsed

| Spec | TS36.213 |
| --- | --- |
| Reason for change: | For description on n0, the indentation of ‘n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+Koffset for FDD or IoT NTN TDD’ is incorrect. It should be under the otherwise branch. Also, the main bullet “For FDD” should also include “IoT NTN TDD”. |
| Summary of change: | Increase the indentation of ‘n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+Koffset for FDD or IoT NTN TDD’ |
| Consequences if not approved: | How to determine n0 is not clear |
| Clauses affected: | 16.5.1 |
| ### 16.5.1 UE procedure for transmitting format 1 narrowband physical uplink shared channel NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14] , while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a NPDCCH with DCI format N0. A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of - n+k0+Koffset DL subframe for FDD or IoT NTN TDD, - k0 NB-IoT UL subframes following the end of n+8 subframe for TN TDD, a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where - subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and - , where the value of is determined as specified in Clause 16.5.1.1, the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.5.1.1), the value of is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the allocated number of subcarriers (as determined in Clause 16.5.1.1) in the corresponding DCI, and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI, otherwise - for FDD or IoT NTN TDD, - if NPUSCH transmission with subcarrier spacing and the UE configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled in the corresponding DCI, - n0 is the first NB-IoT UL slot, ns, starting after the end of subframe n+k0+Koffset that fulfills (5nf+ns) mod 4=0 - otherwise, - n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+Koffset - for TN TDD, n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8 subframe ==omitted== |  |

Agreement

Update the RAN1#122 agreement as follows:

Agreement

The set of D/U-subframes in a non-anchor carrier is the same as the set of D/U-subframes in the anchor carrier, and are time-aligned.

FFS: specification impact, if any. No specification impact

Agreement

Endorse the TP below

TP1 (36.211)

### 10.2.6	Narrowband reference signal (NRS)

Before a UE obtains operationModeInfo:

-	If frame structure type 1 is used in FDD, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #9 and in subframes #0 not containing NSSS.

-	If frame structure type 1 is used in IoT NTN TDD, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0, #3, #4, #6, #7, #8 and in subframes #9 not containing NSSS within the D consecutive downlink subframes according to the TDD pattern.

On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating guardband or standalone.

-	If frame structure type 1 is used in FDD, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, and in subframes #0 not containing NSSS, and in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions.

-	If frame structure type 1 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes.

-	If frame structure type 2 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, subframes #0 not containing NSSS, in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions, and in NB-IoT downlink subframes.

-	If frame structure type 1 is used in IoT NTN TDD, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0, #3, #4, #6, #7, #8 and in subframes #9 not containing NSSS within the D consecutive downlink subframes according to the TDD pattern.

<Unchanged parts omitted>

LS Reply on precompensation for NB-IoT NTN TDD mode

Agreement

Reply to RAN4 LS in R1-2506731 is endorsed as follows:

RAN1 has reached the following agreement on segmented precompensation at RAN1#121, related to the working assumption on the previous RAN1 LS: Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation The RAN4 CR may not be aligned with the latest RAN1 agreement. RAN1 would like to ask RAN4 if they have concerns on the updated agreement, and: - If RAN4 has concerns on the RAN1 agreement, RAN1 will modify its specifications to introduce segmented precompensation within the set of 8 consecutive uplink subframes. - If RAN4 has no concerns on the RAN1 agreement, we recommend RAN4 to update their specifications to clarify that segmented precompensation is not supported in IoT NTN TDD mode.

R1-2508063 [DRAFT] Reply LS on precompensation for NB-IoT NTN TDD mode	RAN1, Moderator (Qualcomm Incorporated)

R1-2508064 Reply LS on precompensation for NB-IoT NTN TDD mode	RAN1, Moderator (Qualcomm Incorporated)

Agreement

The draft LS reply to RAN4 in R1-2508063 is endorsed. The final LS is agreed R1-2508064.

LS on OCC for IoT-NTN TDD mode