3GPP TSG RAN WG1 #122		                                   R1-2506421

Bengaluru, India, Aug 25th – 29th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.11

Agenda Item:		9.11

Document for: 	Endorsement

  - UE features for MCE for NR Phase 3

Proposal 1-0:

No prerequisite FG is defined for FG 66-1 and 66-2

Proposal 1-1:

Update FG 66-1/2 as follows, i.e.:

- Any of the following carrier type and/or SCS is not supported for the set of cells reported in FG 66-1/2 (i.e., FG 66-1 and 66-2 are updated as follows:):

  - FR1 unlicensed band

  - FR2-2

- No more (carrier type, SCS) is supported for the scheduling cell in FG 66-1/2

- Remove [and/or different carrier type] from FG name and component 2

- Remove “FFS: Whether this FG can indicate two sets of (carrier type, SCS) for the cells in the set as (FR2, 60kHz) and (FR2, 120kHz)” from FG 66-1 and 66-2

| 66-1 | Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS [and/or different carrier type] | 1. UE supports monitoring DCI format 1_3 for DL scheduling with different SCS [and/or different carrier type] for the cells in the set 2. Supported applicable combinations of the following from the band combination: - Two [or three] sets of (carrier type, SCS) for the cells in the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)} - A set of (carrier type, SCS) for scheduling cell: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz)} FFS: Other set of (carrier type, SCS) for the cells in the set and/or scheduling cell FFS: Whether this FG can indicate two sets of (carrier type, SCS) for the cells in the set as (FR2, 60kHz) and (FR2, 120kHz) FFS: Whether/how to define separate FG for reporting three sets of (carrier type, SCS) for the cells in the set FFS: Whether/how to define separate FG for reporting unlicensed band in the sets of (carrier type, SCS) for the cells in the set FFS: Other component(s) |
| --- | --- | --- |
| 66-2 | Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS [and/or different carrier type] | 1. UE supports monitoring DCI format 0_3 for UL scheduling with different SCS [and/or different carrier type] for the cells in the set 2. Supported applicable combinations of the following from the band combination: - Two [or three] sets of (carrier type, SCS) for the cells in the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)} - A set of (carrier type, SCS) for scheduling cell: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz)} FFS: Other set of (carrier type, SCS) for the cells in the set and/or scheduling cell FFS: Whether this FG can indicate two sets of (carrier type, SCS) for the cells in the set as (FR2, 60kHz) and (FR2, 120kHz) FFS: Whether/how to define separate FG for reporting three sets of (carrier type, SCS) for the cells in the set FFS: Whether/how to define separate FG for reporting unlicensed band in the sets of (carrier type, SCS) for the cells in the set FFS: Other component(s) |

Proposal 1-1’:

For “consequence if the feature is not supported by the UE” column in FG 66-1 and 66-2, adopt below:

- For FG 66-1, “Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS is not supported”

- For FG 66-1: “Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS is not supported”

Proposal 1-2:

Regarding the number of sets of (carrier type, SCS) for the cells in the set,

- Confirm 66-1/2 are for two sets of (carrier type, SCS) for the cells in the set, i.e.,

  - Remove “or three” in FG 66-1/2

  - Remove “FFS: Whether/how to define separate FG for reporting three sets of (carrier type, SCS) for the cells in the set” in FG 66-1/2

- Define separate two FGs (e.g., FG 66-1a and 66-2a, for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling, respectively), for the support of three sets of (carrier type, SCS) for the cells in the set

  - FG 66-1a and 2a have FG 66-1 and 2 as a prerequisite FG, respectively

Proposal 1-2’:

Remove “FFS: Whether/how to define separate FG for reporting unlicensed band in the sets of (carrier type, SCS) for the cells in the set” in FG 66-1 and 66-2

Proposal 1-3:

Add the following components in FG 66-1:

- (2 in FG 49-1b) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or SCell, and a if set of cells includes only SCells.

- (4 in FG 49-1b) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}

- (5 in FG 49-1b) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8}

- (6 in FG 49-1b) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}

- (7 in FG 49-1b) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1b

- (8 in FG 49-1b) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both}

- (9 in FG 49-1b) Support Type-2 for ‘Antenna port(s)’ field

Proposal 1-4:

Add the following components in FG 66-2:

- (2 in FG 49-2b) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or SCell, and a if set of cells includes only SCells.

- (4 in FG 49-2b) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}

- (5 in FG 49-2b) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8}

- (6 in FG 49-2b) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}

- (7 in FG 49-2b) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both}

- (8 in FG 49-2b) Support Type-2 for ‘Antenna port(s)’ field

Proposal 1-6:

For monitoring SS set(s) for DCI format 1_3 and 0_3, add the following component for FG 66-1/2

x) Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases - 1) Search space set configuration for DCI format 1_3 for the set of cells is provided only on the scheduling cell, or; - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells - UE supporting FG 66-x can additionally report whether the UE support following case - 3) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells

Note: Above implies for UE supporting FG 66-1 and/or 66-2, case 1) and 2) have to be supported, and support of case 3) is optional (i.e., component x indicates whether to support case 3))

Proposal 1-7:

Inform RAN2 that RAN1 agrees to support FG 49-5a/5b/6/7/8/9/10/12/12a/13/14 as optional features for FG 66-1 and/or 66-2 by updating those Rel-18 FGs’ prerequisite FGs as follows in Rel-19.

- FFS: Whether/how to define FG 49-4a/4b/4c/4d for FG 66-1 and/or 66-2

| Index | Feature group | Prerequisite feature groups |
| --- | --- | --- |
| 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | At least one of {49-1, 49-1b, 66-1} |
| 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | At least one of {49-1, 49-1b, 66-1} |
| 49-6 | Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3 | At least one of {49-1, 49-1b, 66-1} |
| 49-7 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer for DCI format 1_3/0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} |
| 49-8 | Triggered HARQ-ACK codebook re-transmission for DCI format 1_3 | At least one of {49-1, 49-1b, 66-1} |
| 49-9 | SCell dormancy indication within active time in DCI format 0_3/1_3 | 6-5, At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} |
| 49-10 | Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3/1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} |
| 49-12 | Unified TCI with joint DL/UL TCI update by DCI format 1_3 for intra-cell and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC | 23-1-1, At least one of {49-1, 49-1b, 66-1} |
| 49-12a | Unified TCI with separate DL/UL TCI update by DCI format 1_3 for intra-cell beam management with more than one MAC-CE activated separate TCI state per CC | 23-10-1, At least one of {49-1, 49-1b, 66-1} |
| 49-13 | Default QCL assumption for multi-cell scheduling by DCI format 1_3 | At least one of {49-1, 49-1b, 66-1} |
| 49-14 | Support of BWP switch indication by DCI format 0_3/1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} for the BC At least one of {6-2, 6-3, 6-4} for at least one band of the BC |

Proposal 2-1:

Adopt the following update:

- Re-index FG 66-4y to 66-4

- Confirm “one or” in component 1 of FG 66-3 and 66-4

Proposal 2-2:

Regarding “FFS: 2. supported HARQ enhancement. details for component 2” in FG 66-3, adopt the following update:

- FFS: 2. supported HARQ enhancement. details for component 2 2. HARQ enhancements for Type 1 HARQ codebook for supporting multi-PDSCH scheduling on a single cell with single DCI and Type 2 HARQ codebook for supporting multi-cell multi-PDSCH scheduling with single DCI

Proposal 2-4:

Regarding “FFS: Whether/how to define component to report the supported number of maximum schedulable PDSCHs (or PUSCHs) per cell”, remove the whole FFS from FG 66-3 and 66-4 add the following component in FG 66-3 and 66-4:

- Add “Maximum number of PDSCHs per cell that can be scheduled by a DCI format 1_3 for multi-cell multi-PDSCH scheduling” in FG 66-3

- Add “Maximum number of PUSCHs per cell that can be scheduled by a DCI format 0_3 for multi-cell multi-PUSCH scheduling” in FG 66-4

- Candidate value for each of the components: {4, 8}

Proposal 2-5:

Regarding “FFS: Whether/how to define component to report the supported maximum total number of schedulable PDSCHs (or PUSCHs) per DCI”, remove the whole FFS from FG 66-3 and 66-4 add the following component in FG 66-3 and 66-4:

- Add “Maximum number of PDSCHs that can be scheduled by a DCI format 1_3 for multi-cell multi-PDSCH scheduling” in FG 60-3

- Add “Maximum number of PUSCHs that can be scheduled by a DCI format 0_3 for multi-cell multi-PUSCH scheduling” in FG 60-4

- Candidate value for each of the components: {8, 12, 16, [32]}

Proposal 2-6:

Regarding prerequisite FGs in FG 66-3 and 66-4:

- Adopt “at least one of {49-1, 49-1b, [66-1]}” for FG 66-3

- Adopt “at least one of {49-2, 49-2b, [66-2]}” for FG 66-4

Proposal 2-7:

- Adopt “Per BC” for FG 66-3 and 66-4

- Adopt “n/a” for each of FDD/TDD differentiation, FR1/FR2 differentiation, interpretation for mixture of FDD/TDD and/or FR1/FR2

- Remove “FFS: other component(s)”

R1-2505197	NR Multi-carrier Enhancements Phase 2 UE features	Nokia

R1-2505251	Discussion on UE features for MCE	Spreadtrum, UNISOC

R1-2505349	UE features for Rel-19 Multi-carrier enhancements	Huawei, HiSilicon

R1-2505403	UE features for MCE for NR Phase 3	vivo

R1-2505450	Discussion on UE feature for Rel-19 Multi-carrier enhancements	Xiaomi

R1-2505570	UE features for multi-carrier enhancements	Samsung

R1-2505692	UE features for multi-carrier enhancement for NR Phase 3	Ericsson

R1-2505725	Discussion on UE feature for multi-cell scheduling with a single DCI	OPPO

R1-2505900	Views on UE features for Rel-19 MCE	Apple

R1-2506027	MCE UE features Phase 3	MediaTek Inc.

R1-2506128	Discussion on UE features for MCE for NR Phase 3	ZTE Corporation, Sanechips

R1-2506204	UE features for MCE for NR Phase 2	Qualcomm Incorporated

R1-2506292		Discussion on UE features for multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.