3GPP TSG RAN WG1 #122	                                   	R1-2506413

Bengaluru, India, Aug 25th – 29th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.7

Agenda Item:		9.7

Document for: 	Endorsement

  - UE features for XR for NR Phase 3

Proposal 1:

Regarding DCI format(s) reported for skipping indication in FG 64-1, adopt the following updates:

| 64-1 | Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1] | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI 2. Supported DCI formats for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour. 4. The minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction FFS: other component (which may be separate FG) | FFS | YES | n/a | UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1] | Per UE |  | Yes |  | Candidate values for component 2: {DCI 0_1 and 1_1, DCI 0_2 and 1_2, DCI 0_3, DCI 1_3} Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms} Note: The behavior described in Component 1 is supported in per-UE basis (i.e., supported in any band(s) that the UE supports) with FR1/FR2 differentiation FFS: Candidate DCI formats for component 2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5:

Adopt the following updates for FG 64-1:

- Put “No” for FDD/TDD differentiation

- Put “No” for capability interpretation for mixture of FDD/TDD and/or FR1/FR2

Proposal 3:

Companies are encouraged to provide their views on the need for the following potential text addition in Note column for FG 64-1:

- “If a UE indicates support for the reception of the indication with DCI format 0_3 and/or 1_3, the UE supports reception of the indication on a band with that DCI format if the UE also indicates support for that DCI format on that band.”

- “If skipping field is configured to DCI format 0_3/1_3, only cells that are intra-band and have same SCS can be co-scheduled”

Proposal 4:

- Adopt no prerequisite FG for FG 64-1

- Add a note “A DCI format reported in Component 2 does not indicate a reception capability of the DCI format”

R1-2505195	UE features for XR for NR Phase 3	Nokia

R1-2505353	UE features for R19 XR	Huawei, HiSilicon

R1-2505400	UE features for XR for NR Phase 3	vivo

R1-2505566	UE features for XR Phase 3	Samsung

R1-2505693	UE features for XR for NR Phase 3	Ericsson

R1-2505723	Discussion on UE features for Rel-19 XR	OPPO

R1-2505990	Discussion on XR features	ZTE Corporation, Sanechips

R1-2506201	UE features for Rel-19 XR for NR Phase3	Qualcomm Incorporated

R1-2506289	Discussion on UE features for Rel-19 XR enhancements	NTT DOCOMO, INC.

R1-2506393	UE features for XR for NR Phase 3	Google