3GPP TSG RAN WG1 #123                                                                                                 R1-2509216

Dallas, USA, November 17th ~ 21st , 2025

Agenda item:	10.1.1.1

Source: 	Moderator (Qualcomm)

Title: 		FL summary 3 on inference related aspects of CSI feedback via two-sided model

Document for:	Discussion and Decision

# Introduction

Based on the conclusion of the Rel-19 study [1], spatial-frequency CSI compression (with temporal aspects) via two-sided AIML model proceeds to the normative work, with following objective [2] for RAN1.

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

The above objectives are further split into three agenda, i.e., inference aspects, inter-vendor collaboration and other aspects including data collection, monitoring and model pairing. This document summarizes the issues regarding agenda item 10.1.1.1 (Inference aspects of two-sided CSI compression) in RAN#122.

## Contact Information

Please provide your contact information.

| Company | Name | E-mail |
| --- | --- | --- |
| Moderator (Qualcomm) | Chenxi Hao | chenxih@qti.qualcomm.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Bingchao Liu | vpourahmadi@lenovo.com liubc2@lenovo.com |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |
| NTT DOCOMO | Xin Wang | wangx@docomolabs-beijing.com.cn |
| Sharp | Pochun Chou | bertrand.pc.chou@sharp-world.com.tw |
| CMCC | Yi Zheng Yuhua Cao | zhengyi@chinamobile.com caoyuhua@chinamobile.com |
| Ericsson | Xinlin Zhang Jingya Li | xinlin.zhang@ericsson.com jingya.li@ericsson.com |
| Futurewei | Zhigang Rong | zrong@futurewei.com |
| MediaTek | Reubengeorge Stephen Pedram Kheirkhah Sangdeh | Reubengeorge.stephen@mediatek.com Pedram.kheirkhah@mediatek.com |
| CEWiT | Shivshankar Advaith | shivshankar@cewit.org.in advaith22@cewit.org.in |
| CATT | Qianrui Li | liqianrui@catt.cn |
| Panasonic | Tetsuya Yamamoto Hidetoshi Suzuki Xuan Tuong Tran (Henry) | yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@jp.panasonic.com xuantuong.tran@sg.panasonic.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| LG Electronics | Minseok Jo (MJ) | ms.jo@lge.com |
| TCL | Pu Yuan Tianqi Wu | Pu.yuan@tcl.com tianqi1.wu@tcl.com |
| NEC | Peng Guan Yi Jiang Shafivulla Sayyed | guan_peng@nec.cn y-jiang_ct@nec.com Sayyed.Shafivulla@india.nec.com |
| Apple | Huaning Niu Yuan Zhu | huaning_niu@apple.com yuan_zhu3@apple.com |
| Vivo | Yuanyuan Wang | yuanyuan.wang.txyj@vivo.com |
| InterDigital | Afshin Haghighat | afshin.haghighat@interdigital.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| China Telecom | Nanxi Li | linanxi@chinatelecom.cn |
| Fujitsu | Xin Wang Liqiang Jin | wangxin@fujitsu.com liqingjin@fujitsu.com |

# Payload and quantization related aspects

## Summary of companies’ proposals

| Company | Proposals |
| --- | --- |
| Futurewei | Proposal 4: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for quantization codebook used for quantizing the feedback, for inter-vendor collaboration Direction A, support Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters associated with an ID. The exchanged quantization codebook can be in the form of the codebook itself or can be parameters of the codebook based on which the codebook can be reconstructed. Proposal 6: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for per-layer payload parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν, at least support one of the PCs for VQ with L = dl,ν (e.g., dl,ν, L,Ql,ν = 128, 128,10). |
| InterDigital | Proposal 3: For both Direction A and Direction C, support hybrid VQ + SQ quantization to reduce the VQ overhead and complexity, while also avoiding the inefficiency of standalone SQ. Proposal 4: For the hybrid VQ-SQ approach add the following definition on payload size. - If hybrid VQ-SQ is supported, it refers to the quantization of a segment of L consecutive real values jointly, and the segment(s) are separately quantized, and independent quantization of a scalar per each segment. - The payload size for layer l and rank ν is determined by dzl,νL×QVQl,ν + L×QSQl,ν, where QVQl,ν is the number of bits per segment, QSQl,ν is the number of bits per scalar. Proposal 5: Support non-uniform scalar quantizer design based on the distribution of latent samples where the UE and NW exchange the non-uniform scalar quantizer parameters. Proposal 6: Support Alt1 for the quantization codebook for inter-vendor collaboration Direction A, where the quantization codebook from NW-side to UE-side is exchanged along with each exchanged dataset or model parameters. |
| Ericsson | Proposal 1 Support SQ as the only quantization method that is used for quantizing the latent space for both model training and model inference. Proposal 2 Support of the following N=5 pairs of {d, Q}: {32, 2}, {64, 2}, {96, 2}, {128, 2}, {192, 2} Proposal 3 Support parameter combinations of {64, 2}, {96, 2}, {128, 2}, {192, 2} for rank = 1 and rank = 2 case. If {32, 2} is supported by a PC, this PC is not expected to support rank 3 and rank 4. For rank 3, the following d1,3,Q, d2,3,Q, d3,3,Q give the best system level throughput for the considered total payload sizes. d1,3,Q, d2,3,Q, d3,3,Q; Total payload size. {32, 2}, {32, 2}, {64, 2}; 256. {64, 2}, {64, 2}, {64, 2}; 384. {64, 2}, {64, 2}, {128, 2}; 512. {128, 2}, {128, 2}, {128, 2}; 768. For rank 4, the following d1,4,Q, d2,4,Q, d3,4,Q, d4,4,Q give the best system level throughput for the considered total payload sizes. d1,4,Q, d2,4,Q, d3,4,Q, d4,4,Q; Total payload size. {32, 2}, {32, 2}, {32, 2}, {32, 2}; 256. {32, 2}, {32, 2}, {64, 2}, {64, 2}; 384. {32, 2}, {32, 2}, {64, 2}, {128, 2}; 512. {64, 2}, {64, 2}, {128, 2}, {128, 2}; 768. Support 5 PCs according to Table 11, where each index maps to a pair of {d, Q} defined in Table 10. Table 10 Indices of {d, Q} pairs Index; {d, Q}. 1; {32, 2}. 2; {64, 2}. 3; {96, 2}. 4; {128, 2}. 5; {192, 2}. Table 11 PCs with supported {d, Q} indices for different ranks. Rank1; Rank2; Rank3; Rank4; Total payload size rank 1/2/3/4. PC1; 1; 1,1; N/A; N/A; 64/128/NA/NA. PC2; 2; 2,2; 1,1,2; 1,1,1,1; 128/256/256/256. PC3; 3; 3,3; 2,2,2; 1,1,2,2; 192/384/384/384. PC4; 4; 4,4; 2,2,4; 1,1,2,4; 256/512/512/512. PC5; 5; 5,5; 4,4,4; 2,2,4,4; 384/768/768/768. |
| Spreadtrum | Proposal 4: For quantization codebook, support use a standardized quantization codebook decided by RAN1. |
| Vivo | Proposal 5: Support Alt 1: exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters Proposal 6: For exchanged quantization codebook, multiple L (e.g., 1, 2) can be supported. Proposal 7: A payload-common codebook can be exchanged from NW-side to UE-side along with each exchanged dataset or model parameters. - dl,ν=32, 64, 128 should be supported for small, medium and large payload sizes - The following PCs can be supported for Rel-20 CSI compression Rank1 X; Rank2 X, X; Rank3 X, X, X; Rank4 X/2, X/2, X, X. PC1; {32, 2}; {32, 2}, {32, 2}; {32, 2}, {32, 2},{32, 2}. PC2; {64, 2}; {64, 2}, {64, 2}; {64, 2}, {64, 2},{64, 2}; {32, 2}, {32, 2},{64, 2},{64, 2}. PC3; {128, 2}; {128, 2}, {128, 2}; {128, 2}, {128, 2},{128, 2}; {64, 2},{64, 2},{128, 2},{128, 2}. |
| CMCC | Proposal 7: For quantization codebook used for quantizing the feedback, at least scalar quantization (SQ) should be supported. Proposal 8: For quantization codebook used for quantizing the feedback, for inter-vendor collaboration Direction A, support Alt1: Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters Proposal 9: For quantization codebook used for quantizing the feedback, dzl,νand Ql,ν should be determined based on NW configuration. |
| Huawei, HiSilicon | Proposal 3: Candidate combinations of {subband pattern, Tx port number, d/Q/L values}, can be represented by the standardized model (Direction C)/model structure (Option 3a-1 Direction A) or delivered dataset (Option 4-1 Direction A) during the inter-vendor training collaboration. - After model pairing procedure, candidate d/Q/L values for the applicable model are aligned. A specific CSI payload size can be indicated by additionally configuring the specific d/Q/L value among the candidate d/Q/L values. Proposal 5: For precoding matrix feedback, regarding the determination of N parameter pairs / triplets, consider at least the following rules: - For rank 1/2, the minimum and maximum per-layer payload size should be comparable with the minimum and maximum payload size of legacy eType II codebook for rank 1, respectively. - E.g., consider 64 bits (e.g., d=32 and Q/L=2 with L=1 for SQ and L=4 for VQ or d=64 and Q/L=1 with L=1 for SQ and L=4 for VQ) for minimum per-layer payload size, and consider 384 bits (e.g., d=128 and Q/L=3 with L=1 for SQ and L=2 for VQ or d=96 and Q/L=4 with L=1 for SQ and L=2 for VQ) for minimum per-layer payload size. - For rank 3/4, consider either even split (i.e., {a, a, a} or {a, a, a, a}) or the payload size of layer 1/2 larger than that of layer 3/4 (i.e., {a, a, b}, {a, b, b} or {a, a, b, b} with “a” corresponding to the larger payload size). - A dedicated per-layer pair/triplet (e.g., d*Q/L=32*1) corresponding to a payload size lower than the smallest payload size of rank 1 should be included in the supported per-layer pool of {d,Q}/{d,Q,L} to enable smaller per-layer payload size of rank 3/4. Proposal 6: For precoding matrix feedback, regarding the set of N parameter pairs / triplets, consider the following two candidate options for {d,Q} or {d, Q/L} as reference for further selection: - Option 1: {32, 1}, {32, 2}, {96, 1}, {32, 4}, {96, 2}, {96, 4} - Option 2: {32, 1}, {64, 1}, {32, 3}, {128, 1}, {64, 3}, {128, 3} - Note: Examples of the PC entries for SQ and VQ and for Option 1 and Option 2 are provided in Table 4 to Table 7 . {d, Q} pool: {32, 1}, {32, 2}, {96, 1}, {32, 4}, {96, 2}, {96, 4} Note: d=32, 96; Q=1, 2, 4. Rank 1; Rank 2; Rank 3; Rank 4. PC1; {32, 2} (64bits); {32, 2}, {32, 2}; {32, 2}, {32, 1}, {32, 1}; {32, 1}, {32, 1}, {32, 1}, {32, 1}. PC2; {96, 1} (96bits); {96, 1}, {96, 1}; {32, 2}, {32, 2}, {32, 2}; {32, 2}, {32, 2}, {32,1}, {32, 1}. PC3; {32, 4} (128bits); {32, 4}, {32, 4}; {32, 4}, {32, 2}, {32, 2}; {32, 2}, {32, 2}, {32, 2}, {32, 2}. PC4; {96, 2} (192bits); {96, 2}, {96, 2}; {32, 4}, {32, 4}, {32, 4}; {96, 1}, {96, 1}, {96, 1}, {96, 1}. PC5; {96, 4} (384bits); {96, 4}, {96, 4}; {96, 4}, {96, 2}, {96, 2}; {96, 2}, {96, 2}, {96, 2}, {96, 2}. …; Note: other PC values can consider to replace rank 3/4 components, e.g., {32, 2}, {32, 1}, {32, 1}=>{32, 1}, {32, 1}, {32, 1} {32, 2}, {32, 2}, {32,1}, {32, 1}=>{32, 1}, {32, 1}, {32, 1}, {32, 1} {96, 1}, {96, 1}, {96, 1}, {96, 1}=>{32, 4}, {32, 4}, {32, 2}, {32, 2}. {d, Q} pool: {32, 1}, {64, 1}, {32, 3}, {128, 1}, {64, 3}, {128, 3} Note: d=32, 64, 128; Q=1, 3. Rank 1; Rank 2; Rank 3; Rank 4. PC1; {64, 1} (64bits); {64, 1}, {64, 1}; {64, 1}, {32, 1}, {32, 1}; {32, 1}, {32, 1}, {32, 1}, {32, 1}. PC2; {32, 3} (96bits); {32, 3}, {32, 3}; {64, 1}, {64, 1}, {64, 1}; {64, 1}, {64, 1}, {32,1}, {32, 1}. PC3; {128, 1} (128bits); {128, 1}, {128, 1}; {128, 1}, {64, 1}, {64, 1}; {64, 1}, {64, 1}, {64, 1}, {64, 1}. PC4; {64, 3} (192bits); {64, 3}, {64, 3}; {128, 1}, {128, 1}, {128, 1}; {32, 3}, {32, 3}, {32, 3}, {32, 3}. PC5; {128, 3} (384bits); {128, 3}, {128, 3}; {128, 3}, {64, 3}, {64, 3}; {64, 3}, {64, 3}, {64, 3}, {64, 3}. …; Note: other PC values can consider to replace rank 3/4 components, e.g., {32, 2}, {32, 1}, {32, 1}=>{32, 1}, {32, 1}, {32, 1} {32, 2}, {32, 2}, {32,1}, {32, 1}=>{32, 1}, {32, 1}, {32, 1}, {32, 1} {96, 1}, {96, 1}, {96, 1}, {96, 1}=>{32, 4}, {32, 4}, {32, 2}, {32, 2}. Proposal 7: For flexible trade-off between quantization performance and quantization complexity/overhead: - To simplify the design for VQ and SQ: - For SQ, uniform quantization is considered for 1 bit, 2bits and 4 bits quantization granularity. The quantization interval can be determined based on a standard range or model structure of encoder - For VQ, a master codebook including the set of codewords (i.e., floating vectors) and fixed length of the vector segment (i.e., L) are considered over all layers/Rx antennas. - Flexible quantization configuration should be supported. - For SQ, multiple quantization granularities are considered. - For VQ, flexible selection of codewords from the master codebook is considered to achieve different quantization granularities (Q values). Proposal 8: For the codebook used for quantizing the feedback in inter-vendor collaboration Direction A, support Alt1 (Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters). Proposal 9: At the inference phase, the quantization codebook (e.g., quantization dictionary for VQ) can be updated by gNB with RRC configuration. |
| NEC | Proposal 4: For determining the payload parameters combinations, support both SQ and VQ. Proposal 5: Multiple (candidate) parameters (e.g., d, Q, L) or parameter combinations should be specified. And the parameter combination used in inference can be determined based on NW configuration and/or UE reporting, which requires further discussion. Proposal 6: Whether to use fixed parameter combination depends on the inter-vendor training. For instance, only a fixed parameter combination is used in the inter-vendor training. Proposal 7: For CSI payload scalability, prioritize those options that enable layer specific or rank specific. Proposal 8: For rank 3/4, at least support the PC combination {a, a, a} or {a, a, a, a} having one PC. For the other PC combinations having more than one PCs, for the PC combinations having the same number of PCs, support one of them is enough, e.g., - For rank 3, support one of {a, a, b} and {a, b, b}. - For rank 4, support {a, a, b, b} + one of {a, b, b, c}, {a, a, b, c} and {a, b, c, c}. |
| CATT | Proposal 5: Prioritize scalar quantization for CSI compression for model inference. Proposal 6: For model inference, support only Ql,v=2 in payload parameters combinations. Support dl,ν=32, 64, 96, 128, 192 Proposal 7: For AI CSI compression, for inference, support 64/128/192 bits per layer for rank1-4, with 256bits per layer only available for rank1 and rank2. |
| China Telecom | Proposal 3: Support Option 3-1 (layer common and rank common), and limit each model to a single or a few encoder output dimensions dl,ν applicable across all supported layers and ranks Proposal 4: Support limiting the encoder output dimension dzl,v to a finite set of selectable options. Proposal 5: Support multiple quantization configurations/codebooks and dynamic adjustment of quantization parameters. Proposal 6: Support associating a set of per-layer payload PCs with the pairing ID. Proposal 7: Support the association of each pairing ID with a specific set of subband and port configurations. |
| Xiaomi | Proposal 1: Support scalar quantization for CSI feedback. Parameter combination; v=1; v=2; v=3; v=4. 1; d1,1=32; d1,2=d2,2=32; d1,3=d2,3=d3,3=32; d1,4=d2,4=d3,4=d4,4=32. 2; d1,1=64; d1,2=d2,2=64; d1,3=d2,3=32, L3=64; d1,4=d2,4=d3,4=d4,4=32. 3; d1,1=96; d1,2=d2,2=96; d1,3=d2,3=d3,3=64; d1,4=d2,4=32, d3,4=d4,4=64. 4; d1,1=128; d1,2=d2,2=128; d1,3=d2,3=64, d3,3=128; d1,4=d2,4=d3,4=d4,4=64. 5; d1,1=128; d1,2=d2,2=128; d1,3=d2,3=64, d3,3=128; d1,4=32, d2,4=d3,4=64, d4,4=96. 6; d1,1=192; d1,2=d2,2=192; d1,3=d2,3=96, d3,3=192; d1,4=d2,4=d3,4=d4,4=96. 7; d1,1=192; d1,2=d2,2=192; d1,3=d2,3=96, d3,3=192; d1,4=64, d2,4=d3,4=96, d4,4=128. Proposal 2: Support N=5, and (dl,ν, Ql,ν)={(32,2), (64,2), (96,2), (128,2), (192,2)}. Proposal 3: The following parameter combinations with Ql,ν=2 in the table can be supported. Proposal 4: Support Alt2, i.e., use a standardized quantization codebook decided by RAN1 if SQ is adopted as quantization method of CSI feedback. |
| ZTE | - Proposal 8: For payload determination of AI/ML CSI feedback, support SQ due to its simplicity on implementation and standardization. FFS whether to support VQ for further performance improvement. - Proposal 9: When the Target CSI is precoder matrix, support the following quantization parameter value: d={32, 64, 96, 128, 192}, Q=2, resulting in scalable per-layer payload size of {64, 128, 192, 256, 384}. - Proposal 10: With the same rank, the payload size of lower layers should be larger than that of higher layers to maximize the system throughput. - Proposal 11: To limit the number of parameter combination configurations, consider candidate payload patterns {a, a, a}, {a, b, b} for rank 3 and {a, a, a, a}, {a, a, b, b} for rank 4. - Proposal 12: For the payload configurations, consider the following 8 parameters combinations configurations. Rank 1; Rank 2; Rank 3; Rank 4. PC1; 64; 64, 64; 64, 64, 64; 64, 64, 64, 64. PC2; 128; 128, 128; 128, 64, 64; 64, 64, 64, 64. PC3; 192; 192, 192; 256, 64, 64; 128, 128, 64, 64. PC4; 192; 192, 192; 128, 128, 128; 128, 128, 64, 64. PC5; 256; 256, 256; 256, 128, 128; 192, 192, 64, 64. PC6; 256; 256, 256; 256, 128, 128; 128, 128, 128, 128. PC7; 384; 384, 384; 256, 256, 256; 192, 192, 192, 192. PC8; 384; 384, 384; 384, 192, 192; 256, 256, 128, 128. Proposal 13: For payload determination of AI/ML CSI feedback, support the quantization parameters (e.g., d, Q, or L) of each layer to be directly configured by the network. |
| OPPO | Proposal 1: Support SQ only with L=1 and fixed Ql,v=2 for specified quantization parameter combinations. Proposal 2: Support {a, a, b} and {a, a, b, b} as the only CSI feedback payload alternative for rank 3 and rank 4, respectively Proposal 3: Support exchanged non-specified quantization parameter combinations along with dataset and quantization codebook (Alt 1) in Option 4-1. Proposal 4: Regarding CSI feedback payload determination, three options of PC configured by NW can be considered for further down-selection: - Option 1: one PC for all layers and ranks - Option 2: one PC for each layer - Option 3: one PC for layer 1, and another PC for payload ratios between other layers and layer 1 |
| LGE | - Consider the methods for association between pairing ID and payload configuration (e.g., via signaling procedure for applicable functionality reporting). - For scalability of payload size and restriction on potential parameter combinations of (d, Q), consider to introduce common/fixed values of Q over rank/layer, where Q is the number of quantization bits and d is the latent message size. - Support rank-specific latent dimension (i.e., dzν) to ensure flexibility of payload size, enabling payload adaptation for different rank, where ν denotes rank. - Support Q=1 of the parameter combinations (PCs) for payload determination. - Proposal #8: Regarding quantization codebook used for quantizing the CSI feedback in inter-vendor collaboration Direction A, support Alt1 (NW trained and exchanged) over Alt2 (fixed codebook). - Proposal #9: Regarding exchange of quantization codebook for CSI feedback, support vector quantization (VQ). - Proposal #10: Further study on following quantization alignment methods. - Via model identification procedure - Via dataset {target CSI, CSI feedback} exchange, where data format for CSI feedback may be specified as - codeword index of NW-determined codebook with additional signaling for quantization configuration (e.g., codebook exchange) - unquantized version with indication of NW’s selection from standardized (3GPP-specified) quantization codebooks |
| KT | - Proposal 3: For a limited set of payload parameter combinations for AI/ML-based CSI compression, reference the payload size range derived from the enhanced Type II codebook with N_3≤13 to ensure that the AI/ML-based CSI compression payload remains comparable in scale to practical 5G configurations. |
| Samsung | Proposal#4: Support scalar quantization codebooks catered for different dynamic range zmin,zmax output of the latent message zjl for j=0,…,dzl,v-1. - Support range [-1,1] and [0,1] and uniform quantization - Support uniform number of quantization bits, i.e., Q=Ql,v for all l,v. At least Q=2 is supported. - For direction A, network exchanges at least one of the specified quantization codebooks along dataset and/or parameter. - For Direction C, the quantization codebook is determined in relation to last layer activation function and Q. Proposal#5: Support the following scalar quantization codebooks catered for the different dynamic range for elements of the latent message zmin,zmax Quantization Table for Q=2, zmin,zmax=-1,1 - Quantized value; - Codeword. - -3/4; - ‘00’. - -1/4; - ‘01’. - 1/4; - ‘10’. - 3/4; - ‘11’. Quantization Table for Q=2, zmin,zmax=0,1 - Quantized value; - Codeword. - 1/8; - ‘00’. - 3/8; - ‘01’. - 5/8; - ‘10’. - 7/8; - ‘11’. Proposal#5: Regarding quantization codebooks, support Alt2: Use a standardized quantization codebook decided by RAN1. - For Direction C, the quantization codebook is determined along the specified reference model - For Direction A, the network indicates one of the specified quantization codebooks along dataset/parameter Proposal#7: Considering the agreed principles for payload size determination, do not support multiple pair of values for dl,ν,Ql,ν that would result in the same payload size Xl,v=dl,ν×Ql,ν. Proposal#8 For payload size determination, support a single value for Ql,ν, i.e., Q=2. - Support various latent message sizes dl,ν∈32, 48, 64, 96, 128, 192 to support various payload sizes Xl,v=dl,ν×Q. Proposal#9: For payload size determination, support parameter values d1,1∈32, 64, 96, 128, 192 , Q=2 and the total payload size across layers of rank v - X1=d1,1×Q for v=1 - 2X1 for v=2,3,4. Proposal#10: For per-layer payload size determination, set the following constraints - l=1vXlv=2X1 and Xlv=Q×dl,v where Q=2, and dl,v∈32, 48, 64, 96, 128, 192. - Support additional value dl,v=48 for v=2,3,4. Proposal#11: For payload size determination, considering the constraints l=1vXl,v=2X1 and Xl,v=Q×dl,v where Q=2, and dl,v∈32, 48, 64, 96, 128, 192, support at least the following payload sizes {d1,1,Q}; Rank1 {X1}; Rank2 {X1, X1}; Rank3 {X13, X23, X33}; Rank4 {X14, X24, X34, X44}. PC1; {32, 2}; {64}; {64,64}; -; -. PC2; {64, 2}; {128}; {128, 128}; {64, 96, 96}; {64, 64, 64, 64}. PC3; {96, 2}; {192}; {192, 192}; {96, 96, 128}; {96, 96, 96, 96}. PC4; {128, 2}; {256}; {256, 256}; {128, 192, 192}; {128, 128, 128, 128}. PC5; {192, 2}; {384}; {384, 384}; {192, 192, 384}; {192, 192, 192, 192}. Note here to support the payload size Xlv=96 with Q=2, the value dl,v=48 should be supported. |
| Fujistu | Proposal 2: - For payload PCs, RAN1 to specify three parameter combinations of {dzl,ν ,Ql,ν, L } or {dzl,ν ,Ql,ν} for low/mediate/high resolutions. Proposal 3: - Regarding the specific configuration of payload PC, RAN1 to consider the following table for PCs. SQ; Rank1; Rank2; Rank3; Rank4. PC1; {64,2}; {64,2}, {64,2}; {64, 2}, {32,2}, {32,2}; {32,2},{32,2}, {32,2}, {32,2}. PC2; {128,2}; {128,2}, {128,2}; {128,2}, {64,2}, {64,2}; {64,2},{64,2}, {64,2}, {64,2}. PC3; {192,2}; {192,2}, {192,2}; {192,2}, {96,2}, {96,2}; {96,2},{96,2}, {96,2}, {96,2}. VQ; Rank1; Rank2; Rank3; Rank4. PC1; {64,4,2}; {64,4,2}, {64,4,2}; {64,4, 2}, {32,4,2}, {32,4,2}; {32,4,2},{32,4,2}, {32,4,2}, {32,4,2}. PC2; {128,4,2}; {128,4,2}, {128,2}; {128,4,2}, {64,4,2}, {64,4,2}; {64,4,2},{64,4,2}, {64,4,2}, {64,4,2}. PC3; {192,4, 2}; {192,4,2}, {192,4,2}; {192,4,2}, {96,4,2}, {96,4,2}; {96,4,2},{96,4,2}, {96,4,2}, {96,4,2}. Proposal 4: - With the consideration of model scalability, the payload PC should be configured separately from pairing ID. Proposal 5: - For inter-vendor collaboration Direction A, RAN1 to support Alt.1 for quantization codebook. - Alt.1: Exchange quantization codebook from NW side to UE side along with exchanged dataset or model parameters. |
| Google | Proposal 7: Do not support to specify the quantization scheme. - The quantization operation can be exchanged during inter-vendor collaboration procedure |
| Nokia | Proposal 4: Adopt scalar quantization as the quantization method for AI/ML-based CSI compression. Proposal 5: Adopt the following scalar quantization configurations as the N=6 per-layer payload configurations dl,ν,Ql,ν: {32, 1}, {32, 2}, {64, 2}, {128, 2}, {96, 3}, and {192, 2}. Proposal 6: For the payload size configuration for inference, if unequal payload sizes per layer are supported in the parameter combinations, specify a fixed reporting order of the layers of a codeword, such that, for example, the layers are reported in order of decreasing strength. Proposal 7: Consider the adoption of incremental per-layer payload configurations for ranks greater than 1 for layers other than the first CSI layer. |
| Lenovo | Proposal 8: At least support scalar quantization method for the CSI bit generation. Proposal 9: RAN1 should specify the codebook for latent message quantization to allow the case that CSI quantization is not part of the UE-side model encoder. Proposal 10: Quantization codebook parameter, Q^(l,ν) and L is configured by the NW according to UE capability it latent message quantization is not part of the model inference. Proposal 11: To support UEs with different capabilities, support multiple PCs for the codebook with a similar payload size. Proposal 13: For inter-vendor collaboration Direction A, support exchange of quantization codebook from NW-side to UE-side. |
| ETRI | Proposal 2: For AI/ML-based CSI compression, prioritize to consider the vector quantization and consider the scalar quantization in the framework of vector quantization, if needed. Proposal 3: For AI/ML-based CSI compression, consider the trained quantization codebook and its exchanges to enable inter-vendor training collaborations. |
| HONOR | Proposal 1: Each combination of dzl,ν, Ql,ν and L can be applicable to a certain set of subband and port configurations, e.g., the size of Nsubband×Nport are the same or at least similar. Proposal 3: Allocating higher payload sizes to weaker layers would be more beneficial in terms of maximizing average SGCS. Proposal 4: For Quantization methods, SQ is preferred. - FFS: whether/how to support fixed vector quantization codebook |
| Ofinno | Proposal #1: Regarding target CSI type for AI/ML-based CSI compression (Case 0), support layer-common and rank-specific configuration of d_z^(l,v). Proposal #2: Regarding a formulation of quantization for latent message, support SQ only. Proposal #3: Adopt Alt 1 (NW trained and exchanged) for quantization codebook configuration. Proposal #4: Regarding latent-based CSI quantization, consider a two-part quantization method on latent vector by splitting coarse part and finer part for each component of the latent vector. |
| Apple | Proposal 5: Supports VQ as the only quantization method for latent quantization: - Support VQ with dl,ν, L=4,Ql,ν=8. - The jointly trained VQ is signaled in inference configuration with/via pairing ID. Proposal 6: Support number of payload sizes to be 4, e.g., dl,ν∈{32, 64, 128, 196}. Proposal 7: Support number of payload configurations to be 4, e.g., payload size for respective layers of each rank is from dl,ν∈{32, 64, 128, 196}. |
| MTK | Proposal 8. Support scalar quantization for quantizing the output of the encoder in the inference stage if down selection is needed. Proposal 9. Prioritize Alt 2, Use a standardized quantization codebook decided by RAN1 at least for SQ. Proposal 10: Based on the candidate values of Q^(l,ν), deprioritize L=d^(l,ν) for VQ Proposal 11: In accordance with observations captured in TR 38.843, prioritize choosing more combinations of d^(l,ν),Q^(l,ν), and L which lead to small (X) and medium (Y) payload sizes rather than large (Z) payload sizes. Proposal 12: Support selecting common combinations of d^(l,ν),Q^(l,ν), and L across different layers as much as possible to ease training rank-common layer-common scalable AI/ML model. Proposal 13: For per-layer 〖{d〗^(l,ν),Q^(l,ν)} values supported by rank-common and layer-common model with scalar quantization, prioritize {32,1}, {32,2}, and {96,1} with 32 bits, 64 bits, and 96 bits for small payload sizes. Proposal 14: For per-layer 〖{d〗^(l,ν),Q^(l,ν)} values supported by rank-common and layer-common model with scalar quantization, prioritize {64,2} and {192,1} with 128 bits and 192 bits for medium payload sizes. Proposal 15: For per-layer 〖{d〗^(l,ν),Q^(l,ν)} values supported by rank-common and layer-common model with scalar quantization, {128,2} with 256 bits, {192,2} with 384 bits for large payload sizes. |
| Quectel | Proposal 2: - Consider the payload information in the two-sided model pairing or training. |
| Qualcomm | Proposal 1: Support both VQ and SQ with exchanged codebook and SQ with standardized codebook in the following mechanism - For a certain pairing ID, only SQ or VQ is supported, but not both - For a certain pairing ID, NW exchanges the codebook of either SQ or VQ; if NW does not exchange any codebook, the standardized SQ codebook is used Proposal 2: For the payload configurations for VQ and SQ - For VQ, support fixed segment size L=2 and codebook size with Q=4 bits per segment. - For SQ, support Q=2 per scalar. - Support common codebooks for all segments / scalars and for all payload options. - Proposal 3: Support N=4 or 5 per-layer pairs / triplets d={32,64,96,128,192} as candidates for determining the parameter combinations across layers and ranks. Proposal 4: Consider following principles for allocating payload across layers: • Even allocation across layers if feasible; otherwise, more bits can be allocated to higher layers. • For a PC, total number of parameter pairs for a certain layer across all ranks is no more than 2. • The per-layer parameter pair is chosen from the identified options, i.e., {32, 2}, {64, 2}, {96, 2}, {128, 2} and {192, 2}. Proposal 5: Support Y=4 parameter combinations as in Table. SQ table; Rank1; Rank2; Rank3; Rank4. PC1; {32, 2}; {32, 2}, {32, 2}; {32, 2}, {32, 2}, {32, 2}; {32, 2}, {32, 2}, {32, 2}, {32, 2}. PC2; {64, 2}; {64, 2}, {64, 2}; {32, 2}, {32, 2}, {64, 2}; {32, 2}, {32, 2}, {32, 2}, {32, 2}. PC3; {128, 2}; {128, 2}, {128, 2}; {64, 2}, {64, 2}, {128, 2}; {64, 2}, {64, 2}, {64, 2}, {64, 2}. PC4; {192, 2}; {192, 2}, {192, 2}; {96, 2}, {96, 2}, {192, 2}; {96, 2}, {96, 2}, {96, 2}, {96, 2}. Proposal 6: For Rel-20 NR CSI feedback via two-sided model, RAN1 consider further define the applicability of each payload configuration to a certain range of subband / port configurations. E.g., 64 / 128 bits per layer are only applied to <=32 ports, 384 bit per layer is only applied for >32 ports. Proposal 7: Further model applicability on subband, port and payload configuration can be aligned during applicability enquiry phase. |
| Transsion Holdings | Proposal 3: Regarding configuration of scalar quantization, the following can be aligned between Network and UE: - The number of bits per latent dimension, i.e., Ql,ν - The minimum and/or maximum range of quantization Proposal 4: Regarding configuration of vector quantization, the following can be aligned between Network and UE: - The number of bits per segment, i.e., Ql,ν - Segment size, i.e., L - The size of codebook Proposal 5: Regarding quantization codebook for inter-vendor collaboration Direction A, support Alt2: - Alt2: Use a standardized quantization codebook decided by RAN1. Proposal 6: Regarding codebook design of vector quantization, the following alternatives can be considered: - Alt-1: A unified codebook is designed for all the segments of latent vector - Alt-2: A dedicated codebook is designed for each segment of latent vector |
| DCM | Proposal 4 - The selected parameter combinations for AI/ML CSI payload size should be comparable to that of the Rel-16 eTypeII codebook with eTypeII parameter combinations. Proposal 5 - Introduce 5 per-layer parameter combinations. Among them, - 4 per-layer parameter combinations result in (64, 128, 256, 384) bit payload size, which is comparable to the payload size of eTypeII codebook and can be used for rank 1 or rank 2. - 1 per-layer parameter combination results in 32 bit payload size, which can be used for partial layers for rank 3 or rank 4 and will not be used for rank 1 or rank 2. - For the parameter combination table for all ranks, consider the following design, - For each row of PC, the per-layer PC for rank 1 and rank 2 is reused for partial layers of rank 3 and rank 4. - The remaining layers for rank 3 and rank 4 adopt the per-layer PCs with a smaller payload size. Per-layer PC (PLPC); dl,ν,Ql,ν; Per-layer payload size. 1; {32, 1}; 32. 2; {32, 2}; 64. 3; {64, 2}; 128. 4; {128, 2}; 256. 5; {192, 2}; 384. Table 3: An example for PCs to decide the total payload size in bit PC; Rank 1; Rank 2; Rank 3; Rank 4. PLPC; Total size; PLPC; Total size; PLPC; Total size; PLPC; Total size. 1; (2); 64; (2, 2); 128; (2, 1, 1); 128; (2, 1, 1, 1); 160. 2; (2); 64; (2, 2); 128; (2, 2, 1); 160; (2, 2, 1, 1); 192. 3; (3); 128; (3, 3); 256; (3, 2, 2); 256; (3, 2, 2, 2); 320. 4; (3); 128; (3, 3); 256; (3, 3, 2); 320; (3, 3, 2, 1); 352. 5; (4); 256; (4, 4); 512; (4, 3, 3); 512; (4, 3, 3, 3); 640. 6; (4); 256; (4, 4); 512; (4, 4, 3); 640; (4, 4, 3, 2); 704. 7; (5); 384; (5, 5); 768; (5, 4, 4); 896; (5, 4, 4, 4); 1152. 8; (5); 384; (5, 5); 768; (5, 5, 4); 1024; (5, 5, 4, 3); 1152. |
| Rakuten | Proposal 1: RAN1 is requested to study a unified quantization codebook reporting mechanism for efficient network operations under the case if adoption of both SQ&VQ is agreed upon. |

## Discussion

In this section, proposals and discussions are elaborated based on companies’ proposals. The discussion will be carried out by quantization, payload configuration for rank-1 and rank > 1 separately.

### Quantization type and codebook:

RAN1 #122bis R19, are captured below (TR38.843, section 6.2.2.8).

From the perspective of quantization methods for CSI feedback, - For the quantization awareness for training, it is beneficial to consider quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) or jointly updated quantization method/parameters (Case 2-2) to avoid severe performance degradation. In particular, it is more beneficial in performance for Case 2-2 over Case 2-1 under vector quantization format (VQ). - For the quantization format, VQ format achieves comparable performance with scalar quantization format (SQ) in general, where VQ achieves better performance than SQ in some cases while worse in some other cases.

Agreement For inter-vendor collaboration Direction C, - Use standardized quantization codebook For inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing), - Standardize configuration(s) of quantization codebook, e.g., scalar or vector quantization, segment size of VQ, codebook size. - FFS: applicability of the above for Case 2 - Exchange quantization codebook of (the selected) standardized configuration(s) from NW-side to UE-side along with each exchanged dataset or model parameters. - FFS: whether quantization codebook may be different across different payload size configurations

Most companies refer to the observations in Rel-18 study. QC provides evaluation on SQ vs. VQ comparisons. And the results show minor performance gain.

Based on the proposals in the tdoc, companies’ views on quantization type are summarized as follows

- SQ only / prioritized / at-least (12)

- Ericsson, Xiaomi, OPPO, Samsung, Nokia, Ofinno (two-part quantization, coarse + finer resolution), CMCC, ZTE, Lenovo, HONOR (VQ need fixed CB), MTK, CATT

- Both SQ and VQ (6)

- vivo (L=2), QC (L=2, with evaluation), Huawei (with multiple resolution (SQ) or master codebook (VQ)), NEC, LGE, Fujistu

- VQ with prioritization (3)

  - L=d and Q=10: Futurewei

    - deprioritization from MTK

  - VQ only with L=4 Q=8: Apple

  - Prioritize VQ: ETRI (SQ is under VQ)

The situation is similar to the last meeting. SQ are more popular than VQ. Based on the offline  discussion in the last meeting, some companies have concern on the VQ due to the limited performance benefit with the cost of additional complexity. Besides, among the VQ fans, the preferred VQ settings are diverge, more discussion is needed. For progress, let us focus on SQ to proceed to payload parameter discussion. The necessity of VQ can be further studied and discussed at a later stage.

Companies’ views on quantization codebook exchange are summarized as follows:

- Alt1 (exchanged): 16

  - Futurewei, InterDigital, Ericsson, vivo, CMCC, Huawei (can be via RRC), ZTE, OPPO, LGE, Fujistu, Google, Lenovo, ETRI, Ofinno, Apple, QC

- Alt2 (specified, single CB): 5

  - Spreadtrum, HONOR, MTK, QC, Transsion Holdings

- Hybrid: 1

  - SS (2 CBs based on activation function, NW exchange the codebook index, #1 or #2)

Majority companies are interested in using learnable codebook considering its flexibility and potential in the field although the performance gain offered by learnable SQ codebook vs. fixed SQ codebook is not significant as commented by some companies in the last meeting. On the other hand, if the codebook choice is upto NW decision, it seems no harm to include a specified codebook as another option.

##### Proposal 2.1 (closed):

Support NW optionally exchanges codebook to the UE side.

- If NW exchanges a SQ codebook, the exchanged SQ codebook is used for this pairing ID.

- If NW does not exchange the codebook, a standardized SQ codebook is used for this pairing ID.

- FFS whether / how VQ is supported

- Note: a common codebook is used for all the scalars, all the layers and all parameter pairs with same quantization resolution Q.

| Companies | Comments |
| --- | --- |
| OPPO | Support. Other than optionally exchange of codebook, we think non-specified parameter combination can also be exchanged optionally. Reasons are given as follows: - The mapping between CSI feedback payload Xn and its associated pair of {dl,v,Ql,v} or triplets of {dl,v,L,Ql,v} can be exchanged along with the dataset and quantization codebook (Alt 1) in Option 4-1. By this way, different NW-vendors may have the freedom and flexibility to implement different kinds of parameter combinations, e.g., VQ, SQ with different dl,v, Ql,v values, which is potential to achieving a higher performance suitable for local region than specified alternatives. Then UE-side can use the exchanged non-specified quantization parameter combination, instead of the specified ones for sequential model training and inference. |
| Futurewei | Not support. One of the key motivations for AI/ML-based CSI spatial/frequency compression is to reduce feedback overhead. However, we don’t see how SQ alone can achieve that goal. On the other hand, VQ can achieve significant reduction on feedback overhead. Although our preference is to support VQ only, for the sake of progress, we are open to support both SQ and VQ. Therefore, we suggest modifying the proposal as follows: ##### Updated Proposal 2.1 (VQ vs SQ, high priority): Support NW optionally exchanges codebook to the UE side. - If NW exchanges a SQ or VQ codebook, the exchanged SQ or VQ codebook is used for this pairing ID. - If NW does not exchange the codebook, a standardized SQ codebook is used for this pairing ID. - FFS whether / how VQ is supported - Note: a common codebook is used for all the scalars, all the layers and all parameter pairs with same quantization resolution Q. |
| Apple | Our results indicate clear performance gain for VQ over SQ. Another limitation for SQ is that it achieves its best performance when Q=2 for the same payload size. This has been shown in multiple companies’ simulation results. This leaves implementation with no potential trade-off option among complexity and performance. For VQ, although we prefer single L value, we see different L values offers a graceful trade-off between complexity and performance, e.g., a larger L leads to better performance at the cost of higher complexity. As such VQ is a more extensible design than SQ. More trainable parameters in VQ also makes it more scalable. We have quickly tested one trained model/parameters supporting four payload sizes. Results are shown in the below figure. We observe the performance advantage of VQ compared to SQ is widened. As such, progress with SQ prioritized does not seem to be a technically reasonable choice. |
| Samsung | We are generally fine with the proposal. But one clarification or study point is whether the exchanged SQ codebook from a set of standardized codebooks. In our contribution we have analysed that it is sufficient to have a uniform quantization over each output of the latent message. If there is a need for NW-exchanged codebook, the exchange is rather to address the different possible range of outputs of latent message [Zmin, Zmax] which results from different implementations for the last layer activation function. Support NW optionally exchanges codebook to the UE side. - If NW exchanges a SQ codebook, the exchanged SQ codebook is used for this pairing ID. - If NW does not exchange the codebook, a standardized SQ codebook is used for this pairing ID. - FFS whether / how VQ is supported - FFS: whether NW exchanges codebook from a specified set of codebooks. - Note: a common codebook is used for all the scalars, all the layers and all parameter pairs with same quantization resolution Q. |
| CMCC | Support. |
| ZTE | For SQ, quantization aware training with fixed quantization method can generally achieve better performance than that with jointly updated quantization method according to TR 38.843. Therefore, there seems no compelling benefit of additionally supporting exchanged SQ codebook on top of standardized SQ codebook. We may consider support standardized codebook first and further study whether exchanged codebook is supported. |
| Huawei, HiSilicon | Agree with Apple and Futurewei, that VQ should be preserved. Unfortunately, in our evaluation with scalable CSI payload size, some SQ configurations cannot converge; some SQ configurations, even converged, achieve much lower performance than VQ. In comparison, any configuration subject to VQ could quickly converge during training. Thus, in our view, VQ should be one option – otherwise, the feature faces a risk that it cannot be supported under some model structures/training methods. Support Futurewei’s changes. |
| InterDigital | Generally ok with the proposal, however, suggest the following for better clarity, Support NW optionally exchanges codebook to the UE side. - If NW exchanges a SQ codebook, the exchanged SQ codebook is associated to the used for this pairing ID. - If NW does not exchange the codebook, a standardized SQ codebook is associated to the used for this pairing ID. - FFS whether / how VQ is supported - Note: a common codebook is used for all the scalars, all the layers and all parameter pairs with same quantization resolution Q. |
| Nokia | If we support exchanging SQ/VQ codebooks, there is no need to limit the number of exchanged codebooks to 1, as they can be linked to the payload configuration of the CSI feedback set in the dataset |
| NEC | OK with InterDigital’s updates. |
| SPRD | Does the current proposal imply that both SQ and VQ are supported? If that's the case, the quantization method is overly complex. We can choose either one based on the perspective of balancing simulation performance and complexity, and take it as the sole solution. |
| Ofinno | Support the proposal. While VQ method may improve compression efficiency to some extent, it also causes increased complexity, codebook training, and challenges in inter-vendor alignment due to jointly quantizing a group of consecutive real values of the latent messages. Considering these trade-offs, SQ can serve a reasonable method to ensure simplicity and inter-operability. |
| CATT | Generally fine with the direction. Based on the study conclusion in Rel-18 and simulations summarized by FL, the performance of scalar quantization and vector quantization are comparable. Considering the fact that SQ has much less implementation complexity and specification effort, we support to progress on SQ. |
| vivo | If the codebook can be exchanged, we don’t see why the VQ codebook cannot be exchanged as well. We prefer either to support a specified SQ only, or to allow codebook exchange for both SQ and VQ |
| ETRI | We also agree that VQ should be preserved in this stage. |
| Xiaomi | We share same view with ZTE. If SQ is support, it is not necessary to support exchanged SQ codebook. |
| Ericsson | We are fine with the proposal. |
| LG Electronics | Agree with Huawei, Apple and Futurewei. The motivation of exchanging the quantization codebook is to achieve higher performance. However, scalar quantization (SQ) shows lower performance than vector quantization (VQ). In this regard, VQ has to be supported especially for exchange of quantization codebook of CSI feedback. In other words, only a fixed codebook may be sufficient for SQ. Thus, VQ is one of the main premises for the quantization codebook exchange. |
| TCL | Generally fine. The benefit for exchanging the codebook should be clarified. |
| Fujitsu | We support codebook exchange for both SQ and VQ. |

Based on the discussion in the offline session, there are good number of companies supporting following options.

- VQ with exchanged codebook:

  - Support: LGE, Huawei, Apple, Futurewei, Fujistu, QC, Lenovo, OPPO, vivo, ETRI

  - Concern: Samsung, Xiaomi,

- SQ with exchanged codebook based on NW development:

  - Support: LGE, Ofinno, QC, Fujistu

  - Concern: Samsung, Huawei, ZTE, Xiaomi, vivo, MTK

- SQ with one or more specified codebook with exchanged index

  - Support: Nokia, Samsung, ZTE, CATT, DCM, MTK, Ericsson, Lenovo, vivo, OPPO, QC

  - Concern: Apple, Huawei, LGE, Fujistu,

Thus, it is widely accepted that standardized quantization is sufficient to provide a good performance if SQ is supported, while the exchanged codebook is beneficial for VQ. Additionally, VQ proponents need a bit more time to converge and convince others. With above observation, following proposal captures the situation and agreed during the online discussion.

##### Proposal 2.1a (closed):

Support NW optionally exchanges codebook to the UE side.

- If NW exchanges a codebook, the exchanged codebook is associated to the used for this pairing ID.

  - FFS SQ / VQ / both is supported

  - FFS the details, e.g., whether exchange the NW-defined codebook or the index of a standardized codebook

- If NW does not exchange the codebook or an index, a standardized SQ codebook is associated to the used for this pairing ID.

- Note: for the same quantization resolution Q, a common codebook is used for all the scalars, all the layers and all parameter pairs for the associated pairing ID with same quantization resolution Q.

Agreement:

Support NW side optionally exchanges codebook to the UE side.

- If NW side exchanges a codebook, the exchanged codebook is associated to the pairing ID.

  - FFS SQ / VQ / both is supported

  - FFS the details, e.g., whether exchange the NW-defined codebook or the  ID of a standardized SQ codebook in case of multiple standardized codebooks.

- If NW side does not exchange a codebook or an ID of a standardized SQ codebook in case of multiple standardized codebooks, a standardized SQ codebook is associated to the pairing ID.

- Note: For the same quantization resolution, a common codebook is used for all the scalars/segment(s), all the layers of pre-coding matrix and all parameter pairs/triplets for the associated pairing ID.

### Per-layer parameter pairs

##### Evaluation results:

Ericsson: “Overall, we can observe that Q=2 generally gives a balanced tradeoff between overhead and performance for all ranks. For the same number of bits given by d*Q, increasing Q while fixing d is typically not as good as increasing d while fixing Q.”

Vivo: “For the same payload size, increasing d yields far greater gains than increasing Q.

- Case 1{multiple d, single Q} incurs a small performance loss (less than 2%) for small payloads due to truncation-based payload scalability model compared with case 2{single d, multiple Q}

- Case 1{multiple d, single Q} has a moderate performance gain (~5.53%) for medium or large payloads compared with case 2{single d, multiple Q}”

| Case 1(baseline): {multiple d, single Q} | {32, 2} | 0.78535 | 0.65379 | 0.5509 | 0.47537 |
| --- | --- | --- | --- | --- | --- |
|  | {48, 2} | 0.82776 | 0.70971 | 0.60929 | 0.53004 |
|  | {64, 2} | 0.84851 | 0.74164 | 0.64519 | 0.56581 |
| Case 2: {single d, multiple Q} | {32, 2} | 0.79614(1.37%) | 0.66615(1.89%) | 0.56169(1.96%) | 0.48369(1.75%) |
|  | {32, 3} | 0.83145(0.45%) | 0.70677(-0.41%) | 0.60393(-0.88%) | 0.5234(-1.25%) |
|  | {32, 4} | 0.84155(-0.82%) | 0.71825(-3.15%) | 0.61589(-4.54%) | 0.53454(-5.53%) |

Huawei: “In terms of SGCS, for VQ, the pool composed with fewer d values and more Q/L values is better than the pool composed with more d values and fewer Q/L values”

- Option 1: {32, 1}, {32, 2}, {96, 1}, {32, 4}, {96, 2}, {96, 4}

- Option 2: {32, 1}, {64, 1}, {32, 3}, {128, 1}, {64, 3}, {128, 3}

CATT: “It can be observed that the performance of Q=2 is higher than Q=3 and Q=4 under the similar payload sizes in all layers.”

Xiaomi: “ Ql,ν=2 achieves better trade-off between performance and overhead compared to Ql,ν=3 and 4.”

ZTE: “With the same scalable per-layer payload size setting of {64, 128, 256}, scalability approach achieved by specific output linear layer for each payload configuration achieve 1.96%~7.57% layer 1 SGCS performance gains and 2.95%~13.32% layer 2 SGCS performance gains over scalability approach achieved by varying quantization parameters”

| Payload size | Case 1: SQ d=64, Q={1, 2, 4} | Case 2: SQ d={32, 64, 128}, Q=2 | Case 3: VQ d={32, 64, 128}, Q=8, L=4 | Case 4: VQ d={128, 256, 512}, Q=8, L=16 |
| --- | --- | --- | --- | --- |
| 64 bits | Layer 1: 0.7118 Layer 2: 0.5820 | Layer 1: 0.7427 Layer 2: 0.6078 | Layer 1: 0.7541 Layer 2: 0.6217 | Layer 1: 0.7041 Layer 2: 0.5784 |
| 128 bits | Layer 1: 0.8053 Layer 2: 0.6957 | Layer 1: 0.8211 Layer 2: 0.7162 | Layer 1: 0.8260 Layer 2: 0.7237 | Layer 1: 0.7958 Layer 2: 0.6919 |
| 256 bits | Layer 1: 0.8191 Layer 2: 0.7133 | Layer 1: 0.8811 Layer 2: 0.8083 | Layer 1: 0.8842 Layer 2: 0.8117 | Layer 1: 0.8714 Layer 2: 0.7943 |

LGE: “Regarding parameter combinations (PCs) for payload determination, there exist cases where Q=1 outperforms the others, given a fixed payload size”, “In order to support small payload sizes (e.g., 32bits or 64 bits per layer), Q=1 seems to be practical and useful”

Nokia: “We propose to select the following pairs: {26, 1}, {26, 2}, {64, 2}, {120, 2}, {104, 3}, and {208, 2} for N=6 pairs. Using the closest candidate values for the latent space dimension, these pairs become {32, 1}, {32, 2}, {64, 2}, {128, 2}, {96, 3}, and {192, 2} with per layer overheads ranging from 32 to 384 bits”

Lenovo: “For SQ in case 1 and case 2, with a same payload size, larger Q^(l,ν) has dominant SGCS performance. For example, {d_z^(l,ν),Q^(l,ν) }={32,4} is obviously better than {d_z^(l,ν),Q^(l,ν) }={64,2} in both Table 1 and Table 2. For SQ in case 3, with a same payload size, larger d_z^(l,ν) has dominant SGCS performance.”

MTK:

| Table ‎2-1: SGCS performance for various candidate combinations of {dl,ν,Ql,ν} Ql,v. 1; 2; 3; 4. dl,v; 32; 0.682; 0.740; 0.770; 0.779. 64; 0.734; 0.799; 0.831; 0.838. 96; 0.773; 0.835; 0.873; 0.883. 128; 0.789; 0.866; 0.897; 0.909. 192; 0.831; 0.903; 0.934; 0.939. | Table ‎2-2: Payload size for various candidate combinations of {dl,ν,Ql,ν} Ql,v. 1; 2; 3; 4. dl,v; 32; 32; 64; 96; 128. 64; 64; 128; 192; 256. 96; 96; 192; 288; 384. 128; 128; 256; 384; 512. 192; 192; 384; 576; 768. |
| --- | --- |

- “Within small payload sizes (less than 100 bits), for payload sizes with 64 bits, {32,2} results slightly better SGCS than {64,1}, and for payload sizes with 96 bits, {96,1} slightly outperforms {32,3}.”

- “Within medium payload sizes (between 100 and 230 bits), for payload sizes with 128 bits, {64,2} results higher SGCS than {32,4} and {128,1}. For payload sizes with 192 bits, {32,3} and {192,1} have similar performance.”

- “Within large payload sizes (more than 230 bits), for payload sizes with 256bits {128,2} outperforms {32,4}, for payload sizes with 384 bits, {192,2} outperforms {96,4} and {128,3}.”

- “Within large payload sizes (more than 230 bits), there are four unique payload sizes with 288, 512, 576, and 768 bits. {128,4} with 512 bits results less SGCS than {192,2} with 384 bits. {96,3} with 288 bits results similar SGCS as {128,2} with 256 bits. {192,4} with 768 bits has the similar performance as {192,3} with 576 bits.”

- “In general, for similar payload sizes, better performance observed using smaller values for Q^(l,ν) and larger values for d_z^(l,ν) with some exception for Q^(l,ν)=1.”

Qualcomm: “For SQ, the best payload options are achieved by setting Q=2 bits per real entry. Different payload is achieved by setting a respective latent dimension dz.”

Observations from companies’ evaluations:

- Changing d fixing Q > than changing Q fixing d, and Q=2 gives a balanced performance (8):

  - Observed by QC, CATT, Xiaomi, Ericsson, MTK, Lenovo, vivo, Samsung

    - 1 source observes the opposite: Huawei

  - A few exceptions are seen with Q=1 at small payload: MTK, LGE

  - One exception is seen with Q=3 at payload size cannot be achieved by Q=2: Nokia

Other principles:

- Comparable payload to eT2: Huawei, Samsung, Nokia, DCM

  - eT2 rank-1 payload range: 63, 93, 113, 171, 229, 282, 333

- More payload pairs for small (X) and medium (Y) payload sizes rather than large (Z) payload sizes: MTK

- No multiple pair for the same payload size: Samsung, QC

- Number of per layer payload pairs

  - N=3: Fujistu

  - N=4: QC, Apple

  - N=5: Ericsson, QC, ZTE, CATT, Xiaomi, DCM

  - N=6: Samsung, Nokia

Summary of the proposed payload pairs for SQ:

| Payload parameter pairs (assuming SQ) | Payload size | Supported companies | Total number of supporters |
| --- | --- | --- | --- |
| {32, 1} | 32 | LGE, MTK, DCM, Nokia, Huawei | 5 |
| {32, 2} | 64 | Ericsson, CATT, Xiaomi, ZTE, vivo, Huawei, Samsung, Nokia, MTK, DCM, QC, Fujistu | 12 |
| {48, 2} | 96 | Samsung | 1 |
| {96, 1} | 96 | Huawei, MTK | 2 |
| {64, 2} | 128 | Ericsson, CATT, Xiaomi, ZTE, QC, vivo, Samsung, Nokia, MTK, DCM, Fujistu | 11 |
| {96, 2} | 192 | Ericsson, CATT, Xiaomi, ZTE, QC, Huawei, Samsung, MTK | 8 |
| {192, 1} | 192 | MTK | 1 |
| {128, 2} | 256 | Ericsson, CATT, Xiaomi, ZTE, QC, vivo, Samsung, Nokia, MTK, DCM, Fujistu | 11 |
| {96, 3} | 288 | Nokia | 1 |
| {96, 4} | 384 | Huawei | 1 |
| {192, 2} | 384 | Ericsson, CATT, Xiaomi, ZTE, QC, Samsung, Nokia, MTK, DCM, Fujistu | 10 |

From above, it is clear that {32, 2}, {64, 2}, {128, 2} and {192, 2} receives sufficient support with solid evaluation results. These four pairs provides a good span of the payload range of eType II.

Other small to medium payload pairs, {32, 1}, {96, 2} , {48, 2}, {96, 1} and {192, 1} may be also beneficial for payloads of rank 3 and rank 4 by providing a good performance complexity. Their necessity would be clearer during the discussion of parameter combination table. How many of them can be adopted without impacting the scalability performance is also needed to be considered.

The rest payload pairs {96, 3} and {96, 4} seems not providing clear benefit compared to {128, 2} and {192, 2}.

##### Proposal 2.2 (closed):

For SQ, at least support per-layer payload parameter pairs {32, 2}, {64, 2}, {128, 2}, {192, 2}

- Further study whether support per-layer payload parameter pairs {32, 1}, {48, 2}, {96, 1}, {96, 2}, {192, 1}

- [Apple, Huawei]: {d, payload}, {d, Q/L}

| Companies | Comments |
| --- | --- |
| Mod | Intention is to proceed to PC table discussions with the 4 pairs in the main bullet. The necessity of other 5 pairs in the sub-bullets become clearer when discussing the payload pairs for rank 3 and rank 4. |
| OPPO | Support |
| Futurewei | Ok. |
| Samsung | Ok with FL’s direction. We want to clarify that supporting the values {48, 2} and {96,2} would latter help us the payload size scaling over the reported rank. |
| CMCC | Support. |
| ZTE | Thanks FL for the summary. We are ok with the main bullet. For the subbullet, we prefer to use a common SQ codebook (i.e., Q=2) to simply the spec design and additionally introduce {96, 2} for better spanning of the etype II payload range. |
| Huawei, HiSilicon | Firstly, agree that up to 384 bits should be supported. This corresponds to the scenario of highly precise CSI for near field UEs. Secondly, for minimum value of CSI payload size, we support {32, 1} due to following reasons: - It should be included in the per-layer pool, because for PC#2, assuming rank1/2 is {32, 2}, its corresponding rank3/4 should have a smaller per layer payload of {32, 1} - It should be included in rank1/2 of lowest PC, so that AI CSI could achieve comparable/smaller payload size than the minimum (PC#1) payload size of eType 2. This corresponds to the scenario of small UCI payload size under multiplexing of large UE number, best UL user experience (most preserved UL resources for UL-SCH), and far field UEs. Example: Rank1; Rank2; Rank3; Rank4. PC#1:; {32, 1}; {32, 1}, {32, 1}; N/A; N/A. PC#2:; {32, 2}; {32, 2}, {32, 2}; {32, 2}, {32, 1}, {32, 1}; {32, 1}, {32, 1}, {32, 1}, {32, 1}. …. at least support per-layer payload parameter pairs {32, 1} (only for rank 1/2 of PC#1), {32, 2}, {64, 2}, {128, 2}, {192, 2} Thirdly, this PC combination can be easily extended to VQ pool with Q/L=2, e.g., Q=8, L=4 |
| MTK | Support |
| Ofinno | Generally fine with the proposal. Regarding the observation, some clarification is needed at least the following, since the quantization performance can be based on: - Whether the latent message per layer is normalized - Whether the quantization range is common or not even if Q=2 |
| CATT | support |
| vivo | We are still hesitant about {192, 2}, as introducing {192, 2} may require considering {96, 2} or {48, 2} for rank 3 or rank 4. A larger number of candidate values increases model scalability complexity, and a larger reporting payload size (e.g., 192) typically provides limited performance gain compared to non-AI solutions. Therefore, {192, 2} can be placed in FFS for further study together with {96, 2} and {48, 2}. |
| ETRI | Agree on the main sentence. For the sub-bullet point, we can discuss with the specific parameter values for each PC. |
| Xiaomi | We are fine with main bullet and prefer the common Q, i.e., Q=2 for any latent message size for simple spec design. |
| China Telecom | Support. |
| Ericsson | We are fine with the main bullet. In our view, having Q=1 introduces significant performance loss and unnecessary implementation effort, thus we propose to keep only Q=2. |
| LG Electronics | As we agreed to limit the total number of per-layer payload pairs across ranks/layers, to focus on popular pairs first and remain the other pairs may be critically harmful, for the reason that there will be very little room/opportunity for the other considered payload pairs such as (32,1), (64,1). Thus, we need more sophisticated rules/principles for per-layer pair {d, Q} selection. Based on our simulation results, not only (32,1) but also (64,1) shows better or similar performance than Q=2 case. Table 1. SGCS and {dz, Q} pairs for different payload sizes Payload size (per-layer); Layer 1; Layer 2; Layer 3; Layer 4. SGCS; (d, Q); SGCS; (d, Q); SGCS; (d, Q); SGCS; (d, Q). 32; 0.791; (32,1); 0.753; (32,1); 0.366; (32,1); 0.244; (32,1). 64; 0.826; (64,1); 0.788; (64,1); 0.408; (64,1); 0.282; (64,1). 0.826; (32,2); 0.787; (32,2); 0.414; (32,2); 0.286; (32,2). 96; 0.845; (96,1); 0.809; (96,1); 0.436; (96,1); 0.310; (96,1). 0.847; (32,3); 0.808; (32,3); 0.440; (32,3); 0.307; (32,3). … (omitted). We should recall that one of the main motivation points for AI/ML-based CSI compression is to reduce signaling overhead of CSI feedback, maintaining overall spectral efficiency. In order to support small payload sizes (e.g., 32bits or 64 bits per layer), (32,1) and (64,1) seem to be practical and useful. Therefore, on top of Huawei’s suggestion, we additionally suggest the following change. For SQ, at least support per-layer payload parameter pairs {32, 1}, {64, 1} {32, 2}, {64, 2}, {128, 2}, {192, 2}.. |
| TCL | OK |
| Fujitsu | OK |

Based on the discussion, the proposed payload pairs seem to be stable. The only controversial part lies in whether the pairs apply to SQ only or can be considered for VQ as well. The proposal is revised in an inclusive manner as following and agreed during online discussion.

##### Proposal 2.2a (closed):

For SQ, at At least support per-layer payload parameter pairs {32, 2}, {64, 2}, {128, 2}, {192, 2}

- Further study whether support per-layer payload parameter pairs {32, 1}, {48, 2}, {96, 1}, {96, 2}, {192, 1}

- The above pair refers to the pair of latent dimension and quantization resolution per dimension, i.e., {d, Q} for SQ or {d, Q/L} for VQ (if VQ is supported).

Agreement:

If target CSI type is pre-coder matrix, at least support per-layer payload parameter pairs {32, 2}, {64, 2}, {128, 2}, {192, 2}

- FFS: Whether support per-layer payload parameter pairs {32, 1}, {48, 2}, {96, 1}, {96, 2}, {192, 1}, {128,10/128}

- Note: The above pair refers to the pair of latent dimension and quantization resolution, i.e., {d, Q} for SQ or {d, Q/L} for VQ.

### Payload combinations across layers / ranks

#### Evaluation results:

Four companies provide evaluation results of the payload pair combinations across layers in rank 3 and rank 4 with throughput and spectral efficiency as performance metric.

Ericsson: Allocating more bits to higher layer is better than allocating more bits to lower layer. Allocating more bits to higher layer is slightly better or same as even allocation, except the moderate to significant loss in the case rank 3 with 768 bits.

“Table 2 compares the throughput for PC1X (256 bits total payload size) with two latent space size combinations. As can be seen, {{32, 2}, {32, 2}, {64, 2}} is better than {{64, 2}, {32, 2}, {32, 2}}, meaning that allocating more bits to the higher layers give better performance.”

Table 2:Throughput for different latent space combinations for rank 3 using 256 bit payload (PC1X).

| PCs | d1,3,Q, d2,3,Q, d3,3,Q | Relative Throughput |  |  |
| --- | --- | --- | --- | --- |
|  |  | 20% RU | 50% RU | 70% RU |
| PC1A (reference) | {32, 2}, {32, 2}, {64, 2} | 100.0% | 100.0% | 100.0% |
| PC1B | {64, 2}, {32, 2}, {32, 2} | 96.7% | 92.3% | 89.7% |

“Table 3 compares the throughput for PC2X (384 bits total payload size) with three latent space size combinations. In this case, both more bits to higher layer (i.e., {32, 2}, {32, 2}, {128, 2}) and equal bits allocation (i.e., {64, 2}, {64, 2}, {64, 2}) give similar performance, while allocating more bits to the first layer introduce significant loss.”

Table 3:Throughput for different latent space combinations for rank 3 using 384 bit payload (PC2X).

| PCs | d1,3,Q, d2,3,Q, d3,3,Q | Relative throughput |  |  |
| --- | --- | --- | --- | --- |
|  |  | 20% RU | 50% RU | 70% RU |
| PC2B (reference) | {32, 2}, {32, 2}, {128, 2} | 100.0% | 100.0% | 100.0% |
| PC2C | {128, 2}, {32, 2}, {32, 2} | 95.8% | 90.0% | 83.4% |
| PC2A | {64, 2}, {64, 2}, {64, 2} | 99.7% | 100.3% | 99.9% |

Table 7 Throughput for different latent space combinations for rank 4 using 384 bit payload (PC2X).

| PCs | d1,4,Q, d2,4,Q, d3,4,Q, d4,4,Q | 20% RU | 50% RU | 70% RU |
| --- | --- | --- | --- | --- |
| PC2B (reference) | {32, 2}, {32, 2}, {64, 2}, {64, 2} | 100.0% | 100.0% | 100.0% |
| PC2A | {64, 2}, {64, 2}, {32, 2}, {32, 2} | 93.9% | 93.4% | 93.8% |

“Table 4 compares the throughput for PC3X (512 bits total payload size) with four latent space size combinations. In this case, even payload distribution across all layers is not possible, and allocating more bits to the 3rd layer is most beneficial.”

Table 4:Throughput for different latent space combinations for rank 3 using 512 bit payload (PC3X).

| PCs | d1,3,Q, d2,3,Q, d3,3,Q | 20% RU | 50% RU | 70% RU |
| --- | --- | --- | --- | --- |
| PC3D (reference) | {128, 2}, {64, 2}, {64, 2} | 100.0% | 100.0% | 100.0% |
| PC3B | {64, 2}, {64, 2}, {128, 2} | 101.4% | 102.9% | 104.4% |
| PC3A | {64, 2}, {96, 2}, {96, 2} | 100.5% | 100.9% | 101.1% |
| PC3C | {96, 2}, {96, 2}, {64, 2} | 100.4% | 100.0% | 98.9% |

Table 8 Throughput for different latent space combinations for rank 4 using 512 bit payload (PC3X).

| PCs | d1,4,Q, d2,4,Q, d3,4,Q, d4,4,Q | 20% RU | 50% RU | 70% RU |
| --- | --- | --- | --- | --- |
| PC3B (reference) | {32, 2}, {32, 2}, {64, 2}, {128, 2} | 100.0% | 100.0% | 100.0% |
| PC3A | {32, 2}, {32, 2}, {96, 2}, {96, 2} | 98.9% | 100.3% | 98.8% |
| PC3C | {64, 2}, {64, 2}, {64, 2}, {64, 2} | 97.1% | 98.2% | 97.6% |
| PC3D | {96, 2}, {96, 2}, {32, 2}, {32, 2} | 91.2% | 90.2% | 87.4% |

Table 5 compares the throughput for PC4X (768 bits total payload size) with three latent space size combinations. In this case, even payload distribution across all layers (i.e., {128, 2}, {128, 2}, {128, 2}) gives the best performance.

Table 5:Throughput for different latent space combinations for rank 3 using 768 bit payload (PC4X).

| PCs | d1,3,Q, d2,3,Q, d3,3,Q | 20% RU | 50% RU | 70% RU |
| --- | --- | --- | --- | --- |
| PC4B (reference) | {128, 2}, {128, 2}, {128, 2} | 100.0% | 100.0% | 100.0% |
| PC4A | {96, 2}, {96, 2}, {192, 2} | 97.0% | 91.2% | 91.9% |
| PC4C | {192, 2}, {96, 2}, {96, 2} | 97.4% | 91.5% | 92.4% |

Table 9 Throughput for different latent space combinations for rank 4 using 768 bit payload (PC4X).

| PCs | d1,4,Q, d2,4,Q, d3,4,Q, d4,4,Q | 20% RU | 50% RU | 70% RU |
| --- | --- | --- | --- | --- |
| PC4D (reference) | {128, 2}, {128, 2}, {64, 2}, {64, 2} | 100.0% | 100.0% | 100.0% |
| PC4B | {64, 2}, {64, 2}, {128, 2}, {128, 2} | 106.5% | 108.5% | 113.8% |
| PC4A | {64, 2}, {96, 2}, {96, 2}, {128, 2} | 104.8% | 106.7% | 113.6% |
| PC4C | {96, 2}, {96, 2}, {96, 2}, {96, 2} | 104.6% | 106.6% | 113.6% |

Vivo: allocating more bits to weaker layer is better than other options.

“For SGCS & SE performance of Rank 3 in Table 8, sorted from largest to smallest alternative is {X,X,X}>{X/2,X/2,X}> {X,X/2,X/2}.”

  - Compared to {X,X/2,X/2}, {X/2,X/2,X} can achieve 1.5%~2.5% gain

  - Compared to {X,X/2,X/2}, {X,X,X} can achieve 8%~9% gain

“For SGCS & SE performance of Rank 4 in Table 9, sorted from largest to smallest alternative is {X,X,X,X}> {X/2,X/2,X, X}>{X,X,X/2,X/2}>{X/2,X/2,X/2, X/2}.”

  - Compared to {X/2,X/2,X/2, X/2}, {X,X,X/2,X/2} can achieve 4%~9% gain

  - Compared to {X/2,X/2,X/2, X/2},  {X/2,X/2,X, X} can achieve 9%~15% gain

  - Compared to {X/2,X/2,X/2, X/2},  {X,X,X,X} can achieve 16%~23% gain

Table 8

|  | X= {dl,ν, Ql,ν} | SGCS_Rank1 X | SGCS_Rank2 {X,X} | SGCS_Rank3 |  |  | SE gain_Rank3 relative to {X/2,X/2, X/2} |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | X,X,X | {X,X/2,X/2} | {X/2,X/2,X} | X,X,X | {X,X/2,X/2} baseline | {X/2,X/2,X} |
| PC-0 | {16,2} | 0.69865 | 0.62553 | / | / |  | / | / | / |
| PC-1 | {32, 2} | 0.76934 | 0.70286 | 0.6465 for 192 | 0.5909 for 128 | 0.5949 for 128 | (8%) | (0%) | (1.6%) |
| PC-2 | {64, 2} | 0.84544] | 0.79146 | 0.7415 for 384 | 0.6719 for 256 | 0.6824 for 256 | (8%) | (0%) | (2.5%) |
| PC-4 | {128, 2} | 0.90067 | 0.86388 | [0.82559] for 768 | 0.7599 for 512 | 0.7773 for 512 | (9%) | (0%) | (2.5%) |

Table 9

|  | X= {dl,ν, Ql,ν} | SGCS_Rank4 |  |  |  | SE gain_Rank4 relative to {X/2,X/2, X/2, X/2} |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X,X,X,X | {X/2,X/2, X/2, X/2} | {X/2,X/, X, X} | {X,X, X/2,X/2} | X,X,X,X | {X/2,X/2, X/2, X/2} baseline | {X/2,X/2 X, X} | {X,X, X/2,X/2} |
| PC-0 | {16,2} | / | / |  | / | / | / |  | / |
| PC-1 | {32, 2} | [0.5999] for 256 | [0.5215] for 128 | [0.5613] For 192 | [0.56015] For 192 | (16%) | (0%) | (10%) | (4%) |
| PC-2 | {64, 2} | [0.6971] for 512 | [0.5999] for 256 | [0.6528] for 384 | [0.64421] for 384 | (23%) | (0%) | (15%) | (9%) |
| PC-4 | {128, 2} | [0.7884] for 1024 | [0.6971] for 512 | [0.7522] For 768 | [0.7332] For 768 | (16%) | (0%) | (9%) | (4%) |

Samsung: allocating more bits to weaker layer is better in rank 3, but even allocation is better at rank 4. Overall, the difference is small.

For per payload size distribution of rank=3, among the considered patterns and considering the total payload size across layers set to 2X1, the following patterns provide highest SE in the descending order.

- the distribution 12,34,34 provides highest SE, i.e., X1,3,X2,3,X2,3={12X1, 34X1, 34X1}.

- the distribution 12,12,1 provides highest SE, i.e., X1,3,X2,3,X2,3={12X1, 12X1, X1}.

For per payload size distribution of rank=4, among the considered patterns and considering the total payload size across layers set to 2X1, the following patterns provide the highest SE.

- the distribution 12,12,12,12 provides highest SE, i.e., X1,4,X2,4,X2,4,X4,4=12X1,12X1,12X1,12X1.

Qualcomm: “Allocating more bits to higher layers is beneficial to spectral efficiency. Maintaining a good quality for layer 1 and layer 2 is also important for rank fall back or MU pairing.”

#### Summary on the evaluation and proposals,

Following observations can be drawn from the evaluation results:

- Allocating bits to higher layer is better than allocating bits to lower layer (Ericsson, vivo, Samsung, Qualcomm)

- For rank 4, allocating bits to higher layer is comparable to even allocation (layer-common)

  - 2 companies show allocating bits to higher layer is slightly better (Ericsson, Qualcomm)

  - 1 company show even allocation is slightly better (Samsung)

- For rank 3,

  - 2 companies show allocating bits to higher layer is better than even allocation (Qualcomm, Samsung)

  - 1 company shows even allocation has moderate to significant gain in one specific case, i.e., 768 bits total (Ericsson)

Following principles are proposed based on the tdocs:

- Even split: Huawei, NEC (at least), QC

- Lower layer should be allocated with more bits than higher layer:

  - Huawei (no eval), ZTE (no eval), DCM (no eval)

- Lower layer should be allocated with less bits than higher layer:

  - Ericsson (Throughput), vivo (SE), Samsung (SE), QC (SE), Nokia (no eval), HONOR (no eval), Xiaomi (no eval)

- Fixed reporting order if layer-specific payload is needed: Nokia

- Dedicated pair for rank 3 and 4, but lower than the smallest payload size of rank 1

  - Needed: Huawei, DCM

  - Not support rank ¾ for the smallest payload size: Ericsson

  - Not needed: QC, Samsung

- Try to selection common {d,Q} across layers / ranks:

  - MTK, QC (only two different options), DCM (reuse rank 1/2 payload for part of rank 3/4), OPPO (common options for layer 1 and 2, i.e., {a, a, b} and {a, a, b, b})

- Support 64/128/192 bits per layer for rank1-4, with 256bits per layer only available for rank1 and rank2: CATT

- Rank-specific d, common Q: LGE

- Payload size derived considering N3<=13 as baseline: KT

- Layer-common, rank-specific d: Ofinno

Based on the evaluation and proposals, the key ingredients lie in performance and complexity aspects.

- From performance perspective, although 4 companies show allocating more bits to higher layer is beneficial, there are still some companies mentioning the importance of lower layer considering rank fallback or MU pairing (Huawei, ZTE, DCM, QC).

  - Nokia mentioned that layer-specific payload should be clarified in the spec the ordering of the layers.

- From complexity perspective, UE / chipset vendors elaborate concerns on the number of different payload pairs per PC row, as it impacts the number of hypothesis for rank determination, and impacts the inference complexity including MAC count and model loading.

With above, FL considers the following guidance for the proposed payload combination in the remaining part of this section:

- When even allocation and allocating more bits to higher layer have comparable performance, prioritize the one with lower complexity – upto M=2 payload pairs per row as in eType II.

- When complexity are similar, prioritize the option yields better performance

- Other cases than above need further discussion.

#### PC led by {32, 2}

The pair {32, 2} works as the smallest payload for rank 1. For parameter combination led by this pair, the major controversial part lies in whether and how to support rank 3 and 4. Companies’ views from their tdoc are summarized in the following table (presenting the payload size where 64 and 32 are resulted by {32, 2} and {32, 1}, respectively).

| Companies | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| QC | 64 | 64, 64 | 64, 64, 64 | 64, 64, 64, 64 |
| Ericsson | 64 | 64, 64 | na | na |
| vivo | 64 | 64, 64 | 64, 64, 64 | na |
| Xiaomi | 64 | 64, 64 | 64, 64, 64 | 64, 64, 64, 64 |
| ZTE | 64 | 64, 64 | 64, 64, 64 | 64, 64, 64, 64 |
| Samsung | 64 | 64, 64 | na | na |
| DCM | 64 | 64, 64 | 64, 32, 32 | 64, 32, 32, 32 |
| Huawei | 64 | 64 | 64, 32, 32 | 32, 32, 32, 32 |

It is worth noting that the payload size for rank 3 and rank 4 are not necessarily comparable with rank 2 for this small payload regime. Thus, there are three possibilities, i.e., 1) layer-common and rank-common pairs same as rank 1 and rank 2, and 2) rank 3 and 4 are not applicable, and 3) using {32,1} as another pair to have same payload as rank 2.

##### Proposal 2.3 (closed)

For the PC with rank-1 payload of 64 bits, down-select from following alternatives

| Pair index 1 | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| Alt1 | {32, 2} | {32, 2}, {32, 2} | {32, 2}, {32, 2}, {32, 2} | {32, 2}, {32, 2}, {32, 2}, {32, 2} |
| Alt2 | {32, 2} | {32, 2}, {32, 2} | NA | NA |
| Alt3.1 | {32, 2} | {32, 2}, {32, 2} | {32, 1}, {32, 1}, {32, 2} | {32, 1}, {32, 1}, {32, 1}, {32, 1} |
| Alt3.2 | {32, 2} | {32, 2}, {32, 2} | {32, 2}, {32, 1}, {32, 1} | {32, 1}, {32, 1}, {32, 1}, {32, 1} |

Note: if Alt 3.1 or Alt3.2 is agreed, specify that {32,2} is applied to the 3rd strongest or the 1st strongest layer respectively.

| Companies | comments |
| --- | --- |
| Mod | Companies please share your preference on the alternatives with technical reasons. |
| OPPO | Prefer Alt 1. Since for PC with rank-1 payload of 64bits, the total CSI feedback payload is not very large. Therefore, we prefer the simplest way as given in Alt 1, wherein the layer 1/2 always use the same {32,2} for different ranks. |
| Futurewei | Since this proposal is for SQ, we suggest adding “For SQ” in the beginning of the proposal to make it clear that this is for SQ as shown below. We are fine with Alt1 considering its simplicity. “For SQ, Ffor the PC with rank-1 payload of 64 bits, down-select from following alternatives” |
| Samsung | We prefer Alt2 - Alt3 and Alt4 require the support of Q=1 in addition to Q=2. We want to keep Q=2 as the only supported value to simplify implementation and specification work. Alt1 is not preferred as the payload size scales linearly up to 4x with the reported rank. This is unwanted behaviour as NW likely configure PC for lower overhead when the uplink resources (PUSCH) are constrained. Thus, the additional payload size from higher rank would not be useful. Moreover, if we support rank=1 payload for {64, 2} and {48,2}, the network may configure the {{32,2}, {48, 2}, {48,2}} for rank3 and {{32,2}, {32,2}, 32,2}, {32,2}} for rank=4 to achieve the same payload sizes without departing the principles we agreed to, i.e., payload size for rank=2,3,4 is double of rank=1. |
| CMCC | Prefer Alt1 to make it simple, especially for the small payload regime where UL resource waste is not severe issue. |
| ZTE | We prefer to further down select between Alt1 and Alt2 with even allocation, in which there is only one payload pair and thus the UE complexity can be reduced. For Alt3.1 and Alt3.2, we don’t think it’s worthy to additionally introduce one more payload pair {32, 1} only for the purpose of ensuring comparable payload across ranks, given that more payload pairs generally mean larger training efforts, complexity and lower performance. |
| Huawei, HiSilicon | Prefer Alt3.1 or Alt3.2. Alt.1 makes NW hard to preserve appropriate UL resources (always preserve 4x UL resources in case rank 4 is determined by UE). Between Alt3.1 and Alt3.2, slightly prefer Alt3.2, since AI CSI, similar to eType II, is mainly designed for MU pairing, so layer 1/2 information is more critical than layer 3/4. |
| InterDigital | We are not clear about the justification for Alt2. The main question for Alt1 vs. Alt3x is whether companies are interested in simplicity of the design or performance. If performance is the metric, then based on most of available simulation results, Al3.1 seem to be the way to go. |
| CATT | We prefer to down-select between Alt1 and Alt2. For low overhead regime, it is more beneficial to have good PMI for lower layer, i.e., layer1 and layer2. Therefore, the performance for Alt3.1 and Alt3.2 might not be good. |
| vivo | If the proposal is to list candidate option, we prefer to add Alt 4 to ensure that the UL resource is always less than 1.5 times that of rank 2. This is because evaluation shows the gain of {x, x, x} is significantly (8%~9%) Pair index 1; Rank1; Rank2; Rank3; Rank4. Alt1; {32, 2}; {32, 2}, {32, 2}; {32, 2}, {32, 2}, {32, 2}; NA. |
| ETRI | Our preference is Alt3.1 or Alt3.2 for preserving the same rule of payload sizes for rank 3/4 along with other PCs. |
| Xiaomi | Considering model training complexity and spec effort, Alt 1 and Alt 2 is preferred compared with Alt 3. Since pair (32,2) is lower overhead regime, the total overhead of rank 3 or 4 can be larger than that of rank 2, which does not against the following principle achieved in RAN1#122 bis meeting. At least for medium to high per-layer payload regime, the total payload size of rank 2, rank 3 and 4 are comparable to each other. |
| Ericsson | Agree with Samsung and Huawei that Alt 1 makes NW difficult to allocation UL resources for CSI reporting due to large total payload uncertainty. Regarding Alt 3, spreading the payload to higher layer by introducing Q=1 will most likely make the overall performance worse, and increase the implementation complexity for both UE and NW. We support Alt 2 in this case. Note that restricting certain parameter to only rank 1 and 2 already exists in eType II PC7/8. |
| LG Electronics | Alt2 is not necessary if Alt1 is supported. For the PC with rank-1 payload of 64 bits, per-layer pair (64, 1) can be also considered, as we already mentioned/commented to Proposal 2.2. We suggest to slightly change the wording with adding a note as follows. “For SQ PC with rank-1 payload pair of (32,1), down-select from following alternatives” (…) Note: other per-layer payers for rank-1 payload of 64 bits e.g., (64,1) is not precluded. Otherwise, we can add alternatives (Alt4) considering (64,1) for this rank-1 64bits payload case. Pair index 1; Rank1; Rank2; Rank3; Rank4. Alt1; {32, 2}; {32, 2}, {32, 2}; {32, 2}, {32, 2}, {32, 2}; {32, 2}, {32, 2}, {32, 2}, {32, 2}. Alt2; {32, 2}; {32, 2}, {32, 2}; NA; NA. Alt3.1; {32, 2}; {32, 2}, {32, 2}; {32, 1}, {32, 1}, {32, 2}; {32, 1}, {32, 1}, {32, 1}, {32, 1}. Alt3.2; {32, 2}; {32, 2}, {32, 2}; {32, 2}, {32, 1}, {32, 1}; {32, 1}, {32, 1}, {32, 1}, {32, 1}. Al4.1; {64, 1}; {64, 1}, {64, 1}; {32, 1}, {32, 1}, {64, 1}; {32, 1}, {32, 1}, {32, 1}, {32, 1}. Al4.1; {64, 1}; {64, 1}, {64, 1}; {64, 1}, {32, 1}, {32, 1}; {32, 1}, {32, 1}, {32, 1}, {32, 1}. |
| Fujitsu | We think layer1/2 information is more important, considering the eigenvalues of higher layers are small, larger number of bits are not that helpful. Also, in MU-MIMO cases, the accuracy of layer1/2 are more important as well. |

After the first round discussion, Alt1 seems to be redundant compared to the agreed pattern in proposal 2.4. Alt3.1 seem to be more aligned with the pattern agreed in proposal 2.4 by allocating more bits to the 3rd layer in rank 3. Perhaps we can focus on these two alternatives.

##### Proposal 2.3a (pair {32, 2}, high priority)

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 64 bits, down-select from following alternatives. The layers are reported in the order of decreasing strength

| Pair index 1 | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| Alt1 | {32, 2} | {32, 2}, {32, 2} | {32, 2}, {32, 2}, {32, 2} | {32, 2}, {32, 2}, {32, 2}, {32, 2} |
| Alt2 | {32, 2} | {32, 2}, {32, 2} | NA | NA |
| Alt3.1 | {32, 2} | {32, 2}, {32, 2} | {32, 1}, {32, 1}, {32, 2} | {32, 1}, {32, 1}, {32, 1}, {32, 1} |
| Alt3.2 | {32, 2} | {32, 2}, {32, 2} | {32, 2}, {32, 1}, {32, 1} | {32, 1}, {32, 1}, {32, 1}, {32, 1} |

#### PC led by {64, 2}

Proposals from tdocs are summarized as following table (presenting the payload size where 64 and 128 are resulted by {32, 2} and {64, 2}, respectively). Considering the even allocation and more bits for weaker layers, the payload combination with size {64, 64, 128} for rank 3 and {64, 64, 64, 64} for rank 4 have more supporters. For rank 3, 1 source shows {64, 64, 128} is comparable {64, 96, 96}. From complexity perspective, the latter introduces 3 pairs for this PC, thus the former is preferred.

| Companies | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| QC | 128 | 128, 128 | 64, 64, 128 | 64, 64, 64, 64 |
| Ericsson | 128 | 128, 128 | 64, 64, 128 | 64, 64, 64, 64 |
| vivo | 128 | 128, 128 | 128, 128, 128 | 64, 64, 128, 128 |
| Xiaomi | 128 | 128, 128 | 64, 64, 128 | 64, 64, 64, 64 |
| ZTE | 128 | 128, 128 | 128, 64, 64 | 64, 64, 64, 64 |
| Samsung | 128 | 128, 128 | 64, 96, 96 (comparable results with {64, 64, 128}) | 64, 64, 64, 64 |
| Fujistu | 128 | 128, 128 | 128, 64, 64 | 64, 64, 64, 64 |
| DCM | 128 | 128, 128 | 128, 64, 64 | 128, 64, 64, 64 |

##### Proposal 2.4 (closed):

For the PC with rank-1 payload of 128 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength.

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {64, 2} | {64, 2}, {64, 2} | {32, 2}, {32, 2}, {64, 2} | {32, 2}, {32, 2}, {32, 2}, {32, 2} |

Note: For rank 3, {64,2} is applied to the 3rd strongest layer.

| Companies | Comments |
| --- | --- |
| OPPO | Not very agree for higher rank. We think that layer 1/2 is most important than layer 3/4. Since even UE reports RI = 3,4 in some cases, NW may still schedule SU/MU in rank 1 or rank 2 transmissions. Therefore, such kind of CSI feedback payload allocation in rank 3/4 may reduce the CSI accuracy of layer 1/2 reporting. |
| Futurewei | Similar to our previous comment, since this proposal is for SQ, we suggest adding “For SQ” in the beginning of the proposal to make it clear that this is for SQ. Also, this proposal is for rank-1 payload of 128 bits, but not 256 bits. Therefore, we suggest the following modifications: “For SQ, Ffor the PC with rank-1 payload of 128 bits and 256 bits, support the following payload combinations across layers and ranks …” |
| Samsung | We prefer {{32,2}, {48,2}, {48,2}} as this configuration achieves better SE over {{32,2}, {32,2}, {64,2}} in our evaluation. We have also noted that supporting the value dl,ν=48 is useful as payload size Xlv=92 achieves better performance for other configurations too. If this is not agreeable, we can come back to rank=3 after progressing a bit more on the other payload size configurations Rank1; Rank2; Rank3; Rank4. {64, 2}; {64, 2}, {64, 2}; FFS; {32, 2}, {32, 2}, {32, 2}, {32, 2}. |
| CMCC | Generally support. Allocating more bits for higher layer for rank 3 is OK. If gNB has high probability to do MU-MIMO paring or rank fallback, maybe gNB can do proper RI restriction configuration to make sure reported RI is fixed to 1 or 2. |
| ZTE | Ok |
| Huawei, HiSilicon | For rank 3, we may further select between {32, 2}, {32, 2}, {64, 2} and {64, 2}, {32, 2}, {32, 2}. |
| Nokia | The text in the note should be part of the proposal rather than a note because it has specs impact. It’s probably better to agree on a specified reporting order of layers in order of decreasing strength, to avoid repeating the same note in multiple proposals |
| Mod | @Samsung, based on FL’s observation, the gap between {{32,2}, {48,2}, {48,2}} and {{32,2}, {32,2}, {64,2}} is minor, considering that {48,2} brings a third payload pair to this PC row, let us try to prioritize complexity in this case. @Huawei, @OPPO, your concerns are well received, but let us try to show more respect to the reported evaluation results. For rank3, allocating more bits to higher layer gives better performance. For MU pairing, maybe CMCC’s comment can address the concern (configuring RI restriction to rank 1 and 2 in advance). Also, please note that eType II also has half payload of layer 1 in rank 3 / 4 compared to rank 1/ 2. @Nokia, let us have a separate proposal for the ordering issue to avoid duplicate work for 2.3~2.6. |
| vivo | Based on this meeting and our own evaluation, as well as feedback from other companies, {X, X, X} shows some additional gain for rank 3. Should we still restrict the total payload size of rank 3 and rank 4 to be equal to that of rank 2? We suggest being more flexible with the payload size for rank 3 and rank 4, allowing it to be up to 1.5 times that of rank 2. Then, we can further down select the options from {{32, 2}, {32, 2}, {64, 2}}, {{64, 2}, {32, 2}, {32, 2}}, and {{64, 2}, {64, 2}, {64, 2}}. |
| Xiaomi | Support |
| Ericsson | Support |
| LG Electronics | Before discussing these specific PC choices, we can first deal with and reach an agreement on the principles regarding which layers (higher or lower) are allocated more bits. |
| Fujitsu | We think {64, 2}, {32, 2}, {32, 2} would be a better choice than {32, 2}, {32, 2}, {64, 2}. |

The PC proposed in Proposal 2.4 is finally agreed in the online session.

Agreement:

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 128 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength.

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {64, 2} | {64, 2}, {64, 2} | {32, 2}, {32, 2}, {64, 2} | {32, 2}, {32, 2}, {32, 2}, {32, 2} |

#### PC led by {128, 2}

Similar to pair {64, 2}, 128 is also integer multiples of 4, but not 3, thus it may be easier to consider the same philosophy as the PC led by pair {64, 2}. That is, even payload is applied for layers of rank 4. For rank 3, the payload of layer 1 and 2 are reduced by 50% than those in rank 1 and rank 2, while the remaining payload size is taken by layer 3 of rank 3.

Companies’ proposals are summarized in the following table (presenting the payload size where 256, 128 and 64 are resulted by {128, 2}, {64, 2} and {32, 2}, respectively), this method is also a bit more popular for the reason of good performance and complexity tradeoff. It is worth noting that, for rank 4, the gap between {128, 128, 128, 128} and {64, 64, 128, 256}is around 2%~3% according to Ericsson’s results, which seems to be a minor loss considering that the latter has 3 pairs for this PC. Similarly, for rank 3, Samsung shows {128, 192, 192} is around 1% better than {128, 128, 256}, but such gain seems to be minor considering that {128, 192, 192} brings 3 pairs for this PC

| Companies | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| QC | 256 | 256, 256 | 128, 128, 256 | 128, 128, 128, 128 |
| Ericsson | 256 | 256, 256 | 128, 128, 256 | 64, 64, 128, 256 (best results, 2% gain over {128, 128, 128, 128}) |
| vivo | 256 | 256, 256 | 256, 256, 256 | 128, 128, 256, 256 (>2x rank1) |
| Xiaomi | 256 | 256, 256 | 128, 128, 256 | 128, 128, 128, 128 |
| Xiaomi | 256 | 256, 256 | 128, 128, 256 | 64, 128, 128, 192 |
| ZTE | 256 | 256, 256 | 256, 128, 128 | 128, 128, 128, 128 |
| ZTE | 256 | 256, 256 | 256, 128, 128 | 192, 192, 64, 64 |
| Samsung | 256 | 256, 256 | 128, 192, 192 | 128, 128, 128, 128 |
| Fujistu | 256 | 256, 256 | 256, 128, 128 | 128, 128, 128, 128 |
| DCM | 256 | 256, 256 | 256, 128, 128 | 256, 128, 128, 128 |
| DCM | 256 | 256, 256 | 256, 256, 128 | 256, 256, 128, 64 (>2x rank1) |

##### Proposal 2.5 (pair {128, 2}, high priority):

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 256 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {128, 2} | {128, 2}, {128, 2} | {64, 2}, {64, 2}, {128, 2} | {64, 2}, {64, 2}, {64, 2}, {64, 2} |

| Companies | comments |
| --- | --- |
| OPPO | Similar comment to {64,2} in proposal 2.4. |
| Futurewei | Similar to our previous comment, since this proposal is for SQ, we suggest adding “For SQ” in the beginning of the proposal to make it clear that this is for SQ. |
| Samsung | Rank=3 can be set to FFS and discussed after concluding on whether to support {96,2} |
| CMCC | Support. |
| ZTE | Ok |
| Huawei, HiSilicon | For rank 3, we may further select between {64, 2}, {64, 2}, {128, 2} and {128, 2}, {64, 2}, {64, 2}. |
| Nokia | Same note as Proposal 2.4 is needed here |
| Mod | Let us try to reuse the same principle as proposal 2.4 since they are power of 2. @Samsung, evaluation results have shown the proposed combination give sufficiently good performance, so let us consider complexity a little bit for this PC. {96, 2} can be useful in proposal 2.6, and let us discuss it there. |
| Xiaomi | Support |
| Ericsson | We are okay with the proposal. |
| LG Electronics | Similar comment to Proposal 2.4. |
| Mod | Given the agreed PC in proposal 2.4. It seems that this PC should be stable. Please continue the discussion. |
| Toyota | Support. This is the easiest for the UE, without any considerable performance loss. |

#### PC led by {192, 2}

Companies’ proposals in their tdoc are summarized in the table below (presenting the payload size where 384, 256, 192 and 128 are resulted by {192, 2}, {128, 2}, {96, 2} and {64, 2}, respectively).

Even allocation in rank 4 is the most popular choice. From performance perspective, even allocation of 192 bits per layer is close to the performance of allocating more bits to layer 3 and layer 4 with {128, 128, 256, 256} bits. Moreover, {128, 128, 256, 256} has higher inference complexity as it uses two different payload pairs than that in rank 1 and 2.

Rank 3 is a bit controversial, 3 companies prefer {192, 192, 384} while 2 companies prefer {256, 256, 256}. From performance perspective, two companies report opposite trend. In terms of complexity, {192, 192, 384} is lower since same payload pair in rank 1, 2 and 4 can be reused.

| Companies | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| QC | 384 | 384, 384 | 192, 192, 384 (2% gain over {256, 256, 256}) | 192, 192, 192, 192 (1~2% loss to {128, 128, 256, 256}) |
| Ericsson | 384 | 384,384 | 256, 256, 256 (2~8% gain over layer-specific patterns) | 128, 128, 256, 256 (1~2% gain over {192, 192, 192, 192}) |
| Xiaomi | 384 | 384, 384 | 192, 192, 384 | 192, 192, 192, 192 |
| Xiaomi | 384 | 384, 384 | 192, 192, 384 | 128, 192, 192, 256 |
| ZTE | 384 | 384, 384 | 256, 256, 256 | 192, 192, 192, 192 |
| ZTE | 384 | 384, 384 | 384, 192, 192 | 256, 256, 128, 128 |
| Samsung | 384 | 384, 384 | 192, 192, 384 | 192, 192, 192, 192 |
| Fujistu | 384 | 384, 384 | 384, 192, 192 | 192, 192, 192, 192 |
| DCM | 384 | 384, 384 | 384, 256, 256 | 384, 256, 256, 256 |
| DCM | 384 | 384, 384 | 384, 384, 192 | 384, 384, 256, 128 |

For this PC, further discussion is needed considering following row.

##### Proposal 2.6 (pair {192, 2}, high priority):

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 384 bits, downselect from following payload combinations across layers and ranks

- Alt1.1: with {96,2} as additional payload pair + upto 2 different pairs

- Alt1.2: with {192, 1} as additional payload pair + upto 2 different pairs

- Alt2: with existing 4 payload pairs + 3 different pairs

- Alt3.1: with {96,2} as additional payload pair + 3 different pairs

- Alt3.2: with {192, 1} as additional payload pair + 3 different pairs

| Pair index 6 | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| Alt1.1 | {192, 2} | {192, 2}, {192, 2} | {96, 2}, {96, 2}, {192, 2} | {96, 2}, {96, 2}, {96, 2}, {96, 2} |
| Alt1.2 | {192, 2} | {192, 2}, {192, 2} | {192, 1}, {192, 1}, {192, 2} | {192, 1}, {192, 1}, {192, 1}, {192, 1} |
| Alt2 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {64, 2}, {64, 2}, {128, 2}, {128, 2} |
| Alt3.1 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {96, 2}, {96, 2}, {96, 2}, {96, 2} |
| Alt3.2 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {192, 1}, {192, 1}, {192, 1}, {192, 1} |

| Companies | Comments |
| --- | --- |
| OPPO | This can be discussed after {64,2} and {128,2} have been confirmed. {192,2} should follow the same principle. |
| Futurewei | Similar to our previous comment, since this proposal is for SQ, we suggest adding “For SQ” in the beginning of the proposal to make it clear that this is for SQ. |
| Samsung | Our results show Alt1.1 performs better. |
| ZTE | We prefer to further down select between Alt1.1, Alt2, and Alt3.1. Generally, we agree with FL that we can strive to reduce the number of payload pairs for each PC if there is no much performance difference. In a typical implementation, each PC corresponds to one different adaptation layer at the encoder output, which is achieved by full connected layer and thus includes quite a bit of model parameters to be download. For Alt1.2 and Alt3.2, we think the benefit of additionally introduce one more SQ codebook is not justified. |
| Huawei, HiSilicon | We suggest adopting the combination of {64, 2}, {128, 2} to fill rank 3/4, rather than creating a new per-layer combination only for rank 3/4 of the largest PC. Thus, we may select between Alt2 and following Alt2a (where layer 1/2 is larger than layer 3/4) Alt2a; {192, 2}; {192, 2}, {192, 2}; {128, 2}, {128, 2}, {128, 2}; {128, 2}, {128, 2}, {64, 2}, {64, 2}. |
| CATT | We prefer Alt1.1, the design principle for this alternative is the same as PC led by {64,2} and {128,2}. Alt1.2 introduces {192,1} with Q=2. Many companies have shown in evaluation results that Q=2 is the optimal choice. Therefore we would rather introduce {96,2} than {192,1}. |
| Xiaomi | We are fine to Alt1.1, Alt2, and Alt3.1 for down selection. |
| Ericsson | We prefer Alt 2 as that gives the best performance in our evaluation. On rank 3, our results indicate that {128, 2}, {128, 2}, {128, 2} is better than {96, 2}, {96, 2}, {192, 2}. One reason is that the gain in throughput (not SGCS) by increasing d from 128 to 192 is less rewarding than increasing d from 96 to 128. |
| LG Electronics | FL mentioned that the inference complexity increases when adopting different/more payload pairs. From our perspective, quantitively analysis (i.e., how much complexity is increased due to the different truncation/adaptation layers) may be needed before down-selection, in terms of performance-complexity trade-off. |

FL note on association between payload configuration with other configurations:

Three sources, CATT, HONOR and QC, proposed that association between payload configuration with other configurations, e.g., Tx port and subband, can be considered. For example, some payload is too small for a full bandwidth or 128 ports reporting or rank 4 reporting, or some other payload is too high for a small number of ports, or small bandwidth. FL thinks the discussion can be postponed after finishing the payload configurations and scalability.

# UCI mapping and omission

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| InterDigital | Proposal 10: Support reusing the two-part CSI reporting design of Type II codebook, where Part I includes at least RI/CQI, and Part II contains bits for compressed CSI. Observation 11: UCI Omission rules and criteria for AIML based CSI feedback can be different than legacy CSI reporting. Proposal 11: Support enhancing UCI omission rules for AIML based CSI feedback based on resources, timing and processing constraints as well as the type of CSI report (i.e., inference, monitoring, data collection |
| Furturewei | Proposal 7: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for UCI mapping for inference, support carrying the CSI feedback in two parts. The first part is of a fixed size and should include CQI for the first codeword, RI, and an indication of the size of the second part. The second part should at least include the content of the UE encoder output and CQI for the second codeword, if applicable. |
| Spreadtrum | Proposal 1: For CSI report in inference, CSI part 1 will include the factor representing the size of part 2, and CSI part 2 will include encoder output. |
| Vivo | - For CSI compression with UE side model, two parts CSI are suggested for reporting - Part 1 contains RI, CQI and potential payload truncation indication of each layer - Part 2 contains bit sequence of CSI feedback with UE side model - New omission rule, e.g., per layer omission, or payload truncation can be considered |
| CMCC | Proposal 10: For AI/ML based CSI compression, one CSI report includes CSI part 1 + CSI part 2. CSI part 1 includes CQI for first codeword, LI and RI; CSI part 2 includes the content of CSI generation part output |
| Huawei, HiSilicon | Proposal 10: For UCI mapping, consider two parts CSI: - CSI part 1 contains CQI, RI, and the CSI payload size information of CSI part 2 if needed. - CSI part 2 contains the bit sequences representing the compressed Target CSI. For precoding matrix: - Consider the priority rule to split the bit sequence per layer into M bit segments with different priorities for UCI omission. - Consider inter-layer first omission: first omit the M-th segment from layer N to layer 1, then the (M-1)-th segment,…, then the 1st segment. |
| Google | Proposal 1: The CSI report configuration for AI/ML based CSI compression is based on the Rel-19 eType2 CSI report configuration • The CSI report configuration can additionally provide a list of model IDs corresponding to different CSI payload size • The UE selects the model according to the maximum payload size for the CSI report to avoid the CSI omission |
| NEC | Proposal 10: In CSI spatial/frequency compression, support two-part as CSI report format. The CSI Part 1 includes CRI, RI, CQI (for first codeword) and information determining the payload size of the compressed PMI, while CSI Part 2 includes compressed PMI. - Whether/which information determining the payload size of the compressed PMI is included in the CSI Part 1 depends on the outcome of discussion about ‘payload determination’. Proposal 11: Study the multiplexing of AI/ML-based CSI feedback with existing Uplink Control Information (UCI) to ensure efficient coexistence on shared uplink resources. Proposal 13: In CSI spatial/frequency compression, for CSI omission, support CSI payload reduction for layer(s), e.g., CSI puncturing/truncating. |
| CATT | Proposal 16: For UCI mapping of AI/ML-based CSI compression, two part CSI mechanism can be reused. The CSI feedback bit for AI model output can have different priorities according to the corresponding layer and/or subband configuration. |
| China Telecom | Proposal 8: Support the reuse of the two-part CSI mechanism, where configurable parameters in CSI Part 1 are used to determine the size of CSI Part 2. Proposal 9: Support carrying the pairing ID in CSI Part 1, allowing NW-side to determine the size of CSI Part 2 and related parameters based on pairing ID. |
| Xiaomi | Proposal 6: For two-sided AI model based CSI feedback, support two-part CSI reporting and CSI part 1 at least including RI and CQI if applicable, CSI part 2 including the output of CSI generation part. Proposal 7: The mapping order of CSI part 1 for two-sided AI model based CSI feedback reuses the mapping order of CSI part 1 for legacy eType II codebook based CSI feedback. Proposal 8: CSI part 2 of two-sided AI model based CSI feedback is divided into multiple groups for UCI mapping, and how to divide CSI part 2 into multiple groups need to further study |
| ZTE | Proposal 14: For CSI feedback using two-sided model, for UCI mapping, the UE may omit a portion of the Part 2 CSI according to pre-defined priority rules. Proposal 15: Consider the following methods for CSI omission for potential down-selection: layer-based CSI omission, subband or subband group based CSI omission, truncation/masking of encoder output. |
| OPPO | Proposal 9: Support to reuse the legacy UCI multiplexing and resource allocation framework for AI/ML based CSI report. Proposal 10: Support CSI omission for CSI part 2 on PUSCH with per-layer truncation before or after quantization. Proposal 5: Regarding CSI feedback payload determination, support UE reported actual CSI feedback payload for each layer in CSI part 1. - Possible combination of NW configure and UE reporting can be further considered |
| LGE | - Study on the CSI encoding method with payload size indication, where part 1 of the CSI report can include either of the following: compression ratio, payload indicator, or the number of multiple information blocks (if applicable). - Consider report of both inference result and monitoring result together when monitoring occasion is occurred. |
| KT | - Proposal 6: Avoid constraining AI/ML-based CSI compression models to mimic the legacy UCI structure by training them to produce human-interpretable latent vector entries with predefined meanings as the latent vector in AI/ML models does not require such structure and enforcing it would limit model flexibility and performance. - Proposal 7: For CSI omission in AI/ML-based CSI compression, study potential specification support to define grouping or hierarchy information for latent vector entries to enable prioritized partial reporting. |
| Samsung | Proposal#12: For two-sided model-based CSI compression, support two parts CSI. The PMI information is mapped to Part 2 CSI. Support Part 2 CSI dropping. FFS: whether/how to define groups for Part 2 CSI including priority formula for PMI bits. FFS: dropping granularity for Part 2 CSI. FFS: content of Part 1 to determine size of Part 2 |
| Sharp | Proposal 1: For CSI feedback via two-sided AI/ML model for CSI compression, support to use two-part CSI reporting: - Part 1 CSI at least include RI and part 2 CSI - FFS: whether to support the indication of the parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν in part 1 CSI or UE follows a specific indication of PC configured by the gNB. - Part 2 CSI includes the quantized latent messages. - For partial omission of part 2 CSI, layer-based omission can be supported. |
| Samsung | Proposal 2: For CSI feedback reporting, further consider the order of the reported bits, e.g. layer dimension first, or segment dimension first. |
| Fujistu | Proposal 6: - For CSI compression Case 0, UCI mapping should follow the legacy framework of CSI Part 1 and Part 2, and the output of AI encoder should be included in CSI Part 2. - The payload size of CSI Part 2 is determined by NW configuration and the rank value. Proposal 7: - For CSI Part 2, RAN1 to study the necessity and the feasibility of grouping the output of AI encoder. - FFS: the number of groups. |
| Nokia | Proposal 11: For the UCI mapping of an inference-based CSI report, reuse legacy design for Rel-16 eType-II, i.e. two-part CSI with Part 1 containing RI (if reported) and CQI and Part 2 containing the quantized latent message. Proposal 12: Postpone the discussion on omission rules until after the payload size and structure is fully decided. The Part 2 omission procedure for inference-based CSI reports can largely follow legacy operation. |
| Lenovo | Proposal 1: Support to associate more than one pairing IDs to a same CSI-ReportConfig and the UE reports which one of them is used for a certain CSI report corresponding to this CSI-ReportConfig. Proposal 7: Support indication of the length of the compressed CSI other than CRI, RI and CQI within the CSI-Report Proposal 12: If there are multiple quantization configurationa for a single payload size of a model determined by a pairing ID, support the feature that the NW configure a payload size for a CSI-ReportConfig, and the UE determines a certain {d_z^(l,ν),Q^(l,ν) } for the CSI report. Proposal 14: Both single part and two parts CSI should be considered for AI based compressed CSI report. |
| ETRI | Proposal 4: For AI/ML-based CSI feedback, investigation of a mechanism that enables flexible adjustment of the payload size and explicit indication of the CSI Part 2 size is required. Proposal 5: For AI/ML-based CSI feedback, adopt type2-r16 UCI mapping as the baseline and investigate a new method for mapping the CSI Part 2 information into groups, for example, by layers. |
| HONOR | Proposal 6: For UCI mapping, support UCI grouping based on portions of each layer, further study the following options: - Option1: Grouping based on output dimension index or segment index. - Option2: Grouping based on quantized bit index. |
| Apple | Proposal 8: For AI based CSI compression inference configuration, the NW configures the maximum payload configuration and a rank restriction. The UE determines the rank and a payload configuration and include the CSI size info in CSI part 1. Proposal 11: For AI based CSI compression, CSI part 1 includes RI/CQI/number of bits per layer, CSI part 2 includes CSI feedback per each layer in order. Proposal 12: For AI based CSI compression, for CSI omission, the CSI feedback is dropped layer by layer. |
| MTK | - Proposal 5: Consider the following options for CSI omission: - Option 1- Layer-wise omission: All feedbacks bits corresponding to a layer will be omitted at a time. - Option 2 - Group-wise omission: group(s) of feedback bits from all layers will be omitted at a time. - Proposal 6: Prioritize group-wise omission where group(s) of feedback bits from all layers are omitted if omission is required. - Proposal 7: Discuss how to group the CSI feedback bits for potential group-wise omission in the following aspects: - Number of groups - Whether grouping changes for layers and ranks - Impact on inter-vendor collaboration |
| Quectel | Proposal 3: - Support AI/ML based CSI omission for CSI part 2 on PUSCH, to maintain the reporting flexibility. Proposal 4: - The AI/ML based CSI report should be portioned into groups, without hierarchical structure. |
| Qualcomm | Proposal 8: UCI omission is a corner case, RAN1 should adopt the simplest solution. Proposal 9: For Rel-20 NR CSI feedback via two-sided model, adopt UCI mapping and omission based on layer indices. |
| Transsion Holdings | Proposal 8: Regarding CSI omission in CSI spatial/frequency compression, the following alternatives can be considered: - Alt-1: Layer-based priority rule - Alt-2: Segment based priority rule for vector quantization - Alt-3: Scalar of latent space based priority rule for scalar quantization. Proposal 9: In CSI spatial/frequency compression, study the necessity of grouping the compressed target CSI and whether the compressed target CSI can be grouped based on layer information. Proposal 7: Regarding the alignment of payload parameters (e.g.,dl,ν, L,Ql,ν), payload parameter can be configured by Network in CSI report configuration or reported by UE in CSI part 1. |
| DCM | Proposal 6 - Reuse the CSI Part 1 and CSI Part 2 of NR CSI for UCI mapping. - In CSI Part 1 - UE reports RI (v) and CQI. - UE reports dzl,v or the sum of dzl,v for l=0, …,v-1 in CSI Part 1, and the UE uses the configured Ql,v values to generate the CSI feedback. - Note: Due to the CSI omission, the reported dzl,v can be smaller than the configured value. - In CSI Part 2 - If necessary, UE reports how many or which scalars in the latent message are reported for each layer. - UE reports dzl,v×Ql,v bits as the quantized latent message for each layer. Proposal 7 - Support the parameter combination-based CSI omissions. - The CSI payload can be truncated by selecting a shorter length of latent message and/or a lower quantization bit width from the specified parameter combinations. - In case of no candidate selection for both latent message length and bit width, layer-based CSI omission can be considered. |
| Rakuten | Proposal 4a: Introduce a minimal CSI Part-2 header at L1 with 1-bit quantization indicator preceding Part-2 (0=SQ, 1=VQ) to avoid ambiguity. When scalability is configured, an optional group index may indicate the pre-configured latent group reported in this occasion. Proposal 4b: Prioritze Omission of CSI Part-2 in following cases using the legacy rules: - if a change in applicability that result in a different Part-2 size or group mapping than configured, the UE omits AI Part-2 for that occasion (Part-1 continues). - under model drift scenario, till the re-training or UCI-loss is adressesed and a new model is activated. |

## Discussion

Two-part UCI is the common understanding as the actual payload would at least depend on the RI reported in CSI part 1. From companies’ proposal, the major controversial parts lie in 1) UCI grouping, mapping and omission order. and 2) whether the payload parameter combination is configured, or can be decided by UE and reported in CSI part 1.

### UCI grouping and omission

Following approaches of UCI grouping are reported by companies:

- Based on segments of each layer (7):

  - Huawei, NEC, ZTE, OPPO, Samsung, HONOR, MTK

- layer-by-layer (4):

  - Apple, QC, ZTE, CATT

- Based on subbands (2):

  - ZTE, CATT

Generally, there are two examples, one is based on layer indices which seem to be simplest approach; another option is based on portions of each layer similar to the eT2 approach. The former maintains the per-layer quality when CSI omission occurs but rank decreases, while the latter maintain the rank with the sacrifice of per-layer CSI quality.

For the second approach (omitting portion of layers), a hierarchical structure of the latent message or quantization is needed to ensure the omitted CSI is still readable by the NW. The key aspect of the discussion is that whether the hierarchical structure leads to performance degradation compared to the non-hierarchy latent message. One source reports SGCS results which shows noticeable performance loss at high payload regime. Furthermore, the omission may result in new payload options, which even increases the inter-vendor training challenges and may even further degrade the performance.

It is important to note that, by prioritizing SQ over VQ, having layer-common rank-common and payload scalable model, the tiny loss incurred in each step are accumulating. Further supporting the hierarchy structure would further accumulate more loss, then it is questionable how much gain we still have compared to eType II.

Thus, although UCI discussion can happen after the payload is stable, it may be good to trigger some discussion or thinking of this aspects from an early stage.

##### Proposal 3.1 (closed)

To facilitate the discussion on UCI grouping and omission, study the performance of hierarchy latent message or hierarchy quantization.

- For hierarchy latent message / quantization resolution, a common model is used for all the payload configurations.

  - Companies should report the hierarchy latent message dimensions (d1, d2, …, dN) or the hierarchy quantization, and how truncation or padding is performed in the training and inference.

- Consider following as benchmarks

  - eType II PC1-8

  - Non-hierarchical latent message / quantization resulted by a model with common backbone and multiple output / input layers or (de)quantization at the encoder / decoder. Same latent dimension and quantization resolution are used as the hierarchy approach.

- Layer-common and rank-common model should be used.

- Per-layer SGCS of the 4 layers as the metric

- The agreed payload pairs should be considered in the evaluation.

| Companies | Comments |
| --- | --- |
| Mod | Intention is to draw attention to the potential performance degradation due to the hierarchy structure of the latent message, and trigger study on relevant issues. CSI omission is a corner case, from FL perspective, we should not optimize such case while taking the risk of losing performance in nominal cases. |
| OPPO | Not very agree. In my understanding, supporting of UCI grouping and omission does not necessarily require hierarchy latent message or hierarchy quantization. UCI grouping and omission can be supported independently without hierarchy structure. Basically, if hierarchy latent message/quantization is considered in inter-vendor collaboration and training phase, the performance loss after UCI grouping and per-layer truncation-based omission may be reduced further. However, without this kind of design and training, UE can still drop part of bits per-layer (e.g., 8bits dropped in 64bits), and NW can use the reported right 56bits and pad omitted 8bits randomly with 0 or 1 on the correct position. This is a practical and simpler assumption in my understanding. We can evaluate it based on this assumption. |
| Futurewei | Need further discussion. |
| Apple | It is good to have some discussion on scalability issue on UCI. We have provided additional results in 2.2.1 showing that VQ’s advantage to SQ is widened when using one common trained model to support multiple payload sizes. |
| CMCC | We are open to discuss it. But if there is no consensus on hierarchy latent message or hierarchy quantization, maybe just segmenting the UCI layer by layer is default way. |
| ZTE | Ok to have further evaluations to justify the necessity of UCI omission. Regarding the first benchmark, we may further clarify that the AI/ML model with hierarchical design shall be compared to the eType II PC1-8 with UCI omission to ensure fair comparison, given that there are also some performance loss for eType II if portions of UCI are omitted. Regarding the second benchmark, it seems that the scalability approach based on specific output adaptation layer is selected while the scalability approach based on truncation/masking of the output layer is precluded. Which scalability approach to be evaluated can be up to company report. So we have the following suggestions. - Consider following as benchmarks - eType II PC1-8 with UCI omission - Non-hierarchical latent message / quantization resulted by a scalable model with common backbone and multiple output / input layers or (de)quantization at the encoder / decoder. Same latent dimension and quantization resolution scalability approach are used as the hierarchy approach. |
| Nokia | There is no need to support omissions of portions of layers and related optimisation efforts given the emergency nature of CSI omissions |
| NEC | Support |
| Mod | @OPPO, to enable the truncation-based UCI omission (as opposed to layer-wise omission), the latent message has to be hierarchy, otherwise NW cannot understand the truncated latent message. The intention is to study the performance difference hierarchy vs non-hierarchy wherein layer-wise omission is the simplest option in the latter. |
| Ofinno | Fine with the evaluation. But in principle, our concern is that omitting even a small subset of latent components in AI-based CSI compression breaks the completeness of the latent representation, preventing the network from reconstructing the channel at all. This fundamental difference means that the omission-based strategies of the legacy CSI framework are not directly applicable. |
| CATT | We are not sure why we need to study this. As FL points out that omitting portions of layer requires a hierarchical structure in latent message, but each company has its own preference on how their models are trained. We don’t think it is the right way to adopt certain model training/implementation methods so as to facilitate certain UCI omission rule. After all, UCI omission is a corner case, as long as we have simple way to solve it (e.g., with layer-by-layer or subband omission), it should be fine. |
| vivo | If x/2 can be supported for higher-rank case or smaller payload size configurations, the UCI omission can be naturally supported, regardless of whether a truncation-based model or a model with multiple input/output layers is used. |
| ETRI | We are Ok to study. |
| Xiaomi | We are general fine with this direction. Hierarchy quantization resolution is not clear to us. Does it mean to support multiple Q? The updated by ZTE could be supported, as the truncation/masking training method should not be precluded. |
| LG Electronics | Similar view with Nokia. |
| TCL | Need to study if hierarchy latent message is necessary. |
| NTT DOCOMO | With the hierarchy quantization, the CSI omission can be based on the agreed parameter combinations (d, L, Q), i.e., UE can degrade the (d, L, Q) PC to a lower size one without much complexity. The CSI omission with finer granularity can be supported without increasing the model training complexity. The reported CSI is always with agreed (d, L, Q) combinations. |
| Fujitsu | OK for further study. |

Based on the discussion, the views on the necessity of hierarchy message are diverse. Companies mention that CSI omission related optimization is not necessary. To facilitate truncation-based CSI omission, the hierarchy structure is needed which may even impact the performance of regular case without CSI omission. Even the evaluation study is performed, the observations would diverge. In light of that, the hierarchy structure can be upto NW and UE implementation, there is no need to restrict such format in the spec.

##### Conclusion 3.1a (latent message, medium priority)

Whether the latent message has a hierarchy structure to support different payload sizes is upto NW and UE implementation. No further discussion in Rel-20 on this aspect.

- Note: Hierarchy structure of a latent message means that a latent message with a lower payload size is a subset of a latent message with a higher payload size

| Companies | views |
| --- | --- |
| Mod | The hierarchy structure can be upto NW and UE implementation, there is no need to restrict such format in the spec. |

### UE reporting actual payload size

This topic was discussed in the last meeting. Around 9 companies (DCM, Huawei, ZTE, Futurewei, CATT, TCL, Fujistu, CMCC) show concerns on the feasibility of UE determining and reporting the actual payload size. However, based on the companies’ proposals submitted to this meeting, it seems that the situation has changed.

Executive summary of companies’ proposals on UE reporting the actual payload information:

- Indication of payload size, payload pair

  - (13): Futurewei, Huawei, vivo, NEC, LGE, OPPO, Sharp, Lenovo, ETRI, Apple, Transsion Holdings, DCM

  - (2) pairing ID: Google, China Telecom

##### Discussion 3.2 (closed)

Is it feasible for UE to determine and report the actual payload size, or payload pair or pairing ID?

| Companies | Views |
| --- | --- |
| Mod | From reading of the proposals, the motivation can be categorized as following: - To avoid CSI omission - To further reduce overhead, similar to the NNZC reporting in eT2. For avoiding CSI omission, FL does not think it is feasible. It should be noted that UCI packing and CSI processing are two orthogonal and independent process. When UE measures and compute the CSI, UE does not know how much the available UL resources is as there would be other UCI multiplexed with the CSI reports. For further reducing overhead, it seems to be a different situation than eT2. The angular-delay domain representation in eT2 may be sparse, but it may not be the case for latent space. Furthermore, indicating the location of these non-zero entries also requires some additional overhead. If the situation remains the same as last meeting, we may conclude that there is no consensus of UE reporting the actual payload size. |
| OPPO | We think it is quite useful to support UE reporting payload size. Support to further discuss this aspect. |
| Futurewei | Since the actual payload size is dependent on UE’s reported RI, the PC used, and whether the allocated PUSCH resource can accommodate the payload for that reported RI, it is necessary and feasible for UE to determine and report the actual payload size. However, in our opinion, the payload pair or PC used by the UE to generate the feedback payload should be configured by the network, instead of being determined and reported by the UE. |
| Apple | Network can configure a maximum payload config and UE can determine and report a payload config that is equal or less than the maximum payload config. |
| Samsung | We prefer the legacy approach. The network configures the parameter configuration. If adjustments to be made on the payload size based on the available uplink resources, partial (group/segment-based) dropping of Part 2 CSI can be considered. |
| CMCC | If UE can further decide the CSI payload size after gNB configure the payload combinations for different ranks, we are afraid inter-vendor collaboration will become more difficult or the inference performance will further deteriorate, considering UE can already report RI and UCI omission may occur. |
| ZTE | Agree with FL’s assessment. |
| Huawei, HiSilicon | Maybe our Tdoc causes some misleading - it says ‘CSI payload size information of CSI part 2 if needed’. We haven’t yet confirmed the feasibility and method on how UE can autonomously determine the payload size, so please tentatively take us out from the supporters. (12 13): Futurewei, Huawei, vivo, NEC, LGE, OPPO, Sharp, Lenovo, ETRI, Apple, Transsion Holdings, DCM |
| MTK | Share same view as Apple. We also believe NW can configure maximum payload size and give UE the flexibility of choosing payload config that fits into the maximum allowed payload. |
| CATT | We agree with the FL. We fail to see the motivation and benefit of UE determining and reporting a payload configuration. Therefore, we prefer legacy behavior. |
| vivo | For current discussion, we share the same view as Samsung that UE reporting only can be considered for UCI omission. |
| ETRI | We see that it is beneficial for UE to adjust per-layer payload sizes from the assigned PC from gNB. For example, using any payload-scalable design UE can adjust and inform the adjustment level from the assigned PC. |
| Panasonic | For actual payload size, we have same view to Apple. |
| Xiaomi | Agree with FL’s assessment. |
| TCL | Support to study. |
| Fujitsu | We think it is OK to allow UE to report its favorite payload, but the final decision is up to NW. |

The discussion has been repeated for 3 meetings, it is unclear of how to determine the actual payload smaller than the payload configuration configured by the NW.

##### Conclusion 3.2a (UE reporting actual payload size, medium priority)

There is no consensus on determining the actual payload size based on UE indications other than RI.

| Companies | views |
| --- | --- |

# CQI assumptions

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Proposal 7 Support option 1a as the method for CQI calculation. |
| Futurewei | Proposal 3: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for CQI determination, if the actual or reference CSI reconstruction model is available at UE, adopt Option 2a to determine CQI at UE: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment. Observation 1: Without the knowledge of the actual decoder at UE side, the reported CQI can be significantly biased, and the amount of bias may change significantly from report to report, and this is a big issue for OLLA to meet the target BLER. The consequence of not reporting CQI correctly is not only hurting the performance but also defeating the purpose of using AI/ML-based CSI feedback compression functionality the difference between the CQI calculated based on the reconstructed CSI and the CQI calculated based on the target CSI will also vary significantly over time, rendering Option 1a and 1b infeasible as it is difficult to configure/set an adjustment for such a big and fluctuated CQI difference and it is difficult for outer-loop link adaptation (OLLA) to compensate such a big difference. |
| Spreadtrum | Proposal 3: For CQI determination in CSI report, support option 1 as starting point. |
| Huawei, HiSilicon | Proposal 4: For CQI determination, consider Option 1a/1b, i.e., it is determined with realistic channel measurement, and with potential adjustment indicated by gNB if needed. |
| InterDigital | Observation 2: In Options 1a and 1c, the potential mismatch between the UE-side measured CSI and the NW-side reconstructed CSI cannot be compensated. In Option 2a, proprietary issues may arise, and it can significantly increase UE complexity. However, Options 1b and 2b do not exhibit such drawbacks, and can provide a same or better accuracy for CQI reporting. Proposal 2: Support at least one of Option 1-b or Option 2-b for CQI determination. |
| Google | Proposal 4: Support the RI/CQI calculation and report based on precoded CSI-RS. |
| CATT | Proposal 16: For inference report of AI/ML-based CSI compression, if CQI in CSI report is configured, CQI is calculated based on target CSI with realistic channel measurement.  Same quantization scheme can be reused as in legacy CQI reporting Proposal 17: For inference report of AI/ML-based CSI compression, RI is determined by UE with rank adaption. RI restriction is supported. |
| Vivo | Therefore, based on the above simulation and analysis, option 1a can be selected since the option 1a is the simplest and most direct choice. Furthermore, option 1a could be rephrased as “CQI can be calculated based on the precoding matrix before compression” since “target CSI with realistic channel measurement” may be unclear. - The impact of different CQIs on system performance is limited. - The SE performances remain essentially the same when using the same PMI and different CQI(e.g., the CQI based on SVD PMI, or the CQI based on eType 2 PMI) - For CQI determination in CSI report, CQI can be calculated based on precoding matrix before compression |
| Xiaomi | Proposal 10: Support Option 1a, CQI is calculated based on target CSI with realistic channel measurement to determine RI/CQI. |
| ZTE | Proposal 7: For CSI feedback using two-sided model, for CQI determination in a CSI report, support Option 1a: CQI is calculated based on target CSI with realistic channel measurement. Proposal 8: For CSI feedback using two-sided model, for RI determination in a CSI report, legacy RI determination can be reused as a starting point. |
| Samsung | Proposal#9: Defer the discussion on CQI determination schemes for two-sided model-based CSI compression until some progress on PMI feedback is achieved. |
| Lenovo | Proposal 5: For AI based CSI compression report, CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation. Proposal 6: For AI based CSI compression report, RI is calculated based on the realistic channel estimation based on the CSI-RS resource indicated by CRI as legacy. |
| Panasonic | Proposal 4: For CQI determination in CSI report, deprioritize Option 1c and 2b. - Option 1c: CQI is calculated based on legacy codebook - Option 2b: CQI is calculated using two-stage approach. UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder. Proposal 5: For CQI determination in CSI report, Option 1a or 1b should be baseline. The necessity of Option 2a can be further discussed. - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform construction model inference with potential adjustment. - The CSI reconstruction part for CQI determination at the UE is a nominal or reference model, which is different from the actual CSI reconstruction part at the network. |
| OPPO | Proposal 7: Regarding the RI determination, support the RI report with min⁡(RI1,RI2), where RI1 is calculated with PMI as target CSI and RI2 is calculated with PMI as legacy codebook. - Type I as the default codebook to calculate RI2 Proposal 8: Regarding the CQI determination, support CQI report with both of CQI1a and CQI1c, where CQI1a is calculated with PMI as target CSI and CQI1c is calculated with PMI as legacy codebook - Type I as the default codebook to calculate CQI1c |
| HONOR | Proposal 4: For CQI determination, support option 1a, CQI is calculated based on target CSI with realistic channel measurement. |
| Nokia | Proposal 8: For calculating the CQI, support CQI calculation based on target CSI (Options 1a). An adjustment of the CQI may be applied by a UE, based on the receiver implementation in a network-transparent manner. |
| LGE | Proposal #2: For CQI determination in CSI compression using two-sided model, consider to prioritize Option 1b (via realistic channel measurement + potential adjustment) or Option 1c (via legacy codebook). |
| Apple | Proposal 9: For AI based CSI compression, CQI/RI is calculated based on target CSI with realistic channel measurement, with potential adjustment. Proposal 10: UE determines the potential adjustment via measuring one or more precoded RSs (CSI-RS and/or DM-RS) in a time window, wherein the one or more precoded RSs are precoded corresponding to the target CSI. |
| NEC | Proposal 8: For CQI determination in CSI report, if a proxy CSI reconstruction part at UE side is proven to be feasible, support Option 2a; if not, support Option 1a/1b. |
| Sharp | Proposal 1: For CQI determination, for option 1c, discuss the precoding information type of the output CSI. Proposal 2: For CQI determination, prioritize option 1c and option 2a-2. For UEs with different capabilities, UEs may support different options based on their capabilities. Proposal 3: Support a type of CQI determination based on UE capability. |
| Transsion Holdings | Proposal 2: Regarding CQI determination, support Option 1b: • Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment |
| Fujistu | Proposal 2:  For CSI compression Case 0, prioritize Option 1a and Option 1b for CQI/RI determination. o Option 1a: CQI is calculated based on target CSI with realistic channel measurement. o Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment. |
| MTK | Proposal 2. Support only a single option for determining CQI. Proposal 3. For CQI determination using two-sided AI/ML models for CSI compression: i) deprioritize sub-options 1c, 2a, and 2b; and ii) Further clarify potential adjustment in option 1b. |
| ETRI | Proposal 1: For AI/ML-based CSI compression, prioritize to consider the Option 1a and 2a for CQI determination method |
| CMCC | Proposal 4: CQI should be reported in the AI/ML based CSI compression report. Proposal 5: Take Option 1 as the assumption of CQI determination for AI/ML based CSI compression: - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation Proposal 6: Take Option 1a as the starting point of CQI determination for AI/ML based CSI compression: - Option 1a: CQI is calculated based on target CSI with realistic channel measurement |
| Ofinno | Proposal #6: Regarding CQI determination, prioritize Option 1a. Also, consider adopting PMI based approach (Option 1c) as a fallback when the CQI from Option 1a is unreliable. Proposal #7: Consider a method to minimize the number of utilized AI models while maintaining accuracy-overhead performance across the full range of supported ranks with consideration of AI/ML processing unit occupation. |
| KT | Proposal 4: Treat the difference between Option 1a and Option 1b as implementation-specific, allowing the potential adjustment in Option 1b to be handled at the implementation level without requiring standardization or impacting the existing CQI definition. Proposal 5: Deprioritize option 1c and 2a due to their high computational complexity and limited implementation feasibility |
| KAIST | Proposal 1: Based on the output of CSI reconstruction part, adjust the estimated CQI by considering the following factors: - Precoder variation across subbands: Adjust the estimated CQI by exploiting the precoder difference in the port-subband domain. - HARQ ACK/NACK-based link adaptation: Utilize recent HARQ feedback to compensate for the estimated CQI. |
| Qualcomm | Proposal 10: Note CQI calculated based on precoded CSI-RS as being already supported. Proposal 11: For CQI options 1a/1b/2a, study the spec impacts to accommodate all implementation options. • RAN4 may define a relaxed testing requirement to accommodate various potential precoder assumptions. • In RAN1, clarify the reported CQI by these methods are different from the legacy CQI. |
| DCM | Proposal 3 - RI and CQI are determined based on the same option. - Support Option 1a as the baseline for both RI and CQI determination. - Option 1b is the UE implementation based on Option 1a. - Not support Option 2a due to the UE complexity and inaccuracy of UE reconstructed CSI, which cannot be compensated at the NW side because the NW does not know the UE implementation. - Not support Option 2b due to the additional round-trip latency for the CQI reporting. |
| Rakuten | Proposal 2: To adopt CQI Option 1b as the study baseline for Rel-20 and define a CQI Adjustment Descriptor (CAD) structure that carries the ΔCQI offset table parameters. - CAD consists of rank, SNR range, deltaCQI entries indexed per pairing ID, where deltaCQI is function of rank and SNR range estimated on UE-side from CSI. - The scheduler at the gNB side accordingly compensates its link adaptation, maintaining transparency to higher-layer scheduling logic. Proposal 3: - For Option-1b, the UE applies a signed ΔCQI after averaging and before quantization and saturates to the valid CQI range. If ΔCQI is not available, ΔCQI=0 (i.e., Option-1a) is assumed. No change to UCI size/packing. - If UCI retransmission fails for an inference-based CSI report, the UE reverts to legacy CSI (Part-1 only) until the next configured CSI opportunity. |

## Discussion

CQI assumptions identified in R18 includes the following.

- Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder.

The summary of companies’ proposals is presented as the table below.

|  | Support | deprioritize |
| --- | --- | --- |
| Option 1a (16) Target CSI | Ericsson, Spreadtrum, Huawei, CATT, Xiaomi, ZTE, Panasonic (baseline), Nokia (adjustment in spec transparent manner), NEC, Fujistu, DCM, HONOR, CMCC, Ofinno, vivo, ETRI |  |
| Option 1b (11) Target CSI + adjustment | Spreadtrum, Huawei, LGE, Apple (adjustment based on precoded RS in a time window), NEC, Transsion Holdings, Fujistu, MTK (further clarification), IDC, KAIST (adjust by precoder variation, A/N), Rakuten |  |
| Option 1c (3 vs. 4) eT2 | Ofinno (fallback), OPPO, Sharp (further discuss precoding information) | Panasonic, MTK, QC, KT |
| Option 2a (4 vs. 3) Proxy decoder, ref decoder | Lenovo, NEC, Futurewei, ETRI | MTK, DCM, KT |
| Option 2b (2 vs. 3) Precoded RS | Google, IDC | Panasonic, MTK, DCM |
| Other | Samsung: Defer the discussion after PMI part is stable. Qualcomm: CQI assumption upto implementation |  |

FL note:

From companies’ proposals, compared to the last meeting, the situation remains same. Collecting companies’ comments mentioned in the last meeting, following are FL’s response and observations

- Boundary between option 1a and 1b are blur, quite a few companies mentioned that the adjustment can be done by NW, or even by the UE, how is can be detected or tested as it is inside the UE and transparent to the NW.

- With above logic, option 2a is also an implementation option. In other words, the CQI is adjusted by running a proxy decoder, which is transparent to the network. More generally, a direct CQI estimator can be used.

- Regarding option 1c,

  - @OPPO, it seems that the benefit is not well acknowledged by the majority and there is strong objection from chipset vendors considering the additional complexity. The CQI of the codebook-based PMI can be obtained by a separate CSI report.

  - @ofinno, the fallback mechanism can be a part of the monitoring discussion.

- Option 2b is already supported by existing spec.

The CQI is based on a PDSCH hypothesis. In legacy, the hypothesis is defined with the respect to the specified overhead for rate-matching, the specified PMI codebook, and it is also largely related to implementation such as channel measurement and PMI algorithm and receiver assumption. For two-sided CSI, the only difference is that the PMI codebook is not specified while the CQI resulted by the PMI is adjusted / derived by one way or another, which are transparent to the network. From this angle, the principle and definition of CQI is not changed, the new part is the implementation.

##### Conclusion 4.1 (CQI, medium priority):

For CSI feedback via two-sided model inference, the CQI calculation method is upto UE implementation.

| Companies | Comments |
| --- | --- |
| Mod | Intention is to conclude that the CQI calculation option 1a/1b/2a (and some other options not listed) are all implementation choices. |
| OPPO | Does it mean reporting both of option 1a and option 1c in one CSI report or two CSI reports are possible? |
| Futurewei | Is the reported CQI based on UE implementation subject to the same accuracy requirement defined by RAN4 as in legacy? |
| Apple | Better to have a bit gNB assistance to allow UE to have a simple and feasible implementation. We can discuss more. |
| ZTE | Per our understanding, the CQI calculation is not totally up to UE implementation. For two-sided model based CSI, the PMI codebook is not specified but the precoding matrix has been specified. As agreed before, the precoding matrix is represented in the port-subband domain, and the mapping between precoding matrix and latent message is subject to the configured pairing ID. Therefore, if we can converge on option 1a for CQI calculation, it actually means that the CQI calculation is based on the specified precoding matrix. |
| Nokia | Ok with this conclusion as the same is true for legacy CQI calculation. However, regarding the UE assumptions for PMI/RI/CQI calculations, we may need to discuss how the PDSCH signals on antenna ports [1000,…,1000+v-1] would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+PCSI-RS-1]. Is the choice of precoder W(i) also up to UE implementation? |
| Mod | The intention is that, the CQI definition remains the same which is based on a reported precoding matrix, while the only difference is that UE may apply adjustment / estimation / method or even running a decoder to derive the CQI rather than calculate the CQI directly using the reported precoding matrix as it is not available. @Futurewei, it is expected that RAN4 would adopt a new requirement, but further discussion may be upto them. @OPPO, option 1c is done by a separate CSI report using legacy codebook. @Nokia, CQI is not redefined, it is till based on the reported W(i), but just the derivation method is upto UE implementation, whether UE uses actual decoder output (if UE has it), nominal / reference / private decoder output, target CSI, or use any other estimation methods to adjust CQI to make it represent the actual decoder output as much as possible. |
| Ofinno | Similar view with ZTE. In legacy, the CQI is derived by the UE but it is based on the indicated precoder (i.e., PMI). But, in two-sided case, there is no reference to derive the CQI currently (if we do not use the legacy PMI) and this is why we consider the options above (especially with realistic channel measurement such as 1a). |
| CATT | We are not sure if such conclusion is help. In our understanding, one way or another, UE has to adopt certain assumption for PMI/CQI calculation as described in TS 38.214. We are not sure if this conclusion is agreed, how the PMI/CQI calculation section for AI CSI compression will be described? In legacy eT2 both the gNB and UE side has the same understanding on the reported PMI. For AI CSI compression, the PMI obtained at gNB with decoder model is not known at UE side unless decoder is available. We are not sure legacy description adding reported W(i) is derived up to UE implementation is sufficient for CQI calculation |
| ETRI | Agree on the CQI calculation option is the implementation choice. Altough it is implementation choice of UE, gNB may need to be informed of the specific option of UE. |
| Panasonic | First, we update our proposal above based on our updated tdoc. We have same question to Futurewei. |
| Xiaomi | If companies have common understanding that Option 2b is already supported by existing spec, we think this conclusion is not needed because CQI could be obtained by using existing solution, i.e., Option 2b. |
| LG Electronics | First of all, our preference has not been captured properly. We support not only Option 1b, but also Option 1c. So, the table above would be corrected as below. Option 1c (34 vs. 4) eT2; Ofinno (fallback), OPPO, Sharp (further discuss precoding information), LG; Panasonic, MTK, QC, KT. Although both Option 1c and Option 2a may have additional UE complexity on top of running encoder model alone, Option 1c is much more preferable because Option 1c, basically, does not incur the model mismatch between UE and NW. However, for Option 2a, alignment between proxy/reference decoder at UE and NW-side actual decoder should be guaranteed. Thus, Option 1c is more practical and more feasible than Option 2a. Moreover, Option 1c provides simplicity and backward compatibility to legacy framework. As per Option 1b, we have to further clarify the term “potential adjustment”, which some companies including us interpret as the NW indication of CQI adjustment (e.g., CQI offset). To our understanding, Option 1b is not just UE implementation and fundamentally different than other options. Thus, Option 1b may be unique solution which is able to reflect NW-side decoder’s output. |
| TCL | Agree, but the UE should acknowledge the NW in the report. |
| Fujitsu | OK. |

# Report configuration

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| Futurewei | Proposal 1: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), support using CSI-ReportConfig for inference measurement and report configuration. Indicate at least the following in CSI-ReportConfig: • An ID for pairing encoder at UE side and decoder at NW side • A new report quantity, e.g., “AIML-cri-RI-PMI-CQI” • Number of TX antenna ports • Number of subbands • CSI-RS resources for channel measurement • Resources for interference measurement Proposal 5: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), support using the following for CSI feedback payload size determination. • The ID for pairing encoder at UE side and decoder at NW side • RI • Quantization codebook associated with the ID |
| Huawei, HiSilocon | Proposal 2: For measurement and report configuration,  Introduce a new CSI report quantity based on latent space. Proposal 12: For AI/ML CSI compression, deprioritize the CBSR discussion. Proposal 13: For Rel-20 CSI feedback via two-sided AIML models, support up to 128 ports, 19 subbands, and rank-4 (with precoding matrix as Target CSI). |
| Google | Proposal 2: For CBSR, support one of the following options: - Option 1: Only 1-bit indication with the value of {0, 1} per beam is supported - Option 2: Introduce the W2 as the target CSI for CBSR with multi-bit indication per beam Proposal 3: Support hybrid AI/ML based and non-AI/ML based CSI measurement and report • UE reports the CSI based on AI/ML if it reports a small RI and the UE can report the CSI based on Type1 codebook if it reports a large RI |
| CATT | Proposal 2: For inference report of AI/ML-based CSI compression, support aperiodic and semi-persistent CSI report. Proposal 3: For AI/ML based CSI compression, rank <= 4 can be supported. Proposal 4: For AI/ML based CSI compression, up to 128 Tx ports can be supported. Proposal 5: For AI CSI compression, support the following options for configurable subband sizes:  Option 1: Reuse the legacy mechanism.  Option 2: Select one subband size for each BWP Proposal 6: The reported subband configuration (i.e., csi-ReportingBand) can be restricted for AI/ML-based CSI compression. Proposal 12: For inference report configuration of AI/ML based CSI compression, new reportQuantity or new parameters dedicated to AI CSI compression can be introduced. Proposal 13: The value of N1,N2 as well as RI restriction can be configured for inference for AI CSI compression. |
| Vivo | Proposal 3: CSI-ReportConfig is used for the configuration of inference results reporting with the following enhancements. • Introduced new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI compression. Proposal 5: Study whether a dataset (direction 4-1), a model parameter direction 3a-1 and/or a specified model in direction C) can be associated with same pairing ID • If yes, one additional ID also needs to be indicated in inference configuration to indicate which method or which reference model is used |
| TCL | Proposal 3: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects: • AI/ML-specific CSI-RS resource configuration for CSI compression • AI/ML-specific fields in CSI-ReportConfig IE • Dedicated report quantities and report configurations for AI |
| ZTE | Proposal 5: For the CSI reporting setting for inference, pairing ID shall be configured to enable the UE to select an encoder compatible with the decoder used by the gNB. Proposal 6: For the CSI reporting setting for inference, support UE to indicate whether the CSI content is generated using the legacy codebook or the deployed AI/ML model. |
| Samsung | Proposal#1: For Target CSI in port-subband domain denoted for layer l by Winl=[Win,0l, Win,1l,⋯,Win, Nsb-1l], support the parameter R∈{1,2} and the legacy eType II rule for the determination of Nsb and the PMI subbands. Proposal#2: Consider expressing the target CSI in the basis representation Winl=W1W2WfH to facilitate configuration of CBSR. Proposal#3: Support at least the following for the CSI measurement and reporting for inference - A single CSI-RS resource - 16, 32 ports - Up to rank 4 - FFS: restrictions on the number of sub-bands - FFS: other configuration aspects, e.g., rank restriction, CBSR |
| Lenovo | Proposal 1: Support to associate more than one pairing IDs to a same CSI-ReportConfig and the UE reports which one of them is used for a certain CSI report corresponding to this CSI-ReportConfig. Proposal 2: Reuse CSI-ReportConfig to configure the CSI report with AI based CSI compression by introducing new reportQuantity for CSI report with AI based CSI compression. Proposal 3: A pair ID is configured with a CSI-ReportConfig used for CSI report with AI based compression. Proposal 4: RAN1 needs to study how to support various subband and/or Tx port configuration with a same pair ID. |
| Panasonic | Proposal 4: At least for inference, new RRC parameter for CSI report configuration to distinguish CSI report of AI/ML-based report and non-AI/ML-based report. Proposal 5: For inference configuration, the same ID as pairing ID for AI/ML-based CSI compression can be used. |
| OPPO | Proposal 7: Support to indicate whether the CSI report is through AI/ML or legacy codebook, following options can be considered:  Option 1: introduce new reportQuantity, e.g., AI-PMI.  Option 2: introduce new codebookType, e.g., codebookAI-r20  Option 3: introduce a new field, e.g., AI-r20, instead of within codebookType field. |
| Nokia | Proposal 1: Include at least the following parameters as part of the inference configuration of AI/ML-based CSI compression via two-sided models: - The gNB antenna port configuration given by the parameters N1 and N2. - The codebook parameter combination - The rank restriction - Codebook subset restriction (CBSR) configuration - A single pairing ID Proposal 2: Use the supported combined antenna port configurations from the Rel-16 and Rel-19 eTypeII codebooks as a starting point for discussing the port configurations to support for AI/ML-based CSI compression. Proposal 3: AI/ML-based CSI compression supports ranks 1-4. |
| LGE | Proposal #2: Consider the methods for association between pairing ID and payload configuration (e.g., via signaling procedure for applicable functionality reporting). Proposal #7: Consider report of both inference result and monitoring result together when monitoring occasion is occurred. |
| Apple | Proposal 1: For AI based CSI compression inference configuration, a paring ID should be configured to identify the matching CSI generation and CSI reconstruction model. Several signaling options can be considered: • Alt 1: Pairing ID is configured together with resourceForChannelMeasurement. • Alt 2: Pairing ID is included within codebookConfig. • Alt 3: Pairing ID is associated directly with reportQuantity. • Alt 4: Pairing ID is introduced as a separate field within CSI-reportConfig. Proposal 2: For AI based CSI compression inference configuration, introduce a new report quantity aiPMI, to distinguish PMI feedback using legacy codebook. Proposal 5: For AI based CSI compression inference configuration, introduce a new IE (e.g., AI-CSICompressionConfig or CodebookConfig-r20) to consolidate configuration parameters relevant to AI based CSI compression. The IE minimum contains rank restriction parameters. Proposal 3: For AI based CSI compression inference configuration, configuration parameters of AI based CSI compression comprises spatial restriction parameters, e.g., restricted DFT beams with an SGCS threshold: - Total number of restricted spatial directions is a small number, e.g., 4; and - Oversampling factor might not be needed. |
| NEC | Proposal 2: In CSI spatial/frequency compression, support inference report configuration reuse the CSI-ReportConfig with additional contents of model ID/ pairing ID and quantization parameters. Proposal 3: In CSI spatial/frequency compression, introduce new reportQuantity for compressed PMI in CSI-ReportConfig. |
| Nokia | Proposal 9: Define an uncompressed and unquantized reference format for the target precoding matrices for inference, for example as a special parameter combination of Rel-16 eType-II, with L=N1N2, Mυ=N3 and with K0=2N1N2N3 unquantized nonzero coefficients per layer. Proposal 10: Support the possibility to configure hard codebook subset restriction (CBSR) for a CSI reporting setting for inference, based on legacy CBSR higher-layer parameter (hard CBSR only). |
| Transsion Holdings | Proposal 1: The following potential spec impacts related to CSI report configuration of CSI compression should be considered: • Signaling to trigger the AI/ML model CSI compression • Pairing between CSI generation model and CSI reconstruction model • Indicate the CSI feedback payload • CSI-RS resource configuration • CSI reporting configuration |
| CMCC | Proposal 1: In addition to up to 32 ports CSI, 48, 64, 128 ports CSI should also be supported in AI/ML based CSI compression. Proposal 2: For up to 32 ports CSI, only one CSI-RS resource can be configured for one CSI report configuration, each CSI-RS resource is configured with up to 32 ports. Proposal 3: For 48, 64 and 128 ports CSI, up to 4 CSI-RS resources can be configured for one CSI report configuration, each CSI-RS resource is configured with up to 32 ports, and these CSI-RS resources are aggregated to form up to 128 CSI ports. |
| ETRI | Proposal 10: For AI/ML-based CSI compression, investigate the feasibility of introducing codebook subset restriction in the AI/ML-based CSI feedback framework. |
| MTK | Proposal 16. For identifying an AI/ML CSI report via two-sided inference support introducing a new report quantity, i.e., Alt 2. |
| DCM | Proposal 1 - Support the restrictions for AI/ML-based CSI compression to achieve similar effects as the CBSR of the legacy codebooks. - Restrict the amplitude of the target CSI in NW-configured directions. - To reduce the UE complexity, only the hard CBSR can be supported. |

## Discussion

Companies’ proposals are similar to the previous two meetings. A bunch of necessary RRC configurations are proposed and listed as follows. The controversial parts are highlighted in yellow and cyan. The gNB antenna port configuration given by the parameters N1 and N2.

- The codebook parameter combination

- The rank restriction

- Codebook subset restriction (CBSR) configuration

- Pairing ID (single or multiple)

- Subband configurations

- Report quantity (new or reusing legacy)

- New RRC parameter to distinguish from legacy codebook

- New codebook type

FL note on distinguishing the ML CSI feedback from the legacy

The motivation of the yellow parts is to distinguish the CSI feedback via two-side model from the legacy. From FL perspective, the reason of having a new RRC parameter in R19 CSI prediction is that all other RRC parameters are the same as the R18 CSI prediction, thus a new indicator is needed. For R19 CSI prediction monitoring report, a new report quantity is needed because the SGCS is a new metric. However, for CSI feedback via two-sided model, the pairing ID is the unique parameter that does not exist in any CSI report before, thus it is sufficient to use pairing ID to represent the feature.

Based on the discussion in the last meeting, new report quantity seems to be popular. However, the CSI feedback of two-sided CSI is a analogous to a new PMI codebook, where CSI report quantity is still CQI, RI, CRI and PMI. Having a new report quantity is misleading.

Thus, companies are encouraged to provide technical reasons of your preference.

##### Discussion 5.1 (closed)

What is the motivation of having new report quantity, or new RRC parameter? If the intention is to distinguish the new CSI feedback via two-sided model from the legacy, why pairing ID is not enough?

| Companies | Views |
| --- | --- |
| Mod | Pairing ID is the unique parameter that does not exist in any legacy CSI reports; thus it is sufficient to use pairing ID to represent the feature since it is mandatory. R19 CSI prediction needs a new RRC parameter because all other configurations are similar to Rel-18 CSI prediction including the codebook. CSI report quantity is still CQI, RI, CRI and PMI. Having a new report quantity is misleading Companies are encouraged to provide technical reasons of your preference. |
| Samsung | Agree with FL. |
| CMCC | Agree with FL’s assessment. |
| Huawei, HiSilicon | OK with Pairing ID. |
| Nokia | Agree with FL’s assessment |
| SPRD | Agree with FL. |
| vivo | In this case, how can specify that the pairing ID should be mandatorily configured in [AI-based] CSI-reportconfig ? |
| Panasonic | Agree with FL. |
| Xiaomi | If CQI is calculated by using Option 2b, CQI will be separately reported with RI/CR/PMI. Hence, CSI report quantity may not include CQI, and a new report quantity may be needed in this case. |
| Ericsson | Agree with FL. |
| TCL | Agree with FL. |
| Apple | With the current RRC IE structure, RI-restriction and CBSR is part of CodebookConfig. Every time a new codebook is introduced, a new CodebookConfig-rxx is added to CSI-ReportConfig IE. Although CBSR is still under discussion whether it should be supported for AI based CSI compression, I think it is not controversial to minimum support rank restriction. If this is the case, it is inevitable that we minimum need a 4 bit bitstring to carry the rank restriction. Unless we would like to augment PairingID as a structure like legacy CodebookConfig structure. I think both ways works. |

Based on the 1st round discussion, following conclusion can be drawn.

##### Conclusion 5.1a (distinguish with other CSI report, low priority)

Configuration of pairing ID is sufficient to distinguish the CSI feedback with two-sided model with other legacy CSI reports.

FL note on CBSR:

There are 5 companies present interest in specifying CBSR:

- Samsung, Nokia: represent W in uncompressed eType II format, and apply hard CBSR

- Apple: hard CBSR with SGCS as metric

- Google, DCM: hard CBSR

Based on the discussion in the last meeting, CBSR is also favoured by Ericsson and ETRI. However, there are a good number of companies, including OPPO, Xiaomi, Huawei, ZTE, CMCC, Futurewei, MTK, CATT, TCL presented concerns on applying CBSR. Particularly, Huawei mentioned that CBSR is not present in the commercialization thus showing its low priority and necessity.  Given that it is pretty early stage of the release, let us continue discuss and share opinions here. More concrete proposals can be brought up at a later stage based on the views and after finishing the target CSI type discussion.

##### Discussion 5.2 (CBSR, low priority):

Please share comments on the necessity, feasibility and how to support CBSR for CSI feedback via two-sided inference.

| Companies | Views |
| --- | --- |
| Mod | Companies please continue share comments. Interested companies please try to convince others, and let us formulate a more concrete proposal if more companies support this topic. |
| Nokia | Supporting a structure for the target CSI for inference, W, as uncompressed and unquantized eType II format does not require a UE to support eType II codebook because no codebook-based compression is needed. However, is serves two purposes: - It better defines the target CSI, W, in the formula for PDSCH signal a UE assumes for PMI/RI/CQI calculation. In this way W is defined in the same way as in eType II but without compression and quantisation It allows the NW to reuse legacy hard CBSR configuration |
| vivo | Given Huawei comments that CBSR is not present in the commercialization, it can be deprioritized |
| Ericsson | CBSR is an essential feature that needs to be supported to make this use more practical. Co-existence with other services, such as radar, satellite, is an aspect that cannot be ignored. This has been supported in legacy reporting, while we don’t see a valid technical reason for not supporting it for the two-sided CSI compression use case. |
| NTT DOCOMO | CBSR is an essential feature for practical network operations. It is used for interference management in real-life deployments. If a UE reports CSI with directions on which NW does not want to transmit signals, there will be CQI mismatch that is hard to be compensated by NW. Therefore, the alignment between NW and UE on which directions can be used is essential. The major concern of CBSR is UE complexity. For this issue, we can consider the following to reduce the implementation complexity for CBSR: - Support only hard CBSR. - Support the limited number of configured directions, e.g., 4. With the above constraints, UE can pre-process the Target CSI with simpler matrix operations without the FFT. |
| Apple | We are open to have CBSR applied to target CSI. But unlike legacy codebook which CBSR is applied during codebook search, CBSR is an extra effort for AI based CSI compression. As such DoCoMo’s proposal is reasonable in principle. Additionally, we think having an SGCS threshold indicated with the hard CBSR can help UE to simplify applying the CBSR to target CSI. Something like below: - Support only hard CBSR. - Support the limited number of configured directions, e.g., 4. - Network indicates a SGCS threshold for the restricted DFT beams, e.g., UE should exclude a layer from the target CSI if the layer has an SGCS with the restricted DFT beam(s) higher than the SGCS threshold. |

# Specification of target CSI type

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| Furturewe | Proposal 2: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for inference, consider channel matrix as target CSI type for further performance improvements. |
| InterDigital | Proposal 1: Support SRS configurations associated with CSI-RS or CSI feedback resources through explicit offsets, dynamic triggering rules. |
| vivo | - For inference/monitoring/data collection purpose, consider channel matrix as a CSI type of target CSI for TDD scenario. - Compared the performance between channel matrix and precoder matrix at least for TDD scenario |
| Huawei, HiSilicon | Proposal 1: For the evaluation assumption of channel matrix as Target CSI type, at least consider the following configurations: - SRS period/hopping: For 100MHz BWP with 272PRBs, - SRS period includes at least 10ms, 20ms, 40ms. - SRS measurement is considered with at least 17 hops each with 16 PRBs. - DL/UL reciprocity error modeling: at least consider channel estimation error to DL channel and UL channel, where DL/UL channel matrix are generated by adding DL/UL channel estimation error depending on DL/UL SNR, respectively. - Frequency granularity: For 100MHz BWP with 272PRBs, at least consider the Encoder input as the DL channel matrixes for 17 subbands, where a certain PRB among the 16 PRBs is selected per subband for generating the DL channel matrix. - DL Rx number: at least consider 4 Rx, where the DL channel vectors per subband per Rx antenna are considered as individual Tokens. Proposal 2: Regarding the Target CSI type, support channel matrix (H) as Target CSI type. - Note: Fusion with SRS measurement is NW implementation and is transparent to the UE. Observation 4: From the evaluation results over different target CSI types, under TDD scenario, with SRS periodicity of 10ms, 20ms, and 40ms, - Channel matrix as Target CSI (with NW implemented fusion) can outperform SRS measurement only with 16.42%~34.75% throughput gains, and 21%~90% SGCS gain over 4 layers. - Channel matrix as Target CSI (with NW implemented fusion) can outperform precoding matrix as Target CSI with 18.26%~30.9% throughput gains, and 17%~90% SGCS gain over 4 layers Proposal 4: If Target CSI is channel matrix, CSI payload size adaptation can be achieved by reporting partial channel matrix. |
| TCL | Proposal 1: For Target CSI, support the precoding matrix as the baseline. - FFS: Study if the full channel matrix can be used in specific scenarios. |
| ZTE | Observation 1: Under the PC1 payload configuration, the AI/ML-based CSI compression with SRS achieves SGCS performance gains of 75%, 39%, and 17.5% compared to pure SRS measurements, traditional eType II Codebook, and SRS prediction, respectively. Observation 2: Under the PC6 payload configuration, the AI/ML-based CSI compression with SRS achieves SGCS performance gains of 130%, 31%, and 54% compared to pure SRS measurements, traditional eType II Codebook, and SRS prediction, respectively. For Rel-20 study on channel matrix as target CSI type, consider the evaluation assumptions, methodology, and KPIs in TR 38.843 for CSI feedback enhancement as a starting point. - FFS on additional assumptions, e.g., SRS configurations, bandwidth. - Note: additional EVM and the corresponding template to collect the results can be updated. |
| KT | - Proposal 1: Study the feasibility of also supporting channel matrix as target CSI type, particularly for scenarios with a large number of transmit antennas. - Proposal 2: Support representation of the channel matrix in the angular-delay domain, as its sparse structure provides a more efficient approach to target CSI formatting with potential for overhead reduction. |
| Qualcomm | - Channel feedback has the issue of disclosing Rx port information and has phase coherency issue if SRS fusion is employed. The frequency granularity and channel gain information may also impact the performance. To address these concerns, covariance matrix may be a good candidate for study. |

## Discussion

Based on the proposals in the contribution, FL observes following proposals on the target CSI type.

- Support the specification of channel matrix as target CSI:

  - Futurewei, Huawei, ZTE

- Study the performance and feasibility of channel matrix as target CSI

  - ZTE, KT

- Study channel covariance matrix as target CSI:

  - Qualcomm

FL note on target CSI type: The situation remains similar to the last meeting. Per guidance from the Madam chair in the last meeting, let us try to have a discussion on how to proceed with this topic. One consideration is that there is a parallel discussion in 6G AIML agenda. Channel feedback, other types of explicit feedback, and SRS fusion receive good attention for 6G, let us collect some thoughts on whether and how we can proceed in this agenda, before going to the discussion on evaluation methodology.

##### Discussion 6.1 (explicit CSI feedback, low priority):

This use case is being studied 6G AIML agenda, is there a need to repeat the study in R20 5GA?

| Companies | Comments |
| --- | --- |
| Futurewei | We are open to the study in R20 5GA considering the potential performance gain already illustrated by multiple companies. We don’t think we need to wait till 6G to grap such a low hanging fruit. |
| ZTE | We are open to further study this use case in 5G-A scope. For 6G scope, the use case study and discussions are premature. There are many new use cases on the table and the allocated time for the discussion of each use case is quite limited. While for 5G-A scope, we have done many works related to channel feedback in previous releases including evaluation assumptions and potential spec impacts, which builds a solid foundation for further study of new target CSI type. |
| Huawei, HiSilicon | Firstly, it is clearly in the 5GA scope as captured in the agreement. Let’s say it in another way around: there is not need to lead a duplicated study at 6G. Agreement: For CSI feedback via two-sided model, support at least precoding matrix as target CSI type. - FFS whether consider raw channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc.. - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports. Secondly, as mentioned by ZTE, there is huge uncertainty to ensure the CSI compression case with SRS fusion (which has only 3 supporters) can survive with the competition of other ~20 new use cases/sub use cases. |
| InterDigital | We are open to discuss explicit CSI feedback. |
| Nokia | No need to duplicate the study in R20 5GA |
| SPRD | No need to repeat the study in R20 5GA |
| vivo | We are open to further study this use case in 5G-A scope, at least we can evaluate the performance firstly especially considering that 10.1 is more focused on the performance and spec impact of CSI. |
| Panasonic | Considering specification effort point of view, to consider it for 6G would be reasonable. |
| China Telecom | We think it can be studied in 5GA. Since lots of related work for CSI feedback has been done in 5GA, it’ll be more mature to study new target CSI type here, we don’t need to wait for 6G. Thus, if time permits, we support to study this use case in 5GA. |
| TCL | No need to repeat the study in R20 5GA |

If there is sufficient interest in studying the use case in R20 5GA, maybe we can start with the following evaluation methdology.

##### Proposal 6.2 (explicit CSI feedback EVM, low priority):

For the study of explicit channel feedback as another target CSI type, consider following evaluation assumption of channel matrix as Target CSI type, at least consider the following configurations:

- Candidate target CSI type

  - DL Channel matrix: a certain PRB among the NRBSB PRBs of a subband is selected.

  - Covariance matrix : Hi is the channel measurement on RB i of a certain suband with size SBsize, covariance matrix is such that 1NRBSBi=0NRBSBHiH⋅Hi

- Benchmark

  - CSI feedback via two-sided model with precoding matrix as target CSI type

  - eType II PC 1-8

  - SRS (if TDD system is considered)

- Frequency band:

  - 100MHz BWP with 272 RBs, subband size NRBSB=16

  - 20MHz BWP with 50 RBs, subband size NRBSB=4

- If TDD system is considered, SRS fusion can be considered as NW implementation

  - NW side model input: CSI feedback, channel measurement from SRS, covariance matrix / precoding matrix calculated from the SRS measurement.

  - SRS period includes at least 10ms, 20ms, 40ms.

  - SRS hopping

    - 17 hops or 5 hops for 100MHz BWP

  - DL/UL reciprocity error modeling:

    - DL/UL channel estimation error subject to DL/UL SNR.

    - DL/UL calibration error

    - Tx/Rx phase drift between each SRS hop

    - Antenna imbalance

| Companies | Comments |
| --- | --- |
| Futurewei | We are supportive on the direction of this proposal and we are open to further discussions on the EVM. |
| ZTE | OK in general. |
| Huawei, HiSilicon | Supportive with this direction. From the R19 study, using channel matrix to facilitate fusion with SRS measurement has significant gains over precoding matrix (which can hardly justify the motivation due to its limited gain over eT2). From our view, this is the only/major motivation of a potential commercialization of two-sided CSI compression. |
| InterDigital | We are ok with the intention of the proposal, however to manage the workload, we prefer to restrict the scope of the proposal to the SRS fusion for TDD systems. |
| Nokia | This can be part of 6G AIML study |
| vivo | One clarification question: Why is the covariance matrix defined as the average covariance matrix, rather than the covariance matrix for a specific PRB? |
| China Telecom | Support the direction. |
| TCL | Support to study. |

# Processing timeline, criteria, active resource and capability

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| Spreadtrum | Proposal 6: For CSI compression, for inference, for PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,  OAPU= 0 or N is reported by UE  OCPU=0 or M is reported by UE  Note: Detailed values of N and M can be further discussed in UE feature.  Note: Combination of OAPU= 0 and OCPU=0 is not allowed Proposal 7: For occupancy duration of CPU and APU for CSI compression. • Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report • Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report Proposal 8: For the determination of CSI report priority value of a CSI report for inference, the existing Pri_iCSI (y,k,c,s) is reused  k = 1 for the CSI report for inference. |
| Huawei, HiSilcon | Proposal 11: Regarding the occupancy of PU for AI/ML-based CSI compression, for inference, OAPU and/or OCPU are occupied with UE capability report.  Combination of OAPU = 0 and OCPU =0 is not allowed.  For the OAPU > 0 and OCPU > 0 case, if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to be updated, and neither of the PUs are occupied.  CPU/APU occupation starts from the latest RS occasion (for SP-CSI report) / DCI (for A-CSI report) till PUSCH carrying the CSI report. Proposal 12: Regarding the CSI processing timeline for AI/ML-based CSI compression, for inference, extend legacy Z_2/ Z_2^' to Z_2+d/ Z_2^'+d, where additional delay d is reported by UE per SCS. Proposal 13: Regarding the CSI priority rules, reuse existing CSI priority 〖Pri〗_iCSI (y,k,c,s) by considering k=1 for inference CSI report. |
| InterDigital | Proposal 7: Support CPU pool sharing between legacy CSI reporting and AI/ML-based CSI. Proposal 8: Support PU pool sharing by different AI/ML functionalities (e.g., AI/ML based CSI reporting, AI/ML based BM, AI/ML based POS). Proposal 9: Support AI/ML Processing Units (PU) occupancy definition per AI/ML functionality (e.g., AI/ML based CSI reporting, AI/ML based BM, AI/ML based POS). |
| Google | Proposal 5: For CPU/APU rule, support to reuse the same mechanism of AI/ML based CSI prediction for AI/ML based CSI compression • UE reports the number of CPUs, number of APUs and the APU pool for AI/ML based CSI compression Proposal 6: For minimum processing delay, support to reuse the same mechanism of AI/ML based CSI prediction for AI/ML based CSI compression • UE reports the additional processing delay for AI/ML based CSI compression on top of the processing delay for Rel-19 eType2 CSI report |
| CATT | Proposal 18: For inference report of AI/ML-based CSI compression, support the following for PU occupancy  OAPU= 0 or N is reported by UE  OCPU=0 or M is reported by UE  The values of N and M are reported by UE  Combination of OAPU= 0 and OCPU=0 is not allowed Proposal 19: For inference report of AI/ML-based CSI compression, the legacy mechanism of occupancy duration for R19 eType II codebook can be reused, if associated monitoring report is not configured. Proposal 20: For CSI compression, for inference, in addition to legacy Z/Z’ for R19 eTypeII codebook, UE report the value of t per SCS  The value of t is reported by UE Proposal 21: For inference report of AI/ML-based CSI compression, the priority of the inference report is the same as CSI report not carrying L1-RSRP or L1-SINR (analogous to Rel-19 eType II codebook). |
| Vivo | Proposal 13: For CSI compression using UE-side model, the similar rule that agreed in CSI predication can be reused, e.g., - For PU occupancy, support Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - the same occupancy duration is used if both CPU and APU are reported non-zero value - the occupied PU of {48,64,128} ports can be OPU=P/32*OPU_reported or OPU_reported Proposal 14: For CSI compression using UE-side model, for inference of up to 32 ports, in addition to legacy Z/Z’ for type II codebook, UE may report the value of t per SCS - Detailed value of t can be discussed in UE feature Proposal 15: For CSI compression using UE-side model, for inference of {48,64,128} ports, the Z value can be P32 times of Z value of 32 port, or equal to Z value of 32 port Proposal 16: For the determination of CSI report priority value of a CSI report for inference, the existing Pri_iCSI (y,k,c,s) is reused - k = 1 for the CSI report for inference Proposal 17: For the determination of CSI report priority value of a CSI report for monitoring, the existing Pri_iCSI (y,k,c,s) is reused - k = 1 for the CSI report for monitoring |
| Xiaomi | Proposal 14: Reuse legacy CSI reporting priority rule definition, and k=1 for two-sided AI model based CSI feedback. Proposal 15: For two-sided AI model based CSI compression, reuse the CSI processing criteria for AI based CSI prediction, and extend legacy Z/Z’ to Z/Z’+t’ for CSI generation part model inference, where value of t’ is reported by UE per SCS. |
| TCL | Proposal 4: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules. Proposal 5: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting. Proposal 5: Consider to reuse the definition of AI/ML PU occupancy in CSI prediction. • FFS if the value of OAPU is the same or not. • FFS if the value of occupancy duration of OAPU is the same or not. Proposal 4: Consider if different values of OCPU should be defined for different target CSI. Proposal 6: Study the selection rule if the AI-related CSI report to be reported is jointly determined by the legacy CPU and AI/ML PU. • Option 1: Selection of CSI report prioritizes the AI/ML PU constraint. • Option 2: Selection of CSI report prioritizes the legacy CPU constraint. • Option 3: Selection of CSI report jointly considers the legacy CPU and AI/ML PU constraint. Proposal 7: The occupation time of CPU and A/ML PU should be considered separately. The duration of the PU occupation time could be configured by the NW. |
| ZTE | Proposal 16: For CSI feedback using two-sided model, dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied for an inference report, where OAPU= 0 or N is reported by UE in UE capability report, and OCPU= 0 or M is reported by UE in UE capability report. Proposal 17: For CSI feedback using two-sided model, for an inference report, Rel-15 CPU occupation time is reused for APU occupation time and/or CPU occupation time. Proposal 18: For CSI feedback using two-sided model, for an inference report, extend legacy Z2 / Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS in UE capability report. Proposal 19: For the determination of CSI report priority, the existing Pri_iCSI (y,k,c,s) is reused, where k = 1 for CSI reports for inference and monitoring. |
| Samsung | Proposal#10: Defer the discussion on CSI processing criteria and priority rule for two-sided model-based CSI compression until some progress on PMI feedback is achieved. |
| Lenovo | Proposal 15: For AI based CSI compression report, for PU occupancy, dedicated AI/ML PU and/or legacy CPU are occupied, and the UE separately report the number of AI/ML PUs and the number of CPUs for a CSI compression report. Proposal 16: For AI based CSI compression report, based on legacy (Z_2,〖Z'〗_2 ), the UE indicates where additional CSI computation is needed. Proposal 17: Specify a few classes for model complexity. The encoder of the AI-based CSI-feedback is then assigned to one of the classes of the model complexity. Proposal 18: Specify a few classes for AI HW capability. The UE reports the HW capability group to the NW. Knowing the active AI model, the gNB is able to determine the required APU. Proposal 19: k=1 is adopted to determine the CSI report priority value for AI based compressed CSI report. |
| Panasonic | Proposal 11: For CSI compression using two-sided model, to calculate the inference report, UE reports the number of dedicated AI/ML PUs and /or legacy CPUs by reusing the mechanism of CSI prediction as the baseline. Proposal 12: For CSI compression using two-sided model, for inference, in addition to legacy processing timeline, UE may report the offset value per SCS. |
| OPPO | Proposal 12: Regarding the aspects of CPU/APU design and occupation time, active resource/port counting, timeline, reuse the related specification outcome for inference in BM and CSI prediction agenda. Proposal 13: Support to distinguish the priority of AI/ML and legacy CSI report for inference, and the priority of CSI report for inference and monitoring. |
| HONOR | Proposal 6: For CPU/APU occupancy, further study whether CPU/APU occupancy is specific for different layers/ranks. |
| LGE | Proposal #10: For CSI compression, study on PU counting rule considering • Different types of CSI reports, e.g., inference or monitoring • Different types of AI/ML models, e.g., encoder, proxy/nominal decoder, intermediate KPI estimator Proposal #11: Study on methods to manage the NW-side computing capability for two-sided model, e.g., NW indication of inference skipping. |
| Apple | Proposal 9: For AI based CSI compression CPU/APU, use R19 CPU/APU framework as a starting point. |
| NEC | Proposal 13: In CSI spatial/frequency compression, to calculate the inference report for compressed CSI, - For PU occupancy, support:  Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,  OAPU= 0 or N is reported by UE  OCPU=0 or M is reported by UE  Note: Detailed values of N and M can be further discussed in UE feature.  Note: Combination of OAPU= 0 and OCPU=0 is not allowed  Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied - For occupancy duration of CPU and APU,  the CPU occupancy time for CSI report with reportQuantity set to 'cri-RI-PMI-CQI' can be reused  the same occupancy duration is used if both CPU and APU are reported non-zero value Proposal 14: In CSI spatial/frequency compression, for a CSI report for inference, extend legacy Z2/Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS for CSI spatial/frequency compression. - Detailed values of d and d’ can be further discussed in UE feature. Proposal 12: In CSI spatial/frequency compression, support to define additional new priority rule for the following collisions: - AI/ML-based CSI report and non-AI/ML-based CSI report. - Inference CSI report and monitoring CSI report. |
| Fujistu | Proposal 8:  The legacy calculation of priority values could be reused for AI/ML-based CSI feedback.  The AI/ML-based CSI report should have a higher priority over the codebook-based CSI report, which could be realized by implementation. |
| MTK | - Proposal 17: Consider a dedicated AI/ML PU for AI/ML-based CSI Compression which will not be shared with other AI/ML functionalities. - Proposal 18: Consider adding additional delay based on UE’s capability report into legacy timeline in order to accommodate delays incurred from loading AI/ML model onto dedicated AI processing unit and its execution. |
| CMCC | Proposal 11: For AI/ML based CSI compression, to calculate the inference report, • For PU occupancy, support o Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or N is reported by UE - OCPU=0 or M is reported by UE - Note: Detailed values of N and M can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied • For occupancy duration of CPU and APU, o the same occupancy duration is used if both CPU and APU are reported non-zero value o if associated monitoring report is not configured, reuse following legacy occupancy duration - For periodic and semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report), occupation starts from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, respective latest CSI-RS/CSI-IM occasion no later than CSI reference resource, until the last symbol of the PUSCH carrying the report - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. Proposal 12: For AI/ML based CSI compression, extend legacy Z/Z’ to Z+d / Z’+d’, where d is reported by UE per SCS • Detailed values of d can be further discussed in UE feature Proposal 13: For AI/ML based CSI compression, for the determination of CSI report priority value of a CSI report for inference, the existing 〖Pri〗_iCSI (y,k,c,s) is reused k = 1 for the CSI report for inference |
| ETRI | Proposal 9: For AI/ML-based CSI compression, adopt the priority rules for the legacy type2-r16 CSI reports. |
| Qualcomm | Proposal 12: Adopt existing framework of CPU and APU counting for CSF via two-sided model, UE may report dedicated CPU and APU cost for this feature. Proposal 13: Adopt existing timeline formulation for CSF via two-sided model, UE may report dedicated additional time requirement t for both the aperiodic CSI reporting and semi-persistent CSI reporting. |
| DCM | Proposal 8 - Rel-16 eType2 CSI is the baseline of the CSI processing criteria and timeline, with the following enhancements  UE reports APU and CPU occupation numbers, reusing the mechanism of CSI prediction as the baseline.  Relaxation of the processing timeline if necessary. |
| Sharp | Proposal 5: For determination of CSI report priority value of a CSI report for inference for CSI compression, the existing PriiCSI(y,k,c,s) is reused where k =1 for the CSI report. Proposal 6: Further consider extending the CSI computation time for P/SP CSI report in CSI compression. |

## Discussion

FL note: From the proposals, almost all companies propose to reuse the legacy CSI processing unit rules and legacy AI processing unit rules. Two companies propose to adopt dedicated processing units pool for CSI compression. In RAN-plenary #108, it has been agreed to adopt two processing unit pools and it’s upto UE to select which pool to use for each feature. FL think this proposal can be handled within the scope of this proposal, i.e., UE selecting a dedicated pool for CSI compression without sharing with other features.

Similarly, most companies propose that legacy timeline framework can be adopted by allowing UE to report additional processing time for this feature. There is one proposal from InterDigital considering model loading time for inactive models. In Rel-19 study, model loading time has been studied, and the assumption is that it can be absorbed into the specified / reported timeline.

For priority rule, lots of company propose to adopt k=1 for AIML CSI compression, just like other non-AI CSI feedback. Some other companies prefer to considering different priority for AIML and non-AI CSI feedback, while some others think this may be handled by NW implementation by assigning different report IDs.

All above aspects can be discussed in a later stage of the release after the PMI type, CQI, payload, UCI mapping become stable and clear.

FL observation 6.1 (not for online / offline session)

- Majority companies propose to adopt legacy CSI processing unit rule and legacy AIML processing unit rule for CSI compression via two-sided model. Specific CPU and APU occupation values can be reported by UE capability

- Majority companies propose to adopt Rel-19 timeline formulation for CSI compression via two-sided model, allowing UE to report additional time requirement on top of existing {Z, Z’} values.

- Majority companies propose to adopt same priority rule to address CSI collision by setting k=1 for CSI compression via two-sided model.

| Companies | Comments |
| --- | --- |
| Futurewei | We agreed with FL’s observation. |

# Other topics proposed by companies to 10.1.1.1

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| TCL | Proposal 8: Ran1 should consider whether/how to transmit the raw CSI for monitoring purpose. • FFS: The overhead reduction scheme based on spectral or temporal processing. Proposal 9: Study the triggering mechanism based on UCI loss event for CSI information reset or retransmission. • Option 1: UCI loss event is determined by NW • Option 2: UCI loss event is determined by UE Proposal 10: Investigate the CSI information synchronization for mitigating the impact of UCI loss. • Option 1: Reset the CSI information to pre-defined values. • Option 2: NW configs or UE initials CSI information retransmission. |
| China Telecom | Proposal 1: For the determination of target CSI type, deprioritize the use of unquantized channel matrix. Proposal 2: Support enhanced e-Type II like codebook with new parameters as the target CSI type, with W2 matrix optimized through techniques such as quantization and decomposition. |
| MTK | Proposal 1. For pairing and applicability report of two-sided AI/ML models using AI/ML model ID, the following procedure is proposed as the starting point of discussion: • Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. • Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. • Step 3: NW sends a sets of AI/ML model IDs available at NW-side and ready to be paired and may provide initial configuration of each inquired AI/ML model ID. • Step 4: UE reports applicable AI/ML model IDs corresponding to the AI/ML model IDs inquired by NW during step 3. • Step 5: NW configures a set of applicable AI/ML models based on the report received in step 4 from UE and may provide updated configurations compared to step 3. |
| DCM | Proposal 2 - Support specifying a phase normalization method for the target CSI. |

## Discussion

There seems to be no essential issues in the section. Please provide your views on any other essential issues that have not yet been captured in the discussion.

| Companies | Comments |
| --- | --- |

# Proposals for online sessions

## Monday

##### Proposal 2.1a (VQ vs SQ, high priority):

Support NW optionally exchanges codebook to the UE side.

- If NW exchanges a codebook, the exchanged codebook is associated to the used for this pairing ID.

  - FFS SQ / VQ / both is supported

  - FFS the details, e.g., whether exchange the NW-defined codebook or the index of a standardized codebook

- If NW does not exchange the codebook or an index, a standardized SQ codebook is associated to the used for this pairing ID.

- Note: for the same quantization resolution Q, a common codebook is used for all the scalars, all the layers and all parameter pairs for the associated pairing ID with same quantization resolution Q.

##### Proposal 2.2a (param-pair, high-priority):

For SQ, at At least support per-layer payload parameter pairs {32, 2}, {64, 2}, {128, 2}, {192, 2}

- Further study whether support per-layer payload parameter pairs {32, 1}, {48, 2}, {96, 1}, {96, 2}, {192, 1}

- The above pair refers to the pair of latent dimension and quantization resolution per dimension, i.e., {d, Q} for SQ or {d, Q/L} for VQ (if VQ is supported).

##### Proposal 2.4 (pair {64, 2}, high priority):

For the PC with rank-1 payload of 128 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength.

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {64, 2} | {64, 2}, {64, 2} | {32, 2}, {32, 2}, {64, 2} | {32, 2}, {32, 2}, {32, 2}, {32, 2} |

##### Proposal 2.5 (pair {128, 2}, high priority):

For the PC with rank-1 payload of 256 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {128, 2} | {128, 2}, {128, 2} | {64, 2}, {64, 2}, {128, 2} | {64, 2}, {64, 2}, {64, 2}, {64, 2} |

## Tuesday

## Wednesday

##### Proposal 2.5 (pair {128, 2}, high priority):

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 256 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength.

| Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- |
| {128, 2} | {128, 2}, {128, 2} | {64, 2}, {64, 2}, {128, 2} | {64, 2}, {64, 2}, {64, 2}, {64, 2} |

##### Proposal 2.3a (pair {32, 2}, high priority)

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 64 bits, down-select from following alternatives in RAN1 #124. The layers are reported in the order of decreasing strength.

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| Alt1 | {32, 2} | {32, 2}, {32, 2} | {32, 2}, {32, 2}, {32, 2} | {32, 2}, {32, 2}, {32, 2}, {32, 2} |
| Alt2 | {32, 2} | {32, 2}, {32, 2} | NA | NA |
| Alt3.1 | {32, 2} | {32, 2}, {32, 2} | {32, 1}, {32, 1}, {32, 2} | {32, 1}, {32, 1}, {32, 1}, {32, 1} |
| Alt3.2 | {32, 2} | {32, 2}, {32, 2} | {32, 2}, {32, 1}, {32, 1} | {32, 1}, {32, 1}, {32, 1}, {32, 1} |

##### Proposal 2.6 (pair {192, 2}, high priority):

If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 384 bits, downselect from following alternatives in RAN1 #124. The layers are reported in the order of decreasing strength.

| Pair index 6 | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| Alt1.1 | {192, 2} | {192, 2}, {192, 2} | {96, 2}, {96, 2}, {192, 2} | {96, 2}, {96, 2}, {96, 2}, {96, 2} |
| Alt1.2 | {192, 2} | {192, 2}, {192, 2} | {192, 1}, {192, 1}, {192, 2} | {192, 1}, {192, 1}, {192, 1}, {192, 1} |
| Alt2 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {64, 2}, {64, 2}, {128, 2}, {128, 2} |
| Alt3.1 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {96, 2}, {96, 2}, {96, 2}, {96, 2} |
| Alt3.2 | {192, 2} | {192, 2}, {192, 2} | {128, 2}, {128, 2}, {128, 2} | {192, 1}, {192, 1}, {192, 1}, {192, 1} |

##### Conclusion 3.1a (latent message, medium priority)

Whether the latent message has a hierarchy structure or non-hierarchy structure to support different payload sizes is upto NW and UE implementation. No further discussion is needed in Rel-20 on this aspect.

- Note: Hierarchy structure of a latent message means that a latent message with a lower payload size is a subset of a latent message with a higher payload size

##### Conclusion 3.2a (UE reporting actual payload size, medium priority)

There is no consensus on determining the actual payload size based on other UE indications than RI. No further discussion is needed in Rel-20 on this aspect.

## Thursday

##### Conclusion 3.1b (latent message, high priority)

Whether the latent message has a hierarchy structure or non-hierarchy structure to support different payload sizes is upto NW and UE implementation.

- Note: Hierarchy structure of a latent message means that a latent message with a lower payload size is a subset of a latent message with a higher payload size

- Note: UCI omission is a separate discussion with the above consideration.

##### Conclusion 5.1a (distinguish with other CSI report, medium priority)

Configuration of pairing ID is sufficient to distinguish the CSI feedback via two-sided model with other legacy CSI reports.

##### Conclusion 3.2a (UE reporting actual payload size, medium priority)

There is no consensus on determining the actual payload size based on other UE indications than RI. No further discussion is needed in Rel-20 on this aspect.

## Friday

# FL closing remark

In this meeting, good progress was made regarding the following aspects

- NW optionally exchanges quantization codebooks, and standardized SQ codebook is used if NW does not exchange the codebook.

- 4 parameter pairs have been identified, with another 5 as candidates for further discussion

- 2 parameter combinations (PC) across ranks are agreed. Alternatives of another 2 PC are listed and as starting point for downselection.

For the next meeting, high priority topics include

- Down-selection of SQ and VQ when NW exchanges codebook

- Specification of SQ codebook

- Finalize remaining parameter combination discussion.

- Study UCI grouping and omission options

- Continue study CQI assumptions

  - Considering option 1a/1b/2a (other CQI estimation algorithm is not precluded) as implementation choices

  - Study includes spec impacts and performance benefit over benchmark of eT2 and option 2b (which is already supported by current spec)

- Study necessity and performance impact by CBSR.

# Previous agreements

## RAN1 #122

Agreement:

For CSI feedback via two-sided model, support at least precoding matrix as target CSI type.

- FFS whether consider raw channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc.

Agreement

Specify the precoding matrix feedback via two-sided model as follows: For a certain layer l and rank ν,

- A precoding matrix is mapped with a latent message zl=z0l, z1l,⋯,zdzl,ν-1l, with l = 1, ..., ν where l is the layer index.

  - The mapping is subject to [at least] a pairing ID configured by high-layer signalling.

  - FFS dzl,νis common or specific for different layers or ranks

  - Note: the terminology “latent message” is used for discussion purposes, the detailed name can be discussed further

- zl is quantized and mapped to bit sequence before reporting.

Agreement

For precoding matrix feedback, for a certain layer l of rank ν, the latent message zl contains dzl,ν real values

- If scalar quantization (SQ) supported, it refers to the quantization of each real value independently,

  - The payload size for layer l and rank ν is determined by dzl,ν×Ql,ν, where Ql,ν is the number of bits per scalar.

- If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized.

  - The payload size for layer l and rank ν is determined by dzl,νL×Ql,ν, where L is the length of the vector segment,  Ql,ν is the number of bits per segment

  - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded.

Agreement:

For precoding matrix feedback, consider the following aspects for payload determination

- Support only SQ, or only VQ or both

- dzl,ν and / or Ql,ν and / or L are fixed values or determined from multiple values based on NW configuration or UE reporting.

- dzl,νand / or Ql,ν is layer-common or layer-specific, rank-common or rank-specific.

- If configured, dzl,νand / or Ql,ν and / or L above are configured separately from pairing ID, or configured by Pairing ID.

  - The configuration may be via other parameters which are used to derive dzl,ν, L and Ql,ν.

Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges.

Note: dzl,ν, Ql,ν and L are independent on subband and port configurations,

- FFS each combination of dzl,ν, Ql,ν and L is applicable to a certain set of subband and port configurations

Agreement:

For inference report configuration, support separate configuration of subband, Tx port and payload size from pairing ID.

Agreement:

For quantization codebook used for quantizing the feedback,

- For inter-vendor collaboration Direction A, select one of the following:

  - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters

  - Alt2: Use a standardized quantization codebook decided by RAN1.

## RAN1 #122bis

Agreement:

Support precoding matrix with port-subband domain representation as target CSI type.

- For a certain layer l, a precoding matrix on N_sb subband and P CSI-RS ports is Winl=[Win,0l, Win,1l,⋯,Win, Nsb-1l] with Win,kl being a P×1 vector.

- Note: Only sub-band PMI reporting is supported.

Agreement:

For determining the payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and ranks, consider following general principles for precoding matrix-based target CSI

- Rank-common and layer-common model with payload-scalable design is considered as the baseline implementation for discussion purpose.

  - Note: layer-specific / rank-specific (if supported) payload parameters are supported by this rank-common and layer-common model with payload-scalable design.

- Limiting the total number of (per-layer) payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and all ranks.

- At least for medium to high per-layer payload regime, the total payload size of rank 2, rank 3 and 4 are comparable to each other.

- Spectral efficiency or system throughput and trade-off with feedback overhead are considered as the performance metric.

Agreement:

When the Target CSI is precoder matrix, in order to down-select to N per-layer payload parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν based on performance, overhead and complexity trade-off results, consider the following candidate values:

- Candiate values for dl,ν: 32, 64, 96, 128, 192

- Candidate values for Ql,ν: 1, 2, 3, 4, (8, 10 only for VQ)

  - 2 is considered as the baseline assumption for SQ.

- Candidate values for L: L=2, 4,  dl,ν

- Candidate values for N: 4, 5, 6

- Note: These N payload PCs of dl,ν,Ql,ν or dl,ν, L,Ql,ν are considered as the pool for the payload PC for each layer and each rank.

- Note: The above is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: Rank-common and layer-common model with payload scalable design is considered.

- Note: The above does not mean support of both SQ and VQ.

- Note: The number of parameters combinations configurations (Y) does not exceed 8.

Agreement:

For payload parameter combinations (PC), consider followings

- Support Y configurable PCs (across layers and ranks) as below, where X_n, Xn_31, Xn_32, Xn_33, Xn_41, Xn_42, Xn_43, Xn_44 represent a pair of dl,ν,Ql,ν or a triplet of dl,ν, L,Ql,ν  chosen from the per-layer PC pool.

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| PC1 | X1 | X1, X1 | X1_31, X1_32, X1_33 | X1_41, X1_42, X1_43, X1_44 |
| PC2 | X2 | X2, X2 | X2_31, X2_32, X2_33 | X2_41, X2_42, X2_43, X2_44 |
| … |  |  |  |  |

- For rank 1 and rank 2, layer-common and rank-common payload is supported.

- For rank 3, consider alternatives of {Xn_31, Xn_32, Xn_33} = {a, a, a}, {a, a, b}, {a, b, b} for potential down selection.

- For rank 4, consider alternatives of {Xn_41, Xn_42, Xn_43, Xn_44} = {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c} for potential down selection.

- Note: The table is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: a, b values in rank3 can be different from that in rank 4.

Agreement:

Consider following for evaluation:

- For down-selection of per-layer pairs of dl,ν,Ql,ν or a triplets of dl,ν, L,Ql,ν , average SGCS across per layer can be considered as performance metric.

- For down-selection of the rank-specific and layer-specific patterns (e.g., {Xn_31, Xn_32, Xn_33} and {Xn_41, Xn_42, Xn_43, Xn_44}), spectral efficiency or throughput should be considered.

Agreement:

For CSI feedback via two-sided AIML models, reuse legacy CSI reporting framework

- Support CSI report based on periodic, semi-persistent and aperiodic CSI-RS resources with legacy measurement resource configurations.

- Support aperiodic report and semi-persistent CSI report on PUSCH and reuse legacy triggering mechanism.

# References

- TR 38.843 v19.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” (Release 19), June 2025

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm (Moderator), 3GPP TSG RAN #108, June 2025