3GPP TSG-RAN WG1 Meeting #123	  R1-2509508

Dallas, USA, November 17 – 21, 2025

NNTT

Agenda Item:	10.1.1.2

Source:	Moderator (Huawei)

Title:	FL summary#4 for other aspects of CSI compression

Document for:	Discussion and Decision

- Introduction

  - [Rd1] Contact info

Before going to the discussions in the following sections, companies are invited to type in the contact person information into the following table.

Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Jianfeng Wang | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| Qualcomm | Chenxi Hao | chenxih@qti.qualcomm.com |
| Huawei, HiSilicon | Keyvan Zarifi | Keyvan.zarifi@huawei.com |
| Ericsson | Jingya Li, Xinlin Zhang | Jingya.li@ericsson.com Xinlin.zhang@ericsson.com |
| MediaTek | Pedram Kheirkhah Sangdeh Reubengeorge Stephen | Pedram.kheirkhah@mediatek.com Reubengeorge.stephen@mediatek.com |
| CATT | Qianrui Li | liqianrui@catt.cn |
| Xiaomi | Qin Mu Min Liu | muqin@xiaomi.com liumin10@xiaomi.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| Panasonic | Tetsuya Yamamoto Hidetoshi Suzuki Xuan Tuong Tran (Henry) | yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@jp.panasonic.com xuantuong.tran@sg.panasonic.com |
| CMCC | Yi Zheng Jiazhen Zhang | zhengyi@chinamobile.com zhangjiazhen@chinamobile.com |
| LGE | Yechan Chun | Yechan.chun@lge.com |
| TCL | Pu Yuan Tianqi Wu | Pu.yuan@tcl.com tianqi1.wu@tcl.com |
| NEC | Peng Guan Shafivulla Sayyed Yi Jiang | guan_peng@nec.cn Sayyed.Shafivulla@india.nec.com y-jiang_ct@nec.com |
| NTT DOCOMO | Xin Wang | wangx@docomolabs-beijing.com.cn |
| Apple | Huaning Niu Yuan Zhu | huaning_niu@apple.com yuan_zhu3@apple.com |
| ZTE | Wenfeng Liu | liu.wenfeng@zte.com.cn |
| InterDigital | Afshin Haghighat | afshin.haghighat@interdigital.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| Sony | Sam Atungsiri Yingshuang Bai | sam.atungsiri@sony.com yingshuang.bai@sony.com |
| Toyota | Konstantinos Dimou | konstantinos.dimou@toyota.com |
| Southeast University | Jiajia Guo Chengyong Jiang | jiajiaguo@seu.edu.cn chengyongjiang@seu.edu.cn |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |

- Data collection

  - Summary of company proposals

### 2.1-1: Generic

| Company | View |
| --- | --- |
| LGE | Proposal #7: For data collection for training, study how to efficiently construct dataset (e.g. data omission/drop, filtering) |
| DOCOMO | Proposal 5 - Support the synchronized and consistent data collection between NW and UE. Proposal 6 - Support the notification of the pairing ID for the synchronized data collection. |
| Samsung | Observation#7: For both NW-side and UE-side data collection, several CSI-ReportConfig related parameters are needed, e.g., CMR, N1N2 configuration, CBSR, codebook config, rank restriction, pairing-related ID etc. A single CSI-ReportConfig can be applied to both NW-side and UE-side data collection for efficiency, configuration, resource/port counting, CSI computation. Proposal#6: For efficiency, a CSI-ReportConfig can be configured for both NW-side and UE-side data collection - Linkage between CSI-LoggedMeasurmentConfig and CSI-ReportConfig for NW-side data collection. - ReportQuantity set to ‘none’ |
|  | Proposal#8: For NW-side and UE-side data collection report configurations, consider - CPU counting/occupancy - CSI processing timeline - Interpretation of other configurations, e.g., CBSR, RI restrictions, if supported. |
| Southeast University | Proposal 1-6: Further discuss the possibility of alleviating data collection requirements by introducing CSI-free unsupervised CSI compression model training scheme. Proposal 1-7: Study whether CSI-free unsupervised CSI compression model training scheme can co-exist with ordinary CSI compression model training scheme. |

### 2.1-2: NW side data collection

| Company | View |
| --- | --- |
| Apple | Observation 1: For NW side data collection, the complexity to obtain Tx eigen vectors are much lower than the complexity of SVD operation on a Tx covariance matrix. Proposal 1: Scale the target CSI by Ntx and apply fixed quantization range, e.g., [-1.82, 1.82], for each of the 2N1N2N3v real/imaginary part. Proposal 2: For NW side data collection option 1, 5bits per real or imaginary part scalar quantization is adopted to evenly quantize each real or imaginary part within range of [-1.82, 1.82]. Proposal 3: Do not support eTypeII based training data set quantization. |
| CATT | Proposal 1: For AI/ML based CSI compression Case 0, regarding the Opt1 scalar quantization of target CSI in NW-side data collection, support separate quantization of the real and imaginary part without normalization (Alt1-1) or with implementation specific normalization (Alt1-2). Proposal 2: For AI/ML based CSI compression Case 0, regarding the Opt1 scalar quantization of target CSI in NW-side data collection, support the same quantization range for the real and imaginary part as [-1,1], i.e., T1min=T2min=-1, T1max=T2max=1. Proposal 3: For AI/ML based CSI compression Case 0, regarding the Opt1 scalar quantization of target CSI in NW-side data collection, support the same quantization bits for the real and imaginary part, i.e., k1=k2. Observation 1: For AI/ML based CSI compression Case 0, regarding the Opt1 scalar quantization of target CSI in NW-side data collection, if UE implementation-specific normalization and truncation is applied, k1=k2=4bits quantization can guarantee less than -1% performance degradation with respect to FP32; if no normalization is applied, k1=k2=6bits quantization can guarantee less than -1% performance degradation with respect to FP32. Proposal 4: For AI/ML based CSI compression Case 0, regarding the Opt1 scalar quantization of target CSI in NW-side data collection, support one of the following quantization bits configurations for the real and imaginary part: - k1=k2=6 bits without normalization - k1=k4=4 bits with UE implementation-specific normalization and truncation Observation 2: The complexity of Opt1 and Opt4 is dominated by SVD or eigenvalue decomposition, which is implementation specific. Proposal 5: For AI/ML based CSI compression Case 0, additional content can be optionally included in NW-side data collection report: - to distinguish the layer/rank - to indicate data quality to facilitate precoding matrix reconstruction from a specific target CSI codebook configuration |
| CMCC | Proposal 1: In CSI compression using two-sided model use case, regarding the target CSI format for NW side data collection, Option 0 and Option 4 can be supported. - Option 0: Reusing legacy e-Type II codebook - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) Proposal 2: In CSI compression using two-sided model use case, for target CSI reporting for NW-side data collection, the rank of the target CSI is configured by gNB and RI reporting in CSI is not needed. Proposal 3: In CSI compression using two-sided model use case, regarding target CSI reporting for NW-side data collection, CQI reporting in CSI is not needed. FFS: whether CRI or LI is needed for reporting or not |
| Ericsson | - For the target CSI format for NW-side data collection, Option 1 is not feasible for NW-side data collection for 128 CSI-RS ports and 19 subbands, when the quantization bits per complex element is greater than or equal to 9. - Compared with Option 1, Option 4 can enable NW to collect more data samples in an RRC message. Proposal 7: If Option 1 is adopted, the real and imaginary parts are quantized separately, using 3 bits per part (6 bits per element). The quantization should be uniform in the interval -c/√(N_ports ) to c/√(N_ports ), where N_ports is the number of CSI-RS ports. - Compared to Option 1, Option 4 provides a better overhead vs accuracy trade-off, since it keeps important elements in the channels more efficiently. Proposal 8 Support Option 4 as the target CSI format for NW-side data collection, due to its feasibility for 128 ports configuration, and better payload vs accuracy trade-off. Pairing ID is not needed for configuring NW-side data collection, and UE shall log at least cell ID, measurement configuration ID and target CSI samples per CSI logged measurement configuration per cell. |
| ETRI | Proposal 1: For AI/ML-based CSI compression, consider NW-side data collection under various environments and UE implementations. Proposal 2: For AI/ML-based CSI compression, consider the use of pairing ID and data format indication as assistant information for NW-side data collection. |
| Fujitsu | Observation-1：Regarding reference amplitude bit number and phase bit number, its legacy value of Ref_bit=4 and Phase_bit=4 can be reused, since no obvious SGCS increase can be observed with enlarged value. Observation-2: Differential amplitude bit number 4 can be selected as its new value, and a larger value than 4 is not helpful to further SGCS increase. Observation-3: Based on simulations for Beta, pv, and L selection respectively, it can be found: - SGCS increase is not significant after Beta=0.5. - SGCS of pv=0.85 and SGCS of pv=0.95 are almost the same. - SGCS increase is not significant after L=12, a lager L value can be considered for layer2 and higher layers Observation-4: Compared to the model trained by FP32 Target CSI, the model trained by Target CSI quantized with eT2-like codebook (new PC, PC9) has no performance loss in large CSI payload case (Z ≥ 230 bits), and negligible performance degradation in low and medium payload cases. Proposal-1: The following two parameter combinations are selected as the new PC for eT2-like codebook to support higher resolution Target CSI quantization. PC; L; Pv; Beta; Ref_bit; Diff_bit; Phase_bit. PC9; 12; 0.95; 0.5; 4; 4; 4. PC10; 16; 0.95; 0.5 or 0.6; 4; 4; 4. Observation-5: From performance perspective, no SGCS performance degradation is observed in the SGCS comparison between the model trained with the Target CSI via Option 4 and the model trained with Target CSI via FP32. Similar observation can be found for the model trained with the Target CSI quantized by Option1/Scalar8. Observation-6: To reach the similar model performance, the payload size of Option1/Scalar8 is four times higher than that of the Optoin4/PC9 based on the evaluations. Observation-7: Option1/Scalar8 cannot meet the container size restriction of one RRC message, i.e. 9Kbyte, in some CSI configurations. Observation-8: The comparison of Option1 and Option4 can be summarized as the following table: Payload; Performance; Complexity. Option1; 4x, some cases are out of the size of one RRC message; Similar performance as that of FP32 (Scalar8); Mainly realted to eigenvector calcuation. Option4; 1x, no RRC message issue; Similar performance as that of FP32 (PC9); Mainly realted to eigenvector calcuation, exsits low complexiy calcuation method. Proposal-2: Support eType II like codebook with new parameter values as the Target CSI format. |
| Google | Proposal 1: For network side data collection, with regard to UE’s memory constraints and computing complexity, option 2 and 3 should be prioritized. (O1, O2)=(8, 8) can be considered for Option 4 if supported |
| HONOR | Observation 1: Option 4 is based on the R16 codebook, resulting in additional UE capabilities, higher implementation complexity and larger specification impacts. Observation 2: Compared to benchmark#1, amplitude/phase quantization with k1= k2=3bits shows moderate-to-severe degradation (-5.62% to -9.23%). Observation 3: For real/imaginary part quantization, per-sample normalization outperforms whole-dataset normalization. Observation 4: For real/imaginary part quantization, independent quantization of real and imaginary components results in negligible differences compared to joint quantization. Proposal 1: For Target CSI format Option 1, support 2-bit quantization across both real and imaginary parts with per-sample normalization. Proposal 2: To ensure consistency between the training and inference, normalization needs to be specified. |
| Huawei, HiSilicon | Observation 1: Regarding Option 1 of Target CSI format, following observations can be made from our simulation results: - k1+k2 should be at least 8 for an acceptable SGCS performance. - Amplitude+phase quantization provides a higher SGCS performance than real+imaginary quantization. - Selecting equal k1 and k2 results in a higher SGCS performance than unequal k1 and k2. - Intra-layer and intra-subband normalization provide a higher SGCS performance than inter-layer and inter-subband normalization, respectively. Proposal 1: For the NW side data collection, for the quantization of Target CSI with Option 1, consider the following: - Consider normalization per layer (for precoding matrix) and per subband to ensure the quantization performance. - The Target CSI is quantized to amplitude and phase. - For precoding matrix, for each individual layer and subband, each vector of 1*2N1N2 is uniformly quantized to the range of [0, 1] for amplitude, and uniformly quantized to the range of (−π, π] for phase. - Regarding k1 and k2 values, consider k1+k2≥8. Proposal 2: For the NW side data collection, for the quantization of Target CSI using Option 1, consider Alt.3 where one Target CSI sample is split into multiple segments, each of which corresponding to a subset of layers (for precoding matrix) and/or subbands and is carried by a separate RRC message. Proposal 3: Consider to select one combination from the following new parameters for Rel-16 eType II CB-like method as a reference: - L= 12; pv = {0.75, 1}; beta = {0.5, 0.8}; reference amplitude = 6 bits; differential amplitude = 4bits; phase = 6 bits. Proposal 4: For the NW side data collection with higher layer reporting, - Target CSI (for precoding matrix) is generated following the number of layers configured by NW. UE additionally reports data quality related information accompanied with each Target CSI sample, such as RSRP and CQI. |
| KT Corporation | - While Option 4 is typically regarded as overhead-efficient, configuring it with L=N1N2, pυ=1, β=1, and 4-bit differential amplitude with 6-bit reference amplitude and phase quantization to minimize quantization error for higher accuracy results in higher bit usage than Option 1 with k1+k2<10. - Option 1 with k1+k2=8 bits attains comparable accuracy to Option 4 with L=N1N2, pυ=1, β=1, and 4-bit differential amplitude with 6-bit reference amplitude and phase quantization while requiring only about 80% of its overhead. - Option 4 is computationally more complex than Option 1, as it includes an additional wideband beam selection step. - If channel matrix is to be supported as a target CSI type, the enhanced Type II codebook may face challenges, since inter-column power relationships must be preserved and the larger magnitude variation of channel matrix entries can increase quantization error. - Reliance on enhanced Type II codebook for NW-side data collection may limit the practical adoption of AI/ML-based CSI compression due to its limited commercialization rate. Support Option 1 with k1+k2=8 bits, as it achieves high accuracy while requiring lower overhead and a reduced computational complexity to reach a comparable level of accuracy. |
| Lenovo | - Option 1 with Scalar 8-bit quantization level for real (4 bits) and imaginary (4 bits) part of each values has small performance loss with 90% overhead reduction, and Scalar 4-bit quantization level for real (2 bits) and imaginary (2 bits) degrades performance significantly. - Option 4 with the new enhanced parameter combinations and similar payload size as Option 1 has better Matric#2 SGCS performance. Proposal 1: Support Option 4 as the target CSI format for NW-side data collection with high layer reporting and further down selections on the new parameters combinations. - Although it is desirable to have larger CSI dataset for training, due to the overhead associated with data collection, it is important to determine the proper size of the CSI dataset which provides enough information for training without imposing large overhead on the network. - It is desirable to include target CSI samples with more informative content to reduce the dataset size, based on the statistics of the input, e.g., lower informative content of some CSI samples or high correlation between one CSI sample (a group of samples) and another CSI sample (group of samples). Proposal 2: Support specification of procedures/signaling enabling transmission of subset of CSI samples with more informative content among the set of measured/collected CSI samples. - As the information content of CSI samples depends on the experienced distortion level, the UE may use distortion level or a quality indicator to determine which samples should be transmitted to the NW or at least send this information along the sample itself. The NW may be able to use this information further during the training of the model. - Support specification of procedures/signaling enabling transmission of subset of CSI samples based on the experienced distortion level or a quality indictor. - Transmission of additional information such as the level of distortion when measuring a sample, quality indicator, or how frequently a particular representative sample occurs, can help the NW side to better train the model. - Support specification of procedures/signaling for transmission of additional information such as sample-group size, quality indicator, distortion level along the transmission of the sample itself. Observation 7: Knowledge of the conditions/additional conditions under which the samples of the training data have been collected is needed can be used to train a Specialized model. Proposal 5: Support specification of procedures/signaling enabling UE to report UE-side additional conditions and also the NW conditions/additional conditions under which the data/samples have been collected. - he applicability of a model can be determined based on the combined additional information of all dataset(s) used for training with associated IDs, which represents the conditions of UE and NW to measure and collect the samples. - Support associating additional information with the dataset/model parameters generated by the NW-side and indicating that to the UE-side along the exchange of the dataset/model parameters. Study the necessary of the ID associated (i.e., associated ID) with the conditions to generate and tune the dataset/model parameters. |
| LGE | Proposal #5: Regarding the target CSI format for NW-side data collection, - Support legacy eType-II codebook (option 0) as a baseline. - Support scalar quantization (option 1)  If one Target CSI sample exceeds 9KBytes, multiple RRC messages can be considered. Proposal #6: Regarding the normalization scope of Target CSI format Option 1, consider normalization within one sample (2N1N2*N3*v) for each real/imaginary or amplitude/phase part. Observation 1: For Option 1, k1=k2=Int4 quantization provides more than 87.5% overhead reduction while maintaining the model performance (Metric#1) compared to the FP32 baseline. When the quantization level is further reduced to k1=k2=Int3, the model performance drops significantly across all layers, showing the limitation of coarse quantization. |
| MediaTek | Proposal 2. Support option 0 for NW-side data collection regardless of whether higher resolution for data collection is needed or not. Observation 1: For Alt 1, real/imaginary quantization with single [Tmin,Tmax] across all subbands, with S=1, (k1,k2)=(3,3) and (4,4) offers better SGCS accuracy and compression trade off among all candidate values. Compared to FP32, (3,3) results in 0.966 SGCS and 90% compression ratio, and (4,4) results in 0.992 SGCS and 87% compression ratio. For Alt 2, real/imaginary quantization with per-subband [Tmin,Tmax] and with S=1, k1, k2=3,3 and (4,4) offer better SGCS accuracy and compression trade off among all candidate values. Compared to FP32, (3,3) results in 0.978 SGCS and 84% compression ratio, and (4,4) results in 0.995 SGCS and 81% compression ratio. For real/imaginary quantization, per-subband [Tmin,Tmax] calculation (Alt 2) improves SGCS by only 0.01 while reduced the compression ration by 5.7% compared to per-sample(s) [Tmin,Tmax] calculation (Alt 1). Observation 4: For phase and amplitude quantization (k1,k2)=(3,5) achieves more than 0.991 SGCS and all combinations of (k,k2) with k1+k2>8 result in slight or no improvement on SGCS while introducing significant degradation in compression ratio. In general (3,5) and (2,5) offers better SGCS-compression tradeoff than other evaluated candidate values. Observation 5: For phase and amplitude quantization, assumption of the fixed range for phase has negligible degradation of SGCS while improves the compression ratio compared to assumption of dynamic phase range. Observation 6: For phase and amplitude quantization, per-subband calculation of [Tmin,Tmax] for phase and amplitude offers 0.007 SGCS improvement and 5.76% degradation on compression ratio compared to per-sample(s) calculation of [Tmin,Tmax] for amplitude and phase. Also, by assuming fixed range for phase, the improvement on SGCS is 0.002 while degradation in compression ratio is 2.88%. Observation 7: For phase/amplitude quantization and real/image quantization, calculation of [Tmin,Tmax] over all S samples does not have a significant impact on SGCS while offer more compression ratio due to a smaller number of [Tmin,Tmax] values. - For reporting [Tmin,Tmax] values in amplitude/phase and real/imaginary quantizations, FP16 has no or negligible impact on SGCS while provide additional compression compared to FP32. Proposal 3. For option 1, deprioritize (k1, k2) values with k1+k2>8 Proposal 4. For option 1 with amplitude/phase quantization, consider calculation of a single [Tmin,Tmax] for amplitude across all samples, ports, and subbands, and assume a fixed range for phase. Proposal 5. For option 1 with real/imaginary quantization, consider calculation of a single [Tmin,Tmax] for each real and imaginary parts over all samples, ports, and subbands. Proposal 6. For option 1, consider FP16 as reporting format for Tmin and Tmax values. For amplitude/phase quantization, prioritize (k1, k2) = (2,4) and (3,5). For real/imaginary quantization, prioritize (k1, k2) = (3,3) and (4,4). Proposal 7. Support only one of real/imaginary quantization and amplitude/phase quantization for option 1. Proposal 8. If higher resolution is required for NW-side data collection, prioritize option 1. Proposal 9. It is up to UE that how many CSI samples can be accumulated and reported to NW for training purpose at a time. Proposal 10. Discuss the assistant information that UE needs to include in reporting CSI samples to NW for training purpose. |
| NEC | For target CSI format, in addition to RRC message size, UE buffer size needs to be considered. Supporting Option 1 (scalar quantization) may cause frequent UAI transmission to notify NW that UE buffer is full. For target CSI format, between Option 1 (scalar quantization) and Option 4 (higher resolution eType II like codebook), support Option 4. |
| Nokia | Observation 3: Elementwise uniform SQ applied to Magnitude-Phase representation of a complex element shows better SGCS performance than elementwise uniform SQ being applied to Real-Imaginary representation in general. Observation 4: When elementwise uniform SQ is applied to Magnitude-Phase representation, increasing the number of bits for phase for a given fixed number of bits per magnitude (fixed k1) decreases the quantization distortion with diminishing gains. Observation 5: When elementwise uniform SQ is applied to Magnitude-Phase representation, for a given total number of bits: - When the total number of bits is an even number, k1 = k2 provides the best SGCS performance. - When the total number of bits is an odd number, k1 = floor(k_total/2) and k2 = ceil(k_total/2) or k2 = (k_total – k1) provides the best performance. The extra bit is allocated to the phase part in this case. Proposal 5: If Option 1 is selected for NW side data collection, agree on quantizing the amplitude and phase parts of each complex element of target CSI sample(s). Proposal 6: For quantizing the amplitude and phase parts in Option 1 of NW-sided data collection, for the given total number of allocation bits, allocate total number of allocation bits/2 bits to the magnitude part and the remaining bits to the phase part. Observation 6: For NW side data collection with higher layer reporting, concerning Option 1 (scalar quantization), input range adjustment of SQ can improve quantization performance. Proposal 7: Considering Option 1 (scalar quantization) for NW side data collection with higher layer reporting, the following input range configuration as a starting point for normalization scheme. - For real-imaginary representation: x1, x2∈ [-T1max, +T1max], where 0 < T1max ≤ 1. - For amplitude-phase representation: x1∈ [0, +T1amp,max], x2∈ [-π, + π], where 0 < T1amp,max ≤ 1. - FFS: How to select/define T1max and/or T1amp,max. Observation 7: For fixed ρv, β, reference amplitude bits (nrab), differential amplitude bits (ndab), and phase bits (npb), increasing the number of beams, i.e., L, results in a better SGCS performance but at a cost of significantly increased feedback overhead. Observation 8: For fixed L, β, reference amplitude bits (nrab), differential amplitude bits (ndab), and phase bits (npb), increasing ρv improves the SGCS performance along with increasing feedback overhead. Observation 9: For fixed L, ρv, reference amplitude bits (nrab), differential amplitude bits (ndab), and phase bits (npb), increasing β will result in a limited improvement in the SGCS performance while the feedback overhead increases significantly. Observation 10: For fixed L, ρv, β, reference amplitude bits (nrab), and differential amplitude bits (ndab), increasing the number of phase bits (npb) provides a minor improvement in SGCS performance but with increased feedback overhead. Observation 11: For fixed L, ρv, β and phase bits (npb), increasing the number of amplitude bits (reference amplitude bits (nrab) and differential amplitude bits (ndab)) provides a minor improvement in SGCS performance but with an significant or insignificant increase in feedback overhead depending on other parameters. Observation 12: For moderate values of target SGCS, R16 eType II codebook with new parameters (Option 4) requires less feedback overhead than SQ (Option 1). Observation 13: Only SQ (Option 1) can achieve a high SGCS value (≥0.95) whereas the SGCS performance of eType II codebook saturates along with significantly increased feedback overhead. Proposal 8: Consider target SGCS values to be observed during practical scenarios to decide between Option 1 and Option 4 for NW side data collection. |
| DOCOMO | Proposal 4 - For NW side data collection, - Support a data quality indicator report for the NW side data collection. Observation 2 - For the scalar quantization of real and imaginary parts, the payload size for one sample is from 3.3 kB to 9.5 kB, and it is less than 9kB for most of the configurations. - Option 4 has lower complexity without the beam combination searching if L = N1 × N2. - The beta parameter can be used to adjust the payload size flexibly. - The New PC #2 omits only 20% least significant coefficients (pv = 1 and beta = 0.8) and results in 8.8 kB payload size. Proposal 7 - The packaging of Target CSI into high-layer sigallings is up to RAN2. If RAN2 requires less than 9 kB for each Target CSI, adopt Option 4 with L = N1 × N2, pv = 1, and beta = 0.8. |
| OPPO | Proposal 1: Support Option 1 with linear normalization and uniform scalar quantization on real and imaginary part as baseline solution for NW-side data collection. Proposal 2: Regarding the determination of linear normalization parameters xmin and xmax, consider: - Alt 1: specified values. - Alt 2: NW configured and/or reconfigured. - Alt 3: UE reported: - Alt 3-1: per sample normalization (S = 1) - Alt 3-2: per reporting normalization (S>1) Proposal 3: For NW-side data collection from serving cell, pairing ID is not needed. Proposal 4: For NW-side data collection not from serving cell, whether pairing ID assigned from previous cells is valid for serving cell requires further study. |
| Panasonic | Proposal 4: Paring ID(s) can be optionally configured in CSI-RerpotConfig for NW-side data collection. Proposal 5: For NW-side data collection, the necessity and feasibility of UE reporting UE-side additional conditions to network should be studied. Instead of informing actual configuration, UE-side associated ID is necessary. |
| Qualcomm | Proposal 1: For NW side data collection, the data format discussion should consider performance and UE complexity / power consumption as high priority. The incurred over-the-air overhead is not a critical issue. Observation 1: For data format used for training data collection, element-wise quantization and spatial compression only achieve similar performance with L>=6. Observation 2: A target CSI sample with element-wise quantization of 7-bit per complex number can be transmitted within one RRC message. Observation 3: Option 4 has lower SVD dimension and lower number of quantization operations than option 1, but the complexity paid in option 1 guarantees the robustness of ground-truth in real world Observation 4: Option 4 has compression operations while option 1 does not. These operations bring zero benefit considering that lowering quantization resolution is sufficient to carry a target CSI sample of option 1 within a RRC message. - Adopt one from the following data format for NW side training data collection - Option 1: Element-wise quantization of target CSI directly (e.g., A-bit for amplitude + B-bit for phase, {A,B}={3,4}) - Option 4: Target CSI with spatial compression only, eT2 format with β=1 and p=1. - TypeII-like, W=W1*W2 with L=6 beams and element-wise quantization of W2 in subband-by-subband manner (A-bit for amplitude + B-bit for phase, {A,B}={3,4}). - eT2 enhancement is a generic problem for CSI enhancement, and is orthogonal to the discussion on data collection or monitoring. Observation 6: 5G data collection and performance monitoring framework are informative examples of 6G study. - For data collection and monitoring of Rel-20 CSI feedback with two-sided model, consider a more generic data format with low complexity rather than using a specific format used for 5G CSI feedback. - Do not support eT2 or eT2 enhancement as data format for NW side monitoring or data collection. |
| Samsung | Proposal#1: For Target CSI in NW-side data collection, for the overhead evaluation of the options for Target CSI, consider parameter values that would fit at least one Target CSI sample of a baseline configuration into 9kB RRC message. - Consider 2N1N2=128, Nsb=19, v=4 as a baseline configuration of a Target CSI sample. Observation#1: For Target CSI in NW-side data collection, for Option#3 and Option#4 below, Option#3 provides a slightly better accuracy than Option#4 while both options provide more than 90% overhead reduction from benchmark (Float32). - Option#3: principal component analysis (PCA)-based enhanced method for overhead saving by reporting the principal components (left and right eigenvectors and eigenvalues) of the spatial-frequency precoding matrix. - Option#4: eType II with new parameter combination, L, ρv, β, with Rel-16 amplitude and phase quantization. Observation#2: For two-sided model-based CSI compression, for Target CSI in NW-side data collection, - Option#0: eType II PC=8 reduces data samples overhead by 98.7% at the expense of considerable performance degradation as compared to a model trained by dataset quantized by Float32. - Option#1: SQ with amp-phase quantization (k1,k2)=(4,3) reduces data samples overhead by 89.0% with similar performance as compared to a model trained by dataset quantized by Float32. - Option#3: Principal component analysis (PCA)-based enhanced method for overhead by reporting the principal component of spatial-frequency precoding matrix, reduces data samples overhead by 94.53% at the expense of marginal performance degradation (0.38%) as compared to a model trained by dataset quantized by Float32. - Option#4: eType II with new parameter combination, ρv=1/2, β=3/4, reduces data samples overhead by 96.1% at the expense of marginal performance degradation as compared to a model trained by dataset quantized by Float32. Note: above evaluation is on 2N1N2=32, Nsb=13, v=1 dataset captured by RAN4. Proposal#2: For two-sided model-based CSI compression, for Target CSI in NW-side data collection, deprioritize, Option 0: Reusing legacy e-Type II codebook. Observation#3: For Target CSI in NW-side data collection, - Option 1, Option 3 and Option 4 require similar number of matrix operations, e.g., SVD, large matrices multiplications - Option 4 requires search operations which are already supported in legacy eType II CSI. Note: Option#3 is principal component analysis (PCA)-based approach by scalar quantizing the d<p,Nsb principal components of per-layer (Nsb×P) precoding matrices. Proposal#3: Clarify Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix as - Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of P=2N1N2 ports, N3 subbands, v layers for precoding matrix, and amplitude + phase parts, which has overall (P+N3)*d*v complex elements, and each complex element has an amplitude part of x1 and phase part of x2. The number of dominant components is d=2, 3, 4. - Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. Consider the candidate values k1=4, k2=3,4. - Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values. Proposal#5: For NW-side data collection, consider the following additional information reporting - Rank information Data quality information, e.g., explicit quantities including eigenvalues, SINR, etc., or implicit quantities such as SGCS, CQI. |
| Sharp | Proposal 1: For CSI compression, for NW side data collection with higher layer reporting, down-selection to one of option 1 and option 4. Proposal 2: For CSI compression, for NW side data collection for training, support that NW can indicate rank information for target CSI reporting. |
| Southeast University | Proposal 1-1: When x1/x2 represent the real and imaginary parts of target CSI samples, set k1=k2 and T1,min=T2,min, T1,max=T2,max as a baseline. When x1/x2 represent the amplitude and phase parts, the bit allocation and range design require further study due to their asymmetric statistical properties and encoding mechanisms. Proposal 1-2: If a down-selection is to be made between using real–imaginary and amplitude–phase inputs for the two-side CSI feedback model, it should be further studied whether and how the chosen model-input modality affects the content and quantization format of NW side data collection. Proposal 1-3: Further study magnitude-aware adaptive phase quantization, where phase precision is coarsened for low-magnitude coefficients and refined for high-magnitude coefficients, to achieve better payload-efficiency without notable performance loss. Proposal 1-4: Study how to reduce data collection signaling overhead for training two-side CSI compression model when S>1. Proposal 1-5: Study the dataset selection and dataset distillation-based method to further reduce the data collection signaling overhead. |
| Spreadtrum, UNISOC | Proposal 1：Whether to configure rank/layer can be discussed after the model training method is determined. |
| TCL | Proposal 1: For scalar quantization of target CSI, it is recommended to quantize the amplitude value and phase value. Proposal 2: The amplitude can be normalized for each dimension value and the dimension(s) within which the amplitude is normalized can be reported by UE for NW side data collection. Proposal 3: The quality information such as CSI-SINR and CQI can be provided along with target CSI samples or the quality threshold can be configured by NW for reliability of target CSI samples. |
| TOYOTA ITC | Observation 2: If Option 4 is adopted for the format of the Target CSI for NW sided AI/ML model training and new values for the eType II CSI codebook parameters are adopted, certain UEs might not be able to meet the granularity/accuracy requirements in the CSI measurement. Proposal 2: If Option 4 is adopted for the format of the Target CSI for NW sided AI/ML model training and new values for the eType II CSI codebook parameters are adopted, UEs to exchange their capabities in terms of granularity/accuracy in terms of - Amount of beams that can be separated - Amplitude granularity/accuracy - Phase granularity/accuracy for various channel quality levels. Proposal 3: With regards to the format of Target CSI to be used for NW sided AI/ML model training and for Option 1, scalar quantization should happen for the real and imaginary values. Proposal 4: With regards to Option 1 for the format of Target CSI to be used for NW sided AI/ML model training, normalization of the quantized values of real and imaginary parts is done to the range [-1, 1]. |
| vivo | Observation 1: There is a minimal to small performance gain with real+imaginary quantization compared to amplitude+phase quantization • Relative to Benchmark#1(FP 32), the performance loss of option 1 quantization with [Int4+int4 for real+imaginary quantization] are less than 0.1%. • Relative to Benchmark#1(FP 32), the performance loss of option 1 quantization with [Int4+int 4 for amplitude+phase]are less than 0.2%. Observation 2: There is a minimal performance loss [0.02%] with per real /imaginary quantization compared to across both real and imaginary quantization for rank 1, and there is a small performance gain [0.2%] with per real /imaginary quantization compared to across both real and imaginary quantization for rank 2 Observation 3: There is a small performance gain [0.2%] with per-layer quantization compared to across-layer quantization for rank 2 Observation 4: There is a minimal to small performance loss [0.2%] when de-normalization processing is not applied, compared with when normalization processing is used for performance metric 1 calculation. Observation 5: There is no loss when using a layer-common model versus a layer-specific model Observation 6: As L increases from L=4 to L=16, the complexity (i.e., SVD complexity) grows from O(43) to O(163) Observation 7: With the same L and [pv, Beta]=[1,1], the complexity decreases from O(L3)*N3+O(16*Tx port)+O(N3) to O(L3)*N3+O(16*Tx port) Observation 8: There is a large performance loss [~10%] between option 4 etype II target CSI format and Benchmark#1(FP 32) Proposal 4: Support option 1 for quantizing Target CSI • Quantization can be performed for real+imaginary part, and - k1=k2=4 - [T1min, T1max] and [T2min, T2max] are [-1 to 1] for uniform quantization • Quantization can be per layer per real/imag quantization, or can be per layer across real/imag quantization |
| Xiaomi | Proposal 5: Option 1 (scalar quantization) is adopted for target CSI reporting for NW side data collection. |
| ZTE, Sanechips | Observation 1: If the target CSI type is precoding matrix in spatial-frequency domain, the precoding vector of each layer has unit norm and real/imaginary parts of all entries are within the range from -1 to 1. Observation 2: Compared with amplitude+phase part quantization, real+imaginary part quantization can help reduce the UE complexity as it eliminates the need for further conversion of the complex element. Observation 3: 8 bits scalar quantization applied to the real+imaginary parts without normalization achieves 75% overhead reduction and almost no performance loss compared with FP32. Observation 4: The payload size of each target CSI sample with 8 bits scalar quantization may be up to ~37.5K bits, which is unaffordable by the current higher layer signaling in MDT framework with 9K Bytes (i.e., 72K bits) limitations. Proposal 7: For NW side data collection with higher layer reporting, for the target CSI format, further justify the performance-overhead tradaoff for high-resolution scalar quantization. Observation 5: In comparison with FP32 (i.e., Benchmark#1), eType-II PC8 (i.e., Benchmark#2), eType-II PC9, and eType-II PC10 demonstrate per-sample overhead reductions of 98.8%, 97.5%, and 96.2%, respectively. Observation 6: For large CSI payload Z ≥ 230 bits per layer, R16 eType II codebook with legacy parameters achieves 98.8% overhead reduction while with layer 1 performance loss of -5.5% for performance Metric#1 and -12.1% for performance Metric#2 compared to FP32. Observation 7: For large CSI payload Z ≥ 230 bits per layer, R16 eType II CB with new parameters achieves 96.2%~97.5% overhead reduction while with layer 1 performance loss of -0.4%~-1.5% and layer 2 performance loss of -0.7%~-2.3% for performance Metric#1 compared to FP32. Observation 8: For large CSI payload Z ≥ 230 bits per layer, R16 eType II CB with new parameters have 2.1~3.2 times overhead while with layer 1 performance gain of 4.3%~5.4% and layer 2 performance gain of 6.4%~8.2% for performance Metric#1 compared to R16 eType II codebook with legacy parameters. Proposal 8: Compared with R16 eType II codebook with legacy parameters, higher resolution codebook based quantization (i.e., R16 eType II codebook with new parameter) for target CSI reporting can achieve better tradeoff between model inference performance and reporting overhead. Proposal 9: For NW side data collection with high layer reporting, consider R16 eType II codebook with new parameters as the target CSI format. Proposal 10: For NW side data collection of CSI compression, the content of the collected data can include pairing ID which is beneficial for both NW side and UE side. Proposal 11: To enable high-quality data collection from the UE to the NW, further study the following options:  The UE reports data quality-related information to the NW, e.g., SINR, CQI, positioning information  The NW configures a data quality threshold to the UE and the UE only reports qualified data |

### 2.1-3: UE side data collection

| Company | View |
| --- | --- |
| Apple | Proposal 4: For UE side data collection, when a UE initiates a data collection procedure for training purposes, it may also recommend a CSI-RS-related configuration such as CSI-RS density and association ID, to optimize the quality of the collected data. |
| CATT | Proposal 6: For AI/ML based CSI compression Case 0, optionally UE can request the specific configuration for UE-side data collection. - Multiple candidate configurations for UE-side data collection can be configured by NW - UE can indicate preferred configuration from candidate configurations for UE-side data collection |
| CMCC | Proposal 4: In CSI compression using two-sided model use case, to enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report - Pairing ID can be optionally indicated in CSI-ReportConfig Proposal 5: In CSI compression using two-sided model use case, the CPU occupancy duration of UE side data collection can reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured. |
| Ericsson | - Pairing ID can be optionally configured for UE-side data collection, if needed. |
| ETRI | Proposal 3: For AI/ML-based CSI compression, consider the use of the pairing ID as assistant information for UE-side data collection. |
| FUTUREWEI | Proposal 1: In AI/ML-based CSI compression using two-sided model Case 0, to enable UE side data collection, consider including Paring ID as additional information in the CSI-ReportConfig for data collection purposes without CSI report. Proposal 2: In AI/ML-based CSI compression using two-sided model Case 0, for UE side data collection, consider setting reportQuantity to “none-CSI-r20” to reflect that no CSI report is needed and it is for Rel-20 CSI compression via two-sided model. |
| HONOR | Proposal 3: For UE-side data collection, the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion. |
| Huawei, HiSilicon | Proposal 5: Pairing ID(s) can be optionally indicated in CSI-ReportConfig for UE side training data collection. |
| InterDigital | Observation 8: Target CSI quantization Option 2 and Option 3 may reduce the signalling overhead at a cost of potential performance degradation. Proposal 8: Support Option 1 and Option 4 for quantizing the Target CSI for NW-side data collection. |
| Lenovo | - To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose with Pairing ID indicated. |
| MediaTek | Proposal 1. For UE-side data collection, CSI-ReportConfig is used for configuring the resources for data collection purpose without CSI report; the corresponding indication is that reportQuantity is set to ‘none-r20-CSI’. |
| NEC | Proposal 2: In Direction A sub option 4-1, the UE side can determine similarity information between the newly collected data/dataset and the exchanged (training) dataset and provide it to the NW side, to assist the NW side to determine whether the newly collected data is needed to avoid unnecessary dataset exchange. |
| Nokia | Proposal 9: For CPU counting of UE side data collection of CSI compression, the same CPU occupancy duration as Rel-19 beam prediction and CSI prediction data collection (with the changes suggested in R1-2508948) shall be applied. |
| OPPO | Proposal 5: Support P/SP-CSI-RS for UE-side data collection, and reuse the same CPU occupancy duration to CSI prediction agenda. Proposal 6: For UE-side data collection, pairing ID is required and can be configured in CSI-ReportConfig. |
| Panasonic | Proposal 1: For UE-side training data collection, in order to identify the scenario / configuration, configuration of ID, which is same ID as paring ID, is necessary at least for the case that UE-side data collection is based on OTA approach. - For the case that UE-side data collection is non-OTA approach, it would not be necessarily required to use the same ID as paring ID. Proposal 2: Paring ID(s) can be optionally configured in CSI-RerpotConfig for UE-side data collection. Proposal 3: For UE-side data collection at least based on non-OTA approach, the necessity and feasibility of UE reporting UE-side additional conditions to network should be studied. Instead of informing actual configuration, UE-side associated ID is necessary. - For the case that UE-side data collection is OTA approach, UE-side additional condition can be added when OTA data is sent to UE-side server. |
| Qualcomm | Proposal 4: For UE side data collection for two-sided CSI compression use case, configure measurement CSI-RS using CSI report config without a report. The CSI report configuration should include a paring ID. |
| Samsung | Proposal#7: For CPU counting of UE side data collection, - the occupancy starts from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement. |
| Sharp | Proposal 3: For UE side data collection, CPU occupancy duration can reuse the legacy CPU occupancy rule for CSI reports configured to 'none', 'none-bm-r19', or 'none-csi-r19'. Proposal 4: For data collection for UE side model, reportQuantity is set to a dedicate value to notify UE of the data collection for CSI compression. |
| Spreadtrum, UNISOC | Proposal 2: For CSI compression, the CPU occupancy duration can reuse the rule of R19 CSI prediction, but expand Z3’. Proposal 3: The pairing ID can be used for UE to collect UE-side target CSI for model training and fine-tuning. |
| TCL | Observation 1: The UE side data collection for training can be performed in both an opportunistic and proactive manner. |
| vivo | Proposal 2: The pairing ID is indicated in CSI-ReportConfig for UE-sided data collection. Proposal 5: Support to indicate the following information in CSI-ReportConfig used for UE side data collection • Pairing ID • The supported configuration(s) of pairing ID (e.g., multiple port configuration, multiple subband configuration, and multiple payload size configuration) |
| Xiaomi | Proposal 6: The paring ID is configured in CSI-ReportConfig for UE side data collection. Proposal 7: UE side data collection based on UE request is supported for CSI compression, and the procedure of UE side data collection for beam management and CSI prediction is reused. |
| ZTE, Sanechips | Proposal 12: For UE-side training data collection, pairing ID can be configured to address data distribution mismatch issues regarding NW-side/UE-side additional conditions. FFS other necessary information. Proposal 13: For CPU counting of UE side data collection for AI/ML CSI compression, the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the last symbol of the P/SP-CSI-RS occasion. |

  - Discussions

- Issue#2-1 NW side data collection - Quantization format of Target CSI

Brief summary:

Supporters of quantization methods:

- Option 0 (legacy eT2 codebook): CMCC, MediaTek, LGE

  - Not support: Samsung,

- Option 1 (Scalar quantization): LGE, InterDigital, CATT, Huawei, HiSilicon, Honor, Qualcomm, vivo, Samsung, Nokia, Apple, OPPO, Xiaomi, NTT DOCOMO, MediaTek, KT, Sharp (down select between Opt 1 and Opt 4), [TCL], [Toyota]

  - Argument 1: option 1 guarantees the robustness of ground-truth in real world

  - Argument 2: Option 1 is more generic (not limited to CSI feedback use case) forward compatible towards 6G.

  - Counter-argument: For 128ports, v=4 layers, and 19 subbands, Option 1 cannot be carried within one RRC message, if  k1+k2≥8

- Option 2 (Type II like codebook with scalar quantized W2): Google

- Option 3 (reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix): Samsung, Google

- Option 4 (eType II like codebook with new parameter): InterDigital, NTT DOCOMO, Lenovo, Huawei, HiSilicon, [Qualcomm?], Ericsson, [Nokia], ZTE, CMCC, NEC, Fujitsu, Sharp (down select between Opt 1 and Opt 4)

  - Argument: option 4 can enable NW to collect more data samples in an RRC message

  - Counter-argument 1: As L increases from L=4 to L=16, the complexity (i.e., SVD complexity) grows from O(43) to O(163)

  - Counter-argument 2: relying on e-Type 2 codebook makes this feature limiting, e.g., not being able to generalize to scenarios (e.g., near field scenario) that the e-Type 2 codebook is not designed for

  - [Counter-argument 3]: substantial performance loss of approximately 10% vivo

  - Not support: Apple, vivo

Overhead issue:

- Alt.1 (lower quantization resolution): Qualcomm, [Ericsson]

  - Qualcomm, Nokia: k1=3, k2=4 can maintain the overhead for max configuration within 9KB.

  - Ericsson: k1=3, k2=3

- Alt.2 (Limite/reduce N3, v values per RRC message):

- Alt.3 (Report one Target CSI sample via multiple RRC messages): LGE, NTT DOCOMO, Huawei, HiSilicon

Option 1 specific issues

- Normalization scope:

  - Specified/aligned normalization scope: Honor, MediaTek, Huawei, HiSilicon, Nokia, OPPO, [Apple], LGE, TCL, Toyota

    - Huawei, HiSilicon, Samsung, CATT: Normalize precoding vector of 1*2N1N2 within per layer, within per subband.

    - OPPO, MediaTek: consider reporting Tmin and Tmax.

    - Apple, Ericsson: Scale the target CSI by Ntx and apply fixed quantization range

    - Qualcomm: the strongest coefficient of each eigen-vector is normalized to 1, and other coefficients are normalized accordingly (similar operation as the W2 quantization of eType II)

    - Nokia: only one value (0.60) needs to be defined as normalization configuration parameter across 2N1N2 ports/ N3 subbands / ν layers / real and imaginary (or amplitude)

    - OPPO: y=x-xminxmax-xmin. dequantization and inverse operation from y to x is performed at NW side. Therefore, to facilitate the inverse operation, xmin and xmax should be aligned between NW-side and UE-side.

    - TCL: the dimension(s) within which the amplitude is normalized can be reported by UE

    - Observations

      - Huawei, HiSilicon, vivo: small performance gain with intra-layer quantization compared to across-layer quantization

      - Huawei, HiSilicon, MediaTek: small performance gain with intra-subband quantization compared to across-subband quantization

        - MediaTek: Compression ratio of intra-subband quantization is larger than across-subband quantization (due to the additional overhead of per subband reported Tmin and Tmax)

      - Huawei, HiSilicon, vivo, CATT: Small performance gain when normalization processing is applied vs not applied

      - vivo, Honor: close performance with across real+imag quantization compared to individual real and imag quantization

      - Honor, LGE: per-sample normalization outperforms across-sample normalization

  - Not support specification: CATT, NTT DOCOMO

    - CATT (UE implementation): For any x1 (x2), if -1<x1p<1 (-1<x2p<1), the real (imaginary) part after normalization and truncation is x1p ( x2p). If x1p≥1 orx1p≤-1 x2p≥1 orx2p≤-1, the real (imaginary) part after normalization and truncation is 1.

- Quantize to amp+pha or real+imag:

  - amp+pha: Samsung, Huawei, HiSilicon, Qualcomm, Nokia, MediaTek (only select one between amp+pha and real+imag), TCL

    - Values of {k1, k2}:

      - Qualcomm, Nokia: k1=3, k2=4

      - MediaTek: k1=3, k2=5, or k1=2, k2=4

      - Nokia: For amp+pha, if k1+k2 is even number, k1=k2 is best; if k1+k2 is odd number, k2=k1+1 is best.

      - Huawei, HiSilicon: k1+k2>=8

        - MediaTek: deprioritize k1+k2>8

      - Huawei, HiSilicon: Support k1=k2 other than unequal k1 and k2

      - Samsung: k1=4, k2=3

    - Range of [T1min, T1max]:

      - [0, 1]: Honor, Huawei, HiSilicon, Qualcomm, Samsung

      - [0, +T1amp,max] where 0 < T1amp,max ≤ 1 Nokia

    - Range of [T2min, T2max]:

      - (−π, π]/(0, 2π): Honor, MediaTek, TCL, Huawei, HiSilicon, Qualcomm (16PSK), Nokia, Samsung (M-PSK), CATT

  - Real+imag: KT, vivo, Honor, Ericsson, [ZTE], OPPO, Apple, CATT, [Lenovo], MediaTek (only select one between amp+pha and real+imag), LGE, Toyota

    - Values of {k1, k2}:

      - k1=k2=2: Honor

      - k1=k2=3: MediaTek, Ericsson

      - k1=k2=4: LGE, MediaTek, vivo, CATT (with UE impelemented normalization and truncation), Lenovo, KT, Huawei, HiSilicon

        - Lenovo, [vivo], LGE: with even smaller k1, k2 values, performance degrades

      - k1=k2=5: Apple

      - k1=k2=6: CATT (without normalization)

      - k1=k2=8: ZTE

      - Equal k1 and k2: Southeast University, CATT

    - Range of [T1min, T1max]/[T2min, T2max]:

      - [-1, 1]: vivo, [ZTE], CATT, LGE, Toyota

      - [0, 1]: Honor

      - x1, x2∈ [-T1max, +T1max], where 0 < T1max ≤ 1 Nokia

      - [-c/Nports, c/Nports]: where c is a number that may vary depending on the scenario, deployment, etc. Ericsson

      - [-1.82, 1.82]: Apple (quantization range to cover 99% probability, e.g., [0.5%, 99.5%], it is roughly [-1.82, 1.82])

      - KT: [-0.14, 0.14] for N1N2=64, [-0.18, 0.18] for N1N2=48, [-0.20, 0.20] for N1N2=24,

      - Dynamical value to be aligned between NW and UE: OPPO, MediaTek

    - Other views

      - OPPO: xmin and xmax should be aligned between NW-side and UE-side

        - Alt1: specified values. Alt2: NW configured values. Alt3: UE reported values.

      - ZTE, CATT: If amplitude/phase quantization is applied, additional complexity will be introduced for converting the CSI samples into polar domain

  - Views on ranges of [T1min, T1max], [T2min, T2max]

    - Ericsson: Different quantization points/ranges may be different for different configurations, deployment scenarios, etc.

    - Nokia restricting the input range for SQ in accordance with the measured data statistics lead to significant quantization performance enhancement. FFS how to select/define T1max and/or T1amp,max

  - Observations:

    - Honor, vivo: real+imag performs [slightly] better than amp+pha

    - Nokia, Huawei, HiSilicon: amp+pha performs [slightly] better than real+imag

Option 2 specific issues:

- Candidate parameter values:

Option 3 specific issues:

- Samsung: Perform SVD to the eigen vector (with dimension of Tx port * Subband Num) constructed matrix of H=c1,1c1,2c2,1c2,2⋯c1,NSB⋯c2,NSB⋮⋮cNport,1cNport,2⋱⋮⋯cNport,NSB, deriving left singular vectors (Nport length) and right singular vectors (Nsubband length). Then, further perform scalar quantization (amplitude, phase) to each element of left singular vectors and right singular vectors, with total number of coefficients of d*( Nport + Nsubband), where d is selected singular value which achieves a compression of NportNSBd(Nport+NSB).

  - Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of P=2N1N2 ports, N3 subbands, v layers for precoding matrix, and amplitude + phase parts, which has overall (P+N3)*d*v complex elements, and each complex element has an amplitude part of x1 and phase part of x2. The number of dominant components is d=2, 3, 4.

  - Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. Consider the candidate values k1=4, k2=3,4.

  - Candidate parameter values: d=2,3,4; amplitude=4 bits; phase=4 bits

Option 4 specific issues:

- Candidate parameter values:

  - NTT DOCOMO: L=N1N2, pv=1, β=0.8.

  - Lenovo (32 ports): L=16, pv=1, β=1. Or, L=12, pv=1, β=0.8.

  - Fujitsu: L= {12, 16}; pv = 0.95; beta = {0.5, 0.6}; {Ref_amp, Diff_amp, Pha} = {4, 4, 4} bits

  - Huawei, HiSilicon: L= 12; pv = {0.75, 1}; beta = {0.5, 0.8}; {Ref_amp, Diff_amp, Pha} = {6, 4, 6} bits

  - Qualcomm: L= 6; pv = 1; beta = 1; {Ref_amp, Diff_amp, Pha} = {4, 3, 4} bits

  - Ericsson:

    - 32 ports: L= 12/10; pv = {0.6, 0.75, 1}; beta = {0.25, 0.5}; {Ref_amp, Diff_amp, Pha} = {4, 3, 4} bits

    - 128 ports: L=16/24/32; pv = {0.6, 0.75, 1}; beta = 1; {Ref_amp, Diff_amp, Pha} = {4, 3, 4} bits

  - Samsung: L= 10; pv = {0.5}; beta = {0.75}; {Ref_amp, Diff_amp, Pha} = {4, 3, 4} bits

  - KT (24/32/64 ports): L= N1N2; pv = 1; beta = 1; {Ref_amp, Diff_amp, Pha} = {6, 4, 6} bits

  - Google: Enhanced (O1, O2)=(8, 8)

  - Observations

    - Increasing L has impact to SGCS performance Huawei, HiSilicon, Nokia

      - Fujitsu: SGCS increase is not significant after L=12

    - Increasing pv has impact to SGCS performance Huawei, HiSilicon, Nokia

      - Fujitsu: SGCS of pv=0.85 and SGCS of pv=0.95 are almost the same

    - Increasing quantization bits of {Ref_amp, Diff_amp, Phase}

      - Diff_amp can be increased; Ref_amp and Phase can be reused Fujitsu

      - has significant impact to SGCS performance. Huawei, HiSilicon

      - has minor impact to SGCS performance Nokia

    - Increasing beta has minor impact to SGCS performance Huawei, HiSilicon, Nokia, Fujitsu

- Complexity analysis:

  - SVD/EVD decomposition dimension

    - Large dimension SVD decomposition complexity is the bottleneck

      - Huawei, HiSilicon: Key complexity issue for Option 4 is 2L dimension EVD decomposition.

      - Apple: the complexity to obtain Tx eigen vectors are much lower than the complexity of SVD operation on a Tx covariance matrix

    - SVD decomposition complexity of Option 4 is smaller/equal to Option 1

      - Fujitsu: For Option 4, the SVD/EVD processing can be done after W1 selection and thus the matrix inverse dimension in SVD/EVD is significantly reduced

      - Samsung: Option 1, Option 3 and Option 4 require similar number of matrix operations, e.g., SVD, large matrices multiplications

      - Qualcomm: Option 4 has lower SVD dimension and lower number of quantization operations than option 1

      - CATT: complexity of Opt1 and Opt4 is dominated by SVD or eigenvalue decomposition

  - Search/selection operation compared to Option 1 Qualcomm, Samsung, KT

    - Qualcomm: Option 4 incurs lots of logical operations such as basis and coefficients selection, and searching, which may not be able to be performed in parallel

    - KT: it includes an additional wideband beam selection step compared with Option 1

Moderator note: From supporters referring to “Brief summary”, there are comparable number of companies supporting Option 1 and Option 4.

- Option 1: 18 supporters

- Option 4: 12 supporters

For Option 4, different UE vendors report different implementations: Apple and Huawei/HiSi mention the major complexity lies in the large dimension SVD decomposition, while Qualcomm, Samsung mention the major complexity lies in the basis/coefficients selection. Considering different UEs may indeed have different implementations, some easy to support Option 4 while others challenging. Then, from NW perspective, if some UE vendors have problem on supporting Option 4, the reporting with Option 1 is also helpful and may not cause much additional efforts on NW management.

So, let’s see if we can support both options with UE capability.

### [Rd1] (Closed) Proposal 2.1 (Option selection):

For NW side data collection with higher layer reporting, regarding Target CSI quantization, support both Option 1 (Scalar quantization) and Option 4 (eType II like codebook with new parameter) based on UE capability.

| Company | View |
| --- | --- |
| Huawei, Hisilicon | In general, we are OK to support both SQ and VQ. To avoid later complications during UE feature discussions, it may be better to further clarify whether one quantization method (eg, SQ) is used as a basic UE capability (basic feature) and the other as an optional UE capability (eg, VQ) or UE capability is defined for both SQ and VQ. In the latter case, it needs to be clarified that a UE that supports AIML-based CSI compression, supports at least one of SQ or VQ. |
| ZTE | Thanks FL for the summary. According to the above observations, Option 4 shows much lower reporting overhead, comparable performance and complexity (at least for some implementations) compared with Option 1. Therefore, Option 4 can be supported subject to UE capability. However, for Option 1, the payload size of each target CSI sample is largely increased, which means only one or less than one sample can be reported each time. Consequently, the UE buffer will become full in a very short time and thus frequent NW request/UE response for data reporting are needed in MDT framework. This way of data collection may cause too much NW and UE efforts. We feel the feasibility of Option 1 needs to be further confirmed by RAN2. |
| OPPO | We are okay. |
| Xiaomi | A single option is preferred. Option 1 is future-proof and has a good trade-off between UE complexity and report accuracy. Maybe we can try whether some companies support Option 4 can also accept Option 1? |
| MediaTek | We also prefer a single option since several companies have proposed to use the same format for data collection and inter-vendor collaboration and we want to avoid any unnecessary complication. In our view, option 1 shows better SGCS performance which has been the motivation to seek new options other than legacy eType II. If the overhead was the major concern, option 0 was workable solution from the very beginning. |
| Ericsson | We think that a single option should be selected. We also agree with ZTE that the feasibility of Option 1 needs to be further confirmed by RAN2. |
| ETRI | We agree on having both options. |
| InterDigital | Support Option 1 and Option 4 for quantizing the Target CSI for NW-side data collection. |
| Panasonic | Deciding either Option 1 or Option 4 is desirable. |
| Fujitsu | We support selecting Option 4 for Target CSI considering its advantage in payload size. As a compromise, we can leave with this proporsal for progress. |
| LGE | We also prefer single option for Target CSI format. |
| KT | We prefer supporting a single option. Between accuracy and overhead, accuracy should be prioritized; therefore, option 1 should be supported. |
| NTT DOCOMO | We also prefer a single option. It is unnecessary to create two options for such feature. Given the condition that some UE vendors have concerns on the complexity of Option 4, Option 1 can be selected. |
| Lenovo | We are Okay with having both options. |

Moderator note: For Option 1 details referring to “Brief summary”:

Quantization to real/imag or amp/pha:

- amp/pha: 6 supporters

- real/imag: 12 supporters

Performance wise, they have similar performance as observed by Honor, vivo, Nokia, Huawei. Complexity wise, ZTE and CATT point out that, compared to real/imag, additional complexity will be introduced for converting the CSI samples into polar domain for amp/pha. To Moderator understanding, the two solutions are not distinguished from performance/complexity wise.

=>Propose to take real/imag which has more supporters.

Normalization/Quantization range (T1 and T2): Huawei, vivo, CATT results show that performing normalization is better than w/o normalization. This may be because the distributions of real/imag parts (as pointed out by Nokia) are not between [-1, 1]. Without normalization, some codepoints (e.g., indicating [-1, -0.4] and [0.4, 1]) will never be used.

In addition, as pointed out by KT and Ericsson, the distribution of real/imag parts may depend on scenario/configuration (KT: [-0.14, 0.14] for N1N2=64, [-0.18, 0.18] for N1N2=48, [-0.20, 0.20] for N1N2=24; Ericsson: [c/Nports, -c/Nports] with c=1.8 => [-0.33, 0.33] for the 32-port case and [-0.16, 0.16] for the 128-port case). Moreover, it is Moderator’s feeling that the original Target CSI is generated by UE proprietary implementation, and there is no mandate that the Target CSI vector must be inherently normalized, with real/imag parts distributed between -1 and 1 (or -0.33 to 0.33). Then, is it possible to specify a hard range of T1 and T2 unless the distribution of values is testable? Or, the UE can scale up/down to the specified range by implementation?

To fit the quantization to a dynamic range, TCL and MediaTek propose that UE additionally reports the quantization range Tmin and Tmax.

On the other hand, as raised by OPPO, if the normalization is performed for real/imag, xmin and xmax need to be additionally reported to gNB so that inverse operation is performed at NW to recover the original Target CSI; otherwise the direction of the Target CSI vector may be distorted.

=> FFS three alternatives on normalization: one with hard range of Tmin and Tmax without specifying normalization; a second with adjustable Tmin and Tmax without specifying normalization but with reported quantization range; a third with UE normalization and reporting xmin and xmax.

In addition, if normalization is performed, from the obervations of Huawei, vivo, MediaTek, Honor, and LGE, that the normalization is performed for each 1*2N1N2 vector individually per layer, per subband, and per sample, which is slightly better than across layers/subbands/samples.

Values of k1 and k2: Pointed out by Lenovo, vivo, LGE, Huawei, performance degrades a lot for k1+k2 smaller than 4+4, such as k1=k2=3. Therefore, the smallest value is 4+4. Larger values of 6, 8 are also provided to enable flexible granularity.

### [Rd1] (Closed) Proposal 2.2 (Option 1 details):

For NW side data collection with higher layer reporting, for Option 1 (scalar quantzation), consider the following:

- Each complex element of the Target CSI sample is quantized to real value and imaginary value

- Support k1=k2=4, 6, and 8, based on configuration

- FFS whether/how to perform normalization method, e.g., down select among the alternatives:

  - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1]. No need to specify normalization.

  - Alt.2: The ranges of [T1min, T1max] and [T2min, T2max] are reported by UE. No need to specify normalization.

  - Alt.3: Normalization is performed for each 1*2N1N2 vector individually per layer, per subband, per sample, and per real/imag part. UE additionally reports the original max and min values which based on which the normalization is performed.

Moderator note:

After reviewing companies’ comments, Moderator realized that amp+phase may be simpler than real+imag, since we only need to consider T1max value for amplitude; T1min for amplitude is naturally 0, and [T2min, T2max] is naturally (−π, π]. Regarding normalization, only need to determine/report T1max value (or, similar to eType II, reports the strongest coefficient index) – in contrast, for real/imag we need to decide whether to report both Tmin/Tmax or only one of max {|Tmin|, |Tmax|} as pointed out by vivo.

Anyway, two sub-options are listed. We can try if we can down select to one method at offline.

### [Rd2] (Closed) Proposal 2.2a (Option 1 details):

For NW side data collection with higher layer reporting, for Option 1 (scalar quantzation), down select between consider the following:

- Option 1-1: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Support k1=k2=3, 4, 6, and 8, based on configuration

    - Note: k1=k2=4, 6, and 8 are at least applicable to some certain configurations of Tx ports/subband number.

- Option 1-2: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Support {k1, k2} = {3,4}, {4,3}, {4,4}, and {8,8}, based on configuration

    - Note: {k1, k2} = {4,4}, and {8,8} are at least applicable to some certain configurations of Tx ports/subband number.

- FFS whether/how to perform normalization method, e.g., down select among the alternatives:

  - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, and [0, 1]/(−π, π] for amplitude/phase quantization. No need to specify normalization.

  - Alt.2: The ranges of T1min, T1max, T2min, and/or T2max [T1min, T1max] and [T2min, T2max] are reported by UE. No need to specify normalization.

  - Alt.3: Normalization is performed for each 1*2N1N2 vector individually per layer, per subband, per sample, and per real/imag part. UE additionally reports the original max and min values which based on which the normalization is performed.

| Company | View |
| --- | --- |
| Mod | It is Moderator’s feeling that there seems no strong need to specify normalization. UE can perform scale up/down to the Target CSI elements to fit the range of [Tmin, Tmax]. E.g., if the original distribution is [-0.2, 0.2], UE can multiply a scaling factor of 5 to all the elements to stretch the distribution to [-1, 1] and then perform quantization. But, considering how to generate Target CSI and whether/how to perform scaling is UE implementation, how can we ensure the UE will stretch the distribution to fit the fixed range? |
| QC | If amp/phase and real/imag quantization has similar performance, we should take amp/phase as same implementation in eType II can be reused. Normlization is applied at the NW side as implementation, whatever UE reports, Alt1/Alt2/Alt3, NW would do some preprocessing, such as normalizing the reported precoding vector of each layer on each subband to be unit. These operation are transparent to UE. Similarly, during inter-vendor collaboration, after receiving the dataset exchanged from NW, UE would do some pre-processing as well, such as normalizing the reported vector to be unit norm. Thus, Alt1 should be the only solution. The normalization and range applied / reported by UE in Alt2 and Alt3 do not have any real effect. |
| vivo | For amp/phase and real/imag quantization, the real and imaginary parts follow a Gaussian distribution, while the amplitude follows a Rayleigh distribution. Uniform quantization seems to be more suitable for the real and imaginary parts Based on our evaluation, we slightly prefer to Alt 3 with modification. Per-layer normalization is favored over per-subband normalization, as the performance gain is limited and the reporting overhead for max value per subband would incur higher overhead. Additionally, reporting only max(¦W¦) for normalization suffices, rather than reporting both max and min values, given that their ranges are similar in the evaluation distribution. |
| Samsung | We considered k1=4bits amplitude and k2=3bits phase quantization. The normalization is by setting the element to largest amplitude to 1, i.e., no need for further normalization. The network can re-normalize the reconstructed precoding vector to a unit norm. Thus, we made the following additional alternative. This has advantage as eType II already supports 4-bits amplitude quantization and 8-PSK. For NW side data collection with higher layer reporting, for Option 1 (scalar quantzation), consider the following: - Each complex element of the Target CSI sample is quantized to real value and imaginary value - Support k1=k2=4, 6, and 8, based on configuration - FFS whether/how to perform normalization method, e.g., down select among the alternatives: - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1]. No need to specify normalization. - Alt.2: The ranges of [T1min, T1max] and [T2min, T2max] are reported by UE. No need to specify normalization. - Alt.3: Normalization is performed for each 1*2N1N2 vector individually per layer, per subband, per sample, and per real/imag part. UE additionally reports the original max and min values which based on which the normalization is performed. - Alt4: Normalization is performed for each 1*2N1N2 vector individually per layer, per subband, per sample, and per real/imag part. The largest amplitude of elements of the vector is normalized to 1. Phase is quantized in the range [0, 2π). |
| Apple | From our simulation, we see that the real and imaginary part distribution of a normalized eigen vector will be the same if scaled by sqrt(N_tx). A quantization range of [-1.82, 1.82] covers centrol 99% probability. I guess you can achieve the same quantization accuracy by quantizing each number within [-1.82/sqrt(N_tx), 1.82/sqrt(N_tx)]. With regard to standardization the quantization, I guess minimum we need to specify that the value quantized is uniformly distributed within a symmetric range around zero? |
| Huawei, HiSilicon | Regarding the quantization of real/imaginary vs. amplitude/phase, based on our simulations for k1+k2 at least equal to 8, we observed better SGCS results with amplitude/phase quantization. Some relevant results from our t-doc R1-2508495 are brought below: Table 5 Option 1 SGCS results (Metric#1 and Metric#2) Metric#1; Metric#2. amplitude+phase; real+imaginary; amplitude+phase; real+imaginary. FP32; 0.918; 0.918; 1; 1. k1=8, k2=8; 0.91; 0.906; 0.9999; 0.9997. k1=4, k2=4; 0.902; 0.891; 0.981; 0.9294. k1=3, k2=5; 0.894; 0.876; 0.968; 0.847. k1=5, k2=3; 0.894; 0.872; 0.936; 0.843. k1=3, k2=3; 0.887; 0.861; 0.911; 0.742. Regarding the FFS, based on our simulation results, Alt.3 (with normalization) provides a better SGCS performance than Alt1 or Alt2 (without normalization). See below Table from our t-doc R1-2508495. However, we are open to further discussions on these alternatives. Table 1 Option 1 SGCS results (Metric#2, with normalization and w/o normalization) amplitude+phase; real+imaginary. with normalization (intra layer); w/o normalization; with normalization (intra layer); w/o normalization. FP32; 1; 1; 1; 1. k1=8, k2=8; 0.9999; 0.9998; 0.9997; 0.9987. k1=4, k2=4; 0.9813; 0.9615; 0.9309; 0.7345. k1=2, k2=2; 0.6571; 0.5317; 0.7011; 0.3005. |
| ZTE | Generally OK if the feasibility of Option 1 (especially for the relatively large k1/k2 values) can be confirmed by RAN2. Among the three alternatives for normalization, Alt1 and Alt3 ensure a common operation for all samples. While for Alt2, it can happen that the different T1/T2 values are reported for different samples or by different UEs, leading to additionally NW efforts for data post-processing such as dequantization. |
| CMCC | Since some evaluation results showed better SGCS with amplitude/phase quantization, we prefer amp/phase quantization similar as in eType II codebook. For simplicity and union normalization range across layers and samples, Alt1 with fixed range is supported. |
| Nokia | In Alt 3, is it correct understanding that the UE reports 4N1N2N3v normalization factors per sample? If so, this seems excessive, unless it’s included in the k1=k2 bits per real/imaginary value |
| OPPO | Generally okay. Regarding normalization, we prefer Alt 2 to align the normalization between NW-side and UE-side. No need to specify it. |
| TCL | We think quantization of amplitude/phase should be reserved for further discussion. In addtion, whether and how to perform normalization need to be separately discussed. So we made the following modification. For NW side data collection with higher layer reporting, for Option 1 (scalar quantzation), consider the following: - FFS whether each complex element of the Target CSI sample is quantized to real value and imaginary value or amplitude and phase value. - Support k1=k2=4, 6, and 8, based on configuration - FFS whether/how to perform normalization method, e.g., down select among the alternatives: - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1]. No need to specify normalization. - Alt.2: The ranges of [T1min, T1max] and [T2min, T2max] are reported by UE. No need to specify normalization. - Alt.3: Normalization is performed for each 1*2N1N2 vector individually per layer, per subband, per sample, and per real/imag part.and UE additionally reports the original max and min values which based on which the normalization is performed. - FFS how to perform normalization method, e.g., down select among the alternatives: - Alt.1: Normalization is performed for each 1*2N1N2 vector individually per layer, per subband, per sample, and per real/imag part. - Alt.2: Normalization is performed across layers/subbands/samples. |
| Xiaomi | We don’t have a strong view on real/imaginary or amplitude/phase due to it’s similar performance. Regarding the normalization, we tend to agree with FL that normalization can be up to UE implementation. Thus, Alt.1 is preferred. |
| MediaTek | We do not see substantial performance difference between amp/phase and real/imag quantization schemes. However, since we have used Alt. 2, amp/phase reduced number of reportings of min/max ranges by fixing the phase range. Regarding normalization, we prefer Alt 2. No need to specify the Tmin/Tmax values based on the synthetic data companies have used in their evaluation |
| Ericsson | Regarding the second bullet - Support k1=k2=4, 6, and 8, based on configuration k1=k2=4, 6, 8, will result in a target CSI sample size larger than the maximum payload size of a RRC message. Hence, we cannot support this bullet without feasibility input from RAN2. Regarding the quantization range and normanlization In our evalutions, the precoder is nomzlized to unit norm, the real and imaginary parts are quantized using uniform SQ in the range of [-c/Nports, -c/Nports], where we find c=1.83 a good option, which is very close to the value 1.82 considered by Apple. |
| CATT | Support. Regarding the normalization, we also think that Alt 1 is the good way to go. In our evaluations, the normalization can be UE-implementation specific and the quantization range can be [-1,1] for both real and imaginary part. UE does not need to exchange the normalization coefficient because the per-layer and per-subband PMI is a uni-norm vector, gNB can rescale after quantization. Many companies including us have provided simulation results showing that such mplementation-specific normalization can provide good performance. Therefore, Alt 1 should be adopted. |
| Fujitsu | Regarding normalization part, we think it is better not to specify the normalization process. Considerring the data disctribution in field may be quite different from synthetic data, how to do normalization could be up to UE’s implementation. |
| LGE | Regarding quantization contents, if real/imag and amp/phase has similar performance, real and imaginary quantization seems more suitable, since it is more simple way and has benfits for computational complexity compared to amp/phase quantization. From our evaluations, k1=k2=4 for real/imag quantization provides similar model performance (Metric #1) compared to FP32 and k1=k2=8 quantization. We didn’t find motivation to support k1=k2=6, 8 for real/imag quantization. Setting tight quantization range or peforming normalization can be beneficial to increase quantization performance. In our view, we prefer normliazation method compared to setting tight quantization range (Alt2), since there seems no need to report max/min values for normalization if normalization scope/range is aligned with NW and UE. |
| Lenovo | We are fine with the updated version. |

Moderator note: As said in Tue online, let’s first do the homework on the necessity of enhancing the RRC signaling – a narrowed set of k1, k2 values are selected to facilitate the cross check over companies. In addition, more study points are added, such as the quantization value of each codepoint, and UE complexity considering the conversion. Last, a note is added to give information of the Target CSI payload size.

### [Rd3] Proposal 2.2b (Option 1 details):

For for Option 1 (scalar quantzation) of NW side data collection with higher layer reporting, down select between Option 1a and Option 1b:

- Option 1a: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Further study at least k1=k2=3, 4, 6, and 8 to justify the performance and necessity of RRC signaling.

  - Further study the quantized value of each codepoint.

- Option 1b: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Further study at least k1+k2=6, 7, 8, and 16 with k1>k2, k1<k2 or k1=k2 to justify the performance and necessity of enhanced RRC signaling.

  - Further study the quantized value of each codepoint.

  - Further study the UE complexity compared with Option 1a, considering the potential conversion to amplitude/phase domain of Option 1b.

- Further study whether/how to perform normalization for Option 1a and Option 1b, e.g., down select among the alternatives:

  - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, and [0, 1]/(−π, π] for amplitude/phase quantization.

  - Alt.2: The ranges of T1min, T1max, T2min, and/or T2max are reported by UE.

  - Alt.3: Normalization is performed for each 1*2N1N2 vector.

- Note: Payload size of a single Target CSI for

  - k1+k2<=7 is smaller than 9Kbytes for all configurations of Tx ports/subband/layer number.

  - k1+k2=8 is smaller than 9Kbytes for all configurations of subband/layer number with 2N1N2<=64.

  - k1+k2=16 is smaller than 9Kbytes for all configurations of subband/layer number with 2N1N2<=32.

Moderator note: Regarding the overhead issue for Option 1, two alternatives from the last meeting are kept for discussions.

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| FP32 (k1+k2=64bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 3328 | 6656 | 13312 |
| 2N1N2=32, N3=19 | 4864 | 9728 | 19456 |
| 2N1N2=64, N3=19 | 9728 | 19456 | 38912 |
| 2N1N2=128, N3=19 | 19456 | 38912 | 77824 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| FP16 (k1+k2=32bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 1664 | 3328 | 6656 |
| 2N1N2=32, N3=19 | 2432 | 4864 | 9728 |
| 2N1N2=64, N3=19 | 4864 | 9728 | 19456 |
| 2N1N2=128, N3=19 | 9728 | 19456 | 38912 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| Scalar8 (k1+k2=16bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 832 | 1664 | 3328 |
| 2N1N2=32, N3=19 | 1216 | 2432 | 4864 |
| 2N1N2=64, N3=19 | 2432 | 4864 | 9728 |
| 2N1N2=128, N3=19 | 4864 | 9728 | 19456 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| Scalar4 (k1+k2=8bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 416 | 832 | 1664 |
| 2N1N2=32, N3=19 | 608 | 1216 | 2432 |
| 2N1N2=64, N3=19 | 1216 | 2432 | 4864 |
| 2N1N2=128, N3=19 | 2432 | 4864 | 9728 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| Scalar3 (k1+k2=6bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 312 | 624 | 1248 |
| 2N1N2=32, N3=19 | 456 | 912 | 1824 |
| 2N1N2=64, N3=19 | 912 | 1824 | 3648 |
| 2N1N2=128, N3=19 | 1824 | 3648 | 7296 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

### [Rd1] (Closed) Proposal 2.3 (Overhead for Option 1):

For NW side data collection with higher layer reporting, regarding Option 1 (scalar quantzation), consider the following solutions to limit the overhead of per RRC message within 9KBytes (max payload size per RRC message) for up to 128 ports

- Alt.1: Using lower quantization resolution per floating element, e.g., using k1+k2=7 bits or 6 bits.

- Alt.2: Report one Target CSI sample via multiple RRC messages.

  - Send LS to RAN2 to inform the solution for confirmation.

| Company | View |
| --- | --- |
| QC | Alt1, lowering the quantization resolution is sufficient. |
| vivo | Alt 2, per layer reporting for NW side data collection can be considered Moreover, single sample report is insufficient for data collection; consequently, several RRC messages are needed to report multiple samples |
| Samsung | Alt1 is a reasonable proposal. Multiple RRC messages for just one sample is not reasonable. |
| Apple | Alt. 2 is apparently a more future proven approach, even if we are able to squeeze in one RRC message today. |
| Huawei, HiSilicon | We are supportive of the proposal and prefer Option 2. Based on our simulations, k1+k2 less than 8 results in an unreliable SGCS performance. Also, if the quantization is over real and imaginary parts, it is hardly justifiable to select unequal values for k1 and k2 and, therefore, k1+k2=7 doesn’t seem acceptable. Compared to k1+k2=7, k1+k2=6 is a more a justifiable choice but it results in a further drop in SGCS performance. |
| ZTE | Alt 1 may not work since it potentially sacrifices the training performance according to companies evaluations. If the large k1/k2 values are necessary from RAN1 requirement perspective, we agree to send an LS to RAN2 for feasibility confirmation of Option 1, given that Option 1 has deviated from the intention of logged data collection. Also, the solutions to address the large payload size of each sample should also be discussed in RAN2, which may include Alt2 or just enlarging the AS layer size. |
| CMCC | Alt1 is ok. |
| Nokia | Concern for Alt 2 because of the excessive overhead. Alt 1 is preferred |
| OPPO | Alt 1 is good. |
| TCL | Alt.1 is preferred. |
| Xiaomi | UE is not always required to report full rank channel information. Thus, k1+k2=8 can be additionally supported. For Alt.2, it’s up to RAN2 to decide. |
| MediaTek | Support Alt 2. |
| Ericsson | Alt 1 is preferred if the target CSI accuracy can be maintained. The target CSI sample sizes for option 1 are calculated assuming max rank = 4, which implies that the CSI compression feature supports CSI feedback for up to rank 4. If this is the common understanding among companies, then, we suggest making an explicit agreement on this. |
| CATT | We would say Alt2 is a more future-proof way if we have further enhance the AI CSI compression for more TX ports, for example 256 ports in the future. It might be good to send an LS to RAN2 for confirmation and we can have a future-proof solution. |
| Fujitsu | Alt 1 is OK. If one Targe CSI is transmitted via multiple RRC messages, additional signalling efforts are needed. |
| LGE | We prefer Alt 2. As we usually consider offline training for CSI compression, it seems more reasonable for maintaing high training performance than reducing overhead for NW-side data collection. Regarding overhead reduction, criteria-based sub-sampling can be used. |
| Lenovo | Although Alternative 1 offers advantages from an overhead standpoint, the requirement to transmit data within a single RRC message should not dictate the choice of quantization parameters. Instead, these parameters should be selected based on performance considerations; ideally, they would also fit within one RRC message. |

Moderator note: From observations, beta may have non-significant impact to performance; pv has moderate impact to performance, but limited when pv is as large as 0.75; L=6 or 12 has controversy; {Ref_amp, Diff_amp, Pha} = {4,3,4} or with increased value has controversy.

In addition, Qualcomm points out that there may be some trouble on complexity for FD basis selection and coefficients selection, so one option with pv=1, beta=1 is preserved.

Based on the above inputs, two sets are provided, based on UE capability.

### [Rd1] (Closed) Proposal 2.4 (Option 4 details):

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), consider the following two sets of parameters based on UE capability.

- L= 6; pv = 1; beta = 1; {Reference amplitude, differential amplitude, phase} = {4, 3, 4} bits

- L= 12; pv = 0.75; beta = 0.5; {Reference amplitude, differential amplitude, phase} = {6, 4, 6} bits

Moderator note: Seems not possible to determine specific sets of values. We can try to first narrow down the ranges.

### [Rd2] (Closed) Proposal 2.4a (Option 4 details):

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further down select between following candidate values: consider the following two sets of parameters based on UE capability.

- L= 6, 12, or N1N2

  - Note: the impact of Tx port number (e.g., as small as 8 or as large as 128) should be considered

- {pv, beta}={0.75, 0.5} or {1, 1};

- {Reference amplitude, differential amplitude, phase} = {4, 3, 4} bits or {6, 4, 6} bits

- L= 6; pv = 1; beta = 1; {Reference amplitude, differential amplitude, phase} = {4, 3, 4} bits

- L= 12; pv = 0.75; beta = 0.5; {Reference amplitude, differential amplitude, phase} = {6, 4, 6} bits

| Company | View |
| --- | --- |
| QC | Suggest to have L=12, p=1, beta=1 in addition for the better tradeoff between complexity, overhead and rich scattering scenario. |
| vivo | The benefits of {Reference amplitude, differential amplitude, phase} = {6, 4, 6} bits require further clarification. Some evaluation results show that increasing the number of phase and amplitude bits yields only a minor improvement. |
| Samsung | If we consider 128 ports, both alternatives may not provide sufficient resolution. Can we discuss these after the baseline configuration, e.g., 128ports, Nsb=19, v=4. |
| Huawei, Hisilicon | We support Option 4. Regarding the two suggested parameter sets, we are a bit concerned that the parameter set with L=6 and relatively smaller quantization resultion for amplitude and phase may not result in a Target CSI report that is accurate enough for NW side training. If we define UE capability for the support of these two parameters sets, the only plausible choice is that, for the UE that supports Option 4, the parameter set with L=6 is mandatorily support and the parameter set with L=12 is optionaly supported. This may lead to the situation that the NW collected data from the UEs that only support L=6 would not have a good quality and would corrupt the overall data gathered from all UEs (including the UEs supporting L=12 parameter set) used for NW side training. |
| ZTE | OK in general. |
| Nokia | For the two combinations we suggest replacing L=6 with L=min{N1*N2,6} and L=12 with L=min{N1*N2,12}. Without this change, no combination is applicable for number of antenna ports ≤ 8 |
| Fujitsu | Generally fine with the proposal. From our simulation, Performance gain can be observed from increasing differential amplitude bit from 3 to 4. {4, 4, 4} bits，can be considrred for the first PC. |
| KT | Overall, it’s fine. However, we have a question regarding the support of option 4. Are we assuming that the target CSI type here is limited to the precoding matrix only? Since the channel matrix as a target CSI type is still on the table, option 4 would require more bits for amplitude and phase to be applicable in that case. Unlike the precoding matrix—which is essentially an eigenvector normalized to 1 on a per-column basis—the channel matrix requires conveying inter-column norm information during quantization. |

- Issue#2-2 NW side data collection – Additional information indicated by NW/reported by UE

Brief summary:

Additional information configured by NW:

- Data quanlity threshold:  ZTE, TCL

- Rank/layer index/number:  Huawei, HiSilicon, CMCC, Sharp, [Spreadtrum]

- Associated ID/Pairing ID: Samsung

Filtered report: Lenovo, LGE, Southeast University

- Lenovo: based on the experienced distortion level or a quality indictor

- LGE: based on predefined criteria (e.g., data quality metrics threshold)

Additional information reported by UE:

- Data quality related info: LGE, CATT, Samsung, ZTE, NTT DOCOMO, Lenovo, Huawei, HiSilicon, Tejas, TCL

  - RSRP, SINR, CQI: Samsung, ZTE, CATT, Huawei, HiSilicon, Tejas, TCL

  - positioning information: ZTE

  - SGCS (between the data sample in Float32 format and data sample in the configurated data reporting format): Samsung

  - Distortion level: Lenovo, Tejas

- Rank/layer info:  CATT, Samsung

  - No need to report RI/CQI: CMCC

- Configurations to generate the Target CSI/CSI feedback, including Tx ports and subband configurations CATT

- Sample group size: Lenovo

- NW-side additional condition, UE side additional condition, associated ID: Lenovo

Moderator note: Same proposal as last meeting.

UE report:

NW configuration:

### [Rd1] Proposal 2.5 (Same as RAN1#122b proposal):

For NW side data collection with higher layer reporting,

- Target CSI (for precoding matrix) is generated following the number of layers configured by NW

- UE additionally reports data quality related information

  - FFS definition of data quality related information, e.g., CQI, RSRP, SINR, etc.

  - FFS data quality related information is per data sample or across data samples

- UE additionally reports the cell ID and configuration ID (e.g., CSI-LoggedMeasurementConfigId) of the corresponding NW side data collection configuration for generation of Target CSI, reusing the R19 AI BM NW side data collection rule.

| Company | View |
| --- | --- |
| QC | The necessity of data quality needs further discussion. Besides, RAN2 may already have mechanism to ensure good quality for the UE, e.g., configuring threshold / event for data collection. With that, reporting data quality is not really needed. |
| vivo | Firstly, regarding the number of layers, we slightly prefer letting the UE report the rank value as legacy, and which can be equal to or smaller than the max rank value configured by the NW. Additionally, we share the same view as QC to reuse the RAN 2 existing mechanism for data quality. |
| Samsung | If UE does not determine the rank, the quantized eigenvalues gives useful information for the network. |
| Apple | Further discussion is needed. |
| Futurewei | Our view is that the need and how to use it by the UE-side for data quality needs to be clarified and agreed. If RAN2 already defined this, it would be good to discuss whether RAN1 will adopt it. |
| Huawei, HiSilicon | Support (with a comment on the second FFS). For the first FFS, we think CQI (due to the possibility of sub-band CSI report) or RSRP (due to its time-domain averaging effect which provides a better long-term quality metric) are more appropriate for the data used for model training. For the second FFS, it may better to first calrify what a “data sample” mean. If it is a 2N1N2*N3 matrix for each MIMO layer, then none of the candidate quality metrics (CQI, RSRP, SINR) are defined per layer and, therefore, it seems that the quality cannot be reported per data sample. |
| ZTE | A clarification question for the 1st bullet: how does the NW configure the number of layers without knowing the rank? Additionally, the last bullet can be deleted since the reporting of additionally information like cell ID is determined by RAN2, similar as the discussions for AI BM. |
| CMCC | Ok with first bullet. The 3rd bullet can be discussed by RAN2. |
| TCL | We need to further discuss whether other information needs to be configured by NW or reported by UE. |
| Xiaomi | The necessity of data quality should be justified. Regarding the last bullet, although it’s straightforward to reuse R19 AI BM NW side data collection, it belongs to RAN2 scope. |
| CATT | Generally fine with the first two bullets. Regarding cell ID and configuration ID, we agree with ZTE and CMCC that such information falls into RAN2 scope. |
| InterDigital | We support UE reporing of data quality related information for target CSI (ground-truths). Data quality reporting may help to reduce the NW-side data collection reporting overhead (e.g., by filtering data based on quality criteria/contraints). |
| Panasonic | On the assistance information for NW-side data collection, it is also useful that UE can log / store its ground-truth CSI together with the UE-side additional condition and to report it to network especially for inter-vendor model training purpose. |
| Fujitsu | We are OK with the first two bullets. Regarding cell ID and configuration ID, it may be RAN2 issues. |
| LGE | In our view, both reporting data quality related information, which is informative for NW, and configuring data quality threshold for collecting qualified data can be considered. |
| Lenovo | We support inclusion of data quality realted information. Further discussion is needed on what exactly needed to be reported. |

- Issue#2-3(Onhold) UE side data collection – additional information

Brief summary:

Other information provided in CSI-reportConfig for UE side data collection

- Supported configuration(s) of pairing ID (e.g., multiple port configuration, multiple subband configuration, and multiple payload size configuration). vivo

- Tx antenna port or subband information ZTE

- Linkage between CSI-LoggedMeasurmentConfig and CSI-ReportConfig for NW-side data collection Samsung

Other information requested by UE for UE side data collection

- UE can indicate preferred configuration: Apple (CSI-RS density and pairing ID), CATT (same as AI BM)

Moderator note: Inputs are limited. Onhold until more inputs are collected.

- Issue#2-4 UE processing related issues for NW/UE side data collection

Brief summary:

CPU occupation

- Same CPU occupancy duration as Rel-19 beam prediction and CSI prediction data collection: Nokia, ZTE, Samsung, OPPO, CMCC, Honor, Sharp

Moderator note: In the last meeting, the CPU occupation duration is pended due to unclear rule for up to 128 ports. According to the Tdoc for this meeting, companies’ views are aligned, that the AI CSI prediction rule can be reused.

### [Rd1] Proposal 2.6:

For CPU counting of UE side data collection, the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion.

| Company | View |
| --- | --- |
| vivo | OK |
| Samsung | If multiple resources are agreegated for larger number of ports, we need to clarify the agreegated resources are considered as one resource for timeline or use the following - the occupancy starts from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement. |
| Futurewei | We are ok with the proposal. |
| Huawei, HiSilicon | Support |
| ZTE | OK with Samsung’s clarification. |
| CMCC | OK with Samsung’s clarification. |
| Nokia | Ok |
| OPPO | Ok |
| Xiaomi | Fine with the proposal. Samsung’s suggestion is a valid point when multiple resources are aggregated. |
| MediaTek | Support |
| CATT | OK |
| LGE | OK with Samsung’s clarification, with deleting SSB. |

- Issue#2-5 Other solutions or issues

### [Rd1] Question 2.7:

For NW/UE side data collection, any other solutions or issues are missed?

| Company | View |
| --- | --- |
| vivo | Agreement: Support the configuration of following parameters in a CSI-ReportConfig with the higher layer parameter reportQuantity set to'none-bm-r19': - TimeGap-r19, i.e., the time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance. - nroftimeinstance-r19, i.e., number of predicted time instances for BM-Case 2. Agreement: For inference report configuration, support separate configuration of subband, Tx port and payload size from pairing ID.. According to BM agreement and inference report configuration, separate configuration of subband, Tx port and payload size from pairing ID can be configured |
| Panasonic | We think the necessity and feasibility of UE reporting UE-side additional conditions for UE-side data collection and/or NW-side data collection should be discussed. |
| Mod | To vivo: The configuration provided for UE side data collection can be discussed after the provision of Pairing ID is agreed. To Panasonic: There seems no evaluation on the necessity of providing UE side additional condition. |
| QC | CPU occupation is needed for NW side monitoring. |
| Samsung | Indication of pairing-related ID for UE-side and NW-side data collection. |
| Xiaomi | UE side data collection based on UE request should be supported for CSI compression, and the procedure of UE side data collection for beam management and CSI prediction is reused. |
| Panasonic | In Rel.19 study, there was some evaluation under NW-side and UE-side data distribution mismatch. Some souces observed that there is performance loss when NW-side dataset does not include UE-side dataset. |
| LGE | Agree with QC that CPU occupation for NW-side monitoring is needed |

- Monitoring

  - Summary of company proposals

### 3.1-1: Generic

| Company | View |
| --- | --- |
| CATT | Proposal 14: For AI/ML based CSI compression Case 0, regarding performance monitoring, support L1 signaling via CSI report. |
| ETRI | Proposal 4: For AI/ML-based CSI compression, study both NW-side and UE-side performance monitoring to leverage their respective advantages. |
| Lenovo | - For monitoring of CSI-compression, configure the UE with additional report-config instructing it how to generate the additional data needed for monitoring of the model, e.g., the quantized version of the RAW CSI for NW-side monitoring or computed performance metric for UE-assisted monitoring. - Support the following combinations for the AI-output CSI-report and the type of monitoring report. Monitoring report type AI-output report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support. - Further specify the required procedure/signalling for model updating, switching or model fall-back as a result of a) model monitoring outcome, or b) request from the UE, or c) change of some configuration parameters. |
| NEC | Observation 3: Both NW and UE side performance monitoring are feasible and a combination of NW and UE side monitoring can be supported. Observation 4: Study is needed on how to define the linkage between monitoring and inference reports to guarantee valid time-domain comparison for performance monitoring. - Proposal 11: Study the mechanism and specification impact of identifying root causes for performance degradation, enabling accurate diagnosis across different root causes with corresponding impacts on LCM actions. |
| DOCOMO | Observation 1 - NW-sided monitoring: The performance can be monitored by the ground truth (Target CSI) report from the UE with some UL overhead. - UE-sided monitoring: - Based on the proxy model or direct KPI estimation: Inaccurate and biased because the UE does not know the CSI reconstruction implementations, and it has limited capability. - Based on the RS from the NW: Introduce DL overhead and UE complexity for additional measurements and CSI processing. - The UE-sided monitoring based on the proxy model or direct KPI estimation is not suitable for standalone usage, but it can serve as a trigger for the NW-sided monitoring. Proposal 1 - Support the NW-side performance monitoring with UE-reported ground truth. - The UE-side performance monitoring with proxy model or direct KPI estimation can be supported given the following conditions, - The NW-side performance monitoring is supported, and the ground truth report can be event-trigger-based. - The UE-side performance metric serves only as a trigger for the ground truth report. |
| OPPO | Proposal 7: Regarding the performance monitoring for AI/ML based CSI compression, support • UE-side monitoring Case 2-1/Case 2-2 • NW-side monitoring according to UE capabilities. Possible combination of UE-side monitoring Case 2-1 and NW-side monitoring. |
| Panasonic | Observation 1: There are at least two purposes for performance monitoring. The monitoring purpose 1 is to check new untested model / parameter behavior. The monitoring purpose 2 is to check current model / parameters are suitable to current environment. Observation 2: If AI/ML models are trained by UE-side and AI/ML model update cycle is non-real time, UE vendor specific monitoring in offline for monitoring purpose 1 sufficiently work. Observation 3: If AI/ML models are trained by network side and AI/ML model update cycle is non-real time, A/B test can be used, and data collected for non-real time performance monitoring for monitoring purpose 1 can also be used for model training. Observation 4: - For monitoring purpose 1, the monitoring mechanisms to detect degradation could be sufficient. - For monitoring purpose 2, both monitoring mechanisms to detect degradation and to identify the cause of degradation would be needed. Observation 5: - As the monitoring mechanism to detect degradation, the following is useful. - NW-side or UE-side eventual KPI-based monitoring - UE-side intermediate KPI monitoring based on the output of the CSI reconstruction model at the UE - As the monitoring mechanism to identify the cause of degradation, the following is useful. - NW-side intermediate KPI monitoring based on the target CSI reported by the UE via legacy eTypeII codebook or eTypeII-like codebook. - UE-side intermediate KPI monitoring based on the output of the CSI reconstruction model indicated by NW. |
| Qualcomm | Observation 7: There are spec-transparent performance monitoring mechanisms, such as throughput comparison, CQI comparison with legacy CSI codebooks, or obtaining ground-truth CSI using existing CSI report with existing codebook for NW side monitoring. Proposal 5: Spec-transparent monitoring can be used; the necessity of specifying dedicated monitoring methods needs justification. • If needed, any standardized performance monitoring for two-sided AIML CSF should consider UE complexity and require additional capability than the feature itself. |
| Rakuten Mobile | Proposal 1: RAN1 to consider the following, proceeding with two-sided model CSI compression for effective model supervision and online finetuning procedures: • To allow UE side monitoring where encoder’s intermediate KPI: SGCS or 1-bit Model-At-Risk flag (MAR) is reported in one inference cycle using L1 trigger signal via UCI when KPI < configured threshold. • To adopt higher-layer NW side monitoring for Immediate/logged MDT to capture paired samples {Target-CSI, Compressed-CSI} over a sample window .This provides ground truth for retraining and KPI auditing under the UE trigger scenario. |
| Samsung | Observation#10: For two-sided model-based CSI compression, the following monitoring types were identified - Type 1: UE-assisted monitoring: UE calculates monitoring metric - Type 1-1: UE-assisted monitoring without precoded CSI-RS - Type 1-2: UE-assisted monitoring with precoded CSI-RS - Type 2: NW-side monitoring: NW calculates monitoring metric from report ground-truth CSI Proposal#11: Study the benefits and cost of the following monitoring types after some progress in the reference model discussion and Target CSI format - Type 1: UE-assisted monitoring: UE calculates monitoring metric - Type 1-1: UE-assisted monitoring without precoded CSI-RS - Type 1-2: UE-assisted monitoring with precoded CSI-RS - Type 2: NW-side monitoring: NW calculates monitoring metric from report ground-truth CSI - Note: cost analysis should include at least measurement and feedback overhead, UE complexity and network complexity. |
| TCL | Proposal 6: The request of target CSI, encoder output and quantizer output could be based on the existing CSI report framework for the root cause analysis. |
| vivo | Proposal 8: Similar to CSI predication, periodic, semi-persistent, and aperiodic CSI-RS are supported, and semi-persistent, and aperiodic monitoring report types are supported for the monitoring of CSI compression. |

### 3.1-2: NW side monitoring

| Company | View |
| --- | --- |
| Apple | Proposal 7: For CSI compression using two-sided model, for NW side performance monitoring, UE report CSI feedback and target CSI pair using immediate MDT with logging framework. The MDT framework can be used for root cause detection following UE side performance monitoring. |
| ASUSTeK | Proposal 3: For NW side performance monitoring, UE transmits assistance information of UE-side encoder’s performance along with ground truth reporting. |
| CATT | Proposal 15: For AI/ML based CSI compression Case 0, support NW-side monitoring based on the target CSI reported by the UE via high resolution codebook. - Monitoring report content is the target CSI, i.e., SF-domain eigenvector - Legacy Rel-19 eType II codebook can be considered as the starting point Proposal 16: Regarding AI/ML based CSI compression Case 0, for NW-side monitoring based on target CSI reporting, monitoring report is separately configured from inference report. - Association between monitoring report and inference report is managed by gNB in a specification transparent manner |
| CMCC | Proposal 14: For CSI compression using two-sided model use case, support L1 signaling for NW side monitoring, and the content is Target CSI only - Quantization format of Target CSI is the same as for NW side data collection. |
| Ericsson | - Eventual KPI based monitoring has low complexity, low overhead, and can capture network MU-MIMO performance. The NW can perform frequent monitoring of eventual KPIs via NW implementation-based solutions and use it as a first step for detecting potential AI/ML feature/functionality failure. - It is necessary to specify UE reporting high resolution target CSI to enable NW-side intermediate KPIs based performance monitoring. - It is necessary to specify UE reporting target CSI and CSI feedback pair to enable performance degradation error cause detection for the two-sided CSI-compression use case. - NW-side monitoring of the two-sided CSI-compression model based on target CSI reporting is expected to be executed infrequently (e.g., event triggered or periodically with a large periodicity), hence, the monitoring data collection overhead for this model monitoring method is in general not an issue. - Support UE reporting multiple target CSI and CSI feedback pairs via high-layer signaling for NW-sided performance monitoring: - Reuse the target-CSI format defined for NW-side training data collection - Enhance the higher-layer signaling mechanism designed for NW-side training data collection to support UE reporting target CSI and CSI feedback pairs |
| ETRI | Proposal 5: For AI/ML-based CSI compression, for NW-side performance monitoring, consider L1 signaling for low latency, and consider report config ID or pairing ID for linkages between AI/ML-based CSI feedback and the ground-truth CSI. |
| Fujitsu | Proposal-3: Support NW-side performance monitoring based on target CSI reporting from UE, by using the format of eT2-like codebook with new parameter values. Proposal-4: Regarding the content of CSI report for NW-side performance monitoring, at least L1 report is supported. - Support the reporting of a paired Target CSI and CSI feedback in one CSI report. |
| FUTUREWEI | Observation 1: For NW-side monitoring, using L1 for reporting Target CSI may have the benefit of allowing NW to receive performance report in a timelier manner compared to L3 reporting. Proposal 3: In AI/ML-based CSI compression using two-sided model Case 0, at least for NW-side monitoring, consider using L1 signaling for Target CSI reporting and adopting the following option for the content: - Option 2: Target CSI only (assuming NW obtains CSI feedback via inference CSI report) |
| Google | Proposal 3: If NW-side performance monitoring is supported, support the NW configures the UE to report the CSI based on eType2 codebook with enhanced parameters based on the eType2 CSI report framework - Support the UE to report CQI additionally to facilitate CSI acquisition |
| HONOR | Proposal 4: Support L1 signaling for NW-side monitoring. Proposal 5: For L1 performance monitoring, reuse the legacy codebook without introducing new codebook parameters. Observation 5: For inference reports with rank > 2, the high-resolution codebook with PC#8 cannot be used for monitoring purposes, which impact monitoring accuracy. Proposal 6: Based on the legacy PC table, for the inference reports with rank > 2, partial layers of the Target CSI can be reported by UE via higher-resolution codebook with PC#8. |
| Huawei, HiSilicon | Proposal 9: For NW side monitoring, reuse the same Target CSI quantization format(s) that are supported for NW side data collection. Proposal 10: For NW side monitoring, support inclusion of the following content in the report: - Paired Target CSI and CSI feedback if L3 reporting is supported - Target CSI if L1 reporting is supported - Note: In this case, NW obtains the associated CSI feedback via inference CSI report. |
| InterDigital | Observation 3: NW-side monitoring can have a significant impact on signaling overhead. Proposal 3: For NW-side model monitoring, support use of predefined vector sets, e.g., basis vectors, and/or redundancy elements. Observation 4: NW-side data collection for model monitoring via L3 signaling could be used when larger data sizes need to be signaled (e.g., if using eTypeII-like high resolution CB), or when there are no critical latency constraints. Proposal 4: For NW-side model monitoring, support L3 signaling for ground-truth data collection, in addition to L1 signaling. |
| Lenovo | - It is NW implementation to configure a dedicated monitoring CSI report via L1 signaling feedback for monitoring the performance. - The higher layer signaling for NW side monitoring with the report content of paired Target CSI and CSI feedback can be supported, where the Target CSI is the same as for NW side data collection. |
| LGE | Proposal #3: For NW-side monitoring, support target CSI reporting via L1 signaling with paired Target CSI and CSI feedback. |
| MediaTek | Proposal 11. For NW-side monitoring with L1 signaling, only support legacy codebook for reporting target CSI. Proposal 12. Support L3 signaling for NW-side monitoring to transfer multiple Target CSI samples to NW. Proposal 13. For NW-side monitoring with L3 signaling , there is no need to report CSI feedback . |
| NEC | - Proposal 7: For NW-side monitoring, support reporting paired compressed CSI and target CSI in one CSI report. - Proposal 8: For NW-side monitoring, if the compressed CSI and target CSI are reported in separate reports, support both L1 signaling and L3 signaling for reporting the target CSI. - Proposal 9: For NW-side monitoring, if the compressed CSI and target CSI are reported in separate reports, discuss whether it is still necessary to report other information besides PMI (e.g., RI, CQI) for the latter report in the time domain. - Proposal 10: NW can configure target CSI reporting based on the SGCS results, with MAC CE to indicate whether target CSI reporting needs to be performed. |
| Nokia | Proposal 10: RAN1 to agree to mandate UEs supporting NW side performance monitoring feature to support eType2 codebook-based CSI feedback as well as AIML-based CSI feedback. Observation 14: For NW side performance monitoring, the required feedback overhead for GT target CSI reporting can be reduced by adjusting its resolution with respect to the resolution of the AIML-enabled CSI acquisition scheme under monitoring. Lower resolution of the CSI inference report being monitored requires lower resolution of the ground truth target CSI. Proposal 11: For NW-side performance monitoring, to reduce the feedback overhead of reporting the ground truth target CSI, introduce a definition of ground truth target CSI with an adjustable resolution with respect to the resolution of the CSI inference report being monitored. |
| DOCOMO | Proposal 2 - If the Layer 3 signalling is adopted for the ground truth reporting, only Target CSI is reported. - The signalling selection is up to RAN2. RAN1 should further study the following issue, - The latency requirements, report occasions, and payload size of the report, which help RAN2 to select the reporting mechanism. - The periodicity of the report, including the support of event-triggered reporting. - The association between the Layer 3 ground truth report and Layer 1 inference report. - Opt. 1: Include a series number in the CSI report for the association purpose. - Opt. 2: Define a timeline considering the characteristics of Layer 3 signallings. Proposal 3 - Support a unified quantization approach with different payload sizes for the Target CSI report for performance monitoring and the NW side data collection. - The unified quantization approach means that the payload for performance monitoring can be obtained from the payload of the NW side data collection by omitting some bits. |
| Ofinno | Proposal#1: Regarding whether/how to support higher layer-based monitoring, clarify target CSI quantization/overhead and higher layer signalling latency requirement for LCM decision. |
| OPPO | Observation 1: Regarding NW-side monitoring Case 1-1, two concerns from overhead and UE capability aspects should be addressed. Proposal 8: Regarding the CSI report configuration for NW-side monitoring, consider: • Option 1: one CSI report configuration for both of inference and monitoring report on the same CSI report resource.  New reportQuantity or combination of two PMIs in reportQuantity is required to indicate UE reports both of inference from AI/ML based CSI and target CSI with legacy codebook for monitoring • Option 2: dedicated CSI report configuration for monitoring report on dedicated CSI report resource different from inference report. Codebook type and parameter combinations for target CSI calculation should be configured |
| Qualcomm | Observation 8: SGCS statistics or average SGCS across multiple inference samples are needed to determine the data distribution drift. Observation 9: High monitoring accuracy may not matter much in monitoring decision, the dominating factor is the natural SGCS variation / statistics across multiple samples. Proposal 6: SGCS statistics or average SGCS across multiple inference samples should be taken into account for the study of the necessity and feasibility of NW side monitoring. Observation 10: Performance monitoring is not time-critical and requires multiple target CSI samples to identify data drift, so there is no fundamental difference between NW side monitoring and NW side data collection. L3 signalling is a more proper solution that allows UE to log multiple target CSI in a flexible manner with lower cost than L1 signalling. Proposal 7: For Rel-20 CSI feedback with two-sided model, support L3 signalling and mechanism for target CSI reporting for NW side monitoring. Framework for NW training data collection can be a starting point. Proposal 8: For NW side monitoring, study the SGCS variation in a dataset A vs. SGCS variation in a dataset B where dataset A is considered as “in-distribution” and dataset B is considered as “out-of-distribution”. Define false-alarm and miss-detection event using dataset A and B. Observation 11: Increasing number of samples used in averaging the SGCS measurement improve the monitoring decision accuracy. Observation 12: Using existing parameter-combinations of eType II codebook is sufficient to achieve good monitoring results. Observation 13: If NW data collection framework is used for NW side performance monitoring, the data format used for data collection can be reused for monitoring. Proposal 9: Further optimization on data format for NW performance monitoring is not needed. Data format for model training can be reused if L1 mechanism is supported for NW side performance monitoring. |
| Sharp | Proposal 5: For NW side monitoring, target CSI reporting via eTypeII codebook can be supported as baseline. - Support target CSI reporting with configured rank by network Proposal 6: For NW side monitoring, support to reuse CSI framework for monitoring report in L1 signalling to carry target CSI via eTypeII codebook. - For monitoring report type, semi-persistent on PUSCH and aperiodic CSI report are supported. - FFS: whether some contents in existing quantity can be omitted and whether a new report quantity is configured. Proposal 7: For NW side monitoring, consider using higher layer signalling to report target CSI of high resolution in the same format as NW side data collection, provided that one of options 1 and 4 was agreed for NW side data collection. |
| Sony | Proposal 1: RAN1 to priorities network-side performance monitoring over UE-side performance monitoring. Proposal 2: RAN1 adopts network-side performance monitoring of AI/ML-based CSI compression in which the UE transmits both compressed CSI and ground truth CSI to the network and the network performs the comparison Proposal 3: RAN1 to consider performance monitoring frequency including: • Periodic monitoring • Event-triggered monitoring based on eventual KPIs measured by the network, such as BLER or throughput to minimize overhead while ensuring timely detection of model degradation. Proposal 4: RAN1 should adopt the same intermediate KPI (SGCS) for monitoring that was adopted for CSI prediction for CSI compression |
| Southeast University | Proposal 2-1: Define two target CSI reporting configurations (Co-located Reporting with combined AI-CSI/legacy CSI, Dedicated Reporting with separate resources linked to inference CSI-RS) for Rel-20 to enhance network deployment flexibility. Proposal 2-2: Define NW configuration of codebook types/parameters for target CSI calculation, and support UE reporting of paired target CSI/CSI feedback via high-layer signaling (reusing NW training data mechanisms) to enable NW-side reference/UE CSI comparison for root-cause identification. |
| Spreadtrum, UNISOC | Proposal 4: For performance monitoring, support NW-side monitoring based on the ground-truth CSI reported by UE with legacy Rel-16 eType II codebook. Proposal 5：L1 signaling is preferred that allows NW to quickly monitor model performance and trigger fallback if necessary. |
| TCL | Proposal 4: For NW side performance monitoring, the label acquisition could be based on the existing CSI report framework, e.g., both target CSI and CSI feedback are configured as the report quantities. |
| vivo | Proposal 6: Target CSI and CSI feedback pair report can be supported for NW side monitoring and error cause identification • The CSI feedback is the output derived from the recovery precoding matrix by target CSI (e.g., quantizing Target CSI) |
| Xiaomi | Proposal 8: Higher layer reporting for performance monitoring at NW side is supported. Proposal 9: The report content including pairs of target CSI and AI based compressed precoder. Proposal 10: The paring ID is indicated to UE for higher layer reporting for performance monitoring at NW side. |
| ZTE, Sanechips | Proposal 14: For AI/ML CSI feedback, support NW-side performance monitoring based on the target CSI reporting with CSI codebook enhancement via higher resolution parameter combination. |

### 3.1-3: UE side monitoring

| Company | View |
| --- | --- |
| Apple | Observation 2: For UE side performance, NW implicitly transmit output CSI using precoded CSI-RS or DMRS to the UE, provides a simple and low-overhead solution. In addition, comparing with legacy codebook based on defined KPIcodebook and KPIactual can help to determine whether fall back decision is required. Proposal 5: For CSI compression using two-sided model, support UE side performance monitoring with pre-coded RS using output CSI. Define monitoring KPI using precoding gain difference between KPIcodebook and KPIactual. Proposal 6: For CSI compression using two-sided model, for UE side performance, support RLF/BFD like mechanism. UE transmit fallback request via the applicability report procedure with UAI. |
| CATT | Proposal 17: For AI/ML based CSI compression Case 0, performance monitoring at UE side based on reference model or proxy model can be deprioritized. Proposal 18: For AI/ML based CSI compression Case 0, support UE-side monitoring based on precoded RS (e.g., CSI-RS) transmitted from NW with RS precoded by the output of the CSI reconstruction model (i.e., decoder output). - Consider monitoring metrics (i.e., report quantity for monitoring report) including SGCS, RSRP/differential RSRP. Proposal 19: Regarding AI/ML based CSI compression Case 0, for UE-side monitoring based on decoder output precoded RS, reuse performance monitoring mechanism via CSI report for AI BM and AI CSI prediction in Rel-19: - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link inference report configuration and monitoring report configuration - For the time behavior of monitoring report, semi-persistent and aperiodic reporting are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For time domain behavior of monitoring RS resources, semi-persistent and aperiodic resources are supported. Legacy rule on CSI report should be applied for allowed time domain behavior combination of RS resources and monitoring report. |
| CMCC | Proposal 12: For CSI compression using two-sided model use case, it is proposed to deprioritize UE side monitoring with proxy model / direct estimator of KPI / reference decoder. Proposal 13: For CSI compression using two-sided model use case, it is proposed to deprioritize UE side monitoring with precoded RS. |
| Ericsson | - Supported UE-side performance monitoring based on UE reporting an intermediate-KPI based performance metric (e.g., a SGCS value range indicator), at least the following spec impacts are identified: - The format of the performance monitoring metric (e.g., a SGCS range indicator) - L1 signaling for UE reporting the monitoring metric (e.g., a SGCS together with the CSI feedback via inference CSI report) - To ensure the quality of UE reported monitoring metric, at least the following spec impacts are identified: - RAN4 testing of the quality of UE reported monitoring metric - Mechanisms (e.g., RRC-message based methods) to support UE reporting target CSI, encoder output, together with the associated intermediate-KPI based performance metric to the NW. - UE-side monitoring based on precoded RS transmission from NW introduces RS signaling overhead, latency and processing complexity at both UE and NW, without clear benefit. - UE-side performance monitoring based on precoded RS transmission from NW is not supported. |
| ETRI | Proposal 6: For AI/ML-based CSI compression, for UE-side performance monitoring, consider L1 signaling to report performance metric such as SGCS. |
| Fujitsu | Proposal-5: Regarding UE-side performance monitoring, AI/ML-based performance monitoring is not supported unless how to guarantee the monitoring model’s performance can be clarified/justified. Proposal-6: Study on the standard impact for UE-side performance monitoring is suggested to be deprioritized. |
| Google | Proposal 2: If UE assisted performance monitoring is supported, support the UE to report the SGCS between the channel measured from a precoded CSI-RS and the target CSI and channel measured from a CSI-RS for AI/ML based CSI report. - Reuse the R19 framework for the SGCS report |
| HONOR | Proposal 5: Deprioritize UE-side performance monitoring, if supported, precoded RS based monitoring can be considered. |
| Huawei, HiSilicon | Proposal 11: There is no strong motivation for specifying the UE side proxy model (Case 2-1/2-2) for monitoring. Proposal 12: Precoded RS based monitoring can be achieved with implementation. |
| InterDigital | Observation 1: UE-side monitoring is beneficial due to the lower latency and reporting overhead compared to NW-side monitoring. Proposal 1: Support UE-side monitoring in addition to NW-side monitoring. Observation 2: For UE-side model monitoring, to avoid an excessive overhead for reporting, an efficient reporting behaviour should be followed. Proposal 2: For UE-side model monitoring, support both time and event-triggered based reporting. |
| Lenovo | - Support Option 1 (i.e., Case 2-1/2-2) for the UE side monitoring via reporting the estimated SGCS in monitoring CSI report via L1 signaling. Observation 9: Model monitoring based on metric calculated at the UE has been specified for BM use case. This confirmed the possibility of having UE-sided monitoring without the need for extra NW-side monitoring for checking the reliability of UE-side monitoring reports. - Conclude that the local decoder model (or proxy model) that is trained using data shared in Direction A sub-option 4-1 and Direction C has a good match with the NW-side decoder model and it can be used for UE-side monitoring and UE-side root-cause determination. - Conclude that UE-sided model monitoring can be used at least for inter-vendor collaboration options based on Direction A sub-option 4-1 and Direction C. |
| LGE | Proposal #4: Deprioritize UE-side performance monitoring. |
| MediaTek | Proposal 14. If UE-side monitoring supported, discuss the following options to specify how UE reports a monitoring event to NW: • UE can calculate and report from a set of specified monitoring metrics. • UE can report specified monitoring output (i.e., occurrence) cause by a monitoring event. Proposal 15. Deprioritize UE-side monitoring due to additional burden on UE with no significant benefit. |
| NEC | Proposal 5: For UE-side monitoring, support SGCS as performance metric, SGCS is calculated based on target CSI and reconstructed CSI. Proposal 6: For UE-side monitoring, support event triggered performance monitoring report. The event is defined based on the SGCS lower than a threshold value for a time duration. |
| Ofinno | Proposal#2: Regarding whether/how to support UE side monitoring, clarify reliability of the performance metric and its definition to ensure testability across different UE implementation. |
| OPPO | Observation 2: Regarding UE-side monitoring Case 2-1/Case 2-2 there is no overhead and UE capability concerns. Proposal 9: UE-side monitoring Case 2-1 and Case 2-2 with SGCS report can be supported according to UE capability. Proposal 10: Regarding SGCS report format in UE-side monitoring, SGCS1 and SGCS2 agreed in CSI prediction agenda can be reused, wherein: - SGCS1 denotes the SGCS between target CSI and proxy model output in Case 2-1 or the direct output of SGCS estimator in Case 2-2. - SGCS2 denotes the SGCS using legacy codebook configured by NW-side within the CSI report configuration. Proposal 11: Regarding the CSI report configuration for UE-side monitoring, consider: - Option 1: one CSI report configuration for both of inference and monitoring report on the same CSI report resource. - New reportQuantity or combination of PMI and SGCS in reportQuantity is required to indicate UE reports both of inference from AI/ML based CSI and SGCS monitoring result - Option 2: dedicated CSI report configuration for monitoring report on dedicated CSI report resource different from inference report. Codebook type and parameter combinations for SGCS2 calculation should be configured Proposal 12: Support UE-triggered NW-side monitoring to reduce the reporting overhead of target CSI and improve the accuracy of UE-side monitoring. |
| Qualcomm | Proposal 10: Do not support UE side monitoring. |
| Samsung | Observation#11: For two-sided model-based CSI compression, for Type 1-1: UE-assisted monitoring without precoded CSI-RS, the UE may calculate performance metric, e.g., SGCS, using - Alt1: Specified reference decoder - Alt2: NW-shared proxy decoder - Alt3: direct metric estimator |
| Sharp | Proposal 8: For UE side monitoring, RAN1 should further conclude the necessity of supporting the UE side monitoring for CSI compression. - If UE side monitoring is supported, study details for each option among UE-side monitoring options agreed in RAN1#117. |
| Southeast University | Proposal 2-3: Specify UE-side performance monitoring based on a lightweight proxy decoder, with requirements including architecture, training, and performance metrics. Proposal 2-4: Define the evaluation metrics as normative performance requirements for UE-side proxy monitoring, specifying the calculation method and minimum/maximum allowed values for each: average SGCS prediction gap, classification precision and recall, and UE-NW reconstruction alignment. Proposal 2-5: Specify signaling and reporting mechanisms to support the proxy decoder framework. The NW shall signal the proxy decoder’s configuration (e.g., architecture ID, distillation parameters) to the UE via RRC signaling during model activation. The UE shall report the proxy’s predicted SGCS (or a SGCS range indicator) to the NW alongside the CSI feedback to enable joint performance analysis. |
| Spreadtrum, UNISOC | Proposal 6: For performance monitoring, support UE-side monitoring based on the recovery CSI indicated by NW or precoded RS transmitted form NW. |
| TCL | Observation 2: The UE side performance monitoring can be performed by UE implementation. Proposal 5: For UE side performance monitoring, UE needs to report the monitoring metric values or monitoring outcome. |
| vivo | Proposal 7: UE side monitoring can be supported for CSI compression with proxy model based solution. • SGCS can be a report format for monitoring |
| Xiaomi | Proposal 11: Precoded RS based solution is preferred comparing with model-based solution for UE side performance monitoring, and the following two options can be considered to monitoring model performance at UE side. - Option 1: Reusing legacy eventual KPI reporting with precoded CSI-RS, e.g., CQI report subject to “cri-RI-CQI”, i.e., no additionaly spec impact. - Option 2: Gap of precoding gain between HT0 vTarget(T0)2 and HT1 vPrecoded(T0)2. Proposal 12: The configuration for UE side performance monitoring is linked with a CSI report configuration for inference which including a paring ID, if UE side performance monitoring is supported. |
| ZTE, Sanechips | Proposal 15: For AI/ML CSI feedback, prioritize NW-side performance monitoring and take UE-side performance monitoring as second priority. |

  - Discussions

- Issue#3-1 (Onhold) Prioritization between NW side monitoring and UE side monitoring

Brief summary:

Prioritize NW side monitoring/Deprioritize UE side monitoring: [NTT DOCOMO], ZTE, CMCC, Qualcomm, LGE, MediaTek, Fujitsu, Honor, Sony

Support both: InterDigital, ETRI

- Issue#3-2 NW side monitoring

Brief summary:

Quantization format of Target CSI

- Follow the same format as NW side data collection: Huawei, HiSilicon, Nokia, CMCC, NTT DOCOMO

- eType II(-like) format with higher resolution parameter combination: ZTE, Tejas, Fujitsu

- legacy eType II codebook: CATT, LGE, Tejas, Sharp, MediaTek, HONOR, Spreadtrum

Content for reporting:

- Option 1: Paired Target CSI and CSI feedback

  - Support: InterDigital, Lenovo, Rakuten, Huawei, HiSilicon (for L3 reporting), Ericsson, vivo, Apple, Xiaomi, Fujitsu (for L1 reporting), Sony, LGE (for L1 reporting), NEC, Southeast University

- Option 2: Target CSI only (assuming NW obtains CSI feedback via inference CSI report)

  - Support: MediaTek, NTT DOCOMO, Huawei, HiSilicon (for L1 reporting), CMCC (for L1 reporting), Futurewei (for L1 reporting), Honor

    - NTT DOCOMO: Include a series number in the CSI report for the association, or define a timeline as the timing reference to locate the associated CSI feedback

  - Not support: Huawei, HiSilicon (for L3 reporting) Xiaomi

    - Huawei, HiSilicon, Ericsson: NW would not know the logged Target CSI corresponds to which reported resources during inference

    - Xiaomi: CSI omission may happen for CSI feedback at inference CSI report

- Other views

  - Introduce a definition of ground truth target CSI with an adjustable resolution with respect to the resolution of the CSI inference report being monitored  Nokia

  - UE may not be needed to report RI, CQI NEC

  - NW to configure the rank Sharp

Signaling to carry the reported data:

- L1: InterDigital, MediaTek (with legacy eType II), Qualcomm (with legacy eType II), CATT (with legacy eType II), Huawei, HiSilicon, [OPPO], LGE, NEC, Sharp, Futurewei, ETRI, Fujitsu, Honor (with legacy eType II), Spreadtrum (with legacy eType II)

  - Separate CSI report configuration for monitoring from inference CATT, Huawei, HiSilicon, OPPO, Lenovo (NW implementation)

    - Association between monitoring report and inference report is managed by gNB in a specification transparent manner CATT, Huawei, HiSilicon

    - Linkage between monitoring and inference by report ID/pairing ID ETRI

  - One CSI report for both inference and monitoring report OPPO

  - For the inference reports with rank > 2, partial layers of the Target CSI can be reported by UE via higher-resolution codebook with PC#8 Honor

- L3/higher layer: InterDigital, NTT DOCOMO, Lenovo, Ericsson, Qualcomm, Apple, Xiaomi, Huawei, HiSilicon, NEC, Rakuten, Sharp (with high resolution format), Southeast University

- UE requested NW side monitoring: OPPO

- Periodic monitoring, NW triggered monitoring: Sony

Moderator note: Companies’ views are a bit diverse. Some do not think this proposal is needed, while others want to introduce some enhancement (configured information or enhanced CB parameters) on top of L1 report. It is Moderator’s feeling that it may be difficult to achieve consensus on L1 level enhancements. Let’s first discuss higher layer signaling.

### [Rd1] (Low priority) Proposed conclusion 3.1 (L1 signaling):

For NW side monitoring, it is NW implementation to perform NW side monitoring with legacy methods, e.g., eType II codebook via L1 signaling, SRS, eventual KPI, etc.

| Company | View |
| --- | --- |
| QC | (copy-paste our comments in the last meeting) We don’t need this proposal. In Rel-19, we concluded that monitoring via legacy CSI codebook is feasible. This conclusion is sufficient. It implies that it’s upto NW to configure a CSI report to acquire a target CSI using legacy eT2 format. From UE perspective, it’s just a CSI report, nothing else. There is no spec impact. If a conclusion is needed, consider following text change “For NW side monitoring, it is NW implementation to perform NW side monitoring without specification impact with legacy methods, e.g., eType II codebook via L signaling CSI report via legacy codebook, SRS, eventual KPI, etc.” |
| Huawei, HiSilicon | Support. |
| Xiaomi | Agree with QC. |
| MediaTek | Support. We are fine with QC’s revision |
| CATT | OK |
| NEC | L1 report still needs to be discussed since timely monitoring can only be achivable by L1 report. |
| Fujitsu | We think L1 report is needed, and eT2-like codebook can be considered for it. |
| LGE | We prefer paired contents for NW-side monitoring reporting, since a dedicated monitoring report requires unnecessary additional measurement at the UE (e.g., buffering, CPU). Also, if dedicated monitoring report is supported, there is case that one of reports can be dropped due to collision. In that case, NW cannot calculate performance metric/output. Thus, it would be good to consider reporting both results of inference and monitoring together when monitoring report occasion is occurred. |

Moderator note: For higher layer signaling to report Target CSI, it seems the report of Target CSI only may not be workable: one thing is that CSI feedback from inference may be an omitted version; another issue is that gNB/UE may miss the UCI/DCI, so that the pairing between CSI feedback (in L1 inference CSI report) and Target CSI (in higher layer reporting) may not be ensured.

Moderator note: Updated the UCI omission part. The intention is to distinguish from L1 CSI feedback which considers UCI omission.

Regarding Lenovo, CMCC, CATT comments, if we adopt high-resolution (e.g., Option 1) for Target CSI format, how is it possible to be carried at L1 signaling? Or, what you have in mind is just reusing legacy eType II CB with NW implementation?
Regarding ZTE comment, for NW side data collection, indication of time stamp is precluded by RAN2. It is Moderator’s feeling that the same thing may happen for monitoring via higher layer.

### [Rd1] Proposal 3.2 (Higher layer signaling):

For NW side monitoring, support the higher layer signaling for reporting.

- Regarding the reporting content, at least consider paired Target CSI and CSI feedback

  - FFS: CSI feedback is the one before or after UCI omission.

- Quantization format of Target CSI is the same as for NW side data collection.

| Company | View |
| --- | --- |
| vivo | Support |
| NTT DOCOMO | We suggest FFS the paired Target CSI and CSI feedback part. Even if we consider L3 signalling, it is not a good idea to waste UL resources transmitting a message that has already been reported to the NW. As proposed, there are several ways to establish the association between the Target CSI and the reported CSI feedback. Let’s further study them. |
| Futurewei | We are ok to get consensus for high layer signaling for NW-side monitoring first while further discussing whether/how L1 signaling can also be supported. We suggest adding an FFS on L1 signaling. For NW side monitoring, support the higher layer signaling for reporting. - Regarding the reporting content, at least consider paired Target CSI and CSI feedback - FFS: CSI feedback is the one before or after UCI omission. - Quantization format of Target CSI is the same as for NW side data collection. FFS: whether/how to support L1 signaling for NW-side monitoring. |
| Huawei, HiSilicon | Support. For the FFS, we think Target CSI is better to be paired with the CSI feedback that is actually transmitted (after UCI omission) otherwise, the monitoring procedure will overestimate the performance of AIML-based CSI compression. |
| Nokia | The usefulness of higher layer reporting for NW-side performance monitoring is questionable. This is not useful for real-time monitoring |
| OPPO | The delay of using higher layer signaling for NW-side monitoring should be considered. We are not okay to say support it currently. |
| Xiaomi | Support. In addition, Paring ID should be indicated to UE for NW side performance monitoring as UE needs to determine which model for CSI feedback generation. The paring ID can be configured in the configuration of higher layer reporting explicitly or indicated implicitly via the linkage with CSI report configuration for inference. |
| MediaTek | Support higher layer signaling. In our view, monitoring based on L1 singaling and a single sample may suffer from high missdetection/false alarm rate which can be cured to some extent by inspecting more samples sent by L3. We see L3 monitoring as a complementary scheme to L1 monitoring, not an alternative to replace it. |
| Ofinno | Similar view with DOCOMO. In addition, while this higher layer signalling alleviates air-interface overhead and complexity issues, it does not provide a kind of timeliness. For further study, need to provide requirements on the necessary latency/report occasions for L3 monitoring. |
| Ericsson | Support. During Rel-19 study, it was already agreed in RAN1#120 that UE reporting target CSI and CSI feedback pair is needed to enable performance degradation error cause detection for the two-sided CSI-compression use case. Sufficient monitoring data samples are needed for NW-side performance monitoring and error cause analysis. High layer signaling defined for NW-side training data collection can be reused and easily extended to support this data sample pair based monitoring data collection. |
| CATT | Not support. We agree with Nokia that the usefulness of higher layer reporting for NW-side performance monitoring needs justification. Since both R19 AIBM and AI CSI prediction have specified L1 performance monitoring, we do not see the need for high layer reporting for NW-side performance monitoring. |
| NEC | Agree with OPPO that L3 ony report cannot satisfy the dalay requirement of monitoring. |
| Panasonic | OK |
| Fujitsu | We share similar view as that of Ericsson. |
| LGE | We are not clear with introducing higher layer signaling for NW-side performance monitoring. L1 signaling seems to be sufficient for NW-side performance monitoring. |
| Mod | To DOCOMO, Nokia, OPPO, CATT, NEC, Offinio, LGE: The precision of L1 signaling with legacy eType II codebook cannot accurately monitor the performance – that has been evaluated at R18. If the intention is to introduce SQ/high resolution eType II codebook, how can that satisfy the L1 CSI processing timeline? |
| Lenovo | Thanks FL for the clarification. However, the delay and overhead of L3 layer signalling must also be considered. Although L3 offers more accurate monitoring, its overhead and latency make it a less suitable choice for model monitoring. |

- Issue#3-3 UE side monitoring

Brief summary:

Supporters to UE side monitoring Option 1 (i.e., Case 2-1/2-2)

- Case 2-1 (based on UE side decoder): vivo, Samsung, Lenovo (UE trained decoder under Direction A, or specified decoder), OPPO, NEC, Tejas, TCL, Sharp, Southeast University

  - Spec impact:

    - vivo: Format (e.g., SGCS) of the metric.

      - Reuse the definition of SGCS for CSI prediction

      - How to generate the metric (e.g., SGCS) is purely UE implementation

    - Vivo: P/SP/AP CSI-RS types. SP/AP CSI report types.

    - Samsung: Specify the proxy decoder, or NW to share the proxy decoder

    - NEC: Format (SGCS) of the metric. Event triggered report.

- Case 2-2 (direct estimation of intermediate KPI): Ericsson, Samsung, OPPO, TCL, Sharp, ETRI, TCL

  - Spec impact:

    - Ericsson: Format (e.g., a SGCS range indicator) and reporting signaling (L1) of the metric.

    - Ericsson: RAN4 testing of the quality of UE reported monitoring metric

    - Ericsson: Mechanisms (e.g., RRC-message based methods) to support UE reporting target CSI, encoder output, together with the associated intermediate-KPI based performance metric to the NW

- Other format:

  - NTT DOCOMO, NEC, Rakuten, TCL: UE-side performance metric serves only as a trigger/event, which can trigger NW-side performance monitoring

  - MediaTek: a set of metrics (e.g., SGCS drift, estimated SGCS, distribution shifts, etc.), or occurrence/not occurrence of a monitoring event

- Not support 2-1/2-2: Huawei, HiSilicon, CATT, CMCC

Content of the reporting for Option 1:

- SGCS: ETRI, Ericsson (range indicator), vivo (between Target CSI and recovered Target CSI), Samsung, NEC

- SGCS1 and SGCS2: OPPO (SGCS1 between Target CSI and recovered Target CSI, or estimated SGCS;  SGCS2 for legacy CB)

Supporters to UE side monitoring Option 2 (based on precoded RS): Google, Huawei, HiSilicon (implementation), CATT, Samsung, Apple,

- Option 2-1: Reusing legacy eventual KPI reporting with precoded CSI-RS, e.g., CQI report subject to “cri-RI-CQI”, i.e., no additionaly spec impact. Huawei, HiSilicon, Xiaomi, Honor (if supported)

- Option 2-2: SGCS between vTarget(T0) and vPrecoded(T0) CATT

  - FL assessment: UE needs to perform channel measurement on another non-precoded CSI-RS to obtain HT1, and then extracts vPrecoded(T0)

  - Xiaomi: Such calculation refers to the inverse operation of HT1. However, inverse of HT1 is not exist if HT1 is not a full rank or column full rank matrices

- Option 2-3: Gap of precoding gain between HT0 vTarget(T0)2 and HT0 vPrecoded(T0)2, or HT0 vTarget(T0)2 and HT1 vPrecoded(T0)2. Samsung, Xiaomi (latter solution), Google (latter solution)

  - FL assessment: Former solution: UE needs to perform channel measurement on another non-precoded CSI-RS to obtain HT1, and then extracts vPrecoded(T0), and mutiplies it with HT0. Latter solution: Seems easy, but there is gap between HT0 and HT1, which may cause inaccuracy.

  - Xiaomi: Former solution is similar to Option 2-2. Latter solution (gap of precoding gain HT0 vTarget(T0)2 and HT1 vPrecoded(T0)2) can be used to evaluate model performance if HT0 and HT1 are similar

- Option 2-4: Gap of precoding gain between HT1 vPrecoded(T0)2 and HT1 vPMI2, where a second precoded RS is transmitted at T1 precoded with non-AI PMI/legacy CB of vPMI. Apple, CATT

  - Content: SGCS, RSRP/differential RSRP CATT

  - FL assessment: UE needs to feedback another non-AI based PMI/legacy CB, i.e.,  vPMI, to NW at T0, and perform channel measurement on another precoded CSI-RS precoded by vPMI.

  - Xiaomi: Option 2-4 requires NW to send two precoded CSI-RS to UE.

- Note: T0 is the time for channel measurement for CSI feedback. T1 is the time when precoded RS is transmitted. HT0 is the measured channel at T0. HT1 is the measured channel at T1. vTarget(T0) is the precoder based on Target CSI at T0. vPrecoded(T0) is the precoder based on recovered Target CSI, and applied to the precoded RS.

- Not support: Ericsson, CMCC

  - Argument 1: UE-specific CSI-RS overhead scaled with UE number

  - Argument 2: Additional latency between CSI feedback and precoded CSI-RS

- Other views:

  - Apple: support RLF/BFD like mechanism, where UE transmit fallback request

Signaling and RS type for UE side monitoring

- L1 signaling: Ericsson, vivo, CATT, Lenovo, ETRI

- Support P/SP/AP CSI-RS resource types vivo,

- Support SP/AP CSI reporting types vivo CATT, Lenovo

Linkage between monitoring report and inference CSI report Xiaomi, CATT

- Monitoring configuration should associate with one CSI report configuration for inference which including the paring ID Xiaomi

- Inference CSI-reportConfigId is configured for monitoring, which is analogous to Rel-19 AI BM monitoring report and AI CSI prediction monitoring report CATT

Moderator note: Companies’ views from last meeting:

Support Option 1 (7): Lenovo, OPPO, vivo, ETRI, TCL, Southeast University, Panasonic

- Against Option 1 (4): ZTE, CATT, Apple, Huawei, HiSilicon

Support Option 2 (6): Huawei, HiSilicon, Apple, CATT, Xiaomi, Honor (no spec impact), Panasonic

- Against Option 2: Ericsson

None of Option 1 and Option 2 (6): Qualcomm, MediaTek, CMCC, LGE, Fujitsu, Toyota

It is quite controversial. No change to the old proposal - The plan is to trigger some offline discussion on this issue, and see if we have any other alternative as way forward.

### [Rd1] Proposal 3.3 (Same as RAN1#122b proposal):

For UE side monitoring, study the following two options, and perform down selection to at most one option:

- Option 1 (i.e., Case 2-1/2-2): UE reports estimated SGCS in monitoring CSI report via L1 signaling.

  - How to generate the estimated SGCS is UE implementation from RAN1 perspective.

  - FFS definition of estimated SGCS, e.g., between Target CSI and reconstructed Target CSI

  - FFS format of estimated SGCS, e.g., statistic value, mean value, or range.

  - FFS association of the monitoring CSI report with the inference CSI report

- Option 2 (i.e., monitoring based on precoded RS): UE reports a metric representing the accuracy or precoding gain of the recovery CSI, via L1 signaling. Select one sub-option in below:

  - Option 2-1: Reusing legacy eventual KPI reporting with precoded CSI-RS, e.g., CQI report subject to “cri-RI-CQI”, i.e., no additionaly spec impact.

  - Option 2-2: SGCS between vTarget(T0) and vPrecoded(T0)

  - Option 2-3: Gap of precoding gain between HT0 vTarget(T0)2 and HT0 vPrecoded(T0)2, or HT0 vTarget(T0)2 and HT1 vPrecoded(T0)2.

  - Option 2-4: Gap of precoding gain between HT1 vPrecoded(T0)2 and HT1 vPMI2, where a second precoded RS is transmitted at T1 precoded with non-AI PMI/legacy CB of vPMI.

  - Note: T0 is the time for channel measurement for inference CSI feedback. T1 is the time when precoded RS is transmitted. HT0 is the measured channel at T0. HT1 is the measured channel at T1. vTarget(T0) is the precoder based on Target CSI at T0. vPrecoded(T0) is the precoder based on recovered Target CSI, and applied to the precoded RS.

  - FFS for Option 2-2, 2-3, and 2-4, association of the monitoring CSI report with the inference CSI report.

| Company | View |
| --- | --- |
| Mod | The comments for the last meeting are kept. Please feel free to add/update your comments if any. |
| vivo | We prefer option 1 to report estimated SGCS. |
| Huawei, HiSilicon | We are not supportive of Option-1 which relies on a proxy decoder model at the UE side. In our view, the use of proxy model has two main challenges: - Generalization performance of the proxy model: Since the proxy CSI reconstruction model is different from the actual NW part CSI reconstruction model, e.g., with a smaller size/simpler architecture and weaker learning capability, the generalization performance of the proxy model will be typically worse than the actual NW part CSI reconstruction model. - Additional LCM of the UE side proxy model: Considering the proxy model is used to monitor the performance of actual model, then a subsequent issue is how to monitor and manage the proxy model. Since the proxy model is one-sided model at the UE side, one potential way is that it is up to UE implementation with level x collaboration and, therefore, its performance can hardly be monitored or trusted by the NW side. Option 2 relies on precoded CSI-RS. This option is overhead friendly for UL report, as it may avoid delivery of Target CSI from UE to NW. In addition, it is more reliable than proxy model-based monitoring in option 1. On the other hand, precoded RS based monitoring may reflect the eventual performance, but may not reflect the intermediate KPI (e.g., SGCS) of the model. However, the eventual performance may not only be impacted by the accuracy of the model but also other factors such as scheduling, MU pairing, channel status, etc. Moreover, this method is applicable only when the channel state does not change rapidly. Otherwise, considering there is a time gap between the time slot when the UE feeds back the Target CSI and the time slot when UE calculates the KPI, the accuracy of the calculated KPI may be degraded if the channel state changes significantly in between. We think, compared with proxy model-based monitoring, precoded RS based monitoring may be a more reliable method for UE side monitoring, but it cannot work in a standalone manner without NW side monitoring. Overall, we think RAN1 should first support NW side monitoring and, if necessary, a variation of Option2-based UE side monitoring can be further supported. |
| Apple | We are not supportive of option 1 for the same reasons mentioned by Huawei/HiSilicon. For option 2, instead of reporting a precoding gain, e.g., L1 signaling, UE can trigger an event for NW side performance monitoring. This makes UE side performance monitoring and NW side performance monitoring complementary to each other. |
| Qualcomm | In general, we don’t support UE side monitoring as NW side monitoring with target CSI reporting is needed anyway. Any other operation at UE side is additional complexity without justified benefit. Even if it is supported, we don’t support L1 reporting of the monitoring metric. Let us first make a decision between NW and UE side monitoring, we don’t need two solutions. If NW monitoring is prioritized based on majority, we don’t need to spend time on this topic. |
| Lenovo | We are supporting option-1 as - It is more accurate than NW-side monitoring - It has less overhead and less delay than NW-side monitoring. Regarding the accuracy of the proxy model: Considering Option 4-1 and Option 3a-1 with target CSI, one common method for developing the encoder is to first train the “nominal decoder” and then use this decoder for training the encoder. Assuming correct training, the UE-side (UE) has access to a accurate Encoder and “nominal decoder” after the training phase. Now for monitoring, we can use the developed “nominal decoder” (which we are sure that it is a good model) as the “proxy model. No need for development of other proxy model and, therefore, the issues regarding the accuracy of the proxy model is not valid (we know that the nominal decoder model is a good match with the NW-side decodor model). The rationale for employing a “nominal decoder” as the proxy model addresses concerns about generalization capability and mitigates the need for additional LCM requirements. In summary, if a "nominal decoder" is present on the UE side, it addresses concerns related to accuracy, generalization, and the LCM requirements for UE-side monitoring. |
| OPPO | We share the same view to Lenovo, and support option 1. UE-side monitoring using proxy decoder (Case 2-1) can provide adequate monitoring accuracy and address the concerns on overhead in NW-side monitoring. We can reuse the similar framework to UE-side assisted monitoring in CSI prediction by reporting SGCS. Therefore, the UE-side monitoring Case 2-1/Case 2-2 should be studied and specified |
| MediaTek | Share same view as QC. If the UE-side monitoring should be complemented by NW-side monitoring, the question will be why UE-side monitoring is needed at the first place. |
| CMCC | We don’t support UE side monitoring since the monitoring accuracy of UE sided monitoring based on proxy model at UE side is lower than NW sided monitoring. Besides, if there is an additional proxy model for monitoring specific on top to CSI generation part for inference at UE side, NW/UE may suffer from more burdens on model storage, complexity, power consumption, and management. |
| CATT | We are not supportive for Opt 1 for the same reasons mentioned by HW. We support Opt 2. |
| Ericsson | Ok to further study and down select between option 1 and option 2. We are not supportive of option 2. For Option 2, the NW needs to first use its decoder to generate the reconstructed CSI and then derive a precoding matrix to precode a RS, and then, transmit the RS to the UE. By measuring on the precoded RS, the UE may derive some link-level KPIs like CQI, BLER, etc., but the UE cannot derive intermediate KPI. This method cannot support a UE to obtain accurate information of the model performance, since only the effective channel and not the precoder is visible to the UE, yet it introduces additional latency, RS signalling overhead, and processing complexity at both UE and NW side. In addition, this method does not scale if considering multiple UEs in the network and many monitoring samples are required per UE to derive a monitoring result, since each UE required dedicated precoded CSI-RS which can result in unacceptable CSI-RS overhead. Moreover, the effect of channel aging can impact the accuracy and reliability of the monitoring result. |
| ZTE | We don’t think option 1 can work. The proxy model shows significant generalization performance loss in terms of monitoring accuracy according to TR 38.843, and it may impose huge burden on the network side to additionally identify/monitor/manage the UE-side proxy model. Additionally, if the estimated SGCS is purely UE implementation, the SGCS accuracy across different UEs varies. It cause confusing for the gNB for the interpretation of the reported metric, and false alarming or miss detection may happen. |
| Xiaomi | We share similar view with HW and apple. Considering the issues on accuracy, generation and LCM requirement for proxy model, we are not supportive of Option-1. If the proxy model is trained as normal decoder to mimic the decoder at NW side, it means that the normal decoder must be trained at UE side for inter-vendor collaboration training Option4-1/3a-1 or Option1. However, based on discussion in AI 10.1.2, UE side does not have to train normal decoder. Thus, Option 1 reqires the restriction on inter-vendor collaboration. For Option 2, we are fine to down selection one option. Considering the monitoring performance and calculation complexity, Opiton2-3 or Opiton 2-4 is preferred. |
| InterDigital | Support |
| Panasonic | Our preference is Option 1 and Option 2-1. Monitoring based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW can be used for calculating eventual KPI (L1-RSRP, hypothetical BLER, etc.). Intermediate KPI-based monitoring based on the output of the CSI reconstruction model at the UE (Case 2-1) can be used periodically during normal operation for each check of AI/ML model. |
| LGE | We have similar view with QC that first make a decision between NW and UE-side monitoring. In our view, UE-side monitoring is not needed. Since UE-side monitoring options have many limitations, such as montitoring accuracy, computional complexity, latency. If UE-side monitoring is supported, since precoded RS based monitoring have channel aging and latency issue, we prefer option1. |
| Fujitsu | For Option 1, how to monitor the performance of proxy models should be clarified first. For Option 2, CSI aging is a big challenge for its reliability. Thus, NW side monitoring should be considered as high priority. |
| ETRI | We shares similar view with Lenovo. For intervendor training option 4-1, the nominal decoder could be developed and served as the proxy model for deriving SGCS for UE-side performance moniroting. In addition, for intervendor trianing direction C, UE-side can acquire the actual standardized decoder. |
| HONOR | Since there is no additionaly spec impact, Option 2-1 is preferred. |
| TCL | support the proposal and prefer Option 1. |
| Toyota | No support for UE side monitoring, due to the latency and higher signaling overhead it generates. |
| Southeast University | We prefer option 1. |
| Ofinno | Fine with the proposal. Regarding whether/how to support this L1 reporting, it should be clarified that the conditions under which the SGCS metric reported by the UE is reliable enough to serve as a decision input for LCM. Also, RAN1 must define the SGCS metric consistently to ensure testability across different UE implementations. |
| InterDigital | Support |

- Issue#3-4 Other solutions or issues

### [Rd1] Question 3.4:

For NW/UE side monitoring, any other solutions or issues are missed?

| Company | View |
| --- | --- |

- Model pairing

  - Summary of company proposals

| Company | View |
| --- | --- |
| Apple | Proposal 8: The paring ID is used for NW side data collection for performance monitoring configuration and report. Proposal 9: When the paring ID is used for UE side data collection for training, the pairing ID is configured together with resourcesForChannelMeasurement (e.g., part of CSI-ResourceConfig). Proposal 10: For model pairing procedure for inference, if different paring ID is used for different dataset and NW uses a mixing of datasets for NW side model training with continuous learning: - A pairing ID list is used in inference configuration as part of the applicability reporting procedure; and - UE will report applicable if one of its pairing IDs matches within the pairing ID list. |
| ASUSTeK | Proposal 1: Support both Option A and Option B of Rel-19 applicability report procedure. Proposal 2: If Pairing ID(s) is included in step-3, no need to report pairing ID in step-4. |
| CATT | Proposal 7: For AI/ML based CSI compression Case 0, for model pairing procedure for inference, pairing ID(s) indicated in Step 3 of applicability report is(are) the same as pairing ID(s) in inter-vendor training collaboration. - For Direction A Opt 3a-1, the pairing IDs are received at UE side during model parameter exchange - For Direction A Opt 4-1, the pairing IDs are received at UE side during dataset exchange - For Direction C, the pairing IDs can be generated at the same time when a pair of encoder and decoder reference models is specified Proposal 8: For AI/ML based CSI compression Case 0, inference model pairing in Step 4 of applicability report can be achieved via UE reporting of configuration ID(s) in which the supported pairing ID(s) and inference related parameters/configuration are configured. Proposal 9: For AI/ML based CSI compression Case 0, for model pairing procedure for inference, support both Option A and Option B for applicability report. - For Option A, entire CSI-ReportConfig for inference configuration is configured in Step 3 - For Option B, the following sets of inference related parameters for applicability check only are configured in Step 3: - pairing ID(s) - Subband configuration - Tx port configuration - Payload size configuration Proposal 10: For AI/ML based CSI compression Case 0, regarding the applicability report, a pairing ID can be associated with multiple configurations of subband, Tx port and payload size. Proposal 11: For AI/ML based CSI compression Case 0, do not support configuration of pairing ID during NW-side data collection. Pairing ID can be optionally configured by the NW during UE-side data collection. Proposal 12: For AI/ML based CSI compression Case 0, regarding the pairing procedure, do not support additional configuration of associated ID. Proposal 13: For AI/ML based CSI compression Case 0, it is unnecessary to configure the pairing ID in performance monitoring report. |
| CMCC | Proposal 6: With inter-vendor collaboration Option 4-1 and 3a-1, the exchanged dataset or model parameters may be assigned with Pairing ID(s) during the dataset/model parameters exchange. Proposal 7: In CSI compression using two-sided model use case, the Pairing ID can be optionally indicated in CSI-ReportConfig for inference configuration. Proposal 8: In Step 4 of applicability reporting procedure, UE reports applicability for CSI-ReportConfig and/or set(s) of inference related parameters, and Pairing ID(s) is not needed to report. Proposal 9: For applicability reporting procedure, both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only) can be supported. Proposal 10: Inference related parameters for Option B at least include the following parameters: - Pairing ID - this doesn’t imply the Pairing ID is always present - Configuration of subband - Configuration of Tx port - Configuration of payload size Proposal 11: In CSI compression using two-sided model use case, the Pairing ID is not used for monitoring configuration. |
| Ericsson | - The ID assignment for Direction A, option 3a-1 and 4-1 shall be studied in RAN2 to ensure the uniqueness of the ID across different NW vendors. - In Direction C, a fully standardized reference model is associated with an ID for pairing related discussion, and the pairing ID is specified together with the reference model. - For CSI compression use case, pairing ID (i.e., the ID associated with an exchanged dataset/model-parameters or a fully standardized reference model) is sufficient for ensuring consistency between model training and inference. Do not support associated ID for this use case. - For model pairing procedure for inference, whether to support Option A or Option B or both shall be left for RAN2 discussion and decision. RAN1 provides input on what inference-related parameters shall be included for Option B to assist RAN2 discussions. - For Option B based model pairing procedure, only Pairing ID(s) is/are indicated in Step 3. - A Pairing ID indicated in Step 3 implicitly indicates other inference related configuration parameters that were sent in the dataset exchange for the same pairing ID (e.g., number of Tx ports, number of subbands, and CSI payload size configurations) - For model pairing procedure for inference, for the content of UE reporting in Step 4, support one from the following two options: - Option 1: applicable pairing ID(s) only. - Option 2: applicable pairing ID(s) and applicable (sub)set of other inference related configurations (e.g., number of Tx ports, number of subbands, and CSI payload size configurations) per pairing ID. |
| ETRI | Proposal 7: For AI/ML-based CSI compression, consider both Option A and Option B in the model pairing procedure for AI/ML-based CSI compression. Proposal 8: For AI/ML-based CSI compression, support multiple configurations and payload lengths under a single pairing ID within the model pairing procedure for inference. |
| Fujitsu | Proposal-7: Pairing information exchange between NW and UE is essential for two-sided models, which is indicated by an ID obtained from inter-vendor training collaboration. Proposal-8: The ID is shared from NW to UE along with the exchanged dataset/model parameters, which is an indicator of the NW-side reconstruction model used to generate the exchanged dataset Proposal-9: For model pairing procedure for inference, support both Option A and Option B. The procedure details are left to RAN2 study. Proposal-10: Pairing ID is separately provided from the configurations for TX ports, sub-bands, and payload size. Proposal-11: There is no need to introduce associated ID for CSI compression. |
| FUTUREWEI | Proposal 4: In AI/ML-based CSI compression using two-sided model Case 0, consider reusing procedures agreed in Rel-19 Beam Management use case as much as possible and additionally consider the following: - Indicate Pairing ID in the procedure (applicable for both Option A and Option B in Step 3) - Adopt both Option A and Option B in Step 3 of the procedure |
| Goolge | Proposal 4: Support the network to configure a list of model IDs in a CSI report configuration - Different models can be associated with different payload size - UE reports CSI corresponding to a model ID that fits for the requirement of the maximum payload size to avoid CSI omission |
| Huawei, HiSilicon | Proposal 6: Regarding the model pairing procedure for inference, support both Option A and Option B. Proposal 7: Regarding the model pairing procedure for inference, one Pairing ID can be indicated per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B. - Reuse Rel-19 rule where the applicability state reported in Step 4 is per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B. Proposal 8: Regarding the model pairing procedure for inference/monitoring: - No need to additionally indicate associated ID to UE in Step 3. - No need to additionally indicate Pairing ID for monitoring. |
| InterDigital | Observation 5: The UE-side model management complexity may be high in case of purely numeric IDs that carry no implicit information. Proposal 5: Study mechanism to manage ID for model pairing with reduced complexity. Observation 6: ID structures that convey (implicit) dataset information may help reduce the UE-side model management complexity. Proposal 6: Support ID structures that convey implicit information to enable reducing the UE-side model management complexity. Observation 7: When the ID associated with the exchanged dataset or the model parameters is also used for the performance target in Options 3a-1 and 4-1, the same ID can be used for monitoring configuration. Proposal 7: Support using the Pairing ID for monitoring configuration. |
| Lenovo | Observation 10: In both options 3a-1 and 4-1, linking a pairing ID to the exchanged dataset or model parameters between the UE vendor and NW vendor could inadvertently reveal the vendor identity of either the UE or NW during the inference stage. Proposal 17: Consider specifying procedures or signaling mechanisms that allow the UE/gNB to identify and select the appropriate paired encoder/decoder, without revealing identifying information of the counterpart (i.e., gNB or UE). Proposal 18: Consider that the Pairing ID associated with dataset/model parameters is unique within the coverage area of the node assigning the pairing ID (e.g. an OAM node/function assigning the pairing ID). Proposal 19: Specify required procedures/signalling enabling the node/function in the network responsible for dataset/model parameters exchange to associate a Pairing ID with datasets/model parameters that need to be exchanged between the NW-side and the UE-side. Proposal 20: Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection and can be also configured for monitoring. Proposal 21: Further clarify that the associated ID for the exchanged dataset/model parameters can be used for further NW-side data collection (if needed) but is not needed for the initial data collection for initial construction of the decoder model (and the corresponding dataset/model parameters). Proposal 22: A procedure should be established to specify when and how the UE or UE-side initiates encoder model development for a particular dataset/model-parameters. Additional study is needed to standardize this process and support effective model deployment. Proposal 23: Pairing ID can be used in Step 3 to indicate the exchanged dataset/model parameters/standardized reference model in the inter-vendor collaboration. |
| LGE | Proposal #1: For applicability report of CSI compression, consider both option A and option B. Proposal #2: Regarding the CSI compression specific inference related parameters for Option B, quantization related information (e.g., quantization parameter combination {D, Q, L}, VQ/SQ, layer/rank specific/common) and scalability related information (e.g., sub-bands, TX ports, payload sizes) can be considered |
| MediaTek | Proposal 16. Pairing ID is the ID that is specified together with AI/ML model in Direction C, and it is the ID which is associated to the exchanged parameter or dataset in Direction A. Proposal 17. Support mandatory inclusion of pairing ID for UE-sided data collection and optional inclusion of pairing ID for NW-side data collection. Proposal 18. For model pairing procedure, other than Pairing ID, no need to additionally indicate associated ID. |
| NEC | Model pairing procedure can be different based on the methods used for inter-vendor collaboration. But the ID used for model paring can be the same for different procedures. Proposal 3: Support applicability/inapplicability reporting of inference configuration with the pairing ID and support the inapplicability report to trigger parameter/dataset transfer for model pairing if needed. Proposal 4: In Direction A sub option 4-1, support to study the use of existing/new pairing IDs when adding additional/new data samples. |
| Nokia | Proposal 1: For Direction C, Pairing ID shall be uniquely determined based on the reference models defined in the spec. Observation 1: NW does not need to provide multiple pairing IDs in Step 3 during the applicability determination as the UE will know from UE training entity which pairing IDs provided by the NW are associated to the same encoder. Proposal 2: Reporting pairing ID(s) in Step 4 of the applicability determination procedure is not needed. Observation 2: NW-configured inference configuration (contains pairing ID) determines UE model selection, including applicability reporting. Proposal 3: Support only Option A for applicability determination on CSI compression. Proposal 4: No Pairing ID is needed for configuring NW-side data collection. |
| Ofinno | Proposal#3: Regarding Option B for applicability report procedure, consider quantization codebook information as an inference-related parameter for AI CSI compression. |
| OPPO | Proposal 13: Support Option B only for applicability report for AI/ML CSI compression: - Pairing ID is indicated in the sets of inference related parameters in Step 3 - UE to check whether the pairing ID is pre-exchanged during inter-vendor collaboration and has been associated with an encoder. The applicability of pairing ID is explicitly reported in Step 4 |
| Panasonic | Proposal 6: Pairing ID should be allocated by MNO. Proposal 7: The same ID as pairing ID, i.e., ID associated with exchanged dataset or the model parameters, can be used for applicability inquiry and reporting. Proposal 8: For inference configuration, the same ID as pairing ID, i.e., ID associated with exchanged dataset or the model parameters, can be used. A paring ID can be associated with multiple inference configurations within the range where scalability is ensured. |
| Rakuten Mobile | Observation 2: As per Rel-19 RAN2 agreement on applicability, Step-3 should deliver the pairing context, and Step-4 should report applicability identifiers only, not the pairing ID itself. Proposal 2: RAN1 to consider both options A & B, as gNB selects Option B for a quick applicability check while Option A for full-inference verification before the model pair activation. Collectively study the reporting content that aligns with step-3/step-4 of rel-19 applicability procedure for both options. |
| Samsung | Observation#4: The network may provide the pairing-related ID, e.g., Associated ID, in the data collection configurations to align data categorization between the UE-side and NW-side with respect to the NW-side additional conditions, e.g., deployment scenario (indoor/outdoor TRPs), TxRU mapping, antenna layout, etc. Observation#5: For Direction C, both NW-side and UE-side may train their respective actual models to be compatible with the fully specified reference encoder and decoder, respectively. Alignment on data categorization through the pairing-related ID, e.g., Associated ID, enables training without NW-side and UE-side data distribution mismatch. Observation#6: For Direction A, NW-side may share the pairing-related ID(s), e.g., Associated ID, with reference encoder (Opt. 3a-1) or dataset (Opt. 4-1). UE-side may train its respective actual encoder based on the shared reference encoder/dataset and UE-side data (Target CSI) collected with the same Associated ID(s). Alignment on data categorization through the Associated ID enables training without NW-side and UE-side data distribution mismatch. Proposal#4: Supported the configuration of Associated ID (or named as a pairing ID) in the measurement and reporting configurations for NW-side and UE-side data collection. For the data collection, with the same Associated ID the UE assumes the same data distribution with respect to the NW-side additional conditions, e.g., deployment scenario, antenna layout assumed for corresponding decoder training. Observation#8: For Direction C, fully specified reference model-based approach, there are four different deployment cases - Case 0: Both UE and NW deploy the specified reference encoder and decoder, respectively - Case 1: NW deploys finetuned decoder, UE implements the specified reference encoder. - Case 2, UE deploys finetuned encoder, NW deploys the reference decoder. - Case 3: Both UE and NW deploy finetuned encoder and decoder, respectively Proposal#9: Consider a unified pairing-related ID for Direction A and Direction C with the pairing-related ID consisting of two parts. - Part 1 indicates a specified ID, e.g., specified reference model. - Part 2 indicates network-assigned ID to ensure consistency between training and inference. Both Part 1 and Part 2 or just Part 2 may be associated with exchanged dataset/model parameters in Direction A, or data collection-related configurations in Direction A and Direction C. Observation#9: The implementation dependency between UE-side beam prediction model and NW-proprietary (undisclosed) beam information is analogous to the dependency of UE’s CSI encoder and the NW-proprietary (undisclosed) actual decoder. Thus, the Associated ID framework adopted for UE-side model-based beam prediction can be reused. Proposal#10: For pairing and applicability reporting, reuse the Associated ID framework adopted in UE-side model-based beam prediction using UE-side model - With the same pairing-related ID, e.g., Associated ID, the UE assumes the same decoder. - FFS: additional information, if necessary, for UE-side to train common encoder across cells. - FFS: additional information subject to inter-vendor collaboration options. |
| Southeast University | Proposal 3-1: Specify a two-tier standardized pairing ID system along with ID registration, management, and UE-side applicability reporting mechanisms to enable secure inter-vendor collaboration while addressing hardware resource asymmetry. |
| Spreadtrum, UNISOC | Proposal 7：In step 4,the UE should report the pairing ID received from gNB, and optionally included supported inference configurations when multiple configurations share the same pairing ID. Proposal 8: Support both option A and option B to configure the pairing ID for applicable functionality reporting. Proposal 9：It is necessary to ensure the uniqueness of the pairing ID across different NW vendors. Details is up to RAN2 discussion. Proposal 10：For CSI compression, there is no need to indicate the pairing ID for monitoring procedure. |
| TCL | Observation 3: The paring ID can be exchanged along with the model parameters (Option 3a-1) or dataset (Option 4-1) for model identification. Proposal 7: The paring ID can be used for model paring procedure for inference, UE-side data collection and NW-side data collection. Proposal 8: It is recommended to introduce a unified ID to streamline the ID-related configuration. |
| TOYOTA ITC | Observation 1: If the outcome of the ongoing RAN 4 study - on the specification of a fully standardized AI/ML model - shows considerable performance degradation for AI/ML models at peer entities not being aligned at a certain level/extent, then, the applicability procedure of Rel. 19 AI/ML assisted (UE-sided) beam management might need to be revisited. Proposal 1: RAN 1 to request RAN 4 to provide clear guidance on the level/extent of AI/ML models for CSI compression (at the UE) and for CSI reconstruction (at the gNB) that is required, so as there is no considerable performance loss due to the partial non-alignment. |
| vivo | Proposal 1: Study whether a dataset (direction 4-1), model parameters (direction 3a-1) and direction C can be associated with the same pairing ID Proposal 3: For applicability check, option B should be supported for checking supported pairing ID, and the following inference related parameters are provided along with pairing ID. • Payload size • Subband number and/or port number |
| Xiaomi | Proposal 1: Both option A (CSI-ReportConfig) and option B (sets of inference related parameters for applicability report only) are adopted for step 3 of applicability report. Proposal 2: For inter-vendor collaboration direction A, the paring ID is exchanged together with the dataset and/or model parameters, the paring ID is linked with the model generated by the exchanged dataset and/or model parameters. Proposal 3: For inter-vendor collaboration direction C, the paring ID including the model ID and associated ID, where the model ID part is used to indicate the fully standardized reference model, and the associated ID is used to align the dataset feature for ensuring the consistency. Proposal 4: The parings ID(s) included in the applicable CSI-reportConfig ID(s) is reported in Step4. |
| ZTE, Sanechips | Proposal 1: Spec enhancements of other aspects should be compatible to all to be specified inter vendor training collaboration options (i.e., Direction A options 3a-1 and 4-1, Direction C) including the associated scalability design. Proposal 2: Spec enhancements specified for AI/ML beam management and CSI prediction use cases should be reused for AI/ML CSI feedback to reduce spec efforts and simplify the spec design, when applicable. Proposal 3: To enable the UE to select an encoder compatible with the decoder used by the gNB, pairing ID shall be defined which may be in the form of specified ID, dataset ID, or model ID, depending on the inter-vendor training collaboration options. Proposal 4: For the applicability report for AI/ML CSI feedback use case, reuse the applicability reporting procedures defined for AI/ML beam management as a starting point. Proposal 5: For the applicability report for AI/ML CSI feedback, support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only). Proposal 6: The CSI feedback specific inference configuration or inference related parameters provided to the UE shall at least include configurations of subband, Tx port and payload size. |

  - Discussions

- Issue#4-1 Model pairing procedure

Brief summary:

Support of Option A and Option B

- Support both: Fujitsu, Futurewei, ASUSTeK, Huawei, HiSilicon, ZTE, Xiaomi, CATT, CMCC, LGE, ETRI, Spreadtrum, Offino, Rakuten

- Option B only: vivo, OPPO

  - Argument: only AI model related parameters are configured in Step3

  - Counter-argument: new RRC parameters need to be defined for applicability report only, and the CSI report configuration for inference still needed

- Option A only: Nokia

  - Argument: applicability reporting can already be performed based on the full inference configuration, and following the path of Option B seems unnecessary for CSI compression Nokia

  - Counter-argument: vivo, Huawei, HiSilicon, ZTE

    - vivo: 1) it is challenge to indicate the pairing ID directly in CSI-ReportConfig if NW side has multiple models and does not know which one the UE supports 2) Subset of preferred configuraitons can be replied to NW for checking, rather than configure multiple CSI-ReportConfig for applicability checking

    - Huawei, HiSilicon: For Option A, 1) if gNB cannot confirm whether the UE can support a CSI-reportConfig in the first place but blindly configure with the CSI-reportConfig, it may suffer the consequence of UE reporting ‘inapplicable’. 2) If the UE replies with ‘inapplicable’, then it means the gNB cannot immediately receive the CSI report, and it needs to blindly try another CSI-reportConfig, causing longer CSI report latency.

    - ZTE: since there is no priori information on UE-side applicability status, many of the inference configurations provided in Step 3 may become useless and result in a waste of air interface resources.

- Up to RAN2: Ericsson

  - Ericsson: RAN1 provides input on what inference-related parameters shall be included for Option B

How to indicate Paring ID(s) in Step 3

- For Option A:

  - one Pairing ID can be indicated per CSI-ReportConfigId: Huawei, HiSilicon, OPPO, Futurewei

- For Option B:

  - Alt.1: One Pairing ID can be indicated per ApplicabilitySetConfigId: Huawei, HiSilicon,

  - Alt.1’: One Pairing ID can be indicated within the set of inference related parameters: OPPO

  - Alt.2: NW directly indicates one or multiple pairing ID(s) Ericsson

    - Ericsson: each pairing ID already implicitly indicates the sets of other inference related parameters

Whether/how Paring ID(s) are reported in Step 4

- For Option A:

  - No need for explicit report, since applicability state reported in Step 4 is per CSI-ReportConfigId which includes all inference configurations: ASUSTeK, CMCC, Huawei, HiSilicon, Nokia, ZTE, Xiaomi, CATT

  - UE reports applicable Pairing ID Fujitsu

- For Option B:

  - Alt.1: No need for explicit report, since applicability state reported in Step 4 is per set of inference parameters (ApplicabilitySetConfigId): ASUSTeK, CMCC, Huawei, HiSilicon, ZTE, CATT,

  - Alt.2: Only report applicable Pairing ID(s) in Step 4: Ericsson, OPPO

  - Alt.3: Reports applicable Pairing ID(s) as well as (sub)set of other inference related configurations, if more than one set of configurations is represented by a Pairing ID  Ericsson

CSI compression specific inference configuration for Option A:

- Antenna port geometry: N1 (number of array rows), N2 (number of array columns) Nokia, Xiaomi

- Bandwidth configuration: number of subbands and location of subbands (i.e., contiguous or some other pattern) Nokia, Xiaomi

- Latent vector dimension Nokia

- Quantization scheme Nokia

- Carrier frequency Nokia

- CSI payload size Xiaomi

CSI compression specific inference related parameters for Option B:

- Payload size config: Fujitsu, vivo, ZTE, CATT (d, Q, L), CMCC, LGE (d, Q, L), Offino

- Quantization information: LGE (VQ/SQ, layer/rank specific/common), Offino

- Subband config: Fujitsu, vivo, ZTE, CATT, CMCC, LGE, Offino

- Tx port config: Fujitsu, vivo, ZTE, CATT, CMCC, LGE, Offino

Moderator note:  13 companies support both Option A and Option B.

Applicability report procedure for R19 AI BM in latest 38.331 (R2-2506530) Step 3-Option A: CSI-reportConfig => CSI-ReportConfigId … Step 3-Option B: OtherConfig-v19xy => ApplicabilityReportConfig-r19 => applicabilitySetConfigId-r19 … Step 4: Option A and Option B: RRCReconfigurationComplete or UAI => ApplicabilityReportList. In ApplicabilityReportList => ApplicabilityInfoReport, UE reports the applicability status per CSI-ReportConfigId (for Option A), and per ApplicabilitySetConfigId (for Option B).

Regarding whether to support Option A and B, to Moderator’s understanding, Option A and Option B may be applicable to different scenarios: for initial applicability check (e.g., a UE newly camps on the cell), Option B is better as it can avoid gNB blindly configuring CSI-reportConfig. After UE feedback applicability via UAI or RRCReconfigurationComplete and gNB is somehow aware of the applicability at UE, Option A may be applicable.

From the comments of last meeting, detailed signaling for Option A/B can be decided by RAN2. Thus, RAN1 can confirm both options are workable, identify inference related parameters of Option B, and add one bullet that signaling is up to RAN2.

### [Rd1] Proposal 4.1:

For model pairing procedure for inference, from RAN1 perspective, Option A and Option B are both feasible.

- Inference related parameters for Option B may include at least: payload size related information (e.g., d, Q, L), subband configuration, and Tx port configuration.

  - FFS quantization codebook.

- Detailed RRC signaling is up to RAN2.

| Company | View |
| --- | --- |
| QC | Prefer to deferring this proposal till the inference configuration is clear. |
| vivo | At least, the main bullet can be supported |
| Futurewei | We are ok with vivo’s comment on agreeing the main text at this time. |
| Huawei, HiSilicon | We are supportive of both Option A and Option B. It is probably better to change “feasible” to “supported” in the main bullet since “feasible” does not mandate specifying either of Option A or B in RAN2 or agreeing on further details of necessary inference related parameters in RAN1. Regarding FFS, it seems that if the quantization codebook is exchanged during inter-vendor collaboration, it needs to be also provided in Step 3 of applicability process as UE may not support it. |
| ZTE | OK in general. It seems pairing ID is missed in the inference related parameters. |
| CMCC | Inference related parameters for Option B can include Pairing ID. |
| Nokia | For option B, subband configuration should include also subbandSize, bwp-info as different subbandSize and bwp-info parameters may correspond to the same subband configuration. For tx port configuration, for ports>32, the number of aggregated resources and the port mapping method should also be included |
| TCL | OK. |
| Xiaomi | Support Option A and Option B. |
| MediaTek | Support both options |
| Ofinno | Support |
| Ericsson | Agree with ZTE and CMCC that pairing ID is missing. For model pairing procedure for inference, whether to support Option A or Option B or both shall be left for RAN2 discussion and decision. RAN1 can provide input on what inference-related parameters shall be included for Option B to assist RAN2 discussions. |
| CATT | OK in general, we are also supportive for both Option A and Option B. |
| ETRI | Agree |
| Fujitsu | OK in general. |
| LGE | Support |
| Lenovo | We are okay with the mian text, we suggest to remove the bullets for now to further progress on what parts of the payload infomraoitn is embeded in the Pairing-ID and what parts needs to be further configured. |

- Issue#4-2 Pairing ID for LCM procedures

Brief summary:

Definition of Pairing ID:

- In the form of specified ID, dataset ID, or model ID, depending on the inter-vendor training collaboration options: ZTE

- Samsung: Part 1 indicates a specified ID, e.g., specified reference model. Part 2 indicates network-assigned ID

Inclusion of Pairing ID in LCM procedures:

- NW side data collection:

  - Support: ZTE, ETRI, MediaTek (optional), Samsung, Apple, TCL, ETRI, Panasonic (optional)

  - Not support: CMCC, Ericsson, Nokia, CATT, OPPO (not needed for serving cell), Lenovo (not needed for initial data collection)

    - Argument 1: UE logs and reports the target CSI samples together with the associated measurement configuration ID and the cell ID, which is sufficient for NW to categorize data by implementation Ericsson, Nokia

    - Argument 2: NW side can manage the dataset categorization and aggregation in an implementation specific manner CATT

- UE side data collection:

  - Support: MediaTek (mandatory), NTT DOCOMO, Lenovo, CATT (optional), Huawei, HiSilicon (optional), Qualcomm, Ericsson (optional), vivo, Samsung, ZTE, Apple, Xiaomi, CMCC (optional), OPPO, Futurewei, Tejas, TCL, Panasonic (optional), ETRI, Spreadtrum

    - Argument 1: If the same Pairing ID is associated with an exchanged model#A/dataset#A (with certain NW side additional condition) during inter-vendor training collaboration, UE side may categorize the newly collected data into the dataset for model#A/dataset#A Huawei, HiSilicon, Qualcomm, Samsung

    - Argument 2: as Pairing ID is anyway indicated to UE for model pairing purpose, it would not cause any harm if it is additionally indicated for UE side data collection purpose Huawei, HiSilicon

    - Argument 3: For direction 3A-1 and direction C, it is possible that no Target CSI is shared, then UE side must collect data by itself vivo

  - vivo: how to configure the pairing ID, especially when two models exist under the same scenarios or deployments

  - CATT: Pairing ID can be optionally configured by the NW during UE-side data collection

  - Apple: A pairing ID list is used in inference configuration as part of the applicability reporting procedure. Every time the NW fine-tuned a decoder that is backward compatible with a deployed encoder, the NW added a new Pairing ID into the list (without removing backward compatible Pairing IDs). UE will report applicable if one of its pairing IDs matches within the pairing ID list.

- Monitoring:

  - Support: InterDigital, Lenovo, Tejas, InterDigital, Xiaomi

    - Xiaomi in the configuration of higher layer reporting explicitly or indicated implicitly via the linkage with CSI report configuration for inference

  - Not support: Huawei, HiSilicon, CATT, CMCC, Spreadtrum

Need of additional associated ID in LCM procedures:

- Needed: Xiaomi

- Not needed: MediaTek, Huawei, HiSilicon, Ericsson, CATT, Fujitsu

Moderator note: On top of the FL proposal of last meeting, “Optional” is removed. Whether the provision is mandatory or optional can be further discussed.

### [Rd1] (Closed) Proposal 4.2 (Inclusion of Pairing ID in LCM procedures):

Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS whether to configure Pairing ID for NW side data collection

- No need to configure Pairing ID(s) for monitoring

### [Rd3] Proposal 4.2a (Inclusion of Pairing ID in LCM procedures):

Support providing Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS mandatory or optional

- FFS whether to configure Pairing ID for NW side data collection

- No need to configure Pairing ID(s) for monitoring

| Company | View |
| --- | --- |
| QC | To us, “optional” = “can be”. We think it should be mandatory. Reason as follows. (copy-paste our comment in the last meeting). We think it is mandatory, not optional. Otherwise, the UE has no idea how to use the collected data for model training. It is worth noting that the dataset exchanged in training collaboration is used to establish a private decoder which mimics the actual decoder employed by the NW, then the data collected by the UE data collection is used to train the actual encoder against the established private decoder. In this procedure, UE needs to be aware of which exchanged dataset the collected UE side data is associated with. The above training procedure is more meaningful for new UE devices/vendors where their data has not yet collected by the NW. For these new UEs, if pairing ID assigned to UE data collection, they would be able to train their model against an existing pairing ID, without necessarily going through a new round of NW side data collection and dataset exchanged with a new pairing ID. |
| vivo | Unlike the associated ID, which is optional, the pairing ID is configured as mandatory for inference. Providing pairing ID as a mandatory for UE-side data collection is beneficial and feasible. |
| Samsung | Same view as QC and vivo. |
| Futurewei | We also think pairing ID is needed for UE-side data collection. |
| Huawei, HiSilicon | Support. For the FFS, we don’t see a strong reason to configure Pairing ID for NW side data collection. To our understanding, UE just needs to report the Target CSI following the provided configuration and does not need to know the reported Target CSI is associated with which Pairing ID. |
| ZTE | Ok. Per our understanding, the same logic applies to pairing ID and associated ID. If the ID is absence, the UE may use some implementation methods to classify the different NW-side additional conditions. Also, the UE may aggregate the collected data into the previous model, and then train a generalize model with mixed datasets. |
| CMCC | Similar as associated ID, we support pairing ID as optional. Without pairing ID, UE may assume collected data with similar NW-side additional conditions and train a generalize model. |
| Xiaomi | Agree with above comments that pairing ID is mandatory. Regarding Pairing ID(s) for monitoring, unlike Rel-19, NW-side monitoring may be in L3 signaling. The pairing ID may be useful for UE to determine which model for CSI feedback generation. |
| MediaTek | For the UE-side data collection, pairing ID can be mandatory as it always associated to a model/exchanged data set which has already a pairing ID. |
| Ericsson | Support. Regarding FFS, we think that Pairing ID is not needed for configuring NW-side data collection. Similar to NW-side data collection for BM use case, for CSI compression use case, the UE shall log at least cell ID, measurement configuration ID and target CSI samples per CSI logged measurement configuration per cell, then, based on the collected dataset, NW can categorize the target CSI samples by its implementation. |
| CATT | We are NOT OK to remove optionally. Even in R19 AI BM, the associated ID for UE side data collection is optionally configured; we can only accept that pairing ID can be optionally configured for UE side data collection. For the first FFS, we also don’t see the reason to configure Pairing ID for NW side data collection. |
| ETRI | Support |
| InterDigital | We support the use of pairing ID for performance monitoring configuration. When the ID associated with the exchanged dataset or the model parameters is also used for the performance target in Options 3a-1 and 4-1, the same ID can be used for monitoring configuration. |
| Panasonic | For the case UE-side data collection is non-OTA approach, the relationship between “what is shared from the NW-side to the UE-side and the related configuration parameters” and “the configuration of data collection for UE-side training” are known by NW-side, and then, it would not be necessarily required to use paring ID itself. Since the necessity of paring ID depends on whether UE-side data collection is based on OTA based or non-OTA based, paring ID configuration can be optional. |
| LGE | Support. For NW-side data collection, in order to provide a pairing ID for NW-side additional conditions, further discussion seems to be needed on which additional condition can be inferred from pairing ID. |
| Lenovo | We suggest to remove the last bullet until further discussion on Monitoring scheme: Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection - FFS whether to configure Pairing ID for NW side data collection - No need to configure Pairing ID(s) for monitoring |

Moderator note:  Same proposal as last meeting.

### [Rd1] Proposed conclusion 4.3 (Same as RAN1#122b proposal):

For model pairing procedure, other than Pairing ID, no need to additionally indicate associated ID in Step 3 for representing NW side additional conditions

| Company | View |
| --- | --- |
| Mod | The comments for the last meeting are kept. Please feel free to add/update your comments if any. |
| vivo | We can postpone the discussion as the definition of associated ID remains unclear, and further discuss the detailed pairing ID for all the inter vendor collaboration direction and finetuning. |
| Huawei, HiSilicon | Support. Pairing ID represents NW side additional condition as well and there is no need for an additional ID. |
| NTT DOCOMO | Support. |
| Samsung | As long as we agree on the following, we are fine on terminologies. - For Direction C if applicable, the corresponding Pairing-related ID may consist of a specified as a fixed value and network assigned ID for data collection. |
| Futurewei | We support this proposal, no need to indicate associated ID for pairing purpose. |
| MediaTek | Support. Associate ID is a redundant concept here in presence of pairining ID |
| CMCC | Support. |
| CATT | support |
| Ericsson | Support |
| ZTE | Support. |
| Xiaomi | We think the associated ID is needed at least for Direction C considering model finetuning. |
| Spreadtrum | Support，pairing ID is enough for model pairing procedure. |
| Panasonic | Support |
| LGE | Support. Associated ID is not needed in step 3 for representing NW-side additional conditions, since Pairing ID which can represent NW-side additional condition is already in step 3. |
| Fujitsu | Support |
| ETRI | In our understanding, associated ID is not cleared defined yet. So we don’t need this proposal at this stage, in our view. |
| HONOR | Support. |
| TCL | support |
| Toyota | Support |
| Huawei, HiSilicon2 | We think there is no need to further discuss the indication of associate ID following the conclusion in RAN1 122b: Conclusion: For Option 4-1 under Direction A in AI/ML based CSI compression, there is no consensus to further study the necessity of sharing assisted information to align model design aspects. |
| Nokia | Support |
| Xiaomi | Update in RAN1#123. Pairing ID has the similar function as associated ID for checking the consistency. However, we think pairing ID should further include model ID (e.g., model ID for the reference model defined in Direction C) when both Direction C and Direction A are utilized simultaneously. In this case, the paring ID including the model ID and associated ID, where the model ID part is used to indicate the fully standardized reference model, and the associated ID is used to align the dataset feature for ensuring the consistency. |
| Ofinno | Same understanding with Huawei |
| Ericsson | Support |

- Issue#4-3 Other solutions or issues

### [Rd1] Question 4.4:

For model pairing, any other solutions or issues are missed?

| Company | View |
| --- | --- |

- [Place holder] Other issues

- Potential proposals for GTW/offline

  - Tuesday offline

### Proposal 2.1 (Option selection):

Working assumption

For NW side data collection with higher layer reporting, regarding Target CSI quantization, support both Option 1 (Scalar quantization) and Option 4 (eType II like codebook with new parameter) based on UE capability.

Alt.1: Working assumption

Alt.2: Agreement

For NW side data collection with higher layer reporting, regarding Target CSI quantization, consider Option 1 (Scalar quantization) as way forward

Strive to ensure the quantization resolution (e.g., with appropriate k1, k2 values) under Option 1.

Option 1 only: MTK, Offinio, LGE, OPPO, Xiaomi, Docomo, Huawei, KT, Nokia, QC, Apple, ETRI, Lenovo, Ericsson (if RRC msg issue resolved), Futurewei, vivo

Samsung: should first determine the k1/k2 values and quantization method.

Option 4 only: Ericsson, Docomo, Fujitsu

Both: CATT, ZTE, OPPO, Huawei, ETRI, Lenovo, Fujitsu

Moderator note: there are comparable number of companies supporting Option 1 and Option 4. - Option 1: 18 supporters - LGE, InterDigital, CATT, Huawei, HiSilicon, Honor, Qualcomm, vivo, Samsung, Nokia, Apple, OPPO, Xiaomi, [NTT DOCOMO], MediaTek, KT, Sharp (down select between Opt 1 and Opt 4), [TCL], [Toyota] - Option 4: 11 supporters InterDigital, Lenovo, Huawei, HiSilicon, [Qualcomm?], Ericsson, [Nokia], ZTE, CMCC, NEC, Fujitsu, Sharp (down select between Opt 1 and Opt 4)

### Proposal 2.2a (Option 1 details):

For NW side data collection with higher layer reporting, for Option 1 (scalar quantzation), down select between the following:

- Option 1-1: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Support k1=k2=3, 4, 6, and 8, based on configuration

    - Note: k1=k2=4, 6, and 8 are at least applicable to some certain configurations of Tx ports/subband number.

- Option 1-2: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Support {k1, k2} = {3,4}, {4,3}, {4,4}, and {8,8}, based on configuration

    - Note: {k1, k2} = {4,4}, and {8,8} are at least applicable to some certain configurations of Tx ports/subband number.

- FFS whether/how to perform normalization method, e.g., down select among the alternatives:

  - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, and [0, 1]/(−π, π] for amplitude/phase quantization. No need to specify normalization.

  - Alt.2: The ranges of T1min, T1max, T2min, and/or T2max [T1min, T1max] and [T2min, T2max] are reported by UE. No need to specify normalization.

  - Alt.3: Normalization is performed for each 1*2N1N2 vector individually per layer, per subband, per sample, and per real/imag part. UE additionally reports the original max and min values which based on which the normalization is performed.

  - Tuesday online

### Proposal 2.2b (Option 1 details):

Alt.1: Working assumption

Alt.2: Agreement

For NW side data collection with higher layer reporting, regarding Target CSI quantization, consider Option 1 (Scalar quantization) as way forward

For Madam Chair’s information

Option 1 only: MTK, Offinio, LGE, OPPO, Xiaomi, Docomo, Huawei, KT, Nokia, QC, Apple, ETRI, Lenovo, Ericsson (if RRC msg issue resolved), Futurewei, vivo Samsung: should first determine the k1/k2 values and quantization method. Option 4 only: Ericsson, Docomo, Fujitsu Both: CATT, ZTE, OPPO, Huawei, ETRI, Lenovo, Fujitsu

### Proposal 2.3a (Overhead for Option 1):

For NW side data collection with higher layer reporting, regarding Option 1 (scalar quantzation), consider the following solutions to limit the overhead of per RRC message within 9KBytes (max payload size per RRC message) for up to 128 ports

- Alt.1: Using lower quantization resolution per floating element, e.g., using k1+k2=7 bits or 6 bits.

- Alt.2: Report one Target CSI sample via multiple RRC messages. E.g., carry Target CSI of partial ports/ subbands/ layers (for precoding matrix) within one RRC message.

- Send LS to RAN2 to check the feasibility of Alt.2 inform the solution for confirmation.

### Proposal 2.2b (Option 1 details):

For NW side data collection with higher layer reporting, for Option 1 (scalar quantzation), down select between the following:

- Option 1-1: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Support at least k1=k2=3, 4, 6, and 8, based on configuration

    - Note: k1=k2=4, 6, and 8 are at least applicable to some certain configurations of Tx ports/subband number.

- Option 1-2: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Support at least {k1, k2} = {3,4}, {4,3}, {4,4}, and {8,8}, based on configuration

    - Note: {k1, k2} = {4,4}, and {8,8} are at least applicable to some certain configurations of Tx ports/subband number.

- FFS whether/how to perform normalization method, e.g., down select among the alternatives:

  - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, and [0, 1]/(−π, π] for amplitude/phase quantization. No need to specify normalization.

  - Alt.2: The ranges of T1min, T1max, T2min, and/or T2max [T1min, T1max] and [T2min, T2max] are reported by UE. No need to specify normalization.

  - Alt.3: Normalization is performed for each 1*2N1N2 vector individually per layer, per subband, per sample, and per real/imag part. UE additionally reports the original max and min values which based on which the normalization is performed.

  - Thursday offline

### [Rd3] Proposal 2.2b (Option 1 details):

For for Option 1 (scalar quantzation) of NW side data collection with higher layer reporting, down select between Option 1a and Option 1b:

- Option 1a: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Further study at least k1=k2=3, 4, 5, 6, and 8 to justify the performance and necessity of RRC signaling enhancement (e.g., by carrying one Target CSI sample with multiple RRC messages) for configurations of k1+k2>=8.

  - Further study the quantized value of each codepoint.

- Option 1b: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Further study at least k1+k2=6, 7, 8, 12, and 16 with k1>k2, k1<k2 or k1=k2 to justify the performance (e.g., by carrying one Target CSI sample with multiple RRC messages) for configurations of k1+k2>=8.

    - Consider Metric#1 (SGCS for a trained model) for the evaluation

  - Further study the quantized value of each codepoint.

- Further study the potential UE complexity between Option 1a and Option 1b, e.g., considering the potential conversion to amplitude/phase domain of Option 1b.

- Consider Metric#1 (SGCS for a trained model) for the evaluation

- The study considers 32 ports, 64 ports, and 128 ports.

- Further study mixded k1,k2 granularities for different port/subband/layer configurations for training scalable model.

- Study the necessity of RRC signaling enhancement, e.g., for a large k1, k2 pair if needed.

Further study whether/how to perform normalization for Option 1a and Option 1b, e.g., select one among the alternatives:

  - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, and [0, 1]/(−π, π] for amplitude/phase quantization. UE may perform scale up/down the fit the fixed range.

    - FFS whether/how to ensure the UE proprietary generated Target CSI fits the specified quantization range

  - Alt.2: The ranges of T1min, T1max, T2min, and/or T2max are reported by UE. UE may not perform scale up/down the fit the fixed range.

  - Alt.3: Normalization is performed for each 1*2N1N2 vector.

- Note: Payload size of a single Target CSI for

  - k1+k2<=7 is smaller than 9Kbytes for all configurations of Tx ports/subband/layer number.

  - k1+k2=8 is smaller than 9Kbytes for all configurations of subband/layer number with 2N1N2<=64.

  - k1+k2=16 is smaller than 9Kbytes for all configurations of subband/layer number with 2N1N2<=32.

For for Option 1 (scalar quantzation) of NW side data collection with higher layer reporting, down select between Option 1a and Option 1b:

- Option 1a: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Further study at least k1=k2=3, 4, 5, 6, and 8 to ensure the performance (e.g., the minimum value that achieves the close performance to upperbound).

  - Further study the quantized value of each codepoint.

- Option 1b: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Further study at least k1+k2=6, 7, 8, 12, and 16 with k1>k2, k1<k2 or k1=k2 to ensure the performance (e.g., the minimum value that achieves the close performance to upperbound).

  - Further study the quantized value of each codepoint.

- Further study the potential UE complexity between Option 1a and Option 1b, e.g., considering the potential conversion to amplitude/phase domain of Option 1b.

- Consider Metric#1 (SGCS for a trained model) for the evaluation.

- The study considers 32 ports, 64 ports, and 128 ports; 19 subbands; rank 4.

- [It is up to companies to study mixed Target CSIs subject to multiple {k1, k2} values each corresponding to a specific port/subband/layer configuration, for training scalable model.]

- Study the necessity of RRC signaling enhancement, e.g., for a large k1, k2 pair if needed.

Further study whether/how to perform normalization for Option 1a and Option 1b, e.g., select one among the alternatives:

  - Alt.1: The ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, and [0, 1]/(−π, π] for amplitude/phase quantization. UE may perform scale up/down to fit the fixed range.

    - FFS whether/how to ensure the UE proprietary generated Target CSI fits the specified quantization range

  - Alt.2: On top of Alt.1, additional specification on the quantization, e.g., at least one element among all real and imag elements in the Target CSI matrix (FFS per layer and/or per subband) correspond to any of Tmin (assuming T1min = T2min) and Tmax (assuming T1max = T2max) in the quantization table, or reporting scaling factor and offset factor.

Example of the LUT for real/imag quantization

| Codepoint | Codeword |
| --- | --- |
| 00 | -0.75 |
| 01 | -0.25 |
| 10 | 0.25 |
| 11 | 0.75 |

  - Alt.2: The ranges of T1min, T1max, T2min, and/or T2max are reported by UE. UE may not perform scale up/down the fit the fixed range.

  - Alt.3: Normalization is performed for each 1*2N1N2 vector.

- Note: Payload size of a single Target CSI for

  - k1+k2<=7 is smaller than 9Kbytes for all configurations of Tx ports/subband/layer number.

  - k1+k2=8 is smaller than 9Kbytes for all configurations of subband/layer number with 2N1N2<=64.

  - k1+k2=16 is smaller than 9Kbytes for all configurations of subband/layer number with 2N1N2<=32.

Huawei results (240bits payload, 32 ports, normalized to [0, 1] for amp or [-1, 1] for real/imag) w/o normalization, Metric#2

Nokia results (Metric#2? 32-port?)

vivo results (up to 256 bits payload. 32 ports? Normalized to [0, 1] for amp or [-1, 1] for real/imag)

Ericsson results (Metric#2 only? Normalized to [-0.33, 0.33] for the 32-port case and [-0.16, 0.16] for the 128-port)

Apple results (up to 192 bits. Normalized to [-0.33, 0.33] for the 32-port case

### [Rd3] Proposal 4.2a (Inclusion of Pairing ID in LCM procedures):

Support providing Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS mandatory or optional

- FFS whether to configure Pairing ID for NW side data collection

### [Rd1] Proposal 4.1:

For model pairing procedure for inference, from RAN1 perspective, Option A and Option B are both feasible.

- Inference related parameters for Option B may include at least: payload size related information (e.g., d, Q, L), subband configuration, and Tx port configuration.

  - FFS quantization codebook.

- Detailed RRC signaling is up to RAN2.

### [Rd1] Proposal 3.2 (Higher layer signaling):

For NW side monitoring, support the higher layer signaling for reporting.

- Regarding the reporting content, at least consider paired Target CSI and CSI feedback

  - FFS: CSI feedback is the one before or after UCI omission.

- Quantization format of Target CSI is the same as for NW side data collection.

  - Thursday online

### Proposal 2.2c (Option 1 details):

For for Option 1 (scalar quantzation) of NW side data collection with higher layer reporting, down select between Option 1a and Option 1b:

- Option 1a: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Further study at least k1=k2=3, 4, 5, 6, and 8 to ensure the performance (e.g., the minimum value that achieves the close performance to upperbound).

  - Further study the quantized value of each codepoint.

- Option 1b: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Further study at least k1+k2=6, 7, 8, 12, and 16 with k1>k2, k1<k2 or k1=k2 to ensure the performance (e.g., the minimum value that achieves the close performance to upperbound).

  - Further study the quantized value of each codepoint.

- Further study the potential UE complexity between Option 1a and Option 1b, e.g., considering the potential conversion to amplitude/phase domain of Option 1b.

- Consider Metric#1 (SGCS for a trained model) for the evaluation.

- The study at least considers 32 ports, 64 ports, and 128 ports; 19 subbands; rank 4.

- It is up to companies to study mixed Target CSIs subject to multiple {k1, k2} values each corresponding to a specific port/subband/layer configuration, for training scalable model.

- Study the necessity of RRC signaling enhancement, e.g., for a large k1, k2 pair if needed.

- Further study whether/how to perform normalization for Option 1a and Option 1b, e.g., the ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, or [0, 1]/(−π, π] for amplitude/phase quantization. Select one among the alternatives:

  - Alt.1: It is UE implementation to perform scale up/down to fit the fixed range.

  - Alt.2: Additional specification on the quantization, e.g.,

    - Alt.2-1: at least one element among all real and imag elements in the Target CSI matrix (FFS per layer and/or per subband) correspond to any of Tmin (where Tmin=T1min=T2min) and Tmax (where Tmax=T1max=T2max) in the quantization table or at least one element among all amplitude elements in the Target CSI matrix (FFS per layer and/or per subband) correspond to any of T1min and T1max

    - Alt.2-2: reporting scaling factor and offset factor used for denormalization.

// For reference. Not part of the proposal.

Example of the LUT for real/imag quantization

| Codepoint (k1=k2=2 bits) | Codeword |
| --- | --- |
| 00 | -0.75 |
| 01 | -0.25 |
| 10 | 0.25 |
| 11 | 0.75 |

Example of the scaling factor and offset factor (y is quantized value; x is original value; xmin is the offset factor; xmax-xmin is the scaling factor)

y=x-xminxmax-xmin

- Note: Payload size of a single Target CSI for

  - k1+k2<=7 is smaller than 9Kbytes for all configurations of Tx ports/subband/layer number.

  - k1+k2=8 is smaller than 9Kbytes for all configurations of subband/layer number with 2N1N2<=64.

  - k1+k2=16 is smaller than 9Kbytes for all configurations of subband/layer number with 2N1N2<=32.

### Proposal 4.2a (Inclusion of Pairing ID in LCM procedures):

Support providing Pairing ID(s) in CSI-ReportConfig for UE side data collection

- FFS mandatory or optional

- FFS whether to configure Pairing ID for NW side data collection

### Proposal 3.2 (Higher layer signaling):

For NW side monitoring, support the higher layer signaling for reporting.

- Regarding the reporting content, at least consider paired Target CSI and CSI feedback

  - FFS: CSI feedback is the one before or after UCI omission.

- Quantization format of Target CSI is the same as for NW side data collection.

- Agreements

  - Agreements for #123

Agreement

For NW side data collection with higher layer reporting, regarding Target CSI quantization, Option 1 (Scalar quantization) is supported.

Note: Supporting target CSI payload size greater than single RRC message may need RAN2 justification.

Agreement:

For Option 1 (scalar quantization) of NW side data collection with higher layer reporting, down select between Option 1a and Option 1b:

- Option 1a: Each complex element of the Target CSI sample is quantized to real value and imaginary value

  - Further study at least k1=k2=3, 4, 5, 6, and 8 to ensure the performance (e.g., the minimum value that achieves the close performance to upperbound).

  - Further study the quantized value of each codepoint.

- Option 1b: Each complex element of the Target CSI sample is quantized to amplitude value and phase value

  - Further study at least k1+k2=6, 7, 8, 12, and 16 with k1>k2, k1<k2 or k1=k2 to ensure the performance (e.g., the minimum value that achieves the close performance to upperbound).

  - Further study the quantized value of each codepoint.

- Further study the potential UE complexity between Option 1a and Option 1b, e.g., considering the potential conversion to amplitude/phase domain of Option 1b.

- Consider Metric#1 (SGCS for a trained model) for the evaluation.

- The study at least considers 32 ports, 64 ports, and 128 ports; 19 subbands; rank 4.

- It is up to companies to study mixed Target CSIs subject to multiple {k1, k2} values each corresponding to a specific port/subband/layer configuration, for training scalable model.

- Study the necessity of RRC signaling enhancement, e.g., for a large k1, k2 pair if needed.

- Further study whether/how to perform normalization for Option 1a and Option 1b, e.g., the ranges of both [T1min, T1max] and [T2min, T2max] are fixed and specified as [-1, 1] for real/imag quantization, or [0, 1]/[0, 2π) for amplitude/phase quantization. Select one among the alternatives:

  - Alt.1: It is UE implementation to perform scale up/down to fit the fixed range.

  - Alt.2: Additional specification on the quantization, e.g.,

    - Alt.2-1: At least one element among all real and imag elements in the Target CSI matrix (FFS per layer and/or per subband) correspond to any of Tmin (where Tmin=T1min=T2min) and Tmax (where Tmax=T1max=T2max) in the quantization table or at least one element among all amplitude elements in the Target CSI matrix (FFS per layer and/or per subband) correspond to T1max

    - Alt.2-2: Reporting scaling factor and offset factor used for denormalization.

    - Alt.2-3: Fixed scaling factor used for normalization.

Agreement:

Support providing Pairing ID(s) in CSI-ReportConfig for UE side data collection

- FFS mandatory or optional

- FFS whether to configure Pairing ID for NW side data collection

- FFS whether multiple Pairing IDs provided in one CSI-ReportConfig for UE side data collection

  - Agreements for #122bis

Agreement:

For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantization), consider the following definition for evaluation:

- Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall 2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2.

  - Note: Companies to report whether  real value and imaginary value are quantized, or amplitude value and phase value are quantized.

- Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part. Uniform quantization is assumed.

  - Note: Companies to report whether/how the normalization is performed, and whether it is normalized within or across 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts.

- Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values.

- Note: It is up to company to use number of samples S>1 in their evaluations. In this case, companies need to provide information how S samples are quantized in their evaluation.

Agreement:

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection.

- L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits

  - Company to report the quantization step for reference amplitude for 6bits.

- No restriction to the total number of non-zero coefficients for a specific rank.

- Other combinations can be considered and reported by companies.

Agreement:

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Performance metric:

  - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

  - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution

  - Overhead per sample and per layer SGCS results are reported per layer

- Note: Complexity should be analysed.

- For Option 1, consider k1+k2=4, 6 ,7, 8, 16, 32

- Other assumptions follow R18 study EVM

Agreement

For CPU counting of UE side data collection,

- OCPU=1

- FFS: The CPU occupancy duration

  - Agreements for #122

Agreement:

For model pairing procedure for inference, consider the applicability report procedure of Rel-19 AI/ML beam management for UE side model as a starting point.

- Pairing ID(s) (i.e., ID for pairing related discussion) is indicated (e.g., in CSI-reportConfig) in Step 3.

  - FFS whether/how to link the Pairing ID with the inter-vendor collaboration.

  - FFS whether/how Paring ID(s) are reported in Step 4.

- FFS whether to support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only).

  - FFS CSI compression specific inference configuration or inference related parameters.

Agreement:

For NW side data collection of CSI compression, discuss the content and format of the collected data by assuming higher layer based signalling.

- Details on signaling is up to RAN2.

Agreement:

To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report.

- P-CSI-RS and /SP CSI-RS are supported.

Agreement:

For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,:

- Option 0: Reusing legacy e-Type II codebook

- Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI.

- Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization.

- Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix.

- Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase)

- Note: the quantization format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling.

- Note: At least take precoding matrix as Target CSI type. FFS channel matrix.

- FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline.

- Potential issues for the next meeting

- NW side data collection: Please companies bring the analysis/evaluation results to justify the following (take the agreed simulation parameters in your evaluation!):

  - Whether to go with Option 1a or 1b, including analysis of performance and complexity. Please only propose one option in your Tdoc!

  - For your preferred option, preferred values of k1 and k2, and whether there is necessity to enhance RRC signaling (please DO NOT forget to check with your RAN2 colleagues, whether the current mechanism can support >9KB Target CSI sample with one or multiple RRC messages)

  - Whether there is need to have additional specification on quantization (i.e., Alt.2), considering the trade-off between restriction to UE and potential data quality. If so, what is the preferred Alt.2 sub-solution.

- Monitoring: We had a short discussion at Thu off-offline discussion. Some companies want to keep at least L1 signaling solution (UE monitoring or NW monitoring) – maybe next meeting we can try to discuss a package to include simple solutions for both NW and UE, L1 and L3 – should be simple and implementable!

- References

- R1-2508326	Discussion on other aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

- R1-2508345	AI/ML CSI Spatial/Frequency Compression: Other Aspects	InterDigital, Inc.

- R1-2508375	Discussion on AIML for CSI compression other aspects	Spreadtrum, UNISOC

- R1-2508396	Other aspects of AIML for CSI compression	Ericsson

- R1-2508419	Discussion on other aspects for CSI compression	vivo

- R1-2508445	Discussion on other aspects of CSI compression	CMCC

- R1-2508495	Discussion on other aspects for CSI compression	Huawei, HiSilicon

- R1-2508537	Discussion on other aspects for CSI compression	TCL

- R1-2508547	Discussion on other aspects of CSI compression	NEC

- R1-2508584	Other aspects for AI/ML CSI compression	CATT

- R1-2508671	Discussion on other aspects of CSI spatial/frequency compression	Xiaomi

- R1-2508694	Discussion on other aspects of CSI compression	ZTE Corporation, Sanechips

- R1-2508714	Other aspects for AI/ML CSI compression	OPPO

- R1-2508750	Discussion on other aspects for CSI compression	LG Electronics

- R1-2508763	Discussion on other aspects of CSI compression	KT Corp.

- R1-2508789	Views on other aspects of CSI compression	Samsung

- R1-2508828	Other Aspects	Tejas Network Limited

- R1-2508861	Views on the lifecycle management for AI/ML-based CSI Compression	Southeast University

- R1-2508885	Discussion on other aspects for AI/ML-based CSI compression	Panasonic

- R1-2508926	Discussion on other aspects of CSI compression	Fujitsu

- R1-2508943	Other Aspects for AI/ML based CSI Compression	Google

- R1-2508950	CSI Compression: Other Aspects	Nokia

- R1-2508955	Other aspects for CSI compression	Lenovo

- R1-2508963	Discussion on other aspects of CSI compression	ETRI

- R1-2508986	Other aspects for CSI compression	HONOR

- R1-2509036	Discussion on other aspects for AI CSI compression	Ofinno

- R1-2509051	Views on the lifecycle management for AI/ML-based CSI Compression	Southeast University

- R1-2509066	Discussion on performance monitoring for CSI compression	Sony

- R1-2509093	On other aspects for AI based CSI spatial/frequency domain compression	Apple

- R1-2509151	Other aspects of AI/ML-based CSI compression	MediaTek Inc.

- R1-2509183	Discussions on other aspects for NR air interface enhancements	Sharp

- R1-2509218	Specification of other aspects of AIML CSI compression	Qualcomm Incorporated

- R1-2509267	Discussion on other aspects of AI/ML CSI compression	NTT DOCOMO, INC.

- R1-2509319	Discussion on other aspects of AIML-based CSI compression	Rakuten Mobile, Inc

- R1-2509367	Discussion on AIML based CSI compression	ASUSTeK

- R1-2509418	Other Aspects of AI/ML Assisted CSI Compression	TOYOTA Info Technology Center