3GPP TSG RAN WG1 #123                                                                               R1-2509597

Dallas, USA, Nov 17th – 21st, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0710 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SRI in STxMP SFN non-codebook PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC, NEC, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For non-codebook PUSCH when multipanelSchemeSFN is configured and when codepoint "00" or "01" of SRS Resource Set indicator is indicated, only one SRI is indicated. However, UE behavior when two SRIs are indicated is captured in current specification for the case when codepoint "00" or "01" of SRS Resource Set indicator is indicated. The current description should be aplicable only when codepoint “10”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For non-codebook PUSCH when multipanelSchemeSFN is configured and when codepoint "00" or "01" of SRS Resource Set indicator is indicated, remove description of UE behavior when two SRIs are indicated for the case when codepoint "00" or "01" of SRS Resource Set indicator is indicated, in order to make sure that UE does not expect to be indicated with two SRIs in this case. The current description is applicable only when codepoint “10”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification leads to misunderstanding that two SRIs can be indicated when codepoint "00" or "01" of SRS Resource Set indicator is indicated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has no isolated impact on network and UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.1.1.2	Non-Codebook based UL transmission

<Unchanged part omitted>

When multipanelSchemeSFN is configured and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', two SRI(s) are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator and srs-ResourceIndicator2 in configuredGrantConfig:

-	When codepoint "10" of SRS Resource Set indicator is indicated, or when srs-ResourceIndicator2 is provided, the first SRI is used to indicate resource(s) to be associated with layer(s) {0…v-1} and the second SRI is used to indicate resource(s) to be associated with  layer(s) {0…v-1}, where  v ≤ Lmax and where Lmax is defined in clauses 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212]. The UE shall expect that SRS resource(s) indicated by the first SRI and SRS resource(s) indicated by the second SRI are corresponding to different PUSCH antenna ports. The UE shall expect that the number of SRS resources associated with two indicated SRIs to be the same.

-	When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI is reserved, the first SRI is used to indicate resources(s) to be associated with layers {0…v-1}, where v ≤ Lmax.

-	Codepoint "11" of SRS Resource Set indicator is reserved.

<Unchanged part omitted>