3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.1

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session notes for AI 8.1 (Maintenance on AI/ML for NR Air Interface)

Document for:	Discussion, Decision

## Maintenance on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

Note: Maximum one contribution. For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- Specification support for beam management

- Specification support for positioning accuracy enhancements

- Specification support for CSI prediction

[122bis-R19-AI/ML] Email discussion on AI/ML – Juan (Qualcomm)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2506760	Remaining Issues of Rel-19 AI/ML for NR Air Interface	Ericsson

R1-2506770	Discussion on maintenance of AI for Air Interface	ZTE Corporation, Sanechips

R1-2506871	Maintenance on AI/ML for NR Air Interface	vivo

R1-2506934	Maintenance of Rel-19 AI/ML for air interface	Huawei, HiSilicon

R1-2506961	Maintenance on AI/ML for NR Air Interface	Xiaomi

R1-2507000	Maintenance of specification support for AI/ML for NR Air Interface	CMCC

R1-2507094	Remaining issues on AI/ML for NR air interface	CATT

R1-2507155	Maintenance on Rel-19 AI/ML for NR air interface	OPPO

R1-2507227	Remaining issue on AI/ML for NR Air Interface	Samsung

R1-2507277	Remaining issues on AI/ML for air interface in Rel-19	Fujitsu

R1-2507302	Remaining Issues on AIML for NR Air Interface	NEC

R1-2507374	Discussion on AI/ML maintenance in 5G NR interface	Panasonic

R1-2507383	Maintenance on AI/ML for NR Air Interface	Nokia

R1-2507394	Maintenance on AI/ML for NR Air interface	LG Electronics

R1-2507442	Maintenance on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface	Lenovo

R1-2507452	Maintenance of AI beam management	Ofinno

R1-2507515	Maintenance for AI/ML for NR Air Interface	Google

R1-2507582	Maintenance on Rel-19 AI/ML	InterDigital, Inc.

R1-2507648	Remaining issues for R19 AI/ML for NR air interface	Apple

R1-2507694	Maintenance on AI/ML for NR air interface	Qualcomm Incorporated

R1-2507774	Maintenance on AI/ML for NR Air Interface	Sharp

R1-2507832	Remaining issues on AI/ML for air interface in Rel-19	Quectel

R1-2507865	Remaining issues on AI PHY	ASUSTeK

### Specification support for beam management

R1-2507998

Agreement:

The following TP for TS 38.214 Clause 5.2.1.4.3b is endorsed.

Reason for change: The slot of transmission occasion for monitoring is unclear for the linkage of the corresponding CSI report for inference.

Summary of change: Clarify the first slot of transmission occasion for monitoring is used for the linkage.

Consequences if not approved: gNB and UE may have different understanding on the linkage.

##### TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting <omitted texts> - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; <omitted texts>

Agreement:

The following TP for 38.214 Clause 6.2.1.3 is endorsed.

Reason for change: P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is missing in the prioritization rules between a transmission of SRS and transmission of a physical signal/channel.

Summary of change: Add P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP in the prioritization rules.

Consequences if not approved: the prioritization rules between a transmission of SRS and transmission of a physical signal/channel with P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is unclear.

6.2.1.3 UE sounding procedure between component carriers <omitted text> The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI/P-CRI/P-SSBRI/RS-PAI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/P-L1-RSRP, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCP/cjtc-Dd/cjtc-F/cjtc-Dd-F/cjtc-P/P-L1-RSRP on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol. <omitted text>

Agreement:

The following TP for 38.214 Clause 6.2.1 is endorsed.

Reason for change: P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is missing in the collision handling procedure between a transmission of SRS and transmission of a physical signal/channel.

Summary of change: Add P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP in the collision handling procedure.

Consequences if not approved: the collision handling procedure between a transmission of SRS and transmission of a physical signal/channel P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is unclear.

6.2.1 UE sounding procedure <omitted text> For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s), or only RS-PAI report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s), or only RS-PAI report(s) and the PUCCH starts no earlier than Tproc,2 after the last symbol of the PDCCH carrying the triggering command for the aperiodic SRS, where Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the triggering command and the SCS configuration of the PUCCH. <omitted text>

Agreement:

Regarding the RAN2 LS from R1-2506715, for the UE-side candidate data collection configuration for BM-Case2, include the following IEs along with the already agreed IEs in RAN2:

- TimeGap-r19, i.e., the expected time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance.

- nroftimeinstance-r19, i.e., number of expected predicted time instances.

R1-2507999

Agreement

Regarding the RAN2 LS R1-2506720, the answer to the question in the LS is as below:

From RAN1 perspective, Option 2 is best option and can resolve the issue. - No additional RAN 1 impact was identified for Option 2.

Agreement:

The following TP for TS 38.214 Clause 5.2.1.4.2 is endorsed.

Reason for change: The configuration restriction of AP RS resource set with no more than 16 resources is too restrictive for Set A for inference.

Summary of change: For Set A for inference, relax the configuration restriction of AP RS resource set with no more than 16 resources.

Consequences if not approved: The configuration of more than 16 resource within Set A for inference is not possible.

##### TS 38.214 Clause 5.2.1.4.2 Report quantity configurations <omitted texts> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-LI-PMI-CQI', UE does not expect the CSI-ReportConfig to be configured with higher layer parameter codebookType set to 'typeII-r16' or 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18', 'typeII-Doppler-PortSelection-r18', 'eTypeII-r19', 'TypeII-FePortSelection-r19' or 'typeII-Doppler-r19'. If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP- Index', 'cri-SINR- Index, 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', or ‘rs-pai-r19', and the CSI-ReportConfig is linked to a resource setting not configured by resourcesForSetA-r19 and configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within the resource setting. <omitted texts>

Agreement:

The draft response LS in R1-2508121 is endorsed. Final LS in R1-2508122

### Specification support for positioning accuracy enhancements

R1-2508009

Agreement:

The following text proposal to TS 38.214 V19.1.0 is endorsed.

| Reason for change | - Capture the agreements related to associated ID for ensuring consistency between training and inference for Rel-19 AI/ML positioning. - For training data provided by PRU, TRP-LocationInfo-Implicit may be provided together with NR-PRU-DL-Info for ensuring consistency between training and inference for Rel-19 AI/ML positioning. |
| --- | --- |
| Summary of change | - Describe the UE behavior on receiving associated ID from the network. - Clarify that the UE may be provided with NR-TRP-LocationInfo-Implicit associated with NR-PRU-DL-Info. |
| Consequences if not approved | For Rel-19 AI/ML positioning, the UE may not be able to ensure consistency between training and inference. |
| Suggested text proposal | TS 38.214 V19.1.0 ##### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource. The UE may be configured by the network with nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN [17, TS 37.355] associated with a dl-PRS-ID. - If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, if provided, are the same as the corresponding information of a serving cell, the UE may assume that the DL PRS is transmitted from the serving cell; - Otherwise, the UE may assume that the DL PRS is not transmitted from a serving cell. If the UE assumes that the DL PRS is transmitted from a serving cell, and if the serving cell is the same as the serving cell defined by the SS/PBCH block, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same serving cell. If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell. The UE may be configured by the network with TRP-LocationInfo-Implicit-Element [17, TS 37.355]. - If TRP-LocationInfo-Implicit-Element is provided, and if nr-PhysCellID or nr-CellGlobalID is provided with nr-AIML-AssociatedID, for TRP(s) in the given cell, the UE may assume that the geographical coordinates of the TRP(s) are consistent for a same nr-AIML-AssociatedID. < Unchanged parts are omitted > The UE may be provided with NR-PRU-DL-Info which contains measurement(s) performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamps associated with the measurement(s), and the location information of the PRU. The UE may be provided with NR-TRP-LocationInfo-Implicit [17, TS 37.355] associated with NR-PRU-DL-Info. < Unchanged parts are omitted > |

Agreement:

Regarding the RAN2 LS R1-2506737, the following response from RAN1 is endorsed.

Question 1: Does a single Associated ID correspond to the location coordinates of one specific TRP within a cell, or does it represent the location information of a group of TRPs in the same cell, each having distinct location coordinates? Response to Question 1: As stated in RAN1#121 agreement, an associated ID is configured per cell (e.g., NCGI-r15). It is provided for a group of TRP(s) in the same cell, not for one specific TRP within a cell. Note: A group of TRP(s) in the same cell can have only one TRP. Question 2: Is the Associated ID in any way related to the identification of the location of Antenna Reference Points (ARPs) associated with DL-PRS Resource Sets and DL-PRS Resources? Response to Question 2: It is up to network implementation whether the associated ID is related to the identification of the location of ARP(s). Question 3: Are Associated IDs unique across different cells? Can cells belonging to different Positioning Frequency Layers (PFLs) share the same Associated ID? Furthermore, if a single cell is part of multiple PFLs, should it be assigned a distinct Associated ID for each PFL? Response to Question 3: As captured in the RAN1#121 agreement, the associated ID is configured per cell, not per PFL. Specific to RAN2’s Question 3: - Associated ID(s) are not required to be unique across different cells. - As associated ID(s) are unique only per cell, sharing associated ID(s) for cells belonging to different PFLs is irrelevant. - The associated ID is not PFL specific. The associated ID is specific only to geographical coordinates of a group of TRP(s) in a cell. Question 4: RAN2 observes that an Associated ID is configured "per-Cell". However, NR DL-PRS Assistance Data support also PRS-only Transmission Points (TPs), which are not associated to any specific cell (i.e., have no NCGI). Can an Associated ID also implicitly indicate the location coordinates of such PRS-only TPs? Response to Question 4: From RAN1’s perspective, in Rel-19, the associated ID cannot be used to implicitly indicate the consistency of location coordinates of such PRS-only TPs, as the PRS-only TPs are not associated with a cell.

The final LS response to RAN2 is endorsed in R1-2508026.

### Specification support for CSI prediction

R1-2508061

Agreement:

The following TP for TS 38.214 is endorsed.

| Reason for change | During non-active periods of cell DTX, UE is not expected to receive P/SP CSI-RS for CSI report associated with report quantity comprising at least RI which means this can be applied to SP inference report. However, whether it is applied for monitoring report is not clear. |
| --- | --- |
| Summary of change | Report quantity ‘csi-pai-r19’ is added. |
| Consequences if not approved | Inconsistent UE behaviour during non-active periods of cell DTX. |
| TS38.214 5.1.6.1 CSI-RS reception procedure << Unchanged parts are omitted >> During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. << Unchanged parts are omitted >> |  |

Agreement:

Regarding the RAN2 LS from R1-2506715, for the UE-side candidate data collection configuration for CSI prediction, include the following IEs:

- CSI-ResourceConfigId

- FFS: whether to include other parameters

R1-2508062

Offline agreement

Adopt the following TP for TS 38.214.

| Reason for change | According to the agreement, Rel-16 eType II codebook is used for calculating both the precoder wk represented by PMI based on the channel measurement corresponding to the monitoring report and the precoder wk represented by PMI based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the inference report. However, the codebook for calculating wkwas not clearly captured by the specification. |
| --- | --- |
| Summary of change | Add the corresponding description of the codebook for calculating wk in TS 38.214. |
| Consequences if not approved | How to select the codebook for calculating wk is not clearly defined in TS 38.214, which is not aligned with the agreement. |
| TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to ‘etypeII-r16’ and it shall: <Unrelated part omitted> - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=n=1NwknHwknwkn.wkn2N, SGCSk2=n=1NwknHwknwkn.wkn2N, where wkn is the predicted precoder represented by predicted PMI for k-th layer, n-th subband and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, n-th subband based on the channel measurement corresponding to the second Reporting Setting, and wk wkn is the precoder represented by PMI for k-th layer and n-th subband, n∈1,2,…,N, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. Both wkn and wkn are calculated based on the codebookConfig within the second Reporting Setting. The UE is expected to be configured with same csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband configurations for the first CSI Reporting Setting and the second CSI Reporting Setting. << Unchanged parts are omitted >> |  |

Offline agreement

Following TP is endorsed as editorial corrections for TS 38.214 for Clauses 5.2.1.4.2, 5.2.1.4.6, 5.2.1.6 and 5.4.

TS38.214 ##### 5.2.1.4.2 Report quantity configurations << Unchanged parts are omitted >> If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to ‘csi-pai-r19’, - the UE shall be configured with inferenceReportConfigId-r19 to link another CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, - when semi-persistent Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, the UE is not expected to be configured with a periodic Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to ‘csi-pai-r19’. - when aperiodic Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, the UE is not expected to be configured with a periodic or semi-persistent Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to ‘csi-pai-r19’. ##### << Unchanged parts are omitted >> ##### 5.2.1.4.6 CSI-PAI reporting When the UE is configured with a first CSI Reporting Setting for reporting CSI prediction, based on a CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 and a second CSI Reporting Setting for reporting CSI-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to ‘csi-pai-r19’, and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of CSI-PAI corresponding to the second CSI Reporting Setting shall consider the following: << Unchanged parts are omitted >> ##### 5.2.1.6 CSI processing criteria The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, or CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, OCPU= OCPU,1 is considered. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’ corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 if a CSI report is not considered within any of M and Mx, the values for OCPU,1 and OCPU,x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report. A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows: << Unchanged parts are omitted >> - OCPU,1=X1 and OCPU,x=Xx, for a CSI report configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19, where the values of X1 and Xx are reported by UE capability. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to ‘none’, or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to ‘none-bm-r19’ or ‘none-csi-r19’, the CPU(s) (including OCPU,1and/or OCPU,x, for CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, or ‘csi-pai-r19’, or ‘rs-pai-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 are occupied for a number of OFDM symbols as follows: << Unchanged parts are omitted >> In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, (excluding the case in which the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to ‘csi-pai-r19’), starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. << Unchanged parts are omitted >> For aperiodic CSI-RS, when the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to ‘csi-pai-r19’, NZP CSI-RS resource is active in a duration starting from the end of the PDCCH containing the request for the report of the corresponding CSI Reporting Setting and ending at the end of the scheduled PUSCH containing the report associated with the report of the linked CSI Reporting Setting. ##### 5.4 UE CSI computation time << Unchanged parts are omitted >> (Z2+14K-1m+t,Z2'+t), with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to ‘typeII-Doppler-r18’ or ‘typeII-Doppler-PortSelection-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, and t is according to UE reported capability as defined in [13, TS 38.306] when the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+w+t,Z2'+t), with (Z2,Z2') of table 5.4-2, where w=56⋅(KP -1) or 56⋅KP symbols, according to the reported UE capability, where the value of KP ∈{1,2,4} is indicated by UE capability, if the CSI report is configured with N4=1, codebookType is set to ‘typeII-Doppler-r18’ or ‘typeII-Doppler-PortSelection-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, and t is according to UE reported capability as defined in [13, TS 38.306] when the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+14K-1m+t,Z2'+t) or (Z2+14K-1m+Z2'+t,2Z2'+t), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to ‘typeII-Doppler-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, and t is according to UE reported capability as defined in [13, TS 38.306] when the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+w+t,Z2'+t) or (Z2+w+Z2'+t,2Z2'+t), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to ‘typeII-Doppler-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, and t is according to UE reported capability as defined in [13, TS 38.306] when the higher layer parameter [RRC_name-r19] csi-InferencePrediction-r19 is configured in CSI-ReportConfig, otherwise t=0, or << Unchanged parts are omitted >>

Agreement ( The previous agreement is revised as the following):

Regarding the RAN2 LS from R1-2506715:

For the UE-side candidate data collection configuration for CSI prediction, include the following IEs:

- CSI-ResourceConfigId

For candidate UE-side data collection, following IE(s) in addition to CSI-ResourceConfigId can be provided.

- CodebookConfig-r18 set to ‘typeII-Doppler-r18’

The draft response LS in R1-2508139 is endorsed. The Final LS is agreed in R1-2508140.

R1-2508142

Agreement

Support the configuration of following parameter in a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none-CSI-r19'

- CodebookConfig-r18 set to ‘typeII-Doppler-r18’