3GPP TSG RAN WG1 #122			R1-230XXXX

Bengaluru, India, Aug. 25th – 29th, 2025

Agenda item:	10.2.1

Source: 	Moderator (CATT)

Title: 	Moderator summary on improvement of SRS capacity and coverage: Round 1

Document for:	Discussion and Decision

- Introduction

In RAN #108, the Rel-20 WID of NR MIMO Phase 6 was approved [1]. In the approved WID, enhancement of SRS capacity and coverage is a part of RAN1 objectives.

The detailed objectives are as follows: RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

This summary includes the following:

- Summary of companies’ views on each of open issues raised by interested companies

- Observations and recommended proposals based on the summary of companies’ views

- Issue 1: Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

Open issues on multiple frequency-domain starting positions for SRS repetition and company views are summarized below.

Table 1. Summary of views on Issue 1

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1-1 | Starting position pattern | K is the number of frequency-domain starting positions. - PF=2 Support: HW, CATT, Samsung, ZTE, Apple Not support: vivo, DCM - K=2 - [0, 1]: HW, CATT, Samsung, ZTE, Apple - [1, 0]: HW - PF =4 Support: HW, CATT, MTK, Samsung, ZTE, Apple, vivo (K=2), DCM (K=2) - K=2 : vivo, DCM - [0, 2]: HW,CATT, ZTE, Apple, Samsung, MTK - [1, 3], [2, 0] , [3, 1]: HW - K=4 - [0,2,1,3]: HW, ZTE, CATT, Apple, Samsung, MTK - [1, 3, 2, 0], [2, 0, 3, 1] and [3, 1, 0, 2]: HW - HONOR, DCM, Lenovo Equally spacing the RB gap among multiple frequency-domain starting offsets: - QC Regarding how to config the “sub-hop” pattern, down-select one of the following two options: - Option 1: Config NsubHop sub-hops, each with different a different starting RB, within a legacy frequency hop of R symbols; - Option 2: Config RsubHop consecutive symbols associated with a same starting RB, within a legacy frequency hop of R symbols; wherein, legacy repetition factor R = NsubHop⋅RsubHop. - Sharp, DCM - configuring two StartRBIndex kF1 and kF2=kF1+PF/2 mod PF - Nokia, Sony - Case 1: R/RRPFS is a positive integer - Case 2: R is not an integer multiple of RRPFS - NEC, xiaomi, Rakuten Mobile - noffsetRPFS=NSCRBmSRS,BSRSkF+khop+khop'mod PFPF. And the new factor of hop khop' is the RPFS frequecy offset within each SRS hop |
| 1-2 | Phase continuity | - Send LS to RAN4 to check the feasibility of maintaining the phase continuity within SRS repetitions with the RPFS start RB index hopping - CATT, Samsung, ZTE, MTK, Sony - QC - Phase coherence over SRS symbols is not required when RB position changes. - InterDigital - RAN4 requirement on DMRS bundling is not relevant for SRS frequency hopping with multiple starting positions. Do not consider RAN4 requirement on DMRS bundling for SRS frequency hopping with multiple starting positions. - Samsung - Introduce a separate UE capability whether phase continuity is satisfied |
| 1-3 | Enhanced RPFS start RB index hopping + legacy RPFS start RB index hopping (enableStartRBHopping= “true”) | Whether/how to support legacy RPFS start RB index hopping and enhanced RPFS simultaneously. - Interested companies: MTK, Samsung, Ericsson |
| 1-4 | Other issues | - Qualcomm, Ericsson - Support periodic, semi-persistent, and aperiodic SRS. - OPPO - For UE supporting RPFH for non-FH cases, multiple frequency-domain starting positions across repetitions can also be supported. |

Companies are encouraged to show views/comments/suggestions in the following tables.

P1-1: Starting position pattern

Proposal 1-1: For the SRS pattern with multiple frequency-domain starting positions within one hop

- The R repetitions are equally divided into K subgroups, each containing R/K consecutive symbols.

- FFS: whether non-consecutive symbols are supported for each subgroup.

- FFS: R is not an integer multiple of K.

- Within each subgroup of R/K symbols, the SRS is transmitted at the same starting position in frequency domain.

- Support K=2 for PF=2 and K=2 or 4 for PF=4

- The starting position patterns across the K different frequency locations are pre-defined, e.g.,

- For PF=2 and K=2, support pattern {0,1}

  - FFS: {1,0}

- For PF =4 and K=2, support pattern {0,2}

  - FFS: {1,3}, {2,0}, {3,1}

- For PF =4 and K=4, support pattern {0,2,1,3}

  - FFS: {1,3,2,0}, {2,0,3,1}, {3,1,0,2}

- Start position pattern is the same during the period for sounding all subbands (hopping period)

FFS: whether/how to support enabling legacy RPFS start RB index hopping and enhanced RPFS start RB index hopping simultaneously.

FFS: whether/how to support multiple frequency-domain starting positions for RPFS without FH..

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P1-2: Phase continuity

Proposal 1-2: Send LS to RAN4 to check the feasibility of maintaining the phase continuity within SRS repetitions with the RPFS start RB index hopping

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P1-3: Supported types of  SRS

Proposal 1-4: Support enhanced RPFS for all SRS resource types, i.e., for periodic, semi-persistent, and aperiodic SRS.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P1-4: Others

Proposal 1-5: TBD.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the other issues. |

- Issue 2: Cross-slot SRS between one U slot and one adjacent S slot

Open issues on cross-slot SRS between one U slot and one adjacent S slot and company views are summarized below.

Table 2. Summary of views on Issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2-1 | Starting slot for SRS transmission | - For a cross-slot SRS resource, the slot offset refers to the first of the two slots spanned by the SRS resource. - Ericsson - For periodic/semi-persistent SRS, the candidate starting slots in which the configured SRS resource may be used for starting SRS transmission are the slots satisfyingNslotframe,μnf+ns,fμ-Toffset mod TSRS=0 - HW, vivo - For aperiodic SRS, higher layer parameter slotOffset and availableSlotOffset decide the starting slot of an SRS resource set. - HW |
| 2-2 | Per slot resource offset for AP SRS | - Samsung, MTK, Ericsson vivo, DCM, Sony, ZTE - Introduce per-SRS resource slot offset for each of SRS resources within an aperiodic SRS resource set - ZTE: - Option-1: To configure the current slot-level offset (i.e., slot offset k and available slot offset t) per SRS resource for a given aperiodic SRS resource set in case of SRS transmission across two adjacent S+U slots. - Option-2: Introduce a newly additional slot offset (denoted as t') per SRS resource on top of slot offset k and available slot offset t that is configured for a given aperiodic SRS resource set in case of SRS transmission across two adjacent S+U slot. |
| 2-3 | Definition of available slot | - The definition of available slot can be extended to multiple consecutive slots satisfying there are UL or flexible symbol(s) for transmitting all the SRS resources in the resource set. - CATT, Samsung, MTK, HW, Ericsson, NEC - ZTE - Option-1: Available slot is determined per SRS resource An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for at least one of the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Option-2: Available slot is determined per slot An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for a subset of SRS resource(s) with same available slot offset value in the SRS resource set, and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Option-3: Available slot is determined per SRS resource set The available slot(s) can include one or more slot(s) satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - DCM - An available slot group is two adjacent slots consisting of UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the AP SRS resource set. - DCI indicates the cross-slot aperiodic SRS resource set is transmitted on (t+1)-th available slot group counting from the slot configured by slotOffset. - RRC configures per SRS resource whether the SRS resource is transmitted on first slot or second slot. |
| 2-4 | Possible scenarios for cross-slot SRS | - Scenario 1: an aperiodic SRS resource set which includes at least one SRS resource with time resource allocated in the first slot, and at least one another SRS resource with time resource allocated in the second slot. - Samsung, MTK, Ericsson, vivo, Sharp, DCM, Sony, Fujitsu, KDDI, Xiaomi - Scenario 2: a periodic, semi-persistent, or aperiodic SRS resource set which includes at least one SRS resource with time resource allocated across two slots. - Samsung, MTK, QC, Apple, NEC，NICT, Spreadtrum, Tejas Network Limited - Down select one from the above scenarios - OPPO |
| 2-5 | Time domain location of cross-slot SRS | - Samsung, Ericsson, Nokia, Apple, HONOR, Rakuten Mobile, TCL, xiaomi - Reuse legacy RRC parameters for time domain resource allocation (i.e., startPosition, nrofSymbols, and repetitionFactor) without restriction on “within a slot” - HW - Support loffset to count symbols backwards from the end of the last slot of P/SP SRS resource or AP SRS resource set so as to accommodate cross-slot SRS transmission, the candidate value of which is extended to {0, 1, …, 27}. - Support to derive l0 by l0=2Nsymbslot-1-loffset so as to accommodate cross-slot SRS transmission. - ZTE, Transsion Holdings: - Option-1: One set of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) is configured to the SRS resource transmitted across two adjacent S+U slots - Option-2: Two sets of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) are configured for the SRS resource transmitted in S slot and U slot, respectively. |
| 2-6 | Maximum number of repetition and SRS symbols | - Maximum number of repetition and SRS symbols beyond 14. - Support: ZTE, vivo, Fujitsu, HONOR, KDDI, Lenovo, Spreadtrum, xiaomi - Not support: CATT, Samsung, MTK, Qualcomm, Ericsson, Tejas Network Limited |
| 2-7 | SRS transmission before PUSCH | - Support transmitting PUSCH after SRS in a slot - CATT, Samsung, MTK, HW, Ericsson, NTT DOCOMO, China Telecom, TCL |
| 2-8 | Multiple aperiodic SRS resource sets in a slot for usage of antenna switching | - For cross-slot SRS, support configuring/triggering multiple aperiodic SRS resource sets in a slot for usage of antenna switching. - Support: CATT, Samsung, Tejas Network Limited - Further study: OPPO |
| 2.9 | Supported type and usage of cross-slot SRS | - At least BM and AS - MTK - Codebook, non-codebook, antenna switching, beam management - Spreadtrum - P/AP/SP SRS - Spreadtrum, Ericsson |
| 2-10 | Other issues | - QC For cross-slot SRS, according to WID, UE does not expect the following conditions changed between the two consecutive slots: TA, UL spatial filter, and transmit power. - RAN1 clarifies that phase continuity is not required for UE, if any of the above conditions are not satisfied. - DCM - Whether WID precludes cross-slot on two adjacent U slots. - Whether cross-slot SRS within one SRS resource is applicable for SRS without repetition. - NEC: - Further discuss a common timing advance (TA), a common UL spatial filter, and common transmit power - Xiaomi - Support only PUSCH mapping Type B for PUSCH transmission. - ZTE - The transmission occasion for SRS transmission should be defined by the total of consecutive SRS symbols - Lenovo - The transmit power for the SRS transmission is calculated based on the parameters determined in the first slot for the SRS transmission. - HONOR - If cross-slot SRS in S slot is overlapped, at least the last symbol in S slot is overlapped, its remaining part in adjacent U slot should be dropped to save the uplink resources. |

P2-1: Starting slot for SRS transmission

Proposal 2-1: For a cross-slot SRS resource, the slot offset refers to the first of the two slots spanned by the SRS resource.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P2-2: Per slot resource offset for AP SRS

Proposal 2-2: For cross-slot SRS, per-SRS resource slot offset for each of SRS resources within an aperiodic SRS resource set is supported.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P2-3: Definition of available slot

Proposal 2-3: The available slot(s) can include one or two consecutive slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P2-4: Possible scenarios for cross-slot SRS

Proposal 2-4: Possible scenarios for cross-slot SRS transmission include:

- Scenario 1: an aperiodic SRS resource set which includes at least one SRS resource with time resource allocated in the first slot, and at least one another SRS resource with time resource allocated in the second slot.

- Scenario 2: a periodic, semi-persistent, or aperiodic SRS resource set which includes at least one SRS resource with time resource allocated across two slots.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P2-5: Time domain location of cross-slot SRS

Proposal 2-5: To determine the time-domain location of cross-slot SRS, down select from the following alternatives:

- Alt 1：Reuse legacy RRC parameters for time domain resource allocation (i.e., startPosition, nrofSymbols, and repetitionFactor) without restriction on “within a slot”.

  - For the index of each SRS symbol l', it is the same as legacy spec., i.e., l'=0, 1, …, NsymbSRS-1.

  - The offset loffset∈0,1,…,13 counts symbols backwards from the end of the starting slot of the resource

    - For SRS resource across slot boundary, 1≤loffset<NsymbSRS-1

    - For SRS resource with in one slot, loffset≥NsymbSRS-1

- Alt 2: modify the definitions of  loffset and l0

  - The offset loffset∈0,1,…,27 counts symbols backwards from the end of the last slot of a SRS resource.

  - l0=2Nsymbslot-1-loffset.

- Alt 3: Two sets of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) are configured for the SRS resource transmitted in S slot and U slot, respectively

  - For the index of each SRS symbol l', it should be that l'=0, 1, …, Nsymb,  SSRS-1 in S slot and l'=Nsymb, SSRS, Nsymb, SSRS+1, …, Nsymb, SSRS+Nsymb,USRS-1 in U slot.

  - Nsymb, SSRS and Nsymb, USRS denoted as the number of consecutive SRS symbols configured in S slot U slot, respectively.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above alternatives. |

P2-6: Maximum number of repetition and SRS symbols

Proposal 2-6: For the maximum number of symbols per cross-slot SRS resource and repetition factor, down select from the following alternatives:

- Alt 1: limited to 14

- Alt 2: Support values over 14

.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P2-7: SRS transmission before PUSCH

- Proposal 2-7: Support transmitting PUSCH and corresponding DMRS after SRS in a slot

.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P2-8: Multiple aperiodic SRS resource sets in a slot for usage of antenna switching

Proposal 2-8: For cross-slot SRS, support configuring/triggering multiple aperiodic SRS resource sets in a slot for usage of antenna switching.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

P2-9: Supported type and usage of cross-slot SRS

Proposal 2-9-1: Support cross-slot SRS for all SRS resource types, i.e., for periodic, semi-persistent, and aperiodic SRS.

Proposal 2-9-1: Support cross-slot SRS for usages of codebook, non-codebook, antenna switching, beam management

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposals. |

P2-10: Others

Proposal 2-10: TBD

| Company | Input |
| --- | --- |
| Mod | Please share your views on the other issues. |

- Other issues

Open issues on enhancement of SRS capacity and coverage are summarized below.

Table 3. Summary for other issues

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3-1 |  |  |

P3-1:

Proposal 3-1: TBD

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above issues. |

- Issues to be discussed in online/offline sessions

5.1 Round 1

- References

- RP-251856	New WID: NR MIMO Phase 6    Samsung (Moderator)

- R1-2505164	Discussion on improvement of SRS capacity and coverage	Spreadtrum, UNISOC

- R1-2505209	Improvement of SRS capacity and coverage	Huawei, HiSilicon

- R1-2505242	NR MIMO Phase 6: SRS Enhancement	InterDigital, Inc.

- R1-2505274	Discussion on improvement of SRS capacity and coverage	ZTE Corporation, Sanechips

- R1-2505286	Discussion on improving of SRS capacity and coverage	TCL

- R1-2505288	Improvement of SRS capacity and coverage	MediaTek Inc.

- R1-2505302	On improvement of SRS capacity and coverage	CATT

- R1-2505408	Discussion on improvement of SRS capacity and coverage	vivo

- R1-2505455	Discussion on the improvement of SRS capacity and coverage	Xiaomi

- R1-2505512	Discussion on SRS capacity and coverage improvement	China Telecom

- R1-2505576	Views on improvement of SRS capacity and coverage	Samsung

- R1-2505635	Improvement of SRS capacity and coverage	Tejas Network Limited

- R1-2505749	Discussion on enhancement of SRS capacity and coverage for MIMO phase 6	OPPO

- R1-2505812	Improvement of SRS capacity and coverage	Lenovo

- R1-2505905	On Rel-20 MIMO SRS capacity and coverage improvement	Apple

- R1-2505936	Discussion on improvement of SRS capacity and coverage	NEC

- R1-2505946	Enhancements for SRS capacity and coverage	Transsion Holdings

- R1-2505965	Discussion on improvement of SRS capacity and coverage	Fujitsu

- R1-2505987	Improvement of SRS Capacity and Coverage	Nokia

- R1-2505997	Discussion on improvement of SRS capacity and coverage	HONOR

- R1-2506110	Discussion on improvements of SRS capacity and coverage	Sony

- R1-2506144	Discussion on Improvement of SRS Capacity and Coverage	Rakuten Mobile, Inc

- R1-2506210	SRS enhancements in 5G MIMO Phase 6	Qualcomm Incorporated

- R1-2506267	Improvement of SRS capacity and coverage	Sharp

- R1-2506297	Discussion on Improvement of SRS capacity and coverage	NTT DOCOMO, INC.

- R1-2506367	On Rel-20 improvement of SRS capacity and coverage	Ericsson

- R1-2506369	Discussion on improvement of SRS capacity and coverage	NICT

- R1-2506381	Views on enhancements for Improvement of SRS capacity and coverage	KDDI Corporation