3GPP TSG-RAN WG1 #122bis	DRAFT R1-2508043

Prague, Czech Republic, October 13 – 17 2025

Source:	Moderator (Nokia)

Title:	Feature Lead summary #3 on 6G waveform

Document for:	Discussion

Agenda item:	11.3.1

Work Item:	FS_6G_Radio

# Introduction

3GPP RAN1#122bis saw 43 contributions submitted to agenda item 11.3.1 6G waveforms. In addition one document was submitted to agenda item 11.3, but moved to 11.3.1 [4]. Altogether these 44  contributions totalled 433 pages, 226 observations and 233 proposals. The observations and proposals can be found in the attachment:

This document summarizes and organizes the proposals and acts as a platform to further facilitate the related discussion.

|  | Tdoc# | Title | Source |
| --- | --- | --- | --- |
| [1] | R1-2506752 | Waveform for 6G Radio Air Interface | Nokia |
| [2] | R1-2506815 | Discussion on waveform for 6GR | Spreadtrum, UNISOC |
| [3] | R1-2506899 | Discussion on Waveform for 6GR air interface | vivo |
| [4] | R1-2506905 | Discussion on waveform for 6GR air interface | THALES, University of Bologna, CTTC, DLR, ESA |
| [5] | R1-2506919 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
| [6] | R1-2506952 | Considerations for 6GR DL waveform | Kyocera |
| [7] | R1-2506990 | Discusson on 6GR Waveform | Xiaomi |
| [8] | R1-2507015 | Discussion on the waveform design for 6G radio | CMCC |
| [9] | R1-2507028 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
| [10] | R1-2507052 | Waveform design for 6GR air interface | Tejas Network Limited |
| [11] | R1-2507059 | Waveform for 6GR air interface | Huawei, HiSilicon |
| [12] | R1-2507118 | Discussions on waveform for 6GR | CATT |
| [13] | R1-2507131 | On waveforms for 6GR | Beijing University of Posts and Telecommunications (BUPT) |
| [14] | R1-2507177 | Discussion on waveform and multiple access for 6G Radio | OPPO |
| [15] | R1-2507185 | Discussion on waveform for 6GR | LG Electronics |
| [16] | R1-2507254 | Discussion on waveform for 6GR | Samsung |
| [17] | R1-2507344 | Waveform for 6GR air interface | InterDigital, Inc. |
| [18] | R1-2507368 | Waveform for 6GR Air Interface | Cohere Technologies |
| [19] | R1-2507381 | Discussion on Uplink Waveform Enhancements in 6G | KT Corp. |
| [20] | R1-2507412 | Discussion on 6G Waveform | NEC |
| [21] | R1-2507418 | Discussion on waveform for 6GR air interface | Panasonic |
| [22] | R1-2507468 | Discussion on waveform for 6GR air interface | Ofinno |
| [23] | R1-2507482 | Discussion on 6GR Waveform | Lenovo |
| [24] | R1-2507507 | Discussion on 6GR waveform | ETRI, University of Surrey |
| [25] | R1-2507513 | On 6G waveforms | Ericsson |
| [26] | R1-2507521 | Waveform for 6GR Air Interface | Google |
| [27] | R1-2507526 | New waveform for 6GR | Shanghai Jiao Tong University, NERCDTV |
| [28] | R1-2507545 | Discussion on Waveforms of 6GR Air Interface | Rakuten Mobile, Inc |
| [29] | R1-2507597 | Considerations for 6GR waveform | Sony |
| [30] | R1-2507603 | Discussion on waveform for 6GR air interface | Ruijie Networks Co. Ltd |
| [31] | R1-2507608 | Waveform for 6GR air interface | MediaTek Inc. |
| [32] | R1-2507678 | Waveforms for 6GR air interface | Apple |
| [33] | R1-2507722 | Waveforms for 6GR | Qualcomm Incorporated |
| [34] | R1-2507747 | Requirements for 6GR Waveform Design | AT&T |
| [35] | R1-2507767 | Discussion on waveform for 6G air interface | Fainity Innovation |
| [36] | R1-2507771 | Study on waveform for 6GR | Sharp |
| [37] | R1-2507816 | Discussion on Waveform | NTT DOCOMO, INC. |
| [38] | R1-2507824 | New waveform for 6GR air interface | NICT |
| [39] | R1-2507837 | Discussion on 6GR waveform design | Hanbat National University |
| [40] | R1-2507886 | Considerations on waveform for 6GR air interface | ITL |
| [41] | R1-2507896 | Discussion on Waveform for 6GR Air Interface | Indian Institute of Tech (M) |
| [42] | R1-2507902 | Views on 6GR waveforms | CEWiT |
| [43] | R1-2507942 | IIT Kanpur’s views on 6GR waveforms | IIT Kanpur |
| [44] | R1-2507951 | Waveform considerations for Uplink and Downlink | Wisig, IITH |

# Proposals

## Baseline waveform for DL for communication

| Nokia | - Proposal 1: CP-OFDM waveform as defined in 5G NR is supported for communications in 6G downlink. - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink |
| --- | --- |
| InterDigital | Proposal 1: CP-OFDM is the only downlink waveform for 6GR; do not support additional DL waveforms for 6GR |
| NTT DOCOMO | Proposal 1: For 6GR study on waveform, - Only OFDM-based waveform(s) should be considered (as described in the SID) - Any new waveform(s), even for OFDM-based, should be justified by clear gain - Unified design across scenarios/use cases is strongly preferred - Following the above, RAN1 can carefully assess the need in 6GR to introduce waveform(s) beyond 5G NR, targeting, e.g., - Potential better coverage by better PAPR performance for uplink - Spectrum efficiency improvement |
| ITL | Proposal 1: For baseline waveform candidates for 6GR - CP-OFDM should be maintained as the baseline waveform for 6G downlink, ensuring backward compatibility, numerology continuity, and efficient support for MIMO/beamforming operations. - Both CP-OFDM and DFT-s-OFDM should be retained as baseline waveforms for 6G uplink, with optional low-PAPR enhancements (e.g., FDSS, FDSS-SE, frequency-selective mapping) studied as means to improve coverage and energy efficiency, particularly for low-power devices. - Fundamentally new waveform families should only be considered if they demonstrate clear and quantifiable performance advantages, while ensuring minimal disruption to NR compatibility and deployment feasibility. Proposal 2: For 6GR waveform with consideration of MRSS Compatibility - MRSS compatibility shall be a mandatory design principle for 6GR waveform development, ensuring seamless coexistence and migration with 5G. - CP-OFDM be maintained as the common baseline waveform structure for downlink, and that uplink baseline waveforms (CP-OFDM/DFT-s-OFDM) remain fully aligned with NR grid design to support MRSS operation. - Any optional enhancements (e.g., low-PAPR techniques) be evaluated under the condition that MRSS alignment is preserved, avoiding disruption to spectrum sharing or coexistence performance. - Fundamentally new waveforms not aligned with MRSS shall only be considered if they demonstrate clear, quantifiable gains that justify the additional complexity and coexistence risks. Proposal 3: For unified or multiple waveforms for 6GR - OFDM-based waveforms be adopted as the unified baseline for 6GR, ensuring simplicity, continuity, and compatibility with NR. - Multiple waveforms not be introduced as parallel baselines, as this would create fragmentation and increase implementation burden. - Optional complementary waveforms may be studied for specialized scenarios (e.g., NTN, ISAC, sensing, low-power IoT), but only considered if they provide clear and quantifiable performance advantages over the baseline. |

### Moderator proposal(s)

Check if there is a need for strengthening the RAN1#122 agreement further

Moderator proposal: Completement the August RAN1#122 agreement with:

- CP-OFDM waveform as defined in 5G NR is supported for communications in 6G downlink.

## Baseline waveform for UL for communication

| Nokia | - Proposal 2: CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported for communications in 6G uplink. - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded as potential additions. |
| --- | --- |
| InterDigital | Proposal 2: CP-OFDM and DFT-s-OFDM are the only waveforms for uplink. Study enhancements for PAPR reduction for DFT-s-OFDM. |
| NEC | Proposal 1: CP-OFDM and DFT-s-OFDM in NR are baseline as 6GR uplink waveform. 6GR could study to support dynamic waveform switching during initial access. |
| NTT DOCOMO | Proposal 1: For 6GR study on waveform, - Only OFDM-based waveform(s) should be considered (as described in the SID) - Any new waveform(s), even for OFDM-based, should be justified by clear gain - Unified design across scenarios/use cases is strongly preferred - Following the above, RAN1 can carefully assess the need in 6GR to introduce waveform(s) beyond 5G NR, targeting, e.g., - Potential better coverage by better PAPR performance for uplink - Spectrum efficiency improvement |
| ITL | Proposal 1: For baseline waveform candidates for 6GR - CP-OFDM should be maintained as the baseline waveform for 6G downlink, ensuring backward compatibility, numerology continuity, and efficient support for MIMO/beamforming operations. - Both CP-OFDM and DFT-s-OFDM should be retained as baseline waveforms for 6G uplink, with optional low-PAPR enhancements (e.g., FDSS, FDSS-SE, frequency-selective mapping) studied as means to improve coverage and energy efficiency, particularly for low-power devices. - Fundamentally new waveform families should only be considered if they demonstrate clear and quantifiable performance advantages, while ensuring minimal disruption to NR compatibility and deployment feasibility. Proposal 2: For 6GR waveform with consideration of MRSS Compatibility - MRSS compatibility shall be a mandatory design principle for 6GR waveform development, ensuring seamless coexistence and migration with 5G. - CP-OFDM be maintained as the common baseline waveform structure for downlink, and that uplink baseline waveforms (CP-OFDM/DFT-s-OFDM) remain fully aligned with NR grid design to support MRSS operation. - Any optional enhancements (e.g., low-PAPR techniques) be evaluated under the condition that MRSS alignment is preserved, avoiding disruption to spectrum sharing or coexistence performance. - Fundamentally new waveforms not aligned with MRSS shall only be considered if they demonstrate clear, quantifiable gains that justify the additional complexity and coexistence risks. Proposal 3: For unified or multiple waveforms for 6GR - OFDM-based waveforms be adopted as the unified baseline for 6GR, ensuring simplicity, continuity, and compatibility with NR. - Multiple waveforms not be introduced as parallel baselines, as this would create fragmentation and increase implementation burden. - Optional complementary waveforms may be studied for specialized scenarios (e.g., NTN, ISAC, sensing, low-power IoT), but only considered if they provide clear and quantifiable performance advantages over the baseline. |

### Moderator proposal(s)

Check if there is a need for strengthening the RAN1#122 agreement further

Moderator proposal:

- CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported for communications in 6G uplink.

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded as potential additions.

## DFT-s-OFDM for DL

| Spreadtrum, UNISOC | Proposal 1：DL DFT-s-OFDM can be studied in 6GR for the following use cases and channel: - Target use case: 6G IoT, NTN, ISAC - Target channel：PDSCH |
| --- | --- |
| Vivo | Proposal 8: Transparent solutions are the baseline of DL low-PAPR waveform for coverage/NW energy saving motivation. Proposal 9: Support to study DFT-s-OFDM waveform for DL wake-up signal for UE energy efficiency. |
| Thales et al. | Proposal 2: Identify the set of NTN scenarios/use cases for which is beneficial to use DFT-s-OFDM in DL. Proposal 3: RAN1 to study the performance of DFT-s-OFDM in the downlink for non-terrestrial network (NTN)-based 6G radio access. |
| Xiaomi | Proposal 2: To support the coverage performance for NTN DL, low-PAPR waveform such as DFT-S-OFDM can be considered. Proposal 3: To support the coverage performance for NTN DL, low-PAPR waveform can be applied to the following channels: - PDCCH at least for CSS (except for type-3) - PDSCH with Msg 4 - PDSCH with SIB1 |
| Lekha Wireless Solutions | Proposal 1: DFT-s-OFDM has benefits in terms of PAPR and receiver performance. Due to the multi-carrier capabilities of CP-OFDM, it is more preferred in DL. However, there are use-cases like coverage-limited cells, small-cell BSs etc., where DFT-s-OFDM can help in power efficient transmission. |
| Huawei | Proposal 12: Study the DL DFT-s-OFDM (including enhancement) waveform for common channels for network energy saving and/or meeting coverage requirements. |
| CATT | Proposal 6: DFT-s-OFDM waveform for downlink is needed to increase the output power of the PA. Proposal 7: DFT-s-OFDM waveform can be applied in NTN downlink with introducing little complexity on the UE side to achieve significant power efficiency improvement. Proposal 8: For lager bandwidth transmission and enable scheduling flexibility, two segments DFT-S-OFDM can be studied. Proposal 9: For improving spectrum efficiency, multiplexing between DMRS and DFT-S-OFDM PUSCH data on a symbol can be studied. |
| Oppo | Proposal 6: DFT-s-OFDM is not supported as additional DL baseline waveform for 6GR, due to limited performance gain, restriction on multiuser scheduling and extra complexity on UE side. - DL DFT-s-OFDM for NTN can be further studied in NTN agenda. |
| LGE | - Proposal 6: DFT-s-OFDM waveform is supported as the additional basis for 6GR in downlink. - DFT transform precoding for DL is available at least for a single UE-dedicated PDSCH. - FFS: Whether or how the DFT transform precoding is applied to the common DL channels (e.g., PDSCH containing common signaling, SS/PBCH). - FFS: Whether or how the DFT transform precoding is applied to multiple PDSCHs for multiple UEs. |
| InterDigital | - Proposal 1: CP-OFDM is the only downlink waveform for 6GR; do not support additional DL waveforms for 6GR |
| NEC | - Proposal 6: Study the support of low PAPR waveforms like DFT-s-OFDM for 6G downlink transmissions. Proposal 7: Study a simplified framework for DL waveform support, where a default waveform is used for initial access, and UE-specific configuration for DFT-s-OFDM is performed semi-statically via RRC signaling. Proposal 8: Study the waveform design for PDCCH in deployments supporting DL DFT-s-OFDM, evaluating two approaches: - The use of CP-OFDM for PDCCH to ensure implementation simplicity and compatibility. - The feasibility of using DFT-s-OFDM for PDCCH to improve performance, including a detailed analysis of the required structural redesign, challenges in supporting multiple users, and the overall system impact. Proposal 9: Study multi-user scheduling techniques for downlink DFT-s-OFDM, including group-based or sub-band DFT, to balance multi-user throughput with low-PAPR properties. |
| Panasonic | Proposal 9: DL DFT-s-OFDM for individual signal / channel (such as LP-WUS / LP-WUR signal) are not required to be concluded for now. The important point it whether to support DFT spreading to overall channel’s method. Proposal 10: To support DFT spreading to overall DL channel’s method is excluded. |
| Ofinno | Proposal 6: Study DFT-s-OFDM as potential additional waveform for downlink in 6GR. |
| Lenovo | Proposal 2: Study adopting DFT-s-OFDM for at least DL data channel (PDSCH) for low data rate MBB, IoT, NTN. Proposal 3: For DFT-s-OFDM in DL, study adopting UE multiplexing using - TDM with per UE DFT - FDM with per UE DFT - FDM with DFT grouping of multiple UEs Proposal 4: Evaluate the feasibility of DFT-s-OFDM in DL for NTN and IoT use cases, focusing on coverage enhancement, NES, and UE power saving. |
| Ericsson | Proposal 1 RAN1 to deprioritize the study of DFT-s-OFDM for downlink due to lack of any significant advantage compared to CP-OFDM even for the potential use cases such as NES, NTN and ISAC, as well as to keep the 6GR system design to a reasonable complexity. |
| Google | Proposal 1: Support DFT-s-OFDM waveform as the DL waveform with regard to the following aspects: - To support the same coverage for FR1 and FR3 - To provide a good coverage for NTN - Compared to other coverage enhancement techniques, e.g., to increase the number of antennas or to transmit the DL signals by multiple repetitions, using DFT-s-OFDM waveform does not require additional complexity for complicated CSI calculation, does not require large delay for beam measurement or multi-repetitions-based DL signal reception, and does not require large overhead for complicated CSI report and more DL-RSs for beam measurement. |
| Ruijie Networks | Proposal 2: Study of multi-layer DFT-s-OFDM waveform for 6GR downlink. |
| Fainity | Proposal #2: For TN-NTN integration and coverage considerations, it is suggested that the downlink waveform set should, at least, include DFT-s-OFDM for the corresponding scenarios. Proposal #3: Regarding the study of DFT-s-OFDM enhancement, R1 is suggested to study multi-layer DFT-s-OFDM for DL transmission |
| Hanbat National University | Proposal 2: In addition to CP-OFDM, DFT-s-OFDM needs to be used as a potential additional waveform for the 6GR. |
| Indian Institute of Tech (M) | Proposal 3: 3GPP should consider the use of DFT-s-OFDM in DL/UL at least for NTN and FR2 use-cases. |
| CEWiT | Proposal 1: Support usage of DFT-s-OFDM in DL. |
| IIT Kanpur | Proposal 2: It is proposed to further study DFT-s-OFDM and consider it as a downlink waveform candidate for 6GR, with a focus on its potential benefits in improving energy efficiency and extending coverage. |

### Moderator proposal(s)

Identify the different DFT-s-OFDM use cases and solution candidates

Moderator proposal: Discuss the following alternatives

Target use cases for DFT-s-OFDM for DL:

- IoT

- NTN

- MBB (low data rates)

- FR2

- Network Energy Saving

- UE low power receiver (Wake-up signal)

- (ISAC)

Target channels

- Unicast PDSCH

- SIB1 PDSCH

- Msg4 PDSCH

- CSS PDCCH

- “Common channels”

- Wake-up signal

- Reference signals

Moderator proposal

- No need to discuss transmit processing of individual signals such as wake-up signals under the waveform agenda item

## Other waveforms

| Vivo | Proposal 6: Support to study AI/ML-based waveform enhancements. |  |
| --- | --- | --- |
| ZTE |  | Proposal 6: DFT-s-OFDM with enhanced time domain resource multiplexing in symbol-level (i.e., eDFT-s-OFDM waveform) can be considered as a candidate waveform technology for 6G waveform design to at least improve the performance in high-speed scenario. Proposal 7: GFB-OFDM should be considered in 6G waveform study as a scheme to support wideband transmission and flexible subband configuration. |
| BUPT |  | Proposal 1 Postpone the study of 6GR waveforms for high mobility enhancements for future releases. |
| LGE |  | - Proposal 5: Study the benefits of Spread OFDM compared to CP-OFDM and DFT-s-OFDM. |
| Cohere |  | - Proposal 1: Zak-OTFS and Zak-OTFS-over-OFDM are considered as 6G potential waveforms and are included in the 6G waveform study. |
| ETRI, University of Surrey |  | - Proposal 1. RAN1 to consider AFDM as an additional waveform candidate for 6G radio. |
| Shanghai Jiao Tong University, NERCDTV |  | Proposal 1: RAN1 to study additional waveform candidate (e.g., AFDM) for 6G that stays structurally compatible with OFDM, enabling reuse of 5GNR/6GR ecosystem components, while targeting enhanced performance in sensing, high mobility, and NTN scenarios. Proposal 2: RAN1 to investigate AFDM as a candidate waveform for 6G radio, with a specific focus on evaluating its robustness in high-mobility scenarios, high-efficiency sensing capabilities, low PAPR characteristics, and integration into an 6GR compatible system architecture. Proposal 3: RAN1 to study and specify the design of AFDM parameters (e.g., c1 and c2), low-complexity receiver algorithms for communication, sensing, and PAPR reduction, and their integration into a 6G-compatible system architecture, to enhance robustness against doubly-selective channels while targeting superior performance in sensing, high-mobility, and NTN scenarios. - Proposal 4: RAN1 to investigate the implications of AFDM on MIMO channel estimation, receiver signal processing, etc., and to study low-complexity techniques to ensure the efficient integration of AFDM with existing multi-antenna systems. |
| Apple |  | Proposal 8: Consider DFT-s-OFDM GMSK as one of the candidate waveforms for low PAPR waveform in uplink. |
| Qualcomm |  | Proposal 3: In 6GR study on waveforms, focus on enhancements to the DFT-S-OFDM family of waveforms. |
| Sharp |  | Proposal 1: RAN1 should study Interlace OFDM for 6GR. |
| NICT |  | Proposal 1: RAN1 to discuss waveforms that is to supress OOBE without affecting signal quality. Proposal 2: RAN1 to consider SP-DFT-s-OFDM as one of candidate UL waveforms for 6GR. Proposal 3: RAN1 to consider SP-OFDM as one of candidate DL and UL waveforms for 6GR |
| Hanbat National University |  | Proposal 1: In addition to CP-OFDM and DFT-s-OFDM, RAN1 is encouraged to study additional OFDM-based waveform for diverse deployment scenarios requiring robustness against timing/frequency offsets and potential to reduce guard band requirements. |
| Indian Institute of Tech (M) |  | Proposal 4: Study the use of single carrier TDMA bursts in the current frame structure for NTN. |
| CEWiT |  | Proposal 2: Support for OFDM-OOK kind of waveforms for low end devices. |
| Wisig, IITH |  | Proposal-1 (π/2-BPSK DFT-s-OFDM F0) replaces F0 with a cell-specific base sequence combined with a pi/2 BPSK orthogonal sequence set, enabling reliable multi-bit encoding over short resources while preserving near-saturation PA operation. Proposal-2 (π/2-BPSK DFT-s-OFDM F1) maintains the long-format structure (4–14 symbols, 2 or more bits) with TDM π/2-BPSK DM-RS, time-domain OCC for optional multi-UE multiplexing, and intra-slot frequency hopping for diversity. Proposal-3 (π/2-BPSK OTFDM single-symbol) time-multiplexes π/2-BPSK DM-RS and control within a single (or two) symbol(s) before DFT, enabling true one-shot UCI with optional OCC and strong spectrum shaping to minimize PAPR. |

### Moderator proposal(s)

List the candidate proposals for new waveforms reduction that can be considered for further studies:

- AFDM

- DFT-s-OFDM

- DFT-s-OFDM GMSK

- eDFT-s-OFDM (DFT-s-OFDM with enhanced time domain resource multiplexing in symbol-level)

- GFB-OFDM

- OFDM-OOK (for low end devices)

- OSDM

- Interlace OFDM

- Single carrier TDMA (for NTN)

- SP-DFT-s-OFDM (for UL)

- SP-OFDM (for DL and UL)

- Spread OFDM

- Zak-OTFS

- OTFDM

## Low-PAPR for DFT-s-OFDM

| Nokia | Proposal 3: Net gain is used to evaluate coverage gain of waveform enhancements. Proposal 4: RF-simulations are used to obtain Tx power gain for net gain evaluation. - Proposal 5: Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio. Proposal 6: Transparent filtering approach assumed for FDSS and FDSS-SE in 6G Radio. |
| --- | --- |
| vivo | Proposal 1: Study waveform adjustments to achieve high UE power efficiency and UL coverage. Proposal 2: The net gain defined as the MPR gain minus the SNR loss required to maintain a target BLER of 0.1 should be evaluated to validate benefits of candidate solutions. Proposal 3: Memoryless PA model defined in RAN4 can be used for MPR evaluation for 6GR. Proposal 4: Study the use of PAPR and DCM gains as an alternative to MPR gain in RAN1. Proposal 5: Support to study CFR-SE based low-PAPR waveform for 6GR UL. |
| ZTE | Proposal 1: To improve coverage, tone reservation should be considered in 6G waveform design as a low-complexity scheme to reduce PAPR along with the compatibility with both UL and DL waveforms. Proposal 2: To improve coverage, Selected Mapping(SLM) should be considered in 6G waveform design as a low-complexity scheme to reduce PAPR along with the compatibility with both UL and DL waveforms. Proposal 3: To improve coverage, FDSS can be considered in 6G waveform design. - FDSS w/o SE is considered as the baseline. Proposal 4: To improve coverage, joint optimization of waveform and modulation can be considered in 6GR. |
| CMCC | Proposal 4. The study focuses on PAPR reduction for DFT-s-OFDM waveform. Proposal 5. The proposed design of FDSS and tone reservation should clarify the difference and improvement comparing to the corresponding work already been done in NR. |
| Lekha Wireless Solutions | Proposal 3: UE transmit power is limited; hence uplink need waveforms with low PAPR for efficient PA usage, robust to mobility, CFO, and Doppler and need spectral efficiency and coexistence with other UL transmissions. Enhancements to current waveforms can be done using windowing and filtering techniques, CP length variations, SC-FDMA variants, and DFT-s-OFDM. Proposal 4: Due to restricted power requirements in uplink, DFT-s-OFDM was chosen for LTE UL instead of plain OFDM because it already offers lower PAPR. Still, additional PAPR reduction techniques are desired. Some good PAPR reduction techniques include DFT precoding, sub-carrier mapping, tone reservation or injection, clipping and filtering, selective mapping, partial transmit sequence, and precoding-based techniques. |
| Tejas Network | Proposal 1: Study of DFT-s-OFDM enhancements for uplink is crucial for realizing the vision of 6GR, particularly to meet the low PAPR requirements essential for improved UL coverage. Proposal 3: A thorough study on the integration of FDSS with DFT-s-OFDM is essential, as it directly addresses critical 6GR KPIs. Careful examination of design trade-offs such as spectral efficiency versus PAPR reduction will be necessary to optimize performance across diverse 6GR use cases. |
| Huawei | Proposal 8: Study how to utilize the signal property of frequency redundancy in the spectrum extension based schemes to enhance multi-user experience. For example, scheduled PRBs for multiple UEs are overlapped partially. Proposal 9: Study I/Q-offset DFT-s-OFDM for lower PAPR DFT-s-OFDM enhancement. Proposal 10: Study pruning QAM under DFT-s-OFDM (including enhancement) waveform for coverage enhancement. |
| CATT | Proposal 1: Lower PAPR schemes shall be studied due to following aspects in 6GR: - Larger FFT size (e.g. from 4096 to 8192) - Larger transmission channel bandwidth - Increased downlink free space path loss in NTN due to large propagation distance. Proposal 4: If FDSS-SE is adopted by 6GR, the scheme on spectrum efficiency improving shall be studied. Proposal 5: It is proposed to study 32-QAM schemes for PAPR reduction for DFT-S-OFDM in 6GR. |
| BUPT | Proposal 3 Allow for non-transparent DMRS (Not necessarily the exactly same signal processing restriction between reference signal and data channel) against 5G NR at least for the evaluation purpose. |
| Oppo | Proposal 7: Postpone the study on UL PAPR reduction schemes to RAN1#124 meeting. - Prioritize the implementation-based schemes without specification impacts. - Study additional gain from schemes with specification impacts. |
| Samsung | Proposal 1: Study low PAPR uplink waveform based on DFT-s-OFDM for 6GR uplink coverage enhancement. Proposal 2: Study frequency domain spectrum shaping (FDSS) on DFT-s-OFDM to reduce PAPR for 6GR uplink coverage enhancement. |
| InterDigital | Proposal 2: CP-OFDM and DFT-s-OFDM are the only waveforms for uplink. Study enhancements for PAPR reduction for DFT-s-OFDM. Proposal 4: The following KPIs relevant for communication should be evaluated when studying PAPR reduction techniques or a new waveform: - Spectral efficiency (bps/Hz) - BLER - Cubic metric - PAPR Proposal 6: Study FDSS with spectrum extension as potential candidate for uplink PAPR reduction for DFT-s-OFDM to support coverage enhancement for 6G |
| KT Corp. | Proposal 1: Study methods that improve spectral efficiency while preserving the benefits of low PAPR, with careful evaluation of trade-offs, complexity, and signaling impact. |
| NEC | Proposal 3: Study the use of Frequency Domain Spectrum Shaping (FDSS) for DFT-s-OFDM in the 6GR uplink to enhance coverage and power efficiency. Proposal 4: The evaluation of FDSS enhancements must incorporate realistic PA models and be validated against RF conformance requirements, including ACLR and EVM. Proposal 5: Study a non-transparent FDSS operation for 6GR, including the signaling of the applied shaping filter, to enable advanced receiver equalization and unlock greater performance benefits. |
| Panasonic | Proposal 6: At least the following evaluation criteria should be considered in the waveform evaluation. - MRSS compatibility - Complexity - Flexible time and frequency domain resource allocation - Specification impact - MIMO capability - Spectral efficiency - Coverage - Net gain - MPR / PAPR - BLER Proposal 7: For MPR / PAPR analysis, how relationship with ACLR, SEM, EVM, spurious emissions, and occupied bandwidth progress will require discussion in RAN1 as PAPR only does not determine the amount of UE Tx power. Proposal 8: RAN1 should assess the need to introduce MPR / PAPR reduction techniques targeting coverage enhancement for UL. |
| Ofinno | Proposal 1: Consider frequency domain spectrum shaping (FDSS) as a candidate scheme for PAPR reduction for DFT-s-OFDM. Proposal 2: Consider FDSS with spectrum extension (FDSS-SE) as a candidate scheme for PAPR reduction for DFT-s-OFDM with at least /2-BPSK and QPSK. |
| Lenovo | Proposal 5: Study and evaluate CP-OFDM waveform enhancement techniques including PAPR/CM reduction techniques such as Selected Mapping (SLM) and Tone Reservation (TR) for coverage enhancement and energy efficiency improvement, and compare to implementation-based techniques in terms of complexity, signal distortion, and spectral efficiency. Proposal 6: Study enhancing DFT-s-OFDM waveform by incorporating PAPR/CM reduction techniques such as FDSS, DFT precoder extension, etc. Proposal 7: The study and evaluation of waveform enhancements should focus on CM characteristic of the waveform |
| Ericsson | Proposal 2 To evaluate different coverage enhancement techniques via PAPR reduction for 6GR, RAN1 to consider MPR-based and system-level simulations. |
| Sony | Proposal 2: RAN1 should study PAPR reduction for DFT-s-OFDM when used with higher order modulation. |
| MediaTek | Proposal 1: Support O-QPSK and O-QAM modulated DFT-s-OFDM for coverage enhancement. Proposal 2: For π/2-BPSK modulated DFT-s-OFDM, support subcarrier truncation for spectral efficiency enhancement. |
| Apple | Proposal 1: The following table details the motivations of Low PAPR waveforms for 6G - Motivation/Information Details - Targeted link direction UL - Motivation Uplink Coverage Enhancement, Energy Efficiency, Mitigation of PA non-linearity - MRSS compatibility Yes - Target channels/signals PUSCH - MIMO (SU and MU-MIMO) compatibility N/A - Target modulations pi/2-BPSK, QPSK - Multi-user multiplexing/scheduling flexibility same as baseline DFT-S-OFDM - Multiplexing/coexistence with baseline waveforms Yes - Impact on synchronization and initial access N/A - Expected specification impact RAN1 Specification, RAN4 Requirements - Receiver Complexity Transparent, Non-transparent - Impact to power consumption Improved PA efficiency Proposal 6: Study UL DFT-S-OFDM with support for FDSS, FDSS-SE and GMSK approximation filters. Proposal 7: Study both transparent and non-transparent receivers Proposal 9: Support specification non-transparent filtering for PAPR reduction in UL DFT-S-OFDM |
| Qualcomm | Proposal 4: For 6GR, study the family of low PAPR waveforms obtained using DFT-S-OFDM with Pi/2 BPSK and truncated mapping. Proposal 7: For 6GR waveform study, for DFT-S-OFDM waveforms, decouple the size of allocation from the DFT size. Define any DFT size that is a product of powers of 2, 3 and 5 as a valid DFT size. Proposal 8: For 6GR waveform study, when considering DFT-S-OFDM waveforms, consider flexible frequency-domain mapping of the DFT output to the spectrum allocation, e.g., frequency-domain multiplexing of DMRS and data, non-contiguous mapping, etc. Proposal 9: For 6GR waveform study, consider multi-tx enhancements for DFT-S-OFDM where different transmit ports transmit over different frequency domain allocations. Proposal 10: For 6GR waveform study, consider feasibility to enhance spectrum utilization for small channel bandwidths using spectrum confinement techniques (e.g. WOLA) of reasonable complexity. |
| Fainity | Proposal #4: R1 is suggested to study how to enhance DFT-s-OFDM for achieving higher energy efficiency and lower PAPR. Proposal #6: R1 is suggested to study an enhanced DFT-s-OFDM be additionally needed or the enhanced DFT-s-OFDM can replace DFT-s-OFDM for the UL waveform set. |
| Indian Institute of Tech (M) | Proposal 1: 3GPP should study the option of enabling mechanisms for PAPR reduction techniques both in CP-OFDM and DFT-s-OFDM Proposal 2: PAPR reduction techniques such as FDSS, FDSS-SE, CFR and CFR-SE along with their specification impacts should be studied. |

### Moderator proposal(s)

List the candidate proposals for DFT-s-OFDM PAPR reduction that can be considered for further studies:

Moderator proposal: The following DFT-s-OFDM PAPR reduction techniques can be considered for further studies:

- Crest Factor Reduction (CFR) with spectrum extension

- DFT precoder extension

- Flexible frequency-domain mapping of the DFT output to the spectrum allocation,

  - frequency-domain multiplexing of DMRS and data,

  - non-contiguous mapping.

  - Decoupling frequency allocation from the DFT size

- Flexible time-domain resource mapping of the DFT

- Frequency domain spectrum shaping (FDSS)

- FDSS with spectrum extension

  - Symmetric spectrum extension

  - Cyclic spectrum extension

- GMSK approximation

- I/Q-offset DFT-s-OFDM

- Interpolation-based π/2-BPSK and QAM DFT-s-OFDM

- O-QPSK and O-QAM modulated DFT-s-OFDM

- Pruning QAM (e.g. 32QAM) modulated DFT-s-OFDM

- Selected mapping (SLM)

- Tone Reservation (TR)

- Subcarrier truncation for π/2-BPSK modulated DFT-s-OFDM

- Multi-Tx enhancements: different transmit ports transmit over different frequency domain allocations.

## Low-PAPR for CP-OFDM

| ZTE | Proposal 1: To improve coverage, tone reservation should be considered in 6G waveform design as a low-complexity scheme to reduce PAPR along with the compatibility with both UL and DL waveforms. Proposal 2: To improve coverage, Selected Mapping (SLM) should be considered in 6G waveform design as a low-complexity scheme to reduce PAPR along with the compatibility with both UL and DL waveforms. |
| --- | --- |
| Lekha Wireless Solutions | Proposal 2: Plain OFDM works well in DL but has some limitations such as high PAPR, sensitivity to phase noise and Doppler, out-of-band emissions, and limited flexibility to mixed numerologies. Hence, enhancements and alternative OFDM-based schemes need to be explored including windowing and filtering, sub-band filtering, cyclic prefix-based enhancements, precoding and MIMO enhancements, and DFT-s-OFDM. |
| CMCC | Proposal 4. The study focuses on PAPR reduction for DFT-s-OFDM waveform. |
| Tejas Network | Proposal 2: A study on enhancing DFT-s-OFDM for downlink in NTN use case would be highly valuable, particularly when power efficiency and coverage are the primary priorities. |
| CATT | Proposal 1: Lower PAPR schemes shall be studied due to following aspects in 6GR: - Larger FFT size (e.g. from 4096 to 8192) - Larger transmission channel bandwidth - Increased downlink free space path loss in NTN due to large propagation distance. Proposal 2: If Selective Mapping (SLM) is adopted for CP-OFDM waveform in 6GR, scheme on reducing the overhead of side information shall be studied. Proposal 3: It is proposed to study Tone Reservation (TR) for CP-OFDM waveform in 6GR. |
| Lenovo | Proposal 5: Study and evaluate CP-OFDM waveform enhancement techniques including PAPR/CM reduction techniques such as Selected Mapping (SLM) and Tone Reservation (TR) for coverage enhancement and energy efficiency improvement, and compare to implementation-based techniques in terms of complexity, signal distortion, and spectral efficiency. |
| Sony | Proposal 1: RAN1 should study PAPR reduction for CP-OFDM that can be applied to the NTN DL. |

### Moderator proposal(s)

List the candidate proposals for CP-OFDM PAPR reduction that can be considered for further studies:

Moderator proposal: The following CP-OFDM PAPR reduction techniques can be considered for further studies:

- Tone Reservation (TR)

- Selected Mapping (SLM)

## UE power class, power boosting and PA model

| Nokia | - Proposal 7: High power class should be the baseline for 6G due to significant enhancement in coverage. Proposal 8: Power boosting features such as the ones specified in NR should be part of the baseline for 6G. |
| --- | --- |
| Huawei | Proposal 2: For RAN1 waveform evaluations, the NR PA model recommended in R4-164542 for below 6GHz is reused at least until any new PA model is updated by RAN4. Proposal 3: Send LS to RAN4 to ask about any updates to PA models, RF requirements and RB allocations for MPR evaluations. |
| Ofinno | Proposal 3: Study the possibility of introducing high power UE (e.g., 26 dBm) as mandatory feature in 6GR from Day 1. Proposal 5: Study the possibility of reduced UE MPR. |
| Apple | Proposal 5: Send a Liaison Statement (LS) to RAN4 to consider the appropriate high-power PA model and the associated RF requirements for a power boost feature in 6G. |
| Hanbat National University | Proposal 4: RAN1 to request RAN4 input on power class and MPR-related parameters. |

### Moderator proposal(s)

The WID states that RAN1 is to coordinate with RAN4 early to be able to provide interim assessment on e.g. waveform evaluation by June 2026. In accordance to this, a number of companies note that a UE PA model and other RAN4 aspects should be requested from RAN4.

Moderator proposal for Monday online:

- Send an LS to RAN4 for appropriate PA model(s) (for both UE and network side models) and other relevant RF parameters to use in waveform evaluations.

## DFT-s-OFDM with Rank > 1 for UL

| Nokia | Proposal 10: Low PAPR waveform study, if any, for multi-layers UL transmission in 6G needs to consider at least fully coherent and partially coherent UEs. Proposal 11: For study of UL multi-layer MIMO enhancement with low PAPR DFT-s-OFDM waveform, the power-limited UEs and power non-limited UEs should be considered, and the net gain should be evaluated as follows: Power limited UE Power non-limited UEs MIMO Net gain [dB] Tx power difference [dB] – Rx SINR Rx SINR difference for 10% BLER [dB] difference for 10% BLER [dB] Proposal 12: Limit UL DFT-s-OFDM to Rank=1 in 6G. |
| --- | --- |
| Vivo | Proposal 7: Support rank 2 for DFT-s-OFDM waveform |
| ZTE | Proposal 5: DFT-s-OFDM with rank 2 for uplink transmission can be considered in 6G waveform study. |
| CMCC | Proposal 6. The target use cases are recommended to be clarified and divided between multi-layer CP-OFDM and multi-layer DFT-s-OFDM before detailed technical study on multi-layer DFT-s-OFDM. |
| Huawei | Proposal 11: At least 2-layers uplink DFT-s-OFDM (including enhancement) waveform with observed coverage gains should be further studied in 6GR MIMO session, e.g., codebook design. |
| LGE | - Proposal 2: Study the supported rank of each waveform (i.e., DFT-s-OFDM and CP-OFDM) for 6GR in uplink. - Option 1: DFT-s-OFDM is used for lower ranks (e.g., up to N, where N < 3), while CP-OFDM is used for both lower and higher ranks. - Option 2: DFT-s-OFDM is used for lower ranks, and CP-OFDM is used for higher ranks only. - Option 3: Both DFT-s-OFDM and CP-OFDM are used across lower and higher ranks. |
| Samsung | Proposal 3: To assess whether multi-rank DFT-s-OFDM can offer significant spectral efficiency gain, focus the study on rank-2 (two-layer uplink transmission on PUSCH) - Proposal 4: To assess whether rank-2 DFT-s-OFDM can offer significant uplink spectral efficiency gain, further investigate its performance in deployment scenarios with primarily line-of -sight channels |
| Ericsson | Proposal 3 Support multi-layer DFT-s-OFDM with at least rank 2 for uplink transmissions in 6GR. |
| Google | Proposal 2: Support the DFT-s-OFDM waveform for multiple layers for UL transmission. |
| Sony | Proposal 3: RAN1 should study multi-layer transmission with DFT-s-OFDM. |
| Ruijie Networks | Proposal 1: Study of multi-layer DFT-s-OFDM waveform for 6GR uplink. |
| Qualcomm | Proposal 5: For 6G Radio, support multi-layer DFT-S-OFDM transmissions in uplink. Proposal 6: Study feasibility of using non-identity precoders for multi-layer DFT-S-OFDM transmissions with focus on the PAPR-precoding gain trade-off of using such precoders. |

### Moderator proposal(s)

Moved to section 4

## Waveform switching

| Nokia | Proposal 9: Dynamic waveform switching is introduced to 6G in the first release. |
| --- | --- |
| LGE | - Proposal 3: Discuss how the UE should be instructed to use a particular waveform for 6GR in uplink. - Option 1: Waveform selection based on transmission rank. - Option 2: Explicit waveform indication via cell-specific configuration, channel-specific, or BWP-specific configuration, including dynamic switching. Option 3: Waveform selection based on frequency band or usage scenario. |
| Xiaomi | Proposal 4: The UL coverage enhancement mechanism in Rel-18 including power domain solution and waveform switch related solution can be taken as a starting point for UL coverage performance guarantee for both TN and NTN. |
| InterDigital | Proposal 3: Support dynamic waveform switching for the uplink for 6GR |
| NEC | Proposal 1: CP-OFDM and DFT-s-OFDM in NR are baseline as 6GR uplink waveform. 6GR could study to support dynamic waveform switching during initial access. |
| Google | Proposal 3: Postpone the dynamic waveform switching related discussion until most of the details for each waveform are finalized. |
| Fainity | Proposal #5: R1 is suggested to study simplified and smooth UL waveform switching mechanisms. |
| Indian Institute of Tech (M) | Proposal 5: 3GPP should support multiplexing of waveforms as required. |
| CEWiT | Proposal 3: Support for waveform selection for different time/frequency/physical channels/physical signals. |
| IIT Kanpur | Proposal 1: An efficient waveform multiplexing mechanism needs to be studied for 6GR. |

### Moderator proposal - deprioritize

Postpone the discussion on waveform switching/selection until the waveform decisions have stabilized.

## Sensing

| ZTE | Proposal 8: Study pulse RS design using OFDM-based waveform for large sensing coverage. - The pulse is applicable for both monostatic and bistatic sensing. - Study the application for communication, e.g., RSRP measurement, time/frequency tracking. |
| --- | --- |
| Huawei | Proposal 6: ISAC waveform design should consider AF (including Main lobe width, PSLR, ISLR), PAPR as sensing performance evaluation metrics. Proposal 7: Take Table 5 as the start point for ISAC waveform evaluation. Proposal 13: Shared communication and sensing waveforms should be studied given the low resources overhead for the support of sensing and communication co-existence, where both communication and sensing requirements for the waveform design should be considered. Proposal 14: The shared communication and sensing waveform study should be together with the waveform study for communication only, considering the mutual effect to each other. Proposal 15: Study CP-OFDM enhancement such as pruning QAM by considering both communication spectral efficiency and sensing performance. Proposal 16: Study frequency modulus ripple reduction techniques for DFT-s-OFDM (including enhancement) waveforms for sensing performance improvement. |
| CATT | Proposal 10: For supporting sensing, OFDM-based wave can be supported for both pulse wave (PW) and Continuous wave (CW): - OFDM-based PW with larger SCS (i.e. 960 kHz or 1920 kHz) than communication - OFDM-based CW with same SCS as communication Proposal 11: For enhance the sensing, the new waveform different to OFDM can be studied, such as LFM (Linear Frequency Modulation), AFDM (Affine Frequency Division Multiplexing) and OCDM (Orthogonal Chirp Division Multiplexing). |
| LGE | Proposal 7: A new waveform such as FMCW is studied for sensing as well as OFDM. |
| Oppo | Proposal 2: Study waveforms to fulfil the requirement of 6G Sensing and 6G NTN (Ubiquitous Connectivity). - Strive for reusing the 6GR baseline waveform for 6G Sensing and 6G NTN. - An additional waveform can be considered if significant gain over the baseline waveform can be justified for a specific vertical scenario, but only supported by the vertical BS/UE. |
| InterDigital | Proposal 5: Waveform for sensing is not covered in Agenda Item 11.3.1 and shall be studied separately in Agenda Item 11.14 |
| Ofinno | Proposal 4: Support dynamic switching between DFT-s-OFDM and CP-OFDM from Day 1. |
| MediaTek | Proposal 3: The 6GR communication waveform design can be a baseline for sensing, and any enhancement or sensing specific waveform design, if needed, will be discussed in sensing specific agenda, i.e., AI 11.14. |
| Qualcomm | Proposal 1: For 6G Radio waveform study, limit initial focus to waveform design for communication use cases. Waveforms and specific waveform enhancements for other use cases such as sensing to be discussed separately. |
| Fainity | Proposal #1: R1 is suggested to first focus on the communication waveform in the early stage of 6GR waveform discussions. |
| Hanbat National University | Proposal 5: RAN1 to study essential extensions of CP-OFDM/DFT-s-OFDM (e.g., windowing techniques, dedicated sensing reference signals, PAPR reduction methods) to achieve target sensing performance. |
| IIT Kanpur | Proposal 3: To support Integrated sensing and communication (ISAC) in 6G, it is proposed to study the use of OFDM–LFM as a joint waveform for sensing and high-throughput communication. |

### Moderator proposal - deprioritize

As the sensing agenda item 11.14 will come online on RAN1#124bis in April 2026, postpone proposals specific to sensing until then.

## Requirements for 6G waveform

| Tejas Networks | Proposal 4: A systematic study of waveform selection based on device type is essential to meet the diverse requirements of 6GR. Careful evaluation and mapping by requirements (power budget, mobility, bandwidth, MIMO) with device types to suitable waveforms will be key to achieving 6GR KPIs including spectral efficiency, coverage extension, power efficiency, and integrated sensing capabilities. |
| --- | --- |
| BUPT | Proposal 2 Strive for PAPR performance of potential additions of 6GR waveforms agnostic of modulation constellations with given modulation level. Proposal 4 The PAPR performance of reference signal should be no worse than the associated data channel. Proposal 7 The PAPR gain of additions of 6GR waveforms should be decent. Proposal 8 The PAPR performance of reference signal should be no worse than the associated data channel. Proposal 9 The spectrum efficiency loss (if occurred) of potential additions of 6GR waveforms for all MCS levels should be imperceptible (e.g. less than 0.1dB) regardless of the exact value of PAPR gain. Proposal 10 Continue to optimize and evaluate spectrum efficiency and PAPR performance tradeoff of potential additions of 6GR waveforms with the Generic DFT-S-OFDM framework as starting point. |
| Oppo | Proposal 1: A unified 6GR baseline waveform is studied to fulfil the requirements of eMBB and 6G IoT. - The baseline waveform is used for 6G HRLLC. Proposal 3: For studying the additional waveform for 6GR, evaluate waveform proposals using agreed 6GR waveform (i.e., CP-OFDM for DL and CP-OFDM/DFT-s-OFDM for UL) as the benchmark, with the consideration of following: - Spectrum efficiency. - Coverage. - NW and UE side complexity. - Compatibility and neutrality for proposals in other areas, i.e., no restriction to or bundling with specific proposals for 6G MIMO, modulation, channel coding, AI/ML enhancements, etc. - Support flexible frequency-domain (e.g., RB-level) and time-domain (e.g., symbol-level) resource allocation. - Support of efficient 5G/6G spectrum sharing. Proposal 4: For studying the 6GR baseline waveform, support up to 2 waveforms in DL and up to 2 waveforms in UL, e.g., one optimized for spectrum efficiency, one optimized for coverage. - At least 1 waveform in DL and 1 waveform in UL are mandatorily supported for all device types, e.g., CP-OFDM in DL and DFT-s-OFDM in UL. - The 2nd waveform can be considered for 6G eMBB s which shares the processing units with the 1st waveform as much as possible. Proposal 5: Only one DL waveform is supported for 6GR initial access procedure. Proposal 8: Study multiple access (MA), targeting a single MA scheme for each waveform, to fulfil the requirement of all 6G usage scenarios using this waveform. Proposal 9: Orthogonal multiple access (OMA) is the baseline for 6GR. Evaluate OMA proposals using 5G NR as the benchmark, with the consideration of following: - Spectrum efficiency. - Coverage. - NW and UE side complexity. - Compatibility and neutrality for proposals in other areas, i.e., no restriction to or bundling with specific proposals for 6G MIMO, modulation, channel coding, AI/ML enhancements, etc. - Support flexible frequency-domain (e.g., RB-level) and time-domain (e.g., symbol-level) resource allocation. - Support of efficient 5G/6G spectrum sharing. |
| LGE | - Proposal 1: Following principles form the foundation for the waveform study in 6GR and guide the evaluation of both continuity with 5G NR and the exploration of new waveform candidates. - To ensure smooth evolution and coexistence with legacy networks, waveform design must maintain compatibility with 5G NR wherever possible. - Minimize complexity and support diverse 6G services such as TN/NTN integration and massive IoT. - Future enhancements or new signal/channel structures should avoid significant increases in implementation complexity to ensure broad feasibility and scalability. - Proposal 4: Discuss whether reference signal design should consider commonality across CP-OFDM and DFT-s-OFDM in both uplink and downlink. |
| NEC | Proposal 2: 6GR strives for a unified waveform baseband generation and upconversion for all channels and signals including PRACH. |
| Panasonic | Proposal 1: 6GR should allow certain time / frequency resources can be different waveform for forward compatibility perspective and to support MRSS. Proposal 2: For 6GR waveform design, time/frequency grid should be allowed to be aligned and orthogonal with NR boundary. Proposal 3: OFDM-based waveform should be supported for 6GR. - The definition of “OFDM-based” is to have subcarrier mapping and IFFT to generate time-domain signal. Proposal 4: Striving for OFDM-based waveforms across all the identified use cases can be sufficient at least for 6G Day 1. Proposal 5: Any enhancements to CP-OFDM or DFT-s-OFDM and/or any newly introduced waveform must demonstrate clear and justified advantages over 5G waveform. |
| Ofinno | Proposal 7: Study use cases which may require larger SCS and/or extended CP in conjunction with the waveforms for communication in 6GR. |
| Lenovo | Proposal 1: Study waveform enhancement techniques targeting 6GR coverage enhancement, energy efficiency improvement and support of sensing while maintaining compatibility with current waveforms’ structures, complexity constraints, and support of MRSS. |
| Rakuten | Proposal 1: In consideration of NR waveform extension and alternative candidates, RAN1 focuses on the following scenarios and channel conditions: - High-Mobility DL Scenarios - including assessment on the resilience to Doppler shifts and inter-carrier interference - Fragmented Spectrum and Sparse Access - including assessment on the adaptability for spectrum allocation in non-contiguous or opportunistic bands. - Low-SNR and Edge-of-Cell Conditions - including investigation on the performance in coverage-limited areas accounting for robustness and power efficiency. - JSAC and Multi-Service Integration - including exploration of structures supporting simultaneous sensing and communication. Proposal 2: In consideration of NR waveform extension and alternative candidates, RAN1 studies the following aspects for the impacts on spectrum sharing and compatibility with NR: - Waveform Coexistence and Guard Band Design - including analysis on the coexistence of CP-OFDM and candidate waveforms within the same band. - Numerology and Timing Alignment - including investigate timing and subcarrier spacing alignment across waveforms. - Control and Data Channel Multiplexing - including assessment on control channel decoding, synchronization signal design, and cross-carrier scheduling impacted by waveform diversity. Proposal 3: In consideration of NR waveform extension and alternative candidates, RAN1 studies the following aspect for the impacts on complexity and power consumption from the Physical Layer perspective: - Power Amplifier Efficiency and PAPR Impacts - including analysis of PA linearity and energy drain. - Baseband Processing Load - including assessment of computational complexity of channel estimation, equalization, and demodulation for candidate waveforms. - RF Front-End Duty Cycles - including analysis of RF activity patterns. - Note: RAN4 may be involved in - Thermal and Energy Budget Constraints - including analysis of power impacted of waveform processing under typical traffic loads. |
| Sony | Proposal 4: RAN1 should study multiplexing of CP-OFDM for reference signals with DFT-s-OFDM for physical channels on the same component carrier. |
| Qualcomm | Proposal 2: Design considerations for 6G waveform study for communication purposes to include: - new spectrum bands and associated requirements, e.g large BW - needs for new deployment scenarios, e.g. suburban macro, FWA, etc. - duplex operation, e.g., subband full duplex - enhancing coverage, e.g. design of low PAPR waveforms - Support for high power transmissions in uplink, e.g., higher power classes, MPR optimizations - integration with use cases such as sensing and positioning - Co-channel and adjacent channel requirements - Support for spatial multiplexing, beamforming, multiple access - Transceiver complexity associated with synthesis and reception; processing latency - Energy/power efficiency - Considerations on backward compatibility and coexistence with 5G - Scheduling flexibility and agility |
| AT&T | Proposal 1: Proponent companies to provide details of enhancements/modifications on CP-OFDM/DFT-s-OFDM compared with CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR (for 6GR uplink) Proposal 2: Proponent companies to provide details of enhancements/modifications on CP-OFDM compared with CP-OFDM waveform as defined in 5G NR (for 6GR downlink) Proposal 3: Proponent companies to provide justification for the enhancements/modifications on CP-OFDM/DFT-s-OFDM, and how to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off, compared with CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR (for 6GR uplink) Proposal 4: Proponent companies to provide justification for the enhancements/modifications on CP-OFDM, and how to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off, compared with CP-OFDM waveform as defined in 5G NR (for 6GR downlink) |
| Fainity | Proposal #1: R1 is suggested to first focus on the communication waveform in the early stage of 6GR waveform discussions. |
| Sharp | Proposal 2: To avoid excessive configurations, excessive UE capabilities and UE capabilities reporting, 6G waveforms should be applied to diverse use cases/device types. |
| NTT DOCOMO | Proposal 1: For 6GR study on waveform, - Only OFDM-based waveform(s) should be considered (as described in the SID) - Any new waveform(s), even for OFDM-based, should be justified by clear gain - Unified design across scenarios/use cases is strongly preferred - Following the above, RAN1 can carefully assess the need in 6GR to introduce waveform(s) beyond 5G NR, targeting, e.g., - Potential better coverage by better PAPR performance for uplink - Spectrum efficiency improvement |

## Evaluation methodology

| Thales et al | Proposal 1: RAN1 to investigate the performance of candidate waveforms under varying carrier frequency and time offsets inherent to satellite links, specifically considering scenarios with significant uncertainty in the UE’s position. This study will focus on conditions relevant to GNSS-free physical layer operation, aiming to identify robust waveform solutions suitable for environments with no GNSS availability. |
| --- | --- |
| ZTE | Proposal 9: The following aspects are recommended to be considered for the 6G waveform evaluation: - Performance metrics: PAPR, BLER,OOBE, Net gain. - PA modelling with more realistic assumption. |
| Xiaomi | Proposal 1: The following net gain evaluation metric is used for 6GR OFDM based waveform determination and RAN1 shall liaise with RAN4, if necessary, on RF dependent net gain assessment to identify the 6GR waveform that is coverage beneficial. Net Gain = ΔSNR+ ΔPAPR |
| CMCC | Proposal 1. For the evaluation of PAPR reduction for OFDM waveform, cubic metric can be used. Proposal 2. For the evaluation of link performance for each candidate OFDM waveform, the gain should be evaluated at the target data rate for cell-edge UE. Proposal 3. The characteristics of the out-of-band emission need to be evaluated for each candidate OFDM waveform, while the detailed metric(s) should be determined by RAN4. |
| Huawei | Proposal 1: Take Table 1 and 2 as the start point for low PAPR waveform enhancement evaluations. Proposal 2: For RAN1 waveform evaluations, the NR PA model recommended in R4-164542 for below 6GHz is reused at least until any new PA model is updated by RAN4. Proposal 3: Send LS to RAN4 to ask about any updates to PA models, RF requirements and RB allocations for MPR evaluations. Proposal 4: Take Table 3 as the start point for UL multi-layer waveform evaluations. Proposal 5: Take Table 4 as the start point for DL common channels waveform evaluation. |
| BUPT | Proposal 5 Baseline metrics for evaluation of potential additions of 6GR waveforms at least include PAPR and spectrum efficiency. Proposal 6 FFS the other metric(s) taking into account PA modeling and implementations if they are representative. |
| InterDigital | Proposal 4: The following KPIs relevant for communication should be evaluated when studying PAPR reduction techniques or a new waveform: - Spectral efficiency (bps/Hz) - BLER - Cubic metric - PAPR |
| Panasonic | Proposal 6: At least the following evaluation criteria should be considered in the waveform evaluation. - MRSS compatibility - Complexity - Flexible time and frequency domain resource allocation - Specification impact - MIMO capability - Spectral efficiency - Coverage - Net gain - MPR / PAPR - BLER Proposal 7: For MPR / PAPR analysis, how relationship with ACLR, SEM, EVM, spurious emissions, and occupied bandwidth progress will require discussion in RAN1 as PAPR only does not determine the amount of UE Tx power. |
| Ericsson | Proposal 2 To evaluate different coverage enhancement techniques via PAPR reduction for 6GR, RAN1 to consider MPR-based and system-level simulations. |
| Sony | Proposal 5: RAN1 should study and compare PAPR reduction techniques with respect to PAPR reduction, implementation complexity, and spectral efficiency. Proposal 6: RAN1 should determine a method for evaluating the the efficiency of a PAPR reduction method; this method should consider both spectral efficiency, implementation complexity and PAPR reduction. |
| Apple | Proposal 2: future low PAPR waveform evaluations should adopt a multi-dimensional metric framework centered on Net Gain, spectral compliance, and realistic RF and receiver assumptions. Proposal 3: For low PAPR waveforms, the Net Gain with a realistic PA model should be used as a metric where, Net Gain = Δ𝑆𝑁𝑅 + ΔPower - Δ𝑆𝑁𝑅 = the gain or loss in SNR compared with basic DFT-S-OFDM due to the use of a low PAPR waveform at a desired BLER (typically a loss). - ΔPower = the gain or loss in UE output power captured by the PAPR difference with the basic DFT-S-OFDM or the Maximum Power Reduction (MPR). Proposal 4: For 6G waveform evaluations, particularly for coverage enhancement, MPR or PAPR should be considered as the metric. If using MPR or power gain, PA model and RF requirement has to be revisited for power boost feature in 6G. |

# Proposals for online sessions #1

## Monday/Tuesday

Moderator proposal (stable)

Send an LS to RAN4 for appropriate PA model(s) (for both UE and network side models) and other relevant RF parameters to use in waveform evaluations.

Draft LS in R1-2508068 (Frank)

Complement the following RAN1#122 agreements (not stable)

Agreement [RAN1#122] CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement [RAN1#122] CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

Moderator proposal; complement the RAN1#122 agreement with (not stable): 
CP-OFDM waveform as defined in 5G NR is supported as the basis for communication in 6GR for downlink

- Moderator proposal; complement the RAN1#122 agreement with (not stable):
CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for communication in 6GR for uplink

Moderator proposal: Discuss the following alternatives on DFT-s-OFDM usage for 6GR downlink (not stable)

Target use cases for DFT-s-OFDM for DL:

- IoT

- NTN

- MBB (low data rates)

- FR2

- Network Energy Saving

- UE low power receiver (Wake-up signal)

- (ISAC)

Target channels

- Unicast PDSCH

- SIB1 PDSCH

- Msg4 PDSCH

- CSS PDCCH

- “Common channels”

- Wake-up signal

- Reference signals

Moderator proposal (not stable)

- No need to discuss transmit processing of individual signals such as wake-up signals under the waveform agenda item

# Questions after Tuesday offline

## Questions on DFT-s-OFDM with Rank > 1 for UL

These questions are not aimed to be discussed yet on Wednesday online session, but later in the week

Discuss which alternatives are seen as practical by the proponents of DFT-s-OFDM with Rank > 1 for UL

- Alt 1a: DFT-s-OFDM is used for lower ranks (e.g. 1 and 2) only, CP-OFDM is used for all ranks.

- Alt 1b: DFT-s-OFDM is used for lower ranks (e.g. 1 and 2) only, CP-OFDM is only used for ranks not used with DFT-s-OFDM

- Alt 1c: Both DFT-s-OFDM and CP-OFDM can be used across lower and higher ranks.

- Something else, please describe

Discuss which alternatives are seen as practical by the proponents of DFT-s-OFDM with Rank > 1 for UL

- DFT-s-OFDM with rank > 1 is applicable/not applicable for

  - Alt 2a: applicable for all bands

  - Alt 2b: not applicable to FDD bands

  - Alt 2c: not applicable to bands < X MHz,

  - Alt 2d: not applicable to FR1 bands

  - Something else, please describe

Company comments

| Company | Comment |
| --- | --- |
| vivo | Alt 1a - We shouldn’t limit the use of CP-OFDM for all ranks. CP-OFDM should be able to work standalone. - Careful study should be carried out for the applicable ranks for DFT-s-OFDM. The applicable ranks should show gain compared with CP-OFDM taking MPR into account. Based on our study, we see gain on rank 2. Hence we can start with rank 1 and 2. Alt 2a |
| Ofinno | We support: Alt 1a, and Alt 2a |
| Ericsson | Alt 1c as a starting point. CP-OFDM is more compatible with fully coherent UL MIMO operation, while DFT-S-OFDM enables better use of PA power especially in non-coherent UL MIMO operation. Power limitation effects can be even stronger with increasing number of UE antennas at higher power classes where all PAs must transmit to reach the power class. On the other hand, the precoding gains from coherence also increase with the number of UE antenna elements. System level simulations are needed to establish gains for a given UE antenna / transmitter configuration and should be used to decide if limitations on rank for a given waveform are justified. Therefore, during the study phase, it is natural to start with Alt 1c to obtain better understanding. Alt 2a. MIMO UE capability generally depends on the band, rather than having specific limitations in specifications for a given MIMO feature. For example, 8 Tx UL MIMO is not limited to FR1 today, nor is beam management limited to FR2. Such strict limitations would greatly complicate the specification as well as restrict UE and network implementations. |
| NTT DOCOMO | We think Alt 1a and Alt 1b are more practical than Alt 1c. To avoid fragmentation, we think it would be good to try to achieve Alt 1b, while we also think it would be important to see whether there is any performance difference between CP-OFDM and DFT-s-OFDM when e.g., 2 ranks are configured. If there is a case where 2-rank with CP-OFDM is better than the one with DFT-s-OFDM, we think it should be fine to consider Alt1a. For the second question, we understand NR didn’t really limit # of ranks that can be implemented in the spec. Instead NR just let UE implementation decide. In our view this direction could still remain and no need to change. Hence we prefer Alt 2a in the spec, i.e., DFT-s-OFDM with rank > 1 is applicable for all bands |
| QC | Prefer Alt 1c as a starting point. Similar views as Ericsson. Prefer Alt 2a. Can be left to a UE’s MIMO capability in any given band. |
| ZTE | We support Alt 1a: CP-OFDM should be supported across all transmission ranks, while DFT-s-OFDM is employed to improve coverage and energy efficiency. Our analysis indicates that even with rank > 1, DFT-s-OFDM maintains a lower PAPR than CP-OFDM, though the improvement becomes marginal beyond rank 4. Rank-2 DFT-s-OFDM for 6G uplink transmission can be considered as the most practical and prioritized extension for 6G, which offers a good balance between performance gain and implementation complexity. We support Alt 2a: DFT-s-OFDM should not be limited to specific frequency bands. And our investigation show that even in FDD mode, where DFT modules will multiplex in frequency domain , DFT-s-OFDM still maintains a lower PAPR than CP-OFDM, though the number of DFT modules should remain limited. |
| OPPO | In general, we suggest to discuss this issue in MIMO agenda. We do see any impact to DFT-s-OFDM waveform processing due to supporting multi-rank. We can focus on the waveform proposals themselves in this agenda. For the first proposal, We support Alt 1a. No reason to restrict number of ranks for CP-OFDM, even if DFT-s-OFDM with multiple layers is supported in 6GR. Alt 1c is completely unreasonable. In high rank region, SINR is quite high in which the power is not limited. No need to support DFT-s-OFDM with high rank. Alt 1b is sub-optimal with unnecessary limit to CP-OFDM usage. For the second proposal, we see no need to limit the bands applicable to multi-rank DFT-s-OFDM. Support Alt 2a. |
| Samsung | While we appreciate the above proposals from the moderator, from our perspective, the maximum rank for DFT-s-OFDM is a critical factor for decision making as described in details in our contribution. Therefore, we suggest the following revision on the formulation: - Alt 1a: DFT-s-OFDM is used for ranks <= Y only, CP-OFDM is used for all ranks. - Alt1a.1: Y=1 - Alt1a.2: Y=2 - Alt1a.3: Y>2 - Alt 1b: DFT-s-OFDM is used for ranks <= Y only, CP-OFDM is only used for ranks not used with DFT-s-OFDM - Alt1a.1: Y=1 - Alt1a.2: Y=2 - Alt1a.3: Y>2 - Alt 1c: Both DFT-s-OFDM and CP-OFDM can be used across all ranks. - DFT-s-OFDM with rank > 1, if supported, is applicable/not applicable for - Alt 2a: applicable for all bands - Alt 2b: not applicable to FDD bands - Alt 2c: not applicable to bands < X MHz, - Alt 2d: not applicable to FR1 bands - Something else, please describe |
| Lenovo | We regard Alt 1a with DFT-s-OFDM limited to rank-2 as a pragmatic and practical starting point. Further applicability constraints can be studied next considering realistic evaluation of net gains and analysis of target scenarios. |
| CEWiT | Support Alt 1a. Alt 1b is unreasonable. For a cell center UE with limited data size (where rank is sufficient), Alt 1b would limit the waveform to DFT-s-OFDM. This is not ideal. Alt 1c is not needed, since PAPR gains of DFT-s-OFDM over CP-OFDM is marginal for higher ranks. Therefore, using DFT-s-OFDM for higher ranks (rank > 2) is not justified. Support Alt. 2a |
| Lekha | Support Alt 1a. Alt 1b doesn’t make sense as it limits the usage of CP-OFDM. Alt 1c can be considered based on the evaluation of 1a. Support Alt 2a |
| CATT | There are two issues in this proposal 1) whether to support multiple layers for DFT-s-OFDM 2) how DFT-s-OFDM with any layer works with CP-OFDM with all layers. We prefer to discuss two issues separately i.e., before we discuss how DFT-s-OFDM with any layer works with CP-OFDM with all layer, we should firstly discuss the performance, the impact on gNB complexity, and the spec impact etc. by introducing multiple layer DFT-s-OFDM to determine whether to support multiple layer for DFT-s-OFDM. |
| Huawei, HiSilicon | Suggest to replace “ranks” with “layers” in order to align terminology with current NR spec. The maximum number of UL-MIMO layers is up to MIMO discussions and thus better not to claim “all” number of layers at this stage. In waveform session here, it is good to confirm 2-layer UL-MIMO for DFT-s-OFDM is beneficial from coverage perspective because its coverage gain is sourced from waveform rather than MIMO. Additionally, confirm that multi-layer UL-MIMO for CP-OFDM is also supported. Suggest a revision of Alt 1a: Revised proposal: - Alt 1a-rev: DFT-s-OFDM with 2 layers is beneficial for coverage and thus supported, CP-OFDM with multi-layer UL-MIMO is supported. The maximum number of layers for both waveforms are up to the future discussions in MIMO session. For the second question, similar to CP-OFDM with multi-layer UL-MIMO, it can be up to UE capability and does not have to decide in RAN1 now. |

# Proposals for online sessions #2

## Wednesday

Feature lead proposal:

- For uplink low-PAPR proposals the primary performance evaluation criterion is Net Gain

  - Net Gain [dB] = Tx power gain - link loss relative to the reference @10% BLER

    - A realistic UE PA model should be used

  - Other metrics, e.g. occupied BW, are to be used, if applicable

Feature lead proposal:

- For downlink low-PAPR proposals the primary evaluation criterion is one of (depending on the proposal)

  - Net Gain [dB] = Tx power gain - link loss relative to the reference @10% BLER

    - A realistic base station PA model should be used

  - Network energy saving

  - Other metrics, e.g. occupied BW, are to be used, if applicable

Feature lead proposal:

- For waveform proposals not motivated by PAPR reduction, the primary evaluation criteria is to be chosen depending on the benefit motivating the proposal

Feature lead proposal:

- Adopt the attached list as the set of waveform related proposals to consider further in the 6G study

  - Additional lines to the list may be added by [end of Thursday]

[list to be attached before the online session]

| Company | Comment |
| --- | --- |
| Ericsson | We would like to propose the following evaluation methodology while evaluating low-PAPR enhancements. For uplink low-PAPR proposals the primary link-level performance evaluation criterion is Net Gain - Net Gain [dB] = Tx power gain - link loss relative to the reference @10% BLER - A realistic UE PA model should be used - Schemes are compared at comparable operating points, such as the same spectral efficiency and/or using the same time/frequency resources - Other metrics, e.g. occupied BW, are to be used, if applicable - System level evaluations taking MPR into account are used to establish the overall benefit of schemes. - The MPR is determined using the PA model |

# Thursday offline

Continuation of the Wed online on the ULlow-PAPR evaluation metrics

Agreement

- For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

    - A realistic PA model should be used

    - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account.

    - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged.

    - Practical modulation and coding should be used

    - Evaluation metrics maybe revisited in the future.

  - Note: Companies to report how to calculate the Tx power gain

- Other metrics, e.g. effective occupied BW, In-band emissions, etc. are to be used, if applicable

Ericsson proposal for system level

- For uplink low-PAPR proposals, the system level evaluations taking MPR into account are used to establish the overall benefit of schemes.

  - The MPR is determined using the PA model

DL evaluation for low-PAPR

Feature lead proposal:

- For downlink low-PAPR proposals the primary evaluation criterion is one of (depending on the proposal)

  - Net Gain [dB] = Tx power gain - link loss relative to the reference @10% BLER

    - A realistic base station PA model should be used

  - Network energy saving

  - Other metrics, e.g. occupied BW, are to be used, if applicable

Questions related to the list of waveform proposals

Feature lead proposal:

- Adopt the collected list as the set of waveform related proposals to consider further in the 6G study

- Following points are to be clarified/confirmed

  - Yellow highlighted are modifications by the FL (DFT-s-OFDM for DL categorized as new waveform for DL)

  - Red highlight is a question raised by FL

      - What to do with proposals under waveform AI that are not categorized as waveform proposals?

      - GFB-OFDM is marked as an enhancement to CP-OFDM and a new waveform

      - OSDM is marked as an enhancement to CP-OFDM and a new waveform

      - Zak-OTFS is marked to be all three, a new waveform, a modification to CP-OFDM and a modification to DFT-s-OFDM

UL DFT-s-OFDM with Rank > 1

- Alt 1a: DFT-s-OFDM is used for lower ranks (e.g. 1 and 2) only, CP-OFDM is used for all ranks.

- Alt 1c: Both DFT-s-OFDM and CP-OFDM can be used across lower and higher ranks.

- Alt 2a: DFT-s-OFDM with Rank > 1 applicable for all bands

Feature lead proposal:

- If support for DFT-s-OFDM with rank > 1 is defined, RAN1 will not limit the applicable bands

- DFT-s-OFDM with Rank > 1 study is

  - Alt1: Limited to max rank = 2

  - Alt2: Not limited to any particular rank at this time

  - Alt3: To be continued under MIMO agenda item starting RAN1#123