3GPP TSG RAN WG1 #123			R1-250xxxx

Dallas, USA, November 17th – 21th, 2025

Agenda item:	7

Source: 	Moderator (vivo)

Title: 	Discussion on PUSCH without repetition when two SRS resource sets are configured

Document for:	Discussion and Decision

- Introduction

In RAN1#123, a draft CR is proposed in [1], the proposed text change is copied for reference.

6.1.2.1 Resource allocation in time domain For PUSCH repetition Type B, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', the SRS resource set association to nominal PUSCH repetitions follows the same method as SRS resource set association to slots in PUSCH Type A repetition by considering nominal repetitions instead of slots. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', the UE expects the value of the SRS resource set indicator field to be set to "00" or "01" in DCI format 0_1 or 0_2, and the PUSCH transmission in case K=1 scheduled or activated by the DCI is associated with the first SRS resource set or the second SRS resource set respectively. For both PUSCH repetition Type A and PUSCH repetition Type B, when a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" or "11" for the SRS resource set indicator, the redundancy version to be applied on the nth transmission occasion (for PUSCH repetition Type A) of the TB, where n = 0, 1, … K-1, or nth actual repetition (for PUSCH repetition Type B, with the counting including the actual repetitions that are omitted) is determined according 6.1.2.3 Resource allocation for uplink transmission with configured grant For PUSCH transmissions with a Type 2 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, the SRS resource set association to K>1 (nominal) repetitions follows MappingPattern in ConfiguredGrantConfig, or the SRS resource set is associated to PUSCH transmission with K=1 as defined in Clause 6.1.2.1 for PUSCH scheduled by DCI format 0_1 and 0_2. For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets with usage set to 'codebook' or 'noncodebook' are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if p0-PUSCH-Alpha2 is provided, the SRS resource set association to (nominal) repetitions is determined as follows. When K = 2, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively.

Since the spec is written from UE perspective, the proponent argued that the UE behavior should be clear if there the DCI field indicates otherwise.

Please provide your views in table below

| Company | comments |
| --- | --- |
| ZTE | Not needed. When the PUSCH transmission in case K=1 scheduled (aka PUSCH without repetition and without MTPR operation), it is intuitive gNB can avoid to indicate any values other than “00” or “01” of SRS resource set indicator field in DCI format 0_1 or 0_2. If we have such kind of CR, do we need to capture all exceptions of error cases (may be endless) in terms of gNB implementation? For instance, if 4-layer based precoder is indicated by TPMI field for the PUSCH transmission, gNB wrongly indicates two DMRS ports by DMRS port indication field. |

Reference

[1] R1-2508401,	‘Draft CR on PUSCH without repetition when two SRS resource sets are configured(Rel-17)’,	vivo