3GPP TSG RAN WG1#125			    R1-2604993

Dalian, China, May 18th – 22nd, 2026

Agenda item:	8.1

Source: 	Moderator (ZTE)

Title: 	Moderator Summary #1 on UE-initiated/event-driven beam management

Document for:	Discussion and Decision

- Introduction

In RAN#102, the Rel-19 WID on NR MIMO phase 5 is approved. In the approved WID, UE-initiated/event-driven beam management is a part of the RAN1 objectives as follows:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Plan

Per Mr. Chair’s guidance, for Rel-19 maintenance, only essential corrections will be considered. Individual CRs will be adopted for this meeting.

Based on company contributions in [1]-[5], the followings are provided in this discussion:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues/TPs/CRs are categorized as follows:

  - Issue 3 – UL signaling medium/container

- Observations and recommended proposals based on the summary of companies’ views

Note 1: While considering the progress of the above issues, proposals for draft CR are provided directly. Please pay attention to pending issues/TPs/CRs discussed before (highlighted in this per FL’s assessment), we may have to close the door if we fail to make progress in this meeting.

Note 2: Please note that the TPs/CRs that already discussed more than two meetings without consensus were NOT provided in the summary for discussion.

- Contact Person

For potential offline discussions, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Apple | Hong He | hhe5@apple.com |
| CATT | Jiayi Yang | yangjiayi@catt.cn |
| CEWiT | Pardh | pardhasarathy.j@cewit.org.in |
| CMCC | Yan LI | liyanwx@chinamobile.com |
| Ericsson | Claes Tidestav | claes.tidestav@ericsson.com |
| ETRI | Cheulsoon Kim | cs.kim@etri.re.kr |
| Fujitsu | David | wangguotong@fujitsu.com |
| Fujitsu | Wang Xin | wangxin@fujitsu.com |
| FUTUREWEI | Weimin Xiao | weimin.xiao@futurewei.com |
| FUTUREWEI | Zhigang Rong | zrong@futurewei.com |
| Google | Alex Liou | alexliou@google.com |
| Huawei/HiSilicon | Keyvan Zarifi | Keyvan.zarifi@huawei.com |
| Huawei/HiSilicon | Fanbo | asen.fanbo@huawei.com |
| HONOR | Guozeng Zheng | zhengguozeng@honor.com |
| Intel | Gary | gang.xiong@intel.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| KDDI | Shunsuke Kamiwatari | sh-kamiwatari@kddi.com |
| LG | Hyungtae Kim | ht.kim@lge.com |
| MediaTek | Rebecca Chen | rebecca.chen@mediatek.com |
| Meta | Avik Sengupta | aviksg@meta.com |
| NEC | Peng GUAN | guan_peng@nec.cn |
| NEC | Yukai GAO | gao_yukai@nec.cn |
| NICT | Kenichi Takizawa | takizawa@nict.go.jp |
| Nokia | Mihai Enescu | mihai.enescu@nokia.com |
| Nokia | Youngsoo Yuk | youngsoo.yuk@nokia.com |
| NTT DOCOMO | Mamoru Okumura | Mamoru.okumura.nz@nttdocomo.com |
| OPPO | Li Guo | guoli@oppo.com |
| Panasonic | Khalid Zeineddine | khalid.zeineddine@eu.panasonic.com |
| Qualcomm | Wooseok Nam | wnam@qti.qualcomm.com |
| Ruijie Networks | Ke Zhong | zhongke@ruijie.com.cn |
| Samsung | Dalin Zhu | dalin.zhu@samsung.com |
| Samsung | Sa Zhang | sa.zhang@samsung.com |
| Sharp | Taka | fukui.takahisa@mail.sharp |
| Sony | Naoki Kusashima | Naoki.Kusashima@sony.com |
| Sony | Jose Flordelis | Jose.Flordelis@sony.com |
| Spreadtrum | Yu Yang | yu.yang2@unisoc.com |
| vivo | Rakesh Tamrakar | rakesh@vivo.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| ZTE | Yang Zhang | zhang.yang220@zte.com.cn |
| Lenovo | Bingchao Liu | Liubc2@lenovo.com |
| Ofinno | Jae-Nam Shim | jshim@ofinno.com |
| Ofinno | Yunjung Yi | yyi@ofinno.com |

- Discussion

- Issue 3 – UL signaling medium/container

Table 3-1 Summary for Issue 3

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 3.1 | First PUCCH for Mode-A and Mode-B —— multiplexing and/or dropping rule on Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH | FL Note: In [1], one TP was proposed to capture the case of only UEIRI multiplexed in PUSCH regarding the procedure of data and control multiplexing in TS 38.212. FL assessment: As discussed in last meeting, most of companies thought this TP is redundant and not needed due to the case of only UEIRI multiplexed on PUSCH was already captured in clause 9.3 of TS 38.213 and clause 6.3.2.1.3B in 38.212 as follows. E.g., the description without “and HARQ-ACK information if any”. Besides, if go with this TP, it seems the case of both UEEIRI and HARQ-ACK multiplexed in PUSCH is rather missing. Please companies provide views on necessity of this TP. Given that this TP was already discussed in last meeting and without consensus, and if we fail to make any progress in this meeting, we may have to close the door for this proposal at all. - TS 38.213 ## 9.3 UCI reporting in physical uplink shared channel ... When the UE would multiplex UEIRI, and HARQ-ACK information if any, in a PUSCH, the UE determines a number of resources for multiplexing the UEIRI bits and HARQ-ACK information, if any, in the PUSCH using βoffsetHARQ−ACK which provides indexes Ioffset,1HARQ−ACK and Ioffset,2HARQ−ACK for the UE to use if the UE multiplexes up to 11, and more than 11 information bits, respectively, where if the PUSCH transmission includes HARQ-ACK information, the UE jointly encodes the HARQ-ACK information and the UEIRI [5, TS 38.212]. .... - TS 38.212 ##### 6.3.2.1.3B UEIRI If UEIRI bits are transmitted on a PUSCH, the UEIRI bit sequence a0, a1, a2, a3, …,aA−1 is determined as follows: - set ai=oiUEIRI for i=0,1, …, OUEIRI−1 and A=OUEIRI, where OUEIRI is the number of PUCCH resources for UEIRI given by Clause 9 of [5, TS38.213], and the UEIRI bit sequence o0UEIRI, o1UEIRI, …, oOUEIRI−1UEIRI is mapped to the PUCCH resources according to an ascending order of pucch-ResourceId. If the associated UEIRI of oiUEIRI is positive, oiUEIRI=1; otherwise, oiUEIRI=0.. Proposal 3.1: Adopt the following changes in Clause 6.2.7 in TS 38.212: - Reason for change: The case of only UEIRI multiplexed on PUSCH is absent in the procedure of data and control multiplexing. - Summary of change: Remove “and HARQ-ACK” to extend the procedure of CG-UCI without HARQ-ACK multiplexed on PUSCH to apply for the case of UEIRI with/without HARQ-ACK. - Consequences if not approved: UE behavior is unclear when only PUCCH with UEIRI collides/overlaps with PUSCH. ### 6.2.7 Data and control multiplexing In case where there are more than one UL-SCH transport blocks for the PUSCH transmission, the UCI information is multiplexed only on the UL-SCH transport block with highest IMCS value for the initial PUSCH, where IMCS is as defined in Clause 6.1.4.1 in [6, TS 38.214]. In case the two transport blocks have the same IMCS value for the initial PUSCH, the UCI information is multiplexed with data only on the first transport block. The PUSCH for UCI multiplexing in this Clause refers to the UL-SCH transport block for UCI multiplexing. If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedure in this clause 6.2.7 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "when higher layer parameter cg-UCI-Multiplexing is configured" with "when UTO-UCI and HARQ-ACK are transmitted on a PUSCH". If a UE would multiplex UEIRI and HARQ-ACK in a PUSCH [5, TS 38.213], the procedure in this clause 6.2.7 applies by replacing CG-UCI with UEIRI in all the notations and texts, and replacing "when higher layer parameter cg-UCI-Multiplexing is configured" with "when UEIRI and HARQ-ACK are transmitted on a PUSCH". UE expects that at most one of CG-UCI, UTO-UCI, or UEIRI to overlap with a PUSCH. <unchanged part is omitted>. Supported by: vivo Not supported by: |
| 3.2 | X symbols for determining available transmission occasion of the second UL channel for ModeB | FL Note: In [2], TP#2-1 was proposed to clarify the timing reference of X symbols for determining available transmission occasion of the second UL channel on Mode B, which should refer to transmission occasion of first PUCCH rather than the transmitted PUSCH based on the CR endorsed in last meeting (cf. R1-2603465), e.g., replace “PUCCH” with “PUSCH with UEIRI”. Otherwise, it may lead to several issues, i.e.: 1) the ambiguity of specification description like “where the periodicity of the PUSCH with UEIRI resource and type 1 CG PUSCH resource is the same”; 2) unexpected report latency if the minimumPucch-PuschOffset value is not large enough (e.g., 0, 1, 2, 4, or 8 symbols within a slot), as shown in Figure 2; 3) Further repeated statement of other cases, e.g., when UEIRI multiplexed in PUCCH format 2/3/4, as shown in Figure 4. - R1-2603465 In the remaining of this clause, for the determination of type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB, “PUCCH” is replaced with “PUSCH with UEIRI” if the UE would multiplex UEIRI in a PUSCH [6, TS38.213].. Figure 2 Second PUSCH transmission occasion based on the PUSCH transmission occasion with UEIRI Figure 4 Second PUSCH transmission occasion based on the first PUCCH transmission occasion FL assessment: It seems to be valid issue per FL’s assessment. Companies are encouraged to provide views of Proposal 3.2 with consideration of the above elaboration. Proposal 3.2: Adopt the following changes in Clause 5.2.1.5.4.1 in TS38.214: - Reason for change: For Mode B UE-initiated beam reporting, if the timing reference of X symbols for determining available second UL channel is based on transmission occasion of a transmitted PUSCH with UEIRI based on the CR endorsed in RAN1#124bis meeting (cf. R1-2603465), it may wrongly lead to: 1) the ambiguity of specification description like “where the periodicity of the PUSCH with UEIRI resource and type 1 CG PUSCH resource is the same”; 2) unexpected report latency if the minimumPucch-PuschOffset value is not large enough (e.g., 0, 1, 2, 4, or 8 symbols within a slot); 3) Further repeated statement of other cases, e.g., when UEIRI multiplexed in PUCCH format 2/3/4. - Summary of change: Clarify that timeline of valid transmission occasion of second PUSCH in Mode B UE-initiated reporting is determined based on the transmission occasion of PUCCH with positive UEIRI regardless of whether UEIRI is multiplexed on other uplink channels. - Consequences if not approved: If this ambiguity is not resolved, ambiguity of specification interpolation may exist, leading to inconsistent UE behavior and potential repeated statements of other cases. - 5.2.1.5.4.1 UE Initiated CSI reporting - 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 <Irrelevant part is omitted> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The differential L1-RSRP values are with a reference to the largest measured L1-RSRP value of the reported nrofReportedRS-UEIBR reference signals. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion, subject to limitations for UE transmission of a PUSCH with SP-CSI reports as described in clauses 9, 11.1, 11.1.1, 11.2A,15 and 17.2 including repetitions if any [6, TS 38.213] or as described in clause 5.2.5, numOfSymbols-ModeB symbols after the end of the transmitted PUCCH transmission occasion with UEIRI transmitted regardless of multiplexing or not if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. <Irrelevant part is omitted> - 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 <Irrelevant part is omitted> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The differential L1-RSRP values are with a reference to the largest measured L1-RSRP value of the reported nrofReportedRS-UEIBR reference signals. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion, subject to limitations for UE transmission of a PUSCH with SP-CSI reports as described in clauses 9, 11.1, 11.1.1, 11.2A,15 and 17.2 including repetitions if any [6, TS 38.213] or as described in clause 5.2.5, numOfSymbols-ModeB symbols after the end of the transmitted PUCCH transmission occasion with UEIRI transmitted regardless of multiplexing or not if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. <Irrelevant part is omitted> - 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 <Irrelevant part is omitted> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ-th highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block with the valueOfQ-th highest L1-RSRP out of the SS/PBCH blocks QCLed with the activated TCI state reference signals. The differential L1-RSRP values are with a reference to the largest measured L1-RSRP value of the reported nrofReportedRS-UEIBR reference signals. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion, subject to limitations for UE transmission of a PUSCH with SP-CSI reports as described in clauses 9, 11.1, 11.1.1, 11.2A,15 and 17.2 including repetitions if any [6, TS 38.213] or as described in clause 5.2.5, numOfSymbols-ModeB symbols after the end of the transmitted PUCCH transmission occasion with UEIRI transmitted regardless of multiplexing or not if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. <Irrelevant part is omitted>. Supported by: ZTE Not supported by: |
| 3.3 | Type 1 CG PUSCH in Mode B configured with applyIndicatedTCI-State in ConfiguredGrantConfig | FL Note: In [3], one TP was proposed to clarify that Type 1 CG PUSCH for Mode B UEI beam reporting can be transmitted in UL mTRP operation in TS 38.300, e.g., Type 1 CG PUSCH in Mode B configured with applyIndicatedTCI-State in ConfiguredGrantConfig. Besides, an LS is needed to be sent to RAN2. FL assessment: Per statement in WID, it assumes that Rel-19 UEI-BR is targeting for sTRP with respect to beam measurement, while it does not limit the PUSCH carrying beam report to be applied in sTRP or mTRP operation. In this sense, it seems to be up to NW configuration on whether Type 1 CG PUSCH in Mode B can be configured with applyIndicatedTCI-State in ConfiguredGrantConfig. Companies are encouraged to provide views of Proposal 3.3a and Proposal 3.3b with consideration of the above. Proposal 3.3a: Send an LS to RAN2 to clarify that Type 1 CG PUSCH for Mode B UEI beam reporting can be configured with applyIndicatedTCI-State in ConfiguredGrantConfig. Proposal 3.3b: Adopt the following changes in Clause 9.2.3.1 in TS 38.300: - Reason for change: For the UEI beam reporting with multiple CSI configurations described in clause 5.2.1.5.4.1d of the TS 38.214 which is applicable for Event-1, Event-2 and Event-7, it is inaccurate/incorrect to state that the reported beams corresponding to the reference signals in the new beams RS resource set need to satisfy the event. - Summary of change: Remove the condition of satisfying the event for the reported beams corresponding to the reference signals from the new beams RS resource set when the UEI beam reporting is with multiple CSI configurations. - Consequences if not approved: UE procedures of beam reporting for when the UEI beam reporting configured with multiple CSI configurations are unclear. ##### 9.2.3.1 Overview <Unchanged parts are omitted> For UE-initiated beam reporting, Mode A and Mode B are supported, where upon detection of an event, UE transmits a UE Initiated Report Indicator (UEIRI) indication in a PUCCH resource to indicate transmission of a beam report. Corresponding to the UEIRI indication, in Mode A, the UE transmits the beam report on a PUSCH indicated by a DCI, while in Mode B, the UE transmits the beam report on a type1 CG PUSCH associated to the PUCCH resource. For UE-initiated beam reporting, single DL TRP is supported for event evaluation, and the type1 CG PUSCH for the beam report can be configured within a BWP supporting UL sTRP or UL mTRP. <Unchanged parts are omitted>. Supported by: CATT Not supported by: |
| 3.4 | Overlapping of PUCCH with UEIRI and other channels/signals | FL Note: In [4], TP#4 was proposed to reuse the same behaviour as SR for overlapping PUCCH with UEIRI and other channels/signals. FL assessment: It is valid issue per FL’s assessment, which is somehow similar/related to the third issue of Proposal 3.2. Companies are encouraged to provide views of Proposal 3.4. Proposal 3.4: Adopt the following changes in Clause 9 in TS 38.213: - Reason for change: UE behaviour is under for overlapping PUCCH with UEIRI and other channels/signals. - Summary of change: Reuse the same behaviour as SR for overlapping PUCCH with UEIRI and other channels/signals. - Consequences if not approved: Unclear UE behaviour. - 9 UE procedure for reporting control information <Unchanged parts are omitted> If a UE is configured for NR-DC operation, the UE does not expect to be configured with a PUCCH-SCell. For the remaining of this clause, unless stated otherwise, any reference to SR is applicable for SR and/or for LRR and/or for UEIRI. A PUSCH or a PUCCH transmission other than PUCCH transmissions with SL HARQ-ACK reports, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from phy-PriorityIndex, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in clause 9.2.4. For a PUSCH transmission with semi-persistent CSI report, a UE determines a priority index from a priority indicator field, if provided, in a DCI format that activates the semi-persistent CSI report. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission other than PUCCH transmissions with SL HARQ-ACK reports, the priority index is 0. <Unchanged parts are omitted>. Supported by: Samsung Not supported by: |
| 3.5 |  | FL Note: In [5], TP#1 was proposed for spec alignment/completion in clause 9 of TS 38.213, i.e., HARQ-ACK information and/or UEIRI, from the overlapping PUCCH(s), to be multiplexed in a PUSCH transmission. FL assessment: It may be optimal to reach such spec alignment/completion that corresponding to the following excerpts in clause 9 of TS 38.213. Companies are encouraged to provide views of Proposal 3.5. - TS 38.213 ## 9.3 UCI reporting in physical uplink shared channel ... If a UE - would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and - the PUSCH and PUCCH transmissions fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE - multiplexes only HARQ-ACK information and/or UEIRI, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic or semi-persistent CSI reports in the PUSCH; - multiplexes only HARQ-ACK information, UEIRI and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic or semi-persistent CSI reports in the PUSCH. .... Proposal 3.5: Adopt the following changes in Clause 9 in TS 38.213: - Reason for change: UE behaviour is under for overlapping PUCCH with UEIRI and other channels/signals. - Summary of change: Reuse the same behaviour as SR for overlapping PUCCH with UEIRI and other channels/signals. - Consequences if not approved: Unclear UE behaviour. - 9 UE procedure for reporting control information **unchanged omitted** If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information and UEIRI, if any in the PUSCH and does not transmit the PUCCH. **unchanged omitted**. Supported by: Ofinno Not supported by: |

Table 3-2 Company input for Issue 3

| Company | Input |
| --- | --- |
| Mod V00 | Please input your comment to proposals of issues 3.1~3.5. |

- Proposals for Tuesday Online

TBD

- References

| 1 | R1-2603623 | Maintenance on NR MIMO phase 5 | vivo |
| --- | --- | --- | --- |
| 2 | R1-2603681 | Maintenance on NR MIMO Phase 5 | ZTE Corporation, Sanechips |
| 3 | R1-2603894 | Maintenance of NR MIMO Phase 5 | CATT |
| 4 | R1-2604152 | Remaining issue on NR MIMO Phase 5 | Samsung |
| 5 | R1-2604210 | Maintenance on NR MIMO Phase 5 | Ofinno |