3GPP TSG RAN WG1#125			    R1-260xxxx

Dalian, China, May 18th – 22nd, 2026

Agenda item:	8.1

Source: 	Moderator (ZTE)

Title: 	Moderator Summary #2 on UE-initiated/event-driven beam management

Document for:	Discussion and Decision

- Introduction

In RAN#102, the Rel-19 WID on NR MIMO phase 5 is approved. In the approved WID, UE-initiated/event-driven beam management is a part of the RAN1 objectives as follows:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Plan

Per Mr. Chair’s guidance, for Rel-19 maintenance, only essential corrections will be considered. Individual CRs will be adopted for this meeting.

Based on company contributions in [1]-[5], the followings are provided in this discussion:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues/TPs/CRs are categorized as follows:

  - Issue 3 – UL signaling medium/container

- Observations and recommended proposals based on the summary of companies’ views

Note 1: While considering the progress of the above issues, proposals for draft CR are provided directly. Please pay attention to pending issues/TPs/CRs discussed before (highlighted in this per FL’s assessment), we may have to close the door if we fail to make progress in this meeting.

Note 2: Please note that the TPs/CRs that already discussed more than two meetings without consensus were NOT provided in the summary for discussion.

- Contact Person

For potential offline discussions, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Apple | Hong He | hhe5@apple.com |
| CATT | Jiayi Yang | yangjiayi@catt.cn |
| CEWiT | Pardh | pardhasarathy.j@cewit.org.in |
| CMCC | Yan LI | liyanwx@chinamobile.com |
| Ericsson | Claes Tidestav | claes.tidestav@ericsson.com |
| ETRI | Cheulsoon Kim | cs.kim@etri.re.kr |
| Fujitsu | David | wangguotong@fujitsu.com |
| Fujitsu | Wang Xin | wangxin@fujitsu.com |
| FUTUREWEI | Weimin Xiao | weimin.xiao@futurewei.com |
| FUTUREWEI | Zhigang Rong | zrong@futurewei.com |
| Google | Alex Liou | alexliou@google.com |
| Huawei/HiSilicon | Keyvan Zarifi | Keyvan.zarifi@huawei.com |
| Huawei/HiSilicon | Fanbo | asen.fanbo@huawei.com |
| HONOR | Guozeng Zheng | zhengguozeng@honor.com |
| Intel | Gary | gang.xiong@intel.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| KDDI | Shunsuke Kamiwatari | sh-kamiwatari@kddi.com |
| LG | Hyungtae Kim | ht.kim@lge.com |
| MediaTek | Rebecca Chen | rebecca.chen@mediatek.com |
| Meta | Avik Sengupta | aviksg@meta.com |
| NEC | Peng GUAN | guan_peng@nec.cn |
| NEC | Yukai GAO | gao_yukai@nec.cn |
| NICT | Kenichi Takizawa | takizawa@nict.go.jp |
| Nokia | Mihai Enescu | mihai.enescu@nokia.com |
| Nokia | Youngsoo Yuk | youngsoo.yuk@nokia.com |
| NTT DOCOMO | Mamoru Okumura | Mamoru.okumura.nz@nttdocomo.com |
| OPPO | Li Guo | guoli@oppo.com |
| Panasonic | Khalid Zeineddine | khalid.zeineddine@eu.panasonic.com |
| Qualcomm | Wooseok Nam | wnam@qti.qualcomm.com |
| Ruijie Networks | Ke Zhong | zhongke@ruijie.com.cn |
| Samsung | Dalin Zhu | dalin.zhu@samsung.com |
| Samsung | Sa Zhang | sa.zhang@samsung.com |
| Sharp | Taka | fukui.takahisa@mail.sharp |
| Sony | Naoki Kusashima | Naoki.Kusashima@sony.com |
| Sony | Jose Flordelis | Jose.Flordelis@sony.com |
| Spreadtrum | Yu Yang | yu.yang2@unisoc.com |
| vivo | Rakesh Tamrakar | rakesh@vivo.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| ZTE | Yang Zhang | zhang.yang220@zte.com.cn |
| Lenovo | Bingchao Liu | Liubc2@lenovo.com |
| Ofinno | Jae-Nam Shim | jshim@ofinno.com |
| Ofinno | Yunjung Yi | yyi@ofinno.com |

- Discussion

- Issue 3 – UL signaling medium/container

Table 3-1 Summary for Issue 3

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 3.4 | Overlapping of PUCCH with UEIRI and other channels/signals | FL Note: According to Ms. Chair’s announce in Tuesday online session, proponents are encouraged to provide further details/reason regarding the necessity of this TP, e.g., how many changes are needed if without such general description. FL assessment: Companies are encouraged to provide views of Proposal 3.4. Proposal 3.4: Adopt the following changes in Clause 9 in TS 38.213: - Reason for change: UE behaviour is under for overlapping PUCCH with UEIRI and other channels/signals. - Summary of change: Reuse the same behaviour as SR for overlapping PUCCH with UEIRI and other channels/signals. - Consequences if not approved: Unclear UE behaviour. # 9 UE procedure for reporting control information <Unchanged parts are omitted> If a UE is configured for NR-DC operation, the UE does not expect to be configured with a PUCCH-SCell. For the remaining of this clause, unless stated otherwise, any reference to SR is applicable for SR and/or for LRR and/or for UEIRI, when applicable. A PUSCH or a PUCCH transmission other than PUCCH transmissions with SL HARQ-ACK reports, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from phy-PriorityIndex, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in clause 9.2.4. For a PUSCH transmission with semi-persistent CSI report, a UE determines a priority index from a priority indicator field, if provided, in a DCI format that activates the semi-persistent CSI report. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission other than PUCCH transmissions with SL HARQ-ACK reports, the priority index is 0. <Unchanged parts are omitted>. |

Table 3-2 Company input for Issue 3

| Company | Input |
| --- | --- |
| Mod V00 | Please input your comment to proposals of issue 3.4. |

- Proposals for Tuesday Online

- TBD

- References

| 1 | R1-2603623 | Maintenance on NR MIMO phase 5 | vivo |
| --- | --- | --- | --- |
| 2 | R1-2603681 | Maintenance on NR MIMO Phase 5 | ZTE Corporation, Sanechips |
| 3 | R1-2603894 | Maintenance of NR MIMO Phase 5 | CATT |
| 4 | R1-2604152 | Remaining issue on NR MIMO Phase 5 | Samsung |
| 5 | R1-2604210 | Maintenance on NR MIMO Phase 5 | Ofinno |