3GPP TSG RAN WG1 #122							R1-2506533

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.2.2

Source: 	Moderator (Samsung)

Title: 	Moderator Summary on Offline Session for Rel-19 CSI enhancements

Document for:	Discussion and Decision

## Introduction

The following proposals were discussed.

## Summary of proposals

### Issue 1 (WID objective 2a and 2b): Type-I and Type-II codebook refinement for up to 128 CSI-RS ports

| 1.3 | Proposal 1.C: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports mode-B, support following TP to accurately referring spatial domain basis vector selection. ##### 5.2.2.2.1a Refined Type I Single-Panel Codebook <Unchanged part omitted> The index i1,2 is given by i1,2∈0,1,…,N1N2LG-1 where LG=3 for υ=5,6 and LG=4 for υ=7,8. The mapping of i1,1 and i1,2 to m1(g) m'1(g') and m2(g) m'2(g') for g=1,…,LG g'=0,…,LG-1 is obtained as in Clause 5.2.2.2.3 by replacing i with g' and replacing L with LG, where the values of C(x,y) are given in Table 5.2.2.2.5-4 and Table 5.2.2.2.1a-5, and m'1(g') and m'2(g') for g'=0,…,LG-1 one to one mapping to m1(g) and m2(g) for g=1,…,LG with g=g'+1,. The index i2,l, for l=1,…,υ and υ=1,2,3,4 is given by i2,l∈{0,1,2,3} and is mapped to cl=i2,l. The mapping of index i2,g, for g=1,…,LG and υ=5,6,7,8, to cl, with l=1,…,υ, is given in Table 5.2.2.2.1a-6. The quantities φcl and vm1l,m2(l) for typeI-codebookMode-r19 = 'modeB' are the same as defined above for 'modeA'. <Unchanged part omitted> Support/fine: NEC, Google, Samsung, OPPO, NTT DOCOMO, Spreadtrum, Fujitsu, vivo, ETRI, Ericsson, Lenovo, Apple, ZTE/Sanechips, CATT, Xiaomi, Qualcomm, Not support (obvious): Nokia, FL assessment: This TP seems to be correct and clearer than the current text. Whether this is needed or not can be discussed. The TP can be found in Section 3.3 | ##### 5.2.2.2.1a Refined Type I Single-Panel Codebook <Unchanged part omitted> The index i1,2 is given by i1,2∈0,1,…,N1N2LG-1 where LG=3 for υ=5,6 and LG=4 for υ=7,8. The i1,1i1,2selection of m1(g) and m2(g) for g=1,…,LG is obtained as in Clause 5.2.2.2.3, where the values of C(x,y) are given in Table 5.2.2.2.5-4 and Table 5.2.2.2.1a-5. The index i2,l, for l=1,…,υ and υ=1,2,3,4 is given by i2,l∈{0,1,2,3} and is mapped to cl=i2,l. The mapping of index i2,g, for g=1,…,LG and υ=5,6,7,8, to cl, with l=1,…,υ, is given in Table 5.2.2.2.1a-6. The quantities φcl and vm1l,m2(l) for typeI-codebookMode-r19 = 'modeB' are the same as defined above for 'modeA'. <Unchanged part omitted> |
| --- | --- | --- |

No offline consensus that this is needed

Proposal 1.G

Reason for change: In RAN1 #118bis meeting [1], SRS port grouping has been agreed for periodic, semi-persistent and aperiodic SRS resource sets. The SRS ports indexing for P/SP SRS resource set and AP SRS resource set are different following the agreement. For P/SP SRS, the 6/8R ports consists of ports within a set, while for AP SRS, the 6/8R ports can consist of ports across multiple sets. While the current description in TS38.214 seems sufficient, the proposed TP can improve the clarity.

Summary of the change: Separated description in TS38.214 for aperiodic from periodic and semi-persistent SRS

Consequences if not approved: Potential lack of clarity in description in TS38.214

< Start of the text proposal > ### 5.2.2.5.1 UE assumptions for CQI/PMI/RI calculation --- unchanged text omitted --- - For a UE configured with one or more SRS resource sets with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType set to 'periodic' or 'semi-persistent', with a total of PSRS=6 or 8 ports across the resources in a set intended for xT6R or xT8R, respectively, if the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI' and the UE is configured with the higher layer parameter SRSPortGrouping-r19, the UE can assume that SRS port group 0 corresponds to codeword 0 and comprises the even PSRS/2 ports, and that SRS port group 1 corresponds to codeword 1 and comprises the odd PSRS/2 ports out of the total PSRS ports. The SRS ports are indexed in an ascending order according to SRS resource ID and port number within each SRS resource. - For a UE configured with one or more SRS resource sets with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType set to 'aperiodic', with a total of PSRS=6 or 8 ports across the resources across the resource set(s) intended for xT6R or xT8R, respectively, if the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI' and the UE is configured with the higher layer parameter SRSPortGrouping-r19, the UE can assume that SRS port group 0 corresponds to codeword 0 and comprises the even PSRS/2 ports, and that SRS port group 1 corresponds to codeword 1 and comprises the odd PSRS/2 ports out of the total PSRS ports. The SRS ports are indexed in an ascending order according to SRS resource set ID, SRS resource ID and port number within each SRS resource. --- unchanged text omitted --- < End of the text proposal >

[Offline Conclusion: For Rel-19 SRS port grouping, whether resources across multiple resource sets can be used follows the legacy principle, i.e. for periodic and semi-persistent SRS resource sets, it is not possible to have the grouped SRS ports resources across multiple SRS resource sets; but for aperiodic SRS resource sets, the grouped SRS ports can come from multiple SRS resource sets]

(To be checked by Huawei for better wording)

| 1.4 | Conclusion 1.D: For Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, following the legacy (Rel-18) principle, a UE shall assume that CSI-RS ports mapped to the same port index p' across the K aperiodic CSI-RS resources, p'=0,1,…,PCSI-RS-1, as described in Clause 7.4.1.5.3 of [4, TS 38.211], share the same antenna port. FL assessment: This proposal has been discussed since RAN1#121. The proposal is reformulated based on vivo’s latest Tdoc [5] The proposal is technically sound. It was argued by vivo that for “KDOPP = {4, 8, 12} CSI-RS resource groups are introduced for Type-II Doppler CSI. This means that within a CMR group, there may be multiple CSI-RS ports with the same CSI-RS port index mapped to different antenna ports.” | Support/fine: vivo, Apple, CATT, Xiaomi, NEC, Not support (NW implementation): Google, Samsung, OPPO, NTT DOCOMO, Spreadtrum, Fujitsu, ETRI, Ericsson, Lenovo, ZTE/Sanechips, |
| --- | --- | --- |

| 1.6 | [117] Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, - N1⋅N2X1⋅X2-bit group-based CBSR - the following (X1, X2) values are supported: P; (N1, N2); (X1, X2). 48; (8,3); (1,1), (2,1), (4,1). (6,4); (1,1), (2,1), (2,2),. 64; (16,2); (1,1), (2,1), (2,2), (4,1), (4,2). (8,4); (1,1), (2,1), (2,2), (4,1), (4,2). 128; (16,4); (1,1), (2,1), (2,2), (4,1), (4,2). (8,8); (1,1), (2,1), (2,2), (4,1), (4,2). Conclusion 1.F: For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, the N1⋅N2X1⋅X2-bit group-based bitmap is identical for all the O1O2 groups FL assessment: This proposal is technically sound. Although the proposal is aligned with the understanding of the FL (and should be to other 😊), it doesn’t hurt to clarify to avoid ambiguity, | Support/fine: Fraunhofer IIS/HHI, Samsung, Spreadtrum, Fujitsu, vivo, Ericsson, Lenovo, Apple, CATT, Xiaomi, NEC, Not support: NTT DOCOMO, Google, ETRI, ZTE/Sanechips, Huawei/HiSi, |
| --- | --- | --- |

| 1.2 | [120bis] Conclusion For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when the Rel-18 SD NES Type-I is configured for the Rel-19 Type-I SP codebook, the powerOffset parameter can be configured in all the respective subConfiguration IEs - The supported values for powerOffset follow the legacy specification Proposal 1.B: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when the Rel-18 SD NES Type-I is configured for the Rel-19 Type-I SP codebook with the powerOffset parameter configured in all the respective subConfiguration IEs, the soft scaling (if configured) is calculated based on powerControlOffset (in linear scale) of the respective CSI-RS resource and powerOffset (in linear scale) in the respective sub-configuration Conclusion: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, joint configuration of the Rel-18 SD NES Type-I, the Rel-19 Type-I SP codebook, and the Rel-19 soft scaling for the Rel-19 Type-I SP codebook is not supported FL assessment: For this proposal to be valid, a UE must be configured with Rel-19 Type-I SP, SD+PD NES, and soft scaling. - But the use of soft scaling together with SD+PD NES has not yet been agreed, at least explicitly. - Even if it were not precluded, this would seem to fall into optimization rather than an essential change. More discussion on the combination is needed. | Support/fine: Google, Fujitsu (open), Lenovo, ZTE/Sanechips, Not support: Samsung, OPPO, NTT DOCOMO (discuss combo first), Spreadtrum, vivo, ETRI, Ericsson, Apple, CATT, Xiaomi (discuss combo first), Qualcomm, Nokia, |
| --- | --- | --- |

### Issue 2 (WID objective 2c): CRI-based CSI for hybrid beamforming (HBF)

| 2.1 | Proposal 2.A: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, - If higher layer parameter mrSelectedResources is not configured for the CSI report, support to associate the NZP CSI-RS resource for interference measurement with the NZP CSI-RS resource for channel measurement with smallest CRI; - Else, support to associate the NZP CSI-RS resource for interference measurement with the first configured NZP CSI-RS resource for channel measurement among the non-reported MR CRIs. Conclusion: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, when one NZP CSI-RS resource for interference measurement is configured, it is associated with all the KS NZP CSI-RS resources for channel measurement FL assessment: For the KS NZP CSI-RS resources for CM, only 1 NZP CSI-RS resource for IM can be configured (regardless whether MR is configured or not). The proposal introduces an association rule where the single NZP CSI-RS resource for IM corresponds to the “first” NZP CSI-RS resource for CM. It is unclear if this association is needed since NZP CSI-RS resource for IM is typically used for interference emulation. It seems proposal 2.A is not agreeable. To conclude this issue, either one of the three alternatives can be agreed: - Alt1. Conclude that when one NZP CSI-RS for IM is configured, it is associated with all the KS CMRs | Support/fine: Huawei/HiSi, Google (ok), NTT DOCOMO, Not support: Samsung, OPPO, Spreadtrum, Fujitsu, vivo, ETRI, Ericsson, Lenovo, Apple, ZTE/Sanechips, CATT (may need clarification), Xiaomi (already agreed to reuse legacy), Qualcomm, |
| --- | --- | --- |

### Issue 3 (WID objective 3): CJT calibration reporting for non-ideal synchronization and backhaul

Proposal 3.A

Reason for change: It was agreed that CJTC Dd report can be linked with Rel-18 Type-II CJT, which implies that the CQI/PMI calculation for Rel-18 Type-II CJT can be configured to assume a previously reported CJTC Dd report. Analogous to Rel-19 Type-II CJT mode-1, this can be reflected in the CQI/PMI calculation equation. However, the current description in TS38.214 already captures the necessary behaviour albeit without equation.

Summary of the change: Added the equation for CQI/PMI calculation when CJTC Dd report is linked with Rel-18 Type-II CJT

Consequences if not approved: Possible lack of clarity in UE behaviour when CJTC Dd report is linked with Rel-18 Type-II CJT

##### 5.2.2.5.1b UE assumptions for CQI/PMI/RI calculation for CJT If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-PMI-CQI', the higher layer parameter codebookType is set to 'typeII-CJT-r18' or ' typeII-CJT-PortSelection-r18', and the corresponding CSI-RS Resource Set for channel measurement is configured with 1≤NTRP≤4 CSI-RS resources, for CQI calculation - a UE should assume PDSCH signals on antenna ports in the set [1000,…,1000+υ-1] for υ layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the N0 selected CSI-RS resources, as given by yσ13000(i)⋮yσ13000+P-1(i)yσ23000(i)⋮yσ23000+P-1(i)⋮yσN03000(i)⋮yσN03000+P-1(i)=Wix0i⋮xυ-1i where W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.8 and 5.2.2.2.9 for codebookType set to 'typeII-CJT-r18' and ' typeII-CJT-PortSelection-r18', respectively, and {σ1,…,σN0} are the indices of the N0 selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN0≤NTRP. A UE should assume that the signals yσj, j=1,…,N0, fully overlap in time and frequency. - if the CSI reports with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' is linked to a CSI report with reportQuantity set to 'cjtc-Dd' by the higher layer parameter linkedCJTCReport, and the two CSI reports are jointly triggered, or separately triggered and the compensation is enabled by the higher layer parameter delayOffsetCompensation, a UE should assume PDSCH signals on antenna ports in the set [1000,…,1000+υ-1] for υ layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the N0 selected CSI-RS resources, as given by yσ13000k⋮yσ13000+P-1k⋮yσnref3000(k)⋮yσnref3000+P-1(k)⋮yσN03000(k)⋮yσN03000+P-1(k)=e-j2πk-k0∆f∆τσ1Wσ1(k)⋮Wσnref(k)⋮e-j2π(k-k0)∆f∆τσNWσN(k)x0k⋮xυ-1k where W(k) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.8 and 5.2.2.2.9 for codebookType set to 'typeII-CJT-r18', and {σ1,…,σN0} are the indices of the N0 selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN0≤NTRP, k is the subcarrier index and k0 is the reference subcarrier index, ∆f is the subcarrier spacing and ∆τσn is within the interval [δin,δin+1) in which the delay offset, Dn,offset is reported by the UE. A UE should assume that the signals yσj, j=1,…,N0, fully overlap in time and frequency.

(to be discussed again in RAN1#122bis to reflect that the delay compensation is digital and only on PDSCH)

# References