3GPP TSG-RAN WG2 Meeting #131bis	R2-2506798

Prague, Czech Republic, October 13th – 17th, 2025

Source:	vivo

Title:	Considerations on 6GR user plane

Agenda Item:	10.3.1

Document for:	Discussion and Decision

- Introduction

In RAN#109, Study on 6G Radio was updated in [1]. In this contribution, we provide our initial considerations on 6GR user plane.

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- Discussion

  - User Plane Functions Overview in 6G

Typically, User Plane involves L2 functions, L2 protocol sublayers and mapping between the L2 functions and the L2 sublayers. Considering that L2 functions are the foundation of User Plane and in order to ease the discussion, we think RAN2 can first try to have a consensus on which L2 functions shall be supported in 6GR Day-1. Then, we can dive into the discussion of division of L2 protocol sublayers as well as the functions mapping with each protocol sublayer.

The L2 functions of NR Rel-15 take the LTE L2 functions as the baseline, and it is mainly designed to support eMBB. Several improvements have been introduced to accelerate L2 processing, to support new QoS flow level handling and to basically support new services of 5G like URLLC. In the later releases, more L2 functions/enhancements are introduced to better support IIoT/URLLC, e.g., EHC and intra-UE prioritization, etc. Furthermore, 5G-A supports new media type like XR, and introduces L2 improvements including DSR and PDU set based handling, etc.

As for 6GR, the typical service scenarios being supported by User Plane includes Immersive Communication, Hyper Reliable and Low-Latency Communication (HRLLC) and Massive Communication [2]. As clarified in [2], immersive communication extends eMBB, and requires ‘support of high reliability and low latency for response’ and ‘larger system capacity for simultaneously connecting numerous devices’. HRLLC extends URLLC and ‘is expected to have more stringent requirements on reliability and latency’. Massive communication extends mMTC of 5G and requires IoT devices with long-life batteries. Low power consumption is one of the basic features of massive communication.

Until NR Rel-19, L2 has enormous functions to support the basic eMBB, URLLC and mMTC. When determining the basic L2 functions in 6GR Day-1 to support above 6G services, we think all those L2 functions until NR Rel-19 needs to be taken into account. However, it is not desirable to directly copy what we have in NR Rel-19 to 6GR Day-1, due to potential standard workload and specification complexity. In our opinion, it is better to prioritize those L2 functions with high commercial opportunities in the first 6G release.

In the following table, we make an initial analysis on the existing L2 functions until NR Rel-19, and give our views on whether to support each one in 6GR Day-1. For the sake of intuitiveness, we mark the functions with different highlighted colours. Green means it is the starting point to be supported in 6GR Day-1, Yellow means RAN2 discussion or coordination with other WGs is needed, and Grey means to not considered as the baseline of 6GR Day-1.

Table 1. Initial analysis of the existing L2 functions in NR until Rel-19

| NR L2 functions | Analysis |
| --- | --- |
| Basic L2 functions in NR Rel-15 QoS flow to DRB mapping, Reflective QoS flow to DRB mapping, RoHC, Security protection (ciphering, integrity protection), PDCP duplication, Out-of-order deliver, Reordering, ARQ, (re)Segmentation/assembly, Random Access, DRX, PHR, Timing Advance Maintenance, HARQ operation, Scheduling information reporting (SR/BSR), Uplink Scheduling (Dynamic grant/Configured grant), LCP, Multiplexing/demultiplexing, Downlink Scheduling (Dynamic assignment/SPS) | Naturally, L2 functions in NR Rel-15 can be taken as the baseline. While it needs to be noted that: - The concept of 6G QoS is being discussed in SA2, basic CN-based static QoS framework can be considered as a starting point and further QoS flow related handling in RAN needs to coordinate with SA2. - For reflective QoS, some companies commented UE’s processing burden on always checking of the ‘RDI’ field in the SDAP header. Besides, the frequency of changing the QoS to DRB mapping in real network is low. RAN2 can discuss if to support reflective QoS flow to DRB mapping in 6G Day-1. - In our opinion, HARQ only is not sufficient for RRC signalling and User Plane traffic running on TCP, which require extreme reliability, e.g., lower than 10-6 error rate. A complimentary L2 level retransmission is still needed. We can take ARQ as the starting point of L2 retransmission mechanism. But as will be clarified in subclause 2.2, some improvements especially for immersive communication are worth discussing case by case. - For other L2 functions of NR Rel-15, they can be taken as the starting point of 6GR L2 design. |
| L2 enhancements for IIoT/URLLC in NR Rel-16/Rel-17 Rel-16: Intra-UE prioritization, EHC, Enhanced PDCP duplication with up to 4 legs, Scheduling enhancements (multiplex active CG/SPS, new logical channel restriction) Rel-17: Survival time support, Enhancements to Intra-UE prioritization/multiplexing | As for those L2 functions/enhancements for IIoT/URLLC: - Intra-UE prioritization/multiplexing is quite complex, and its deployment in NR is questionable even though it is specified in Rel-16. We prefer not to consider it in 6GR Day-1. - EHC and survival time support are L2 enhancements specific for IIoT/TSN traffic. The necessity will depend if Operators would like to support 6G integration with IIoT in 6GR Day-1. RAN2 can case by case discuss if to support EHC and Survival time support in Day-1. - Scheduling related enhancements and enhanced PDCP duplication belong to further improvements on the top of Rel-15 basic features. We think it is fine to take them as the starting point of 6GR L2. |
| L2 enhancements for XR in NR Rel-18/Rel-19 Rel-18: Refined long BSR, DSR, DRX enhancements (rational DRX cycle, retransmission less CG), CG enhancements (multiple CG PUSCH transmission occasions with a CG period, unused CG occasion indication), PDU set based discard, PSI-based discard Rel-19: Delay-aware LCP, Enhanced DSR, RLC retransmission related enhancements, Uplink XR rate control | Immersive communication is likely to contribute a major portion of traffic volume in the future network. It is desirable to support immersive communication in 6GR Day-1. Immersive communication demands high data rate in bounded latency with high reliability. It is challenging to support simultaneously connecting numerous devices in radio interface. Besides, devices like XR glasses with form factor limitation put forward demanding power saving requirements. In NR, the major L2 enhancements for XR can be categorized into the following groups: - Scheduling report information enhancements to assist delay-aware and better resource allocation of network. - Scheduling enhancements to guarantee delay-critical data transmission. - Power saving enhancements by considering XR traffic patterns. - XR awareness at RAN to improve experience in congestion. XR is a typical service of immersive communication. The functions specified for XR can be taken as the baseline of 6GR L2 to support immersive communication in Day-1. Any enhancements can be discussed case by case. |
| L2 enhancements for NTN in Rel-17 HARQ disabled mode (LCP restriction, DRX timer control) | As clarified in the SID [1], 6GR shall “Aim at a harmonized 6G Radio design for TN and NTN, including their integration.” The L2 enhancements for NTN in NR are further enhancements on the top of Rel-15 functions, which can also be taken as the baseline for 6GR design. |
| Enhanced L2 functions for IAB | The main impact of IAB and MBS is on the architecture. The impacts on L2 User Plane are minor. We think it is not urgent to support IAB/MBS in 6GR Day-1. |
| Enhanced L2 functions for MBS |  |
| Other L2 functions Rel-17: UDC | UDC is useful to extend the coverage of voice. But considering the fact of deployment scope of UDC in NR, as well as its complexity, we think RAN2 needs to discuss whether to support UDC in 6GR Day-1. |

Based on the above analysis, we think at least the basic L2 functions in NR Rel-15 as well as some further enhancements in later releases (e.g., PDCP enhancements, HARQ disabled mode), the L2 enhancements introduced for XR as mentioned in the above table can be taken as the starting point for the design of 6GR L2 functions.

As mentioned above, in order to better support new services requirements of 6G and further reduce power consumption, as well as to reduce L2 processing complexity and L2 overhead, there might need some rearrangements of L2 functions to L2 sublayers, introduction or removal of some L2 functions, or even removal of some L2 protocol sublayers. We think the investigation of L2 function enhancements/harmonization, new L2 functions and re-allocation of L2 functions to L2 sublayer needs to be considered on a case-by-case basis and shall be provided with clear justification.

At least the basic L2 functions in NR Rel-15, some enhancements on the top of Rel-15 L2 functions in later releases (e.g., PDCP duplication enhancement, HARQ disable mode, etc.) and the L2 enhancements introduced for XR can be taken as the starting point of 6GR L2 functions. Potential L2 function enhancements/harmonization, new L2 functions or re-mapping of L2 functions to L2 sublayer can be considered on a case-by-case basis with clear justification.

In the following table, we further list some motivations and potential new functions for 6GR L2 from our perspective.

Table 2. Potential new L2 functions for 6GR

| Motivation | New L2 functions |
| --- | --- |
| The security protection for user plane traffic at L2 shall be inherited. More and more control information is conveyed via MAC CEs in NR. The security risks of MAC CEs draw companies’ attention. In LTM, companies expressed concerns on that NCC is transmitted via MAC CE in plaintext. Besides, some researches pointed out that illegal attackers may be able to obtain user’s moving trajectory by monitoring specific MAC CEs, e.g., SCell activation/deactivation MAC CE. In order to avoid those potential security risks, we think 6GR can provide security protection for critical MAC CEs. | Security protection in MAC, e.g., for MAC CE |
| As we will further clarify in subclause 2.3, current one-to-one mapping between IP SDU to RLC SDU may result in low processing efficiency of security protection, as well as high L2 header overhead. Concatenation operation at high L2 sublayer can be considered to resolve those issues. | Upper layer concatenation |

In the following subclauses, we will provide our further views on potential L2 enhancements for 6GR case by case.

  - 6G QoS Requirements

In [4], some emerging new traffic characteristics have been studied for 6GR, which may require new QoS functionalities or enhancements to current QoS framework in NR. The following figure shows the traffic characteristics for generative AI services described in TR 22.870 clause 6.25 [3]. A single IP flow may have dynamically changed traffic characteristics according to different AI tasks. And latency requirements may vary differently pending to different users or computing status.

Figure 1. traffic characteristic of typical GenAI applications

The current static QoS mechanism may pose a challenge for the experience of such emerging 6GR services. To achieve better user experience, 6GR needs to consider dynamic QoS change. The potential dynamic QoS control may also require more dynamic RAN resource allocation and transmission (re-)configuration. Such dynamic QoS change may not only from AF/CN but require more UE involvement. It could bring higher efficiency for service adaption especially in RAN, since future 6G terminals probably has AI capability and has more information to better understand end user needs and related services. Thus, whether and how to support the UE and network QoS collaboration to improve performance need further study. On the other hand, RAN related information like radio congestion, energy consumption or computing status may also have impacts to data transmission, which are the requirements for the QoS design. In NR, similar mechanisms, like ECN, rate control and PSI-based PDU discard, have been specified in later 5G release. While in 6GR, we have the chance to define a unified mechanism from 6G Day-1 considering the requirements for all potential services.

Besides, most services other than voice would be mapped to non-GBR, however, it may have difficulty to cope with the sudden high data rate traffic according to the new services, which may lead to a decline in service quality. As a result, how to better support QoS targets than the best effort approach of existing non-GBR services while less intensive resource than current GBR type of services in RAN could be considered.

Based on the design experience from previous generations, the QoS framework has generally been determined and specified in SA2, since QoS inherently involves cross-layer interactions and requires coordination across different system functions. Traditionally, RAN2 has not directly specified QoS mechanisms but has provided the necessary hooks and support from the RAN side.

In the context of 6GR, the notion of dynamic QoS, potentially with UE involvement, has been raised above. Compared with earlier generations, this introduces stronger dependency on RAN-side requirements and capabilities. For example, dynamic QoS adaptation may require real-time measurement input, resource availability awareness, or latency-sensitive scheduling decisions at the RAN side. Therefore, it is important that RAN2 identifies and documents the RAN-related requirements in advance, so that SA2 can take them into account when designing the overall QoS framework for 6GR.

Study RAN impacts and requirements to support dynamic QoS adaption e.g., 6GR node controlled and UE involved. Coordination with SA2 is needed, e.g., some requirements on QoS design from RAN could be provided to SA2.

  - Highly Efficient Security and Processing

In 5G systems, a DRB can be configured to enable Integrity Protection. Meanwhile, RAN2 has discussed issues related to integrity-protected data rate, e.g., by default, a UE supporting integrity-protected DRBs has a minimum data rate capability requirement but cannot support a relatively high or unlimited integrity protection data rate due to processing capability limitations. Consequently, additional integrity protection and verification operations pose challenges to the UE’s processing capability.

In contrast, the peak data rate in 6G will be significantly higher than that in 5G, and the number and complexity of organized PDUs in UE upon UL grant arrival will also increase significantly.

To improve the processing efficiency of integrity protection operations and the assembly of a large number of L2 sub-PDUs in the case of 6G ultra-high data rate, PDCP SDU concatenation is an effective solution. The following figure gives an example of PDCP SDU concatenation.

Figure 2. PDCP SDU Concatenation

From the example of PDCP SDU concatenation above, we can observe that PDCP SDU concatenation can reduce L2 overhead by decreasing the number of L2 sub-header and MAC-Is, which can also simplify L2 processing and reduce delay. The following Table 3 provides the number of bytes reduced for each sub-header at typical SN lengths, such as 12-bit and 18-bit SNs. Table 4 analyses the proportional reduction in L2 header overhead for typical SDU sizes and numbers of concatenated PDCP SDUs, demonstrating that the reduction is particularly significant for small SDU sizes.

Table 3. Reduced overhead per PDCP SDU

| Reduced | 12-bit SN | 18-bit SN |
| --- | --- | --- |
| PDCP overhead | 2 bytes (Header) | 3 bytes (Header) |
|  | 4 bytes (MAC-I) |  |
| RLC header | 2 bytes | 3 bytes |
| MAC subheader | 1 byte |  |

Note：Increased LI num in concatenated PDCP header equals to the reduced LI num in MAC header. Hence, the overhead impact of LI does not need to be taken into account separately.

Table 4. Reduced overhead in some typical scenarios

| L2 overhead proportion reduction (18-bit SN) |  |  |  |
| --- | --- | --- | --- |
| Num of concat. SDUs\SDU size | 40-byte SDU | 576-byte SDU | 1500-byte SDU |
| 5 | 30% -> 8% | 2.3% -> 0.73% | 0.87% -> 0.28% |
| 10 | 30% -> 5.3% | 2.3% -> 0.54% | 0.87% -> 0.21% |
| 20 | 30% -> 3.9% | 2.3% -> 0.44% | 0.87% -> 0.17% |

In the second step, we present simulation results to demonstrate the benefits of PDCP SDU concatenation for improving security efficiency. For PDCP security configuration of DRB, ciphering is mandatory and integrity protection is optional. Integrity protection typically consumes more processing time than ciphering and represents the bottleneck in security processing. We use three typical SDU sizes: 40 bytes, 576 bytes and 1500 bytes, which represent small packets, medium packets (e.g., one of typical video packets) and IP frame packets, respectively.

| Figure 3. Processing Time Reduction percent for 40-byte SDU |  |
| --- | --- |
| Figure 4. Processing Time Reduction percent for 576-byte SDU | Figure 5. Processing Time Reduction percent for 1500-byte SDU |

The benefits of processing time reduction are observed across all scenarios with different security algorithms and typical data packet sizes, demonstrating that PDCP SDU concatenation improves processing efficiency for both integrity protection and ciphering. However, as the number of concatenated SDUs increases, the gain does not grow proportionally. Therefore, a moderate number of SDU concatenations is proposed, e.g., no more than 20 for small packets and no more than 10 for other cases.

Furthermore, PDCP SDU concatenation may be avoided in scenarios that data arrival with non-negligible intervals. This means concatenation prioritizes the SDUs already waiting processing in the buffer and does not need to delay the processing of the previous packet(s) to await subsequent packet(s) for concatenation. When PDCP processing is timely, concatenation may be not needed. Otherwise, appropriate concatenation is performed. Additionally, when link quality deteriorates and TB size decreases, PDCP SDU concatenation should be avoided or the number of concatenated SDUs is reduced to minimize the risks of segmentation in lower layer. PDCP SDU concatenation may occur in the following reasonable scenarios, e.g., when integrity protection processing causes to queuing and waiting, continuous data arrivals, one PDU set, in case of good link quality, etc. Details can be left to UE implementation and AI algorithms can further enhance the efficiency of PDCP SDU concatenation.

RAN2 to investigate solution to improve L2 security processing efficiency and also reduce L2 sub-header overhead & processing complexity, e.g. concatenation in PDCP.

  - 6G L2 Transmission Efficiency Consideration

One of the most interesting things in Internet recent years is the rapid usage of QUIC protocol. QUIC is designed to resolve several performance bottlenecks of the traditional TCP protocol. One important feature of QUIC is to avoid the Head-of-Line (HOL) blocking issue of TCP.

TCP is designed to support lossless transmission and in-order delivery for application data. When multiple independent service flows are multiplexed into the same TCP connection, any packet loss of one service flow will block all other service flows to be delivered to upper layer, even though packets of those flows have been successfully received. This phenomenon has been called as HOL blocking issue. In QUIC, even though multiple service flows can still be multiplexed to the same QUIC connection, each service flow can be allocated to a dedicated stream and transmission of different streams is isolated. Packet loss of one stream will not affect other streams. In other words, HOL blocking issue is avoided.

From the lesson of Internet protocol, we can find that if independent traffic flows are multiplexed into a single queue, there might exist risks of HOL blocking issue. When we closely look at the NR L2 protocol, we can find several multiplexing operations exist, which may also incur HOL blocking issue.

- Case a): In case that several independent streams are multiplexed in a single QUIC connection, the current 5G system has no way to distinguish those streams. In most cases, all traffic of the QUIC connection will be mapped to a single QoS flow and will further be mapped to a single DRB in RAN. We can imagine a situation where one packet is lost during radio interface transmission, then all the following successfully received packets have to be reordered in PDCP layer, even though those packets and the lost packet belong to different streams. Such blocking issue caused by the lost packet will incur unnecessary queueing latency. Hence, we can find that even though QUIC has the capability to resolve HOL blocking issue, its usage may be affected by current 5G design. Companies in SA2 already mentioned QUIC content awareness problem in 6G. We think RAN2 can keep aware of the SA2 progress and identify potential RAN impacts when necessary.

- Case b): NR supports to map multiple QoS flows to a single DRB if they have similar QoS requirements. Such mechanism is useful to reduce the complexity of RAN resource allocation. But it should be noted that if the QoS flows are independent at upper layer, packet loss of one QoS flow will easily incur HOL blocking issue to other irreverent QoS flows. PDCP re-ordering seems necessary only within the same QoS flow or even IP flow, e.g., re-ordering among different flows may cause unnecessary delay and congestion.

- Case c): NR MAC supports multiplexing function, which will multiplex traffic from different logical channels to a single MAC PDU. We note that in L1 specification, it specifies that a TB can only be delivered to MAC layer when it passes the CRC verification. It means that failure decoding of partial code blocks (CBs) or code block groups (CBGs) will block other CB/CBGs being timely decoded and processed by upper layers. Besides, in NR RLC, if some segments are received while others are lost for one SDU, the received ones will finally be dropped. If concatenation is supported in upper layer, there might exist a bunch of IP packets even in one segment. Processing the segment instead of direct dropping will improve resource efficiency and user experience.

- PDCP re-ordering seems necessary only within the same QoS flow or even IP flow, e.g., re-ordering among different flows may cause unnecessary delay and congestion.

- Partial data successfully received in lower layer is better to be delivered to higher layer for continuous processing instead of dropping, which needs no more radio resources and can improve reception experience and radio resource efficiency.

In current 5G system, there are two separate SNs in RLC and PDCP layer, which are used for ARQ & (re-)segmentation functions and security & re-ordering functions respectively. These functions should be all supported in 6G. We can investigate the possibility of joint SN design in 6G in order to avoid duplicated header overhead and simplify L2 processing. For example, it can be analyzed and evaluated whether ARQ & (re-)segmentation functions can be merged into security & re-ordering functions. There are overlapping parts, e.g. receiving window mechanism, between ARQ and re-ordering functions, which can be jointly considered with fast L2 retransmission mechanisms. (Re-)Segmentation is a real-time processing and adds segmentation information according to UL grant and LCP allocated resource size. It seems feasible to re-use SN of security for (re)segmentation in the cases of intra-node or non-split functional location.

However, in 5G, there are several architectures, e.g. DC, CU-DU split with F1 interface, split bearer for the cases of CA duplication/ L2 SL Relay/ IAB, which all have impacts on function/sub-layer split between two nodes or two entities. If these architectures would be supported in 6G Day-1 or later release, the joint design of L2 SN would also be impacted. We provide one example of function split with joint SN design in figure 6: higher part of a common layer locates in higher node and lower part of that locates in lower node.

Figure 6. Function split with joint SN

In summary, joint SN design will introduce some benefits, e.g. overhead reduction, avoidance of duplicated functions and then L2 efficiency improvement. Meanwhile, it is slightly inconvenient to support the split architectures and inter-working with 5G. RAN2 can further evaluate the feasibility and detailed impacts.

- Two separate SNs for ARQ & (re-)segmentation function and security & re-ordering function may be considered to jointly design in 6G.

Hence, we propose:

Study solutions to reduce L2 HOL (Head of Line) blocking impacts and investigate joint sequence numbering in order to improve L2 transmission efficiency in 6G.

  - Fast L2 Retransmission

NR RLC was designed for either low latency (i.e. UM mode) or high reliability (i.e. AM mode) and packet loss for Rel-15 AM mode transmission would lead to RLF. For new emerging services like immersive traffic and mobile AI, both high reliability and low latency are required according to [2][3]. RLC AM mode is normally configured for these services. However, due to current NR RLC mechanism and parameters setting in the field, the retransmission normally takes 30ms-50ms, and up to hundreds of ms in worse cases. This cannot meet the latency requirement for above emerging new services. In NR R19, remaining-time-based RLC retransmission/polling were introduced. However, such add-on solution may have some design limitation e.g. potential radio resource waste for “blind retransmission” and extra status report.

In 6GR, we have the chance to re-visit the issue and design in more efficient way without considering backward compatibility. Normally, ARQ is used to cover HARQ residual errors from improper MCS or NACK-to-ACK detection. Current ARQ design in NR is based on timer triggered status reporting due to lack of information of HARQ transmission status for DL and UL. However, the timer cannot be determined very accurately due to different UE radio condition, interference situation and network congestion status. Large value of configured timer would lead to extra delay while small value may result in unnecessary retransmission that further burden the radio in congestion cases.

Look into the details on the issue that ARQ is lack of information of HARQ transmission. For DL transmission, due to NACK to ACK error, RAN node would trigger improper HARQ new transmission with NDI toggled. UE cannot trigger fast status report due to lack of information of RLC PDU to TB mapping in the receiving side even HARQ error can be detected.

Figure 7. NACK-to-ACK error for DL data transmission

For UL transmission, RAN node may stop schedule HARQ retransmission since MCS is inappropriate. UE cannot identify whether the previous HARQ transmission is successful or not. RAN node also need to wait the RLC timer expire to trigger the status report due to lack of information of RLC PDU to TB mapping even HARQ error can be detected.

Figure 8. HARQ retransmission abandon for MCS adaption

As a result, feedback mechanism for ARQ could be re-visited to enable fast L2 retransmission. For example, the HARQ transmission status could be used to trigger ARQ retransmission efficiently. Since such enhanced feedback information can be obtained naturally close to HARQ, lower L2 (e.g. MAC) data rebuilding including re-segmentation or/and multiplexing could also be further investigated. It is believed that fast L2 retransmission would not only help to meet performance requirements for hyper reliable and low-latency communication, but also enable hardware friendly design to save UE buffer occupation and further reduce header overhead.

Study solutions to better support new emerging services with low latency and high reliability requirements than NR. Effective feedback to enable fast ARQ-like L2 retransmission should be considered.

  - Buffer Status and Delay Status Reporting

In NR, Buffer Status Report (BSR) was introduced in R15 and enhanced BSR with finer granularity was introduced in R18. For supporting delay-aware handling at gNB, e.g. scheduling, single entry Delay Status Report (DSR) for reporting at most single pair of buffer size/remaining time per LCG and multiple entry DSR for reporting multiple pairs of buffer size/remaining times per LCG were introduced in R18 and R19 respectively. Similar functionalities need to be supported in 6GR, while the existing BSR/DSR procedure in 5G should be considered as a starting point for 6GR. As BSR, DSR and the related enhancements were introduced gradually in different releases, the UEs of earlier releases cannot benefit from BSR/DSR enhancements in NR. For 6GR, there is an opportunity to review and jointly consider these similar BSR/DSR functions in 6G Day-1as there is no restriction of backward compatibility for 6G Day-1:

- Support BSR in Day-1: As in NR, BSR should be supported as a mandatory capability of UE in 6GR Day-1. With even larger carrier bandwidth and better spectrum utilization efficiency for 6GR than NR, it is supposed that 6GR should be able to provide even higher uplink data rate than NR. This means that 6GR can serve traffics with even higher data rates. For one instance, immersive communication with ultra-HD video of high data rate could be one of the most promising services to be served in 6GR, which means the UL data volume of 6GR can be much larger than NR. Considering this, BSR for 6GR should be designed to report even larger data volume, with the trade-off between overhead and granularity considered.

- Support DSR in Day-1: For 6GR, to achieve better user experience, the delay budget for data transmission of delay sensitive service (e.g. XR) can be even shorter than that in NR, it is essential to support delay aware handling in order to fulfil the tight delay budget. To ensure the uplink data transmission latency management for 6GR is at least not worse than 5G, delay aware handling should be supported in the first release of 6GR. Considering this, DSR function should be supported in the first release of 6GR.

- Consider harmonization of BSR and DSR: In NR, BSR is triggered based on the data arrival/availability and used to report the whole buffer status per LCG, while DSR is triggered based on remaining time threshold and used to report the buffer sizes of urgent data and the associated remaining times per LCG. Currently, BSR procedure and DSR procedure are relatively independent in NR specification, for instance,

  - Both pending BSR and pending DSR can trigger scheduling request when there is no uplink grant for BSR and DSR transmission, respectively;

  - BSR MAC CE and DSR MAC CE are generated and transmitted separately;

  - BSR cancellation and DSR cancellation are operated separately.

6GR gives us an opportunity to rethink about these two functions. On issue is whether these two functions should be fully isolated as in NR. However, a large portion of the information contained in BSR and DSR is overlapping, especially for the traffic with critical latency requirements, which introduces an additional burden for the UE, in NR. We may consider a harmonized design for BSR and DSR in 6GR. For example, the BSR could carry delay information and be reported as such, or the DSR could include more data volume information for non-delay-critical data. Anyway, such kinds of detailed designs could be further studied.

6GR should study the delay aware handling with the supporting of BSR/DSR functionalities, and harmonized design of BSR and DSR should be considered, from Day-1.

  - UE power saving

UE power saving has always been one of the critical aspects of RAN design. With the transition towards 6G, this issue becomes even more prominent due to wider range of device types (e.g. Smart phone, XR, industrial IoT, metaverse, etc.), and the increasingly diverse services (e.g. eMBB, XR/immersive, Massive IoT, Sensing, AI, Voice, Regulatory service, etc.), based on the 6G study as shown in RP-252960. In RRC connected mode, periodic PDCCH monitoring is a major contributor to UE power consumption.

In 5G Day-1, C-DRX was introduced as the first standardized mechanism, where UEs monitor PDCCH during OnDurationTimer running. However, since UEs must continuously monitor PDCCH during the regardless of traffic presence, this still leads to non-negligible power consumption.

Therefore, additional mechanisms were introduced in later releases (e.g., DCP in Rel-16, LP-WUS/WUR in Rel-18/19) to further reduce unnecessary PDCCH monitoring. Considering the increasing diversity of devices and use cases in 6G, a more efficient, unified, and flexible design for PDCCH monitoring optimization is required from Day-1.

- UE power saving demand in 6G is stronger than in 5G, and periodic PDCCH monitoring remains as a significant source of power consumption. Therefore, it is essential to support efficient mechanisms for wake-up based PDCCH monitoring from Day-1.

In Rel-16, the DCP feature was standardized to allow UEs to wake up before OnDurationTimer to monitor DCP, which could skip subsequent PDCCH monitoring in case there is no scheduling. 
In Rel-18/19, the LP-WUS/WUR feature was studied and then standardized, introducing wake-up signals to trigger UE monitoring of PDCCH. Two options were specified, which are shown in Fig.8.

- Option 1-1: Similar to DCP, where LP-WUS is coupled with C-DRX OnDuration.

- Option 1-2: More flexible, allowing LP-WUS to be monitored at any time outside active time, which is decoupled from C-DRX.

Figure 9. LP-WUS operation in 5G

While both features target the same issue, i.e. reducing PDCCH monitoring power consumption. The coexistence of DCP and LP-WUS leads to functional duplication. In practice, LP-WUS (especially Option 1-2) achieves better power saving due to a separate wake-up receiver design, and lower latency performance with enough flexibility. For 6G, redundant features should be avoided, and a unified mechanism should be prioritized.

Besides, 5G Rel-19 standardized both Option 1-1 and Option 1-2 for LP-WUS. However, Option 1-2 can fully realize the functionality of Option 1-1 while being more flexible and efficient, as it decouples wake-up signaling from C-DRX PDCCH monitoring. Thus, standardizing both options was unnecessary and led to redundant complexity. For 6G, there is no need to standardize multiple duplicated options for the same functionality. A single, flexible solution (e.g., equivalent to 5G LP-WUS Option 1-2) is sufficient.

- Multiple duplicated features increased complexity while providing limited incremental benefit. A single unified wake-up solution for PDCCH monitoring reduction should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

The effectiveness of PDCCH monitoring power saving depends on the availability of mechanisms such as LP-WUS/WUR. In 5G, such features only became available in later releases, limiting the overall power saving gain in earlier deployments. Given the importance of UE power saving, PDCCH monitoring optimization (e.g., LP-WUS/WUR) should be supported from Day-1 in 6G, instead of being introduced gradually in later releases.

Finally, the design of PDCCH monitoring optimization requires cross-WG coordination:

- RAN1: Study signalling and receiver design aspects of LP-WUS/WUR, including waveform and detection.

- RAN2: Define procedures for LP-WUS monitoring, and PDCCH monitoring after waking up.

Tight coordination between RAN1 and RAN2 is needed to ensure that both physical layer design (signalling/receiver aspects) and higher-layer procedures for PDCCH monitoring are matched.

For RRC connected state in 6GR, RAN2 considers to support LP-WUS/WUR for PDCCH monitoring from Day-1:

  - A unified mechanism should be introduced, avoiding duplicated or redundant features;

  - There is no need to couple with C-DRX PDCCH monitoring, i.e. Option 1-2 in 5G is reused as the starting point;

  - Coordination with RAN1 is needed.

- Conclusion

In this paper, we provide our initial 6GR user plane consideration and have the following observations and proposals:

Proposal 1: At least the basic L2 functions in NR Rel-15, some enhancements on the top of Rel-15 L2 functions in later releases (e.g., PDCP duplication enhancement, HARQ disable mode, etc.) and the L2 enhancements introduced for XR can be taken as the starting point of 6GR L2 functions. Potential L2 function enhancements/harmonization, new L2 functions or re-mapping of L2 functions to L2 sublayer can be considered on a case-by-case basis with clear justification.

6G QoS related:

Proposal 2: Study RAN impacts and requirements to support dynamic QoS adaption e.g., 6GR node controlled and UE involved. Coordination with SA2 is needed, e.g., some requirements on QoS design from RAN could be provided to SA2.

L2 Processing and transmission efficiency:

Proposal 3: RAN2 to investigate solution to improve L2 security processing efficiency and also reduce L2 sub-header overhead & processing complexity, e.g. concatenation in PDCP.

Observation 1:PDCP re-ordering seems necessary only within the same QoS flow or even IP flow, e.g., re-ordering among different flows may cause unnecessary delay and congestion.

Observation 2:Partial data successfully received in lower layer is better to be delivered to higher layer for continuous processing instead of dropping, which needs no more radio resources and can improve reception experience and radio resource efficiency.

Observation 3:Two separate SNs for ARQ & (re-)segmentation function and security & re-ordering function may be considered to jointly design in 6G.

Proposal 4: Study solutions to reduce L2 HOL (Head of Line) blocking impacts and investigate joint sequence numbering in order to improve L2 transmission efficiency in 6G.

Proposal 5: Study solutions to better support new emerging services with low latency and high reliability requirements than NR. Effective feedback to enable fast ARQ-like L2 retransmission should be considered.

MAC related aspects:

Proposal 6: 6GR should study the delay aware handling with the supporting of BSR/DSR functionalities, and harmonized design of BSR and DSR should be considered, from Day-1.

Observation 4: UE power saving demand in 6G is stronger than in 5G, and periodic PDCCH monitoring remains as a significant source of power consumption. Therefore, it is essential to support efficient mechanisms for wake-up based PDCCH monitoring from Day-1.

Observation 5: Multiple duplicated features increased complexity while providing limited incremental benefit. A single unified wake-up solution for PDCCH monitoring reduction should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

Proposal 7: For RRC connected state in 6GR, RAN2 considers to support LP-WUS/WUR for PDCCH monitoring from Day-1:

i  A unified mechanism should be introduced, avoiding duplicated or redundant features;

ii  There is no need to couple with C-DRX PDCCH monitoring, i.e. Option 1-2 in 5G is reused as the starting point;

iii  Coordination with RAN1 is needed.

- Reference

- RP-252912	Revised SID: Study on 6G Radio

- Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond". (https://www.itu.int/dms_pubrec/itu-r/rec/m/R-REC-M.2160-0-202311-I!!PDF-E.pdf).

- TR22.870 Study on 6G Use Cases and Service Requirements

- S2-256637 QoS Framework for 6G

- TS 23.501