3GPP TSG RAN WG2 #131bis			R2-2506806

Prague, Czech Republic, Oct 13th – 17th, 2025

Source: 	Fainity Innovation

Title:	Views on 6GR

Agenda item:	10.2

Document for:	Discussion & Decision

Introduction

Study on 6G Radio (6GR) with enumerated objectives in RP-251881 [1] was approved in RAN Plenary #108. As planned by RAN2 chair for the 1st SID meeting, companies were invited to provide principles, guidelines, deployment scenarios and architectures, and other general aspects for 6GR. Accordingly, we prepare this document to share our views on these aspects. The agenda of this document is arranged that we will describe the preferred principles for 6GR design in section 2. In the subsequent sections, we illustrate our observations include pain points from 5G, for deployment scenario and diverse device types aspects. Consequently, the discussions and proposals for each aspect are provided.

Design principles for 6GR

From our perspective, three high-level principles should be always kept in mind during the study and normative stages of 6GR:

- Simple framework: 6GR aims for a simple, less configurable air interface. This includes streamlined upper layer control and configuration, mobility and measurement framework, and MAC operations. The simplification efforts can be made by:

  - Avoiding multiple deployment options and migration options and aiming to use a common protocol stack for all use cases and RRC states.

  - A unified mobility handling design, measurement and reporting framework for L1 beam level mobility and L3 cell level mobility. In addition, massive RRC configurations and various mobility options should be avoided.

  - Modular structure to support diverse device types whereas a basic module is commonly used to perform initial access and other category/feature modules could be introduced via a simple UE capability signaling.

  - Harmonizing CA/DC/BWP into a common framework and leveraging a unified activation and deactivation control signaling to support spectrum utilization and operations.

- Sufficient coverage: 6GR targets sufficient coverage across all frequency ranges which include full range of FR1 (up to 7.125 GHz), the range between FR1 and FR2-1 (including around 7GHz), and FR2-1 (24.25 GHz – 52.6 GHz), specifically, at least the control channels upon FR2-1 shall have similar coverage with FR1 under the same power constraints. In addition, ubiquitous voice coverage is also the goal via the TN/NTN integration. The efforts might include:

  - Uplink coverage enhancement by carrier/waveform selection whereas uplink waveform and/or beam selection with RA enhancement e.g., one or multiple PRACH attempts at different resource could be worked together with power control and power headroom reporting schemes.

  - DL/UL decoupling together with cell free network (or called as UE-centric clustering and mobility management) is utilized by distributed massive MIMO, e.g., multiple TRPs / asymmetric TRPs, to support respective purposes and functions.

  - Voice over NTN and services continuity during TN/NTN mobility, especially 5G NTN handover to/from 6G TN (Inter-RAT Handover) should be studied.

- Significant energy efficiency for both NW and UE: 6GR focuses on lower energy consumption for both network and UE sides while keeping similar QoS performance. Meanwhile, above two principles should also be sustained. The design aspects may include:

  - Joint NW and UE power consumption model and assumptions are first investigated.

  - Revisit all 5G UE and NW power saving features and jointly take them into account for all RRC states and mobility scenarios, especially on PDCCH monitoring and RRM measurement relaxation.

  - Consider Low-Power receiver architecture upon initial access and leveraging the architecture to support energy efficient measurements. In addition, exchange information related to 6G NB power state and UE power state to enable excellent alignment and consequent controls.

  - Widely apply AI/ML to generate the inference to eliminate the necessary transmissions, receptions, and measurements for RRC_CONNECTED UE.

Proposal #1: The design principles for 6GR are to prioritize a simple framework while ensuring sufficient coverage across different frequency ranges and scenarios and achieving significant energy efficiency for both network and UE.

Proposal #2: R2 is suggested to study simplification for at least deployment options and migration options.

Proposal #3: R2 is suggested to identify the potential L2/L3 aspects on 6G coverage and energy efficiency.

Discussions

Aspects of 6G deployment scenarios and migration from 5G

Pain points: The numerous NSA options developed in 5G with early/normal/late drop specifications introduced significant complexity in standardization and also confused the market and verticals. However, development with only one EN-DC seeing widespread commercial adoption. Specifically, the control plane in 5G experienced growing complexity due to incremental deployment options and feature additions, leading to bloated specifications that were difficult to interpret and implement. On the other hand, using NSA connectivity resulted in significant increases in UE battery consumption. Unfortunately, the prevalence of NSA operations also ultimately delayed the broader adoption of 5G SA deployments which impacting SA's full potential and enforce subscribers/verticals doubt the success of 5G. Likely, 6G SA deployment option is promising through the lessons learned from 5G and a smooth transition from 5G to 6G is expected. However, software upgrades from existing 5G networks are not always feasible. The migration path needs to leverage existing 5G infrastructure without being solely dependent on software upgrades. Following, we enumerate the additional relevant design aspects to be discussed under 6G SA and its migration:

MRSS: MRSS aims to utilize existing 5G spectrum for 6G, allowing operators to dynamically re-farm bands like 600/700/2600/3500MHz and support smooth migration without upgrading too much deployment and hardware. This also requires 5G NR compatible waveforms, numerology, and channel coding for efficient sharing. Unlike 5G DSS, MRSS not only passively punctuates the overlapped resources, but aggressively reuse or coordinate the control signaling across 5G and 6G is expected to take place upon a shared band. Per our understanding, the MRSS scenarios may include the combination of following properties: (1) co-location /non-colocation of 5G/6G BS (2) intra-vendor/inter-vendor (or multiple-vendors) deployment (3) UE transparency / UE-aware operation mode (4) shared/non-shared control signaling. Upon MRSS scenario, the measurement and mobility between 5G NR and 6GR may become a tricky case whereas both RATs may comply with beamforming in the same band. Thereafter, how to design an L1-based beam-level mobility and configure the relevant measurement (e.g., whether a measurement gap is necessary) and reporting configurations (e.g., reporting type and UCI reporting signaling) to support smooth migration should be studied. As highlighted by the above design principle, a simple framework preventing too many options is imperative for the market. We suggest R2 to down select the practical scenarios and then identify the necessary operations accordingly.

Observation#1: MRSS scenario creates ambiguous measurement conditions (inter-RAT vs.., intra-frequency) and how to enable efficient mobility will impact the migration from 5G to 6G.

Proposal #4: R2 is suggested to study the practical MRSS scenario and then identify the necessary operations for corresponding inter-RAT mobility.

TN-NTN harmonization: NTN environments present unique challenges like high frequency and time offsets due to satellite movement and large RTT. Unfortunately, 5G NTN standardization was carried out in later releases, and solutions that were amended on top of the existing NR design. This non-native integration in 5G required significant changes in UE implementation for NTN commercialization. Moreover, 5G NTN suffered from a lack of advanced dynamic resource allocation, beam management, and interference mitigation with TN. This resulted in poor TN-NTN handover performance, hindering seamless mobility. On the other hand, 5G NTN is trying to being commercialized in recent years, and introducing a new 6G solution may lead to market fragmentation. To address these, 6G aims for a harmonization design for 6G TN and 5G/6G NTN from the beginning and the goal is to shoot for ubiquitous NTN connectivity especially on voice services. To deal with the issues, LTM with early DL beam activation and early UL synchronization allows these time-consuming procedures to be performed in advance and we believe the LTM framework can serve as a good baseline (i.e., NTN can be configured as an LTM candidate) for studying seamless TN/NTN connectivity. Under NTN especial characteristics, the serving cell may configure appropriate L1/L3 measurement reporting with event-triggered conditions.

Observation#2: 6G TN and 5G/6G NTN harmonization and migration involve complicated deployment scenarios, but we shall aim an unified solution to simplify the coordination.

Proposal #5: By leveraging LTM framework, TN and NTN could be harmonized via L1 beam-level /L3 cell-level mobility and then perform cell switching if appropriately.

Aspects of how to design a single RAT to support diverse device types

Paint points: The diverse devices with various needs cause excessive complexity and fragmentation in standardization. 5G's "toolbox-like" air interface, with numerous options and configurations, resulted in high implementation complexity, slow time-to-market, poor feature adoption, creating market fragmentation. Meanwhile, 5G NSA introduced tight interoperability issues, increased radio complexity at the UE side, but had a very low commercialization ratio despite huge standardization efforts to specific device type. Additionally, achieving better power efficiency on the UE side especially for small form factor and battery-constrained IoT devices is very critical. However, past energy-saving mechanisms were often introduced late in 5G releases, leading to backward compatibility limitations and difficulties in commercialization. Consequently, while introducing new device types in later releases, such as RedCap and XR devices in 5G, it faced stronger obstacles and had less chance for widespread deployment due to design constraints and legacy requirements. In summary, the challenge for 6GR is to overcome the fragmentation and complexity inherited from 5G by adopting a more lean, scalable, and modular design that natively supports a diverse range of devices and services from Day 1, ensuring a balance between flexibility and implementation cost. The relevant design aspects are further discussed below:

UE capability framework: 5G NR's UE capability framework has led to excessive configurations, numerous UE capabilities, and extensive reporting, resulting in high complexity. There are too many use cases, and the standardization framework is too rigid. This leads to "free rider" and "sticking" problems, where operators are reluctant to invest in certain features if UEs don't uniformly support or report them, and UEs won't report unless mandated. In 6G, it is desired to support more diverse device types which have different form factors, power consumption characteristics, and capabilities. It is challenging to efficiently handle capability signaling for such a wide variety of devices while enabling 6G functions. Specifically, once capabilities are reported, they are largely static; changes of UE capabilities require releasing and re-establishing the RRC connection. This is problematic if a UE needs to temporarily adjust capabilities due to conditions like low power or overheating. Meanwhile, how to effectively manage capabilities for diverse device types, particularly distinguishing between "UE category" and "UE capability" is problematic under different device types. For example, in 4G (LTE Release 8), "UE categories" (e.g., Cat 1, Cat 2) simply grouped capabilities like maximum data rates. However, with Carrier Aggregation (CA) introduced in R10, this became overly complicated, as total data rate alone couldn't differentiate between various configurations. 5G moved away from explicit categories for smartphones, relying on granular, individual capability indications, which increased the amount of information to be signaled. An efficient UE capability retrieval and flexible update mechanism should be considered in 6G.

Observation#3: UE capability retrieval should be enhanced to minimize the reporting overhead and enable the flexible update of capabilities.

Proposal #6: R2 is suggested to design a modular UE capability retrieval mechanism with UE-initiated dynamic capability updates for supporting diverse device types.

Waveform and numerology: Considering various requirements across different device types and services, it is difficult to adopt a single waveform to fulfill all needs. Alternatively, a basic waveform set with dynamic switching based on the channel environment is expected to resolve most cases. The remaining use cases might be supported via other category-added modules which may introduce a separate waveform function if the need is identified. Regarding the basic waveform, CP-OFDM for DL and CP-OFDM/DFT-S-OFDM for UL is widely commercialized with affordable performance thus it can be reused for the basic waveform set for 6G. The switching of waveform is proposed to be L1-based and fully dynamic by enabling single-symbol transmissions for reduced latency and energy saving. On the other hand, although applying different numerologies to respective BWPs within the same band can lead to different latency performance, 6G aims to streamline numerologies, potentially avoiding multiple numerologies for the same band or sub-range. Thus, per-band numerology and appropriate band selection during cell selection might act as a common procedure for diverse device types and then the common procedure could simplify the configuration through a fixed numerology setting in each band from both UE and NW perspective. The open issue is at which step to provision the band-specific numerology during initial access and how to modify the cell selection criteria with providing potential assistance information to redirect various devices to a suitable band.

Observation#4: Regardless of different device types, a common waveform and numerology could simplify the initial access procedure, afterwards, NW and UE might negotiate the targeting application and adapt the physical setting.

Proposal #7: A common waveform set, and initial access procedure is mandated for all device types. R2 can consider a new cell selection criterion with assistance information for redirecting various device types to the band with appropriate numerology.

Conclusions

In this contribution, we share our views and observations on 6G and propose:

Observation#1: MRSS scenario creates ambiguous measurement conditions (inter-RAT vs.., intra-frequency) and how to enable efficient mobility will impact the migration from 5G to 6G.

Observation#2: 6G TN and 5G/6G NTN harmonization and migration involve complicated deployment scenarios, but we shall aim at a unified solution to simplify the coordination.

Observation#3: UE capability retrieval should be enhanced to minimize the reporting overhead and enable the flexible update of capabilities.

Observation#4: Regardless of different device types, a common waveform and numerology could simplify the initial access procedure, afterwards, NW and UE might negotiate the targeting application and adapt the physical setting.

Proposal #1: The design principles for 6GR is to prioritize a simple framework while ensuring sufficient coverage across different frequency ranges and scenarios and achieving significant energy efficiency for both network and UE.

Proposal #2: R2 is suggested to study simplification for at least deployment options and migration options.

Proposal #3: R2 is suggested to identify the potential L2/L3 aspects on 6G coverage and energy efficiency.

Proposal #4: R2 is suggested to study the practical MRSS scenario and then identify the necessary operations for corresponding inter-RAT mobility.

Proposal #5: By leveraging LTM framework, TN and NTN could be harmonized via L1 beam-level /L3 cell-level mobility and then perform cell switching if appropriately.

Proposal #6: R2 is suggested to design a modular UE capability retrieval mechanism with UE-initiated dynamic capability updates for supporting diverse device types.

Proposal #7: A common waveform set, and initial access procedure is mandated for all device types. R2 can consider a new cell selection criterion with assistance information for redirecting various device types to the band with appropriate numerology.

References

- RP- 251881, New SID: Study on 6G Radio