3GPP TSG-RAN WG2 Meeting #131bis                                                               R2-2506818

Prague, Czech Republic, Oct. 13th-17th

Source:	CATT

Title:	Discussion on MAC open issues

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the following remaining open issues on MAC:

- MAC Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587)

- MAC Issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

Our proposals are summarized in Section 3.

Discussion

  - [MAC Issue 1]

In RAN2#130 meeting, the following agreements were achieved on RA-RNTI collision issue for PRACH adaptation.

No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. RAN2 understand there is no MAC impact. Send an LS to RAN1 to inform RAN2 agreement.

RAN2 sent an LS to RAN1. Then RAN1 replied the LS [1]. According to the replied LS, from RAN1 perspective, there is no proper place in RAN1 specifications to capture the impact on f_id since f_id is not defined in RAN1 specifications. The index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id.

Furthermore, RAN1 provided the below RAN1 agreements in the replied LS, and kindly requests RAN2 to take into account with regards to the impact on f_id of valid RO in TS38.321.

Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

Adaptation of PRACH is supported to be configured as additional PRACH resources for legacy R15 PRACH resources and RACH partitioning. Based on the previous discussion, there is no RA-RNTI collision issue within legacy R15 PRACH resources and PRACH resources for RACH partitioning. Hence, we think the RA-RNTI collision issue for PRACH adaptation only exists within same RACH-ConfigCommon. Then, there are two possible options for reinterpretation of f_id for the additional PRACH resources:

- Option 1: f_id for the additional PRACH resources is always reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs within the same RACH-ConfigCommon.

- Option 2: f_id for the additional PRACH resources is reinterpreted as the frequency-domain index of the PRACH occasion plus the number of FDMed legacy ROs within the same RACH-ConfigCommon when additional PRACH resources and legacy PRACH resources are overlapped in time domain, otherwise, it is interpreted as the frequency-domain index of the PRACH occasion.

Available configured additional PRACH resources of option 2 are more than those of option 1. But with option 2, the network needs to consider whether additional PRACH resources and legacy PRACH resources are overlapped in time domain when Msg1-FDM for the additional PRACH resources is configured while the UE has two alternatives for reinterpretation of f_id for the additional PRACH resources. To simplify UE behavior and network implementation, we slightly prefer option 1.

Proposal 1: RAN2 confirm f_id for the additional PRACH resources is always reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs within the same RACH-ConfigCommon. Capture this in TS 38.321.

The corresponding TP is provided in TP1 in Annex.

Furthermore, RAN2 need to clarify in the field description of msg1-FDM in addlRACH-Config-Adapt that the network does not configure the total value of msg1-FDM-r19 in addlRACH-Config-Adapt and msg1-FDM (without suffix) in rach-ConfigGeneric configured in the associated RACH-ConfigCommon is more than 8.

Proposal 2: Clarify in the field description of msg1-FDM in addlRACH-Config-Adapt that the network ensures that the total value of msg1-FDM-r19 in addlRACH-Config-Adapt and msg1-FDM (without suffix) in rach-ConfigGeneric configured in the associated RACH-ConfigCommon is not more than 8.

The corresponding TP is provided in TP2 in Annex.

  - [MAC Issue 2]

Based on the discussion in [POST131][111][NES], RAN2 agreed to support the SUL and UL for OD-SIB1 procedure. Thus, the below procedure on Redcap UE over SUL may be impacted, the above MAC issue 2 is proposed to see if "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring, and whether network can know that for a redcap UE performing an OD-SIB1 request.

TS 38.321 5.15.1 Downlink and Uplink <skip> Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall: 1> if initialUplinkBWP-RedCap is configured for the selected carrier: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap. 1> else: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP. 1> if initialDownlinkBWP-RedCap is configured: 2> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP. 2> else: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. 1> else: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

For the case of a UE moving from Cell A to NES Cell, the UE doesn’t have initialUplinkBWP-RedCap/initialDownlinkBWP-RedCap information as yellow part, as which is configured in NES Cell’s SIB1, it means that above procedure has no impact to UEs moving from Cell A to NES Cell.

For the case of a UE already camping in NES Cell, if it is Redcap UE, it may have already stored the SIB1 information on RedCap feature. Based on above green part for SI request, if the selected carrier is SUL, the UE will monitor PDCCH on the BWP configured by initialDownlinkBWP. In addition, it can be seen that the procedure on initialDownlinkBWP-RedCap will only be used on normal carrier, more details could be seen in below procedure in TS 38.331.

TS 38.331 5.2.2.3.3 Request for on demand system information <skip> 1> else if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigRedCap and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met: 2> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigRedcap corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 2> if acknowledgement for SI request is received from lower layers: 3> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

si-RequestConfigRedCap Configuration of Msg1 resources for initialUplinkBWP-RedCap that the (e)RedCap UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting

Based on above procedure in TS 38.331, for normal UL carrier, it can be seen that RedCap UE will trigger the Msg.1 using the RRACH preambles and PRACH resource(s) in si-RequestConfigRedCap IE if initialUplinkBWP-RedCap is configured, then network could identify the OD-SI request is from RedCap UE or not, if it is RedCap UE, the network could transmit the RAR information on the BWP configured by initialDownlinkBWP-RedCap.

Observation 1: In legacy SI request procedure, when initiaUplinkBWP-RedCap is configured, the network could identify the UE is RedCap or not over the normal UL carrier based on the used PRACH preamble(s) and PRACH resource(s) in si-RequestConfigRedcap, then transfer the RAR on the BWP configured by initialDownlinkBWP-RedCap for RedCap UE.

For OD-SIB1 request procedure, until now, there is no discussion on the RedCap-specific PRACH preamble(s) and PRACH resource, then the network could not identify the OD-SIB1 request is from Redcap UE or not. Considering this enhancement is also in the scope of RAN1, it is suggested to not support this function. Then, the correct procedure is, if RedCap UE has stored the initialDownlinkBWP-RedCap, when OD-SIB1 request is triggered, no matter it is on SUL or normal UL carrier, the RedCap UE will always monitor the PDCCH on the BWP configured by initialDownlinkBWP, the change is seen in TP3 in Annex.

Proposal 3: Not to introduce OD-SIB1 specific RA resource for RedCap UE.

Proposal 4: If RedCap UE has stored the initialDownlinkBWP-RedCap, when OD-SIB1 request is triggered, no matter it is on SUL or normal UL carrier, the RedCap UE will always monitor the PDCCH on the BWP configured by initialDownlinkBWP. The change in TP3 in Annex is accepted.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: In legacy SI request procedure, when initiaUplinkBWP-RedCap is configured, the network could identify the UE is RedCap or not over the normal UL carrier based on the used PRACH preamble(s) and PRACH resource(s) in si-RequestConfigRedcap, then transfer the RAR on the BWP configured by initialDownlinkBWP-RedCap for RedCap UE.

Proposal 1: RAN2 confirm f_id for the additional PRACH resources is always reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs within the same RACH-ConfigCommon. Capture this in TS 38.321.

Proposal 2: Clarify in the field description of msg1-FDM in addlRACH-Config-Adapt that the network ensures that the total value of msg1-FDM-r19 in addlRACH-Config-Adapt and msg1-FDM (without suffix) in rach-ConfigGeneric configured in the associated RACH-ConfigCommon is not more than 8.

Proposal 3: Not to introduce OD-SIB1 specific RA resource for RedCap UE.

Proposal 4: If RedCap UE has stored the initialDownlinkBWP-RedCap, when OD-SIB1 request is triggered, no matter it is on SUL or normal UL carrier, the RedCap UE will always monitor the PDCCH on the BWP configured by initialDownlinkBWP. The change in TP3 in Annex is accepted.

Reference

- R1-2506587, Reply LS on RA-RNTI for PRACH adaptation

Annex

  - TP1 of TS 38.321

5.1.3	Random Access Preamble transmission

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission; and

1>	if the Random Access procedure is not initiated by the PDCCH order for an LTM candidate cell:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell as preamble re-transmission; and

1>	if the PDCCH order indicates the same LTM candidate cell and the same SSB as the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

1>	except for contention-free Random Access Preamble for beam failure recovery request and contention-free Random Access Preamble triggered by a PDCCH order for an LTM candidate cell, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX, and PREAMBLE_RECEIVED_TARGET_POWER.

1>	if the Random Access Procedure is triggered by a PDCCH order for an LTM candidate cell:

2>	consider this Random Access procedure completed.

1>	if LBT failure indication is received from lower layers for this Random Access Preamble transmission:

2>	if lbt-FailureRecoveryConfig is configured:

3>	perform the Random Access Resource selection procedure (see clause 5.1.2).

2>	else:

3>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

3>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

4>	if the Random Access Preamble is transmitted on the SpCell:

5>	indicate a Random Access problem to upper layers;

5>	if this Random Access procedure was triggered for SI request:

6>	consider the Random Access procedure unsuccessfully completed.

4>	else if the Random Access Preamble is transmitted on an SCell:

5>	consider the Random Access procedure unsuccessfully completed.

3>	if the Random Access procedure is not completed:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2).

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), for the RACH occasion which is configured by addlRACH-Config-Adapt f_id is the index of the PRACH occasion in the frequency domain plus the value of msg1-FDM in rach-ConfigGeneric included in associated RACH-ConfigCommon which is included the configuration of the RACH occasion, otherwise f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

  - TP2 of TS 38.331

–	RandomAccessAdaptConfig

The IE RandomAccessAdaptConfig is used to configure adaptive random access occasions.

RandomAccessAdaptConfig information element

-- ASN1START

-- TAG-RANDOMACCESSADAPTCONFIG-START

RandomAccessAdaptConfig-r19 ::=    SEQUENCE {

prach-ConfigurationIndex-r19            INTEGER (0..255),

msg1-FDM-r19                           ENUMERATED {one, two, four, eight},

msg1-FrequencyStart-r19                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

prach-SubsetMask-Index-Adaptation-r19   ENUMERATED {one, two, three, four},

ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                                 OPTIONAL,   -- Need M

validityDurationForAddlRACH-Adaptation-r19                       ENUMERATED {n2, n4, n8, n16},

valueKforAssociationPatternPeriodsForPRACH-subsetMask-r19        ENUMERATED {n2, n4, n8, n16}                     OPTIONAL,   -- Need S

...

}

-- TAG-RANDOMACCESSADAPTCONFIG-STOP

-- ASN1STOP

RandomAccessAdaptConfig field descriptions

msg1-FDM The number of PRACH transmission occasions FDMed in one time instance. (see TS 38.211 [16], clause 6.3.3.2). The network ensures the total value of msg1-FDM-r19 in addlRACH-Config-Adapt and msg1-FDM (without suffix) in rach-ConfigGeneric configured in the associated RACH-ConfigCommon is not more than 8.

validityDurationForAddlRACH-Adapt Indicates the validity time duration for availability indication of additional PRACH resources indicated by DCI 1_0 with P-RNTI, time unit is one SI modification period (see TS 38.213, subclause 8.1).

valueKforAssociationPatternPeriodsForPRACH The value of Kmask used for mapping of mask index to association periods per Kmask association pattern periods (See TS 38.213, subclause 8.1). Absence of this field indicates the value 1.

  - TP3 of TS 38.321

5.15.1 Downlink and Uplink <skip> Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall: 1> if initialUplinkBWP-RedCap is configured for the selected carrier: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap. 1> else: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP. 1> if initialDownlinkBWP-RedCap is configured: 2> if the Random Access procedure was initiated for OD-SIB1 request (as specified in TS 38.331 [5]) and the Random Access Resources for OD-SIB1 request have been explicitly provided by RRC: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP. 2> else if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP. 2> else: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. 1> else: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP.