3GPP TSG-RAN WG2 #131bis	R2-2506862

Prague, Czech Republic, Oct. 13th-17th

Agenda item:		8.4.1

Source:				CATT

Title:				  	List of open issues for LP-WUS 38.304 CR

Document for:	   Discussion and Decision

- Introduction

Per instruction from the chairlady, this document is to collect correction phase open issues identified for Rel-19 LPWUS 38.304 CR. More specifically, please provide here open issues that were identified but not addressed during the running CR review phase or we expect to come from other WGs.

Please provide your input no later than Friday September 26 18:00 UTC.  After the deadline, rapporteur will identify issues that are difficult to resolve and should be discussed by contributions, if any, as suggested by the chairlady.

- Open issues

  - LP-WUS in idle/inactive mode

| Company | Open issue | Rapporteur response |
| --- | --- | --- |
| Ericsson | In our view a moving UE in an urban environment cannot be allowed to not measure for up to 20 sec, i.e. this would impact cell reselection and paging performance. However a stationary UE can be allowed to not measure neighbour cells until it starts moving again. Thus an LP-WUS UE should be allowed to use LR measurements to evaluate “low mobility” and “not-at-cell-edge” criterion. | Since last meeting we agreed not support low mobility using LR, but we are open to discuss. Added as an open issue: 38304-1: Discuss whether “low mobility” criterion based on LR measurements needs to be introduced for LP-WUS. |
| Ericsson | It was already captured that the LP-WUS beam selection is left to UE implementation: In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation. Based on RAN2 discussion and agreement about the LP-WUS beam selection it is captured in 38.304: In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation. In our understanding the sentence above says that how the UE measures LR-RSRP/LR-RSRQ in multi-beam operations is up to UE implementation. We do not think that is the case, and if so, it should not only be captured for multi-beam operations. We think the sentence should be clarified or removed. [Chenli] I assume they are two different aspects: the first part is for LP-WUS monitoring, while the second part is for RRM measurement via LR. | Share the same view with vivo that they are two different aspects. But we are open to discuss how to capture the agreement agreed in RAN2#131 meeting. Added as an open issue: 38304-2: Discuss how to capture the agreement “LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations. We assume this conclusion does not impact the cell reselection procedure.” |
| Xiaomi | We have captured that in TS 38.304 CR draft : “ If the UE in RRC_IDLE and RRC_INACTIVE is unable to monitor the LP-WUS occasion in all MO corresponding to its PO, the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2. ” We would prefer to keep the same way as PEI: “If the UE is unable to monitor the PEI occasion (i.e. all valid PDCCH MO for PEI) corresponding to its PO, e.g. during cell re-selection, the UE monitors the associated PO according to clause 7.1.” Because here this invalid MO is caused by higher layer, so it should be captured in RAN2. Otherwise, people would confused what is the invalid MO here meaning. Hence, we should keep this in MAC not in PHY. [Chenli] Prefer not to capture it, as we have discussed it before. The collision here also includes the case the “MR and LR cannot operate simultaneously”, which is caused by PHY. | Actually, “e.g. during cell re-selection” is added in the initial version. But it was removed as some companies prefer not to add this considering there are other cases as well. In addition, according to the agreement in RAN2#131, cell re-selection was not captured as an example. Therefore, sharing the same view with vivo, we prefer to have a general description and keep current spec as it is. |
| Xiaomi | As agreed in TS 38.300 draft CR: “ In addition to monitoring a codepoint associated with its subgroup ID, a UE configured with LP-WUS monitoring also monitors a common codepoint associated with all subgroups in a PO.” Do we need to add a sentence to clarify UE’s behaviour when monitors the 2 type of code points as in LTE: “ A UE supporting GWUS can be configured to monitor a WUS group and a common WUS. Upon detecting either of them, UE shall monitor POs as defined in clause 7.4. ” | As the common codepoint has already been captured in RAN1 specification and the stage 2 specification, we slightly prefer not to capture it in TS 38.304. |
| Ericsson | PEI is transmitted on all transmitted beams indicated in ssb-PositionsInBurst. But LP-WUS may be transmitted on a subset of the transmitted beams as indicated in lpwus-LPSS-BeamSubset-r19. Therefore it is suggested to add: In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams for which LP-WUS is configured and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation. | Ok. Will be updated in the 304 CR. |
| Ericsson | Perhaps good to add a clarification that PEI can be disabled in the UE, for example: The UE supporting LP-WUS and LP-WUS has not been disabled in the UE, may monitor LP-WUS in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If LP-WUS configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting LP-WUS (except for the UEs expecting MBS group notification) may start LP-WUS monitoring using LP-WUS parameters in system information according to the procedure described below if the entry condition in clause 7.x.1 is fulfilled. The UE may stop LP-WUS monitoring and follows the paging monitoring procedure as described in clause 7.1 or 7.2 if the exit condition in clause 7.x.1 is fulfilled. | We think it’s better to wait the progress on SA2, CT1 and RAN3 for LP-WUS enable/disable issue. We can capture something if needed after they finish the work on NAS based per-UE enable/disable for LP-WUS. |
| Ericsson | RAN3 agreed: [Last Used Cell] The anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID. Rev in R3-255944 Agreed unseen Rev in R3-255942 Agreed unseen Rev in R3-255943 Agreed unseen Therefore we should also capture in 38.304, similar as with PEI: If lastUsedCellOnly is configured in system information of a cell, the UE monitors LP-WUS in this cell only if the UE most recently received RRCRelease without noLastCellUpdate in this cell. Otherwise (i.e., if lastUsedCellOnly is not configured in system information of a cell), the UE monitors LP-WUS in the camped cell. [Chenli] The RAN3 agreement which said anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message could be applied to the scenario that LP-WUS is not only used in last used cell. It means that the LP-WUS can be used in any cell, i.e., don’t introduce lastUsedCellOnly for LP-WUS, also needs the agreement in RAN3. The background for RAN3 discussion on this issue is, they think this part is related to the lastUsedCellOnly, but all RAN3 agreed that lastUsedCellOnly should be determined in RAN2. Thus, the above RAN3 conclusion doesn't mean the “last used cell” was agreed in RAN3, and RAN3 has closed this issue. | Share the same view with vivo. RAN3 agreement is not related with“last used cell”. So we prefer to keep current spec as it is. |

  - RRM relaxation/offloading

| Company | Open issue | Rapporteur response |
| --- | --- | --- |
| Xiaomi | - An open issue is proposed by us and was left as an open issue in the Rapp’s summary as 38304-12 (R2-250523 Discussion of [Post130][211][LPWUS] Running CR for 38.304 (CATT)): The impact on legacy low-mobility criteria for MR need to be considered. An example is: For case1, when UE exits using fully offload, the UE turn on the MR. UE can start the re-evaluation for low mobility criteria for legacy relaxed measurement criterion if configured. For such case, we need to add such case for setting the reference Srxlev value as shown below. 5.2.4.9.1 Relaxed measurement criterion for UE with low mobility The relaxed measurement criterion for UE with low mobility is fulfilled when: - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows: - After selecting or reselecting a new cell or UE turns on MR when exit of case1, or - If (Srxlev - SrxlevRef) > 0, or - If the relaxed measurement criterion has not been met for TSearchDeltaP: - The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell. After discussing with companies F2F in India, most companies think it is reasonable and suggested to postpone it and give a TP for the maintenance phase. | Added as an open issue: 38304-3: FFS the impact on legacy low-mobility criteria for MR when the UE exits serving cell measurement offloading case. |

- Conclusion

According to clause 2, the following open issues are identified that can be addressed in company contributions:

Proposal 1: 38304-1: Discuss whether “low mobility” criterion based on LR measurements needs to be introduced for LP-WUS.

Proposal 2: 38304-2: Discuss how to capture the agreement “LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations. We assume this conclusion does not impact the cell reselection procedure.”

Proposal 3: 38304-3: FFS the impact on legacy low-mobility criteria for MR when the UE exits serving cell measurement offloading case.