3GPP TSG-RAN WG2 Meeting #131bis	R2-2506873

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.9.4

Source: 	CATT

Title: 	Correction on the any cell selection state for NB-IoT

Document for: 	Discussion and Decision

1	Introduction

In Rel-19, the applicable cell any cell selection state is supported for NB-IoT, this contribution gives correction related to the any cell selection state for NB-IoT.

2	Discussion

According to the CR of TS 36.304 [1], the any cell selection state for NB-IoT is enhanced as follows.

### 5.2.8 Any Cell Selection state For NB-IoT Any Cell Selection state is defined in clause 5.2.8a. This state is applicable for RRC_IDLE and RRC_INACTIVE state. In this state, the UE shall perform cell selection process to find a suitable cell. If the cell selection process fails to find a suitable cell after a complete scan of all RATs and all frequency bands supported by the UE, the UE shall attempt to find an acceptable cell of any PLMN to camp on, trying all RATs that are supported by the UE and searching first for a high quality cell, as defined in clause 5.1.2.2. The UE, which is not camped on any cell, shall stay in this state. ### 5.2.8a Any Cell Selection state for NB-IoT In this state, the UE shall attempt to find a suitable cell of any PLMN to camp on and searching first for a high quality cell, as defined in clause 5.1.2.2. If the cell selection process fails to find a suitable cell after a complete scan of all RATs and all frequency bands supported by the UE and the UE is capable of PWS reception, the UE shall attempt to find an acceptable cell of any PLMN to camp on in current RAT and searching first for a high quality cell, as defined in clause 5.1.2.2. The UE not capable of PWS reception, which is not camped on any cell, shall stay in this state until a suitable cell is found.

The second paragraph specified the UE behaviour when it is not camped on any cell. However, the behaviour of UE capable of PWS reception cannot be covered by the text, which should be added. We commented this issue during the post email discussion stage [1]. But the rapporteur replied that the modification is already reflected in the change in second paragraph (guess the rapporteur wanted to say the first paragraph, but wrote the second paragraph by mistake?), that this change is not essential.

Suggest making the following modifications: The UE, which is not camped on any cell, shall stay in this state until a suitable cell or an acceptable cell (only applicable to PWS capable UE) is found. Rapp : The modified part is already reflected in the change in second paragraph. Hence this change is not essential

Considering the first paragraph is to specify the searching behaviour when the UE is in any cell selection state, while the second paragraph is to specify under what condition the UE shall stay in the any cell selection state. These two paragraphs describe different aspects of the any cell selection state. From our perspective, it is necessary to include the behaviour of PWS reception capable UE in the second paragraph. It is proposed that:

Proposal 1: Specify that the PWS reception capable UE, which is not camped on any cell, shall stay in this state until a suitable cell or an acceptable cell is found. Adopt the TP with CATT change mark in Annex

3	Conclusion

In this contribution, correction related to the any cell selection state for NB-IoT is discussed with the following proposal:

Proposal 1: Specify that the PWS reception capable UE, which is not camped on any cell, shall stay in this state until a suitable cell or an acceptable cell is found. Adopt the TP with CATT change mark in Annex

4	Reference

- R2-2506556, R19 IoT NTN 36.304 CR.

5	Annex

### 5.2.8a	Any Cell Selection state for NB-IoT

In this state, the UE shall attempt to find a suitable cell of any PLMN to camp on and searching first for a high quality cell, as defined in clause 5.1.2.2. If the cell selection process fails to find a suitable cell after a complete scan of all frequency bands supported by the UE, and it is capable of PWS reception the UE shall attempt to find an acceptable cell of any PLMN to camp and searching first for a high quality cell, as defined in clause 5.1.2.2.

The UE not capable of PWS reception, which is not camped on any cell, shall stay in this state until a suitable cell is found. The UE capable of PWS reception, which is not camped on any cell, shall stay in this state until a suitable cell or an acceptable cell is found.