3GPP TSG RAN WG2 #131bis			R2-2506889

Prague, Czech Republic, Oct 13th – 17th, 2025

Source: 	Fainity Innovation

Title:	Views on 6G Mobility

Agenda item:	10.4

Document for:	Discussion & Decision

Introduction

Study on 6G Radio (6GR) with enumerated objectives in RP-251881 [1] was approved in RAN Plenary #108. As planned by RAN2 chair for the 1st SID meeting, companies were invited to provide general principles for 6G mobility, regarding mobility types, measurements, and intra-/inter-RAT mobility. Accordingly, we prepare this document to share our views on these aspects. The agenda of this document is arranged that we will describe the preferred principles for 6G mobility section 2. In the subsequent sections, we provide our observations and suggested directions for the principles mentioned in section 2.

Design principles for 6G Mobility

From our perspective, four high-level principles should be always kept in mind during the study and normative stages of 6GR:

- Simple mobility framework: 6GR aims for a simple, less configurable air interface, and mobility is no exception. Too many mobility schemes and complicated configurations results in increased UE complexity and less interest on the implementations should be avoided.

- Balance between energy efficiency and mobility performance: To achieve a green and sustainable communication system, one of the key concepts is to improve energy efficiency in all aspects. How to strike a balance between the power consumption at both UE and NW sides while guaranteeing the mobility performance is the point where we can make effort.

- Smooth mobility between TN and NTN: NTN is a promising technique to ensure ubiquitous connectivity and thus can be considered a complement to the TN coverage. In such scenarios, the mobility between TN and NTN plays an important role to ensure the service continuity.

- Integration of AI/ML: As studied in Release 19, AI/ML shows the possibility of improvements in two directions: 1) reduction of reference signals and 2) advanced preparation by time domain prediction. This can be further studied to achieve better mobility performance like interruption time reduction, HO failure rate reduction, and signalling overhead reduction.

Proposal #1: The design principles for 6G mobility are to prioritize a simple mobility framework, a balance between energy efficiency and mobility performance, smooth mobility between TN and NTN, and the integration of AI/ML.

Discussions

Simple Mobility Framework

Pain points: In 5G, there are diverse options for mobility, including the basic handover, CHO, DAPS, LTM, and C-LTM, each of which has its benefits and drawbacks. However, too many options for mobility lead to massive measurement configurations (like measurement object for L3 cell level measurement, LTM CSI resource set for L1 beam level measurement), reporting configurations (like report configuration and measurement identity for handover and CHO, and LTM CSI report configuration for LTM and C-LTM), and capability negotiation overhead (like the support of enhanced mobility options such as LTM and C-LTM). The increased UE complexity and configuration overhead can dispel the willingness of implementation. As a result, we think that a simple mobility framework is the first goal for the 6G mobility.

Make-before-break and break-before-make: From the evolution of cellular networks, the break-before-make handover solution (i.e., the UE stops the user data transmission to the source cell before the UE starts the user data transmission to the target cell, as shown in Figure 9.2.3.2.1-1 in TS 38.300 [2]) is prevalently adopted due to its simplicity for implementation. On the contrary, the make-before-break solution (i.e., the UE starts the user data transmission to the target cell before the UE stops the user data transmission to the source cell) provides an ideally seamless handover without interruption at the cost of high complexity for Layer 1 to Layer 3 implementation and coordination between cells, such as doubled physical and MAC layer implementations and handling of duplicated packets. For the current market, there seems to be no prominent need for the seamless feature of the make-before-break solution. As a result, we think break-before-make solutions should be prioritized as a 6G day 1 study item. In contrast, the make-before-break solutions can be postponed until the necessity is identified in any future study.

Convergence of mobility solutions: To achieve a unified mobility solution, we think the comparison between different mobility options is needed. For basic handover evolved from 4G, since the target cell’s RRC configuration is executed upon the UE’s reception, it is more flexible for the target cell to determine the configuration without reserving resources. However, the interruption time for RACH procedure and the possibility of failure due to decoding failure of HO command (the RRC configuration of the target cell) are the main problems. For CHO discussed in Release 16, the failure probability is reduced by early transmission of the target cell’s configuration; in addition, the failure recovery is supported, which allows the UE to possibly fast recover the connection. Still, the interruption time for RACH procedure remains an unsolved problem. For LTM designed in Release 18 (and inter-CU LTM in Release 19), the interruption time is prominently reduced by early RACH and early DL/UL beam activation, and the fast recovery is also supported in case of failure. With the enormous shrink in interruption time, the cost of measurement, report, and other messages related to the early UL/DL procedure becomes an issue. It is possible that the UE measures and reports lots of RSs, performs lots of early preamble transmissions, and just for a single LTM cell switch. As for C-LTM in Release 19, it further enhances the LTM by allowing the UE to autonomously execute the cell switch, although the early UL/DL is still controlled by the network. Similar to LTM, C-LTM presents the same issues as the measurement, report, and control message overhead.

According to the comparison, LTM achieves better interruption performance and allows subsequent LTM cell switch, which is a desirable feature for 6G mobility; as for the burden of measurement, report, and control messages for early UL/DL, we believe that an intelligent network can optimize to reduce them as much as possible by its implementation, so we propose to take LTM as the baseline for the 6G mobility.

Leaner configuration for measurement and report: Besides the multiple mobility procedure options, the configuration for measurement and reporting also needs simplification. In RRM framework, both L3 cell level measurement and L1 beam level measurement can be reported. On the other hand, in CSI framework, L1 beam level measurement and CSI acquisition can be reported. This introduces a complicated reporting framework for the mobility procedure, especially for the LTM and C-LTM. Take C-LTM for example, the condition is configured as a measurement identity, which is associated with a L3 report configuration; the reporting of L1 beam level measurement is configured as a L1 CSI report configuration; while the UE reports the L1 measurement results via UCI for periodic and semi-persistent report and via MAC CE for event-triggered report. We think this is too complicated, especially the cross-layer involvement from L1 to L3 in just a single mobility procedure. Consequently, a simplified framework for measurement and report configuration is desirable.

Observation #1: There is no prominent need for the seamless feature of the make-before-break mobility solutions.

Observation #2: LTM has desirable features for 6G mobility like shorter interruption and subsequent cell switch, and the measurement, report, and control message overhead can be intelligently handled by the network implementation.

Proposal #2: For 6G mobility framework, R2 is suggested to study LTM as the baseline for enhancements and improvements.

Balance between Energy Efficiency and Mobility Performance

Paint points: To make sure that the mobility decision is made based on the latest link information, it is intuitive for the UE to measure and report different reference signals from different cells with possibly short periodicity. With the evolution of 5G mobility schemes, the measurement and RS overhead increase from SSB only (e.g., basic handover and CHO) to SSB with CSI-RS (LTM and C-LTM). The effort of transmitting RS and control messages at the NW side, and the measuring and reporting procedure at the UE side contributes a proper factor to power consumption. Therefore, we suggest to consider the energy efficiency while targeting the mobility performance.

Reduction of SSB: From Release 18, NES has been researched for the reduction of power consumption at the network side, and the reduction of SSB is proposed as one of the solutions. SSB periodicity adaptation, on-demand SSB, and SSB-less SCell are the main ideas for SSB reduction, and it will achieve better power efficiency if we apply the reduction of SSB to more relaxed scenarios like in PCells. However, from the perspective of mobility, the reduction on SSB in PCells may cause the inaccuracy of RRM measurement and may further lead to inaccurate handover or cell switch decisions, which could severely degrade the mobility performance. Additionally, for IDLE UEs, SSB serves as an entrance to access a cell, so SSB reduction on PCell also poses risk on the performance of initial access. Thus, we propose to study how to maximally reduce SSB without harming the mobility performance.

Control signalling: With the design of LTM and C-LTM in Release 18 and 19, the control signalling increases as there is more frequent L1 measurement report, MAC CE for early beam activation, and MAC CE for early TA acquisition. On one hand, these control signallings can be minimized by an intelligent network with accurate predictions; on the other hand, it is beneficial to consider the coexistence of the important control signallings with the NES UP solutions such as cell DTRX.Observation #3: There is a tradeoff between energy efficiency and mobility performance for SSB reduction.

Observation #4: With the performance upgrade brought by LTM and C-LTM, additional control signallings are present and need carefully handled.

Proposal #3: For the balance between energy efficiency and mobility performance, R2 is suggested to study the reduction on SSB in non-SCell scenarios and the reduction on control signallings.

Smooth Mobility between TN and NTN

Pain points: To aim at ubiquitous connectivity, NTN is a promising technology for complementing the coverage hole of TN. Because of the satellite movement and long propagation delay, NTN environments face unique challenges such as high frequency and time offsets. However, since NTN is not a 5G day 1 feature, the study and the standardization of 5G NTN has commenced in Release 16 and 17 respectively, resulting in that the solutions are forced to built on top of the existing Release 15 NR design. In addition, 5G NTN lacks dynamic resource allocation, beam management, and interference mitigation with TN, causing poor mobility performance between TN and NTN. As 5G NTN is going to be commercialized in recent years and introducing a new 6G NTN solution may cause market fragmentation, it is better to design a compromised solution that harmonizes 5G NTN and 6G TN, without preventing the evolution of 6G NTN in later releases.

Inter-RAT mobility: As previously mentioned, the first scenario for TN-NTN mobility in 6G is the handover between 5G NTN and 6G TN. As a result, an inter-RAT mobility solution is the first issue to be solved. Fortunately, there is existing example for design reference; e.g., the LTE-NR mobility can be taken as a baseline for 5G NTN and 6G TN mobility. In our view, the inter-RAT mobility for LTE-NR should be further upgraded to accommodate the NTN characteristics like long RTT and satellite movement. Therefore, we suggest that the inter-RAT mobility is studied with the consideration of integration of TN-NTN mobility.

LTM as a baseline for TN-NTN mobility: To enable a smooth mobility, the interruption time is expected to be as short as possible, and LTM is a good choice as a baseline since the early beam activation and early RA procedure allows these time-consuming procedures to be performed in advance. The issue would then be the adaptation of LTM for a TN-NTN mobility. Thus, we propose to take LTM as baseline and study the enhancements to fit the LTM concept into a smooth TN-NTN mobility.

Observation #5: The scenario of TN-NTN mobility is expected to prioritize the mobility between 5G NTN and 6G TN.

Observation #6: The early UL/DL synchronization procedure in LTM enables a smooth mobility between TN and NTN.

Proposal #4: R2 is suggested to consider the TN-NTN mobility while designing inter-RAT mobility for 6G, and LTM can be considered to be a baseline.

Integration of AI/ML

AI/ML for mobility performance enhancement: In Release 19, RAN2 has studied the feasibility of AI/ML on mobility and found that AI/ML helps improve the accuracy of link quality estimation by measurement result prediction. This implies the possibility of using LTM as a baseline for simplified mobility framework. With more accurate link quality estimation, the NW can make better decisions on early beam activation, early RA initiation, and cell switch at a more appropriate timing. The better decisions prevents excess control signalling; for example, the NW can avoid early beam activation and early RA towards other cells, or avoid ping-pong between early beam activation and deactivation for a candidate cell.

AI/ML for power saving in mobility: Another benefit observed in RAN2 Release 19 AI/ML Mobility study is the potential to reduce the effort for transmission (from the NW’s perspective) and measurement and report (from the UE’s perspective) on reference signals. As we have mentioned in Section 3.2, SSB reduction can be further exploited with the extension to PCell scenarios. The aid of AI/ML will be a simple and reliable approach to realize the goal. For example, the concern on estimation inaccuracy stemmed from SSB reduction can be compensated with the aid of AI/ML.

Integration of AI and mobility: According to the above two points, AI/ML is a promising support to pave the way for a simplified and power efficient mobility framework while simultaneously keeping or enhancing the mobility performance. Considering these benefits, we suggest RAN2 shall try to complete the mobility framework at early stage of study and then the use of AI/ML for mobility could be studied afterwards.

Observation #7: In Release 19, RAN2 has identified the benefit of AI/ML in two aspects, where the enhanced accuracy on link quality estimation can shade the drawback of LTM for simplified mobility, and the reduced effort for NW transmitting and UE measuring SSB can exploit the power saving gain.

Proposal #5: R2 is suggested to study the integration of AI/ML to mobility, including the enhanced accuracy and RS reduction. This can be started after the study of mobility framework is completed and an early checkpoint for the mobility framework is suggested..

Conclusions

In this contribution, we share our views and observations on 6G and propose:

Observation #1: There is no prominent need for the seamless feature of the make-before-break mobility solutions.

Observation #2: LTM has desirable features for 6G mobility like shorter interruption and subsequent cell switch, and the measurement, report, and control message overhead can be intelligently handled by the network implementation.

Observation #3: There is a tradeoff between energy efficiency and mobility performance for SSB reduction in non-SCell scenarios.

Observation #4: With the performance upgrade brought by LTM and C-LTM, additional control signallings are present and need carefully handled.

Observation #5: The first scenario of TN-NTN mobility is expected to be the mobility between 5G NTN and 6G TN.

Observation #6: The early UL/DL synchronization procedure in LTM enables a smooth mobility between TN and NTN.

Observation #7: In Release 19, RAN2 has identified the benefit of AI/ML in two aspects, where the enhanced accuracy on link quality estimation can shade the drawback of LTM for simplified mobility, and the reduced effort for NW transmitting and UE measuring SSB can exploit the power saving gain.

Proposal #1: The design principles for 6G mobility are to prioritize a simple mobility framework, a balance between energy efficiency and mobility performance, smooth mobility between TN and NTN, and the integration of AI/ML.

Proposal #2: For 6G mobility framework, R2 is suggested to study LTM as the baseline for enhancements and improvements.

Proposal #3: For the balance between energy efficiency and mobility performance, R2 is suggested to study the reduction on SSB in non-SCell scenarios and the reduction on control signallings.

Proposal #4: R2 is suggested to consider the TN-NTN mobility while designing inter-RAT mobility for 6G, and LTM can be considered to be a baseline.

Proposal #5: R2 is suggested to study the integration of AI/ML to mobility, including the enhanced accuracy and RS reduction. This can be started after the mobility framework is stable.

References

- RP- 251881, New SID: Study on 6G Radio

- TS 38.300. “NR and NG-RAN Overall Description,” Version 18.6.0