3GPP TSG-RAN WG2 Meeting #131bis	R2-2506891

# Prague, CZ, 13 – 17 Oct 2025

Agenda item:		10.3.2

Source:	T-Mobile USA

Title:	6G Service Aware RAN RAN2 consideration

Document for:		Discussion and Decision

# Introduction

In RAN#108 meeting, following Radio Interface protocol architecture and procedures for 6GR objective was introduced [1]:

One of the detailed objectives of the study are:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

This contribution highlights the importance of Service Aware RAN (SAR) and proposes that RAN2 investigate protocol enhancements to enable SAR capabilities in 6G.

# Discussion

## Background

In today’s 5G system, 5QI defines the service quality profile set by the core network. Each 5QI corresponds to thresholds (priority, latency, error rate) and is conveyed to the RAN. The RAN, along with additional parameters, uses the 5QI to derive MAC scheduling priorities.

While this mechanism is sufficient for less stringent services (e.g., web browsing, non-real-time video streaming), it falls short for real-time, high-performance applications such as XR, which require dynamic and tight coupling of throughput, latency, jitter etc. Without over-dimensioning, the RAN struggles to guarantee these requirements, leading to tradeoffs between service innovation and overall spectrum efficiency.

3GPP has introduced RvQoE (RAN-Visible Quality of Experience) to enable passing of application attributes (e.g., buffer level) to the AS via RRC containers. However, commercial deployment faces challenges:

- Device ecosystems often restrict OS-to-AS application information sharing.

- Operators lack visibility and control over the wide variety of applications traversing the network.

- RRC-container–based signaling introduces additional overhead, which is non-trivial in heavily loaded networks.

Thus, a more fundamental approach is required — one where the RAN natively adapts to applications at the protocol level. This is the intent behind Service Aware RAN for 6G.

We believe Service Aware RAN is a RAN system that can identify and differentiate service types, application-level intents, and QoS requirements, adapting its behavior dynamically in real-time to optimize user experience and network resource utilization.

## Observations

Observation 1: The current 5G RAN lacks visibility in real-time application-level performance attributes, limiting effective resource allocation that balances application requirements and spectrum efficiency.

Observation 2: The 5G QoS framework is too coarse to allow fine-grained, dynamic RAN resource allocation based on real-time application performance.

Observation 3: The 5G RAN has no direct awareness of application-level requirements or characteristics, reducing its ability to support new service types and resilience for critical applications.

Observation 4: While eMBB remains vital, emerging services such as XR and AI traffic present fundamentally different profiles and must be treated as primary service classes in 6G specification work.

## Proposal

Proposal 1: RAN2 should study enhancements to the Radio Interface architecture and protocols with service awareness as a design principle, including novel scheduling methodologies.

# Conclusion

This contribution emphasizes the need for Service Aware RAN in 6G. Building on the identified limitations of 5G, we observe:

Observation 1: Lack of application-level performance visibility in RAN.

Observation 2: Coarse granularity of the 5G QoS framework.

Observation 3: Limited ability of RAN to optimize resources for new service types.

Observation 4: Necessity of treating XR, AI, and other emerging services as primary in 6G.

Accordingly:

Proposal 1: RAN2 should study enhancements to the Radio Interface architecture and protocols with service awareness as a design principle, including novel scheduling methodologies.

# References

[1] RP-251881, NTT Docomo (Moderator), "Study on 6G Radio"

[2] RP-251986, T-Mobile USA, AT&T, Nokia, Ericsson, Telstra, Verizon, “Requirements for Service Aware 6G RAN”

[3] R3-256716, T-Mobile USA, “6G Service Aware RAN Architecture Option”