3GPP TSG-RAN WG2 Meeting #131bis										R2-2506894

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.3.1

Source: 	Sharp

Title:  	Initial Considerations for 6GR User Plane

Document for: 	Discussion

# Introduction

This document discusses 6GR user plane aspects that should be considered at the early stage of the study.

# Discussion

## User Plane Protocol Stack

| MAC | - Mapping between logical channels and transport channels; - Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels; - Scheduling information reporting; - Error correction through HARQ (one HARQ entity per cell in case of CA); - Priority handling between UEs by means of dynamic scheduling; - Priority handling between logical channels of one UE by means of logical channel prioritisation; - Priority handling between overlapping resources of one UE; - Padding. |
| --- | --- |
| RLC | - Transfer of upper layer PDUs; - Sequence numbering independent of the one in PDCP (UM and AM); - Error Correction through ARQ (AM only); - Segmentation (AM and UM) and re-segmentation (AM only) of RLC SDUs; - Reassembly of SDU (AM and UM); - Duplicate Detection (AM only); - RLC SDU discard (AM and UM); - RLC re-establishment; - Protocol error detection (AM only). |
| PDCP | - Transfer of data (user plane or control plane); - Maintenance of PDCP SNs; - Header compression and decompression using the ROHC protocol; - Header compression and decompression using EHC protocol; - Compression and decompression of uplink PDCP SDUs: DEFLATE based UDC only; - Ciphering and deciphering; - Integrity protection and integrity verification; - Timer based SDU discard; - For split bearers, routing; - Duplication; - Reordering and in-order delivery; - Out-of-order delivery; - Duplicate discarding. |
| SDAP | - Mapping between a QoS flow and a data radio bearer; - Marking QoS flow ID (QFI) in both DL and UL packets |

Table 1. Functions supported by NR User Plane protocols [1].

User Plane (UP) protocols are related to data processing. For serving high data rate expected for 6G communications, 6G UP protocols should be lean and efficient. Table 1 shows the list of functions supported by NR UP sublayers. We see that NR has duplicated functions, especially in RLC with other sublayers. In Table 1, duplicate or similar functions are highlighted with colour codes. For example, both PDCP and RLC have sequence numbering. ARQ can be replaced by HARQ for a certain degree. Thus, we can consider lean or no RLC for the efficiency.

Observation 1: NR UP protocol stack has duplicated functions, especially in RLC.

Proposal 1: RAN2 aims to have an efficient UP architecture, including 1) remove duplicated functions among PDCP/RLC/MAC (e.g., Sequence numbering, ARQ) and 2) moving some RLC functionalities to PDCP or MAC (e.g., lean or no RLC).

SDAP is a sublayer to support QoS flow to DRB mapping. The main reason to introduce SDAP sublayer is a case that so many QoS flows are being served by a UE and 1-to-1 mapping between QoS flow and DRB may require a significantly large number of DRBs. However, most of UEs serve a limited number of QoS flows simultaneously. QoS flow to DRB mapping seems to be redundant. Even without SDAP sublayer, the increased number of DRBs supported by the UE will not be changed drastically. Considering one more independent layer, UE process does not have a big gain.

Observation 2: Most of UEs serve a limited number of QoS flows simultaneously. QoS flow to DRB mapping seems to be redundant.

Proposal 2: 6GR does not have SDAP sublayer.

## Radio Bearer/Logical Channel Concept

Figure 1. DL Layer 2 structure in NR [1]

Figure 1 shows Layer 2 structure in NR downlink. The concept of Radio Bearer/Logical Channel is supported in NR and LTE which realizes the management of UP data with different characteristics and priorities. Considering that various service types will be supported by 6GR, radio bearer and logical channel could still be helpful to fulfil the specific requirements like latency, error recovery, etc. for different services. Although RLC entity is associated with a radio bearer/logical in NR, we don’t expect that it implies an additional requirement for RLC if we reuse this concept in 6GR.

Observation 3: Considering the service types will be supported by 6GR, radio bearer and logical channel could still be helpful to fulfil the specific requirements like latency, error recovery, etc.

Proposal 3: 6GR reuses concepts of radio bearer and logical channel as in NR.

## Unified Compressions

NR standards employ multiple compression mechanisms across various releases i.e., ROHC, UDC, EHC. ROHC is a Rel-15 basic feature mainly targeted to voice packets. UDC is a specific UL compression mechanism transplanted from LTE UDC. EHC is for Ethernet-based applications like industrial IoT. All those mechanisms were standardized independently, but those are all compression mechanisms which have commonalities. For the sake of 6GR simplification, it is advisable to adopt a unified compression mechanism framework, to avoid independent features with overlapping goals.

Observation 4: In NR, multiple compression mechanisms, i.e., ROHC, UDC, EHC. ROHC were introduced independently, but those are all compression mechanisms which have commonalities.

Proposal 4: RAN2 to study a unified compression framework including ROHC, UDC, and EHC.

## Unified Buffer Status Reporting

Notifying buffer status information of the UE to the network enables the network to provide moderate resources to the UE, which can avoid wasting of resource or delaying the data transmission. In NR, both BSR and DSR are used to indicate the buffer status of the UE. BSR MAC CE is used to report the total data volume buffered in UE whereas DSR MAC CE is used to report the delay status of the buffered data. There are some duplications between the two mechanisms, which result in signalling overhead or even inefficiency. This should be avoided when defining signalling mechanisms of such information delivery in 6G. 6GR should study a unified buffer status reporting for 6G which considers both total buffered data volume and delay status of the buffered data.

Observation 5: BSR and DSR have duplication which results in signalling overhead or even inefficiency.

Proposal 5: RAN2 to study a unified buffer status reporting including total buffered data volume and delay status of the buffered data.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals:

Observation 1: NR UP protocol stack has duplicated functions, especially in RLC.

Proposal 1: RAN2 aims to have an efficient UP architecture, including 1) remove duplicated functions among PDCP/RLC/MAC (e.g., Sequence numbering, ARQ) and 2) moving some RLC functionalities to PDCP or MAC (e.g., lean or no RLC).

Observation 2: Most of UEs serve a limited number of QoS flows simultaneously. QoS flow to DRB mapping seems to be redundant.

Proposal 2: 6GR does not have SDAP sublayer.

Observation 3: Considering the service types will be supported by 6GR, radio bearer and logical channel could still be helpful to fulfil the specific requirements like latency, error recovery, etc.

Proposal 3: 6GR reuses concepts of radio bearer and logical channel as in NR.

Observation 4: In NR, multiple compression mechanisms, i.e., ROHC, UDC, EHC. ROHC were introduced independently, but those are all compression mechanisms which have commonalities.

Proposal 4: RAN2 to study a unified compression framework including ROHC, UDC, and EHC.

Observation 5: BSR and DSR have duplication which results in signalling overhead or even inefficiency.

Proposal 5: RAN2 to study a unified buffer status reporting including total buffered data volume and delay status of the buffered data.

# References

[1] TS 38.300	NR and NG-RAN Overall Description, Stage-2