3GPP TSG-RAN WG2 Meeting #131bis										R2-2506898

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.4

Source: 	Sharp

Title:  	Initial Considerations for 6GR Mobility

Document for: 	Discussion

# Introduction

This document discusses 6GR mobility aspects that should be considered at the early stage of the study.

# Discussion

## Mobility Mechanisms

NR introduced various mobility mechanisms across multiple releases on top of the traditional L3 handover. Basically, the L3 handover relies on RRC signalling. This classic mechanism is useful in traditional macro-cell deployment with moderate UE mobility and allowed interruption time. CHO supports quick link switching based on preconfigured condition, and it is useful in high mobility scenario. LTM reduces multi-layer processing and allows prompt mobility-related actions. C-LTM further enhances the robustness for prompt mobility scenarios. Also, RACH-less operation reduces the handover interruption time significantly. We think all those options are useful and the network should have the flexibility to select the implemented mobility mechanisms.

The network should therefore have the flexibility to configure and utilize the most suitable mobility mechanisms depending on deployment scenario, UE capability, and service requirements. Intra-cell mobility, e.g. beam-level mobility, is important for higher-frequency (e.g., mmWave/THz) scenario with beamforming. It relies on L1/L2 procedures for fast beam tracking. If feasible, it is preferred to consider intra-cell and inter-cell mobility mechanisms together in a unified framework of 6G mobility. “Unified” refers to defining a single, consistent mobility framework that aligns procedures and reporting formats across multiple mechanisms (e.g., L3 handover, CHO, LTM, C-LTM, RACH-less, beam-level mobility) under common principles. This ensures coherent configuration, measurement reporting, and UE behaviour without fragmented, mechanism-specific signalling.

DAPS requires two uplink transmission links at the UE side. It is considered as a UE complexity. Also, DAPS operation itself is complicated. Moreover, the similarity with other mobility mechanisms is low, so it is difficult to unify the mobility mechanisms including DAPS. Considering the complexity, we do not see a big motivation of DAPS as a 6G feature.

Proposal 1: 6GR support unified mobility framework including L3 Handover, CHO, LTM, C-LTM, RACH-less operation and beam-level mobility. 6GR does not support DAPS.

NR supports L3 measurement reporting and L1 measurement reporting. Generally, L3 measurement result is included in RRC message and reported to the gNB to decide to trigger L3 Handover, configure CHO, configure LTM and C-LTM. L1 measurement result is included in UCI or MAC CE and reported to the gNB to decide to trigger LTM. In 6GR, to support unified mobility as mentioned above, these measurement reporting mechanisms should be unified.

Proposal 2: 6GR support unified measurement reporting mechanism.

## Intra-RAT vs Inter-RAT

Lossless handover is a key aspect of intra-RAT mobility. Lossless handover avoids performance degradation due to packet loss during MAC reset procedure. This concept should be kept in 6G.

Inter-RAT mobility typically results in lossy handover where ongoing data may be lost during the handover. Considering an initial deployment of 6G, full coverage may not be always available, so frequent 5G-6G handovers are likely to be triggered often. In this scenario, lossy inter-RAT mobility may degrade the user service quality. Thus, in 6G, lossless inter-RAT mobility needs to be discussed and studied.

Proposal 3: For both Intra-RAT and Inter-RAT mobility from/to 6G, lossless handover is a baseline.

In MR-DC, a UE may have multiple physical links with both LTE and NR simultaneously. As captured in the SID, DC is not prioritized at this moment. Instead of DC, UE can be connected to only one RAT at a time point. The switching can be done by 6G inter-RAT mobility mechanism.

Proposal 4: 5G-6G mobility is supported by 6G inter-RAT mobility mechanism.

## IDLE/INACTIVE mode mobility

NR cell (re)selection can serve a baseline for 6GR IDLE/INACTIVE mode mobility. Furthermore, as the 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR [1], 6GR IDLE/INACTIVE mobility should further support the inter-RAT cell reselection (i.e., 6GR and NR). Whether inter-RAT cell reselection between 6GR and E-UTRA is supported can depend on RAN progress. In addition, considering the advance of native AI as a fundamental characteristic in 6G, supporting AI/ML in IDLE/INACTIVE mode mobility such as cell (re)selection is worthy of investigation.

Proposal 5: 6GR IDLE/INACTIVE mode mobility considers inter-RAT cell reselection between 6GR and NR.

AI/ML can be used by the UEs in IDLE/INACTIVE mode. For instance, in cell reselection, AI/ML can facilitate measurement result prediction, the selection of the best or suitable cell, frequency/cell priority handling etc. Note that we do not propose any particular AI/ML use cases. However, it is important to consider that UEs in idle/inactive mode should be able to know whether the network supports AI/ML features. We see that it will enhance the 6GR network efficiency.

Proposal 6: RAN2 to study cell reselection mechanism considering AI/ML support of the network/cell.

## RLM/RLF

In both 4G and 5G, RLM/RLF is one of the key procedures. According to RLM procedure, UE can know if the RRC connection is still available and if it is still within the serving cell’s coverage due to its moving. Then, based on the RLM/RLF declaration, the UE can make the decision to recover the connection via serving cell or some potential target cell. By this way, UE encountering RLF will not only autonomously maintain the connection via network during the mobility but also avoid causing interference to other normal UEs. For the same reasons, in 6GR, RLM/RLF is needed. We think RLM/RLF in 5G is in good shape and it can be a baseline for further 6GR RLM/RLF discussion.

Proposal 7: RLM/RLF framework in 5G can be a baseline for 6GR RLM/RLF discussion.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: 6GR support unified mobility framework including L3 Handover, CHO, LTM, C-LTM, RACH-less operation and beam-level mobility. 6GR does not support DAPS.

Proposal 2: 6GR support unified measurement reporting mechanism.

Proposal 3: For both Intra-RAT and Inter-RAT mobility from/to 6G, lossless handover is a baseline.

Proposal 4: 5G-6G mobility is supported by 6G inter-RAT mobility mechanism.

Proposal 5: 6GR IDLE/INACTIVE mode mobility considers inter-RAT cell reselection between 6GR and NR.

Proposal 6: RAN2 to study cell reselection mechanism considering AI/ML support of the network/cell.

Proposal 7: RLM/RLF framework in 5G can be a baseline for 6GR RLM/RLF discussion.

# References

[1] 3GPP TR 38.914 V0.2.0 (2025-09), “Study on 6G Scenarios and Requirements (Release 20)”