3GPP TSG-RAN WG2 Meeting #131bis	                              R2-2506901

Prague, Czech Republic, October 13 - October 17, 2025

Agenda item:	9.2.2

Source:	CMCC

Title:	Discussion on Topology 2 for A-IoT

Document for:	Discussion

Introduction

In revised WID for A-IoT, the following objectives are listed [1]:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE-readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE-reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE-reader is only valid within the serving cell. - Resource allocation to the UE-reader shall be via Uu RRC signaling.

In this contribution, we would like to provide our views on the issues related to A-IoT Topology 2.

Discussion

2.1 Relationship with R19 Topology 1

In Release 19, A-IoT topology 1 has been specified to support A-IoT. When it starts specifying topology 2 in R20, some clarification is necessary on the relationship between topology 1 and topology 2. To be specific, the objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified. Therefore, it is obvious that the A-IoT-enabled gNBs for R19 and those for R20 are not co-located. In other words, we do not need to consider the scenario where the same gNB supports both Topology 1 and Topology 2 for device 1.

Observation 1: The objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified.

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 is not considered in R20.

2.2 General analysis of topology 2

### 2.2.1 Protocol stack of topology 2

In the revised WID, it has been clearly stated that [1]:

- Specify RRC-based architecture, in coordination with SA2.

It is clear that only RRC-based solution is specified for R20 topology 2. The protocol stack of Topology 2 is given in Figure 1, where the design of A-IoT radio interface in R19 is fully reused and UE-reader communicates with A-IoT enabled gNB via the control plane.

Figure 1. Protocol stack of Topology 2.

Observation 2: Only RRC-based solution is specified for R20 topology 2.

Proposal 2: The protocol stack of Topology 2 as shown in Figure 1 can serve as the baseline.

### 2.2.2 Uu Enhancement for A-IoT

#### 2.2.2.1 Radio bearer for transmitting A-IoT related message(s)

For Topology 2, it is essential to study the necessary Uu enhancements to support A-IoT operations of the UE-reader. This includes transmitting inventory or command requests to the UE-reader, allocating or releasing of A-IoT radio resources, and reporting of inventory reports or command responses. Only UE-readers in RRC_CONNECTED state are considered for these operations, which ensures that network has an active UE context and is capable of performing inventory or command operations reliably, avoiding unnecessary signaling attempts for idle or inactive UE.

Considering that messages related to inventory and command procedures are delivered between the gNB and the UE-reader, RAN2 should specify an appropriate radio bearer between the UE-reader and the gNB to carry those messages. Since only RRC based solution is in WID, only SRB should be considered. Further considering that A-IoT use cases typically do not have stringent latency requirements, a low-priority SRB, such as SRB3 or SRB4/5 (currently used for QoE measurement reporting), seems to be suitable. From our point of view, this approach balances implementation simplicity with sufficient reliability for the A-IoT message exchange. While, we are also open to discuss whether a new SBR dedicated for A-IoT is needed.

Proposal 3: SRB is used to carry all A-IoT messages between the UE-reader and the gNB. FFS to reuse the legacy SRB or design a new SRB

#### 2.2.2.2 A-IoT related message(s)

From our perspective, there are several types of A-IoT related messages that need to be specified between UE-reader and gNB. The Inventory/Command Trigger message, which is sent from the gNB to the UE-reader to initiate inventory or command procedure. This message may also include the configuration of A-IoT radio resources, so that the UE-reader can perform the corresponding procedure directly. The A-IoT Resource Request/Release message, through which the UE-reader can request additional resources if needed, or release excess resources if UE-reader completes the inventory/command procedure in advance. Another one is the Inventory/Command Response message, which is sent by the UE-reader to report the inventory reports or command responses collected from A-IoT device to the gNB.Besides, we think that it is also necessary to request UE-reader to report the inventory reports and/or command responses to gNB before handover/RRC state transition with Inventory/Command Response Request message.

Proposal 4: At least the following A-IoT related messages between UE-reader and gNB should be specified:

- gNB->UE-reader: Inventory/Command Trigger message, to initiate the inventory or command procedure for UE-reader, it may also contain the configuration of A-IoT radio resource.

- gNB->UE-reader: A-IoT Radio Resource Allocation/Release message, to allocate or release A-IoT radio resource of the UE-reader.

- gNB-UE-reader: Inventory/Command Response Request message, to request UE-reader to report the inventory reports and/or command responses to gNB, before, e.g., HO or RRC state transition.

- UE-reader->gNB: A-IoT Radio Resource Request/Release Request message, to request to allocate or release A-IoT radio resource by UE-reader.

- UE-reader->gNB: Inventory/Command Response message, to report the inventory reports and/or command responses to gNB.

#### 2.2.2.3 Dealing with inventory reports/command responses

With the consideration that forwarding inventory reports/command responses device by device from the UE-reader may cause significant signalling and energy overhead, it is recommended that the UE-reader can cache some inventory reports/command responses and report them to the gNB in a consolidated manner at an appropriate time.

Besides, another important issue to be discussed is how the UE-reader handles the collected D2R upper-layer data (i.e., inventory reports/command responses) during RLF case, i.e., whether to discard it. From our perspective, the collected inventory reports/command responses are buffered in RRC entity before composing the RRC signaling and deliver it to the lower layers. For the inventory reports/command responses that still buffered in RRC entity during RLF, UE-reader can maintain them.

In terms of the inventory reports/command responses which have been delivered to PDCP/RLC/MAC layers, we have the following observation. In legacy NR, for SRBs, when PDCP entity is re-established, all PDCP PDUs and SDUs are discarded. For inventory report and command response, we do not think they are important enough to require modifying the behavior of PDCP entity re-establishment. Besides, it is also possible that UE-reader selects the suitable cell wihch is a LTM candicated cell and executes LTM successfully. In this case, RLC entity or PDCP entity may not be re-established, so the inventory report and command response may not be discarded. Therefore, there is no need to ensure that inventory reports or command responses are not discarded.

Finally, if UE-reader can not find a suitable cell and enters RRC_IDLE after RLF is declared, just like releasing security keys, measurement reports, etc., UE-reader should release all collected D2R upper-layer data directly. Therefore, we think there is no need to ensure that inventory reports or command responses are not discarded during RLF.

Proposal 5: UE-reader is allowed to buffer and concatenate multiple messages (i.e. inventory reports/command responses) from A-IoT device in RRC entity.

Proposal 6: During RLF, UE-reader can maintain the inventory reports/command responses that still buffered in RRC entity.

Proposal 7: During RLF, for the inventory reports/command responses which have been delivered to lower layers (i.e. PDCP/RLC/MAC/PHY layers), there is no need to ensure that they are not discarded.

#### 2.2.2.4 Overall procedure

Based on above analysis, an example of the basic procedure of Topology 2 is given in Figure 2, and RAN2 can further study whether/what additional messages may be needed. In general, the inventory and command procedures between UE-reader and A-IoT devices should reuse Rel-19 mechanisms as much as possible. This allows implementation to leverage existing procedures, reduces the need for new signaling definitions, and maintains backward compatibility. In terms of the Uu, RAN2 is asked to discuss the necessary enhancement as introduced in section 2.2.2.1, 2.2.2.2 and 2.2.2.3. To be specific, the basic procedure shown in Figure 2 includes:

- gNB transmits the Inventory/Command Trigger message, which includes the configuration of A-IoT radio resource, to initiate the inventory or command procedure for UE-reader;

- UE-reader initiates inventory/ command procedure in accordance with that in the R19;

- If the allocated A-IoT radio resource is not sufficient, UE-reader transmits the A-IoT Radio Resource Request message to request additional A-IoT radio resource from the gNB;

- In response to A-IoT Radio Resource Request message, the gNB transmits the A-IoT Radio Resource Allocation message to UE-reader to allocate additional A-IoT radio resource;

- UE-reader continues the inventory/ command procedure with the allocated additional A-IoT radio resource;

- After the inventory/ command procedure is completed, UE-reader transmits the Inventory/Command Response message to report the inventory reports and/or command responses to gNB.

Figure 2: Example of inventory or command A-IoT devices in Topology 2.

Proposal 8: Reuse Rel-19 inventory and command procedures for the A-IoT radio interface between the UE-reader and the A-IoT device(s).

2.3 Resource of A-IoT radio interface

### 2.3.1 Resource allocation for UE-reader

The A-IoT radio resource includes both RA resources and data transmission resources. For topology 1, the paging message contains the RA resource for A-IoT device to perform access and the resource to transmit Msg3 and D2R data are allocated in R2D message by the gNB reader. As to topology 2, the A-IoT radio between the UE-reader and A-IoT device should also be controlled by network. To avoid interference from UE-readers which are close in distance, the D2R and R2D resource allocation are UE specific. That's to say, the shared D2R and R2D transmission resources are not considered.

In Topology 1, the interference between gNB-readers is not severe, since the gNB generally has good interference coordination capability based on network implementation. However, in Topology 2, as one UE-reader can not obtain any information of other UE-readers, efficient interference coordination among UE-readers becomes really challenging. Therefore, it is important to avoid interference among UE-readers when allocating A-IoT radio resources by gNB. Dedicated and orthogonal resource allocation is recommended so that no resources are shared between UE-readers, preventing collisions and ensuring predictable performance.

What’s more, scheduling and priority handling over A-IoT radio resource should ensure that messages triggered by the gNB are processed promptly. For other messages or lower-priority operations, a default FIFO/FILO treatment can be assumed.

Proposal 9: A-IoT radio resources for A-IoT paging, A-IoT access and upper-layer data transmission are configured by gNB simultaneously.

Proposal 10: The D2R and R2D resource allocation are UE-reader specific, the shared D2R and R2D transmission resources among multiple UE-readers are not considered.

Proposal 11: A-IoT radio resources for UE-readers should be allocated in an orthogonal manner, such that the resources are not shared with or interfered by other UEs.

Proposal 12: Study whether prioritization is applied in A-IoT radio resource allocation for Topology 2, e.g., whether certain type of service (e.g. sensor) has higher priority than others (e.g. inventory, command).

### 2.3.2 Validity of A-IoT radio resources during mobility

As mentioned in the WID, the UE-reader's A-IoT resource is valid only within the serving cell. For the scenario that the UE-reader handovers to another cell, where the A-IoT service has been started or while it is already in the middle of inventory or command procedure when the UE-reader receives HandoverCommand, should be discussed. From our perspective, some interaction between the UE-reader and the gNB before the handover is necessary and valuable to lower adverse impact of the handover procedure on the A-IoT radio. For example, the inventory report or command response that has been buffered at UE side may be triggered to transmit to the source gNB before handover. For the inventory case, although the procedure may be still on going, the partial result is still beneficial. Because the motivation of A-IoT inventory service request is to find the target device(s), the reported device IDs can be identified in core network and subsequent command can be handled toward them.

Taking the basic handover procedure into account, the UE-reader reports measurement results so that the network can decide whether to trigger a handover procedure. Once the UE-reader receives the HandoverCommand, which contains the configuration of target gNB, from the source gNB, it begins accessing the target gNB. Before sending this HandoverCommand, the source gNB may request the UE-reader to report any partial inventory or command feedback. Moreover, the gNB can implicitly instruct the UE-reader, within the HandoverCommand, to terminate the A-IoT service and release the A-IoT resource by no A-IoT radio resource configuration of target gNB included in HandoverCommand.

Proposal 13: Support A-IoT service continuity (release/suspend/continue) for UE-reader HO case.

Proposal 14: The gNB can request the UE-reader to report any partial inventory report or command response before HO.

Proposal 15: Support network to terminate the A-IoT service and release the A-IoT resource, e.g., gNB can implicitly instruct the UE-reader to do this within the HandoverCommand.

### 2.3.3 Validity of A-IoT radio resources during RLF

- Since it has been clarified that A-IoT radio resource of the UE-reader is only valid within the serving cell in the WID and it is possible for the UE-reader to change the serving cell when RLF occurs, for example, the UE-reader the UE has successfully completed RRC re-establishment in a new cell that was not its last serving cell. Therefore, one important issue to be discussed is the validity of A-IoT radio interface resource. During the R19 SI, the following scenarios are discussed [2]:

- Scenario 1: In RRC_CONNECTED, the UE-reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: - Option 1: UE-reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or - Option 2: UE-reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE-reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration. - Scenario 2: In RRC_CONNECTED, the UE-reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE-reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE-reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

- Considering the mobility nature of UE-reader, we think it is more suitable to suspend the A-IoT radio interface resource when RLF occurs (i.e., Scenario 2, Option 1). As for the starting time for UE-reader suspending the A-IoT radio interface resource, there are following two options:

- Option 1: when UE declares RLF;

- Option 2: when UE selects a new suitable cell which is different from the serving cell where it declares RLF or T311 expires.

- For us, we prefer it to be aligned with the time for UE-reader declaring RLF (i.e., Option 1) to avoid the potentialinterference as much as possible.

- In terms of whether the A-IoT radio interface resource can be resumed, the following cases are discussed.

- Case 1: UE-reader can not find a suitabe cell within a certain time after RLF was declared.

- In this case, the UE-reader enters RRC_IDLE as specified in TS 38.300, therefore it should release the A-IoT radio interface resource.

- Case 2: UE-reader can find a suitable cell, and the network can find and verify a valid UE context after RLF declared.

- In this case, if the configuration of A-IoT radio interface is determined to be added to UE context by RAN3, then the UE-reader can be instructed to resume the A-IoT radio interface resource in the latter selected cell.

- Case 3: UE-reader can find a suitable cell, but the network cannot find or verify a valid UE context after RLF declared.

- In this case, when UE-reader enters RRC_CONNECTED again, we are open to discuss whether it can request to resume/reconfig the A-IoT radio interface resource from latter selected cell.

Proposal 16: RAN2 is asked to determine that when UE-reader declares RLF, it suspends the A-IoT radio interface resource at the same time.

Proposal 17: RAN2 is asked to discuss the following three cases when determining whether UE-reader can resume the suspended A-IoT radio interface resource after RLF declariation:

- Case 1: UE-reader can not find a suitabe cell within a certain time after RLF was declared.

- Case 2: UE-reader can find a suitable cell, and the network can find and verify a valid UE context after RLF declared.

- Case 3: UE-reader can find a suitable cell, but the network cannot find or verify a valid UE context after RLF declared.

2.4 Behavior of UE-reader when RRC state transits

- In addition to RLF, there are also other events (such as dataInactivityTimer expires, handover failure etc.) which may cause the RRC state transition of UE-reader. Aligned with the behavior described in RLF case, the following principle can be summarized:

- When UE-reader transits from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE:

- it releases the A-IoT radio interface resource; and,

- it discards the inventory report and command response (if any).

Proposal 18: When UE-reader transits from RRC_CONNECTED state to RRC_IDLE/RRC_INACTIVE state:

- it releases the A-IoT radio interface resource; and,

- it discards the inventory report and command response (if any).

2.5 ID related issue

### 2.5.1 Transaction ID

The transaction ID contained in the A-IoT paging message is a service request related ID. That is, from the devices' perspective, the transaction ID can be used to identify the A-IoT service request. When the device receives the A-IoT paging message with same transaction ID, the device may initiate re-access, if it is failed to access before; if the transaction ID in the paging message is new, the device responds to the new service request. In topology 1, The transaction ID can be generated by reader based on CN corelation ID. While, in topology 2, the transaction ID generation should be further discussed. At least, UE-reader generation or gNB coordination can be considered. For the case that the UE-reader generates the transaction ID based on the correlation ID, it seems to be aligned with the mechanism in topology 1. However, considering the mobility of UE-reader, the transaction ID may be conflicted and interference. For the case of gNB coordination, the UE-reader connected to same gNB can avoid using same transaction ID at same time. In addition, the transaction ID allocation can be along with the A-IoT service request delivery via SRB.

Proposal 19: For topology 2, the following two alternatives can be considered to generate the transaction ID:

- Alt 1: UE-reader generates based on the corelation ID;

- Alt 2: gNB coordinates among UE-readers connected to it.

### 2.5.2 AS ID

The AS ID is allocated in Msg2/R2D Upper Layer Data Transfer message, and can be carried in the subsequent R2D message to address the device. In R19, during A-IoT CBRA, the AS ID can be indicated to reuse the RN16 in Msg1 if no collisions or can be re-allocated by the reader. If such AS ID acquirement way is reused in topology 2, collision and interference issues should be considered for UE-reader. Different from the transaction ID, the number of AS ID may be very large for inventory service and the AS ID allocation is about to real-time. It is not an efficient way for the device to request an AS ID from the gNB during the RA procedure. Thus, the follow 2 alternatives can be considered to avoid the AS ID collision issue: 1) UE-reader implementation based, i.e., with UE identification as part of the AS ID; 2) gNB allocation based, i.e., pre-configured AS ID group,i.e. gNB specifies the range within which the UE-reader can generate the AS ID.

Proposal 20: The following 2 alternatives can be considered to generate the AS ID:

- Alt 1: UE-reader implementation based, i.e., with UE identification as part of the AS ID;

- Alt 2: gNB allocation based, i.e., pre-configured AS ID group.

Conclusions

Based on the discussions mentioned above, in this contribution we provide some discussions on A-IoT Topology 2:

Observation 1: The objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified.

Observation 2: Only RRC-based solution is specified for R20 topology 2.

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 is not considered in R20.

Proposal 2: The protocol stack of Topology 2 as shown in Figure 1 can serve as the baseline.

Proposal 3: SRB is used to carry all A-IoT messages between the UE-reader and the gNB. FFS to reuse the legacy SRB or design a new SRB

Proposal 4: At least the following A-IoT related messages between UE-reader and gNB should be specified:

- gNB->UE-reader: Inventory/Command Trigger message, to initiate the inventory or command procedure for UE-reader, it may also contain the configuration of A-IoT radio resource.

- gNB->UE-reader: A-IoT Radio Resource Allocation/Release message, to allocate or release A-IoT radio resource of the UE-reader.

- gNB-UE-reader: Inventory/Command Response Request message, to request UE-reader to report the inventory reports and/or command responses to gNB, before, e.g., HO or RRC state transition.

- UE-reader->gNB: A-IoT Radio Resource Request/Release Request message, to request to allocate or release A-IoT radio resource by UE-reader.

- UE-reader->gNB: Inventory/Command Response message, to report the inventory reports and/or command responses to gNB.

Proposal 5: UE-reader is allowed to buffer and concatenate multiple messages (i.e. inventory reports/command responses) from A-IoT device in RRC entity.

Proposal 6: During RLF, UE-reader can maintain the inventory reports/command responses that still buffered in RRC entity.

Proposal 7: During RLF, for the inventory reports/command responses which have been delivered to lower layers (i.e. PDCP/RLC/MAC/PHY layers), there is no need to ensure that they are not discarded.

Proposal 8: Reuse Rel-19 inventory and command procedures for the A-IoT radio interface between the UE-reader and the A-IoT device(s).

Proposal 9: A-IoT radio resources for A-IoT paging, A-IoT access and upper-layer data transmission are configured by gNB simultaneously.

Proposal 10: The D2R and R2D resource allocation are UE-reader specific, the shared D2R and R2D transmission resources among multiple UE-readers are not considered.

Proposal 11: A-IoT radio resources for UE-readers should be allocated in an orthogonal manner, such that the resources are not shared with or interfered by other UEs.

Proposal 12: Study whether prioritization is applied in A-IoT radio resource allocation for Topology 2, e.g., whether certain type of service (e.g. sensor) has higher priority than others (e.g. inventory, command).

Proposal 13: Support A-IoT service continuity (release/suspend/continue) for UE-reader HO case.

Proposal 14: The gNB can request the UE-reader to report any partial inventory report or command response before HO.

Proposal 15: Support network to terminate the A-IoT service and release the A-IoT resource, e.g., gNB can implicitly instruct the UE-reader to do this within the HandoverCommand.

Proposal 16: RAN2 is asked to determine that when UE-reader declares RLF, it suspends the A-IoT radio interface resource at the same time.

Proposal 17: RAN2 is asked to discuss the following three cases when determining whether UE-reader can resume the suspended A-IoT radio interface resource after RLF declariation:

- Case 1: UE-reader can not find a suitabe cell within a certain time after RLF was declared.

- Case 2: UE-reader can find a suitable cell, and the network can find and verify a valid UE context after RLF declared.

- Case 3: UE-reader can find a suitable cell, but the network cannot find or verify a valid UE context after RLF declared.

Proposal 18: When UE-reader transits from RRC_CONNECTED state to RRC_IDLE/RRC_INACTIVE state:

- it releases the A-IoT radio interface resource; and,

- it discards the inventory report and command response (if any).

Proposal 19: For topology 2, the following two alternatives can be considered to generate the transaction ID:

- Alt 1: UE-reader generates based on the corelation ID;

- Alt 2: gNB coordinates among UE-readers connected to it.

Proposal 20: The following 2 alternatives can be considered to generate the AS ID:

- Alt 1: UE-reader implementation based, i.e., with UE identification as part of the AS ID;

- Alt 2: gNB allocation based, i.e., pre-configured AS ID group.

References

- RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2, RAN#109.

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR (Release 19), V19.0.0.