3GPP TSG-RAN WG2 Meeting #131bis     	                       R2-2506909

Prague, CZ, 13th-17th October, 2025

Agenda item:	10.3.3

Source:	CMCC

Title:	Consideration on general AI/ML framework and data collection

WID/SID:	FS_6G_Radio

Document for:	Discussion

- Introduction

Based on 6G SID [1], it can be observed that AI/ML data collection, management and transfer are crucial component of 6G.

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

As per Chair lady’s guidance, this contribution will discuss the general AI/ML framework considerations, collection and transfer of various type of data (including AI/ML data, sensing, etc. ).

- Discussion

- 2.1 The existing mechanism in 5G-A

During the study on 5G NR and AI integration, a unified Life Cycle Management (LCM) for AI/ML for NR air interface is defined, as illustrated in Fig.1 [2], which includes data collection, model training, management, inference, etc..

Figure 1. Functional framework for AI/ML for NR Air Interface

However, the existing 5G-A network architecture remains unchanged to support the enhancements on AI/ML, which has limitations on AI data collection and model transfer/delivery.

- Data Collection and transmission

For the transmission of collected data, the legacy CP/UP solutions have the following restrictions:

- The existing SRB cannot transmit the large volumes of data for model training (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes). If a large amount of AI data needs to be transmitted using SRB, the current solution is to use multiple RRC messages, which will lead to signaling overload.

- The existing DRB cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. Therefore, the large volumes of data collected by the UE cannot be transmitted to the gNB directly via DRB.

On the other hand, the data for model training is collected per use case (e.g. beam management, mobility, CSI prediction) and per model type (e.g. UE-sided model, NW-sided model), which can lead to duplicated data collection and reporting, especially for basic measurement results. For example, L1 beam measurement results can be used for both AI/ML based beam management and AI/ML for mobility use cases. For beam management use case, L1-RSRPs and/or beam-IDs need to be collected by UE for both NW-sided model and UE-sided model training, therefore, the duplicated data collection within UE and duplicated reporting via air interface would occur if the standardized solution is adopted for UE side data collection.

- Model transfer/delivery

For model transfer/delivery from gNB to UE, especially for use cases with two-sided model, the challenges are similar to data transmission:

- The existing SRB cannot transmit the large model (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes).

- The existing DRB cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. Therefore, the model trained at gNB cannot be transferred to the UE directly via DRB.

Observation 1: There are some challenges for AI data collection and model transfer/delivery in 5G-A, including:

- Limitations on transmission of large volume of AI data and AI/ML model

- Duplicated data collection and reporting, such as the same measurement results for different use cases, or the similar collected data for NW-sided and UE-sided model training

- 2.2 Discussion on AI/ML framework

- In 5G-A, various use cases have been studied with the model deployed at UE side or NW side (i.e. one-sided model), or both (i.e. two-sided model). The unified LCM framework in [2], which provides a general functional architecture for both model-ID-based LCM and functionality-based LCM, is also applied for both one-sided and two-sided model.

- Taking 5G-A unified LCM framework as baseline, a unified LCM for different granularity, different use cases, or different model types should be supported in 6G.

- Proposal 1: 6G RAN should support a unified LCM, and take LCM in 5G-A as starting point.

- 2.3 Discussion on data collection

As we mentioned above, there are some challenges for AI data collection and model transfer/delivery in 5G-A, thus 6G RAN should resolve the issues from Day 1, i.e. 6G RAN should support the transmission of large volume of AI data (e.g. for model training) and model transfer/delivery to ensure the smooth operation of AI applications. Moreover, it is crucial to avoid duplicated data collection across different use cases or model types in order to minimize UE power consumption and signaling overload.

Proposal 2: 6G RAN should support large volume of data transmission (e.g. for model training) and model transfer/delivery, as well as avoid duplicated data collection and reporting.

Regarding how to support large volume of AI data and model transfer/delivery, the following potential solutions can be further considered and studied:

- Option 1: Extend RRC message size, including UL and DL

- Option 2: Introduce a new radio bearer (e.g. AI bearer) which is terminated between UE and base station for data/model transmission

For option1, it can be considered to extend the size of single RRC message, or extend the number of RRC segmentation.

For option 2, the new radio bearer is dedicated for AI data or model transmission, which is separate from legacy DRB and SRB.

The potential solutions and details can be further discussed and studied.

Proposal 3: It is proposed to study the following potential solutions for large volume of data transmission and model transfer/delivery.

- Option 1: Extend RRC message size

- Option 2: Introduce a new radio bearer (e.g. AI bearer) which is terminated between UE and base station

Furthermore, in addition to large volume of data/model transmission, the diversified data types introduced by 6G new applications/services must also be taken into account. According to the 6G SID [1], 6G data transfer design should support the various types of data, including AI/ML, sensing, etc. Data collection for AI/ML has been discussed in 5G-A with multiple AI/ML enabled use cases, while data collection for sensing still requires further discussion.

For sensing scenario, 6G is expected to support all sensing modes as illustrated in Fig.2, including both mono-static and bi-static with both TRP and UE involved to explore more use cases for improvement of sensing performance.

Figure 2. Sensing mode

In our understanding, the following data types can be considered for sensing service:

- Sensing raw data (e.g., IQ data): It contains all the information obtained by an ISAC system without signal processing, including time-domain samples or sub-carrier-domain samples of original received signals from each antenna port.

- Delay/angular/Doppler spectrum: Spectrum is the first-stage processing output derived from sensing raw data. It could retain most essential information while significantly reducing data volume compared to the raw data. Delay spectrum, angular spectrum and Doppler spectrum are fundamental for most of the sensing algorithms.

- Cloud of points: It is extracted by implementing detection algorithms on delay, angular and Doppler spectrum. This stage typically discards most of the information, resulting in a relatively low data volume while preserving critical parameters.

- Sensing result: It is the final output of sensing signal processing. A sensing result is described by target’s attributes, including location, velocity and type, which has the lowest data volume.

Since ISAC is led by RAN1, the details of data type and data content need RAN1’s inputs for further discussion. However, it can be observed that the volume of sensing data is substantial.

- Observation 2: RAN1’s inputs are needed for the details of data type and data content of sensing.

To meet the diversified data collection requirements for different services or use cases and simplify the data collection management, it is crucial to consider both the efficiency and complexity of data collection in 6G. Therefore, a unified RAN data collection framework is essential, which should support diverse data collection from 6G new services, such as AI and sensing.

Proposal 4: A unified RAN data collection framework should be supported for diversified data collected from 6G new services, e.g. AI and sensing.

- Furthermore, the requirements or principles on data collection should be discussed in 6G. During the discussion on AI PHY in 5G-A, the following requirements were agreed for UE side data collection in the way forward [3] in RAN#105 meeting, including data security, full controllability and visibility by MNO.

Requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

Moreover, the definition of full controllability and visibility in TR 38.843 [2] are listed as follows for information.

- Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer.

- Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA.

- For 6G RAN, we believe that the above requirements can be taken as a good starting point for UE side data collection.

- Proposal 5: The following requirements in 5G-A can be taken as starting point for AI data collection and transfer in 6G:

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose. This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

Given the large amount of data collected in 6G, requiring user consent for all collected data will significantly prolong the time for network optimization, AI service provision and 6G commercial deployment. Therefore, it is expected to discuss which kind of data really requires user consent.

During the discussion in 5G-A, SA3 agreed that the user consent requirements can be configured by OAM. It means the operator can configure which data requires user consent and which data does not, thereby ensuring 3GPP system is in compliance with local laws and regulations. The details are specified in section 4.9 of TS 32.422 [4] and highlighted as follows. In our understanding, the user consent should be respected for UE privacy information (e.g. UE location), but it is unnecessary for legacy L1/L3 measurement results since they can be obtained within the existing RAN framework without user consent actually.

4.9 User consent handling in MDT for NG-RAN 4.9.1 Signalling based MDT In case of signalling based MDT, getting user consent before activating the MDT functionality is required because of privacy and legal obligations. It is the Operator’s responsibility to collect user consent before initiating an MDT for a specific IMSI, IMEI number or SUPI. Collecting the user consent shall be done via customer care process. The user consent information availability should be considered as part of the subscription data and as such this shall be provisioned in the UDM database. The following figure summarizes the functionality. Figure 4.9.1.1: Example for delivering user consent information in signalling based MDT When the IMSI/IMEI(SV)/SUPI based MDT is activated, it is targeted the UDM. The UDM shall check the MDT user consent requirements configured by OAM. If one or more MDT measurement name(s) is/are specified in MDT activation, the UDM shall: - For user-consent-required MDT measurement name(s), start a Trace Session for the given subscriber if the user consent is available. - For user-consent-required MDT measurement name(s), not start a Trace Session for the given subscriber if the user consent is not available. - For user-consent-not-required MDT measurement name(s), start a Trace Session for the given subscriber without considering user consent. <omitted>

For data collection in 6G, the same principle can be followed that user consent can be configured by OAM.

Proposal 6: It is proposed to follow 5G-A mechanism that the user consent can be configured by OAM.

For RAN-centric use cases, we understand that data (e.g. the measurement results in air interface) should be collected, managed and applied solely within RAN side, and there is no need to send all collected data to CN. That is, 6G RAN and CN can maintain their own data repositories, wherein the data collected and used solely within RAN side can be managed and stored in a RAN data repository. Of course, 6G RAN can also share its data to other data consumers if needed, such as CN or OAM.

Furthermore, the RAN data repository can also coordinate data collection request from multiple data consumers to minimize the duplicated data collection and reporting. From the perspective of data sharing, we believe that the RAN data repository owned by MNO is reasonable, since it is secured and can support interoperability across different vendors.

Figure 3. An example for RAN data repository

- Proposal 7: 6G RAN should maintain its own data repository, where data collected and used solely within RAN side can be stored and managed. Furthermore, the data can be shared to different data consumers (e.g. OAM, CN) if needed.

# 3	Conclusion

Here are the observations and proposals for AI/ML framework and data collection in 6G.

Observation 1: There are some challenges for AI data collection and model transfer/delivery in 5G-A, including:

- Limitations on transmission of large volume of AI data and AI/ML model

- Duplicated data collection and reporting, such as the same measurement results for different use cases, or the similar collected data for NW-sided and UE-sided model training

Observation 2: RAN1’s inputs are needed for the details of data type and data content of sensing.

- Proposal 1: 6G RAN should support a unified LCM, and take LCM in 5G-A as starting point.

Proposal 2: 6G RAN should support large volume of data transmission (e.g. for model training) and model transfer/delivery, as well as avoid duplicated data collection and reporting.

Proposal 3: It is proposed to study the following potential solution for large volume of data transmission and model transfer/delivery.

- Option 1: Extend RRC message size

- Option 2: Introduce a new radio bearer (e.g. AI bearer) which is terminated between UE and base station

Proposal 4: A unified RAN data collection framework should be supported for diversified data collected from 6G new services, e.g. AI and sensing.

- Proposal 5: The following requirements in 5G-A can be taken as starting point for AI data collection and transfer in 6G:

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose. This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

Proposal 6: It is proposed to follow 5G-A mechanism that the user consent can be configured by OAM.

Proposal 7: 6G RAN should maintain its own data repository, where data collected and used solely within RAN side can be stored and managed. Furthermore, the data can be shared to different data consumers (e.g. OAM, CN) if needed.

# 4 Text Proposal

Based on the discussion above we propose the following text proposal.

## X.X AI/ML framework and data collection

### x.x.x AI/ML framework

6G RAN should support a unified LCM, and take LCM in 5G-A as starting point.

Figure X. Functional framework for AI/ML for NR Air Interface

### x.x.x Data collection

A unified RAN data collection framework should be supported for diversified 6G new services, e.g. AI and sensing. Furthermore, 6G RAN should support large volume of data transmission (e.g. for model training) and model transfer/delivery, as well as avoid duplicated data collection and reporting. The potential solutions should be further studied.

- For data collection and transfer, the following requirements can be taken as starting point:

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected, where user consent can be configured by OAM.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose. This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

6G RAN should maintain its own data repository, where data collected and used solely within RAN side can be stored and managed. Furthermore, the data can be shared to different data consumers (e.g. OAM, CN) if needed.

# 5	Reference

- RP-252912 Revised SID: Study on 6G Radio

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-242351 Moderator's summary on offline discussion about RAN2 aspects on REL-19 WI NR_AIML_air

- TS 32.422 Telecommunication management; Subscriber and equipment trace; Trace control and configuration management